3GPP TSG-RAN #95-e	RP-220313

# Online, 17-23 March 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 44.318 | CR | 0128 | rev | 1 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Inclusive language review |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17 |  |  |  |  |  | Date: |  |  | 2022-03-09 |
| Category: | D |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | RAN#90 agreed to support the usage of inclusive language in 3GPP and suggested the replacement of related terms in 3GPP specifications which can be understood as non-inclusive (RP-202179). |  |  |  |  |  |  |  |  |
| Summary of change: |  | The term "Black List" (and derivations thereof) have been replaced by the term "Exclude List". Field names and names of information elements have also been updated accordingly. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification will contain non-inclusive language. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1, 6.2.3.3, 6.4.4, 10.1.8, 10.1.8.2, 10.1.9, 10.1.9.1, 11.2, 11.2.58, 12.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from RP-210335, postponed at RAN #91-e. |  |  |  |  |  |  |  |  |

First change

### 6.2.1	Registration initiation by the MS

This procedure shall be triggered by the MS under the following conditions:

-	the MS has gained IP connectivity (i.e. an IP address is allocated to the MS in the "transport IP" layer); and

-	the MS has stored information about a Default GANC.

The Registration procedure is initiated towards the Default GANC after a successful Discovery procedure or after a failed Registration towards a Serving GANC, where no GAN PLMN list was provided to the MS from the Default GANC. The Registration procedure is also initiated towards the Default GANC when no more PLMNs can be selected from the GAN PLMN List received from the Default GANC.

As part of the Registration procedure, the MS may receive a list of PLMN identities when trying to register towards the Default GANC. This list of PLMN identities is passed to the GANC selection process. Following PLMN selection by the GANC selection process, and if registered to a Serving GANC, the MS shall first deregister from the current Serving GANC as defined in sub-clause 6.4.1, and then attempt the registration procedure towards the Serving GANC-SEGW and GANC associated with the selected PLMN.

This procedure is also triggered towards the Default GANC, if the MS at any time wishes to perform manual PLMN selection or "User reselection" irrespective of whether the MS is in manual or automatic PLMN selection mode. If the MS is already successfully registered to a Serving GANC, and a manual PLMN selection or "User reselection" is initiated, the MS shall first deregister from the current Serving GANC as defined in sub-clause 6.4.1 and then initiate registration towards the Default GANC and include an indication that a list of PLMN identities is requested for manual PLMN selection or user reselection. After receiving the requested PLMN list from the Default GANC the MS shall then re-register on the last registered Serving GANC.

GANC information required to initiate the Registration procedure consists of a GANC-SEGW address, a GANC address and a GANC TCP port number. The MS shall initiate the Registration procedure towards a Serving GANC if the corresponding GANC information is available in the stored Serving GANC table. If successful, the stored Serving GANC table in the MS may be updated to contain address information of the successfully registered Serving GANC and the corresponding GSM-CGI, UTRAN cell identity or E-UTRAN cell identity, if available, or AP-ID, if no GSM CGI,  UTRAN cell identity or E-UTRAN cell identity is available (see sub-clause 6.2.3.1). UTRAN cell identity consists of the Location Area Identity (LAI) and Cell Identity (defined in [40]). E-UTRAN cell identity consists of the Tracking Area Identity (TAI) and Cell Identity (defined in [55]). The Default GANC is in control of whether or not the MS is allowed to store GANC information in the stored Serving GANC table.

If the MS is in GERAN, UTRAN or E-UTRAN coverage, it shall check if it has stored Serving GANC information for the current GSM CGI in case of GERAN coverage, UTRAN cell identity in case of UTRAN Coverage, or E-UTRAN cell identity in case of E-UTRAN coverage, or if the MS is not in GERAN/UTRAN/E-UTRAN coverage, it shall check if it has stored Serving GANC information for the current AP-ID:

-	if found, the MS shall initiate the GAN Registration procedure towards the stored Serving GANC

-	if not found, the MS shall initiate the GAN Registration procedure towards the Default GANC

The Location Exclude List contains information about forbidden Locations acquired while attempting GAN registration and is only valid if the MS is in GERAN or UTRAN coverage. It can contain Location information on 3 different levels: Country (i.e. MCC), PLMN (i.e. MCC and MNC) and Location Area (i.e. MCC, MNC and LAC). If the MS is in a location identified by an entry in the Location Exclude List, the MS shall not initiate the GAN Registration procedure. The Location Exclude List shall be deleted upon power cycle.

The AP Exclude List contains information about denied APs. If the MS is being provided IP connectivity by an AP that is on the AP Exclude List, the MS shall not initiate discovery or GAN registration, until the relevant AP has been removed from the AP Exclude List. The AP Exclude List shall be deleted upon power cycle.

To initiate the GAN registration procedure the MS shall:

-	If the MS has stored an IP address of the GANC-SEGW, and the MS does not already have an established secure connection to this GANC-SEGW, the MS establishes a secure connection towards the GANC-SEGW according to sub-clause 4.2;

-	If the MS has stored a FQDN of the GANC-SEGW, the MS performs a public DNS query to retrieve the IP-address of the GANC-SEGW, if the MS does not already have an established secure connection to this GANC-SEGW, the MS establishes the secure connection towards the GANC-SEGW according to sub-clause 4.2. The MS shall not store the IP address retrieved from DNS for subsequent procedures (apart from DNS resolver caching).

Following successful establishment of secure tunnel:

-	If the MS holds an IP address of the GANC, the MS establishes a TCP connection to the GANC at the stored TCP port to be used for Registration with this GANC. If no TCP port has been stored for this GANC, the default TCP port (see sub-clause 12.2.1) shall be used;

-	If the MS holds a FQDN of the GANC, the MS performs a DNS query "inside the secure tunnel" to retrieve the IP-address of the GANC. The MS establishes a TCP connection to the GANC at the stored TCP port to be used for Registration with this GANC. If no TCP port has been stored for this GANC, the default TCP port (see sub-clause 12.2.1) shall be used. The MS shall not store the retrieved IP address for subsequent procedures (apart from DNS resolver caching);

-	The MS shall only establish a single TCP connection to the GANC over the IPsec tunnel.

After successful establishment of a secure connection to the SEGW and a TCP connection to the GANC, the MS sends a GA-RC REGISTER REQUEST message to the GANC on the TCP connection, including the following information elements:

-	The Geographical Location IE, if available;

-	AP Location IE, if available;

-	Required GAN Services IE, if required;

-	If a GAN PLMN list was received during the last registration attempt with the Default GANC and the GANC selection process is finished unsuccessfully (i.e. the MS was not successfully registered to a Serving GANC and to the associated PLMN in the GAN PLMN List IE), then the MS includes the received GAN PLMN List information in the GAN PLMN List IE in the GA-RC REGISTER REQUEST message;

-	Else if the MS failed to register during the last registration it attempted with a GANC (Default or Serving), then Register reject Cause IE, Redirection Counter IE and address information (i.e. IP Address or the Fully Qualified Domain/Host Name IE) of the last GANC and last GANC-SEGW;

-	Else if the MS was redirected during the last registration it attempted with a GANC (Default or Serving):

-	Redirection Counter IE and address information (i.e. IP address or FQDN) of the last GANC and last GANC-SEGW

When GA-RC layer has submitted the GA-RC REGISTER REQUEST message to the TCP layer, it shall start a timer TU3904

Next change

#### 6.2.3.3	Registration rejected

When the MS receives GA-RC REGISTER REJECT message it shall:

-	stop the timer TU3904,

-	if the MS is in the process of GANC selection;

-	ignore the received Register Reject Cause Information Element; and;

-	proceed as defined in sub-clause 6.2.4.5.

-	else extract the Register Reject Cause information element and act as following depending on the value of the Reject Cause IE:

-	'Network Congestion' 
- create a random value between zero and the received value in IE 'TU3907 Timer' and 
- add this value to the received value in IE 'TU3907 Timer', and use this as the new value for TU3907
- start timer TU3907 according to the new calculated value and wait for it to expire.

-	'AP not allowed'
- release the TCP connection established to the GANC, if still established,
- release the secure connection towards the SEGW associated with the GANC, as defined in sub-clause 4.5
- store the AP-ID in the AP Exclude List and not initiate a new Register Request from this AP, until the AP-ID is removed from the AP Exclude List i.e. as a result of power-cycle.

-	'Location not allowed'
-	release the TCP connection established to the GANC, if still established,
-	release the secure connection towards the SEGW associated with the GANC, as defined in sub-clause 4.5
-	update the Location Exclude List according to the received information elements Location Exclude List indicator and Location Area Identification (i.e. the same as that included in the GA-RC REGISTER REQUEST message) and not initiate a new Register Request from the locations indicated by the updated Location Exclude List, until they are removed from the Location Exclude List i.e. as a result of power-cycle.

-	'IMSI not allowed'
-	release the TCP connection established to the GANC, if still established. 
-	release the secure connection towards the SEGW associated with the GANC, as defined in sub-clause 4.5
-	not initiate a new GAN Registration procedure until the next power-on.

-	'Invalid GANC' or 'Unspecified'
-	release the TCP connection established to the GANC, if still established. 
-	release the secure connection towards the SEGW associated with the GANC, as defined in sub-clause 4.5
-	act as defined in sub-clause 6.2.4.5

-	'Geo Location not known'
-	release the TCP connection established to the GANC, if still established. 
-	release the secure connection towards the SEGW associated with the GANC, as defined in sub-clause 4.5
-	not retry registration from this AP until the location is provided or until the next power-on.

The MS shall also:

-	Update the stored Serving GANC table as follows if the received Reject cause was not 'Network Congestion' or 'Geo Location not known':

-	If the MS is in GERAN/UTRAN/E-UTRAN coverage it shall remove information related to the current GSM CGI, UTRAN cell identity or E-UTRAN cell identity, if it exists in the table.

-	If the MS is not in GERAN/UTRAN/E-UTRAN coverage it shall remove information related to the AP-ID, if exists in the table.

Next change

### 6.4.4	Reception of GA-RC DEREGISTER by MS

When the MS receives the GA-RC DEREGISTER message, it shall:

-	if the MS is in the process of PLMN selection;

-	ignore the received Register Reject Cause Information Element, and

-	proceed as defined in sub-clause 6.2.4.5.

-	else extract the Reject Cause information element and act as following depending on the value of the Reject Cause IE:

-	'Network Congestion'

-	release all local GAN resources (e.g. MS is in active call over GAN)

-	release the TCP connection towards the current GANC and

-	release the secure connection towards the GANC-SEGW, as defined in sub-clause 4.5

-	create a random value between zero and the received value in IE 'TU3907 Timer' and

-	add this value to the received value in IE 'TU3907 Timer', and use this as the new value for TU3907

-	start timer TU3907 according to the new calculated value

-	'AP not allowed'

-	release the TCP connection established to the GANC, if still established,

-	release the secure connection towards the SEGW associated with the GANC, as defined in sub-clause 4.5,

-	store the AP-ID in the AP Exclude List of and not initiate a new Register Request from this AP, until the AP-ID is removed from the AP Exclude List i.e. as a result of power-cycle.

-	'Location not allowed'

-	release the TCP connection established to the GANC, if still established,

-	release the secure connection towards the SEGW associated with the GANC, as defined in sub-clause 4.5,

-	update the Location Exclude List according to the received information elements Location Exclude List indicator and Location Area Identification and not initiate a new Register Request from that Location, until the Location is removed from the Location Exclude List i.e. as a result of power-cycle.

-	'IMSI not allowed'

-	release the TCP connection established to the GANC, if still established.

-	release the secure connection towards the SEGW associated with the GANC as defined in sub-clause 4.5,

-	not initiate a new Registration procedure until the next power-on.

-	'Unspecified'

-	release the TCP connection established to the GANC, if still established.

-	release the secure connection towards the SEGW associated with the GANC as defined in sub-clause 4.5,

-	act as if a "Lower layer failure in the MS" has occurred as defined in sub-clause 6.2.4.2

-	'Invalid GANC'

-	release the TCP connection established to the GANC, if still established.

-	release the secure connection towards the SEGW associated with the GANC as defined in sub-clause 4.5,

-	act as defined in sub-clause 6.2.4.5 "Registration Failure"

-	'Geo Location not known'

-	release the TCP connection established to the GANC, if still established.

-	release the secure connection towards the SEGW associated with the GANC as defined in sub-clause 4.5,

-	not retry registration from this AP until the location is provided or until the next power-on.

The MS shall also,

-	Update the stored Serving GANC table as following if the received Reject cause was not 'Network Congestion' or 'Geo Location not known'

-	If the MS is in GERAN/UTRAN/E-UTRAN coverage

-	Remove information related to the current GSM CGI, UTRAN cell identity or E-UTRAN cell identity, if exists in the table

-	If the MS is not in GERAN/UTRAN/E-UTRAN coverage

-	Remove information related to the AP-ID, if exists in the table.

Next change

### 10.1.8	GA-RC REGISTER REJECT

The GAN uses this message to reject the registration request from a MS.

Direction:	GANC to MS

Table 10.1.8.1: GA-RC REGISTER REJECT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length | Value | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Length Indicator | Length Indicator 11.1.1.1 | M | V | 2 |  |  |
|  | GA-RC Protocol Discriminator | Protocol Discriminator 11.1.1.2 | M | V | ½ | 0000 |  |
|  | Skip Indicator | Skip Indicator 11.1.1.3 | M | V | ½ | 0000 |  |
|  | GA-RC REGISTER REJECT Message Type | Message Type 11.1.1.4 | M | V | 1 |  |  |
| 21 | Register Reject Cause | Register Reject Cause 11.2.21 | M | TLV | 3 |  |  |
| 16 | TU3907 Timer | TU3907 Timer 11.2.16 | C | TLV | 4 |  |  |
| 58 | Location Exclude List indicator | Location Exclude List indicator 11.2.58 | C | TLV | 3 |  |  |
| 5 | Location Area Identification | Location Area Identification 11.2.5 | C | TLV | 7 |  |  |

Next change

#### 10.1.8.2	Location Exclude List indicator

The Location Exclude List indicator IE shall be included if the Register Reject Cause 'Location not allowed' is returned to the MS.

Next change

#### 10.1.9	GA-RC DEREGISTER

The GANC on receiving this message removes all contexts associated with the MS in the GANC. The MS on receiving this message releases all GA-RC, GA-CSR and GA-PSR resources.

Direction: MS to GANC, GANC to MS

Table 10.1.9.1: GA-RC DEREGISTER message content

| IEI | Information Element | Type/Reference | Presence | Format | Length | Value | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Length Indicator | Length Indicator 11.1.1.1 | M | V | 2 |  |  |
|  | GA-RC Protocol Discriminator | Protocol Discriminator 11.1.1.2 | M | V | ½ | 0000 |  |
|  | Skip Indicator | Skip Indicator 11.1.1.3 | M | V | ½ | 0000 |  |
|  | GA-RC DEREGISTER Message Type | Message Type 11.1.1.4 | M | V | 1 |  |  |
| 21 | Deregister Cause | Register Reject Cause 11.2.21 | M | TLV | 3 |  |  |
| 58 | Location Exclude List indicator | Location Exclude List indicator 11.2.58 | C | TLV | 3 |  |  |
| 5 | Location Area Identification | Location Area Identification 11.2.5 | C | TLV | 7 |  |  |
| 16 | TU3907 Timer | TU3907 Timer 11.2.16 | C | TLV | 4 |  |  |

Next change

##### 10.1.9.1	Location Exclude List indicator

The Location Exclude List indicator IE shall be included if the Register Reject Cause 'Location not allowed' is returned to the MS.

Next change

## 11.2	IE Type and Identifiers

Table 11.2.1: IE type and identifiers for Unlicensed Radio Resources management

| IE | Identifier | Reference |
| --- | --- | --- |
| Mobile Identity | 1 | 11.2.1 |
| GAN Release Indicator | 2 | 11.2.2 |
| Radio Identity | 3 | 11.2.3 |
| GERAN Cell Identity | 4 | 11.2.4 |
| Location Area Identification | 5 | 11.2.5 |
| GERAN/UTRAN coverage Indicator | 6 | 11.2.6 |
| GAN Classmark | 7 | 11.2.7 |
| Geographical Location | 8 | 11.2.8 |
| GANC-SEGW IP Address | 9 | 11.2.9 |
| GANC-SEGW Fully Qualified Domain/Host Name | 10 | 11.2.10 |
| Redirection Counter | 11 | 11.2.11 |
| Discovery Reject Cause | 12 | 11.2.12 |
| GAN Cell Description | 13 | 11.2.13 |
| GAN Control Channel Description | 14 | 11.2.14 |
| Cell Identifier List | 15 | 11.2.15 |
| TU3907 Timer | 16 | 11.2.16 |
| GSM RR/UTRAN RRC State | 17 | 11.2.17 |
| Routing Area Identification | 18 | 11.2.18 |
| GAN Band | 19 | 11.2.19 |
| GA-RC/GA-CSR/GA-PSR State | 20 | 11.2.20 |
| Register Reject Cause | 21 | 11.2.21 |
| TU3906 Timer | 22 | 11.2.22 |
| TU3910 Timer | 23 | 11.2.23 |
| TU3902 Timer | 24 | 11.2.24 |
| L3 Message | 26 | 11.2.26 |
| Channel Mode | 27 | 11.2.27 |
| Mobile Station Classmark 2 | 28 | 11.2.28 |
| RR Cause | 29 | 11.2.29 |
| Cipher Mode Setting | 30 | 11.2.30 |
| GPRS Resumption | 31 | 11.2.31 |
| Handover From GAN Command | 32 | 11.2.32 |
| UL Quality Indication | 33 | 11.2.33 |
| TLLI | 34 | 11.2.34 |
| Packet Flow Identifier | 35 | 11.2.35 |
| Suspension Cause | 36 | 11.2.36 |
| TU3920 Timer | 37 | 11.2.37 |
| QoS | 38 | 11.2.38 |
| GA-PSR Cause | 39 | 11.2.39 |
| User Data Rate | 40 | 11.2.40 |
| Routing Area Code | 41 | 11.2.41 |
| AP Location | 42 | 11.2.42 |
| TU4001 Timer | 43 | 11.2.43 |
| Location Status | 44 | 11.2.44 |
| Cipher Response | 45 | 11.2.45 |
| Ciphering Command RAND | 46 | 11.2.46 |
| Ciphering Command MAC | 47 | 11.2.47 |
| Ciphering Key Sequence Number | 48 | 11.2.48 |
| SAPI ID | 49 | 11.2.49 |
| Establishment Cause | 50 | 11.2.50 |
| Channel Needed | 51 | 11.2.51 |
| PDU in Error | 52 | 11.2.52 |
| Sample Size | 53 | 11.2.53 |
| Payload Type | 54 | 11.2.54 |
| Multi-rate Configuration | 55 | 11.2.55 |
| Mobile Station Classmark 3 | 56 | 11.2.56 |
| LLC-PDU | 57 | 11.2.57 |
| Location Exclude List indicator | 58 | 11.2.58 |
| Reset Indicator | 59 | 11.2.59 |
| TU4003 Timer | 60 | 11.2.60 |
| AP Service Name | 61 | 11.2.61 |
| GAN Service Zone Information | 62 | 11.2.62 |
| RTP Redundancy Configuration | 63 | 11.2.63 |
| UTRAN Classmark | 64 | 11.2.64 |
| Classmark Enquiry Mask | 65 | 11.2.65 |
| UTRAN Cell Identifier List | 66 | 11.2.66 |
| Serving GANC table indicator | 67 | 11.2.67 |
| Registration indicators | 68 | 11.2.68 |
| GAN PLMN List | 69 | 11.2.69 |
| Required GAN Services | 71 | 11.2.71 |
| Broadcast Container | 72 | 11.2.72 |
| 3G Cell Identity | 73 | 11.2.73 |
| MS Radio Identity | 96 | 11.2.3 |
| GANC IP Address | 97 | 11.2.9 |
| GANC Fully Qualified Domain/ Host Name | 98 | 11.2.10 |
| IP address for GPRS user data transport | 99 | 11.2.9 |
| UDP Port for GPRS user data transport | 100 | 11.2.25 |
| GANC TCP port | 103 | 11.2.25 |
| RTP UDP port | 104 | 11.2.25 |
| RTCP UDP port | 105 | 11.2.25 |
| GERAN Received Signal Level List | 106 | 11.2.70 |
| UTRAN Received Signal Level List | 107 | 11.2.70b |
| PS Handover to GERAN Command | 108 | 11.2.74 |
| PS Handover to UTRAN Command | 109 | 11.2.75 |
| PS Handover to GERAN PSI | 110 | 11.2.76 |
| PS Handover to GERAN SI | 111 | 11.2.77 |
| TU4004 Timer | 112 | 11.2.78 |
| GAN Mode Indicator | 79 | 11.2.79 |
| CN Domain Identity | 80 | 11.2.80 |
| GAN Iu Mode Cell Description | 81 | 11.2.81 |
| 3G UARFCN | 82 | 11.2.82 |
| RAB ID | 83 | 11.2.83 |
| RAB ID List | 84 | 11.2.84 |
| GA-RRC Establishment Cause | 85 | 11.2.85 |
| GA-RRC Cause | 86 | 11.2.86 |
| GA-RRC Paging Cause | 87 | 11.2.87 |
| Intra Domain NAS Node Selector | 88 | 11.2.88 |
| CTC Activation List | 89 | 11.2.89 |
| CTC Description | 90 | 11.2.90 |
| CTC Activation Ack List | 91 | 11.2.91 |
| CTC Activation Ack Description | 92 | 11.2.92 |
| CTC Modification List | 93 | 11.2.93 |
| CTC Modification Ack List | 94 | 11.2.94 |
| CTC Modification Ack Description | 95 | 11.2.95 |
| PTC Activation List | 115 | 11.2.96 |
| PTC Description | 116 | 11.2.97 |
| PTC Activation Ack List | 117 | 11.2.98 |
| PTC Activation Ack Description | 118 | 11.2.99 |
| PTC Modification List | 119 | 11.2.100 |
| PTC Modification Ack List | 120 | 11.2.101 |
| PTC Modification Ack Description | 121 | 11.2.102 |
| RAB Configuration | 122 | 11.2.103 |
| Multi-rate Configuration 2 | 123 | 11.2.104 |
| Selected Integrity Protection Algorithm | 124 | 11.2.105 |
| Selected Encryption Algorithm | 125 | 11.2.106 |
| CN Domains to Handover | 126 | 11.2.107 |
| 3G Security Capability | 74 | 11.2.108 |
| NAS Synchronisation Indicator | 75 | 11.2.109 |
| GANC TEID | 76 | 11.2.110 |
| MS TEID | 77 | 11.2.110 |
| UTRAN RRC Message | 78 | 11.2.111 |
| SRNS Relocation Info | 127 | 11.2.111 |
| MS Radio Access Capability | 128 | 11.2.112 |
| Handover Reporting Control | 129 | 11.2.113 |
| Tracking Area Identity | 130 | 11.2.114 |
| E-UTRAN Cell Identity | 131 | 11.2.115 |
| E-UTRAN Camping Indicator | 132 | 11.2.116 |
| Bandwidth-efficient Mode Indicator | 133 | 11.2.117 |

Next change

### 11.2.58	Location Exclude List indicator

The Location Exclude List indicator information element is used to indicate which part of the Location Area Identification is to be added in the Location Exclude List.

| 8 | 7 | 6 | 5 | 4 |  | 3 | 2 | 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Location Exclude List indicator IEI |  |  |  |  |  |  |  |  |  | octet 1 |  |
| Length of Location Exclude List indicator contents |  |  |  |  |  |  |  |  |  | octet 2 |  |
| spare |  |  |  |  |  | LBLI |  |  |  | octet 3 |  |

Figure 11.2.58.1: Location Exclude List indicator information element

Table 11.2.58.1: Location Exclude List indicator information element

| LBLI, Location Exclude List indicator (octet 3) Bits |  |  |  |
| --- | --- | --- | --- |
| 3 | 2 | 1 |  |
| 0 | 0 | 0 | MCC |
| 0 | 0 | 1 | MCC and MNC |
| 0 | 1 | 0 | MCC, MNC and LAC |
| All other values are undefined in Release 1. |  |  |  |

Next change

### 12.2.3	MS system parameters

| Parameter Name | Value |
| --- | --- |
| Up Register Max Retries | 3 |
| Up Connect Attempt Count | 3 |
| AP Exclude list size minimum | 1 |
| Location Exclude List size minimum | 1 |

| Parameter Name | Initial Value |
| --- | --- |
| Serving GANC Table indicator | "Do Not Store" |