# 3GPP TSG-RAN WG4 Meeting # 102-e 														R4-2206569

# Electronic Meeting, 21 February– 3 March, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.101 | CR | 5851 | rev | 1 | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for 36.101 on LTE REFSENS exception simplification |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_CA_R17_2BDL_1BUL-Core LTE_CA_R17_3BDL_1BUL-Core LTE_CA_R17_xBDL_1BUL-Core LTE_CA_R17_2BDL_2BUL-Core LTE_CA_R17_xBDL_2BUL-Core |  |  |  |  |  | Date: |  |  | 2022-02-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Referring to RAN5 LS R4-2117027, RAN4 has agreed to adopt RAN5’s suggestion option 2a. This CR is to simplify the LTE REFSENS specification and remove the redundant contents. After simplification, the core requirements for higher order CA band combinations are unchanged. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Table; Proposal. Table 7.3.1A-0a; Two band combinations can be kept and others can be removed.. Table 7.3.1A-0b; Two band combinations can be kept and others can be removed.. Table 7.3.1A-0bA; This table can be kept.. Table 7.3.1A-0bB; This table can be kept.. Table 7.3.1A-0bC; This table can be removed.. Table 7.3.1A-0bD; This table can be removed.. Table 7.3.1A-0bD1; This table can be removed.. Table 7.3.1A-0bD2; This table can be removed.. Table 7.3.1A-0bD3; This table can be removed.. Table 7.3.1A-0bD4; This table can be removed.. Table 7.3.1A-0bE; Two band combinations can be kept and others can be removed.. Table 7.3.1A-0bF; Two band combinations can be kept and others can be removed.. Table 7.3.1A-0d; Two band combinations can be kept and others can be removed.. Table 7.3.1A-0eA; Two band combinations can be kept and others can be removed.. Table 7.3.1A-0eC; This table can be kept.. Table 7.3.1A-0eD; This table can be kept.. Table 7.3.1A-0f; No need to further simplify. Table 7.3.1A-0g; No need to further simplify. Table 7.3.1A-0h; No need to further simplify. Table 7.3.1A-1; No need to further simplify. Table 7.3.1A-2; No need to further simplify. Table 7.3.1A-3; No need to further simplify. Table 7.3.1A-4; No need to further simplify. Table 7.3.1A-5; This table can be removed since the fall-back combination with single carrier is covered.. Table 7.3.1A-6; This table can be removed since the fall-back combination with single carrier is covered.. |
| Consequences if not approved: |  | LTE REFSENS simplications are not implemented. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.1A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 36.521 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <<Start of Change>>

### 7.3.1A	Minimum requirements (QPSK) for CA

For inter-band carrier aggregation with one component carrier per operating band and the uplink assigned to one E-UTRA band the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.3.1-1, Table 7.3.1-1a and Table 7.3.1-2. The reference sensitivity is defined to be met with all downlink component carriers active and one of the uplink carriers active. The uplink resource blocks shall be located as close as possible to the primary downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.6-1). The primary downlink operating band is the downlink band of the active uplink operating band. The UE shall meet the requirements specified in subclause 7.3.1 with the following exceptions.

For the bands supporting 4 antenna ports which are in Table 7.3.1-1a, the minimum requirements for reference sensitivity in the reference sensitivity exception tables shall be modified by the amount given in ΔRIB,4R in Table 7.3.1-1a for the applicable E-UTRA bands unless otherwise specified.

For the bands supporting 8 antenna ports which are in Table 7.3.1-1aa, the minimum requirements for reference sensitivity in the reference sensitivity exception tables shall be modified by the amount given in ΔRIB,8R in Table 7.3.1-1aa for the applicable E-UTRA bands unless otherwise specified.

For the UE that supports any of the E-UTRA CA configurations given in Table 7.3.1A-0a, exceptions to the requirements for a band(s) specified in subclause 7.3.1 are allowed when the band(s) is impacted by harmonic interference from the uplink transmission in a lower-frequency band of the same CA configuration. For these exceptions, the UE shall meet the requirements specified in Table 7.3.1A-0a and Table 7.3.1A-0b. These exceptions also apply to any higher order CA or DC combination containing one of the exception combinations in this clause as subset.

Table 7.3.1A-0a: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions due to harmonic issue)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_1A-28A5,6,14 | 133 |  |  | -89.8 | -89.4 | -89 | -88.7 | FDD |
| CA_1A-1A-28A5,6,14 | 1 |  |  | -89.8 | -89.4 | -89 | -88.7 | FDD |
| CA_2A-46A15,16 | 2 |  |  | -70 | -67 | -65.2 | -64 | FDD |
| CA_2A-2A-46A15,16 | 2 |  |  | -70 | -67 | -65.2 | -64 | FDD |
| CA_2A-48A9,10 | 48 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_2A-48A11 | 48 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_2A-71A36 | 2 |  |  | -93.4 | -94 | -92.5 | -91.4 | FDD |
| CA_2A-71A37 | 2 |  |  | -96.8 | -94 | -92.5 | -91.4 | FDD |
| CA_2A-71A15,16 | 71 |  |  | -70.4 | -70.4 | -70.4 | -70.4 | FDD |
| CA_2A-2A-71A36 | 2 |  |  | -93.4 | -94 | -92.5 | -91.4 | FDD |
| CA_2A-2A-71A37 | 2 |  |  | -96.8 | -94 | -92.5 | -91.4 | FDD |
| CA_2A-2A-71A15,16 | 71 |  |  | -70.4 | -70.4 | -70.4 | -70.4 | FDD |
| CA_2A-48C9,10 | 48 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_2A-48C11 | 48 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_2A-48D9,10 | 48 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_2A-48D11 | 48 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_3A-8A4 CA_3A-8B4 | 3 |  |  | N/A | N/A | N/A | N/A | FDD |
| CA_3A-31A12,13 | 333 |  |  | -86.9 | -86.4 | -86 | -85.6 | FDD |
| CA_3A-42A9,10 | 4233 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_3A-42A11 | 4233 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_3A-42A-42A9,10 | 4233 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_3A-42A-42A11 | 4233 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_3A-3A-42A11 | 4233 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_4A-12A5,6 | 433 | -89.2 | -89.2 | -90 | -89.5 | -89 | -88.5 | FDD |
| CA_4A-17A5,6 | 433 |  |  | -90 | -89.5 |  |  | FDD |
| CA_4A-28A5,6 | 433 |  |  | -89.8 | -89.4 | -89 | -88.7 | FDD |
| CA_5A-38A8,19 CA_5A-41A8,19 | 5 |  |  | N/A | N/A |  |  | FDD |
|  | 38,41 |  |  | N/A | N/A | N/A | N/A | TDD |
| CA_7A-8A5,6 CA_7A-8B5,6 | 733 |  |  | -88 | -87.4 | -87 | -86.7 | FDD |
| CA_8A-41A8 | 41 |  |  | N/A | N/A | N/A | N/A | TDD |
| CA_8A-42A12,13 | 4233 |  |  | -84.8 | -84.7 | -84.6 | -84.5 | TDD |
| CA_11A-28A9,10 | 1133 |  |  | -75.2 | -75.2 |  |  | FDD |
| CA_12A-66A5,6 | 6633 | -88.7 | -88.7 | -89.5 | -89 | -88.5 | -88 | FDD |
| CA_18A-41A19 CA_18A-41C19 | 41 |  |  | N/A | N/A | N/A | N/A | TDD |
| CA_20A-32A-42A12,13 | 20 |  |  | -97 |  |  |  | FDD |
|  | 32 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 4233 |  |  | -84.8 | -84.7 | -84.6 | -84.5 | TDD |
| CA_20A-38A8 CA_20A-38C8 | 20 |  |  | N/A | N/A | N/A | N/A | FDD |
| CA_20A-40A15,16 | 2033 |  |  | -60.7 | -60.7 | -60.7 | -60.7 | FDD |
| CA_20A-40A-40A15,16 | 2033 |  |  | -60.7 | -60.7 | -60.7 |  | FDD |
| CA_20A-40C15,16 | 2033 |  |  | -60.7 | -60.7 | -60.7 |  | FDD |
| CA_20A-40D15,16 | 20 |  |  | -60.7 | -60.7 | -60.7 |  | FDD |
| CA_20A-42A12,13, CA_20A-42A-42A12,13 | 4233 |  |  | -84.8 | -84.7 | -84.6 | -84.5 | TDD |
| CA_20A-41A5,6 CA_20A-41C5,6 CA_20A-41D5,6 | 41 |  |  | -85.1 | -84.7 | -84.8 | -84.6 | TDD |
| CA_21A-28A4,21 | 21 |  |  | N/A | N/A | N/A |  | FDD |
| CA_26A-41A8,19 | 26 |  |  | N/A | N/A | N/A |  | FDD |
|  | 41 |  |  | N/A | N/A | N/A | N/A | TDD |
| CA_28A-32A9,10 | 32 |  |  | -72.2 | -72.2 | -72.2 | -72.2 | FDD |
| CA_28A-32A11 | 32 |  |  | -97.6 | -95.2 | -93.7 | -93.0 | FDD |
| CA_28A-40A15,16 | 28 |  |  | -60.7 | -60.7 | -60.7 | -60.7 | FDD |
| CA_28A-42A17,18 | 4233 |  |  | -85.7 | -85.4 | -85.1 | -84.9 | TDD |
| CA_28A-66A5,6 | 66 |  |  | -89.5 | -88.9 | -88.5 | -88.2 | FDD |
| CA_26A-38A15, 16 CA_26A-38C15, 16 | 26 |  |  | -73.2 | -70.3 | -70.2 |  | FDD |
| CA_48A-66A9,10 | 48 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_48A-66A11 | 48 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_48A-66B9,10 | 48 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_48A-66B11 | 48 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_48A-66C9,10 | 48 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_48A-66C11 | 48 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_48A-48A-66A9,10 | 48 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_48A-48A-66A11 | 48 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_48A-66A-66A9,10 | 48 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_48A-66A-66A11 | 48 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_48A-48A-66A-66A9,10 | 48 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_48A-48A-66A-66A11 | 48 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_48A-48A-66B9,10 | 48 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_48A-48A-66B11 | 48 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_48A-48A-66C9,10 | 48 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_48A-48A-66C11 | 48 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_48C-66A-66A9,10 | 48 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
|  | 66 |  |  | -99.3 | -96.3 | -94.5 | -93.2 | FDD |
| CA_48C-66A-66A11 | 48 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
|  | 66 |  |  | -99.3 | -96.3 | -94.5 | -93.2 | FDD |
| CA_48C-66B9,10 | 48 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_48C-66B11 | 48 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_48C-66C9,10 | 48 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_48C-66C11 | 48 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_48C-66A9,10 | 48 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_48C-66A11 | 48 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_48E-66A 9,10 | 48 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_48E-66A 11 | 48 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_70A-71A5,6 | 70 |  |  | -90 | -89.5 | -89.2 |  | FDD |
| CA_70C-71A5,6,35 | 70 |  |  | -90 | -89.5 | -89.2 | -89 | FDD |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5A. NOTE 2: Reference measurement channel is A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD/FS3 as described in Annex A.5.1.1/A.5.2.1/A.5.4.1. NOTE 3: The signal power is specified per port NOTE 4: No requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the low band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of the high band. The reference sensitivity for all active downlink component carriers is only verified when this is not the case (the requirements specified in clause 7.3.1 apply unless otherwise specified). NOTE 5: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 6: The requirements should be verified for UL EARFCN of a low band (superscript LB) such that in MHz and with the carrier frequency of a high band in MHz and the channel bandwidth configured in the low band. NOTE 7: Void. NOTE 8: No requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the low band for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of the high band. The reference sensitivity is only verified when this is not the case (the requirements specified in clause 7.3.1 apply). NOTE 9: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range FHD above and below the edge of this downlink transmission bandwidth. The value FHD depends on the E-UTRA configuration: FHD = 10 MHz for CA_3A-42A, CA_3A-3A-42A, CA_3A-42A-42A, CA_1A-3A-20A-32A-42A, CA_3A-42A-43A, CA_3A-32A-42A-43A, CA_1A-3A-42A, CA_2A-13A-48A-66A, CA_2A-48A, CA_2A-48C, CA_2A-48D, CA_48A-66A, CA_3A-7A-42A, CA_3A-19A-42A, CA_3A-20A-42A, CA_3A-28A-42A, CA_1A-3A-7A-42A, CA_5A-48A-66A, CA_5A-48A-66A-66A, CA_13A-48A-66A, CA_13A-48A-66A-66A, CA_13A-48A-66B, CA_13A-48A-66C, CA_13A-48A-48A-66A, CA_13A-48C-66A, CA_13A-48D-66A, CA_13A-48A-48C-66A, CA_28A-32A, CA_48A-66A-66A, CA_48A-66B , CA_48A-66C, CA_48A-48A-66A, CA_48C-66A, CA_48A-48A-66A-66A, CA_48A-48A-66B, CA_48A-48A-66C, CA_48C-66B, CA_48C-66C, CA_48E-66A, CA_1A-3A-18A-42A, CA_1A-3A-19A-42A, CA_1A-3A-32A-42A, CA_1A-3A-41A-42A, CA_3A-7A-20A-42A, CA_3A-20A-32A-42A, CA_3A-28A-41A-42A, CA_3A-18A-42A, CA_3A-18A-42C, CA_3A-8A-42A and CA_3A-8A-42C. FHD = 0MHz for CA_11A-28A, CA_1A-11A-28A and CA_3A-11A-28A. NOTE 10: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 11: The requirements are only applicable to channel bandwidths with a carrier frequency at MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. NOTE 12: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 4th transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 13: The requirements should be verified for UL EARFCN of a low band (superscript LB) such that in MHz and with the carrier frequency of a high band in MHz and the channel bandwidth configured in the low band. NOTE 14: For the UE that supports CA_1A-18A-28A or CA_1A-19A-28A, no requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the low band for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of the high band. The reference sensitivity should only be verified when this is not the case (the requirements specified in clause 7.3.1 apply). NOTE 15: These requirements apply when there is at least one individual RE within the downlink transmission bandwidth of the victim (lower) band for which the 3rd harmonic is within the uplink transmission bandwidth or the uplink adjacent channel’s transmission bandwidth of an aggressor (higher) band. NOTE 16: The requirements should be verified for UL EARFCN of the aggressor (higher) band (superscript HB) such that in MHz and with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. NOTE 17: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 5th transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 18: The requirements should be verified for UL EARFCN of a low band (superscript LB) such that in MHz and with the carrier frequency of a high band in MHz and the channel bandwidth configured in the low band. NOTE 19: No requirements apply for the case that there is at least one individual RE within the uplink transmission bandwidth of the relative higher band and when the frequency range of relative higher band’s uplink channel bandwidth or uplink 1st adjacent channel bandwidth is fully or partially overlapped with the 3 times of the frequency range of the relative lower band’s downlink channel bandwidth. The reference sensitivity is only verified when this is not the case (the requirements specified in clause 7.3.1 apply). NOTE 20: Void NOTE 21: No requirements apply when there is at least one individual RE on band 28 uplink outside frequencies 728 – 738 MHz. The reference sensitivity is only verified when all configured RE’s are confined within frequencies 728 – 738 MHz (the requirements specified in clause 7.3.1 of [6] apply). NOTE 22: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range FHD above and below the edge of this downlink transmission bandwidth. The value FHD depends on the E-UTRA configuration: FHD = 10 MHz for CA_1A-3A-21A-42A. NOTE 23: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 24: The requirements are only applicable to channel bandwidths with a carrier frequency at MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. NOTE 25: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range FHD above and below the edge of this downlink transmission bandwidth. The value FHD depends on the E-UTRA configuration: FHD = 10 MHz for CA_3A-19A-21A-42A. NOTE 26: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 27: The requirements are only applicable to channel bandwidths with a carrier frequency at MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. NOTE 28: No requirements apply when there is at least one individual RE within the uplink transmission bandwidth of either Band 5 or Band 8 for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of Band 41. The reference sensitivity for all active downlink component carriers is only verified when this is not the case (the requirements specified in clause 7.3.1 apply). NOTE 29: The B41 requirements are modified by -0.1dB when carrier frequency of the assigned E-UTRA channel bandwidth is within 2545-2690 MHz. NOTE 30: Void NOTE 31: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range FHD above and below the edge of this downlink transmission bandwidth. The value FHD depends on the E-UTRA configuration: FHD = 10 MHz for CA_2A-48A-48A and CA_2A-48A-48C NOTE 32: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range FHD above and below the edge of this downlink transmission bandwidth. The value FHD depends on the E-UTRA configuration: FHD = 10 MHz for CA_2A-48A-66A, CA_2A-48A-66A-66A and CA_2A-48C-66A NOTE 33: Applicable for the operations with 2 or 4 antenna ports supported in the band with carrier aggregation configured. NOTE 34: Void NOTE 35: These exceptions for the intra-band class C carriers should be verified one carrier at a time, according to note 6 frequency arrangements. No exceptions apply for the carrier which is not under REFSENS exception test. NOTE 36: These requirements apply when the lower edge frequency of the 5 MHz uplink channel in Band 71 is located at or below 668 MHz and the downlink channel in Band 2 is located with its upper edge at 1990 MHz. NOTE 37: These requirements apply when the lower edge frequency of the 10 MHz, 15 MHz, or 20 MHz uplink channel in Band 71 is located at or below 668 MHz and the downlink channel in Band 2 is located with its upper edge at 1990 MHz. Note 38: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 3nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range FHD above and below the edge of this downlink transmission bandwidth. The value FHD depends on the E-UTRA configuration: FHD = 15 MHz for CA_26A-41A, CA_25A-26A-41A. |  |  |  |  |  |  |  |  |

Table 7.3.1A-0b: Uplink configuration for the low band (exceptions due to harmonic issue)

| E-UTRA Band / Channel bandwidth of the high band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex mode |
| CA_1A-28A | 28 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_1A-1A-28A | 28 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_2A-46A | 46 |  |  |  |  |  | 100 | TDD |
| CA_2A-2A-46A | 46 |  |  |  |  |  | 100 | TDD |
| CA_2A-71A | 71 |  |  | 25 | 251 | 201 | 201 | FDD |
| CA_2A-71A | 2 |  |  | 25 | 50 | 50 | 50 | FDD |
| CA_2A-2A-71A | 71 |  |  | 25 | 50 | 50 | 50 | FDD |
| CA_2A-48A CA_2A-48A-48A CA_2A-48A-48C CA_2A-48D | 2 |  |  | 25 | 50 | 501 | 501 | FDD |
| CA_3A-31A | 31 |  |  | 5 | 5 | 5 | 5 | FDD |
| CA_3A-42A | 3 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_3A-3A-42A | 3 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_3A-42A-42A | 3 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_4A-12A | 12 | 2 | 5 | 8 | 16 | 20 | 20 | FDD |
| CA_4A-17A | 17 |  |  | 8 | 16 |  |  | FDD |
| CA_4A-28A | 28 |  |  | [8] | [16] | [25] | [25] | FDD |
| CA_7A-8A CA_7A-8B | 8 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_8A-42A | 8 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_11A-28A | 28 |  |  | 12 | 25 |  |  | FDD |
| CA_12A-66A | 12 | 2 | 5 | 8 | 16 | 20 | 20 | FDD |
| CA_20A-32A-42A | 20 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_20A-40A3 CA_20A-40D3 | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_20A-40C3 | 40 |  |  | 25 | 50 | 75 |  | TDD |
| CA_20A-40A-40A3 | 40 |  |  | 25 | 50 | 75 |  | TDD |
| CA_20A-41A CA_20A-41C CA_20A-41D | 20 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_20A-42A, CA_20A-42A-42A | 20 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_26A-38A CA_26A-38C | 38 |  |  | 25 | 50 | 75 |  | TDD |
| CA_28A-32A | 28 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_28A-40A | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_28A-42A | 28 |  |  | 5 | 10 | 15 | 20 | FDD |
| CA_28A-66A | 28 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_48A-66A | 66 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_48A-48A-66A | 66 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_48A-48A-66A-66A | 66 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_48A-48A-66B | 66 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_48A-48A-66C | 66 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_48C-66A-66A | 66 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_48C-66B | 66 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_48C-66C | 66 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_48A-66A-66A | 66 |  |  | 121 | 251 | 361 | 501 | FDD |
| CA_48A-66B | 66 |  |  | 121 | 251 | 361 | 501 | FDD |
| CA_48A-66C | 66 |  |  | 121 | 251 | 361 | 501 | FDD |
| CA_48C-66A | 66 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_48E-66A | 66 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_70A-71A | 71 |  |  | 8 | 16 | 20 |  | FDD |
| CA_70C-71A | 71 |  |  | 8 | 16 | 20 | 20 | FDD |
| NOTE 1: refers to the UL resource blocks, which shall be centred within the transmission bandwidth configuration for the channel bandwidth. NOTE 2: the UL configuration applies regardless of the channel bandwidth of the low band unless the UL resource blocks exceed that specified in Table 7.3.1-2 for the uplink bandwidth in which case the allocation according to Table 7.3.1-2 applies. NOTE 3: 3 refers to the UL resource blocks shall be located between 2373-2400MHz. NOTE 4: These configurations apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 5: These configurations apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a high band. |  |  |  |  |  |  |  |  |

For the UE that supports any of the E-UTRA CA configurations given in Table 7.3.1A-0bA, exceptions to the requirements for a band(s) specified in subclause 7.3.1 are allowed when the band(s) is impacted by the uplink being active within a specified frequency range as noted in Table 7.3.1A-0bA. For these exceptions, the UE shall meet the requirements specified in Table 7.3.1A-0bA and Table 7.3.1A-0bB. These exceptions also apply to any higher order CA or DC combination containing one of the exception combinations in this clause as subset.

Table 7.3.1A-0bA: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions for two bands due to close proximity of UL to DL channel)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_1A-3A4 | 39 |  | [-95.7] | -94 | -91.5 | -90 | -89 | FDD |
| CA_1A-3A5 | 3 |  | -98.7 | -97 | -94 | -92.2 | -91 | FDD |
| CA_1A-1A-3A4 | 39 |  |  | -94 | -91.5 | -90 | -89 | FDD |
| CA_1A-1A-3A5 | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD |
| CA_1A-1A-3A-3A4 | 39 |  | [-95.7] | -94 | -91.5 | -90 | -89 | FDD |
| CA_1A-1A-3A-3A5 | 3 |  | -98.7 | -97 | -94 | -92.2 | -91 | FDD |
| CA_1A-3A-3A4 | 39 |  |  | -94 | -91.5 | -90 | -89 | FDD |
| CA_1A-3A-3A5 | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD |
| CA_1A-3C4 | 39 |  |  | -94 | -91.5 | -90 | -89 | FDD |
| CA_1A-3C5 | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD |
| CA_1A-1A-3C4 | 39 |  |  | -94 | -91.5 | -90 | -89 | FDD |
| CA_1A-1A-3C5 | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD |
| CA_1C-3A4 | 39 |  |  | -94 | -91.5 | -90 | -89 | FDD |
| CA_1C-3A5 | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD |
| CA_18A-28A6 | 28 |  |  | -94 | -92.5 |  |  | FDD |
| CA_19A-28A7 | 28 |  |  | -94 | -92 |  |  | FDD |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5A. NOTE 2: Reference measurement channel is A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 NOTE 3: The signal power is specified per port NOTE 4: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz. For each channel bandwidth in Band 3, the requirement applies regardless of channel bandwidth in Band 1. NOTE 5: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. For each channel bandwidth in Band 3, the requirement applies regardless of channel bandwidth in Band 1. NOTE 6: These requirements apply when the uplink is active in Band 18 and the downlink channels in Band 28 are confined within the restricted frequency range specified for this CA configuration (Table 5.5A-2). For each channel bandwidth in Band 28, the requirement applies regardless of channel bandwidth in Band 18. NOTE 7: These requirements apply when the uplink is active in Band 19 and the downlink channels in Band 28 are allocated at the middle of the restricted frequency range specified for this CA configuration (Table 5.5A-2). For each channel bandwidth in Band 28, the requirement applies regardless of channel bandwidth in Band 19. NOTE 8: Void NOTE 9: Applicable for the operations with 2 or 4 antenna ports supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |

Table 7.3.1A-0bB: Uplink configuration for the uplink band (exceptions for two bands due to close proximity of UL to DL channel)

| E-UTRA Band / Channel bandwidth of the affected DL band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex mode |
| CA_1A-3A1, 2 | 1 |  |  | 25 | 25 | 25 | 25 | FDD |
| CA_1A-3A1, 3 | 1 |  |  | 25 | 45 | 45 | 45 | FDD |
| CA_1A-1A-3A 1, 2 | 1 |  |  | 25 | 25 | 25 | 25 | FDD |  |
| CA_1A-1A-3A 1, 3 | 1 |  |  | 25 | 45 | 45 | 45 | FDD |  |
| CA_1A-1A-3A-3A1, 2 | 1 |  |  | 25 | 25 | 25 | 25 | FDD |  |
| CA_1A-1A-3A-3A1, 3 | 1 |  |  | 25 | 45 | 45 | 45 | FDD |  |
| CA_1A-3C1, 2 | 1 |  |  | 25 | 25 | 25 | 25 | FDD |
| CA_1A-3C1, 3 | 1 |  |  | 25 | 45 | 45 | 45 | FDD |
| CA_1A-1A-3C1, 2 | 1 |  |  | 25 | 25 | 25 | 25 | FDD |
| CA_1A-1A-3C1, 3 | 1 |  |  | 25 | 45 | 45 | 45 | FDD |
| CA_1C-3A1, 2 | 1 |  |  | 25 | 25 | 25 | 25 | FDD |
| CA_1C-3A1, 3 | 1 |  |  | 25 | 45 | 45 | 45 | FDD |
| CA_18A-28A4 | 18 |  |  | 18 | 18 |  |  | FDD |
| CA_19A-28A4 | 19 |  |  | 18 | 18 |  |  | FDD |
| NOTE 1: refers to the UL resource blocks shall be located as close as possible to the downlink channel in Band 3 but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.6-1) in the uplink channel in Band 1. NOTE 2: UL allocation when the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz NOTE 3: UL allocation when the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. NOTE 4: refers to the UL resource blocks shall be located as close as possible to the downlink channel in Band 28 but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.6-1). |  |  |  |  |  |  |  |  |

Table 7.3.1A-0bC: Void

Table 7.3.1A-0bD: Void

Table 7.3.1A-0bD1: Void

Table 7.3.1A-0bD2: Void

Table 7.3.1A-0bD3: Void

Table 7.3.1A-0bD4: Void

For the UE that supports any of the E-UTRA CA configurations given in Table 7.3.1A-0bE, exceptions to the requirements for a band(s) specified in subclause 7.3.1 are allowed when the band(s) is impacted by uplink being active in the applicable active UL bands of the same CA configuration in Table 7.3.1A-0bE. For these exceptions, the UE shall meet the reference sensitivities specified in Table 7.3.1A-0bE and Table 7.3.1A-0bF. These exceptions also apply to any higher order CA or DC combination containing one of the exception combinations in this clause as subset.

Table 7.3.1A-0bE: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions due to cross band isolation issues of TDD and FDD bands)

| EUTRA CA Configuration | EUTRA band | Channel bandwidth |  |  |  |  |  | Duplex mode | Applicable active UL band |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |  |  |  |
| CA_1A-7A-38A | 719 |  |  | -93.3 | -90.7 | -89.2 | -88.1 | FDD | 1 |  |
|  | 38 |  |  | -93.3 | -90.7 | -89.2 | -88.1 | TDD |  |  |
| CA_1A-40A | 4019 |  |  | [-93.4] | -91.9 | -90.4 | -89.4 | TDD | 1 |  |
| CA_1A-40A | 119 |  |  | -91.7 | [-89.5] | [-87.9] | [-86.9] | FDD | 40 |  |
| CA_1A-40C | 4019 |  |  | [-93.4] | -91.9 | -90.4 | -89.4 | TDD | 1 |  |
| CA_1A-40C | 119 |  |  | -91.7 | [-89.5] | [-87.9] | [-86.9] | FDD | 40 |  |
| CA_3A-7A-38A | 719 |  |  | [-93.8] | [-91.2] | [-89.7] | [-88.6] | FDD | 3 |  |
|  | 38 |  |  | [-93.8] | [-91.2] | [-89.7] | [-88.6] | TDD |  |  |
| CA_3C-7A-38A | 719 |  |  | [-93.8] | [-91.2] | [-89.7] | [-88.6] | FDD | 3 |  |
|  | 38 |  |  | [-93.8] | [-91.2] | [-89.7] | [-88.6] | TDD |  |  |
| CA_3A-40A | 4019 |  |  | -95.4 | -92.9 | -91.3 | -90.2 | TDD | 3 |  |
| CA_3A-40A | 319 | [-97.4] | [-95.3] | -94.2 | -91.2 | -89.5 | -88.3 | FDD | 40 |  |
| CA_3A-40A-40A | 4019 |  |  |  | -92.9 |  | -90.2 | TDD | 3 |  |
| CA_3A-40A-40A | 319 |  |  | -94.2 | -91.2 |  |  | FDD | 40 |  |
| CA_3A-40C | 4019 |  |  | -95.4 | -92.9 | -91.3 | -90.2 | TDD | 3 |  |
| CA_3A-40D | 4019 |  |  | -95.4 | -92.9 | -91.3 | -90.2 | TDD | 3 |
| CA_3A-40D | 319 |  |  | -94.2 | -91.2 | -89.5 | -88.3 | FDD | 40 |
| CA_3A-40E | 4019 |  |  |  |  | -91.3 | -90.2 | TDD | 3 |  |
| CA_3A-40E | 319 |  |  | -94.2 | -91.2 | -89.5 | -88.3 | FDD | 40 |  |
| CA_3A-40C | 319 |  |  | -94.2 | -91.2 | -89.5 | -88.3 | FDD | 40 |  |
| CA_3C-40A | 4019 |  |  | -95.4 | -92.9 | -91.3 | -90.2 | TDD | 3 |  |
| CA_3C-40A | 319 |  |  | -94.2 | -91.2 | -89.5 | -88.3 | FDD | 40 |  |
| CA_3C-40C | 4019 |  |  | -95.4 | -92.9 | -91.3 | -90.2 | TDD | 3 |  |
| CA_3C-40C | 319 |  |  | -94.2 | -91.2 | -89.5 | -88.3 | FDD | 40 |  |
| CA_3A-41A5 | 319 |  | [-95.3] | [-94] | [-91] | [-89.2] | [-87.9] | FDD | 41 |  |
|  | 4119 |  |  | [-93.3] | [-90.7] | [-89.2] | [-88.1] | TDD | 3 |  |
| CA_3A-41C5 | 319 |  |  | [-94] | [-91] | [-89.2] | [-87.9] | FDD | 41 |  |
|  | 4119 |  |  | [-93.3] | [-90.7] | [-89.2] | [-88.1] | TDD | 3 |  |
| CA_3A-41D5 | 319 |  |  | [-94] | [-91] | [-89.2] | [-87.9] | FDD | 41 |  |
|  | 4119 |  |  | [-93.3] | [-90.7] | [-89.2] | [-88.1] | TDD | 3 |  |
| CA_3A-3A-41A | 3 |  | [-95.3] | [-94] | [-91] | [-89.2] | [-87.9] | FDD | 41 |
|  | 41 |  |  | [-93.3] | [-90.7] | [-89.2] | [-88.1] | TDD | 3 |
| CA_3C-41A5 | 319 |  |  | [-94] | [-91] | [-89.2] | [-87.9] | FDD | 41 |  |
|  | 4119 |  |  | [-93.3] | [-90.7] | [-89.2] | [-88.1] | TDD | 3 |  |
| CA_3C-41C5 | 319 |  |  | [-94] | [-91] | [-89.2] | [-87.9] | FDD | 41 |  |
|  | 4119 |  |  | [-93.3] | [-90.7] | [-89.2] | [-88.1] | TDD | 3 |  |
| CA_3C-41D5 | 319 |  |  | [-94] | [-91] | [-89.2] | [-87.9] | FDD | 41 |
|  | 4119 |  |  | [-93.3] | [-90.7] | [-89.2] | [-88.1] | TDD | 3 |
| CA_7A-40A | 4019 |  |  | -96.3 | -93.6 | -92 | -90.9 | TDD | 7 |  |
|  | 719 |  |  | -97.1 | -94.3 | -92.7 | -91.5 | FDD | 40 |  |
| CA_7A-40C | 4019 |  |  | -96.3 | -93.6 | -92 | -90.9 | TDD | 7 |  |
|  | 719 |  |  | -97.1 | -94.3 | -92.7 | -91.5 | FDD | 40 |  |
| CA_7A-40D | 4019 |  |  | -96.3 | -93.6 | -92 | -90.9 | TDD | 7 |  |
|  | 719 |  |  | -97.1 | -94.3 | -92.7 | -91.5 | FDD | 40 |  |
| CA_7A-40E | 4019 |  |  | -96.3 | -93.6 | -92 | -90.9 | TDD | 7 |  |
|  | 719 |  |  | -97.1 | -94.3 | -92.7 | -91.5 | FDD | 40 |  |
| CA_7A-42A | 4219 |  |  | -95.6 | -93 | -91.5 | -90.4 | TDD | 7 |  |
|  | 719 |  |  | -96.2 | -93.2 | -91.5 | -90.3 | FDD | 42 |  |
| CA_7A-42A-42A | 4219 |  |  | -95.6 | -93 | -91.5 | -90.4 | TDD | 7 |  |
|  | 719 |  |  | -96.2 | -93.2 | -91.5 | -90.3 | FDD | 42 |  |
| CA_25A-41A, CA_25A-25A-41A, CA_25A-41C, CA_25A-25A-41C, CA_25A-41D, CA_25A-25A-41D, CA_25A-41E, CA_25A-25A-41E, CA_25A-41F, CA_25A-25A-41F | 25 |  |  | [-95.9] | [-92.9] | [-91.1] | [-89.9] | FDD | 41 |  |
| CA_28A-40A | 40 |  |  | -95.1 | -92.9 | -91.4 | -90.5 | TDD | 28 |  |
| CA_28A-40A | 28 |  |  | -96.8 | -94.1 | -92.5 | -89.8 | FDD | 40 |  |
| CA_28A-40C | 40 |  |  | -95.1 | -92.9 | -91.4 | -90.5 | TDD | 28 |  |
| CA_28A-40C | 28 |  |  | -96.8 | -94.1 | -92.5 | -89.8 | FDD | 40 |  |
| CA_28A-40D | 40 |  |  | -95.1 | -92.9 | -91.4 | -90.5 | TDD | 28 |  |
| CA_28A-40D | 28 |  |  | -96.8 | -94.1 | -92.5 | -89.8 | FDD | 40 |  |
| CA_38A-66A CA_38C-66A | 66 |  |  | -97.6 | -94.6 | -92.8 | -91.6 | FDD | 38 |  |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5A NOTE 2: Reference measurement channel is A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 NOTE 3: The signal power is specified per port NOTE 4: These requirements apply regardless of the channel bandwidth and the location of UL band. NOTE 5: The B41 requirements are modified by -0.5dB when carrier frequency of the assigned E-UTRA channel bandwidth is within 2545-2690 MHz. NOTE 6: The antenna isolation for MSD calculation is assumed as 10 dB. For conducted mode REFSENS test such antenna isolation is not observed as the antennas are disconnected. Additionally antenna isolation assumption is under discussion depending on the frequency range NOTE 7: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range FHD above and below the edge of this downlink transmission bandwidth. The value FHD depends on the E-UTRA configuration: FHD = 10 MHz for CA_3A-42A, CA_3A-42C, CA_3A-42D, CA_3A-3A-42C, CA_3A-42A-42C, CA_1A-3A-42A, CA_1A-3A-42C, CA_1A-3A-42D, CA_3A-19A-42A, CA_3A-19A-42C, CA_1A-3A-19A-42A, CA 3A-41A-42A, CA_3A-41A-42C, CA_3A-41C-42A, and CA_3A-41C-42C. NOTE 8: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 9: The requirements are only applicable to channel bandwidths with a carrier frequency at MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. NOTE 10: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. NOTE 11: Void NOTE 12: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz. For each channel bandwidth in Band 3 and Band 41, the requirement applies regardless of channel bandwidth in Band 1. NOTE 13: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. For each channel bandwidth in Band 3 and Band 41, the requirement applies regardless of channel bandwidth in Band 1. NOTE 14: The B41 requirements also apply to the supported CA_1A-41A. NOTE 15: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz. For each channel bandwidth other than Band 1, the requirement applies regardless of channel bandwidth in Band 1 NOTE 16: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. For each channel bandwidth other than Band 1, the requirement applies regardless of channel bandwidth in Band 1. NOTE 17: The B41 requirements are modified by -0.1dB when carrier frequency of the assigned E-UTRA channel bandwidth is within 2545-2690 MHz. NOTE 18: No requirements apply when there is at least one individual RE within the downlink transmission bandwidth of band5 for which the 3rd receiver harmonic is within the uplink transmission bandwidth of band41. The reference sensitivity is only verified when this is not the case (the requirements specified in clause 7.3.1 apply). NOTE 19: Applicable for the operations with 2 or 4 antenna ports supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |  |  |

Table 7.3.1A-0bF: Uplink configuration for reference sensitivity (exceptions due to cross band isolation issues of TDD and FDD bands)

| E-UTRA Band / Channel bandwidth of the affected DL band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | E-UTRA Band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex Mode |
| CA_1A-7A-38A | 1 |  |  | 25 | 45 | 451 | 451 | FDD |
| CA_1A-40A | 1 |  |  | 25 | 50 | 75 | 100 | FDD |
|  | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_3A-7A-38A | 3 |  |  | 25 | 50 | 501 | 501 | FDD |
| CA_3C-7A-38A | 3 |  |  | 25 | 50 | 501 | 501 | FDD |
| CA_3A-40A CA_3A-40C CA_3A-40D CA_3A-40E | 3 | 6 | 15 | 25 | 50 | 501 | 501 | FDD |
|  | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_3A-40A-40A | 3 |  |  | 25 | 50 |  |  | FDD |
|  | 40 |  |  |  | 50 |  | 100 | TDD |
| CA_3A-41A CA_3A-3A-41A | 3 |  | 15 | 25 | 50 | 501 | 501 | FDD |
|  | 41 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_3A-41C CA_3C-41A CA_3C-41D | 3 |  |  | 25 | 50 | 501 | 501 | FDD |
|  | 41 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_3A-41D | 3 |  |  | 25 | 50 | 501 | 501 | FDD |
|  | 41 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_3C-41C | 3 |  |  | 25 | 50 | 501 | 501 | FDD |
|  | 41 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_7A-40A, CA_7A-40C, CA_7A-40D, CA_7A-40E | 7 |  |  | 25 | 50 | 75 | 751 | FDD |
|  | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_7A-42A, CA_7A-42A-42A | 7 |  |  | 25 | 50 | 75 | 751 | FDD |
|  | 42 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_25A-41A, CA_25A-41C, CA_25A-41D, CA_25A-41E, CA_25A-41F, CA_25A-25A-41A, CA_25A-25A-41C, CA_25A-25A-41D, CA_25A-25A-41E, CA_25A-25A-41F | 25 |  |  | 25 | 50 | 75 | 100 | FDD |
|  | 41 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_28A-40A, CA_28A-40C | 28 |  |  | 25 | 251 | 251 | 251 | FDD |
|  | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_38A-66A CA_38C-66A | 38 |  |  | 100 | 100 | 100 | 100 | TDD |
| NOTE 1: 1 refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.6-1). NOTE 2: the UL configuration applies regardless of the channel bandwidth of the low band unless the UL resource blocks exceed that specified in Table 7.3.1-2 for the uplink bandwidth in which case the allocation according to Table 7.3.1-2 applies. NOTE 3: UL allocation when the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz. NOTE 4: UL allocation when the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. NOTE 5: 5 refers to Band 20; in the case of 15MHz channel bandwidth, the UL resource blocks shall be located at RBstart 11 and in the case of 20MHz channel bandwidth, the UL resource blocks shall be located at RBstart 16 |  |  |  |  |  |  |  |  |

For band combinations including operating bands without uplink band (as noted in Table 5.5-1), the requirements are specified in Table 7.3.1A-0d and for any uplink band with uplink configuration specified in Table 7.3.1-2. These requirements also apply to any higher order CA or DC combination containing one of the exception combinations in this clause as subset.

Table 7.3.1A-0d: Reference sensitivity QPSK PREFSENS (CA with a SDL band)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_1A-32A | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 32 |  |  | -100 | -97 | -95.2 | -94 |  |
| CA_2A-29A | 2 |  |  | -98 | -95 | -93.2 | -92 | FDD |
|  | 29 |  | -98.7 | -97 | -94 |  |  |  |
| CA_2A-2A-29A | 2 |  |  | -98 | -95 | -93.2 | -92 | FDD |
|  | 29 |  |  | -97 | -94 |  |  |  |
| CA_2C-29A | 2 |  |  | -98 | -95 | -93.2 | -92 | FDD |
|  | 29 |  |  | -97 | -94 |  |  |  |
| CA_3A-32A | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD |
|  | 32 |  |  | -99.5 | -96.5 | -94.7 | -93.5 |  |
| CA_3C-32A | 35 |  |  | -97 | -94 | -92.2 | -91 | FDD |
|  | 32 |  |  | -99.5 | -96.5 | -94.7 | -93.5 |  |
| CA_3A-69A | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD |
|  | 69 |  |  | -100 | -97 | -95.2 | -94 |  |
| CA_4A-29A | 4 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 29 |  | -98.7 | -97 | -94 |  |  |  |
| CA_5A-29A | 5 |  |  | -98 | -95 |  |  | FDD |
|  | 29 |  |  | -97 | -94 |  |  |  |
| CA_7A-29A CA_7A-7A-29A CA_7C-29A | 7 |  |  | -98 | -95 | -93.2 | -92 | FDD |
|  | 29 |  |  | -97 | -94 |  |  |  |
| CA_7A-32A | 7 |  |  |  | -95 | -93.2 | -92 | FDD |
|  | 32 |  |  | -100 | -97 | -95.2 | -94 |  |
| CA_20A-32A | 20 |  |  | -97 | -94 | -91.2 | -90 | FDD |
|  | 32 |  |  | -100 | -97 | -95.2 | -94 |  |
| CA_20A-75A | 20 |  |  | -97 | -94 | -91.2 | -90 | FDD |
|  | 75 |  |  | -100 | -97 | -95.2 | -94 |  |
| CA_20A-76A | 20 |  |  | -97 | -94 | -91.2 | -90 | FDD |
|  | 76 |  |  | -100 |  |  |  |  |
| CA_20A-67A | 20 |  |  | -97 | -94 | -91.2 | -90 | FDD |
|  | 67 |  |  | -100 | -97 | -95.2 | -94 |  |
| CA_23A-29A | 23 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 29 |  | -98.7 | -97 | -94 |  |  |  |
| CA_29A-30A | 29 |  |  | -97 | -94 |  |  | FDD |
|  | 30 |  |  | -99 | -96 |  |  |  |
| CA_29A-66A | 29 |  |  | -97 | -94 |  |  | FDD |
|  | 66 |  |  | -99.5 | -96.5 | -94.7 | -93.5 |  |
| CA_29A-66C | 29 |  |  | -97 | -94 |  |  | FDD |
|  | 66 |  |  | -99.5 | -96.5 | -94.7 | -93.5 |  |
| CA_29A-70A | 29 |  |  | -97 | -94 |  |  | FDD |
|  | 70 |  |  | -100 | -97 | -95.2 | -94 |  |
| CA_29A-70C | 29 |  |  | -97 | -94 |  |  | FDD |
|  | 70 |  |  | -100 | -97 | -95.2 | -94 |  |
| CA_32A-42A | 32 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 42 |  |  | -98.5 | -95.5 | -93.7 | -92.5 | TDD |
| CA_32A-43A | 32 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 43 |  |  | -98.5 | -95.5 | -93.7 | -92.5 | TDD |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5A. NOTE 2: Reference measurement channel is A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 NOTE 3: The signal power is specified per port. NOTE 4: Void NOTE 5: Applicable only if operation with 4 antenna ports is supported in the band with carrier aggregation configured. NOTE 6: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz. For each channel bandwidth in Band 3, the requirement applies regardless of channel bandwidth in Band 1. NOTE 7: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. For each channel bandwidth in Band 3, the requirement applies regardless of channel bandwidth in Band 1. |  |  |  |  |  |  |  |  |

Table 7.3.1A-0e: Void

For band combinations including operating band 46 (Table 5.5-1), the requirements are specified in Table 7.3.1A-0eA for the uplink in any band other than band 46 with the uplink configuration specified in Table 7.3.1-2 and Table 7.3.1A-0eC. These requirements also apply to any higher order CA or DC combination containing one of the combinations in this clause as subset.

For band combinations including operating band 49 (Table 5.5-1), the requirements are specified in Table 7.3.1A-0eA for the uplink in any band other than Band 49 with uplink configurations specified in Table 7.3.1-2 and measurement exclusion region in Table 7.3.1A-0eD. These requirements also apply to any higher order CA or DC combination containing one of the combinations in this clause as subset.

Table 7.3.1A-0eA: Reference sensitivity QPSK PREFSENS (CA with band 46 or Band 49)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_1A-46A CA_1A-46C CA_1A-46D CA_1A-46E | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 46 |  |  |  | -93 |  | -90 | TDD |
| CA_2A-46A CA_2A-46E CA_2A-2A-46A CA_2A-2A-46D CA_2A-46C CA_2A-46A-46A CA_2A-2A-46C CA_2A-46A-46C CA_2A-46D CA_2A-46A-46D | 2 |  |  | -98 | -95 | -93.2 | -92 | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_2A-49A9 | 2 |  |  | -97.8 | -94.8 | -93.0 | -91.7 | FDD |
|  | 49 |  |  |  | -95.5 |  | -92.5 | TDD |
| CA_3A-46A CA_3A-46C CA_3A-46D CA_3A-46E | 3 |  | -98.7 | -97 | -94 | -92.2 | -91 | FDD |
|  | 46 |  |  |  | -93 |  | -90 | TDD |
| CA_3C-46A | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_3A-3A-46A CA_3C-46C CA_3C-46D CA_3A-3A-46C | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_4A-46A CA_4A-46C CA_4A-46A-46A CA_4A-46A-46C CA_4A-46D CA_4A-46A-46D | 4 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_5A-46A CA_5A-46C CA_5A-46D CA_5A-46E CA_5B-46A CA_5B-46C CA_5B-46D CA_5B-46E | 5 |  |  | -98 | -95 |  |  | FDD |
|  | 46 |  |  |  | -93 |  | -90 | TDD |
| CA_7A-46A CA_7A-46C CA_7A-7A-46C CA_7A-46D CA_7A-46E CA_7C-46A CA_7C-46C CA_7C-46D CA_7C-46E | 7 |  |  | -98 | -95 | -93.2 | -92 | FDD |
|  | 46 |  |  |  | -93 |  | -90 | TDD |
| CA_7A-7A-46A CA_7A-7A-46D CA_7A-7A-46E | 7 |  |  | -98 | -95 | -93.2 | -92 | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_8A-46A CA_8A-46D CA_8A-46E CA_8B-46C CA_8B-46D | 8 | -102.2 | -99.2 | -97 | -94 |  |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_8A-46C | 8 | -102.2 | -99.2 | -97 | -94 |  |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_8B-46A | 8 | -102.2 | -99.2 | -97 | -94 |  |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_11A-46A | 11 |  |  | -100 | -97 |  |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_11A-46C | 11 |  |  | -100 | -97 |  |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_11A-46D | 11 |  |  | -100 | -97 | -95.2 |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_11A-46E | 11 |  |  | -100 | -97 |  |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_12A-46A CA_12A-46C CA_12A-46D | 12 |  |  | -97 | -94 |  |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_12A-46E | 12 |  |  | -97 | -94 |  |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_13A-46A CA_13A-46A-46A CA_13A-46A-46C CA_13A-46A-46D CA_13A-46C CA_13A-46D CA_13A-46E | 13 |  |  | -97 | -94 |  |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_19A-46A CA_19A-46C CA_19A-46D CA_19A-46E | 19 |  |  | -100 | -97 | -95.2 |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_21A-46A CA_21A-46C CA_21A-46D CA_21A-46E | 21 |  |  | -100 | -97 | -95.2 |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_25A-46A CA_25A-46C CA_25A-46D | 25 | -101.2 | -98.2 | -96.5 | -93.5 | -91.7 | -90.5 | FDD |
|  | 46 |  |  |  | -93 |  | -90 | TDD |
| CA_26A-46A | 26 |  | -99.7 | -97.57 | -94.57 |  |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_28A-46A CA_28A-46C CA_28A-46D CA_28A-46E | 28 |  |  | -98.5 | -95.5 | -93.7 | -91 | FDD |
|  | 46 |  |  |  | -93 |  | -90 | TDD |
| CA_29A-46A-66A | 29 |  |  | -97 | -94 |  |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
|  | 66 |  |  | -99.5 | -96.5 | -94.7 | -93.5 | FDD |
| CA_39A-46A | 39 |  |  | -100 | -97 | -95.2 | -94 | TDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_39A-46C | 39 |  |  | -100 | -97 | -95.2 | -94 | TDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_39C-46A | 39 |  |  | -100 | -97 | -95.2 | -94 | TDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_39A-46E CA_39C-46C CA_39C-46D | 39 |  |  | -100 | -97 | -95.2 | -94 | TDD |
|  | 46 |  |  |  |  |  | -90 |  |
| CA_40A-46A CA_40A-46D CA_40A-46E CA_40C-46C CA_40C-46D CA_40D-46A CA_40D-46C | 40 |  |  | -100 | -97 | -95.2 | -94 | TDD |
|  | 46 |  |  |  | -93 |  | -90 | TDD |
| CA_40A-46C | 40 |  |  | -100 | -97 | -95.2 | -94 | TDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_40C-46A | 40 |  |  | -100 | -97 | -95.2 | -94 | TDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_41A-46A CA_41A-46C CA_41A-46D CA_41A-46E CA_41C-46A CA_41C-46C CA_41C-46D CA_41D-46A CA_41D-46C | 41 |  |  | -98 | -95 | -93.2 | -92 | TDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_42A-46A | 42 |  |  | -99 | -96 | -94.2 | -93 | TDD |
|  | 46 |  |  |  |  |  | -83 | TDD |
| CA_46A-48A CA_46A-48A-48A CA_46A-48C CA_46A-48D CA_46A-48E CA_46C-48A CA_46C-48A-48A CA_46C-48C CA_46C-48D CA_46C-48E CA_46D-48A CA_46D-48C CA_46D-48A-48A CA_46E-48A CA_46E-48C | 46 |  |  |  |  |  | -83 | TDD |
|  | 48 |  |  | -99 | -96 | -94.2 | -93 | TDD |
| CA_46A-66A CA_46A-46A-66A CA_46A-66A-66A CA_46A-66C CA_46A-46C-66A CA_46A-46D-66A CA_46C-66A CA_46C-66A-66A CA_46D-66A CA_46D-66A-66A CA_46E-66A CA_46E-66A-66A | 46 |  |  |  |  |  | -90 | TDD |
|  | 66 |  |  | -99.5 | -96.5 | -94.7 | -93.5 | FDD |
| CA_46A-70A | 46 |  |  |  |  |  | -90 | TDD |
|  | 70 |  |  | -100 | -97 | -95.2 |  | FDD |
| CA_46A-71A CA_46C-71A CA_46D-71A | 46 |  |  |  |  |  | -90 | FDD |
|  | 71 |  |  | -97.2 | -94.2 | -92.0 | -87.5 | TDD |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5A. NOTE 2: Reference measurement channel is A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD/FS3 as described in Annex A.5.1.1/A.5.2.1/A.5.4.1. NOTE 3: The signal power is specified per port. NOTE 4: Void NOTE 5: The requirement for B46 does not apply when there is at least one individual RE within the B46 downlink transmission bandwidth which falls into the reference sensitivity exclusion region as specified in Table 7.3.1A-0eC. NOTE 6: Void NOTE 7: 7 indicates that the requirement is modified by -0.5 dB when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. NOTE 8: When Band 46 have self interference problems by dual uplink CA, then the requirements not apply in exclusion zone which is frequency range within (harmonics frequency region + FHD) and IMD frequency region as follow. NOTE 9: The requirement for B49 does not apply when there is at least one individual RE within the B49 downlink transmission bandwidth which falls into the reference sensitivity exclusion region as specified in Table 7.3.1A-0eD. NOTE 10: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band which excludes band 46 and a range FHD above and below the edge of this downlink transmission bandwidth. The value FHD depends on the E-UTRA configuration: FHD = 10 MHz for CA_2-46-48, CA_46-48-66, and CA_2-46-48-66. For harmonic issue not related with band 46, the uplink configuration of CA_2-48, CA_48-66 and CA_2-48-66 in Table 7.3.1A-0b can be used. NOTE 11: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 12: The requirements are only applicable to channel bandwidths with a carrier frequency at MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. IMD frequency range DL_CA configuration; UL_CA configuration; Exclusion zone center frequency; Exclusion zone BW. CA_1A-3A-46A; CA_1A-3A; 2*fc_1A + fc_3A; 2*BW_1A + BW_3A. CA_1A-3A-46A; CA_1A-3A; fc_1A – 2*fc_3A; BW_1A + 2*BW_3A. IMD frequency range DL_CA configuration; UL_CA configuration; Exclusion zone center frequency; Exclusion zone BW. CA_1A-5A-46A; CA_1A-5A; 2*fc_1A + 2*fc_5A; 2*BW_1A + 2*BW_5A. CA_1A-7A-46A; CA_1A-7A; 3*fc_7A - fc_1A; 3*BW_7A + BW_1A. CA_5A-7A-46A; CA_5A-7A; 2*fc_7A + fc_5A; 2*BW_7A + BW_5A. IMD frequency range DL_CA configuration; UL_CA configuration; Exclusion zone center frequency; Exclusion zone BW. CA_2A-5A-46D; CA_2A-5A; 3*fc_5A + 1*fc_2A; 3*BW_5A + 1*BW_2A. CA_2A-5A-46D; CA_2A-5A; 2*fc_5A – 3*fc_2A; 2*BW_5A + 3*BW_2A. IMD frequency range DL_CA configuration; UL_CA configuration; Exclusion zone center frequency; Exclusion zone BW. CA_5A-46D-66A; CA_5A-66A; 3*fc_66A + 1*fc_5A; 3*BW_66A + 1*BW_5A. CA_5A-46D-66A; CA_5A-66A; 2*fc_66A – 3*fc_5A; 2*BW_66A + 3*BW_5A. CA_5A-46D-66A; CA_5A-66A; 2*fc_66A + 3*fc_5A; 2*BW_66A + 3*BW_5A. IMD frequency range DL_CA configuration; UL_CA configuration; Exclusion zone center frequency; Exclusion zone BW. CA_13A-46D-66A; CA_13A-66A; 2*fc_66A + 2*fc_13A; 2*BW_66A + 2*BW_13A. CA_13A-46D-66A; CA_13A-66A; 2*fc_66A + 3*fc_13A; 2*BW_66A + 3*BW_13A. IMD frequency range DL_CA configuration; UL_CA configuration; Exclusion zone center frequency; Exclusion zone BW. CA_2A-13A-46D CA_2A-13A-46A-46D CA_2A-13A-46A-46C CA_2A-13A-46C CA_2A-13A-46A-46A CA_2A-13A-46A CA_2A-13A-46E; CA_2A-13A; 3*fc_13A + 1*fc_2A; 3*BW_13A + 1*BW_2A. IMD frequency range DL_CA configuration; UL_CA configuration; Exclusion zone center frequency; Exclusion zone BW. CA_2A-46E-66A CA_2A-46D-66A CA_2A-46C-66A CA_2A-46A-66A; CA_2A-66A; 2*fc_2A + 1*fc_66A 2*fc_66A + 1*fc_2A 1*fc_2A – 4*fc_66A 1*fc_66A – 4*fc_2A; 2*BW_2A + 1*BW_66A 2*BW_66A + 1*BW_2A 1*BW_2A + 4*BW_66A 1*BW_66A + 4*BW_2A. IMD frequency range DL_CA configuration; UL_CA configuration; Exclusion zone center frequency; Exclusion zone BW. CA_2A-46E-48A, CA_2A-46D-48A CA_2A-46D-48C CA_2A-46C-48C CA_2A-46A-48C CA_2A-46C-48A CA_2A-46A-48A; CA_2A-48A; 1*fc_2A + 1*fc_48A 2*fc_48A – 1*fc_2A; 1*BW_2A + 1*BW_48A 2*BW_48A + 1*BW_2A. IMD frequency range DL_CA configuration; UL_CA configuration; Exclusion zone center frequency; Exclusion zone BW. CA_46D-48C-66A CA_46C-48C-66A CA_46A-48C-66A CA_46D-48A-66A CA_46C-48A-66A CA_46A-48A-66A; CA_48A-66A; 1*fc_48A + 1*fc_66A 2*fc_48A – 1*fc_66A; 1*BW_48A + 1*BW_66A 2*BW_48A + 1*BW_66A. |

Table 7.3.1A-0eB: Void

Table 7.3.1A-0eC specifies the Band 46 reference measurement exclusion region for different licensed component carriers and channel bandwidth. The exclusion region is defined according to the licensed component carrier channel bandwidth. The UL configurations to be adopted for the test are specified in Table 7.3.1-2. The exclusion region in Table 7.3.1A-0eC is specified for the case of 10MHz and 20MHz channel bandwidth in Band 46.

Table 7.3.1A-0eC: Band 46 Reference sensitivity measurement exclusion region in MHz.

| Licensed Component Carriers / E-UTRA Band / Harmonic order / Channel BW in UL |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Licensed Component Carriers | Harmonic order | 5MHz | 10MHz | 15MHz | 20MHz |
| 1 | 3 | +/- 15 | +/- 23 | +/- 35 | +/- 45 |
| 2 | 3 | +/- 15 | +/- 23 | +/- 35 | +/- 45 |
| 3 | 3 | +/- 15 | +/- 23 | +/- 35 | +/- 45 |
| 4 | 3 | +/- 15 | +/- 23 | +/- 35 | +/- 45 |
| 5 | 7 | +/- 10 | +/- 20 |  |  |
| 71 | 2 | +/- 15 | +/- 25 | +/- 38 | +/- 50 |
| 8 | 6 | +/- 10 | +/- 20 |  |  |
| 11 | 4 | +/- 15 | +/- 25 |  |  |
| 12 | 8 | +/- 10 | +/- 20 |  |  |
| 13 | 7 | +/- 10 | +/- 20 |  |  |
| 19 | 7 | +/- 10 | +/- 20 | +/- 30 |  |
| 21 | 4 | +/- 15 | +/- 25 | +/- 38 |  |
| 25 | 3 | +/- 15 | +/- 23 | +/- 35 | +/- 45 |
| 26 | 7 | +/- 10 | +/- 20 | +/- 30 |  |
| 28 | 7 | +/- 10 | +/- 20 | +/- 30 | +/- 40 |
| 28 | 8 | +/- 10 | +/- 20 | +/- 30 | +/- 40 |
| 39 | 3 | +/- 15 | +/- 23 | +/- 35 | +/- 45 |
| 41 | 2 | +/- 15 | +/- 25 | +/- 38 | +/- 50 |
| 66 | 3 | +/- 15 | +/- 23 | +/- 35 | +/- 45 |
| NOTE 1: Even though UL harmonic does not fall directly into Band 46 the exclusion region still applies. NOTE 2: The center of the exclusion region is obtained by multiplying the UL channel center frequency by the harmonic order. |  |  |  |  |  |

Table 7.3.1A-0eD specifies the Band 49 reference measurement exclusion region for different licensed component carriers and channel bandwidth. The exclusion region is defined according to the licensed component carrier channel bandwidth. The UL configurations to be adopted for the test are specified in Table 7.3.1-2.

Table 7.3.1A-0eD: Band 49 reference sensitivity measurement exclusion region in MHz.

| Licensed Component Carriers / E-UTRA Band / Harmonic order / Channel BW in UL |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Licensed Component Carriers | Harmonic order | 5MHz | 10MHz | 15MHz | 20MHz |
| 21 | 2 | +/- 12.5 | +/- 25 | +/- 37.5 | +/- 50 |
| NOTE 1: Even though UL harmonic does not fall directly into Band 49 the exclusion region still applies. NOTE 2: The center of the exclusion region is obtained by multiplying the UL channel center frequency by the harmonic order. |  |  |  |  |  |

In all cases for single uplink inter-band CA, unless given by Table 7.3.1-3 for the band with the active uplink carrier, the applicable reference sensitivity requirements shall be verified with the network signalling value NS_01 (Table 6.2.4-1) configured.

For inter-band carrier aggregation with one component carrier per operating band (up to four downlinks) and the uplink assigned to two E-UTRA bands the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.3.1-1, Table 7.3.1-1a and Table 7.3.1-2. The reference sensitivity is defined to be met with all downlink component carriers active and both of the uplink carriers active.

For E-UTRA CA configurations with uplink and downlink assigned to two E-UTRA bands given in Table 7.3.1A-0f, the reference sensitivity is defined only for the specific uplink and downlink test points which are specified in Table 7.3.1A-0f. For E-UTRA CA configurations with uplink assigned to two E-UTRA bands and downlink assigned to three E-UTRA bands given in Table 7.3.1A-0g, the reference sensitivity is defined only for the specific uplink and downlink test points which are specified in Table 7.3.1A-0g. For these test points the reference sensitivity requirement specified in Table 7.3.1-1 and Table 7.3.1-1a are relaxed by the amount of the corresponding parameter MSD given in Table 7.3.1A-0f and Table 7.3.1A-0g.

The allowed exceptions defined in Table 7.3.1A-0a and Table 7.3.1A-0b for inter-band carrier aggregation with a single active uplink are also applicable for dual uplink operation.

Table 7.3.1A-0f: 2DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_1A-3A | 1 | 1950 | 5 | 25 | 2140 | 23 | FDD | IMD3 |
|  | 3 | 1760 | 5 | 25 | 1855 | N/A |  | N/A |
| CA_1A-8A | 1 | 1965 | 5 | 25 | 2155 | 6 | FDD | IMD4 |
|  | 8 | 887.5 | 5 | 25 | 932.5 | N/A |  | N/A |
| CA_2A-4A | 2 | 1860 | 20 | 502 | 1940 | 5 | FDD | IMD3 |
|  | 4 | 1752.5 | 5 | 25 | 2152.5 | N/A |  | N/A |
| CA_2A-4A | 2 | 1868.3 | 5 | 25 | 1948.3 | N/A | FDD | N/A |
|  | 4 | 1735 | 5 | 25 | 2135 | 5 |  | IMD5 |
| CA_2A-46A | 2 | 1880 | 5 | 25 | 1960 | 12.0 | FDD | IMD34 |
|  | 46 | 5720 | 20 | 100 | 5720 | N/A |  | N/A |
| CA_2A-48A | 2 | 1852.5 | 5 | 25 | 1932.5 | [12] | FDD | IMD4 |
|  | 48 | 3625 | 20 | 100 | 3625 | N/A | TDD | N/A |
| CA_2A-49A | 2 | 1852.5 | 5 | 25 | 1932.5 | [12] | FDD | IMD4 |
|  | 49 | 3625 | 20 | 100 | 3625 | N/A | TDD | N/A |
| CA_2A-66A | 2 | 1855 | 5 | 25 | 1935 | 20 | FDD | IMD3 |
|  | 66 | 1775 | 5 | 25 | 2175 | N/A |  | N/A |
| CA_2A-66A | 2 | 1883.3 | 5 | 25 | 1963.3 | N/A | FDD | N/A |
|  | 66 | 1750 | 5 | 25 | 2150 | 4 |  | IMD5 |
| CA_3A-5A | 3 | 1771 | 10 | 50 | 1866 | 4 | FDD | IMD4 |
|  | 5 | 838 | 5 | 25 | 883 | N/A |  | N/A |
| CA_3A-5A | 3 | 1721 | 10 | 50 | 1816 | N/A | FDD | N/A |
|  | 5 | 838 | 5 | 25 | 883 | 24 |  | IMD24 |
| CA_3A-7A | 3 | 1730 | 5 | 25 | 1825 | N/A | FDD | N/A |
|  | 7 | 2535 | 10 | 50 | 2655 | 13 |  | IMD4 |
| CA_3A-8A | 3 | 1755 | 10 | 50 | 1850 | N/A | FDD | N/A |
|  | 8 | 900 | 5 | 25 | 945 | 8 |  | IMD44 |
| CA_3A-8A | 3 | 1747.5 | 10 | 50 | 1842.5 | 6.4 | FDD | IMD5 |
|  | 8 | 897.5 | 5 | 25 | 942.5 | N/A |  | N/A |
| CA_3A-18A | 3 | 1721 | 5 | 25 | 1816 | 4 | FDD | IMD4 |
|  | 18 | 823 | 5 | 25 | 868 | N/A |  | N/A |
| CA_3A-19A | 3 | 1771 | 5 | 25 | 1866 | 4 | FDD | IMD4 |
|  | 19 | 838 | 5 | 25 | 883 | N/A |  | N/A |
| CA_3A-19A | 3 | 1721 | 5 | 25 | 1816 | N/A | FDD | N/A |
|  | 19 | 838 | 5 | 25 | 883 | 27 |  | IMD24 |
| CA_3A-20A | 3 | 1775 | 5 | 25 | 1870 | 4 | FDD | IMD4 |
|  | 20 | 840 | 5 | 25 | 799 | N/A |  | N/A |
| CA_3A-20A | 3 | 1735 | 5 | 25 | 1830 | N/A | FDD | N/A |
|  | 20 | 847 | 5 | 25 | 806 | 9 |  | IMD4 |
| CA_3A-26A | 3 | 1771 | 5 | 25 | 1866 | 4 | FDD | IMD4 |
|  | 26 | 838 | 5 | 25 | 883 | N/A |  | N/A |
| CA_3A-26A | 3 | 1721 | 5 | 25 | 1816 | N/A | FDD | N/A |
|  | 26 | 838 | 5 | 25 | 883 | 26 |  | IMD24 |
| CA_3A-38A | 3 | 1713 | 5 | 25 | 1808 | 8.2 | FDD | IMD4 |
|  | 38 | 2617 | 5 | 25 | 2617 | N/A | TDD | N/A |
| CA_3A-41A | 3 | 1740 | 5 | 25 | 1835 | 8.2 | FDD | IMD4 |
|  | 41 | 2657.5 | 5 | 25 | 2657.5 | N/A | TDD | N/A |
| CA_3A-42A | 3 | 1740 | 5 | 25 | 1835 | 29.8 | FDD | IMD24 |
|  | 42 | 3575 | 5 | 25 | 3575 | N/A | TDD | N/A |
| CA_3A-42A | 3 | 1765 | 5 | 25 | 1860 | 8.0 | FDD | IMD44 |
|  | 42 | 3435 | 5 | 25 | 3435 | N/A | TDD | N/A |
| CA_4A-5A | 4 | 1721 | 5 | 25 | 2121 | N/A | FDD | N/A |
|  | 5 | 838 | 5 | 25 | 883 | 26 |  | IMD24 |
| CA_4A-7A | 4 | 1730 | 5 | 25 | 2130 | N/A | FDD | N/A |
|  | 7 | 2535 | 5 | 25 | 2655 | 15 |  | IMD4 |
| CA_5A-7A | 5 | 834 | 5 | 25 | 879 | 12 | FDD | IMD34 |
|  | 7 | 2547 | 10 | 50 | 2667 | N/A |  | N/A |
| CA_5A-66A | 5 | 838 | 5 | 25 | 883 | 30 | FDD | IMD24 |
|  | 66 | 1721 | 5 | 25 | 2121 | N/A |  | N/A |
| CA_7A-20A | 7 | 2512 | 10 | 50 | 2632 | N/A | FDD | N/A |
|  | 20 | 851 | 5 | 25 | 810 | 12 |  | IMD34 |
| CA_7A-26A | 7 | 2556 | 5 | 25 | 2676 | N/A | FDD-FDD | N/A |
|  | 26 | 837 | 5 | 25 | 882 | 16.0 |  | IMD3 |
|  | 7 | 2567.5 | 5 | 25 | 2687.5 | 2.5 | FDD-FDD | IMD5 |
|  | 26 | 816.5 | 5 | 25 | 861.5 | N/A |  | N/A |
| CA_8A-20A | 20 | 849.5 | 5 | 25 | 808.5 | 25 | FDD | IMD34 |
|  | 8 | 890.5 | 5 | 25 | 935.5 | N/A |  | N/A |
|  | 20 | 847.5 | 5 | 25 | 806.5 | N/A |  | N/A |
|  | 8 | 892.5 | 5 | 25 | 937.5 | 25 |  | IMD34 |
| CA_8A-41A | 8 | 882.5 | 5 | 25 | 927.5 | 12.1 | FDD | IMD34 |
|  | 41 | 2685 | 10 | 50 | 2685 | N/A | TDD | N/A |
| CA_19A-42A3 | 19 | N/A | N/A | N/A | N/A | N/A | FDD | N/A |
|  | 42 | N/A | N/A | N/A | N/A | N/A | TDD | N/A |
| CA_21A-28A | 21 | 1450.4 | 5 | 25 | 1498.4 | [2.5] | FDD | IMD5 |
|  | 28 | 735.5 | 5 | 25 | 790.5 | N/A | TDD | N/A |
| CA_21A-42A3 | 21 | N/A | N/A | N/A | N/A | N/A | FDD | N/A |
|  | 42 | N/A | N/A | N/A | N/A | N/A | TDD | N/A |
| CA_28A-42A | 28 | 705.5 | 5 | 25 | 760.5 | [5.5] | FDD | IMD5 |
|  | 42 | 3582.5 | 5 | 25 | 3582.5 | N/A | TDD | N/A |
| CA_40A-42A | 40 | 2350 | 5 | 25 | 2350 | N/A | TDD | N/A |
|  | 42 | 3500 | 5 | 25 | 3550 | 5 |  | IMD4 |
|  | 40 | 2350 | 5 | 25 | 2350 | 5 | TDD | IMD4 |
|  | 42 | 3525 | 5 | 25 | 3525 | N/A |  | N/A |
| NOTE 1: Both of the transmitters shall be set min(+20 dBm, PCMAX_L,c) as defined in subclause 6.2.5A NOTE 2: RBSTART = 0 NOTE 3: No requirements apply when there is at least one individual RE within the intermodulation generated by the dual uplink is within the downlink transmission bandwidth of the FDD band. The reference sensitivity should only be verified when this is not the case (the requirements specified in clause 7.3.1 apply). NOTE 4: This band is subject to IMD5 also which MSD is not specified. NOTE 5: Void NOTE 6: For operations with 4 antenna ports, the MSD in the applicable bands shall be modified by the absolute value of ΔRIB,4R in Table 7.3.1-1a when MSD > 0. |  |  |  |  |  |  |  |  |

Table 7.3.1A-0g: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA | EUTRA CA | EUTRA band | UL Fc | UL BW | UL | DL Fc | DL BW | MSD | Duplex mode |  |
| DL Configuration | UL Configuration |  | (MHz) | (MHz) | CLRB | (MHz) | (MHz) | (dB) |  |  |
| CA_1A-3A-28A | CA_1A-28A | 1 | 1975 | 5 | 25 | 2165 | 5 | N/A | FDD | N/A |
|  |  | 28 | 710.5 | 5 | 25 | 765.5 | 5 | N/A |  | N/A |
|  |  | 3 | 1723.5 | 5 | 25 | 1818.5 | 5 | 4.0 |  | IMD5 |
|  | CA_3A-28A | 3 | 1780 | 5 | 25 | 1875 | 5 | N/A | FDD | N/A |
|  |  | 28 | 710.5 | 5 | 25 | 765.5 | 5 | N/A |  | N/A |
|  |  | 1 | 1949 | 5 | 25 | 2139 | 5 | 11.0 |  | IMD4 |
| CA_1A-3A-40A | CA_1A-3A | 1 | 1950 | 5 | 25 | 2140 | 5 | N/A | FDD | N/A |
|  |  | 3 | 1735 | 5 | 25 | 1830 | 5 | N/A | FDD | N/A |
|  |  | 40 | 2380 | 5 | 25 | 2380 | 5 | 8.0 | TDD | IMD5 |
| CA_1A-3A-41A | CA_1A-3A | 1 | 1977.5 | 5 | 25 | 2167.5 | 5 | N/A | FDD | N/A |
|  |  | 3 | 1712.5 | 5 | 25 | 1807.5 | 5 | N/A | FDD | N/A |
|  |  | 41 | 2507.5 | 5 | 25 | 2507.5 | 5 | 5.0 | TDD | IMD5 |
| CA_1A-3A-42A | CA_1A-3A | 1 | 1922.5 | 5 | 25 | 2112.5 | 5 | N/A | FDD | N/A |
|  |  | 3 | 1782.5 | 5 | 25 | 1877.5 | 5 | N/A | FDD | N/A |
|  |  | 42 |  |  |  | 3425 | 5 | 13.0 | TDD | IMD4 |
| CA_1A-5A-7A | CA_1A-7A | 1 | 1968 | 5 | 25 | 2158 | 5 | N/A | FDD | N/A |
|  |  | 7 | 2512 | 10 | 50 | 2632 | 10 | N/A |  | N/A |
|  |  | 5 | 835 | 5 | 25 | 880 | 5 | 1.0 |  | IMD5 |
| CA_1A-5A-40A | CA_1A-5A | 1 | 1977.5 | 5 | 25 | 2167.5 | 5 | N/A | FDD | N/A |
|  |  | 5 | 826.5 | 5 | 25 | 871.5 | 5 | N/A | FDD | N/A |
|  |  | 40 | 2305 | 10 | 50 | 2305 | 10 | 9.0 | TDD | IMD4 |
| CA_1A-7A-26A | CA_1A-7A | 1 | 1965 | 5 | 25 | 2155 | 5 | N/A | FDD | N/A |
|  |  | 7 | 2510 | 10 | 50 | 2630 | 10 | N/A |  | N/A |
|  |  | 26 | 830 | 5 | 50 | 875 | 5 | 3.5 |  | IMD5 |
| CA_1A-7A-28A | CA_1A-7A | 1 | 1935 | 5 | 25 | 2125 | 5 | N/A | FDD | N/A |
|  |  | 7 | 2510 | 10 | 50 | 2630 | 10 | N/A |  | N/A |
|  |  | 28 | 730 | 10 | 50 | 785 | 10 | 4.5 |  | IMD5 |
|  | CA_1A-28A | 1 | 1935 | 5 | 25 | 2125 | 5 | N/A | FDD | N/A |
|  |  | 28 | 730 | 10 | 50 | 785 | 10 | N/A |  | N/A |
|  |  | 7 | 2545 | 10 | 50 | 2665 | 10 | 28.0 |  | IMD2 |
| CA_1A-8A-20A | CA_1A-8A | 1 | 1925 | 5 | 25 | 2115 | 5 | N/A | FDD | N/A |
|  |  | 8 | 910 | 5 | 25 | 955 | 5 | N/A |  | N/A |
|  |  | 20 | 846 | 5 | 25 | 805 | 5 | 11.5 |  | IMD4 |
| CA_1A-28A-42A | CA_1A-28A | 1 | 1955 | 5 | 25 | 2145 | 5 | N/A | FDD | N/A |
|  |  | 28 | 735 | 5 | 25 | 790 | 5 | N/A | FDD | N/A |
|  |  | 42 | 3425 | 5 | 25 | 3425 | 5 | 15.0 | TDD | IMD3 |
|  | CA_28A-42A | 28 | 710.5 | 5 | 25 | 765.5 | 5 | N/A | FDD | N/A |
|  |  | 42 | 3560 | 5 | 25 | 3560 | 5 | N/A | TDD | N/A |
|  |  | 1 | 1949 | 5 | 25 | 2139 | 5 | 11.0 | FDD | IMD3 |
| CA_2A-12A-30A | CA_2A-12A | 2 | 1885 | 5 | 25 | 1965 | 5 | N/A | FDD | N/A |
|  |  | 12 | 708.5 | 5 | 25 | 738.5 | 5 | N/A |  | N/A |
|  |  | 30 | 2308 | 5 | 25 | 2353 | 5 | 12.0 |  | IMD4 |
| CA_2A-2A-4A-5A | CA_2A-5A | 2 | 1900 | 5 | 25 | 1980 | 5 | N/A | FDD | N/A |
|  |  | 5 | 834 | 5 | 25 | 879 | 5 |  |  | N/A |
|  |  | 4 | 1732 | 5 | 25 | 2132 | 5 | 7.6 |  | IMD4 |
| CA_2A-4A-13A | CA_2A-13A | 2 | 1855 | 5 | 25 | 1935 | 5 | N/A | FDD | N/A |
|  |  | 13 | 782 | 5 | 25 | 751 | 5 |  |  | N/A |
|  |  | 4 | 1746 | 5 | 25 | 2146 | 5 | 7.6 |  | IMD4 |
|  | CA_4A-13A | 4 | 1750 | 5 | 25 | 2150 | 5 | N/A | FDD | N/A |
|  |  | 13 | 780 | 5 | 25 | 749 | 5 |  |  | N/A |
|  |  | 2 | 1860 | 5 | 25 | 1940 | 5 | 6.2 |  | IMD4 |
| CA_2A-2A-5A-66A-66A, CA_2A-5A-66A, CA_2A-5A-66B, CA_2A-5A-66C, CA_2A-5B-66A, CA_2A-5B-66B, CA_2A-5B-66C, CA_2A-2A-5A-66A, CA_2A-2A-5A-66B, CA_2A-2A-5A-66C, CA_2A-5A-66A-66A | CA_2A-5A | 2 | 1900 | 5 | 25 | 1980 | 5 | N/A | FDD | N/A |
|  |  | 5 | 834 | 5 | 25 | 879 | 5 |  |  | N/A |
|  |  | 66 | 1712 | 5 | 25 | 2132 | 5 | 7.2 |  | IMD4 |
| CA_2A-5B-66A-66A | CA_2A-5A | 2 | 1900 | 5 | 25 | 1980 | 5 | N/A | FDD | N/A |
|  |  | 5 | 834 | 5 | 25 | 879 | 5 |  |  | N/A |
|  |  | 66 | 1712 | 5 | 25 | 2132 | 5 | 7.2 |  | IMD4 |
| CA_2A-13A-66A-66B | CA_2A-13A | 2 | 1860 | 5 | 25 | 1940 | 5 | N/A | FDD | N/A |
|  |  | 13 | 782 | 5 | 25 | 751 | 5 |  |  | N/A |
|  |  | 66 | 1736 | 5 | 25 | 2156 | 5 | 7.2 |  | IMD4 |
| CA_2A-13A-66A-66B | CA_13A-66A | 2 | 1880 | 5 | 25 | 1960 | 5 | 6.2 | FDD | IMD4 |
|  |  | 13 | 782 | 5 | 25 | 751 | 5 | N/A |  | N/A |
|  |  | 66 | 1762 | 5 | 25 | 2162 | 5 |  |  | N/A |
| CA_2A-48A-66A CA_2A-48C-66A | CA_48A-66A | 2 | 1880 | 5 | 25 | 1960 | 5 | 28.3 | FDD-TDD | IMD2 |
|  |  | 48 | 3695 | 5 | 25 | 3695 | 5 | N/A |  | N/A |
|  |  | 66 | 1735 | 5 | 25 | 2135 | 5 | N/A |  | N/A |
| CA_2A-48A-66A CA_2A-48C-66A | CA_2A-48A | 2 | 1905 | 5 | 25 | 1985 | 5 | N/A | FDD-TDD | N/A |
|  |  | 48 | 3560 | 5 | 25 | 3560 | 5 | N/A |  | N/A |
|  |  | 66 | 1755 | 5 | 25 | 2155 | 5 | 12.1 |  | IMD4 |
| CA_3A-5A-7A | CA_3A-5A | 3 | 1780 | 10 | 50 | 1875 | 10 | N/A | FDD | N/A |
|  |  | 5 | 845 | 5 | 25 | 890 | 5 | N/A |  | N/A |
|  |  | 7 | 2505 | 10 | 50 | 2625 | 10 | 30.0 |  | IMD21 |
|  | CA_3A-7A | 3 | 1725 | 10 | 50 | 1820 | 10 | N/A | FDD | N/A |
|  |  | 7 | 2565 | 10 | 50 | 2685 | 10 | N/A |  | N/A |
|  |  | 5 | 840 | 5 | 25 | 885 | 5 | 19.0 |  | IMD3 |
| CA_3A-7A-8A CA_3C-7A-8A | CA_3A-7A | 3 | 1735 | 5 | 25 | 1830 | 5 | N/A | FDD | N/A |
|  |  | 7 | 2530 | 10 | 50 | 2650 | 10 |  |  | N/A |
|  |  | 8 | 895 | 5 | 25 | 940 | 5 | 18.0 |  | IMD3 |
|  | CA_3A-8A | 3 | 1780 | 5 | 25 | 1875 | 5 | N/A | FDD | N/A |
|  |  | 8 | 890 | 5 | 25 | 935 | 5 |  |  | N/A |
|  |  | 7 | 2550 | 10 | 50 | 2670 | 10 | 29.0 |  | IMD2+IMD34 |
| CA_3A-7A-20A | CA_3A-7A | 3 | 1737 | 5 | 25 | 1832 | 5 | N/A | FDD | N/A |
|  |  | 7 | 2543 | 10 | 50 | 2663 | 10 | N/A |  | N/A |
|  |  | 20 | 847 | 10 | 20 | 806 | 10 | 10.5 |  | IMD2 |
|  | CA_3A-20A | 3 | 1775 | 10 | 50 | 1870 | 10 | N/A | FDD | N/A |
|  |  | 20 | 855 | 5 | 25 | 896 | 5 | N/A |  | N/A |
|  |  | 7 | 2510 | 10 | 50 | 2630 | 10 | 26.0 |  | IMD21 |
| CA_3A-7A-26A | CA_3A-7A | 3 | 1720 | 5 | 25 | 1815 | 5 | N/A | FDD | N/A |
|  |  | 7 | 2560 | 10 | 50 | 2680 | 10 | N/A |  | N/A |
|  |  | 26 | 835 | 5 | 25 | 880 | 5 | 17.5 |  | IMD3 |
| CA_3A-7A-26A | CA_3A-26A | 3 | 1780 | 5 | 25 | 1875 | 5 | N/A | FDD | N/A |
|  |  | 26 | 845 | 5 | 25 | 890 | 5 | N/A |  | N/A |
|  |  | 7 | 2505 | 10 | 50 | 2625 | 10 | 29.0 |  | IMD21 |
| CA_3A-7A-28A | CA_3A-7A | 3 | 1747 | 5 | 25 | 1842 | 5 | N/A | FDD | N/A |
|  |  | 7 | 2543 | 5 | 25 | 2663 | 5 | N/A |  | N/A |
|  |  | 28 | 741 | 5 | 25 | 796.0 | 5 | 20.0 |  | IMD2 |
|  | CA_3A-28A | 3 | 1712.5 | 5 | 25 | 1807.5 | 5 | N/A | FDD | N/A |
|  |  | 28 | 743 | 5 | 25 | 798 | 5 | N/A |  | N/A |
|  |  | 7 | 2562 | 5 | 25 | 2682 | 5 | 17.0 |  | IMD3 |
|  | CA_7A-28A | 7 | 2543 | 5 | 25 | 2663 | 5 | N/A | FDD | N/A |
|  |  | 28 | 710.5 | 5 | 25 | 765.5 | 5 | N/A |  | N/A |
|  |  | 3 | 1737.5 | 5 | 25 | 1832.5 | 5 | 26.0 |  | IMD2 |
| CA_3A-7A-32A | CA_3A-7A | 3 | 1775 | 5 | 25 | 1870 | 5 | N/A | FDD | N/A |
|  |  | 7 | 2510 | 10 | 50 | 2630 | 10 | N/A |  | N/A |
|  |  | 32 | - | - | - | 1470 | 5 | 10.5 |  | IMD4 |
| CA_3A-8A-20A | CA_3A-8A | 3 | 1720 | 5 | 25 | 1815 | 5 | N/A | FDD | N/A |
|  |  | 8 | 910 | 5 | 25 | 955 | 5 | N/A |  | N/A |
|  |  | 20 | 851 | 5 | 25 | 810 | 5 | 27.0 |  | IMD2 |
| CA_3A-8A-38A | CA_3A-8A | 3 | 1720 | 5 | 25 | 1815 | 5 | N/A | FDD-TDD | N/A |
|  |  | 8 | 890 | 5 | 25 | 935 | 5 | N/A |  | N/A |
|  |  | 38 | 2610 | 5 | 25 | 2610 | 5 | 26.4 |  | IMD2 |
|  | CA_3A-8A | 3 | 1750 | 5 | 25 | 1845 | 5 | N/A | FDD-TDD | N/A |
|  |  | 8 | 900 | 5 | 25 | 945 | 5 | N/A |  | N/A |
|  |  | 38 | 2600 | 5 | 25 | 2600 | 5 | 15.7 |  | IMD3 |
| CA_3A-11A-18A | CA_3A-11A | 3 | 1725 | 5 | 25 | 1820 | 5 | N/A | FDD | N/A |
|  |  | 11 | 1440 | 5 | 25 | 1448 | 5 | N/A |  | N/A |
|  |  | 18 | 825 | 5 | 25 | 870 | 5 | 4.9 |  | IMD5 |
|  | CA_11A-18A | 11 | 1432 | 5 | 25 | 1481 | 5 | N/A | FDD | N/A |
|  |  | 18 | 820 | 5 | 25 | 865 | 5 | N/A |  | N/A |
|  |  | 3 | 1753 | 5 | 25 | 1848 | 5 | 4.0 |  | IMD5 |
| CA_3A-11A-26A | CA_3A-11A | 3 | 1725 | 5 | 25 | 1820 | 5 | N/A | FDD | N/A |
|  |  | 11 | 1440 | 5 | 25 | 1448 | 5 | N/A |  | N/A |
|  |  | 26 | 825 | 5 | 25 | 870 | 5 | 4.9 |  | IMD5 |
|  | CA_3A-26A | 3 | 1782.5 | 5 | 25 | 1877.5 | 5 | N/A | FDD | N/A |
|  |  | 26 | 816.5 | 5 | 25 | 861.5 | 5 | N/A |  | N/A |
|  |  | 11 | 1435.5 | 5 | 25 | 1483.5 | 5 | 5.0 |  | IMD5 |
|  | CA_11A-26A | 11 | 1440 | 5 | 25 | 1488 | 5 | N/A | FDD | N/A |
|  |  | 26 | 824 | 5 | 25 | 869 | 5 | N/A |  | N/A |
|  |  | 3 | 1761 | 5 | 25 | 1856 | 5 | 4.5 |  | IMD5 |
| CA_3A-19A-21A | CA_19A-21A | 19 | 832.5 | 5 | 25 | 877.5 | 5 | N/A | FDD | N/A |
|  |  | 21 | 1460.4 | 5 | 25 | 1508.4 | 5 | N/A |  | N/A |
|  |  | 3 | 1774.6 | 5 | 25 | 1869.6 | 5 | 4.0 |  | IMD5 |
|  |  | 3 | 1760 | 5 | 25 | 1855 | 5 | N/A | FDD | N/A |
| CA_3A-20A-38A | CA_3A-20A | 20 | 850 | 5 | 25 | 809 | 5 | N/A |  | N/A |
|  |  | 38 | 2610 | 5 | 25 | 2610 | 5 | 28.4 | TDD | IMD21 |
| CA_3A-21A-28A | CA_3A-21A | 3 | 1782 | 5 | 25 | 1877 | 5 | N/A | FDD | N/A |
|  |  | 21 | 1451 | 5 | 25 | 1499 | 5 | N/A |  | N/A |
|  |  | 28 | 734 | 5 | 25 | 789 | 5 | 3.0 |  | IMD5 |
| CA_3A-28A-41A | CA_3A-41 | 3 | 1720 | 5 | 25 | 1815 | 5 | N/A | FDD | N/A |
|  |  | 41 | 2510 | 5 | 25 | 2510 | 5 | N/A | TDD | N/A |
|  |  | 28 | 735 | 5 | 25 | 790 | 5 | 26.0 | FDD | IMD21 |
| CA_3A-41A-42A | CA_41A-42A | 41 | 2640 | 10 | 50 | 2640 | 10 | N/A | TDD | N/A |
|  |  | 42 | 3425 | 10 | 50 | 3425 | 10 |  | TDD | N/A |
|  |  | 3 | 1760 | 5 | 25 | 1855 | 5 | 16.0 | FDD | IMD3 |
| CA_5A-46D-66A | CA_5A_46D | 5 | 834 | 5 | 25 | 879 | 5 | N/A | FDD-TDD | N/A |
|  |  | 46 | 5491 | 20 | 100 | 5491 | 20 |  |  | N/A |
|  |  | 66 | 1755 | 5 | 25 | 2155 | 5 | 0.3 |  | IMD5 |
| CA_13A-48A-66A | CA_13A-48A | 13 | 782 | 5 | 25 | 751 | 5 | N/A | FDD-TDD | N/A |
|  |  | 48 | 3695 | 5 | 25 | 3695 | 5 | N/A |  | N/A |
|  |  | 66 | 1731 | 5 | 25 | 2131 | 5 | 17.1 |  | IMD3 |
| CA_19A-21A-42A | CA_19A-21A | 19 | 842.5 | 5 | 25 | 887.5 | 5 | N/A | FDD | N/A |
|  |  | 21 | 1450.4 | 5 | 25 | 1498.4 | 5 | N/A | FDD | N/A |
|  |  | 42 | 3508.7 | 5 | 25 | 3508.7 | 5 | 13.0 | TDD | IMD4 |
|  | CA_21A-42A | 21 | 1460.4 | 5 | 25 | 1508.4 | 5 | N/A | FDD | N/A |
|  |  | 42 | 3500 | 5 | 25 | 3500 | 5 | N/A | FDD | N/A |
|  |  | 19 | 836.2 | 5 | 25 | 881.2 | 5 | 13.0 | TDD | IMD4 |
| CA_28A-41A-42A | CA_41A-42A | 41 | 2672 | 10 | 50 | 2672 | 10 | N/A | TDD | N/A |
|  |  | 42 | 3460 | 10 | 50 | 3460 | 10 |  | TDD | N/A |
|  |  | 28 | 733 | 5 | 25 | 788 | 5 | 26.0 | FDD | IMD2 |
| CA_1A-21A-42A6 | CA_1A-42A | 1 |  |  |  |  |  |  | FDD | N/A |
|  |  | 42 |  |  |  |  |  |  | TDD | N/A |
|  |  | 21 |  |  |  |  |  |  | FDD | N/A |
| CA_2A-5A-48A CA_2A-5A-48C CA_2A-5A-48D | CA_5A-48A | 2 | 1882 | 5 | 25 | 1962 | 5 | 15.6 | FDD-TDD | IMD3 |
|  |  | 5 | 839 | 5 | 25 | 884 | 5 | N/A |  | N/A |
|  |  | 48 | 3640 | 5 | 25 | 3640 | 5 | N/A |  | N/A |
| CA_2A-5A-48C CA_2A-5A-48D | CA_2A-5A | 2 | 1905 | 5 | 25 | 1985 | 5 | N/A | FDD-TDD | N/A |
|  |  | 5 | 844 | 5 | 25 | 889 | 5 | N/A |  | N/A |
|  |  | 48 | 3593 | 5 | 25 | 3593 | 5 | 16.6 |  | IMD3 |
| CA_2A-13A-48A CA_2A-13A-48C CA_2A-13A-48D | CA_13A-48A | 2 | 1903.5 | 5 | 25 | 1983.5 | 5 | 15.6 | FDD-TDD | IMD3 |
|  |  | 13 | 784.5 | 5 | 25 | 753.5 | 5 | N/A |  | N/A |
|  |  | 48 | 3552.5 | 5 | 25 | 3552.5 | 5 | N/A |  | N/A |
| CA_2A-48A-66A, CA_2A-48D-66A, CA_2A-48E-66A, CA_2A-48A-66A-66A, CA_2A-48C-66A-66A, CA_2A-48D-66A-66A, CA_2A-48E-66A-66A | CA_2A-66A | 2 | 1855 | 5 | 25 | 1935 | 5 | N/A | FDD-TDD | N/A |
|  |  | 48 | 3625 | 5 | 25 | 3625 | 5 | 32.0 |  | IMD2 |
|  |  | 66 | 1770 | 5 | 25 | 2190 | 5 | N/A |  | N/A |
| CA_2A-14A-66A, CA_2A-2A-14A-66A, CA_2A-14A-66A-66A, CA_2A-2A-14A-66A-66A, CA_2A-14A-66A-66A-66A | CA_2A-14A | 2 | 1870 | 5 | 25 | 1950 | 5 | N/A | FDD | N/A |
|  |  | 14 | 793 | 5 | 25 | 763 | 5 | N/A |  | N/A |
|  |  | 66 | 1734 | 5 | 25 | 2154 | 5 | 7.2 |  | IMD4 |
|  | CA_14A-66A | 2 | 1874 | 5 | 25 | 1954 | 5 | 6.2 | FDD | IMD4 |
|  |  | 14 | 793 | 5 | 25 | 763 | 5 | N/A |  | N/A |
|  |  | 66 | 1770 | 5 | 25 | 2190 | 5 | N/A |  | N/A |
| NOTE 1: This band is subject to IMD3 also which MSD is not specified. NOTE 1: Both of the transmitters shall be set min(+20 dBm, PCMAX_L,c) as defined in subclause 6.2.5A NOTE 2: RBSTART = 0 NOTE 3: Void NOTE 4: This MSD requirement apply with both IMD2 and IMD3 products should be generated. NOTE 5: For operations with 4 antenna ports, the MSD in the applicable bands shall be modified by the absolute value of ΔRIB,4R in Table 7.3.1-1a when MSD > 0. NOTE 6: Due to the spectrum holdings of the operator, the deployed frequency ranges do not result MSD to interested downlink channel. Therefore, no requirements apply for this CA configuration. |  |  |  |  |  |  |  |  |  |  |

For intra-band contiguous carrier aggregation the throughput of each component carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.3.1-1, Table 7.3.1-1a, Table 7.3.1-1A, Table 7.3.1-1B, Table 7.3.1-1C, Table 7.3.1A-0h and Table 7.3.1A-1. For operating bands with an unpaired DL part (as noted in Table 5.5-1), the power levels in Table 7.3.1-1 and Table 7.3.1-1a also apply for an SCC assigned in the unpaired part. The requirement is verified using an uplink CA configuration with the largest number of carriers supported by the UE. Table 7.3.1A-0h, Table 7.3.1A-1 and Table 7.3.1A-2 specifies the maximum number of allocated uplink resource blocks for which the intra-band contiguous carrier aggregation reference sensitivity requirement shall be met. The PCC and SCC allocations as defined in Table 7.3.1A-0h, Table 7.3.1A-1 and Table 7.3.1A-2 form a contiguous allocation where TX–RX frequency separations of the component carriers are as defined in Table 5.7.4-1. In case downlink CA configuration has additional SCC(s) compared to uplink CA configuration those are configured furthers away from uplink band. For UE(s) supporting one uplink carrier, the uplink configuration of the PCC shall be in accordance with Table 7.3.1-2 and the downlink PCC carrier center frequency shall be configured closer to uplink operating band than any of the downlink SCC center frequency. Unless given by Table 7.3.1-3, the reference sensitivity requirements shall be verified with the network signalling value NS_01 (Table 6.2.4-1) configured.

Table 7.3.1A-0h: Intra-band contiguous CA uplink configuration for reference sensitivity for Bandwidth Class B

| CA configuration / CC combination / NRB_agg / Duplex mode |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Uplink CA configuration | 25RB+25RB |  | 50RB+25RB |  | 50RB+50RB |  | 75RB+25RB |  | Duplex Mode |
|  | PCC | SCC | PCC | SCC | PCC | SCC | PCC | SCC |  |
| CA_5B | N/A | N/A | 25 | 0 | 25 | 0 | N/A | N/A | FDD |
| CA_8B | N/A | N/A | 25 | 0 | 25 | 0 | N/A | N/A | FDD |
| CA_66B | 25 | 25 | 50 | 25 | 50 | 50 | 75 | 25 | FDD |
| NOTE 1: The carrier centre frequency of SCC in the UL operating band is configured closer to the DL operating band. NOTE 2: The transmitted power over both PCC and SCC shall be set to PUMAX as defined in subclause 6.2.5A. NOTE 3: The UL resource blocks in both PCC and SCC shall be confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.6-1). NOTE 4: The UL resource blocks in PCC shall be located as close as possible to the downlink operating band, while the UL resource blocks in SCC shall be located as far as possible from the downlink operating band. NOTE 5: In case a CA configuration consists of CC channel bandwidths which are unequal in bandwidth the PCC channel bandwidth shall be the larger one for reference sensitivity test. |  |  |  |  |  |  |  |  |  |

Table 7.3.1A-1: Intra-band contiguous CA uplink configuration for reference sensitivity for Bandwidth Class C

| CA configuration / CC combination / NRB_agg / Duplex mode |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Uplink CA configuration | 100RB+25RB |  | 100RB+50RB |  | 75RB+75RB |  | 75RB+50RB |  | 100RB+75RB |  | 100RB+100RB |  | Duplex Mode |
|  | PCC | SCC | PCC | SCC | PCC | SCC | PCC | SCC | PCC | SCC | PCC | SCC |  |
| CA_1C | N/A | N/A | N/A | N/A | 75 | 54 | N/A | N/A | N/A | N/A | 100 | 30 | FDD |
| CA_3C | 50 | 0 | 50 | 0 | N/A | N/A | N/A | N/A | 50 | 0 | 50 | 0 | FDD |
| CA_7C | N/A | N/A | 75 | 0 | 75 | 0 | 75 | 0 | 75 | 0 | 75 | 0 | FDD |
| CA_38C | N/A | N/A | N/A | N/A | 75 | 75 | N/A | N/A | N/A | N/A | 100 | 100 | TDD |
| CA_39C | 100 | 25 | 100 | 50 | N/A | N/A | N/A | N/A | 100 | 75 | N/A | N/A | TDD |
| CA_40C | N/A | N/A | 100 | 50 | 75 | 75 | N/A | N/A | 100 | 75 | 100 | 100 | TDD |
| CA_41C | 100 | 25 | 100 | 50 | 75 | 75 | 75 | 50 | 100 | 75 | 100 | 100 | TDD |
| CA_42C | 100 | 25 | 100 | 50 | N/A | N/A | N/A | N/A | 100 | 75 | 100 | 100 | TDD |
| CA_48C | 100 | 25 | 100 | 50 | N/A | N/A | N/A | N/A | 100 | 75 | 100 | 100 | TDD |
| CA_66C | 100 | 25 | 100 | 50 | 75 | 75 | 75 | 50 | 100 | 75 | 100 | 100 | FDD |
| NOTE 1: The carrier centre frequency of SCC in the UL operating band is configured closer to the DL operating band. NOTE 2: The transmitted power over both PCC and SCC shall be set to PUMAX as defined in subclause 6.2.5A. NOTE 3: The UL resource blocks in both PCC and SCC shall be confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.6-1). NOTE 4: The UL resource blocks in PCC shall be located as close as possible to the downlink operating band, while the UL resource blocks in SCC shall be located as far as possible from the downlink operating band. NOTE 5: In case a CA configuration consists of CC channel bandwidths which are unequal in bandwidth the PCC channel bandwidth shall be the larger one for reference sensitivity test. NOTE 6: Void. NOTE 7: Void |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3.1A-2: Intra-band contiguous CA uplink configuration for reference sensitivity for Bandwidth Class D

| CA configuration / CC combination / NRB_agg / Duplex mode |  |  |  |  |
| --- | --- | --- | --- | --- |
| Uplink CA configuration | CC combination | Number of uplink resource blocks per CC |  |  |
|  |  | PCC | SCC | SCC |
| CA_40D | 75RB+75RB+75RB 100RB+75RB+75RB 100RB+100RB+50RB 100RB+100RB+75RB 100RB+100RB+100RB | NOTE 6 |  |  |
| CA_41D | 75RB+75RB+75RB 100RB+75RB+75RB 100RB+100RB+50RB 100RB+100RB+75RB 100RB+100RB+100RB | NOTE 6 |  |  |
| NOTE 1: The carrier centre frequency of SCC in the UL operating band is configured closer to the DL operating band. NOTE 2: The transmitted power over both PCC and SCC shall be set to PUMAX as defined in subclause 6.2.5A. NOTE 3: The UL resource blocks in both PCC and SCC shall be confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.6-1). NOTE 4: The UL resource blocks in PCC shall be located as close as possible to the downlink operating band, while the UL resource blocks in SCC shall be located as far as possible from the downlink operating band. NOTE 5: In case a CA configuration consists of CC channel bandwidths which are unequal in bandwidth the PCC channel bandwidth shall be the larger one for reference sensitivity test. NOTE 6: All uplink CCs are fully allocated |  |  |  |  |

For intra-band non-contiguous carrier aggregation with one uplink carrier and two or more downlink sub-blocks, the throughput of each downlink component carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) and parameters specified in Table 7.3.1-1, Table 7.3.1-1a, Table 7.3.1-1A, Table 7.3.1-1B, Table 7.3.1-1C and Table 7.3.1A-3 with the reference sensitivity power level increased by  RIBNC given in Table 7.3.1A-3 for the SCC(s). For aggregation of more than two downlink FDD carriers with one uplink carrier the reference sensitivity is defined only for the specific uplink and downlink test points which are specified in Table 7.3.1A-3. The requirements apply with all downlink carriers active. Unless given by Table 7.3.1-3, the reference sensitivity requirements shall be verified with the network signalling value NS_01 (Table 6.2.4-1) configured.

Table 7.3.1A-3: Intra-band non-contiguous CA with one uplink configuration for reference sensitivity

| CA configuration | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | UL PCC allocation | ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- |
| CA_1A-1A | 25RB + 25RB | 0.0 < Wgap ≤ 50.0 | 251 | 0.5 | FDD |
|  | 25RB + 50RB | 0.0 < Wgap ≤ 45.0 | 251 | 0.5 |  |
|  | 25RB + 75RB | 0.0 < Wgap ≤ 40.0 | 251 | 0.5 |  |
|  | 25RB + 100RB | 0.0 < Wgap ≤ 35.0 | 251 | 0.5 |  |
|  | 50RB + 25RB | 0.0 < Wgap ≤ 45.0 | 501 | 0.5 |  |
|  | 50RB + 50RB | 0.0 < Wgap ≤ 40.0 | 501 | 0.5 |  |
|  | 50RB + 75RB | 0.0 < Wgap ≤ 35.0 | 501 | 0.5 |  |
|  | 50RB + 100RB | 0.0 < Wgap ≤ 30.0 | 501 | 0.5 |  |
|  | 75RB + 25RB | 0.0 < Wgap ≤ 40.0 | 751 | 0.5 |  |
|  | 75RB + 50RB | 0.0 < Wgap ≤ 35.0 | 751 | 0.5 |  |
|  | 75RB + 75RB | 0.0 < Wgap ≤ 30.0 | 751 | 0.5 |  |
|  | 75RB + 100RB | 0.0 < Wgap ≤ 25.0 | 751 | 0.5 |  |
|  | 100RB + 25RB | 20.0 < Wgap ≤ 35.0 | 901 | 0.5 |  |
|  |  | 0.0 < Wgap ≤ 20.0 | 1001 | 0.5 |  |
|  | 100RB + 50RB | 15.0 < Wgap ≤ 30.0 | 901 | 0.5 |  |
|  |  | 0.0 < Wgap ≤ 15.0 | 1001 | 0.5 |  |
|  | 100RB + 75RB | 10.0 < Wgap ≤ 25.0 | 901 | 0.5 |  |
|  |  | 0.0 < Wgap ≤ 10.0 | 1001 | 0.5 |  |
|  | 100RB + 100RB | 5.0 < Wgap ≤ 20.0 | 901 | 0.5 |  |
|  |  | 0.0 < Wgap ≤ 5.0 | 1001 | 0.5 |  |
| CA_2A-2A | 25RB+25RB | 30.0 < Wgap ≤ 50.0 | 121 | 5.3 | FDD |
|  |  | 0.0 < Wgap ≤ 30.0 | 251 | 0 |  |
|  | 25RB+50RB | 25.0 < Wgap ≤ 45.0 | 121 | 4.4 |  |
|  |  | 0.0 < Wgap ≤ 25.0 | 251 | 0 |  |
|  | 25RB+75RB | 20.0 < Wgap ≤ 40.0 | 121 | 4.2 |  |
|  |  | 0.0 < Wgap ≤ 20.0 | 251 | 0 |  |
|  | 25RB+100RB | 15.0 < Wgap ≤ 35.0 | 121 | 3.8 |  |
|  |  | 0.0 < Wgap ≤ 15.0 | 251 | 0 |  |
|  | 50RB+25RB | 15.0 < Wgap ≤ 45.0 | 121 | 5.9 |  |
|  |  | 0.0 < Wgap ≤ 15.0 | 321 | 0 |  |
|  | 50RB+50RB | 10.0 < Wgap ≤ 40.0 | 121 | 4.6 |  |
|  |  | 0.0 < Wgap ≤ 10.0 | 321 | 0 |  |
|  | 50RB+75RB | 5.0 < Wgap ≤ 35.0 | 121 | 4.1 |  |
|  |  | 0.0 < Wgap ≤ 5.0 | 321 | 0 |  |
|  | 50RB+100RB | 0.0 < Wgap ≤ 30.0 | 121 | 4.0 |  |
|  | 75RB+25RB | 10.0 < Wgap ≤ 40.0 | 1212 | 6.7 |  |
|  |  | 0.0 < Wgap ≤ 10.0 | 361 | 0 |  |
|  | 75RB+50RB | 5.0 < Wgap ≤ 35.0 | 1212 | 5.4 |  |
|  |  | 0.0 < Wgap ≤ 5.0 | 361 | 0 |  |
|  | 75RB+75RB | 0.0 < Wgap ≤ 30.0 | 1212 | 4.6 |  |
|  | 75RB+100RB | 0.0 < Wgap ≤ 25.0 | 1212 | 4.2 |  |
|  | 100RB+25RB | 0.0 < Wgap ≤ 35.0 | 1613 | 7.2 |  |
|  | 100RB+50RB | 0.0 < Wgap ≤ 30.0 | 1613 | 5.8 |  |
|  | 100RB+75RB | 0.0 < Wgap ≤ 25.0 | 1613 | 5.0 |  |
|  | 100RB+100RB | 0.0 < Wgap ≤ 20.0 | 1613 | 4.6 |  |
| CA_3A-3A | 15RB+25RB | 47.0 < Wgap ≤ 67.0 | 121 | 4.5 | FDD |
|  |  | 0.0 < Wgap ≤ 47.0 | 151 | 0 |  |
|  | 25RB+15RB | 47.0 < Wgap ≤ 67.0 | 121 | 4.9 |  |
|  |  | 0.0 < Wgap ≤ 47.0 | 251 | 0 |  |
|  | 25RB+25RB | 45.0 < Wgap ≤ 65.0 | 121 | 4.7 |  |
|  |  | 0.0 < Wgap ≤ 45.0 | 251 | 0 |  |
|  | 25RB+50RB | 40.0 < Wgap ≤ 60.0 | 121 | 3.8 |  |
|  |  | 0.0 < Wgap ≤ 40.0 | 251 | 0 |  |
|  | 25RB+75RB | 35.0 < Wgap ≤ 55.0 | 121 | 3.6 |  |
|  |  | 0.0 < Wgap ≤ 35.0 | 251 | 0 |  |
|  | 25RB+100RB | 30.0 < Wgap ≤ 50.0 | 121 | 3.4 |  |
|  |  | 0.0 < Wgap ≤ 30.0 | 251 | 0 |  |
|  | 50RB+25RB | 30.0 < Wgap ≤ 60.0 | 129 | 5.1 |  |
|  |  | 0.0 < Wgap ≤ 30.0 | 321 | 0 |  |
|  | 50RB+50RB | 25.0 < Wgap ≤ 55.0 | 129 | 4.3 |  |
|  |  | 0.0 < Wgap ≤ 25.0 | 321 | 0 |  |
|  | 50RB+75RB | 20.0 < Wgap ≤ 50.0 | 129 | 3.8 |  |
|  |  | 0.0 < Wgap ≤ 20.0 | 321 | 0 |  |
|  | 50RB+100RB | 15.0 < Wgap ≤ 45.0 | 129 | 3.4 |  |
|  |  | 0.0 < Wgap ≤ 15.0 | 321 | 0 |  |
|  | 75RB+25RB | 25.0 < Wgap ≤ 55.0 | 1210 | 6.0 |  |
|  |  | 0.0 < Wgap ≤ 25.0 | 321 | 0 |  |
|  | 75RB+50RB | 20.0 < Wgap ≤ 50.0 | 1210 | 4.7 |  |
|  |  | 0.0 < Wgap ≤ 20.0 | 321 | 0 |  |
|  | 75RB+75RB | 15.0 < Wgap ≤ 45.0 | 1210 | 4.2 |  |
|  |  | 0.0 < Wgap ≤ 15.0 | 321 | 0 |  |
|  | 75RB+100RB | 10.0 < Wgap ≤ 40.0 | 1210 | 3.8 |  |
|  |  | 0.0 < Wgap ≤ 10.0 | 321 | 0 |  |
|  | 100RB+25RB | 15.0 < Wgap ≤ 50.0 | 1611 | 6.5 |  |
|  |  | 0.0 < Wgap ≤ 15.0 | 321 | 0 |  |
|  | 100RB+50RB | 10.0 < Wgap ≤ 45.0 | 1611 | 5.1 |  |
|  |  | 0.0 < Wgap ≤ 10.0 | 321 | 0 |  |
|  | 100RB+75RB | 5.0 < Wgap ≤ 40.0 | 1611 | 4.5 |  |
|  |  | 0.0 < Wgap ≤ 5.0 | 321 | 0 |  |
|  | 100RB+100RB | 0.0 < Wgap ≤ 35.0 | 1611 | 4.1 |  |
| CA_4A-4A | NOTE 6 | NOTE 7 | NOTE 8 | 0.0 | FDD |
| CA_5A-5A | 25RB+25RB | NOTE 7 | 121 | 5.3 | FDD |
|  | 25RB+50RB | NOTE 7 | 121 | 4.4 |  |
|  | 50RB+25RB | NOTE 7 | 121 | 5.9 |  |
|  | 50RB+50RB | NOTE 7 | 121 | 4.6 |  |
| CA_7A-7A | 25RB+25RB | 0< Wgap ≤ 60 | 25 | 0.0 | FDD |
|  | 25RB+50RB | 0 < Wgap ≤ 55 | 25 | 0.0 |  |
|  | 25RB+75RB | 0 < Wgap ≤ 50 | 25 | 0.0 |  |
|  | 25RB+100RB | 0 < Wgap ≤ 45 | 25 | 0.0 |  |
|  | 50RB+25RB | 30 < Wgap ≤ 55 | 321 | 0.0 |  |
|  |  | 0 < Wgap ≤ 30 | 50 | 0.0 |  |
|  | 50RB+50RB | 25.0 < Wgap ≤ 50.0 | 321 | 0.0 |  |
|  |  | 0.0 < Wgap ≤ 25.0 | 50 | 0.0 |  |
|  | 50RB+75RB | 20 < Wgap ≤ 45 | 321 | 0.0 |  |
|  |  | 0 < Wgap ≤ 20 | 50 | 0.0 |  |
|  | 50RB+100RB | 15 < Wgap ≤ 40 | 321 | 0.0 |  |
|  |  | 0 < Wgap ≤ 15 | 50 | 0.0 |  |
|  | 75RB+25RB | 20.0 < Wgap ≤ 50.0 | 321 | 0.0 |  |
|  |  | 0.0 < Wgap ≤ 20.0 | 501 | 0.0 |  |
|  | 75RB+50RB | 20.0 < Wgap ≤ 45.0 | 321 | 0.0 |  |
|  |  | 0.0 < Wgap ≤ 20.0 | 501 | 0.0 |  |
|  | 75RB+75RB | 15.0 < Wgap ≤ 40.0 | 321 | 0.0 |  |
|  |  | 0.0 < Wgap ≤ 15.0 | 501 | 0.0 |  |
|  | 75RB+100RB | 10 < Wgap ≤ 35 | 321 | 0.0 |  |
|  |  | 0 < Wgap ≤ 10 | 501 | 0.0 |  |
|  | 100RB+25RB | 25 < Wgap ≤ 45 | 321 | 0.0 |  |
|  |  | 0 < Wgap ≤ 25 | 451 | 0.0 |  |
|  | 100RB+50RB | 20 < Wgap ≤ 40 | 321 | 0.0 |  |
|  |  | 0 < Wgap ≤ 20 | 451 | 0.0 |  |
|  | 100RB+75RB | 15.0 < Wgap ≤ 35.0 | 361 | 0.0 |  |
|  |  | 0.0 < Wgap ≤ 15.0 | 501 | 0.0 |  |
|  | 100RB+100RB | 15.0 < Wgap ≤ 30.0 | 321 | 0.0 |  |
|  |  | 0.0 < Wgap ≤ 15.0 | 451 | 0.0 |  |
| CA_12A-12A | 25RB+25RB | 0.0 < Wgap ≤ 7.0 | 518 | 3 | FDD |
| CA_23A-23A | NOTE 6 | NOTE 7 | NOTE 8 | 0.0 | FDD |
| CA_25A-25A | 25RB+25RB | 30.0 < Wgap ≤ 55.0 | 101 | 5.0 | FDD |
|  |  | 0.0 < Wgap ≤ 30.0 | 251 | 0.0 |  |
|  | 25RB+50RB | 25.0 < Wgap ≤ 50.0 | 101 | 4.5 |  |
|  |  | 0.0 < Wgap ≤ 25.0 | 251 | 0.0 |  |
|  | 25RB+75RB | 20 < Wgap ≤ 45 | 101 | 4.3 |  |
|  |  | 0 < Wgap ≤ 20 | 251 | 0 |  |
|  | 25RB+100RB | 15 < Wgap ≤ 40 | 101 | 4.1 |  |
|  |  | 0 < Wgap ≤ 15 | 251 | 0 |  |
|  | 50RB+25RB | 15.0 < Wgap ≤ 50.0 | 104 | 5.5 |  |
|  |  | 0.0 < Wgap ≤ 15.0 | 321 | 0.0 |  |
|  | 50RB+50RB | 10.0 < Wgap ≤ 45.0 | 104 | 5.0 |  |
|  |  | 0.0 < Wgap ≤ 10.0 | 321 | 0.0 |  |
|  | 50RB+75RB | 5 < Wgap ≤ 40 | 104 | 4.5 |  |
|  |  | 0 < Wgap ≤ 5 | 321 | 0 |  |
|  | 50RB+100RB | 0 < Wgap ≤ 35 | 104 | 4.2 |  |
|  | 75RB+25RB | 10 < Wgap ≤ 45 | 1014 | 7.6 |  |
|  |  | 0 < Wgap ≤ 10 | 321 | 0 |  |
|  | 75RB+50RB | 5 < Wgap ≤ 40 | 1014 | 6.7 |  |
|  |  | 0 < Wgap ≤ 5 | 321 | 0 |  |
|  | 75RB+75RB | 0 < Wgap ≤ 35 | 1014 | 5.6 |  |
|  | 75RB+100RB | 0 < Wgap ≤ 30 | 1014 | 4.8 |  |
|  | 100RB+25RB | 0 < Wgap ≤ 40 | 1215 | 8 |  |
|  | 100RB+50RB | 0 < Wgap ≤ 35 | 1215 | 6.7 |  |
|  | 100RB+75RB | 0 < Wgap ≤ 30 | 1215 | 6.1 |  |
|  | 100RB+100RB | 0 < Wgap ≤ 25 | 1215 | 5.7 |  |
| CA_25A-25A-25A | 25RB+25RB+25RB | Wgap_L = Wgap_H = 25 MHz | 101 | 5.0 | FDD |
|  |  | Wgap_L = Wgap_H = 5 MHz | 251 | 0.0 |  |
|  | 50RB+50RB+50RB | Wgap_L = Wgap_H = 17.5 MHz | 104 | 5.0 |  |
|  | 75RB+75RB+75RB | Wgap_L = Wgap_H = 10 MHz | 1014 | 5.6 |  |
|  | 100RB+100RB+100RB | Wgap_L = Wgap_H = 2.5 MHz | 1215 | 5.7 |  |
| CA_40A-40A | NOTE 6 | NOTE 7 | NOTE 8 | 0.0 | TDD |
| CA_40A-40C | NOTE 6 | NOTE 7 | NOTE 8 | 0.0 | TDD |
| CA_40C-40C | NOTE 6 | NOTE 7 | NOTE 8 | 0.0 | TDD |
| CA_41A-41A | NOTE 6 | NOTE 7 | NOTE 8 | 0.0 | TDD |
| CA_41A-41C | NOTE 6 | NOTE 7 | NOTE 8 | 0.0 | TDD |
| CA_41A-41D | NOTE 6 | NOTE 7 | NOTE 8 | 0.0 | TDD |
| CA_41C-41C | NOTE 6 | NOTE 7 | NOTE 8 | 0.0 | TDD |
| CA_41A-41A-41A | NOTE 6 | NOTE 7 | NOTE 8 | 0.0 | TDD |
| CA_41A-41A-41C | NOTE 6 | NOTE 7 | NOTE 8 | 0.0 | TDD |
| CA_42A-42A | NOTE 6 | NOTE 7 | NOTE 8 | 0.0 | TDD |
| CA_42A-42C | NOTE 6 | NOTE 7 | NOTE 8 | 0.0 | TDD |
| CA_42A-42D | NOTE 6 | NOTE 7 | NOTE 8 | 0.0 | TDD |
| CA_42C-42C | NOTE 6 | NOTE 7 | NOTE 8 | 0.0 | TDD |
| CA_48A-48A | NOTE 6 | NOTE 7 | NOTE 8 | 0.0 | TDD |
| CA_48A-48A-48A | NOTE 6 | NOTE 7 | NOTE 8 | 0.0 | TDD |
| CA_48A-48A-48A-48A | NOTE 6 | NOTE 7 | NOTE 8 | 0.0 | TDD |
| CA_48A-48C | NOTE 6 | NOTE 7 | NOTE 8 | 0.0 | TDD |
| CA_48A-48D | NOTE 6 | NOTE 7 | NOTE 8 | 0.0 | TDD |
| CA_48A-48E | NOTE 6 | NOTE 7 | NOTE 8 | 0.0 | TDD |
| CA_48C-48C | NOTE 6 | NOTE 7 | NOTE 8 | 0.0 | TDD |
| CA_48C-48D | NOTE 6 | NOTE 7 | NOTE 8 | 0.0 | TDD |
| CA_66A-66A | NOTE 6 | NOTE 7 | NOTE 8, NOTE 16 | 0.0 | FDD |
| CA_66A-66A-66A | NOTE 6 | NOTE 7 | NOTE 8, NOTE 16 | 0.0 | FDD |
| CA_66A-66B | NOTE 6 | NOTE 7 | NOTE 8, NOTE 16 | 0.0 | FDD |
| CA_66A-66C | NOTE 6 | NOTE 7 | NOTE 8, NOTE 16 | 0.0 | FDD |
| NOTE 1: 1 refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission. NOTE 2: Wgap is the sub-block gap between the two sub-blocks. NOTE 3: The carrier center frequency of PCC in the UL operating band is configured closer to the DL operating band. NOTE 4: 4 refers to the UL resource blocks shall be located at RBstart=33. NOTE 5: For the TDD intra-band non-contiguous CA configurations, the minimum requirements apply only in synchronized operation between all component carriers. NOTE 6: All combinations of channel bandwidths defined in Table 5.6A.1-3. NOTE 7: All applicable sub-block gap sizes. NOTE 8: The PCC allocation is same as Transmission bandwidth configuration NRB as defined in Table 5.6-1. In case of uplink sub-block is TDD intra-band contiguous CA then the uplink PCC and SCC allocations are the same as NRB_agg defined in Table 7.3.1A-1. NOTE 9: 9 refers to the UL resource blocks shall be located at RBstart=25. NOTE 10: 10 refers to the UL resource blocks shall be located at RBstart=35. NOTE 11: 11 refers to the UL resource blocks shall be located at RBstart=50. NOTE 12: 12 refers to the UL resource blocks shall be located at RBstart=39. NOTE 13: 13 refers to the UL resource blocks shall be located at RBstart=57. NOTE 14: 14 refers to the UL resource blocks shall be located at RBstart=44. NOTE 15: 15 refers to the UL resource blocks shall be located at RBstart=62. NOTE 16: The carrier center frequency of PCC in the DL operating band is configured closer to the UL operating band. NOTE 17: Void NOTE 18: 18 refers to the UL resource blocks shall be located at RBstart=12. NOTE 19: For operations with 4 antenna ports, the MSD in the applicable bands shall be modified by the absolute value of ΔRIB,4R in Table 7.3.1-1a when MSD > 0. |  |  |  |  |  |

For intra-band non-contiguous carrier aggregation with two uplink and downlink carriers the reference sensitivity is defined to be met with both downlink and uplink carriers activated. The downlink PCC and SCC minimum requirements for reference sensitivity power level as specified in Table 7.3.1-1, Table 7.3.1-1a, Table 7.3.1-1A, Table 7.3.1-1B and Table 7.3.1-1C are increased by amount of ΔR2UL_PCC and ΔR2UL_SCC  which are defined in Table 7.3.1A-4 when uplink PCC and SCC allocations are according to the Table 7.3.1A-4.

Table 7.3.1A-4: Intra-band non-contiguous CA with two uplinks configuration for reference sensitivity

| CA configuration | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | UL PCC allocation | UL SCC allocation | ΔR2UL_PCC (dB) | ΔR2UL_SCC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_4A-4A | NOTE 2 | NOTE 3 | NOTE 4 | NOTE 5 | 0.0 | 0.0 | FDD |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5A. NOTE 2: All combinations of channel bandwidths defined in Table 5.6A.1-3. NOTE 3: All applicable sub-block gap sizes. NOTE 4: The PCC allocation is same as Transmission bandwidth configuration NRB as defined in Table 5.6-1. NOTE 5: The SCC allocation is same as Transmission bandwidth configuration NRB as defined in Table 5.6-1. |  |  |  |  |  |  |  |

For combinations of intra-band and inter-band carrier aggregation, the requirement is defined with an uplink configuration in accordance with Table 7.3.1A-3 when the uplink is active in the band supporting two or more non-contigous component carriers, Table 7.3.1A-1 when the uplink (up to two contiguously aggregated uplink carriers) is active in a band supporting two contiguous component carriers and in accordance with Table 7.3.1-2 when an uplink is active in a band supporting one carrier per band. The downlink PCC shall be configured closer to the uplink operating band than the downlink SCC(s) when the uplink is active in band(s) supporting contiguous aggregation. The carrier center frequency of PCC in the UL operating band is configured closer to the DL operating band when the uplink is active in band(s) supporting non-contiguous aggregation. For these uplink configurations, the UE shall meet the reference sensitivity requirements for intra-band non-contiguous carrier aggregation of two or more downlink sub-blocks, the requirements for intra-band contiguous carrier aggregation for the contiguously aggregated downlink carriers and for any remaining component carrier(s) the requirements specified in subclause 7.3.1. For the two or more component carriers within the same band, RIBNC = 0 dB for all sub-block gaps (Table 7.3.1A-3) when the uplink is active in another band. All downlink carriers shall be active throughout the tests and the requirements for the downlinks shall be met with all uplink carriers active in each band capable of UL operation. For component carriers configured in Band 46, the said requirements for intra-band carrier aggregation of downlink carriers are replaced by the requirements in Table 7.3.1A-0eA for the uplink in any band other than band 46 with the uplink configuration specified in Table 7.3.1-2. Unless given by Table 7.3.1-3, the reference sensitivity requirements shall be verified with the network signalling value NS_01 (Table 6.2.4-1) configured.

Table 7.3.1A-5: Void

Table 7.3.1A-6: Void

## <<End of Change>>