3GPP TSG-RAN4 Meeting #102-e	  R4-2204043

# Online, , 21th Feb – 3rd March 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-3 | CR | 0681 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Big CR for Rel-17 Dual Connectivity (DC) of 1 LTE band (1DL/1UL) and 1 NR band (1DL/1UL) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CHTTL |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | DC_R17_1BLTE_1BNR_2DL2UL-Core |  |  |  |  |  | Date: |  |  | 2022-03-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Completed DC configurations after RAN4#101-bis-e and RAN4#102-e are specified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | EN-DC configuration specific requirements are specified. Completed EN-DC configurations from the latest TR (R4-2204045): Inter-band EN-DC in FR1: DC_11A_n1A DC_5A_n1A DC_5A_n3A DC_2A_n25A Inter-band NE-DC in FR1: DC_n28A_8B DC_n41A_3C DC_n41C_3A DC_n41A_8B DC_n41C_8A DC_n41A_39C DC_n41C_39A DC_n41A_40C DC_n41C_40A Some configurations are completed via draft CR approach. This CR covers the following endorsed draft CR. Endorsed draft CR in RAN4#101-bis-e: R4-2200185 Draft CR for 38.101-3: corrections of MOP in DC_42C_n1A R4-2202162 draftCR to add DC_3C_n20A to 38.101-3 R4-2202164 draft CR 38.101-3 to add new configurations for DC_2_n78 and DC_71_n78 R4-2206379 Draft Correction CR R17 TS38.101-3 on MSD for DC_18_n28 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Completed DC configurations are not specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5B.4, 5.5B.4a, 6.2B.1.3, 6.2B.4.2.3.1, 6.5B.3.3.2, 7.3B.2.3, 7.3B.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 38.521-3 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of changes >>

5.5B.4	Inter-band EN-DC within FR1

5.5B.4.1	Inter-band EN-DC configurations within FR1 (two bands)

Table 5.5B.4.1-1: Inter-band EN-DC configurations within FR1 (two bands)

| EN-DC configuration |  | Uplink EN-DC configuration (NOTE 1) |  | Single UL allowed |  | DL interruption allowed (Note 14) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A_n3A DC_1C_n3A |  | DC_1A_n3A DC_1C_n3A |  | DC_1_n3 |  |  |  |
| DC_1A_n5A |  | DC_1A_n5A |  | No |  |  |  |
| DC_1A_n7A DC_1A_n7B |  | DC_1A_n7A |  | No |  |  |  |
| DC_1A-1A_n7A DC_1A-1A_n7B |  | DC_1A_n7A |  | No |  |  |  |
| DC_1A_n8A |  | DC_1A_n8A |  | No |  |  |  |
| DC_1A_n20A |  | DC_1A_n20A |  | No |  |  |  |
| DC_1A_n28A |  | DC_1A_n28A |  | No |  |  |  |
| DC_1A-1A_n28A |  | DC_1A_n28A |  | No |  |  |  |
| DC_1A_n38A DC_1C_n38A |  | DC_1A_n38A |  | No |  |  |  |
| DC_1A_n40A DC_1A_n40B |  | DC_1A_n40A |  | No |  |  |  |
| DC_1A_n41A7 |  | DC_1A_n41A |  | No |  |  |  |
| DC_1A_n50A |  | DC_1A_n50A |  | No |  |  |  |
| DC_1A_n51A |  | DC_1A_n51A |  | No |  |  |  |
| DC_1A_n71A DC_1A_n71B |  | DC_1A_n71A |  | No |  |  |  |
| DC_1A_n77A7 DC_1A_n77C7 |  | DC_1A_n77A |  | DC_1_n77 |  | No |  |
| DC_1A_n77(2A)7 DC_1A_n77(3A)7 |  | DC_1A_n77A |  | DC_1_n77 |  | No |  |
| DC_1A_n78A7 DC_1A_n78C7 |  | DC_1A_n78A |  | No |  | No |  |
| DC_1A_n78(2A)7 |  | DC_1A_n78A |  | No |  | No |  |
| DC_1A-1A_n78A |  | DC_1A_n78A |  | No |  | No |  |
| DC_1A_n79A7 DC_1A_n79C7 |  | DC_1A_n79A |  | No |  | No |  |
| DC_2A_n5A |  | DC_2A_n5A |  | No |  |  |  |
| DC_2A-2A_n5A |  | DC_2A_n5A |  | No |  |  |  |
| DC_2A_n7A |  | DC_2A_n7A |  | No |  |  |  |
| DC_2A_n7(2A) |  | DC_2A_n7A |  | No |  |  |  |
| DC_2A_n12A |  | DC_2A_n12A |  | No |  |  |  |
| DC_2A_n25A11, 13, 20 |  | n25A |  | DC_2_n25 |  |  |  |
| DC_2A_n28A |  | DC_2A_n28A |  | No |  |  |  |
| DC_2A_n30A |  | DC_2A_n30A |  | No |  |  |  |
| DC_2A-2A_n30A |  | DC_2A_n30A |  | No |  |  |  |
| DC_2A_n38A |  | DC_2A_n38A |  | No |  |  |  |
| DC_2A-2A_n38A |  | DC_2A_n38A |  | No |  |  |  |
| DC_2A_n41A DC_2A_n41C DC_2C_n41A |  | DC_2A_n41A DC_2C_n41A |  | No |  |  |  |
| DC_2A_n41(2A) |  | DC_2A_n41A |  | No |  |  |  |
| DC_2A-2A_n41A |  | DC_2A_n41A |  | No |  |  |  |
| DC_2A_n46A |  | DC_2A_n46A |  | No |  |  |  |
| DC_2A_n48A DC_2A_n48B |  | DC_2A_n48A |  | No |  |  |  |
| DC_2A_n66A |  | DC_2A_n66A |  | DC_2_n66 |  |  |  |
| DC_2A_n66(2A) |  | DC_2A_n66A |  | DC_2_n66 |  |  |  |
| DC_2A-2A_n66A |  | DC_2A_n66A |  | DC_2_n66 |  |  |  |
| DC_2A_n71A DC_2A_n71B DC_2C_n71A |  | DC_2A_n71A DC_2C_n71A |  | No |  |  |  |
| DC_2A-2A_n71A |  | DC_2A_n71A |  | No |  |  |  |
| DC_2A_n77A DC_2A_n77C |  | DC_2A_n77A |  | DC_2_n77 |  |  |  |
| DC_2A-2A_n77A DC_2A-2A_n77C |  | DC_2A_n77A |  | DC_2_n77 |  |  |  |
| DC_2A_n78A |  | DC_2A_n78A |  | DC_2_n78 |  |  |  |
| DC_2A-2A_n78(2A) |  | DC_2A_n78A |  | DC_2_n78 |  |  |  |
| DC_2A_n78(2A) |  | DC_2A_n78A |  | DC_2_n78 |  |  |  |
| DC_2A-2A_n78A |  | DC_2A_n78A |  | DC_2_n78 |  |  |  |
| DC_3A_n1A DC_3C_n1A |  | DC_3A_n1A DC_3C_n1A |  | DC_3_n1 |  |  |  |
| DC_3A-3A_n1A |  | DC_3A_n1A |  | DC_3_n1 |  |  |  |
| DC_3A_n5A DC_3C_n5A |  | DC_3A_n5A DC_3C_n5A |  | DC_3_n5 |  |  |  |
| DC_3A_n7A DC_3A_n7B DC_3C_n7A DC_3C_n7B |  | DC_3A_n7A DC_3A_n7B DC_3C_n7A |  | No |  |  |  |
| DC_3A-3A_n7A DC_3A-3A_n7B |  | DC_3A_n7A |  | No |  |  |  |
| DC_3A_n8A |  | DC_3A_n8A |  | No |  |  |  |
| DC_3A-3A_n8A |  | DC_3A_n8A |  | No |  |  |  |
| DC_3A_n20A DC_3C_n20A |  | DC_3A_n20A |  | No |  |  |  |
| DC_3A_n28A DC_3C_n28A |  | DC_3A_n28A DC_3C_n28A |  | No |  |  |  |
| DC_3A_n34A |  | DC_3A_n34A |  | No |  |  |  |
| DC_3A_n38A DC_3C_n38A |  | DC_3A_n38A |  | No |  |  |  |
| DC_3A_n40A DC_3A_n40B |  | DC_3A_n40A |  | No |  |  |  |
| DC_3A_n41A7 DC_3C_n41A |  | DC_3A_n41A DC_3C_n41A |  | DC_3_n41 |  | No |  |
| DC_3A_n50A |  | DC_3A_n50A |  | No |  |  |  |
| DC_3A_n51A |  | DC_3A_n51A |  | No |  |  |  |
| DC_3A_n71A DC_3A_n71B |  | DC_3A_n71A |  | No |  |  |  |
| DC_3A_n77A7 DC_3A_n77C7 DC_3C_n77A7 |  | DC_3A_n77A DC_3C_n77A |  | DC_3_n77 |  | No |  |
| DC_3A_n77(2A)7 DC_3A_n77(3A)7 DC_3C_n77(2A)7 |  | DC_3A_n77A DC_3C_n77A |  | DC_3_n77 |  | No |  |
| DC_3A-3A_n77A7 |  | DC_3A_n77A |  | DC_3_n77 |  | No |  |
| DC_3A_n78A7 DC_3A_n78C7 DC_3C_n78A7 |  | DC_3A_n78A DC_3C_n78A |  | DC_3_ n78 |  | No |  |
| DC_3A_n78(2A)7 DC_3C_n78(2A)7 |  | DC_3A_n78A |  | DC_3_n78 |  | No |  |
| DC_3A-3A_n78A7 |  | DC_3A_n78A |  | DC_3_n78 |  | No |  |
| DC_3A_n79A7 DC_3A_n79C7 DC_3C_n79A7 |  | DC_3A_n79A DC_3C_n79A |  | No |  | No |  |
| DC_4A_n2A |  | DC_4A_n2A |  | No |  |  |  |
| DC_4A_n5A |  | DC_4A_n5A |  | DC_4_n5 |  |  |  |
| DC_4A_n7A |  | DC_4A_n7A |  | No |  |  |  |
| DC_4A_n28A |  | DC_4A_n28A |  | No |  |  |  |
| DC_4A_n38A |  | DC_4A_n38A |  | No |  |  |  |
| DC_4A_n41A |  | DC_4A_n41A |  | No |  |  |  |
| DC_4A_n78A |  | DC_4A_n78A |  | No |  |  |  |
| DC_4A_n78(2A) |  | DC_4A_n78A |  | No |  |  |  |
| DC_5A_n1A |  | DC_5A_n1A |  | No |  |  |  |
| DC_5A_n2A DC_5B_n2A |  | DC_5A_n2A |  | No |  |  |  |
| DC_5A-5A_n2A |  | DC_5A_n2A |  | No |  |  |  |
| DC_5A_n3A |  | DC_5A_n3A |  | DC_5_n3 |  |  |  |
| DC_5A_n7A |  | DC_5A_n7A |  | DC_5_n7 |  |  |  |
| DC_5A_n7(2A) |  | DC_5A_n7A |  | DC_5_n7 |  |  |  |
| DC_5A_n12A |  | DC_5A_n12A |  | No |  |  |  |
| DC_5A_n30A |  | DC_5A_n30A |  | No |  |  |  |
| DC_5A_n38A |  | DC_5A_n38A |  | DC_5_n38 |  |  |  |
| DC_5A_n40A |  | DC_5A_n40A |  | No |  |  |  |
| DC_5A_n48A DC_5A_n48B |  | DC_5A_n48A |  | No |  |  |  |
| DC_5A_n66A DC_5B_n66A |  | DC_5A_n66A |  | DC_5_n66 |  |  |  |
| DC_5A-5A_n66A |  | DC_5A_n66A |  | DC_5_n66 |  |  |  |
| DC_5A_n77A DC_5A_n77C |  | DC_5A_n77A |  | No |  |  |  |
| DC_5A_n77(2A) |  | DC_5A_n77A |  | No |  |  |  |
| DC_5A_n71A |  | DC_5A_n71A |  | No |  |  |  |
| DC_5A_n78A7 DC_5A_n78C7 |  | DC_5A_n78A |  | No |  | No |  |
| DC_5A_n78(2A)7 |  | DC_5A_n78A |  | No |  | No |  |
| DC_5A_n79A |  | DC_5A_n79A |  | No |  | No |  |
| DC_7A_n1A DC_7C_n1A |  | DC_7A_n1A DC_7C_n1A |  | No |  |  |  |
| DC_7A-7A_n1A |  | DC_7A_n1A |  | No |  |  |  |
| DC_7A_n2A DC_7C_n2A |  | DC_7A_n2A |  | No |  |  |  |
| DC_7A_n3A DC_7C_n3A |  | DC_7A_n3A DC_7C_n3A |  | No |  |  |  |
| DC_7A_n5A DC_7C_n5A |  | DC_7A_n5A DC_7C_n5A |  | DC_7_n5 |  |  |  |
| DC_7A-7A_n5A |  | DC_7A_n5A |  | DC_7_n5 |  |  |  |
| DC_7A_n8A |  | DC_7A_n8A |  | No |  |  |  |
| DC_7A-7A_n8A |  | DC_7A_n8A |  | No |  |  |  |
| DC_7A-7A_n78A7 DC_7A-7A_n78C7 |  | DC_7A_n78A |  | No |  |  |  |
| DC_7A-7A_n78(2A)7 |  | DC_7A_n78A |  | No |  |  |  |
| DC_7A_n20A |  | DC_7A_n20A |  | No |  |  |  |
| DC_7A_n25A DC_7C_n25A |  | DC_7A_n25A |  | No |  |  |  |
| DC_7A-7A_n25A |  | DC_7A_n25A |  | No |  |  |  |
| DC_7A_n28A DC_7C_n28A |  | DC_7A_n28A DC_7C_n28A |  | No |  |  |  |
| DC_7A_n40A |  | DC_7A_n40A |  | Yes |  |  |  |
| DC_7A_n51A |  | DC_7A_n51A |  | No |  |  |  |
| DC_7A_n66A DC_7C_n66A |  | DC_7A_n66A |  | No |  |  |  |
| DC_7A-7A_n66A |  | DC_7A_n66A |  | No |  |  |  |
| DC_7A_n71A |  | DC_7A_n71A |  | No |  |  |  |
| DC_7A_n77A7 DC_7C_n77A |  | DC_7A_n77A |  | No |  |  |  |
| DC_7A_n77(2A) DC_7C_n77(2A) |  | DC_7A_n77A |  | No |  |  |  |
| DC_7A-7A_n77A7 |  | DC_7A_n77A |  | No |  |  |  |
| DC_7A-7A_n77(2A) |  | DC_7A_n77A |  | No |  |  |  |
| DC_7A_n78A7 DC_7C_n78A7 DC_7A_n78C7 |  | DC_7A_n78A DC_7C_n78A |  | No |  |  |  |
| DC_7A_n78(2A)7 DC_7C_n78(2A)7 |  | DC_7A_n78A DC_7C_n78A |  | No |  |  |  |
| DC_7A_n79A DC_7A_n79C |  | DC_7A_n79A |  | No |  |  |  |
| DC_8A_n1A |  | DC_8A_n1A |  | No |  |  |  |
| DC_8A_n2A |  | DC_8A_n2A |  | DC_8_n2 |  |  |  |
| DC_8A_n3A |  | DC_8A_n3A |  | No |  |  |  |
| DC_8A_n7A |  | DC_8A_n7A |  | No |  |  |  |
| DC_8A_n20A |  | DC_8A_n20A |  | Yes |  |  |  |
| DC_8A_n28A |  | DC_8A_n28A |  | No |  |  |  |
| DC_8A_n34A |  | DC_8A_n34A |  | No |  |  |  |
| DC_8A_n39A |  | DC_8A_n39A |  | No |  |  |  |
| DC_8A_n40A7 |  | DC_8A_n40A |  | No |  |  |  |
| DC_8A_n41A7 DC_8A_n41C |  | DC_8A_n41A |  | No |  | No |  |
| DC_8A_n41(2A) |  | DC_8A_n41A |  | No |  | No |  |
| DC_8A_n77A7 |  | DC_8A_n77A |  | No |  | No |  |
| DC_8A_n77(2A)7 DC_8A_n77(3A)7 |  | DC_8A_n77A |  | No |  | No |  |
| DC_8A_n78A7 |  | DC_8A_n78A |  | No |  | No |  |
| DC_8A_n78(2A)7 |  | DC_8A_n78A |  | No |  | No |  |
| DC_8A_n79A7 DC_8A_n79C |  | DC_8A_n79A DC_8A_n79C |  | No |  | No |  |
| DC_8A_n93A |  | DC_8A_n93A_ULSUP-TDM |  | N/A |  |  |  |
| DC_8A_n94A |  | DC_8A_n94A_ULSUP-TDM |  | N/A |  |  |  |
| DC_11A_n1A |  | DC_11A_n1A |  | No |  |  |  |
| DC_11A_n3A |  | DC_11A_n3A |  | No |  |  |  |
| DC_11A_n28A |  | DC_11A_n28A |  | No |  |  |  |
| DC_11A_n41A7 |  | DC_11A_n41A |  | No |  |  |  |
| DC_11A_n77A7 |  | DC_11A_n77A |  | No |  | No |  |
| DC_11A_n77(2A)7 DC_11A_n77(3A)7 |  | DC_11A_n77A |  | No |  | No |  |
| DC_11A_n78A7 |  | DC_11A_n78A |  | No |  | No |  |
| DC_11A_n79A7 |  | DC_11A_n79A |  | No |  |  |  |
| DC_12A_n2A |  | DC_12A_n2A |  | No |  |  |  |
| DC_12A_n5A |  | DC_12A_n5A |  | No |  |  |  |
| DC_12A_n7A |  | DC_12A_n7A |  | No |  |  |  |
| DC_12A_n7(2A) |  | DC_12A_n7A |  | No |  |  |  |
| DC_12A_n25A |  | DC_12A_n25A |  | No |  |  |  |
| DC_12A_n30A |  | DC_12A_n30A |  | No |  |  |  |
| DC_12A_n38A |  | DC_12A_n38A |  | No |  |  |  |
| DC_12A_n41A |  | DC_12A_n41A |  | No |  |  |  |
| DC_12A_n66A |  | DC_12A_n66A |  | No |  |  |  |
| DC_12A_n66(2A) |  | DC_12A_n66A |  | No |  |  |  |
| DC_12A_n71A |  | DC_12A_n71A18,19 |  | DC_12_n71 |  |  |  |
| DC_12A_n77A |  | DC_12A_n77A |  | DC_12_n77 |  |  |  |
| DC_12A_n78A |  | DC_12A_n78A |  | DC_12_n78 |  |  |  |
| DC_12A_n78(2A) |  | DC_12A_n78A |  | DC_12_n78 |  |  |  |
| DC_13A_n2A |  | DC_13A_n2A |  | No |  |  |  |
| DC_13A_n5A |  | DC_13A_n5A |  | DC_13_n5 |  |  |  |
| DC_13A_n7A |  | DC_13A_n7A |  | No |  |  |  |
| DC_13A_n7(2A) |  | DC_13A_n7A |  | No |  |  |  |
| DC_13A_n25A |  | DC_13A_n25A |  | No |  |  |  |
| DC_13A_n48A DC_13A_n48B |  | DC_13A_n48A |  | No |  |  |  |
| DC_13A_n66A |  | DC_13A_n66A |  | No |  |  |  |
| DC_13A_n71A |  | DC_13A_n71A |  | No |  |  |  |
| DC_13A_n77A DC_13A_n77C |  | DC_13A_n77A |  | No |  |  |  |
| DC_13A_n78A |  | DC_13A_n78A |  | No |  |  |  |
| DC_13A_n78(2A) |  | DC_13A_n78A |  | No |  |  |  |
| DC_14A_n2A |  | DC_14A_n2A |  | No |  |  |  |
| DC_14A_n5A |  | DC_14A_n5A |  | DC_14_n5 |  |  |  |
| DC_14A_n30A |  | DC_14A_n30A |  | No |  |  |  |
| DC_14A_n66A |  | DC_14A_n66A |  | No |  |  |  |
| DC_14A_n77A |  | DC_14A_n77A |  | No |  |  |  |
| DC_18A_n3A |  | DC_18A_n3A |  | No |  |  |  |
| DC_18A_n28A |  | DC_18A_n28A |  | No |  |  |  |
| DC_18A_n41A16 |  | DC_18A_n41A |  | No |  |  |  |
| DC_18A_n77A7 DC_18A_n77(2A)7 |  | DC_18A_n77A |  | No |  | No |  |
| DC_18A_n78A7 |  | DC_18A_n78A |  | No |  | No |  |
| DC_18A_n78(2A)7 |  | DC_18A_n78A |  | No |  | No |  |
| DC_20A_n91A |  | DC_20A_n91A_ULSUP-TDM |  | N/A |  |  |  |
| DC_20A_n92A |  | DC_20A_n92A_ULSUP-TDM |  | N/A |  |  |  |
| DC_18A_n79A7 |  | DC_18A_n79A |  | No |  |  |  |
| DC_19A_n1A |  | DC_19A_n1A |  | No |  |  |  |
| DC_19A_n77A7 DC_19A_n77C7 |  | DC_19A_n77A |  | No |  |  |  |
| DC_19A_n77(2A)7 |  | DC_19A_n77A |  | No |  |  |  |
| DC_19A_n78A7 DC_19A_n78C7 |  | DC_19A_n78A |  | No |  | No |  |
| DC_19A_n78(2A)7 |  | DC_19A_n78A |  | No |  | No |  |
| DC_19A_n79A7 DC_19A_n79C7 |  | DC_19A_n79A |  | No |  | No |  |
| DC_20A_n1A |  | DC_20A_n1A |  | No |  |  |  |
| DC_20A_n3A |  | DC_20A_n3A |  | No |  |  |  |
| DC_20A_n7A |  | DC_20A_n7A |  | DC_20_n7 |  |  |  |
| DC_20A_n8A |  | DC_20A_n8A |  | DC_20_n8 |  |  |  |
| DC_20A_n28A8, 11,13 |  | DC_20A_n28A |  | No |  |  |  |
| DC_20A_n38A |  | DC_20A_n38A |  | No |  |  |  |
| DC_20A_n41A |  | DC_20A_n41A |  | DC_20_n41 |  |  |  |
| DC_20A_n50A |  | DC_20A_n50A |  | No |  |  |  |
| DC_20A_n51A |  | DC_20A_n51A |  | No |  |  |  |
| DC_20A_n77A7 |  | DC_20A_n77A |  | No |  |  |  |
| DC_20A_n78A7 DC_20A_n78C7 |  | DC_20A_n78A |  | No |  |  |  |
| DC_20A_n78(2A)7 |  | DC_20A_n78A |  | No |  |  |  |
| DC_21A_n1A |  | DC_21A_n1A |  | No |  |  |  |
| DC_21A_n28A17 |  | DC_21A_n28A |  | DC_21_n28 |  |  |  |
| DC_21A_n77A7 DC_21A_n77C7 |  | DC_21A_n77A |  | No |  |  |  |
| DC_21A_n77(2A)7 |  | DC_21A_n77A |  | No |  |  |  |
| DC_21A_n78A7 DC_21A_n78C7 |  | DC_21A_n78A |  | No |  | No |  |
| DC_21A_n78(2A)7 |  | DC_21A_n78A |  | No |  | No |  |
| DC_21A_n79A7 DC_21A_n79C7 |  | DC_21A_n79A |  | No |  | No |  |
| DC_25A_n41A |  | DC_25A_n41A |  | No |  |  |  |
| DC_25A-25A_n41A |  | DC_25A_n41A |  | No |  |  |  |
| DC_25A_n77A |  | DC_25A_n77A |  | DC_25_n77 |  |  |  |
| DC_25A-25A_n77A |  | DC_25A_n77A |  | DC_25_n77 |  |  |  |
| DC_25A_n78A |  | DC_25A_n78A |  | DC_25_n78 |  |  |  |
| DC_25A-25A_n78A |  | DC_25A_n78A |  | DC_25_n78 |  |  |  |
| DC_26A_n25A |  | DC_26A_n25A |  | No |  |  |  |
| DC_26A_n41A |  | DC_26A_n41A |  | No |  |  |  |
| DC_26A_n77A7 |  | DC_26A_n77A |  | No |  |  |  |
| DC_26A_n78A7 |  | DC_26A_n78A |  | No |  |  |  |
| DC_26A_n79A7 |  | DC_26A_n79A |  | No |  |  |  |
| DC_28A_n1A |  | DC_28A_n1A |  | No |  |  |  |
| DC_28A_n2A |  | DC_28A_n2A |  | No |  |  |  |
| DC_28A_n3A |  | DC_28A_n3A |  | No |  |  |  |
| DC_28A_n5A8 |  | DC_28A_n5A |  | No |  |  |  |
| DC_28A_n7A DC_28A_n7B |  | DC_28A_n7A DC_28A_n7B |  | No |  |  |  |
| DC_28A_n51A |  | DC_28A_n51A |  | No |  |  |  |
| DC_28A_n8A |  | DC_28A_n8A |  | No |  |  |  |
| DC_28A_n40A |  | DC_28A_n40A |  | No |  |  |  |
| DC_28A_n41A7 |  | DC_28A_n41A |  | No |  |  |  |
| DC_28A_n50A |  | DC_28A_n50A |  | No |  |  |  |
| DC_28A_n66A |  | DC_28A_n66A |  | No |  |  |  |
| DC_28A_n77A7 DC_28A_n77C7 |  | DC_28A_n77A |  | No |  | No |  |
| DC_28A_n77(2A)7 |  | DC_28A_n77A |  | No |  | No |  |
| DC_28A_n78A7 DC_28A_n78C7 |  | DC_28A_n78A |  | No |  | No |  |
| DC_28A_n78(2A)7 |  | DC_28A_n78A |  | No |  | No |  |
| DC_28A_n79A7 DC_28A_n79C7 |  | DC_28A_n79A |  | No |  |  |  |
| DC_30A_n2A |  | DC_30A_n2A |  | No |  |  |  |
| DC_30A_n5A |  | DC_30A_n5A |  | No |  |  |  |
| DC_30A_n66A |  | DC_30A_n66A |  | No |  |  |  |
| DC_30A_n77A |  | DC_30A_n77A |  | No |  |  |  |
| DC_38A_n1A |  | DC_38A_n1A |  | No |  |  |  |
| DC_38A_n3A |  | DC_38A_n3A |  | No |  |  |  |
| DC_38A_n8A |  | DC_38A_n8A |  | No |  |  |  |
| DC_38A_n28A |  | DC_38A_n28A |  | No |  |  |  |
| DC_38A_n78A7 |  | DC_38A_n78A |  | No |  |  |  |
| DC_38A_n79A DC_38A_n79C |  | DC_38A_n79A |  | No |  |  |  |
| DC_39A_n40A3 |  | DC_39A_n40A |  | No |  |  |  |
| DC_39A_n41A3 DC_39C_n41A3 |  | DC_39A_n41A DC_39C_n41A |  | No |  | No |  |
| DC_39A_n78A5,7 |  | DC_39A_n78A |  | No |  |  |  |
| DC_39A_n79A7 DC_39A_n79C7 |  | DC_39A_n79A |  | No |  | No |  |
| DC_40A_n1A DC_40C_n1A |  | DC_40A_n1A |  | No |  |  |  |
| DC_40A_n41A3 DC_40A_n41C3 DC_40C_n41A3 |  | DC_40A_n41A |  | No |  |  |  |
| DC_40A_n41(2A)3 |  | DC_40A_n41A |  | No |  |  |  |
| DC_40A_n77A |  | DC_40A_n77A |  | No |  |  |  |
| DC_40A_n78A DC_40C_n78A |  | DC_40A_n78A DC_40C_n78A |  | No |  |  |  |
| DC_40A_n78(2A) DC_40C_n78(2A) |  | DC_40A_n78A DC_40C_n78A |  | No |  |  |  |
| DC_40A_n79A7,12 DC_40A_n79C7,12 DC_40C_n79A7,12 |  | DC_40A_n79A |  | No |  | No |  |
| DC_41A_n1A DC_41C_n1A |  | DC_41A_n1A DC_41C_n1A |  | No |  | DC_41A_n1A DC_41C_n1A |  |
| DC_41A_n3A7 DC_41C_n3A7 |  | DC_41A_n3A DC_41C_n3A |  | No |  |  |  |
| DC_41A_n28A7 DC_41C_n28A7 |  | DC_41A_n28A DC_41C_n28A |  | No |  |  |  |
| DC_41A_n77A DC_41C_n77A |  | DC_41A_n77A DC_41C_n77A |  | No |  |  |  |
| DC_41A_n77(2A) DC_41C_n77(2A) |  | DC_41A_n77A DC_41C_n77A |  | No |  |  |  |
| DC_41A_n78A DC_41C_n78A DC_41D_n78A |  | DC_41A_n78A DC_41C_n78A |  | No |  |  |  |
| DC_41A_n78(2A) DC_41C_n78(2A) |  | DC_41A_n78A DC_41C_n78A |  | No |  |  |  |
| DC_41A_n79A6,7 DC_41A_n79C6,7 DC_41C_n79A6,7 |  | DC_41A_n79A DC_41C_n79A |  | No |  | No |  |
| DC_42A_n1A7 DC_42C_n1A7 |  | DC_42A_n1A DC_42C_n1A |  | No |  |  |  |
| DC_42A_n3A7 DC_42C_n3A7 |  | DC_42A_n3A DC_42C_n3A |  | DC_42_n3 |  |  |  |
| DC_42A_n28A7 DC_42C_n28A7 |  | DC_42A_n28A DC_42C_n28A |  | No |  |  |  |
| DC_42A_n51A |  | DC_42A_n51A |  | No |  |  |  |
| DC_42A_n77A3,4,9,11 DC_42A_n77C3,4,9,11 DC_42C_n77A3,4,9,11 DC_42C_n77C3,4,9,11 DC_42D_n77A3,4,9,11 DC_42D_n77C DC_42E_n77A3,4,9,11 DC_42E_n77C |  | N/A |  | N/A |  |  |  |
| DC_42A_n77(2A)3,4,9,11 DC_42C_n77(2A)3,4,9,11 |  | N/A |  | N/A |  |  |  |
| DC_42A_n78A3,4,9,11 DC_42A_n78C3,4,9,11 DC_42C_n78A3,4,9,11 DC_42C_n78C3,4,9,11 DC_42D_n78A3,4,9,11 DC_42D_n78C DC_42E_n78A3,4,9,11 DC_42E_n78C |  | N/A |  | N/A |  |  |  |
| DC_42A_n79A9,15 DC_42A_n79C9,15 DC_42C_n79A9,15 DC_42C_n79C9,15 DC_42D_n79A9,15 DC_42D_n79C9,15 DC_42E_n79A9,15 DC_42E_n79C9,15 |  | N/A |  | N/A |  |  |  |
| DC_46A_n77A2 |  | N/A |  | N/A |  |  |  |
| DC_46A_n78A2 DC_46C_n78A2 DC_46D_n78A2 DC_46E_n78A2 |  | N/A |  | N/A |  |  |  |
| DC_48A_n5A DC_48C_n5A DC_48D_n5A DC_48E_n5A |  | DC_48A_n5A |  | No |  |  |  |
| DC_48A_n12A |  | DC_48A_n12A |  | No |  |  |  |
| DC_48A_n25A DC_48C_n25A DC_48D_n25A |  | DC_48A_n25A |  | No |  |  |  |
| DC_48A_n46A DC_48B_n46A DC_48C_n46A DC_48D_n46A DC_48E_n46A DC_48A_n46B DC_48B_n46B DC_48C_n46B DC_48D_n46B DC_48E_n46B DC_48A_n46C DC_48B_n46C DC_48C_n46C DC_48D_n46C DC_48E_n46C DC_48A_n46D DC_48B_n46D DC_48C_n46D DC_48D_n46D DC_48E_n46D DC_48A_n46E DC_48B_n46E DC_48C_n46E DC_48D_n46E DC_48E_n46E |  | DC_48A_n46A DC_48B_n46A |  | No |  |  |  |
| DC_48A_n66A DC_48C_n66A DC_48D_n66A DC_48E_n66A |  | DC_48A_n66A |  | No |  |  |  |
| DC_48A_n71A DC_48B_n71A DC_48C_n71A DC_48D_n71A |  | DC_48A_n71A |  | No |  |  |  |
| DC_48A-48A_n71A DC_48A-48A-48A_n71A |  | DC_48A_n71A |  | No |  |  |  |
| DC_48A_n77A3. 4. 9, 11 DC_48C_n77A3. 4. 9, 11 DC_48A_n77C3. 4. 9, 11 DC_48C_n77C3. 4. 9, 11 DC_48D_n77A3. 4. 9, 11 DC_48D_n77C3. 4. 9, 11 DC_48E_n77A3. 4. 9, 11 |  | N/A |  | N/A |  |  |  |
| DC_66A_n2A DC_66B_n2A DC_66C_n2A |  | DC_66A_n2A |  | DC_66_n2 |  |  |  |
| DC_66A-66A_n2A |  | DC_66A_n2A |  | DC_66_n2 |  |  |  |
| DC_66A-66A-66A_n2A |  | DC_66A_n2A |  | DC_66_n2 |  |  |  |
| DC_66A_n5A DC_66B_n5A DC_66C_n5A |  | DC_66A_n5A |  | DC_66_n5 |  |  |  |
| DC_66A-66A_n5A |  | DC_66A_n5A |  | DC_66_n5 |  |  |  |
| DC_66A-66A-66A_n5A |  | DC_66A_n5A |  | DC_66_n5 |  |  |  |
| DC_66A_n7A |  | DC_66A_n7A |  | No |  |  |  |
| DC_66A_n7(2A) |  | DC_66A_n7A |  | No |  |  |  |
| DC_66A-66A_n7A |  | DC_66A_n7A |  | No |  |  |  |
| DC_66A-66A_n7(2A) |  | DC_66A_n7A |  | No |  |  |  |
| DC_66A_n12A |  | DC_66A_n12A |  | No |  |  |  |
| DC_66A_n25A |  | DC_66A_n25A |  | DC_66_n25 |  |  |  |
| DC_66A_n28A |  | DC_66A_n28A |  | No |  |  |  |
| DC_66A_n30A |  | DC_66A_n30A |  | No |  |  |  |
| DC_66A_n38A |  | DC_66A_n38A |  | No |  |  |  |
| DC_66A-66A_n30A |  | DC_66A_n38A |  | No |  |  |  |
| DC_66A-66A_n38A |  | DC_66A_n38A |  | No |  |  |  |
| DC_66A_n41A DC_66A_n41C |  | DC_66A_n41A |  | No |  |  |  |
| DC_66A_n41(2A) |  | DC_66A_n41A |  | No |  |  |  |
| DC_66A_n46A |  | DC_66A_n46A |  | No |  |  |  |
| DC_66A_n48A DC_66A_n48B |  | DC_66A_n48A |  | No |  |  |  |
| DC_66A-66A_n48A DC_66A-66A_n48B |  | DC_66A_n48A |  | No |  |  |  |
| DC_66A_n71A DC_66C_n71A DC_66A_n71B |  | DC_66A_n71A |  | No |  |  |  |
| DC_66A-66A_n71A |  | DC_66A_n71A |  | No |  |  |  |
| DC_66A_n77A DC_66A_n77C |  | DC_66A_n77A |  | DC_66_n77 |  |  |  |
| DC_66A-66A_n77A DC_66A-66A_n77C DC_66A-66A-66A_n77C |  | DC_66A_n77A |  | DC_66_n77 |  |  |  |
| DC_66A-66A-66A_n77A |  | DC_66A_n77A |  | DC_66_n77 |  |  |  |
| DC_66A_n78A |  | DC_66A_n78A |  | No |  |  |  |
| DC_66A_n78(2A) |  | DC_66A_n78A |  | No |  |  |  |
| DC_66A-66A_n78A |  | DC_66A_n78A |  | No |  |  |  |
| DC_66A-66A_n78(2A) |  | DC_66A_n78A |  | No |  |  |  |
| DC_71A_n2A |  | DC_71A_n2A |  | No |  |  |  |
| DC_71A_n5A |  | DC_71A_n5A |  | No |  |  |  |
| DC_71A_n38A |  | DC_71A_n38A |  | No |  |  |  |
| DC_71A_n41A |  | DC_71A_n41A |  | No |  |  |  |
| DC_71A_n48A |  | DC_71A_n48A |  | No |  |  |  |
| DC_71A_n66A |  | DC_71A_n66A |  | No |  |  |  |
| DC_71A_n78A |  | DC_71A_n78A |  | No |  |  |  |
| DC_71A_n78(2A) |  | DC_71A_n78A |  | No |  |  |  |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 2: Restricted to E-UTRA operation when inter-band carrier aggregation is configured. The downlink operating band for Band 46 is paired with the uplink operating band (external E-UTRA band) of the carrier aggregation configuration that is supporting the configured Pcell. NOTE 3: The minimum requirements apply only when there is non-simultaneous Tx/Rx operation between E-UTRA and NR carriers. This restriction applies also for these carriers when applicable EN-DC configuration is part of a higher order EN-DC configuration. NOTE 4: The minimum requirements for intra-band non-contiguous EN-DC apply. When UE capability interBandContiguousMRDC is indicated, the minimum requirements for intra-band-contiguous EN-DC also should be met in addtion to intra-band non-contiguous EN-DC. The intra-band requirements also apply for these carriers when applicable EN-DC configuration is a subset of a higher order EN-DC configuration. NOTE 5: The frequency range above 3600 MHz for Band n78 is not used in this combination. NOTE 6: The frequency range below 2506 MHz for Band 41 is not used in this combination. NOTE 7: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability. NOTE 8: The frequency range in band n28 is restricted for this band combination to 703 - 733 MHz for the UL and 758-788 MHz for the DL. NOTE 9: The combination is not used alone as fall back mode of other band combinations in which UL in Band 42 or Band 48 is not used. NOTE 10: Void. NOTE 11: The minimum requirements for inter-band EN-DC apply when the maximum power spectral density imbalance between downlink carriers is within 6 dB. The power spectral density imbalance condition also applies for these carriers when applicable EN-DC configuration is a subset of a higher order EN-DC configuration. NOTE 12: Applicable for frequency range above 4800 MHz for Band n79 in this combination. NOTE 13: The minimum requirements apply for synchronized DL carriers with a maximum receive time difference ≤ 3 usec. The requirements also apply for these carriers when applicable EN-DC configuration is a subset of a higher order EN-DC configuration. NOTE 14: Applicable when dynamic switching between two uplink carriers is conducted. The DL interruption requirements for NR DL carrier(s) and E-UTRA DL carrier(s) are specified in clause 8.2.1.2.14 of 38.133 [15] and clause 7.32.2.12 of 36.133 [16] respectively. NOTE 15: Simultaneous Rx/Tx capability does not apply for UEs supporting band 42 with a n77 implementation only. Same restrictions are applied to related higher order configurations. NOTE 16: The frequency range in band n41 is restricted for this band combination to 2595 – 2645 MHz. NOTE 17: The frequency range in band n28 is restricted for this band combination to 728 - 738 MHz for the UL and 783 - 793 MHz for the DL. This restriction applies also for these band combinations when applicable EN-DC configuration is part of a higher order EN-DC configuration. NOTE 18: Only single switched UL is supported. NOTE 19: The implementation with 4 antennas is targeted for FWA form factor for this band combination. NOTE 20: The combination is not used alone as fallback mode of other band combinations in which UL in Band 2 is not used. |  |  |  |  |  |  |  |

<< Next changes >>

5.5B.4a	Inter-band NE-DC within FR1

5.5B.4a.1	Inter-band NE-DC configurations within FR1 (two bands)

Table 5.5B.4a.1-1: Inter-band NE-DC configurations within FR1 (two bands)

| NE-DC configuration | Uplink NE-DC configuration (NOTE 1) | Single UL allowed |
| --- | --- | --- |
| DC_n1A_28A | DC_n1A_28A | No |
| DC_n3A_1A | DC_n3A_1A | DC_n3_1 |
| DC_n3A_8A | DC_n3A_8A | No |
| DC_n8A_1A | DC_n8A_1A | No |
| DC_n8A_3A | DC_n8A_3A | No |
| DC_n28A_3A DC_n28A_3C | DC_n28A_3A | No |
| DC_n28A_8A DC_n28A_8B | DC_n28A_8A | No |
| DC_n28A_20A3 | DC_n28A_20A | No |
| DC_n28A_34A | DC_n28A_34A | No |
| DC_n28A_39A DC_n28A_39C | DC_n28A_39A | No |
| DC_n28A_40A DC_n28A_40C | DC_n28A_40A | No |
| DC_n41A_3A DC_n41A_3C DC_n41C_3A | DC_n41A_3A | No |
| DC_n41A_8A DC_n41A_8B DC_n41C_8A | DC_n41A_8A | No |
| DC_n41A_34A2 | DC_n41A_34A | No |
| DC_n41A_39A2 DC_n41A_39C2 DC_n41C_39A2 | DC_n41A_39A | No |
| DC_n41A_40A2 DC_n41A_40C2 DC_n41C_40A2 | DC_n41A_40A | No |
| DC_n77A_1A DC_n77(2A)_1A | DC_n77A_1A | DC_n77_1 |
| DC_n77A_3A DC_n77(2A)_3A | DC_n77A_3A | DC_n77_3 |
| DC_n77A_8A DC_n77(2A)_8A | DC_n77A_8A | No |
| DC_n78A_1A | DC_n78A_1A | No |
| DC_n78A_3A | DC_n78A_3A | No |
| DC_n78A_3C | DC_n78A_3A | No |
| DC_n78A_5A | DC_n78A_5A | No |
| DC_n78A_7A | DC_n78A_7A | No |
| DC_n78A_7A-7A | DC_n78A_7A | No |
| DC_n78A_8A | DC_n78A_8A | No |
| DC_n78A_26A | DC_n78A_26A | No |
| NOTE 1: Uplink NE-DC configurations are the configurations supported by the present release of specifications. NOTE 2: The minimum requirements apply only when there is non-simultaneous Tx/Rx operation between E-UTRA and NR carriers. This restriction applies also for these carriers when applicable EN-DC configuration is part of a higher order EN-DC configuration. NOTE 3: The frequency range in band n28 is restricted for this band combination to 703 - 733 MHz for the UL and 758-788 MHz for the DL. |  |  |

<< Next changes >>

6.2B.1.3	Inter-band EN-DC within FR1

For inter-band EN-DC of E-UTRA and NR in FR1, the following UE Power Classes define the maximum output power for any transmission bandwidth within the aggregated channel bandwidth. The maximum output power is measured as the sum of the maximum output power at each UE antenna connector. The period of measurement shall be at least one sub frame (1ms). UE maximum output power shall be measured over all component carriers from different bands. If each band has separate antenna connectors, maximum output power is measured as the sum of maximum output power at each UE antenna connector.

Table 6.2B.1.3-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC configuration | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| DC_1A_n3A |  |  | 23 | +2/-3 |
| DC_1A_n5A |  |  | 23 | +2/-3 |
| DC_1A_n7A |  |  | 23 | +2/-3 |
| DC_1A_n8A |  |  | 23 | +2/-3 |
| DC_1A_n20A |  |  | 23 | +2/-3 |
| DC_1A_n28A |  |  | 23 | +2/-3 |
| DC_1A_n38A |  |  | 23 | +2/-3 |
| DC_1A_n40A |  |  | 23 | +2/-3 |
| DC_1A_n41A |  |  | 23 | +2/-3 |
| DC_1A_n50A |  |  | 23 | +2/-3 |
| DC_1A_n51A |  |  | 23 | +2/-3 |
| DC_1A_n71A |  |  | 23 | +2/-3 |
| DC_1A_n77A DC_1A_n84A_ULSUP-TDM_n77A |  |  | 23 | +2/-3 |
| DC_1A_n78A DC_1A_n84A_ULSUP-TDM_n78A | 266 | +2/-3 | 23 | +2/-3 |
| DC_1A_n79A DC_1A_n84A_ULSUP-TDM_n79A |  |  | 23 | +2/-3 |
| DC_1A_n80A |  |  | 23 | +2/-3 |
| DC_2A_n5A |  |  | 23 | +2/-3 |
| DC_2A_n7A |  |  | 23 | +2/-3 |
| DC_2A_n12A |  |  | 23 | +2/-3 |
| DC_2A_n25A7 |  |  | 23 | +2/-3 |
| DC_2A_n28A |  |  | 23 | +2/-3 |
| DC_2A_n30A |  |  | 23 | +2/-3 |
| DC_2A_n38A |  |  | 23 | +2/-3 |
| DC_2A_n41A | 266 | +2/-3 | 23 | +2/-3 |
| DC_2A_n46A |  |  | 23 | +2/-3 |
| DC_2A_n48A |  |  | 23 | +2/-3 |
| DC_2A_n66A |  |  | 23 | +2/-3 |
| DC_2A_n71A |  |  | 23 | +2/-3 |
| DC_2A_n77A | 266 | +2/-3 | 23 | +2/-3 |
| DC_2A_n78A |  |  | 23 | +2/-3 |
| DC_3A_n1A |  |  | 23 | +2/-3 |
| DC_3A_n5A DC_3C_n5A |  |  | 23 | +2/-3 |
| DC_3A_n7A |  |  | 23 | +2/-3 |
| DC_3A_n8A |  |  | 23 | +2/-3 |
| DC_3A_n20A |  |  | 23 | +2/-3 |
| DC_3A_n28A |  |  | 23 | +2/-3 |
| DC_3A_n34A |  |  | 23 | +2/-3 |
| DC_3A_n38A |  |  | 23 | +2/-3 |
| DC_3A_n40A |  |  | 23 | +2/-3 |
| DC_3A_n41A, DC_3C_n41A, DC_3C_n41A, | 266 | +2/-3 | 23 | +2/-3 |
| DC_3A_n50A |  |  | 23 | +2/-3 |
| DC_3A_n51A |  |  | 23 | +2/-3 |
| DC_3A_n71A |  |  | 23 | +2/-3 |
| DC_3A_n77A DC_3C_n77A |  |  | 23 | +2/-3 |
| DC_3A_n78A DC_3C_n78A | 266 | +2/-3 | 23 | +2/-3 |
| DC_3A_n79A DC_3C_n79A |  |  | 23 | +2/-3 |
| DC_3A_n80A_ULSUP-TDM_n41 DC_3C_n80A_ULSUP-TDM_n41 |  |  | 23 | +2/-3 |
| DC_3A_n80A_ULSUP-TDM_n77A |  |  | 23 | +2/-3 |
| DC_3A_n80A_ULSUP-TDM_n78A |  |  | 23 | +2/-3 |
| DC_3A_n80A_ULSUP-TDM_n79A |  |  | 23 | +2/-3 |
| DC_3A_n82A |  |  | 23 | +2/-3 |
| DC_3A_n84A |  |  | 23 | +2/-3 |
| DC_4A_n2A |  |  | 23 | +2/-3 |
| DC_4A_n5A |  |  | 23 | +2/-3 |
| DC_4A_n7A |  |  | 23 | +2/-3 |
| DC_4A_n28A |  |  | 23 | +2/-3 |
| DC_4A_n38A |  |  | 23 | +2/-3 |
| DC_4A_n41A |  |  | 23 | +2/-3 |
| DC_4A_n78A |  |  | 23 | +2/-3 |
| DC_5A_n2A |  |  | 23 | +2/-3 |
| DC_5A_n7A |  |  | 23 | +2/-3 |
| DC_5A_n12A |  |  | 23 | +2/-3 |
| DC_5A_n30A |  |  | 23 | +2/-3 |
| DC_5A_n38A |  |  | 23 | +2/-3 |
| DC_5A_n40A |  |  | 23 | +2/-3 |
| DC_5A_n48A |  |  | 23 | +2/-3 |
| DC_5A_n66A |  |  | 23 | +2/-3 |
| DC_5A_n71A |  |  | 23 | +2/-3 |
| DC_5A_n77A | 266 | +2/-3 | 23 | +2/-3 |
| DC_5A_n78A | 266 | +2/-3 | 23 | +2/-3 |
| DC_5A_n79A |  |  | 23 | +2/-3 |
| DC_7A_n1A |  |  | 23 | +2/-3 |
| DC_7A_n2A |  |  | 23 | +2/-3 |
| DC_7A_n3A |  |  | 23 | +2/-3 |
| DC_7A_n5A DC_7C_n5A |  |  | 23 | +2/-3 |
| DC_7A_n8A |  |  | 23 | +2/-3 |
| DC_7A_n20A |  |  | 23 | +2/-3 |
| DC_7A_n25A |  |  | 23 | +2/-3 |
| DC_7A_n28A |  |  | 23 | +2/-3 |
| DC_7A_n40A |  |  | 23 | +2/-3 |
| DC_7A_n51A |  |  | 23 | +2/-3 |
| DC_7A_n66A |  |  | 23 | +2/-3 |
| DC_7A_n71A |  |  | 23 | +2/-3 |
| DC_7A_n77A |  |  | 23 | +2/-3 |
| DC_7A_n78A DC_7C_n78A | 266 | +2/-3 | 23 | +2/-3 |
| DC_7A_n79A |  |  | 23 | +2/-3 |
| DC_7A_n80A |  |  | 23 | +2/-3 |
| DC_8A_n1A |  |  | 23 | +2/-3 |
| DC_8A_n2A |  |  | 23 | +2/-3 |
| DC_8A_n3A |  |  | 23 | +2/-3 |
| DC_8A_n7A |  |  | 23 | +2/-3 |
| DC_8A_n20A |  |  | 23 | +2/-3 |
| DC_8A_n28A |  |  | 23 | +2/-3 |
| DC_8A_n34A |  |  | 23 | +2/-3 |
| DC_8A_n39A |  |  | 23 | +2/-3 |
| DC_8A_n40A |  |  | 23 | +2/-3 |
| DC_8A_n41A, |  |  | 23 | +2/-3 |
| DC_8A_n77A |  |  | 23 | +2/-3 |
| DC_8A_n78A | 266 | +2/-3 | 23 | +2/-3 |
| DC_8A_n79A DC_8A_n79C |  |  | 23 | +2/-3 |
| DC_8A_n80A |  |  | 23 | +2/-3 |
| DC_8A_n81A_ULSUP-TDM_n41 |  |  | 23 | +2/-3 |
| DC_8A_n81A_ULSUP-TDM_n78A |  |  | 23 | +2/-3 |
| DC_8A_n81A_ULSUP-TDM_n79A |  |  | 23 | +2/-3 |
| DC_11A_n1A |  |  | 23 | +2/-3 |
| DC_11A_n3A |  |  | 23 | +2/-3 |
| DC_11A_n28A |  |  | 23 | +2/-3 |
| DC_11A_n41A |  |  | 23 | +2/-3 |
| DC_11A_n77A |  |  | 23 | +2/-3 |
| DC_11A_n78A |  |  | 23 | +2/-3 |
| DC_11A_n79A |  |  | 23 | +2/-3 |
| DC_12A_n2A |  |  | 23 | +2/-3 |
| DC_12A_n5A |  |  | 23 | +2/-3 |
| DC_12A_n7A |  |  | 23 | +2/-3 |
| DC_12A_n25A |  |  | 23 | +2/-3 |
| DC_12A_n30A |  |  | 23 | +2/-3 |
| DC_12A_n38A |  |  | 23 | +2/-3 |
| DC_12A_n41A |  |  | 23 | +2/-3 |
| DC_12A_n66A |  |  | 23 | +2/-3 |
| DC_12A_n71A7 |  |  | 23 | +2/-3 |
| DC_12A_n77A | 266 | +2/-3 | 23 | +2/-3 |
| DC_12A_n78A |  |  | 23 | +2/-3 |
| DC_13A_n2A |  |  | 23 | +2/-3 |
| DC_13A_n5A |  |  | 23 | +2/-3 |
| DC_13A_n7A |  |  | 23 | +2/-3 |
| DC_13A_n25A |  |  | 23 | +2/-3 |
| DC_13A_n48A |  |  | 23 | +2/-3 |
| DC_13A_n66A |  |  | 23 | +2/-3 |
| DC_13A_n71A |  |  | 23 | +2/-3 |
| DC_13A_n77A | 266 | +2/-3 | 23 | +2/-3 |
| DC_13A_n78A |  |  | 23 | +2/-3 |
| DC_14A_n2A |  |  | 23 | +2/-3 |
| DC_14A_n5A |  |  | 23 | +2/-3 |
| DC_14A_n30A |  |  | 23 | +2/-3 |
| DC_14A_n66A |  |  | 23 | +2/-3 |
| DC_14A_n77A | 266 | +2/-3 | 23 | +2/-3 |
| DC_18A_n3A |  |  | 23 | +2/-3 |
| DC_18A_n28A |  |  | 23 | +2/-3 |
| DC_18A_n41A |  |  | 23 | +2/-3 |
| DC_18A_n77A |  |  | 23 | +2/-3 |
| DC_18A_n78A |  |  | 23 | +2/-3 |
| DC_18A_n79A |  |  | 23 | +2/-3 |
| DC_19A_n1A |  |  | 23 | +2/-3 |
| DC_19A_n77A |  |  | 23 | +2/-3 |
| DC_19A_n78A |  |  | 23 | +2/-3 |
| DC_19A_n79A |  |  | 23 | +2/-3 |
| DC_20A_n1A |  |  | 23 | +2/-3 |
| DC_20A_n3A |  |  | 23 | +2/-3 |
| DC_20A_n7A |  |  | 23 | +2/-3 |
| DC_20A_n8A |  |  | 23 | +2/-3 |
| DC_20A_n38A |  |  | 23 | +2/-3 |
| DC_20A_n28A |  |  | 23 | +2/-3 |
| DC_20A_n41A |  |  | 23 | +2/-3 |
| DC_20A_n50A |  |  | 23 | +2/-3 |
| DC_20A_n51A |  |  | 23 | +2/-3 |
| DC_20A_n77A |  |  | 23 | +2/-3 |
| DC_20A_n80A |  |  | 23 | +2/-3 |
| DC_20A_n78A |  |  | 23 | +2/-3 |
| DC_20A_n82A_ULSUP-TDM_n78A |  |  | 23 | +2/-3 |
| DC_20A_n83A |  |  | 23 | +2/-3 |
| DC_21A_n1A |  |  | 23 | +2/-3 |
| DC_21A_n28A |  |  | 23 | +2/-3 |
| DC_21A_n77A |  |  | 23 | +2/-3 |
| DC_21A_n78A |  |  | 23 | +2/-3 |
| DC_21A_n79A |  |  | 23 | +2/-3 |
| DC_25A_n41A |  |  | 23 | +2/-3 |
| DC_25A_n77A |  |  | 23 | +2/-3 |
| DC_25A_n78A |  |  | 23 | +2/-3 |
| DC_26A_n25A |  |  | 23 | +2/-3 |
| DC_26A_n41A |  |  | 23 | +2/-3 |
| DC_26A_n77A |  |  | 23 | +2/-3 |
| DC_26A_n78A |  |  | 23 | +2/-3 |
| DC_26A_n79A |  |  | 23 | +2/-3 |
| DC_28A_n1A |  |  | 23 | +2/-3 |
| DC_28A_n2A |  |  | 23 | +2/-3 |
| DC_28A_n3A |  |  | 23 | +2/-3 |
| DC_28A_n5A |  |  | 23 | +2/-3 |
| DC_28A_n7A DC_28A_n7B |  |  | 23 | +2/-3 |
| DC_28A_n8A |  |  | 23 | +2/-3 |
| DC_28A_n40A |  |  | 23 | +2/-3 |
| DC_28A_n41A |  |  | 23 | +2/-3 |
| DC_28A_n50A |  |  | 23 | +2/-3 |
| DC_28A_n51A |  |  | 23 | +2/-3 |
| DC_28A_n66A |  |  | 23 | +2/-3 |
| DC_28A_n77A |  |  | 23 | +2/-3 |
| DC_28A_n78A |  |  | 23 | +2/-3 |
| DC_28A_n79A |  |  | 23 | +2/-3 |
| DC_28A_n83A_ULSUP-TDM_n41A |  |  | 23 | +2/-3 |
| DC_28A_n83A_ULSUP-TDM_n78A |  |  | 23 | +2/-3 |
| DC_30A_n2A |  |  | 23 | +2/-3 |
| DC_30A_n5A |  |  | 23 | +2/-3 |
| DC_30A_n66A |  |  | 23 | +2/-3 |
| DC_30A_n77A | 266 | +2/-3 | 23 | +2/-3 |
| DC_38A_n1A |  |  | 23 | +2/-3 |
| DC_38A_n3A |  |  | 23 | +2/-3 |
| DC_38A_n8A |  |  | 23 | +2/-3 |
| DC_38A_n28A |  |  | 23 | +2/-3 |
| DC_38A_n78A |  |  | N/A | N/A |
| DC_38A_n79A |  |  | 23 | +2/-3 |
| DC_39A_n40A |  |  | 23 | +2/-3 |
| DC_39A_n41A DC_39C_n41A | 265 | +2/-3 | 23 | +2/-3 |
| DC_39A_n78A |  |  | 23 | +2/-3 |
| DC_39A_n79A | 265 | +2/-3 | 23 | +2/-3 |
| DC_40A_n1A |  |  | 23 | +2/-3 |
| DC_40A_n41A DC_40C_n41A |  |  | 23 | +2/-3 |
| DC_40A_n77A |  |  | N/A | N/A |
| DC_40A_n78A |  |  | 23 | +2/-3 |
| DC_40C_n78A |  |  | 23 | +2/-3 |
| DC_40A_n79A |  |  | 23 | +2/-3 |
| DC_41A_n1A DC_41C_n1A |  |  | 23 | +2/-3 |
| DC_41A_n3A DC_41C_n3A |  |  | 23 | +2/-3 |
| DC_41A_n28A DC_41C_n28A |  |  | 23 | +2/-3 |
| DC_41A_n77A DC_41C_n77A |  |  | 23 | +2/-3 |
| DC_41A_n78A DC_41C_n78A |  |  | 23 | +2/-3 |
| DC_41A_n79A DC_41C_n79A | 265 | +2/-3 | 23 | +2/-3 |
| DC_42A_n1A DC_42C_n1A |  |  | 23 | +2/-3 |
| DC_42A_n3A DC_42C_n3A |  |  | 23 | +2/-3 |
| DC_42A_n28A DC_42C_n28A |  |  | 23 | +2/-3 |
| DC_42A_n51A |  |  | 23 | +2/-3 |
| DC_42A_n77A |  |  | N/A | N/A |
| DC_42A_n78A |  |  | N/A | N/A |
| DC_42A_n79A |  |  | N/A | N/A |
| DC_48A_n5A |  |  | 23 | +2/-3 |
| DC_48A_n12A |  |  | 23 | +2/-3 |
| DC_48A_n25A |  |  | 23 | +2/-3 |
| DC_2A_n46A |  |  | 23 | +2/-3 |
| DC_48A_n66A |  |  | 23 | +2/-3 |
| DC_48A_n71A |  |  | 23 | +2/-3 |
| DC_66A_n2A |  |  | 23 | +2/-3 |
| DC_66A_n5A |  |  | 23 | +2/-3 |
| DC_66A_n7A |  |  | 23 | +2/-3 |
| DC_66A_n12A |  |  | 23 | +2/-3 |
| DC_66A_n25A |  |  | 23 | +2/-3 |
| DC_66A_n28A |  |  | 23 | +2/-3 |
| DC_66A_n30A |  |  | 23 | +2/-3 |
| DC_66A_n38A |  |  | 23 | +2/-3 |
| DC_66A_n41A | 266 | +2/-3 | 23 | +2/-3 |
| DC_66A_n46A |  |  | 23 | +2/-3 |
| DC_66A_n48A |  |  | 23 | +2/-3 |
| DC_66A_n71A |  |  | 23 | +2/-3 |
| DC_66A_n77A | 266 | +2/-3 | 23 | +2/-3 |
| DC_66A_n78A DC_66A-66A_n78A |  |  | 23 | +2/-3 |
| DC_66A_n86A_ULSUP-TDM_n78A |  |  | 23 | +2/-3 |
| DC_71A_n2A |  |  | 23 | +2/-3 |
| DC_71A_n5A |  |  | 23 | +2/-3 |
| DC_71A_n38A |  |  | 23 | +2/-3 |
| DC_71A_n41A |  |  | 23 | +2/-3 |
| DC_71A_n48A |  |  | 23 | +2/-3 |
| DC_71A_n66A |  |  | 23 | +2/-3 |
| DC_71A_n78A |  |  | 23 | +2/-3 |
| NOTE 1: An uplink DC configuration in which at least one of the bands has NOTE 3 in Table 6.2.1-1 in TS 38.101-1 or NOTE 2 in Table 6.2.2-1 in TS 36.101 is allowed to reduce the lower tolerance limit by 1.5 dB when the transmission bandwidths of at least one of the bands is confined within FUL_low and FUL_low + 4 MHz or FUL_high - 4 MHz and FUL_high. NOTE 2: PPowerClass, EN-DC is the maximum UE power specified without taking into account the tolerance NOTE 3: For inter-band EN-DC the maximum power requirement should apply to the total transmitted power over all component carriers (per UE). NOTE 4: Power Class 3 is the default power class unless otherwise stated. NOTE 5: The UE is not required to support PC2 within each individual cell group. Power class support within each individual cell group is signaled separately by the UE. NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signaled separately by the UE. NOTE 7: Only single switched UL is supported. |  |  |  |  |

If a UE supports a different power class than the default UE power class for an E-UTRA TDD and NR TDD Inter-band EN-DC band combination and the supported power class enables higher maximum output power than that of the default power class:

–	if the field of UE capability maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is absent and the percentage of NR uplink symbols transmitted in a certain evaluation period is larger than 30% (The exact evaluation period is no less than one radio frame); or

–	if the field of UE capability maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is not absent and the percentage of NR uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 as defined in TS38.331 (The exact evaluation period is no less than one radio frame); or

–	if the IE p-maxUE-FR1 as defined in TS 38.331 is provided and set to the maximum output power of the default power class or lower;

–	shall apply all requirements for the default power class to the supported power class and set the configured transmitted power as specified sub-clause 6.2B.4;

–	Else if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the default power class and the percentage of NR uplink symbols transmitted in a certain evaluation period is less than or equal to maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 as defined in TS 38.331; or

–	if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the default power class and the percentage of NR uplink symbols transmitted in a certain evaluation period is less than or equal to 30% when maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is absent. (The exact evaluation period is no less than one radio frame):

–	shall apply all requirements for the supported power class and set the configured transmitted power class as specified in sub-clause 6.2B.4.

If a UE supports a different power class than the default UE power class for an E-UTRA FDD and NR TDD EN-DC band combination and the supported power class enables higher maximum output power than that of the default power class:

If UE indicating the two capabilities maxUplinkDutyCycle-FDD-TDD-EN-DC1 and maxUplinkDutyCycle-FDD-TDD-EN-DC2:

–	if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the default power class, and the percentage of EUTRA uplink symbols transmitted in a certain evaluation period is between 40% and 70%, and the percentage of NR uplink symbols transmitted in a certain evaluation period is less than or equal tomaxUplinkDutyCycle-FDD-TDD-EN-DC1as defined in TS 38.331 (The exact evaluation period is no less than one radio frame); or

–	if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the default power class, and the percentage of EUTRA uplink symbols transmitted in a certain evaluation period is no larger than 40%, and the percentage of NR uplink symbols transmitted in a certain evaluation period is less than or equal to maxUplinkDutyCycle-FDD-TDD-EN-DC2 as defined in TS 38.331 (The exact evaluation period is no less than one radio frame)

–	shall apply all requirements for the supported power class and set the configured transmitted power class as specified in sub-clause 6.2B.4.

–	else

–	shall apply all requirements for the default power class and set the configured transmitted power as specified sub-clause 6.2B.4;

else

–	shall apply all requirements for the supported power class and set the configured transmitted power as specified sub-clause 6.2B.4;

6.2B.1.3a	Inter-band NE-DC within FR1

For inter-band NE-DC of E-UTRA and NR in FR1, the following UE power classes define the maximum output power for any transmission bandwidth within the aggregated channel bandwidth. The maximum output power is measured as the sum of the maximum output power at each UE antenna connector. The period of measurement shall be at least one sub frame (1 ms). UE maximum output power shall be measured over all component carriers from different bands. If each band has separate antenna connectors, maximum output power is measured as the sum of maximum output power at each UE antenna connector.

Table 6.2B.1.3a-1: Maximum output power for inter-band NE-DC (two bands)

| NE-DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_n1A_28A | 23 | +2/-3 |
| DC_n3A_1A | 23 | +2/-3 |
| DC_n3A_8A | 23 | +2/-3 |
| DC_n8A_1A | 23 | +2/-3 |
| DC_n8A_3A | 23 | +2/-3 |
| DC_n28A_3A DC_n28A_3C | 23 | +2/-3 |
| DC_n28A_8A | 23 | +2/-3 |
| DC_n28A_20A | 23 | +2/-3 |
| DC_n28A_34A | 23 | +2/-3 |
| DC_n28A_39A | 23 | +2/-3 |
| DC_n28A_40A DC_n28A_40C | 23 | +2/-3 |
| DC_n41A_3A | 23 | +2/-3 |
| DC_n41A_8A | 23 | +2/-3 |
| DC_n41A_34A | 23 | +2/-3 |
| DC_n41A_39A | 23 | +2/-3 |
| DC_n41A_40A | 23 | +2/-3 |
| DC_n77A_1A | 23 | +2/-3 |
| DC_n77A_3A | 23 | +2/-3 |
| DC_n77A_8A | 23 | +2/-3 |
| DC_n78A_1A | 23 | +2/-3 |
| DC_n78A_3A | 23 | +2/-3 |
| DC_n78A_5A | 23 | +2/-3 |
| DC_n78A_7A | 23 | +2/-3 |
| DC_n78A_8A | 23 | +2/-3 |
| DC_n78A_26A | 23 | +2/-3 |

<< Next changes >>

6.2B.4.2.3.1	ΔTIB,c for EN-DC two bands

Table 6.2B.4.2.3.1-1: ΔTIB,c due to EN-DC(two bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_1_n3 | 1 | 0.3 |
|  | n3 | 0.3 |
| DC_1_n5 | 1 | 0.3 |
|  | n5 | 0.3 |
| DC_1_n7, | 1 | 0.5 |
| DC_1-1_n7 | n7 | 0.6 |
| DC_1_n8 | 1 | 0.3 |
|  | n8 | 0.3 |
| DC_1_n20 | 1 | 0.3 |
|  | n20 | 0.3 |
| DC_1_n28 | 1 | 0.3 |
|  | n28 | 0.6 |
| DC_1_n38 | 1 | 0.5 |
|  | n38 | 0.5 |
| DC_1_n40 | 1 | 0.5 |
|  | n40 | 0.5 |
| DC_1_n41 | 1 | 0.5 |
|  | n41 | 0.5 |
| DC_1_n50 | 1 | 0.5 |
|  | n50 | 0.5 |
| DC_1_n51 | 1 | 0.6 |
|  | n51 | 0.6 |
| DC_1_n71 | 1 | 0.3 |
|  | n71 | 0.3 |
| DC_1_n77 | 1 | 0.6 |
|  | n77 | 0.8 |
| DC_1_n78 | 1 | 0.3 |
|  | n78 | 0.8 |
| DC_2_n5 | 2 | 0.3 |
| DC_2-2_n5 | n5 | 0.3 |
| DC_2_n7 | 2 | 0.5 |
|  | n7 | 0.5 |
| DC_2_n12 | 2 | 0.3 |
|  | n12 | 0.3 |
| DC_2_n28 | 2 | 0.3 |
|  | n28 | 0.3 |
| DC_2_n30 | 2 | 0.5 |
| DC_2-2_n30 | n30 | 0.3 |
| DC_2_n38, | 2 | 0.5 |
| DC_2-2_n38 | n38 | 0.9 |
| DC_2_n41, | 2 | 0.5 |
| DC_2-2_n41 | n41 | 0.41 |
|  |  | 0.92 |
| DC_2_n48 | 2 | 0.6 |
|  | n48 | 0.8 |
| DC_2_n66 | 2 | 0.5 |
| DC_2-2_n66 | n66 | 0.5 |
| DC_2_n71, | 2 | 0.3 |
| DC_2-2_n71 | n71 | 0.3 |
| DC_2_n77 DC_2-2_n77 | 2 | 0.6 |
|  | n77 | 0.8 |
| DC_2_n78, | 2 | 0.6 |
| DC_2-2_n78 | n78 | 0.8 |
| DC_3_n1 | 3 | 0.3 |
|  | n1 | 0.3 |
| DC_3_n5 | 3 | 0.3 |
|  | n5 | 0.3 |
| DC_3_n8, | 3 | 0.3 |
| DC_3-3_n8 | n8 | 0.3 |
| DC_3_n7, | 3 | 0.5 |
| DC_3-3_n7 | n7 | 0.5 |
| DC_3_n20 | 3 | 0.3 |
|  | n20 | 0.3 |
| DC_3_n28 | 3 | 0.3 |
|  | n28 | 0.3 |
| DC_3_n34 | 3 | 0.5 |
|  | n34 | 0.5 |
| DC_3_n38 | 3 | 0.5 |
|  | n38 | 0.5 |
| DC_3_n40 | 3 | 0.5 |
|  | n40 | 0.5 |
| DC_3-n41 | 3 | 0.5 |
|  | n41 | 0.33 |
|  |  | 0.84 |
| DC_3_n50 | 3 | 0.5 |
|  | n50 | 0.5 |
| DC_3_n51 | 3 | 0.3 |
|  | n51 | 0.3 |
| DC_3_n71 | 3 | 0.3 |
|  | n71 | 0.3 |
| DC_3_n77, DC_3-3_n77 | 3 | 0.6 |
|  | n77 | 0.8 |
| DC_3_n78, DC_3-3_n78 | 3 | 0.6 |
|  | n78 | 0.8 |
| DC_4_n2 | 4 | 0.5 |
|  | n2 | 0.5 |
| DC_4_n5 | 4 | 0.3 |
|  | n5 | 0.3 |
| DC_4_n7 | 4 | 0.5 |
|  | n7 | 0.5 |
| DC_4_n28 | 4 | 0.3 |
|  | n28 | 0.6 |
| DC_4_n38 | 4 | 0.5 |
|  | n38 | 0.8 |
| DC_4_n41 | 4 | 0.5 |
|  | n41 | 0.81 |
|  |  | 1.32 |
| DC_4_n78 | 4 | 0.6 |
|  | n78 | 0.8 |
| DC_5_n1 | 5 | 0.3 |
|  | n1 | 0.3 |
| DC_5_n2, DC_5-5_n2 | 5 | 0.3 |
|  | n2 | 0.3 |
| DC_5_n3 | 5 | 0.3 |
|  | n3 | 0.3 |
| DC_5_n7 | 5 | 0.3 |
|  | n7 | 0.3 |
| DC_5_n12 | 5 | 0.8 |
|  | n12 | 0.4 |
| DC_5_n30 | 5 | 0.3 |
|  | n30 | 0.3 |
| DC_5_n38 | 5 | 0.3 |
|  | n38 | 0.3 |
| DC_5_n40 | 5 | 0.3 |
|  | n40 | 0.3 |
| DC_5_n48 | 5 | 0.3 |
|  | n48 | 0.3 |
| DC_5_n66, DC_5-5_n66 | 5 | 0.3 |
|  | n66 | 0.3 |
| DC_5_n71 | 5 | 0.5 |
|  | n71 | 0.5 |
| DC_5_n77 | 5 | 0.6 |
|  | n77 | 0.8 |
| DC_5_n78 | 5 | 0.6 |
|  | n78 | 0.8 |
| DC_7_n1, DC_7-7_n1 | 7 | 0.6 |
|  | n1 | 0.5 |
| DC_7_n2 | 7 | 0.5 |
|  | n2 | 0.5 |
| DC_7_n3 | 7 | 0.5 |
|  | n3 | 0.5 |
| DC_7_n5, | 7 | 0.3 |
| DC_7-7_n5 | n5 | 0.3 |
| DC_7_n8, | 7 | 0.3 |
| DC_7-7_n8 | n8 | 0.6 |
| DC_7_n20 | 7 | 0.3 |
|  | n20 | 0.3 |
| DC_7_n25 | 7 | 0.5 |
|  | n25 | 0.5 |
| DC_7_n28 | 7 | 0.3 |
|  | n28 | 0.3 |
| DC_7_n40 | 7 | 0.5 |
|  | n40 | 0.6 |
| DC_7_n51 | 7 | 0.3 |
|  | n51 | 0.3 |
| DC_7_n71 | 7 | 0.3 |
|  | n71 | 0.6 |
| DC_7_n66, DC_7-7_n66 | 7 | 0.5 |
|  | n66 | 0.5 |
| DC_7_n77, DC_7-7_n77 | 7 | 0.5 |
|  | n77 | 0.8 |
| DC_7_n78, DC_7-7_n78 | 7 | 0.5 |
|  | n78 | 0.8 |
| DC_7_n79 | 7 | 0.5 |
|  | n79 | 0.8 |
| DC_8_n1 | 8 | 0.3 |
|  | n1 | 0.3 |
| DC_8_n2 | 8 | 0.3 |
|  | n2 | 0.3 |
| DC_8_n3 | 8 | 0.3 |
|  | n3 | 0.3 |
| DC_8_n7 | 8 | 0.6 |
|  | n7 | 0.3 |
| DC_8_n20 | 8 | 0.4 |
|  | n20 | 0.4 |
| DC_8_n28 | 8 | 0.6 |
|  | n28 | 0.5 |
| DC_8_n34 | 8 | 0.3 |
|  | n34 | 0.3 |
| DC_8_n39 | 8 | 0.3 |
|  | n39 | 0.3 |
| DC_8_n40 | 8 | 0.3 |
|  | n40 | 0.3 |
| DC_8_n41 | 8 | 0.3 |
|  | n41 | 0.3 |
| DC_8_n77 | 8 | 0.6 |
|  | n77 | 0.8 |
| DC_8_n78 | 8 | 0.6 |
|  | n78 | 0.8 |
| DC_11_n1 | 11 | 0.3 |
|  | n1 | 0.3 |
| DC_11_n3 | 11 | 0.8 |
|  | n3 | 0.9 |
| DC_11_n28 | 11 | 0.4 |
|  | n28 | 0.6 |
| DC_11_n41 | 11 | 0.3 |
|  | n41 | 0.3 |
| DC_11_n77 | 11 | 0.4 |
|  | n77 | 0.8 |
| DC_11_n78 | 11 | 0.4 |
|  | n78 | 0.8 |
| DC_12_n2 | 12 | 0.3 |
|  | n2 | 0.3 |
| DC_12_n5 | 12 | 0.4 |
|  | n5 | 0.8 |
| DC_12_n7 | 12 | 0.3 |
|  | n7 | 0.3 |
| DC_12_n25 | 12 | 0.3 |
|  | n25 | 0.3 |
| DC_12_n30 | 12 | 0.3 |
|  | n30 | 0.3 |
| DC_12_n38 | 12 | 0.3 |
|  | n38 | 0.3 |
| DC_12_n41 | 12 | 0.3 |
|  | n41 | 0.3 |
| DC_12_n66 | 12 | 0.8 |
|  | n66 | 0.3 |
| DC_12_n71 | 12 | 1 |
|  | n71 | 1 |
| DC_12_n77 | 12 | 0.5 |
|  | n77 | 0.8 |
| DC_12_n78 | 12 | 0.5 |
|  | n78 | 0.8 |
| DC_13_n2 | 13 | 0.3 |
|  | n2 | 0.3 |
| DC_13_n5 | 13 | 0.5 |
|  | n5 | 0.5 |
| DC_13_n7 | 13 | 0.5 |
|  | n7 | 0.5 |
| DC_13_n25 | 13 | 0.3 |
|  | n25 | 0.3 |
| DC_13_n48 | 13 | 0.3 |
|  | n48 | 0.3 |
| DC_13_n66 | 13 | 0.3 |
|  | n66 | 0.3 |
| DC_13_n71 | 13 | 0.5 |
|  | n71 | 0.5 |
| DC_13_n77 | 13 | 0.5 |
|  | n77 | 0.8 |
| DC_13_n78 | 13 | 0.5 |
|  | n78 | 0.8 |
| DC_14_n2 | 14 | 0.3 |
|  | n2 | 0.3 |
| DC_14_n5 | 14 | 0.5 |
|  | n5 | 0.5 |
| DC_14_n30 | 14 | 0.3 |
|  | n30 | 0.3 |
| DC_14_n66 | 14 | 0.3 |
|  | n66 | 0.3 |
| DC_14_n77 | 14 | 0.5 |
|  | n77 | 0.8 |
| DC_18_n3 | 18 | 0.3 |
|  | n3 | 0.3 |
| DC_18_n28 | 18 | 0.5 |
|  | n28 | 0.5 |
| DC_18_n41 | 18 | 0.3 |
|  | n41 | 0.33 |
| DC_18_n77 | 18 | 0.3 |
|  | n77 | 0.8 |
| DC_18_n78 | 18 | 0.3 |
|  | n78 | 0.8 |
| DC_19_n1 | 19 | 0.3 |
|  | n1 | 0.3 |
| DC_19_n77 | 19 | 0.3 |
|  | n77 | 0.8 |
| DC_19_n78 | 19 | 0.3 |
|  | n78 | 0.8 |
| DC_20_n1 | 20 | 0.3 |
|  | n1 | 0.3 |
| DC_20_n3 | 20 | 0.3 |
|  | n3 | 0.3 |
| DC_20_n7 | 20 | 0.3 |
|  | n7 | 0.3 |
| DC_20_n8 | 20 | 0.4 |
|  | n8 | 0.4 |
| DC_20_n28 | 20 | 0.5 |
|  | n28 | 0.5 |
| DC_20_n38 | 20 | 0.5 |
|  | n38 | 0.3 |
| DC_20_n41 | 20 | 0.3 |
|  | n41 | 0.3 |
| DC_20_n50 | 20 | 0.3 |
|  | n50 | 0.4 |
| DC_20_n51 | 20 | 0.5 |
|  | n51 | 0.5 |
| DC_20_n77 | 20 | 0.6 |
|  | n77 | 0.8 |
| DC_20_n78 | 20 | 0.6 |
|  | n78 | 0.8 |
| DC_21_n1 | 21 | 0.3 |
|  | n1 | 0.3 |
| DC_21_n28 | 21 | 0.4 |
|  | n28 | 0.3 |
| DC_21_n77 | 21 | 0.4 |
|  | n77 | 0.8 |
| DC_21_n78 | 21 | 0.4 |
|  | n78 | 0.8 |
| DC_25_n41, DC_25-25_n41 | 25 | 0.5 |
|  | n41 | 0.41 |
|  |  | 0.92 |
| DC_25_n77 DC_25-25_n77 | 25 | 0.6 |
|  | n77 | 0.8 |
| DC_25_n78 DC_25-25_n78 | 25 | 0.6 |
|  | n78 | 0.8 |
| DC_26_n25 | 26 | 0.3 |
|  | n25 | 0.3 |
| DC_26_n41 | 26 | 0.3 |
|  | n41 | 0.3 |
| DC_26_n77 | 26 | 0.3 |
|  | n77 | 0.8 |
| DC_26_n78 | 26 | 0.3 |
|  | n78 | 0.8 |
| DC_28_n1 | n1 | 0.3 |
|  | 28 | 0.6 |
| DC_28_n2 | 28 | 0.3 |
|  | n2 | 0.3 |
| DC_28_n3 | 28 | 0.3 |
|  | n3 | 0.3 |
| DC_28_n5 | 28 | 0.5 |
|  | n5 | 0.5 |
| DC_28_n7 | 28 | 0.3 |
|  | n7 | 0.3 |
| DC_28_n8 | 28 | 0.5 |
|  | n8 | 0.6 |
| DC_28_n40 | 28 | 0.3 |
|  | n40 | 0.3 |
| DC_28_n41 | 28 | 0.3 |
|  | n41 | 0.3 |
| DC_28_n50 | 28 | 0.3 |
|  | n50 | 0.4 |
| DC_28_n51 | 28 | 0.5 |
|  | n51 | 0.5 |
| DC_28_n66 | 28 | 0.6 |
|  | n66 | 0.3 |
| DC_28_n77 | 28 | 0.5 |
|  | n77 | 0.8 |
| DC_28_n78 | 28 | 0.5 |
|  | n78 | 0.8 |
| DC_30_n2 | 30 | 0.3 |
|  | n2 | 0.5 |
| DC_30_n5 | 30 | 0.3 |
|  | n5 | 0.3 |
| DC_30_n66 | 30 | 0.5 |
|  | n66 | 0.8 |
| DC_30_n77 | 30 | 0.5 |
|  | n77 | 0.8 |
| DC_38_n1 | 38 | 0.5 |  |
|  | n1 | 0.5 |  |
| DC_38_n3 | 38 | 0.5 |  |
|  | n3 | 0.5 |  |
| DC_38_n8 | 38 | 0.6 |  |
|  | n8 | 0.3 |  |
| DC_38_n28 | 38 | 0.3 |
|  | n28 | 0.3 |
| DC_38_n78 | n78 | 0.5 |
| DC_38_n79 | 38 | 0.3 |
|  | n79 | 0.8 |
| DC_39-n41 | 39 | 0.5 |
|  | n41 | 0.5 |
| DC_39_n78 | 39 | 0.3 |
|  | n78 | 0.8 |
| DC_39_n79 | 39 | 0.3 |
|  | n79 | 0.8 |
| DC_40_n1 | n1 | 0.5 |
|  | 40 | 0.5 |
| DC_40_n415 | 40 | 0.5 |
|  | n41 | 0.5 |
| DC_40_n77 | n77 | 0.5 |
| DC_40_n78 | n78 | 0.56 |
| DC_40_n79 | 40 | 0.3 |
|  | n79 | 0.8 |
| DC_41_n1 | 41 | 0.5 |
|  | n1 | 0.5 |
| DC_41_n3 | 41 | 0.33 |
|  |  | 0.84 |
|  | n3 | 0.5 |
| DC_41_n28 | 41 | 0.3 |
|  | n28 | 0.3 |
| DC_41_n77 | 41 | 0.3 |
|  | n77 | 0.8 |
| DC_41_n78 | 41 | 0.3 |
|  | n78 | 0.8 |
| DC_41_n79 | 41 | 0.3 |
|  | n79 | 0.8 |
| DC_42_n1 | 42 | 0.8 |
|  | n1 | 0.3 |
| DC_42_n3 | 42 | 0.8 |
|  | n3 | 0.6 |
| DC_42_n28 | 42 | 0.5 |
|  | n28 | 0.8 |
| DC_42_n51 | 42 | 0.6 |
|  | n51 | 0.8 |
| DC_48_n5 | 48 | 0.3 |
|  | n5 | 0.3 |
| DC_48_n12 | 48 | 0.3 |
|  | n12 | 0.3 |
| DC_48_n25 | 48 | 0.8 |
|  | n25 | 0.6 |
| DC_48_n46 | 48 | 0.8 |
| DC_48_n66 | 48 | 0.8 |
|  | n66 | 0.6 |
| DC_48_n71 DC_48-48_n71 DC_48-48-48_n71 | 48 | 0.3 |
|  | n71 | 0.3 |
| DC_66_n2 | 66 | 0.5 |
| DC_66-66_n2 DC_66-66-66_n2 | n2 | 0.5 |
| DC_66_n5, DC_66-66_n5, DC_66-66-66_n5 | 66 | 0.3 |
|  | n5 | 0.3 |
| DC_66_n7, | 66 | 0.5 |
| DC_66-66_n7 | n7 | 0.5 |
| DC_66_n12 | 66 | 0.8 |
|  | n12 | 0.3 |
| DC_66_n25 | 66 | 0.5 |
|  | n25 | 0.5 |
| DC_66_n28 | n28 | 0.6 |
|  | 66 | 0.3 |
| DC_66_n30 | 66 | 0.5 |
| DC_66-66_n30 | n30 | 0.8 |
| DC_66_n38, | 66 | 0.5 |
| DC_66-66_n38 | n38 | 0.5 |
| DC_66_n41 | 66 | 0.5 |
|  | n41 | 0.81 |
|  |  | 1.32 |
| DC_66_n48, DC_66-66_n48 | 66 | 0.6 |
|  | n48 | 0.8 |
| DC_66_n71, | 66 | 0.3 |
| DC_66-66_n71 | n71 | 0.3 |
| DC_66_n77 DC_66-66_n77 DC_66-66-66_n77 | 66 | 0.6 |
|  | n77 | 0.8 |
| DC_66_n78, | 66 | 0.6 |
| DC_66-66_n78 | n78 | 0.8 |
| DC_71_n2 | 71 | 0.3 |
|  | n2 | 0.3 |
| DC_71_n5 | 71 | 0.5 |
|  | n5 | 0.5 |
| DC_71_n38 | 71 | 0.6 |
|  | n38 | 0.3 |
| DC_71_n41 | 71 | 0.6 |
|  | n41 | 0.3 |
| DC_71_n48 | 71 | 0.3 |
|  | n48 | 0.3 |
| DC_71_n66 | 71 | 0.3 |
|  | n66 | 0.3 |
| DC_71_n78 | 71 | 0.5 |
|  | n78 | 0.8 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545 MHz. NOTE 3: Applicable for the frequency range of 2515 – 2690 MHz. NOTE 4: Applicable for the frequency range of 2496 - 2515 MHz. NOTE 5: Applicable for UE supporting inter-band EN-DC without simultaneous Rx/Tx. NOTE 6: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. |  |  |

<< Next changes >>

6.5B.3.3	Inter-band EN-DC within FR1

##### 6.5B.3.3.1	General spurious emissions

The general spurious emissions requirements specified in clause 6.6.3.1 of TS 36.101 [4], clause 6.5.3.1 of TS 38.101-1 [2] and TS 38.101-2 [3] apply for each component carrier. For the case of inter-band EN-DC with a single carrier per cell group, the general spurious emissions requirements also apply with both downlink carrier and both both uplink carriers active. Limits on configured maximum output power for the uplink according to clause 6.2B.4 apply.

NOTE:	The general spurious emission requirements with both uplink carriers active are allowed to be verified for only a single inter-band EN-DC configuration per NR band. Furthermore, the requirements are allowed to be verified by measuring spurious emissions at the specific frequencies where second and third order intermodulation products generated by the two transmitted carriers can occur.

Table 6.5B.3.3.1-1: (Void)

6.5B.3.3.2	Spurious emission band UE co-existence

This clause specifies the requirements for the specified EN-DC, for coexistence with protected bands. The requirements in Table 6.5B.3.3.2-1 apply on each component carrier with all component carriers are active.

NOTE:	For inter-band EN-DC with uplink assigned to one LTE band and one NR band the requirements in Table 6.5B.3.3.2-1 could be verified by measuring spurious emissions at the specific frequencies where second and third order intermodulation products generated by the two transmitted carriers can occur;

Table 6.5B.3.3.2-1: Requirements

| EN-DC Configuration |  |  | Spurious emission |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Protected band |  | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |  |
| DC_1_n3 |  |  | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 38, 40, 41, 43, 44, 50, 51, 65, 67, 72, 73, 74, 75, 76 NR Band n79 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA band 3, 34 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  |  |  | E-UTRA band 22, 42, 52 NR Band n77, n78 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | Frequency range |  | 1880 |  | 1895 | -40 | 1 | 5, 67 |  |
|  |  |  | Frequency range |  | 1895 |  | 1915 | -15.5 | 5 | 5, 7, 16 |  |
|  |  |  | Frequency range |  | 1915 |  | 1920 | +1.6 | 5 | 5, 7, 16 |  |
| DC_1_n5 |  |  | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 21, 22, 26, 28, 31, 38, 40, 42, 43, 50, 51, 65, 73, 74 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA band 3,34 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  |  |  | E-UTRA band 41, 52 NR Band n77, n78, n79 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
| DC_1_n7 |  |  | E-UTRA Band 1, 5, 7, 8, 20, 22, 26, 27, 28, 31,32, 40, 42, 43, 50, 51, 52, 65, 67, 72, 74, 75, 76 NR Band n78, n79 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | band n77 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | band 3, 34 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  |  |  | Frequency range |  | 1880 |  | 1895 | -40 | 1 | 5,16 |  |
|  |  |  | Frequency range |  | 1895 |  | 1915 | -15.5 | 5 | 5, 7, 16 |  |
|  |  |  | Frequency range |  | 1915 |  | 1920 | +1.6 | 5 | 5, 7, 16 |  |
|  |  |  | Frequency range |  | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |  |
|  |  |  | Frequency range |  | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |  |
|  |  |  | Frequency range |  | 2595 | - | 2620 | -40 | 1 | 5, 6 |  |
| DC_1_n8 |  |  | E-UTRA Band 11, 20, 21, 28, 31, 32, 38, 40, 45, 50, 51, 65, 67, 68, 69, 72, 73, 74, 75, 76 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA band 3, 7, 22, 41, 42, 43, 52 NR Band n77, n78, n79 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | E-UTRA Band 1, 8, 34 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  |  |  | Frequency range |  | 1880 |  | 1895 | -40 | 1 | 5, 16 |  |
|  |  |  | Frequency range |  | 1895 |  | 1915 | -15.5 | 5 | 5, 7, 16 |  |
|  |  |  | Frequency range |  | 1915 |  | 1920 | +1.6 | 5 | 5, 7, 16 |  |
| DC_1_n20 |  |  | E-UTRA Band 1, 3, 7, 8, 22, 31, 32, 40, 43, 50, 51, 65, 67, 68, 72, 74, 75, 76 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 38, 42, 69 NR Band n77, n78 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | E-UTRA Band 20, 34 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  |  |  | Frequency range |  | 758 | - | 788 | -50 | 1 |  |  |
| DC_1_n28 |  |  | E-UTRA Band 5, 7, 8, 18, 19, 20, 26, 27, 31, 38, 40, 41, 72, 73 NR band n79 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 1, 22, 32, 42, 43, 50, 51, 52, 65, 74, 75, 76 NR band n77, n78 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | E-UTRA band 3, 34 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  |  |  | E-UTRA Band 11, 21 |  | FDL_low | - | FDL_high | -50 | 1 | 9, 11 |  |
|  |  |  | E-UTRA Band 1, 65 |  | FDL_low | - | FDL_high | -50 | 1 | 9, 10 |  |
|  |  |  | Frequency range |  | 470 | - | 694 | -42 | 8 | 5, 17 |  |
|  |  |  | Frequency range |  | 470 | - | 710 | -26.2 | 6 | 14 |  |
|  |  |  | Frequency range |  | 758 | - | 773 | -32 | 1 | 5 |  |
|  |  |  | Frequency range |  | 773 | - | 803 | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 662 | - | 694 | -26.2 | 6 | 5 |  |
|  |  |  | Frequency range |  | 1880 | - | 1895 | -40 | 1 | 5,16 |  |
|  |  |  | Frequency range |  | 1895 | - | 1915 | -15.5 | 5 | 5, 7, 16 |  |
|  |  |  | Frequency range |  | 1915 | - | 1920 | +1.6 | 5 | 5, 7, 16 |  |
| DC_1_n38 |  |  | E-UTRA Band 1, 3, 5, 8, 20, 22, 27, 28, 31, 32, 34, 40, 42, 43, 50, 51, 65, 67, 68, 72, 74, 75, 76 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
| DC_1_n40 |  |  | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 20, 21, 22, 26, 27, 28, 31, 32, 38, 41, 42, 43, 44, 45, 50, 51, 52, 65, 67, 68, 69, 72, 73, 74, 75, 76 NR band n78 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | Band 3, 34 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  |  |  | NR band n77, n79 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | Frequency range |  | 1880 |  | 1895 | -40 | 1 | 5, 16 |  |
|  |  |  | Frequency range |  | 1895 |  | 1915 | -15.5 | 5 | 5, 7, 16 |  |
|  |  |  | Frequency range |  | 1915 |  | 1920 | +1.6 | 5 | 5, 7, 16 |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
| DC_1_n41 |  |  | E-UTRA Band 3, 4, 5, 8, 12, 13, 14, 17, 19, 20, 21, 24, 26, 27, 28, 29, 30, 31, 32, 42, 43, 44, 45, 50, 51, 52, 66, 67, 68, 71, 72, 73, 75, 76, 85 NR Band n78 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 34 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  |  |  | NR Band n77, n79 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | E-UTRA Band 40 |  | FDL_low | - | FDL_high | -40 | 1 |  |  |
|  |  |  | Frequency range |  | 1880 |  | 1895 | -40 | 1 | 5, 8 |  |
|  |  |  | Frequency range |  | 1895 |  | 1915 | -15.5 | 5 | 5, 7, 8 |  |
|  |  |  | Frequency range |  | 1915 |  | 1920 | +1.6 | 5 | 5, 7, 8, 20 |  |
|  |  |  | E-UTRA Band 11, 18, 19, 21, 74 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
| DC_1A_n50A |  |  | E-UTRA Band 3, 4, 5, 7, 8, 12, 13, 17, 18, 19, 20, 26, 27, 28, 29, 31, 38, 40, 41, 42, 43, 44, 48, 52, 66, 67, 68, 69, 72, 73, 85 NR Band n78 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 34 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  |  |  | NR Band n77, n79 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | Frequency range |  | 1880 |  | 1895 | -40 | 1 | 5,16 |  |
|  |  |  | Frequency range |  | 1895 |  | 1915 | -15.5 | 5 | 5, 7, 16 |  |
|  |  |  | Frequency range |  | 1915 |  | 1920 | +1.6 | 5 | 5, 7, 16 |  |
| DC_1_n51 |  |  | E-UTRA Band 7, 12, 13, 17, 20, 22, 27, 28, 29, 31, 38, 44, 48, 67, 68, 69, 72, 73 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 3, 34 |  | FDL_low | - | FDL_high | -50 | 1 | 5, 2 |  |
|  |  |  | Frequency range |  | 1880 | - | 1895 | -40 | 1 | 5, 16 |  |
|  |  |  | Frequency range |  | 1895 | - | 1915 | -15.5 | 5 | 5, 7, 16 |  |
|  |  |  | Frequency range |  | 1915 | - | 1920 | +1.6 | 5 | 5, 7, 16 |  |
|  |  |  | E-UTRA Band 5, 6, 8, 26, 30, 40, 41, 42, 43, 46 NR Band n77, n78, n79, |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
| DC_1_n71 |  |  | E-UTRA Band 1, 5, 26, |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 41 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | E-UTRA Band 71 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
| DC_1_n77 DC_1_n84_ULSUP-TDM_n77 |  |  | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 40, 41, 65, 74 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 1880 | - | 1895 | -40 | 1 | 5, 8 |  |
|  |  |  | Frequency range |  | 1895 | - | 1915 | -15.5 | 5 | 5, 7, 8 |  |
|  |  |  | Frequency range |  | 1915 | - | 1920 | +1.6 | 5 | 5, 7, 8 |  |
| DC_1_n78 DC_1_n84_ULSUP-TDM_n78 |  |  | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 40, 41, 65, 74 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 1880 | - | 1895 | -40 | 1 | 5, 8 |  |
|  |  |  | Frequency range |  | 1895 | - | 1915 | -15.5 | 5 | 5, 7, 8 |  |
|  |  |  | Frequency range |  | 1915 | - | 1920 | +1.6 | 5 | 5, 7, 8 |  |
| DC_1_n79 DC_1_n84_ULSUP-TDM_n79 |  |  | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 21, 26, 28, 34, 40, 41, 42, 65, 74 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 1880 | - | 1895 | -40 | 1 | 5, 8 |  |
|  |  |  | Frequency range |  | 1895 | - | 1915 | -15.5 | 5 | 5, 7, 8 |  |
|  |  |  | Frequency range |  | 1915 | - | 1920 | +1.6 | 5 | 5, 7, 8 |  |
| DC_1_n80 |  |  | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 38, 40, 41, 43, 44, 45, 50, 51, 65, 67, 68, 69, 72, 73,74, 75, 76, NR Band n79 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 3, 34 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  |  |  | E-UTRA Band 22, 42, NR Band n77, n78 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
| DC_2_n5 |  |  | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 28, 29, 30, 42, 50, 51, 66, 70, 71, 74, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | NR Band n77 |  | FDL_low | - | FDL_high | -50 | 1 | 2, 5 |  |
|  |  |  | E-UTRA Band 2, 25, 48 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | E-UTRA Band 41, 43, 53 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
| DC_2_n7 |  |  | E-UTRA Band 2, 4, 5, 7, 12, 13, 14, 17, 26, 27, 28, 29, 30, 42, 50, 51, 66, 74, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 43 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | Frequency range |  | 2570 | - | 2575 | 1.6 | 5 | 5, 6, 7 |  |
|  |  |  | Frequency range |  | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |  |
|  |  |  | Frequency range |  | 2595 | - | 2620 | -40 | 1 | 5, 6 |  |
| DC_2_n12 |  |  | E-UTRA Band 5, 13, 14, 17, 24, 26, 27, 30, 41, 50, 53, 71, 74 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 25, 85 NR band n12 |  | FDL_low | - | FDL_high | -50 | 1 | 3 |  |
|  |  |  | E-UTRA Band 2 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  |  |  | E-UTRA Band 4, 51, 66, 70, NR Band n77 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
| DC_2_n28 |  |  | E-UTRA Band 5, 26, 27, 41 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 4, 10, 42, 43, 50, 51, 66, 74 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | E-UTRA band 2, 25 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 11, 21 |  | FDL_low | - | FDL_high | -50 | 1 | 9, 11 |  |
|  |  |  | E-UTRA Band 1, 65 |  | FDL_low | - | FDL_high | -50 | 1 | 9, 10 |  |
|  |  |  | Frequency range |  | 470 | - | 710 | -26.2 | 6 | 14 |  |
|  |  |  | Frequency range |  | 758 | - | 773 | -32 | 1 | 5 |  |
|  |  |  | Frequency range |  | 773 | - | 803 | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 2570 | - | 2575 | 1.6 | 5 | 5, 6, 7 |  |
|  |  |  | Frequency range |  | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |  |
|  |  |  | Frequency range |  | 2595 | - | 2620 | -40 | 1 | 5, 6 |  |
| DC_2_n30 |  |  | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 27, 29, 30, 41, 48, 53, 66, 70, 71, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 2, 25 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  |  |  | NR Band n77 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
| DC_2_n38 |  |  | E-UTRA Band 4, 5, 12, 13, 14,17, 27, 28, 29, 30, 42, 50, 51, 66, 74, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 2 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  |  |  | E-UTRA Band 43 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
| DC_2_n41 |  |  | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 42, 48, 50, 51, 66, 70, 71, 74, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 2, 25 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  |  |  | E-UTRA Band 43, NR Band n77 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
| DC_2A_n48A |  |  | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 29, 30, 41, 50, 51, 66, 70, 71, 74, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 2, 25 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
| DC_2_n66 |  |  | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 41, 50, 51, 66, 70, 71, 74, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 2, 25 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  |  |  | E-UTRA Band 42, 48, NR Band n77 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
| DC_2_n71 |  |  | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 29, 30, 48, 66 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 2, 25, 41, 70, NR Band n77 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | E-UTRA Band 71 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
| DC_2_n77 |  |  | E-UTRA Band 4, 5, 12, 13, 14, 17, 26, 29, 30, 41, 65, 66, 70, 71 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 2, 25 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
| DC_2_n78 |  |  | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 41, 50, 51, 66, 70, 71, 74, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 2, 25 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
| DC_3_n1 |  |  | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 38, 40, 41, 43, 44, 50, 51, 65, 67, 72, 73, 74, 75, 76 NR Band n79 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA band 3, 34 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  |  |  | E-UTRA band 22, 42, 52 NR Band n77, n78 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | Frequency range |  | 1880 |  | 1895 | -40 | 1 | 5,16 |  |
|  |  |  | Frequency range |  | 1895 |  | 1915 | -15.5 | 5 | 5, 7, 16 |  |
|  |  |  | Frequency range |  | 1915 |  | 1920 | +1.6 | 5 | 5, 7, 16 |  |
| DC_3_n5 |  |  | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 21, 26, 28, 31, 38, 40, 43, 50, 51, 65, 73, 74 NR Band n79 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA band 3,34 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  |  |  | E-UTRA Band 22, 42, 52 Band n77, n78 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
| DC_3_n7 |  |  | E-UTRA Band 1, 5, 7, 8, 20, 26, 27, 28, 31, 32, 33, 34, 40, 43, 44, 50, 51, 65, 67, 72, 74, 75, 76 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA band 3 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  |  |  | E-UTRA band 22, 42 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | Frequency range |  | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |  |
|  |  |  | Frequency range |  | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |  |
|  |  |  | Frequency range |  | 2595 | - | 2620 | -40 | 1 | 5, 6 |  |
| DC_3_n8 |  |  | E-UTRA Band 1, 11, 20, 21, 28, 31, 32, 33, 34, 38, 39, 40, 45, 50, 51, 65, 67,68, 69, 72, 73, 74, 75, 76 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA band 3, 8 |  | FDL_low | - | FDL_high | -50 | 1 | 2, 5 |  |
|  |  |  | E-UTRA band 7, 22, 41, 42, 43, 52 NR Band n77, n78, n79 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
| DC_3_n20 |  |  | E-UTRA Band 1, 7, 8, 31, 32, 33, 34, 40, 43, 50, 51, 65, 67, 72, 74, 75, 76 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 3 NR band n20 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  |  |  | E-UTRA Band 22, 38, 42, 52 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | Frequency range |  | 758 | - | 788 | -50 | 1 |  |  |
| DC_3_n28 |  |  | E-UTRA Band 1, 42, 43, 50, 51, 65, 74, 75, 76 NR band n77, n78, n79 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | E-UTRA band 1 |  | FDL_low | - | FDL_high | -50 | 1 | 9, 11 |  |
|  |  |  | E-UTRA band 3 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  |  |  | E-UTRA Band 5, 7, 8, 18, 19, 20, 26, 27, 31, 34, 38, 40, 41, 72 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 11, 21 |  | FDL_low | - | FDL_high | -50 | 1 | 9, 10 |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 13 |  |
|  |  |  | Frequency range |  | 470 | - | 710 | -26.2 | 6 | 14 |  |
|  |  |  | Frequency range |  | 758 | - | 773 | -32 | 1 | 5 |  |
|  |  |  | Frequency range |  | 773 | - | 803 | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 9 |  |
| DC_3_n34 |  |  | E-UTRA Band 1, 7, 8, 11, 18, 19, 20, 21, 26, 28, 31, 32, 33, 38, 39, 40, 41, 43, 44, 45, 50, 51, 65, 67, 69,72, 73, 74, 75, 76, 79 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 22, 42, 52 NR Band n78 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | E-UTRA Band 3 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
| DC_3_n38 |  |  | E-UTRA Band 1, 5, 8, 20, 27, 28, 31, 32, 33, 34, 40, 43, 50, 51, 65, 67, 68, 72, 74, 75, 76 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 22, 42 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
| DC_3_n40 |  |  | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 33, 34, 38, 39, 41, 43, 44. 45, 50, 51, 65, 67, 68, 69, 72, 73, 74, 75, 76 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 3 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  |  |  | E-UTRA Band 22, 42, 52 NR band n77, n78, n79 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
| DC_3_n41, DC_3_n80_ULSUP-TDM_n41 |  |  | E-UTRA Band 1, 5, 8, 11, 18, 19, 21, 26, 27, 28, 34, 39, 44, 45, 50, 51, 65, 73, 74 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 42, 52 NR Band n77, n78, n79 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | E-UTRA Band 40 |  | FDL_low | - | FDL_high | -40 | 1 |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
| DC_3_n50 |  |  | E-UTRA Band 5, 7, 8, 12, 13, 17, 18, 19, 20, 26, 27, 28, 29, 31, 38, 40, 41, 43, 44, 52, 67, 68, 69, 72, 73 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 1, 2, 4, 33, 34, 39, 42, 48, 65, 66 NR Band n77, n78, n79 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 |  |  |
| DC_3_n51 |  |  | E-UTRA Band 7, 8, 12, 13, 17, 20, 27, 28, 31, 33, 38, 67, 68, 69, 72, 73 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 3 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  |  |  | E-UTRA Band 1, 5, 6, 22, 26, 30, 34, 36, 40, 41, 42, 43, 44, 46, 48, 65, 71 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
| DC_3_n71 |  |  | E-UTRA Band 5, 26, |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 41 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | E-UTRA Band 3, 71 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
| DC_3_n77 DC_3_n80_ULSUP-TDM_n77 |  |  | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 39, 40, 41, 65, 74 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
| DC_3_n78 DC_3_n80_ULSUP-TDM_n78 |  |  | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 39, 40, 41, 65, 74 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
| DC_3_n79 DC_3_n80_ULSUP-TDM_n79 |  |  | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 28, 34, 39, 40, 41, 65, 74 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 42 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
| DC_3_n82 |  |  | E-UTRA Band 1, 3 7, 8, 20,31, 32, 33, 34, 40, 43, 50, 51, 65, 67, 68, 72,74, 75, 76 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 22, 38, 42, 69 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
| DC_3_n84 |  |  | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 38, 40, 41, 43, 44, 45, 50, 51, 65, 67, 68, 69, 72, 73,74, 75, 76 NR Band n79 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 3 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  |  |  | NR Band n77, n78 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
| DC_4_n2 |  |  | E-UTRA Band 4, 5, 10, 12, 13, 14, 17, 22, 24, 26, 27, 28, 29, 30, 41, 50, 51, 53, 66, 70, 71, 74, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 2, 25 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  |  |  | E-UTRA Band 42, 43, NR Band n77, n78 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
| DC_4_n5 |  |  | Bands 1, 2, 3, 4, 5, 6, 7, 8, 10, 12, 13, 14, 17, 24, 25, 28, 29, 30, 34, 38, 40, 43, 45, 50, 51, 65, 66, 70, 71, n71, 85, n257 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 26 |  | 859 | - | 869 | -27 | 1 |  |  |
|  |  |  | Bands 41, 42, 48, 52 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | E-UTRA Band 18, 19 |  | FDL_low | - | FDL_high | -40 | 1 |  |  |
|  |  |  | E-UTRA Band 11, 21 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
| DC_4_n7 |  |  | E-UTRA Band 2, 4, 5, 7, 10, 12, 13, 14, 17, 26, 27, 28, 29, 30, 43, 50, 51, 66, 74, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 42 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | Frequency range |  | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |  |
|  |  |  | Frequency range |  | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |  |
| DC_4_n28 |  |  | E-UTRA Band 2, 5, 7, 25, 26, 27, 38, 41 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 4, 10, 42, 43, 50, 51, 66, 74, NR band n77, n78 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | Frequency range |  | 470 | - | 694 | -42 | 8 | 5, 17 |  |
|  |  |  | Frequency range |  | 470 | - | 710 | -26.2 | 6 | 14 |  |
|  |  |  | Frequency range |  | 662 | - | 694 | -26.2 | 6 | 5 |  |
|  |  |  | Frequency range |  | 758 | - | 773 | -32 | 1 | 5 |  |
|  |  |  | Frequency range |  | 773 | - | 803 | -50 | 1 |  |  |
| DC_4_n38 |  |  | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 27, 28, 29, 30, 43, 50, 51, 66, 74, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 42 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
| DC_4_n41 |  |  | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 27, 28, 29, 30, 48, 50, 51, 66, 70, 71, 74, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 42, 48 NR Band n77 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
| DC_4_n78 |  |  | E-UTRA Band 5, 7, 26, 28, 41 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
| DC_5_n1 |  | E-UTRA Band 5, 7, 8, 11, 18, 19, 21, 22, 26, 28, 31, 38, 40, 42, 43, 50, 51, 65, 73, 74 NR band n1 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  | E-UTRA band 3, 34 | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  |  | E-UTRA band 41, 52 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
| DC_5_n2 |  |  | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 28, 29, 30, 42, 50, 51, 66, 70, 71, 74, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 25 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  |  |  | NR Band n2 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  |  |  | E-UTRA Band 26 |  | 859 | - | 869 | -27 | 1 |  |  |
|  |  |  | E-UTRA Band 41, 43, 53 NR Band n77 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
| DC_5_n3 |  | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 21, 26, 28, 31, 38, 40, 43, 50, 51, 65, 73, 74 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  | E-UTRA band 34 NR band n3 | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  |  |  | E-UTRA Band 22, 42, 52 Band n77, n78 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
| DC_5_n7 |  |  | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 14, 17, 28, 29, 30, 31, 34, 40, 42, 43, 65, 66, 71, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 52 NR Band n77, n78 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | E-UTRA band 26 |  | 859 | - | 869 | -27 | 1 |  |  |
|  |  |  | Frequency range |  | 2570 | - | 2575 | +1.6 | 5 | 5, 7, 6 |  |
|  |  |  | Frequency range |  | 2575 | - | 2595 | -15.5 | 5 | 5, 7, 6 |  |
|  |  |  | Frequency range |  | 2595 | - | 2620 | -40 | 1 | 5, 14 |  |
| DC_5_n12 |  |  | E-UTRA Band 2, 5, 13, 14, 17, 24, 25, 26, 30, 43, 50, 71, 74 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Bands 4, 41, 42, 48, 51, 66, 70, NR Band n77 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | E-UTRA Band 12, 85 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
| DC_5_n30 |  |  | E-UTRA Band 2, 4, 5, 7,12, 13, 14, 17, 24, 25, 26, 29, 30, 38, 48, 66, 70, 71, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 41, 53 NR Band n77 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
| DC_5_n38 |  |  | E-UTRA Band 1, 2, 3, 4, 5, 8, 12, 13, 14, 17, 28, 29, 30, 31, 34, 40, 42, 43, 50, 51, 65, 66, 74, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 52 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
| DC_5_n40 |  |  | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 21, 28, 31, 34, 38, 42, 43, 45, 65, 73, 74 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 26 |  | 859 | - | 869 | -27 | 1 |  |  |
|  |  |  | E-UTRA Band 41, 52 NR band n77, n78, n79 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
| DC_5_n48 |  |  | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 29, 30, 50, 51, 66, 70, 71, 74, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 26 |  | 859 | - | 869 | -27 | 1 |  |  |
|  |  |  | E-UTRA Band 41 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
| DC_5_n66 |  |  | E-UTRA Band 1, 2, 3, 4, 5, 6, 7, 8, 12, 13, 14, 17, 24, 25, 28, 29, 30, 34, 38, 40, 43, 45, 50, 51, 65, 66, 70, 71, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 26 |  | 859 | - | 869 | -27 | 1 |  |  |
|  |  |  | E-UTRA Band 41, 42, 48, 52, NR Band n77 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
| DC_5_n71 |  |  | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 30, 48, 66, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 2, 25, 41, 70, NR Band n77 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | E-UTRA Band 29 |  | FDL_low | - | FDL_high | -38 | 1 | 5 |  |
|  |  |  | E-UTRA Band 71 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
| DC_5_n77 |  |  | E-UTRA Band 2, 4, 12, 13, 14, 17, 25, 26, 28, 29, 30, 40, 65, 66, 70, 71 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 41 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
| DC_5_n78 |  |  | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 14, 17, 24, 25, 28, 29, 30, 31, 34, 38, 40, 45, 65, 66, 70 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 26 |  | 859 | - | 869 | -27 | 1 |  |  |
|  |  |  | E-UTRA Band 41 |  | FDL_low | - | FDL_high | -50 | 1 | 2, 7 |  |
| DC_5_n79 |  |  | Bands 1, 2, 3, 4, 5, 7, 8, 12, 13, 14, 17, 24, 25, 28, 29, 30, 31, 34, 38, 40, 42, 43, 45, 48, 50, 51, 65, 66, 70, 71, 73, 74, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 26 |  | 859 | - | 869 | -27 | 1 |  |  |
|  |  |  | Bands 41, 52 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
| DC_7_n1 |  |  | Band 1, 5, 7, 8, 20, 22, 26, 27, 28, 31,32, 40, 42, 43, 50, 51, 52, 65, 67, 72, 74, 75, 76, n78,n79 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | band n77 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | band 3, 34 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  |  |  | Frequency range |  | 1880 |  | 1895 | -40 | 1 | 5,16 |  |
|  |  |  | Frequency range |  | 1895 |  | 1915 | -15.5 | 5 | 5, 7,16 |  |
|  |  |  | Frequency range |  | 1915 |  | 1920 | +1.6 | 5 | 5, 7,16 |  |
|  |  |  | Frequency range |  | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |  |
|  |  |  | Frequency range |  | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |  |
|  |  |  | Frequency range |  | 2595 | - | 2620 | -40 | 1 | 5, 6 |  |
| DC_7_n2 |  |  | E-UTRA Band 4, 5, 7, 10, 12, 13, 14, 17, 26, 27, 28, 29, 30, 42, 50, 51, 66, 74, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 43 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | E-UTRA band 2 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 2570 | - | 2575 | 1.6 | 5 | 5, 6, 7 |  |
|  |  |  | Frequency range |  | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |  |
|  |  |  | Frequency range |  | 2595 | - | 2620 | -40 | 1 | 5, 6 |  |
| DC_7_n3 |  |  | E-UTRA Band 1, 5, 7, 8, 20, 26, 27, 28, 31, 32, 33, 34, 40, 43, 50, 51, 65, 67, 68, 72, 74, 75, 76 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA band 3 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  |  |  | E-UTRA band 22, 42, 52 NR band n78, n77 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | Frequency range |  | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |  |
|  |  |  | Frequency range |  | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |  |
|  |  |  | Frequency range |  | 2595 | - | 2620 | -40 | 1 | 5, 6 |  |
| DC_7_n5 |  |  | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 14, 17, 22, 26, 28, 29, 30, 31, 40, 42, 43, 50, 51, 65, 66, 74, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 52 NR Band n77, n78 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | Frequency range |  | 2570 | - | 2575 | +1.6 | 5 | 5, 7, 6 |  |
|  |  |  | Frequency range |  | 2575 | - | 2595 | -15.5 | 5 | 5, 7, 6 |  |
|  |  |  | Frequency range |  | 2595 | - | 2620 | -40 | 1 | 5, 14 |  |
| DC_7_n8 |  |  | E-UTRA Band 1, 20, 28, 31, 32, 33, 34, 40, 50, 51, 65, 67, 68, 72, 74, 75, 76 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA band 3, 7, 22, 42, 43, 52 NR Band n77, n78 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | E-UTRA Band 8 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  |  |  | Frequency range |  | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |  |
|  |  |  | Frequency range |  | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |  |
|  |  |  | Frequency range |  | 2595 | - | 2620 | -40 | 1 | 5, 6 |  |
| DC_7_n20 |  |  | E-UTRA Band 1, 3, 7, 8, 22, 31, 32, 33, 34, 40, 43, 50, 51, 65, 67, 68, 72, 74, 75, 76 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 42, 52 NR band n78, n77 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | E-UTRA Band 20 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
| DC_7_n25 |  |  | E-UTRA Band 4, 5, 12, 13, 14, 17, 26, 27, 28, 29, 30, 42, 66, 71, 85 NR Band n77 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 43 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | E-UTRA Band 2 NR Band n25 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  |  |  | Frequency range |  | 2570 | - | 2575 | 1.6 | 5 | 5, 6, 7 |  |
|  |  |  | Frequency range |  | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |  |
|  |  |  | Frequency range |  | 2595 | - | 2620 | -40 | 1 | 5, 6 |  |
| DC_7_n28 |  |  | E-UTRA Band 2, 3, 5, 7, 8, 20, 26, 27, 31, 34, 40, 72 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 1, 4, 42, 43, 50, 51, 65, 66, 74, 75, 76 NR band n78 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | E-UTRA band 1 |  | FDL_low | - | FDL_high | -50 | 1 | 9, 10 |  |
|  |  |  | Frequency range |  | 758 | - | 773 | -32 | 1 | 5 |  |
|  |  |  | Frequency range |  | 773 | - | 803 | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |  |
|  |  |  | Frequency range |  | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |  |
|  |  |  | Frequency range |  | 2595 | - | 2620 | -40 | 1 | 5, 6 |  |
| DC_7_n40 |  |  | E-UTRA Band 1, 3, 5, 7, 8, 20, 22, 26, 27, 28, 31, 32, 33, 34, 42, 43, 50, 51, 52, 65, 67, 68, 72, 74, 75, 76, 77, 78 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |  |
|  |  |  | Frequency range |  | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |  |
|  |  |  | Frequency range |  | 2595 | - | 2620 | -40 | 1 | 5, 6 |  |
| DC_7_n51 |  |  | E-UTRA Band 2, 3, 5, 8, 26, 30, 31, 32, 33, 34, 40, 48, 72 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 2570 | - | 2575 | +1.6 | 5 | 5, 7, 16 |  |
|  |  |  | Frequency range |  | 2575 | - | 2595 | -15.5 | 5 | 5, 7, 16 |  |
|  |  |  | Frequency range |  | 2595 | - | 2620 | -40 | 1 | 5 |  |
|  |  |  | E-UTRA Band 1, 4, 12, 13, 14, 17, 20, 22, 23, 27, 28, 29, 42, 43, 44, 46, 65, 66, 67, 68 NR Band n77, n78, n79, |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
| DC_7_n66 |  |  | E-UTRA Band 2, 4, 5, 7, 12, 13, 14, 17, 26, 27, 28, 29, 30, 43, 50, 51, 66, 74, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 42 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | Frequency range |  | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |  |
|  |  |  | Frequency range |  | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |  |
|  |  |  | Frequency range |  | 2595 | - | 2620 | -40 | 1 | 5, 6 |  |
| DC_7_n71 |  |  | E-UTRA Band 4, 5, 12, 13, 14, 17, 26, 30, 66, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 2, 70 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | E-UTRA Band 29 |  | FDL_low | - | FDL_high | -38 | 1 | 5 |  |
|  |  |  | Frequency range |  | 2570 | - | 2575 | 1.6 | 5 | 5, 6, 7 |  |
|  |  |  | Frequency range |  | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |  |
|  |  |  | Frequency range |  | 2595 | - | 2620 | -40 | 1 | 5, 6 |  |
| DC_7_n77 |  |  | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 33, 34, 40, 50, 51, 65, 66, 67, 68, 72, 74, 75, 76 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |  |
|  |  |  | Frequency range |  | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |  |
|  |  |  | Frequency range |  | 2595 | - | 2620 | -40 | 1 | 5, 6 |  |
| DC_7_n78 |  |  | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 33, 34, 40, 50, 51, 65, 66, 67, 68, 72, 74, 75, 76 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |  |
|  |  |  | Frequency range |  | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |  |
|  |  |  | Frequency range |  | 2595 | - | 2620 | -40 | 1 | 5, 6 |  |
| DC_7_n79 |  |  | E-UTRA Band 1, 3, 5, 8, 28, 34, 40, 42, 65, 74 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |  |
|  |  |  | Frequency range |  | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |  |
|  |  |  | Frequency range |  | 2595 | - | 2620 | -40 | 1 | 5, 6 |  |
| DC_8_n1 |  |  | E-UTRA Band 20, 28, 31, 32, 38, 40, 50, 51, 65, 67, 72, 73, 74, 75, 76 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA band 3, 7, 22, 41, 42, 43, 52 NR Band n77, n78, n79 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | E-UTRA Band 1, 8, 34 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  |  |  | E-UTRA band 11, 21 |  | FDL_low | - | FDL_high | -50 | 1 | 12 |  |
|  |  |  | Frequency range |  | 860 | - | 890 | -40 | 1 | 5, 12 |  |
|  |  |  | Frequency range |  | 1880 |  | 1895 | -40 | 1 | 5, 16 |  |
|  |  |  | Frequency range |  | 1895 |  | 1915 | -15.5 | 5 | 5, 7, 16 |  |
|  |  |  | Frequency range |  | 1915 |  | 1920 | +1.6 | 5 | 5, 7, 16 |  |
| DC_8_n2 |  |  | E-UTRA Band 2, 8, 28, 50, 51, 74 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 41, 42, 43 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
| DC_8_n3 |  |  | E-UTRA Band 1, 20, 28, 31, 32, 33, 34, 38, 39, 40, 44, 50, 51, 65, 67, 72, 73, 74, 75, 76 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA band 3, 8 |  | FDL_low | - | FDL_high | -50 | 1 | 2, 5 |  |
|  |  |  | E-UTRA band 11, 21 |  | FDL_low | - | FDL_high | -50 | 1 | 12 |  |
|  |  |  | E-UTRA band 7, 22, 41, 42, 43, 52 NR Band n77, n78, n79 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3.12 |  |
|  |  |  | Frequency range |  | 860 | - | 890 | -40 | 1 | 5. 12 |  |
| DC_8_n7 |  |  | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 10, 12, 13, 14, 17, 20, 22, 26, 27, 28, 29, 30, 31, 32, 33, 34, 40, 42, 43, 45, 50, 51, 52, 65, 66, 67, 68, 69, 72, 73, 74, 75, 76, 85, |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA band 3, 7, 22, 41, 42, 43, 52 NR Band n77, n78, n79 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | E-UTRA Band 8 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  |  |  | E-UTRA Band 11, 21 |  | FDL_low | - | FDL_high | -50 | 1 | 12 |  |
|  |  |  | Frequency range |  | 860 | - | 890 | -40 | 1 | 5, 12 |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 12 |  |
|  |  |  | Frequency range |  | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |  |
|  |  |  | Frequency range |  | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |  |
|  |  |  | Frequency range |  | 2595 | - | 2620 | -40 | 1 | 5, 6 |  |
| DC_7_n80 |  |  | E-UTRA Band 1, 5, 7, 8, 20, 26, 27, 28, 31, 32, 33, 34, 40, 43, 50, 51, 65, 67, 68, 72, 74, 75, 76. NR Band n79 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 3 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  |  |  | E-UTRA Band 22, 42, NR Band n77, n78 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | Frequency range |  | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |  |
|  |  |  | Frequency range |  | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |  |
|  |  |  | Frequency range |  | 2595 | - | 2620 | -40 | 1 | 5, 6 |  |
| DC_8_n20 |  |  | E-UTRA Band 1, 31, 32, 33, 34, 40, 50, 51, 65, 67, 68, 72, 74, 75, 76 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 3, 7, 22, 38, 42, 43, 52, 69 NR band n77, n78 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | E-UTRA Band 8, 20 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  |  |  | Frequency range |  | 758 | - | 788 | -50 | 1 |  |  |
| DC_8_n28 |  |  | E-UTRA Band 20, 31, 34, 38, 40, 72 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA band 3, 7, 22, 41, 42, 43, 50, 51, 65, 73, 74, 75, 76 NR Band n77, n78, n79 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | E-UTRA Band 1 |  | FDL_low | - | FDL_high | -50 | 1 | 2, 9, 11 |  |
|  |  |  | E-UTRA Band 8 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  |  |  | E-UTRA Band 11, 21 |  | FDL_low | - | FDL_high | -50 | 1 | 9, 10, 12 |  |
|  |  |  | Frequency range |  | 470 | - | 694 | -42 | 8 | 5, 17 |  |
|  |  |  | Frequency range |  | 470 | - | 710 | -26.2 | 6 | 14 |  |
|  |  |  | Frequency range |  | 662 | - | 694 | -26.2 | 6 | 5 |  |
|  |  |  | Frequency range |  | 758 | - | 773 | -32 | 1 | 5 |  |
|  |  |  | Frequency range |  | 773 | - | 803 | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 860 | - | 890 | -40 | 1 | 5, 12 |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 9, 12 |  |
| DC_8_n34 |  |  | E-UTRA Band 1, 20, 28, 31, 32, 33, 38, 39, 40, 45, 50, 51, 65, 67, 69,72, 73, 74, 75, 76 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 3, 7, 22, 41, 42, 43, 52 NR Band n78, n79 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | E-UTRA Band 8 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  |  |  | E-UTRA Band 11, 21 |  | FDL_low | - | FDL_high | -50 | 1 | 12 |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 12 |  |
|  |  |  | Frequency range |  | 860 | - | 890 | -40 | 1 | 5, 12 |  |
| DC_8_n39 |  |  | E-UTRA Band 1, 28, 34, 40, 45, 50, 51, 73, 74 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | UTRA Band 22, 41, 42, 52 NR Band n77, n78, n79 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | E-UTRA Band 8 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
| DC_8_n40 |  |  | E-UTRA Band 1, 5, 11, 18, 19, 20, 21, 26, 28, 31, 32, 33, 34, 38, 39,, 45, 50, 51, 65, 67, 68, 69, 72, 73, 74, 75, 76 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 3, 7, 22, 41, 42, 43, 52 NR band n77, n78, n79 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | E-UTRA Band 8 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
| DC_8_n41, DC_8_n81_ULSUP-TDM_n41 |  |  | E-UTRA Band 1, 11, 21, 28, 34, 39, 45, 50, 51, 65, 73, 74 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA band 3, 42, 52 NR Band n77, n78, n79 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | E-UTRA Band 8 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  |  |  | E-UTRA Band 40 |  | FDL_low | - | FDL_high | -40 | 1 |  |  |
|  |  |  | Frequency range |  | 860 | - | 890 | -40 | 1 | 5, 12 |  |
|  |  |  | Frequency range |  | 1884.5 |  | 1915.7 | -41 | 0.3 | 3 |  |
| DC_8_n77 |  |  | E-UTRA Band 1, 20, 28, 31, 32, 33, 34, 38, 39, 40, 44, 45, 50, 51, 65, 67, 68, 69, 72, 73, 74, 75, 76 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA band 3, 7, 41 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | E-UTRA Band 8 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  |  |  | E-UTRA Band 11, 21 |  | FDL_low | - | FDL_high | -50 | 1 | 12 |  |
|  |  |  | Frequency range |  | 860 | - | 890 | -40 | 1 | 5, 12 |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 12 |  |
| DC_8_n78 DC_8_n81_ULSUP-TDM_n78 |  |  | E-UTRA Band 1, 20, 28, 34, 39, 40, 65, 74 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 3, 7, 41 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | E-UTRA Band 8 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  |  |  | E-UTRA Band 11, 21 |  | FDL_low | - | FDL_high | -50 | 1 | 12 |  |
|  |  |  | Frequency range |  | 860 | - | 890 | -40 | 1 | 5, 12 |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 12 |  |
| DC_8_n79 DC_8_n81_ULSUP-TDM_n79 |  |  | E-UTRA Band 1, 8, 28, 34, 39, 40, 65, 74 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 3,41,42 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | E-UTRA Band 11, 21 |  | FDL_low | - | FDL_high | -50 | 1 | 12 |  |
|  |  |  | Frequency range |  | 860 | - | 890 | -40 | 1 | 5, 12 |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
| DC_8_n80 |  |  | E-UTRA Band 1, 20, 28, 31, 32, 33, 34, 38, 39, 40, 45, 50, 51, 65, 67, 68, 69, 72, 73, 74, 75, 76 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 3, 8 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  |  |  | E-UTRA Band 3, 7, 22, 41, 42, 43, 52 NR Band n77, n78, n79 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | E-UTRA Band 11, 21 |  | FDL_low | - | FDL_high | -50 | 1 | 13 |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
| DC_8A_93A_ULSUP-TDM, DC_8A_94A_ULSUP-TDM |  |  | E-UTRA Band 1, 20, 28, 31, 32, 33, 34, 38, 39, 40, 45, 50, 51, 52, 65, 67, 68, 69, 72, 73, 74, 75, 76 |  | FDL_low | - | FDL_high | -50 -50 | 1 1 |  |  |
|  |  |  | E-UTRA band 3, 7, 22, 41, 42, 43 NR Band n77, n78 |  | FDL_low | - | FDL_high | -50 | 1 | 2, 5 |  |
|  |  |  | E-UTRA 8 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
| DC_11_n1 |  |  | E-UTRA Band 1, 11, 18, 19, 21, 27, 28, 40, 42, 65 NR Band n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | E-UTRA Band 3, 34 | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  |  |  | Frequency range | 945 | - | 960 | -50 | 1 |  |  |
|  |  |  | Frequency range | 1880 | - | 1895 | -40 | 1 | 5, 16 |  |
|  |  |  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 5, 7, 16 |  |
|  |  |  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 5, 7, 16 |  |
|  |  |  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |  |
|  |  |  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |  |
| DC_11_n3 |  |  | E-UTRA Band 1, 11, 18, 19, 21, 28, 34, 40, 65 NR band n79 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA band 3 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  |  |  | E-UTRA Band 42 NR band n77, n78 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | Frequency range |  | 945 | - | 960 | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
|  |  |  | Frequency range |  | 2545 | - | 2575 | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 2595 | - | 2645 | -50 | 1 |  |  |
| DC_11_n28 |  |  | E-UTRA Band 3, 18, 19, 34, 40 NR band n79 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA band 42, 65, 74 NR band n77, n78 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | E-UTRA band 1 |  | FDL_low | - | FDL_high | -50 | 1 | 9, 11 |  |
|  |  |  | E-UTRA Band 11, 21 |  | FDL_low | - | FDL_high | -50 | 1 | 9, 10 |  |
|  |  |  | Frequency range |  | 470 | - | 710 | -26.2 | 6 | 14 |  |
|  |  |  | Frequency range |  | 773 | - | 803 | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 945 | - | 960 | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 9 |  |
|  |  |  | Frequency range |  | 2545 | - | 2575 | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 2595 | - | 2645 | -50 | 1 |  |  |
| DC_11_n41 |  |  | E-UTRA Band 1, 3, 11, 18, 19, 21, 28, 34, 42, 65 NR Band n77, n78 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | NR Band n79 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | Frequency range |  | 945 | - | 960 | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
| DC_11_n77 |  |  | E-UTRA Band 1, 3, 18, 19, 28, 34, 40, 65 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 945 | - | 960 | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
|  |  |  | Frequency range |  | 2545 | - | 2575 | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 2595 | - | 2645 | -50 | 1 |  |  |
| DC_11_n78 |  |  | E-UTRA Band 1, 3, 18, 19, 28, 34, 40, 65 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 945 | - | 960 | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
|  |  |  | Frequency range |  | 2545 | - | 2575 | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 2595 | - | 2645 | -50 | 1 |  |  |
| DC_11_n79 |  |  | E-UTRA Band 1, 3, 18, 19, 28, 34, 40, 42, 65 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 945 | - | 960 | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
|  |  |  | Frequency range |  | 2545 | - | 2575 | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 2595 | - | 2645 | -50 | 1 |  |  |
| DC_12_n2 |  |  | E-UTRA Band 5, 13, 14, 17, 24, 26, 27, 30, 41, 50, 53, 71, 74 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 12, 25, 85 |  | FDL_low | - | FDL_high | -50 | 1 | 3 |  |
|  |  |  | E-UTRA Band 2 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  |  |  | E-UTRA Band 4, 51, 66, 70, NR Band n77 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
| DC_12_n5 |  |  | E-UTRA Band 2, 5, 13, 14, 17, 24, 25, 26, 30, 43 50, 71, 74 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Bands 4, 41, 42, 48, 51, 66, 70, NR Band n77 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | E-UTRA Band 12, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
| DC_12_n66 |  |  | E-UTRA Band 2, 5, 13, 14, 17, 25, 26, 27, 30, 41, 53, 71, 74 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 4, 48, 50, 51, 66, 70 NR Band n77 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | E-UTRA Band 12, 85 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
| DC_12_n7 |  |  | E-UTRA Band 2, 5, 7, 13, 14, 17, 26, 27, 30, 74 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 4, 50, 51,66 NR Band n78 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | E-UTRA Band 12, 85 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  |  |  | Frequency range |  | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |  |
|  |  |  | Frequency range |  | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |  |
|  |  |  | Frequency range |  | 2595 | - | 2620 | -40 | 1 | 5, 6 |  |
| DC_12_n25 |  |  | E-UTRA Band 5, 13, 14, 17, 24, 26, 27, 30, 41, 53, 71 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 4, 48, 66, 70. NR Band n77 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | E-UTRA Band 2, 12, 25, 85 |  | FDL_low | - | FDL_high | -50 | 1 | 15 |  |
| DC_12_n30 |  |  | E-UTRA Band 2, 5, 13, 14, 17, 24, 25, 26, 27, 30, 41, 48, 53, 71 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 4, 66, 70, NR Band n77 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | E-UTRA Band 12, 85 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
| DC_12_n38 |  |  | E-UTRA Band 2, 5, 13. 14. 17, 27, 30, 74 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 4, 50, 51, 66 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | E-UTRA band 12, 85 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
| DC_12_n41 |  |  | E-UTRA Band 2, 5, 13, 14, 17, 24, 25, 26, 27, 30, 71, 74 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA band 4, 48, 50, 51, 66, 70 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | E-UTRA band 12, 85 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
| DC_12_n77 |  |  | E-UTRA Band 2, 5, 7. 13, 17, 24, 25, 26, 27, 30, 41, 53, 70, 71 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 4, 66 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | E-UTRA band 12, 85 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
| DC_12_n78 |  |  | E-UTRA Band 2, 5, 7. 13, 17, 25, 26, 41, 71 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 4, 66 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | E-UTRA band 12 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
| DC_13_n2 |  |  | E-UTRA Band 4, 5,12,13,17, 26, 29, 41, 48, 66, 70, 71 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 2,14, 25 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  |  |  | E-UTRA Band 30 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | Frequency range |  | 769 | - | 775 | -35 | 0.00625 | 5 |  |
|  |  |  | Frequency range |  | 799 | - | 805 | -35 | 0.00625 | 5 |  |
| DC_13_n5 |  |  | E-UTRA Band 2, 4, 5, 12, 13, 17, 25, 29, 48, 50, 51, 66, 70, 71, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 26 |  | 859 | - | 869 | -27 | 1 |  |  |
|  |  |  | E-UTRA Band 24, 30, 41, 53 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | E-UTRA Band 14 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  |  |  | Frequency range |  | 769 | - | 775 | -35 | 0.00625 | 5 |  |
|  |  |  | Frequency range |  | 799 | - | 805 | -35 | 0.00625 | 5 |  |
| DC_13_n7 |  |  | E-UTRA Band 2, 4, 5, 7, 12, 13, 17,25，26, 27, 29, 50, 51, 66，74, 85 NR Band n78 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 30 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | E-UTRA Band 14 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  |  |  | Frequency range |  | 769 | - | 775 | -35 | 0.00625 | 5 |  |
|  |  |  | Frequency range |  | 799 | - | 805 | -35 | 0.00625 | 5 |  |
|  |  |  | Frequency range |  | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |  |
|  |  |  | Frequency range |  | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |  |
|  |  |  | Frequency range |  | 2595 | - | 2620 | -40 | 1 | 5, 6 |  |
| DC_13_n25 |  |  | E-UTRA Band 4, 5,12,13,17, 26, 29, 41, 48, 66, 70, 71 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 2,14 NR Band n25 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  |  |  | E-UTRA Band 30 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | Frequency range |  | 769 | - | 775 | -35 | 0.00625 | 5 |  |
|  |  |  | Frequency range |  | 799 | - | 805 | -35 | 0.00625 | 5 |  |
| DC_13_n48 |  |  | E-UTRA Band 2, 4, 5, 12, 13, 17, 25, 26, 27, 29, 41, 50, 51, 66, 70, 71, 74, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 14 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  |  |  | E-UTRA Band 24, 30 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | Frequency range |  | 769 | - | 775 | -35 | 0.00625 | 5 |  |
|  |  |  | Frequency range |  | 799 | - | 805 | -35 | 0.00625 | 5 |  |
| DC_13_n66 |  |  | E-UTRA Band 2, 4, 5, 12, 13, 17, 25, 26, 27, 29, 41, 50, 51, 53, 66, 70, 71, 74, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 14 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  |  |  | E-UTRA Band 30, 48, NR Band n77 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | Frequency range |  | 769 | - | 775 | -35 | 0.00625 | 5 |  |
|  |  |  | Frequency range |  | 799 | - | 803 | -35 | 0.00625 | 5 |  |
| DC_13_n71 |  |  | E-UTRA Band 4, 5, 12, 13, 17, 26, 48, 66, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 2, 24, 25, 30, 41, 70, NR Band n77 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | E-UTRA Band 29 |  | FDL_low | - | FDL_high | -38 | 1 | 5 |  |
|  |  |  | E-UTRA Band 14, 71 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  |  |  | Frequency range |  | 769 | - | 775 | -35 | 0.00625 | 5 |  |
|  |  |  | Frequency range |  | 799 | - | 805 | -35 | 0.00625 | 5 |  |
| DC_13_n77 |  |  | E-UTRA Band 2, 4, 5, 10, 12, 13, 17, 25, 26, 29, 41, 66, 70, 71 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 14 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  |  |  | E-UTRA Band 24, 30 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | Frequency range |  | 769 | - | 775 | -35 | 0.00625 | 5 |  |
|  |  |  | Frequency range |  | 799 | - | 805 | -35 | 0.00625 | 5 |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
| DC_14_n2 |  |  | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 27, 29, 30, 41, 48, 53, 66, 70, 71, 85, NR Band n77 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA band 2, 25 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | Frequency range |  | 769 | - | 775 | -35 | 0.00625 | 5 |  |
|  |  |  | Frequency range |  | 799 | - | 805 | -35 | 0.00625 | 5 |  |
| DC_14_n5 |  |  | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 24, 25, 26, 29, 30, 48, 66, 70, 71, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA band 41, 53 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
| DC_14_n30 |  |  | E-UTRA Band 2, 4, 5,12, 13, 14, 17, 24, 25, 26, 27, 29, 30, 41, 48, 53, 66, 70, 71, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | NR Band n77 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | Frequency range |  | 769 | - | 775 | -35 | 0.00625 | 5 |  |
|  |  |  | Frequency range |  | 799 | - | 805 | -35 | 0.00625 | 5 |  |
| DC_14_n77 |  |  | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 27, 29, 30, 41, 53, 66, 70, 71, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 769 | - | 775 | -35 | 0.00625 | 5 |  |
|  |  |  | Frequency range |  | 799 | - | 805 | -35 | 0.00625 | 5 |  |
| DC_14_n66 |  |  | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 25, 26, 27, 29, 30, 41, 53, 66, 70, 71, 85, NR Band n77 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA band 48 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | Frequency range |  | 769 | - | 775 | -35 | 0.00625 | 5 |  |
|  |  |  | Frequency range |  | 799 | - | 805 | -35 | 0.00625 | 5 |  |
| DC_18_n28 |  |  | E-UTRA Band 11, 21 |  | FDL_low | - | FDL_high | -50 | 1 | 9, 10 |  |
|  |  |  | E-UTRA Band 1, 65 |  | FDL_low | - | FDL_high | -50 | 1 | 2, 9, 11 |  |
|  |  |  | E-UTRA Band 42, 43 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | NR Band n77, n78, n79 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | E-UTRA Band 3, 34, 40 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 470 | - | 710 | -26.2 | 6 | 14 |  |
|  |  |  | Frequency range |  | 758 | - | 773 | -32 | 1 | 5 |  |
|  |  |  | Frequency range |  | 773 | - | 799 | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 799 | - | 803 | -40 | 1 | 5 |  |
|  |  |  | Frequency range |  | 860 | - | 890 | -40 | 1 |  |  |
|  |  |  | Frequency range |  | 945 | - | 960 | -50 | 1 | 5 |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
|  |  |  | Frequency range |  | 2545 | - | 2575 | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 2595 | - | 2645 | -50 | 1 |  |  |
| DC_18_n41 |  |  | E-UTRA Band 1, 3, 11, 19, 21, 28, 34, 42, 65 NR Band n79 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | NR Band n77, n78 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | Frequency range |  | 945 | - | 960 | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
| DC_13_n78 |  |  | E-UTRA Band 2, 5, 7, 12, 13, 25, 26, 41, 66 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 769 | - | 775 | -35 | 0.00625 | 5 |  |
|  |  |  | Frequency range |  | 799 | - | 805 | -35 | 0.00625 | 5 |  |
| DC_18_n3 |  |  | E-UTRA Band 1, 3, 11, 18, 19, 21, 28, 34, 40, 65 NR Band n79 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 42 NR Band n77, n78 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | Frequency range |  | 945 | - | 960 | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
|  |  |  | Frequency range |  | 2545 | - | 2575 | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 2595 | - | 2645 | -50 | 1 |  |  |
| DC_18_n77 |  |  | E-UTRA Band 1, 3, 11, 21, 28, 34, 40, 65, 74 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 945 | - | 960 | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
|  |  |  | Frequency range |  | 2545 | - | 2575 | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 2595 | - | 2645 | -50 | 1 |  |  |
| DC_18_n78 |  |  | E-UTRA Band 1, 3, 11, 21, 28, 34, 40, 65, 74 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 945 | - | 960 | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
|  |  |  | Frequency range |  | 2545 | - | 2575 | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 2595 | - | 2645 | -50 | 1 |  |  |
| DC_18_n79 |  |  | E-UTRA Band 1, 3, 11, 21, 28, 34, 40, 42, 65, 74 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 945 | - | 960 | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
|  |  |  | Frequency range |  | 2545 | - | 2575 | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 2595 | - | 2645 | -50 | 1 |  |  |
| DC_19_n1 |  |  | E-UTRA Band 1, 11, 21, 28, 40, 42, 65, 74 NR Band n79 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | NR Band n77, n78 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | E-UTRA Band 3, 34 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  |  |  | Frequency range |  | 945 | - | 960 | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 1880 | - | 1895 | -40 | 1 | 5, 16 |  |
|  |  |  | Frequency range |  | 1895 | - | 1915 | -15.5 | 5 | 5, 7, 16 |  |
|  |  |  | Frequency range |  | 1915 | - | 1920 | +1.6 | 5 | 5, 7, 16 |  |
|  |  |  | Frequency range |  | 2545 | - | 2575 | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 2595 | - | 2645 | -50 | 1 |  |  |
| DC_19_n77 |  |  | E-UTRA Band 1, 3, 11, 21, 28, 34, 40, 65, 74 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 945 | - | 960 | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
|  |  |  | Frequency range |  | 2545 | - | 2575 | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 2595 | - | 2645 | -50 | 1 |  |  |
| DC_19_n78 |  |  | E-UTRA Band 1, 3, 11, 21, 28, 34, 40, 65, 74 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 945 | - | 960 | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
|  |  |  | Frequency range |  | 2545 | - | 2575 | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 2595 | - | 2645 | -50 | 1 |  |  |
| DC_19_n79 |  |  | E-UTRA Band 1, 3, 11, 21, 28, 34, 40, 42, 65, 74 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 945 | - | 960 | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
|  |  |  | Frequency range |  | 2545 | - | 2575 | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 2595 | - | 2645 | -50 | 1 |  |  |
| DC_20_n1 |  |  | E-UTRA Band 1, 3, 7, 8, 20, 22, 31, 32, 34, 40, 43, 50, 51, 65, 67, 68, 72, 75, 76 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 38, 42, 69 NR Band n77, n78 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | Frequency range |  | 758 | - | 788 | -50 | 1 |  |  |
| DC_20_n3 |  |  | E-UTRA Band 1, 7, 8, 31, 32, 33, 34, 40, 43, 50, 51, 65, 67, 72, 74, 75, 76 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 20 E-UTRA Band 3 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  |  |  | E-UTRA Band 22, 38, 42, 52 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | Frequency range |  | 758 | - | 788 | -50 | 1 |  |  |
| DC_20_n7 |  |  | E-UTRA Band 1, 3, 7, 8, 22, 31, 32, 33, 34, 40, 43, 50, 51, 65, 67, 68, 72, 74, 75, 76 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 42, 52 NR band n78, n77 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | E-UTRA Band 20 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
| DC_20_n8 |  |  | E-UTRA Band 1, 28, 31, 32, 34, 65, 75, 76 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 3, 7, 22, 38, 42, 43 NR Band n78 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
| DC_20_n38 |  |  | E-UTRA Band 1, 3, 8, 22, 31, 32, 33, 34, 40, 43, 50, 51, 65, 67, 68, 72, 74, 75, 76 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 42, 52 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | E-UTRA Band 20 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
| DC_20_n41 |  |  | E-UTRA Band 1, 2, 4, 24, 25, 30, 31, 32, 33, 34, 39, 43, 48, 50, 51, 65, 66, 70, 72, 73, 74, 75, 76 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 3, 8, 12, 13, 14, 17, 42, 44, 45, 52, 67, 68, 71, 85 NR Band n77, n78, n79 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | E-UTRA Band 40 |  | FDL_low | - | FDL_high | -40 | 1 |  |  |
|  |  |  | Frequency range |  | 758 | - | 788 | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 9, 11, 21 |  | FDL_low | - | FDL_high | -50 | 1 | 19 |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 19 |  |
| DC_20_n28 DC_20_n83 |  |  | E-UTRA Band 3, 7, 8, 31, 34 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 1, 22, 32, 38, 42, 43, 65, 75, 76 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
| DC_20_n50 |  |  | E-UTRA Band 2, 3, 7, 12, 17, 31, 33, 39, 43, 48, 65, 67, 68, 72, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 1, 4, 5, 8, 13, 34, 38, 40, 41, 42, 52, 69 NR Band n77, n78 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | Frequency range |  | 758 | - | 788 | -50 | 1 |  |  |
| DC_20_n51 |  |  | E-UTRA Band 1, 3, 4, 8, 17, 22, 28, 29, 31, 40, 43, 48, 65, 66, 68, 72 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 20 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  |  |  | Frequency range |  | 758 | - | 788 | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 2, 7, 25, 32, 33, 34, 35, 36, 37, 38, 39, 41, 42, 46, 69, 70 NR Band n77, n78, n79, |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
| DC_20_n77 |  |  | E-UTRA Band 1, 3, 7, 8, 31, 32, 33, 34, 40, 50, 51, 65, 67, 68, 72, 74, 75, 76 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 20 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  |  |  | E-UTRA Band 38, 69 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
| DC_20_n78, DC_20_n82_ULSUP-TDM_n78 |  |  | E-UTRA Band 1, 3, 7, 8, 31, 32, 33, 34, 40, 50, 51, 65, 67, 68, 72, 74, 75, 76 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 20 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  |  |  | E-UTRA Band 38, 69 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
| DC_20_n80 |  |  | E-UTRA Band 1, 7, 8, 28, 31, 32, 33, 34, 40, 43, 50, 51, 65, 67, 68, 72, 74, 75, 76. NR Band n79 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 3, 20 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  |  |  | E-UTRA Band 22, 42, NR Band n77, n78 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
| DC_20A_91A_ULSUP-TDM, DC_20A_92A_ULSUP-TDM |  |  | E-UTRA Band 1, 3, 7, 8, 22, 31, 32, 33, 34, 40, 43, 50, 51, 65, 67, 68, 72, 74, 75, 76 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 20 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  |  |  | E-UTRA Band 38, 42, 69, NR Band n77, n78 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | Frequency range |  | 758 | - | 788 | -50 | 1 |  |  |
| DC_21_n1 |  |  | E-UTRA Band 1, 18, 19, 28, 40, 42, 65 NR Band n78, n79 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | NR Band n77 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | E-UTRA Band 3, 34 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  |  |  | Frequency range |  | 945 | - | 960 | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 1880 | - | 1895 | -40 | 1 | 5, 16 |  |
|  |  |  | Frequency range |  | 1895 | - | 1915 | -15.5 | 5 | 5, 7, 16 |  |
|  |  |  | Frequency range |  | 1915 | - | 1920 | +1.6 | 5 | 5, 7, 16 |  |
|  |  |  | Frequency range |  | 2545 | - | 2575 | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 2595 | - | 2645 | -50 | 1 |  |  |
| DC_21_n28 |  |  | E-UTRA Band 1, 42, 65, NR Band n77, n78 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | E-UTRA Band 1 |  | FDL_low | - | FDL_high | -50 | 1 | 9, 11 |  |
|  |  |  | E-UTRA Band 3, 18, 19, 34, 40 NR Band n79 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 470 | - | 694 | -42 | 8 | 5, 17 |  |
|  |  |  | Frequency range |  | 470 | - | 710 | -26.2 | 6 | 14 |  |
|  |  |  | Frequency range |  | 662 | - | 694 | -26.2 | 6 | 5 |  |
|  |  |  | Frequency range |  | 758 | - | 773 | -32 | 1 | 5 |  |
|  |  |  | Frequency range |  | 773 | - | 803 | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 945 | - | 960 | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 9 |  |
|  |  |  | Frequency range |  | 2545 | - | 2575 | -50 | 1 |  |  |
| DC_21_n77 |  |  | E-UTRA Band 1, 3, 18, 19, 21, 28, 34, 40, 65 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 945 | - | 960 | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
|  |  |  | Frequency range |  | 2545 | - | 2575 | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 2595 | - | 2645 | -50 | 1 |  |  |
| DC_21_n78 |  |  | E-UTRA Band 1, 3, 18, 19, 21, 28, 34, 40, 65 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 945 | - | 960 | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
|  |  |  | Frequency range |  | 2545 | - | 2575 | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 2595 | - | 2645 | -50 | 1 |  |  |
| DC_21_n79 |  |  | E-UTRA Band 1, 3, 18, 19, 21, 28, 34, 40, 42, 65 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 945 | - | 960 | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
|  |  |  | Frequency range |  | 2545 | - | 2575 | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 2595 | - | 2645 | -50 | 1 |  |  |
| DC_25_n41 |  |  | E-UTRA Band 4, 5, 12, 13 , 14, 17, 24, 26, 27, 28, 29, 30, 42, 45, 48, 66, 70, 71 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 2, 25 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  |  |  | NR Band n77 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
| DC_25_n77 |  |  | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 25, 26, 29, 30, 41, 65, 66, 70, 71 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
| DC_25_n78 |  |  | E-UTRA Band 2, 5, 7, 12, 13, 25, 26, 28, 41, 66 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
| DC_26_n25 |  |  | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 29, 30, 42, 48, 66, 70, 71, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 2, 25 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  |  |  | E-UTRA Band 41, 43, 53 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
| DC_26_n41 |  |  | E-UTRA Band 1, 2, 3, 4, 5, 11, 12, 13 , 14, 17, 18, 19, 21, 24, 25, 26, 29, 30, 31, 34, 39, 42, 43, 48, 50, 51, 65, 66, 70, 71, 74 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 1884.5 |  | 1915.7 | -41 | 0.3 | 3 |  |
|  |  |  | Frequency range |  | 703 | - | 799 | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 799 | - | 803 | -40 | 1 | 5 |  |
|  |  |  | Frequency range |  | 945 | - | 960 | -50 | 1 |  |  |
| DC_26_n77 |  |  | E-UTRA Band 1, 3, 5, 11, 18, 19, 21, 26, 34, 39, 40, 65, 74 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 41 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | Frequency range |  | 703 | - | 799 | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 799 | - | 803 | -40 | 1 | 5 |  |
|  |  |  | Frequency range |  | 945 | - | 960 | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
|  |  |  | Frequency range |  | 2545 | - | 2575 | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 2595 | - | 2645 | -50 | 1 |  |  |
| DC_26_n78 |  |  | E-UTRA Band 1, 3, 5, 11, 18, 19, 21, 26, 34, 39, 40, 65, 74 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 41 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | Frequency range |  | 703 | - | 799 | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 799 | - | 803 | -40 | 1 | 5 |  |
|  |  |  | Frequency range |  | 945 | - | 960 | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
|  |  |  | Frequency range |  | 2545 | - | 2575 | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 2595 | - | 2645 | -50 | 1 |  |  |
| DC_26_n79 |  |  | E-UTRA Band 1, 3, 5, 11, 18, 19, 21, 26, 34, 39, 40, 41, 65, 74 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 41 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | Frequency range |  | 703 | - | 799 | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 799 | - | 803 | -40 | 1 | 5 |  |
|  |  |  | Frequency range |  | 945 | - | 960 | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
|  |  |  | Frequency range |  | 2545 | - | 2575 | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 2595 | - | 2645 | -50 | 1 |  |  |
| DC_28_n1 |  |  | E-UTRA Band 5, 7, 8, 18, 19, 20, 26, 27, 31, 38, 40, 41, 72, 73 NR band n79 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 1, 22, 32, 42, 43, 50, 51, 52, 65, 74, 75, 76 NR band n77, n78 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | E-UTRA Band 1 |  | FDL_low | - | FDL_high | -50 | 1 | 9, 10 |  |
|  |  |  | E-UTRA Band 11, 21 |  | FDL_low | - | FDL_high | -50 | 1 | 9, 11 |  |
|  |  |  | E-UTRA band 3, 34 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  |  |  | Frequency range |  | 470 | - | 694 | -42 | 8 | 5, 17 |  |
|  |  |  | Frequency range |  | 470 | - | 710 | -26.2 | 6 | 14 |  |
|  |  |  | Frequency range |  | 662 | - | 694 | -26.2 | 6 | 5 |  |
|  |  |  | Frequency range |  | 758 | - | 773 | -32 | 1 | 5 |  |
|  |  |  | Frequency range |  | 773 | - | 803 | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 1880 | - | 1895 | -40 | 1 | 5,16 |  |
|  |  |  | Frequency range |  | 1895 | - | 1915 | -15.5 | 5 | 5, 7, 16 |  |
|  |  |  | Frequency range |  | 1915 | - | 1920 | +1.6 | 5 | 5, 7, 16 |  |
| DC_28_n2 |  |  | E-UTRA Band 5, 26, 27, 41 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 4, 10, 42, 43, 50, 51, 66, 74 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | E-UTRA band 2, 25 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 11, 21 |  | FDL_low | - | FDL_high | -50 | 1 | 9, 11 |  |
|  |  |  | E-UTRA Band 1, 65 |  | FDL_low | - | FDL_high | -50 | 1 | 9, 10 |  |
|  |  |  | Frequency range |  | 470 | - | 710 | -26.2 | 6 | 14 |  |
|  |  |  | Frequency range |  | 758 | - | 773 | -32 | 1 | 5 |  |
|  |  |  | Frequency range |  | 773 | - | 803 | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 2570 | - | 2575 | 1.6 | 5 | 5, 6, 7 |  |
|  |  |  | Frequency range |  | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |  |
|  |  |  | Frequency range |  | 2595 | - | 2620 | -40 | 1 | 5, 6 |  |
| DC_28_n3 |  |  | E-UTRA Band 1, 22, 42, 43, 50, 51, 65, 74, 75, 76, NR Band n77, n78 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | E-UTRA Band 1 |  | FDL_low | - | FDL_high | -50 | 1 | 9, 11 |  |
|  |  |  | E-UTRA Band 3, 5, 7, 8, 18, 19, 20, 26, 27, 31, 34, 38, 40, 41, 72, 73 NR Band n79 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 11, 21 |  | FDL_low | - | FDL_high | -50 | 1 | 9, 10 |  |
|  |  |  | Frequency range |  | 470 | - | 710 | -26.2 | 6 | 14 |  |
|  |  |  | Frequency range |  | 758 | - | 773 | -32 | 1 | 5 |  |
|  |  |  | Frequency range |  | 773 | - | 803 | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 9 |  |
| DC_28_n5 |  |  | E-UTRA Band 2, 3, 5, 7, 8, 14, 18, 19, 24, 25, 26, 28, 30, 31, 34, 38, 40, 70, 71 |  | FDL_low | - | FDL_high | -50 |  |  |  |
|  |  |  | E-UTRA Band 4, 22, 32, 41, 42, 43, 45, 48, 50, 51, 52, 65, 66, 73, 74, 75, 76 NR Band n77, n78, n79 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | E-UTRA Band 1 |  | FDL_low | - | FDL_high | -50 | 1 | 2, 9, 11 |  |
|  |  |  | E-UTRA Band 11, 21 |  | FDL_low | - | FDL_high | -50 | 1 | 2, 9, 10 |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 9 |  |
|  |  |  | Frequency range |  | 470 | - | 694 | -42 | 8 | 5, 17 |  |
|  |  |  | Frequency range |  | 470 | - | 710 | -26.2 | 6 | 14 |  |
|  |  |  | Frequency range |  | 662 | - | 694 | -26.2 | 6 | 5 |  |
|  |  |  | Frequency range |  | 758 | - | 773 | -32 | 1 | 5 |  |
|  |  |  | Frequency range |  | 773 | - | 803 | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 773 | - | 803 | -50 | 1 |  |  |
| DC_28_n7 |  |  | E-UTRA Band 2, 3, 5, 8, 20, 26, 27, 31, 34, 40, 72 NR band n7 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 4, 22, 32, 42, 43, 50, 51, 52, 65, 66, 74, 75, 76 NR band n77, n78 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | E-UTRA band 1 |  | FDL_low | - | FDL_high | -50 | 1 | 2, 9, 10 |  |
|  |  |  | Frequency range |  | 758 | - | 773 | -32 | 1 | 5 |  |
|  |  |  | Frequency range |  | 773 | - | 803 | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |  |
|  |  |  | Frequency range |  | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |  |
|  |  |  | Frequency range |  | 2595 | - | 2620 | -40 | 1 | 5, 6 |  |
| DC_28_n8 |  |  | E-UTRA Band 20, 31, 34, 38, 40, 72 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA band 3, 7, 22, 41, 42, 43, 50, 51, 52, 65, 73, 74, 75, 76 NR Band n77, n78, n79 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | E-UTRA Band 8 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  |  |  | E-UTRA Band 11, 21 |  | FDL_low | - | FDL_high | -50 | 1 | 9, 10 |  |
|  |  |  | E-UTRA Band 1 |  | FDL_low | - | FDL_high | -50 | 1 | 9, 11 |  |
|  |  |  | Frequency range |  | 470 | - | 694 | -42 | 8 | 5, 17 |  |
|  |  |  | Frequency range |  | 470 | - | 710 | -26.2 | 6 | 14 |  |
|  |  |  | Frequency range |  | 662 | - | 694 | -26.2 | 6 | 5 |  |
|  |  |  | Frequency range |  | 758 | - | 773 | -32 | 1 | 5 |  |
|  |  |  | Frequency range |  | 773 | - | 803 | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 9 |  |
| DC_28_n40 |  |  | E-UTRA Band 3, 5, 7, 8, 18, 19, 20, 26, 27, 28,31, 34, 38, 41, 72 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA band 1, 11, 21, 22, 32, 42, 43, 50, 51, 52, 65, 73, 74, 75, 76 NR Band, n77, n78, n79 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
| DC_28_n41 DC_28_n83_ULSUP-TDM_n41 |  |  | E-UTRA Band 4, 14, 18, 19, 20, 26, 27, 39, 42, 43, 48, 50, 51, 52, 65, 66, 71, 73 NR Band n77, n78, n79 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | E-UTRA Band 1 |  | FDL_low | - | FDL_high | -50 | 1 | 2, 9, 11 |  |
|  |  |  | E-UTRA Band 2, 3, 5, 8, 24, 25, 30, 31, 34, 70, 72 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 11, 21, 74, 75, 76 |  | FDL_low | - | FDL_high | -50 | 1 | 2, 9, 10 |  |
|  |  |  | E-UTRA Band 40 |  | FDL_low | - | FDL_high | -40 | 1 |  |  |
|  |  |  | Frequency range |  | 470 | - | 694 | -42 | 8 | 5, 17 |  |
|  |  |  | Frequency range |  | 470 | - | 710 | -26.2 | 6 | 14 |  |
|  |  |  | Frequency range |  | 662 | - | 694 | -26.2 | 6 | 5 |  |
|  |  |  | Frequency range |  | 758 | - | 773 | -32 | 1 | 5 |  |
|  |  |  | Frequency range |  | 773 | - | 803 | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 9 |  |
| DC_28_n50 |  |  | E-UTRA Band 4, 40, 42, 43, 48, 65, 66, 73 NR Band n77, n78, n79 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | E-UTRA Band 1 |  | FDL_low | - | FDL_high | -50 | 1 | 2, 9, 10 |  |
|  |  |  | E-UTRA Band 2, 3, 5, 7, 8, 18, 19, 25, 26, 27, 31, 34, 38, 39, 41, 72 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 470 | - | 694 | -42 | 8 | 5, 17 |  |
|  |  |  | Frequency range |  | 470 | - | 710 | -26.2 | 6 | 14 |  |
|  |  |  | Frequency range |  | 662 | - | 694 | -26.2 | 6 | 5 |  |
|  |  |  | Frequency range |  | 758 | - | 773 | -32 | 1 | 5 |  |
|  |  |  | Frequency range |  | 773 | - | 803 | -50 | 1 |  |  |
| DC_28_n51 |  |  | E-UTRA Band 2, 3, 5, 7, 8, 25, 26, 31, 34, 38, 40, 41, 72 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 4, 20, 22, 24, 32, 42, 43, 45, 46, 65, 66, 71, 73 NR band n78, n79 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | E-UTRA Band 1 |  | FDL_low | - | FDL_high | -50 | 1 | 2, 9, 10 |  |
|  |  |  | Frequency range |  | 470 | - | 694 | -42 | 8 | 5, 17 |  |
|  |  |  | Frequency range |  | 470 | - | 710 | -26.2 | 6 | 14 |  |
|  |  |  | Frequency range |  | 662 | - | 694 | -26.2 | 6 | 5 |  |
|  |  |  | Frequency range |  | 758 | - | 773 | -32 | 1 | 5 |  |
|  |  |  | Frequency range |  | 773 | - | 803 | -50 | 1 |  |  |
| DC_28_n66 |  |  | E-UTRA Band 2, 5, 7, 25, 26, 27, 38, 41 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 4, 10, 42, 43, 50, 51, 66, 74, NR band n77, n78 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | Frequency range |  | 470 | - | 694 | -42 | 8 | 5, 17 |  |
|  |  |  | Frequency range |  | 470 | - | 710 | -26.2 | 6 | 14 |  |
|  |  |  | Frequency range |  | 662 | - | 694 | -26.2 | 6 | 5 |  |
|  |  |  | Frequency range |  | 758 | - | 773 | -32 | 1 | 5 |  |
|  |  |  | Frequency range |  | 773 | - | 803 | -50 | 1 |  |  |
| DC_28_n77 |  |  | E-UTRA Band 3, 5, 7, 8, 18, 19, 20, 26, 34, 39, 40, 41 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 1, 65, 74 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | E-UTRA Band 1 |  | FDL_low | - | FDL_high | -50 | 1 | 9, 11 |  |
|  |  |  | E-UTRA Band 11, 21 |  | FDL_low | - | FDL_high | -50 | 1 | 9, 10 |  |
|  |  |  | Frequency range |  | 758 | - | 773 | -32 | 1 |  |  |
|  |  |  | Frequency range |  | 773 | - | 803 | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 9 |  |
| DC_28_n78 DC_28_n83_ULSUP-TDM_n78 |  |  | E-UTRA Band 3, 5, 7, 8, 18, 19, 20, 26, 34, 39, 40, 41 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 1, 65, 74 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | E-UTRA Band 1 |  | FDL_low | - | FDL_high | -50 | 1 | 9, 11 |  |
|  |  |  | E-UTRA Band 11, 21 |  | FDL_low | - | FDL_high | -50 | 1 | 9, 10 |  |
|  |  |  | Frequency range |  | 758 | - | 773 | -32 | 1 |  |  |
|  |  |  | Frequency range |  | 773 | - | 803 | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 9 |  |
| DC_28_n79 |  |  | E-UTRA Band 3, 5, 8, 18, 19, 34, 39, 40, 41 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 1, 42, 65, 74 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | E-UTRA Band 1 |  | FDL_low | - | FDL_high | -50 | 1 | 9, 11 |  |
|  |  |  | E-UTRA Band 11, 21 |  | FDL_low | - | FDL_high | -50 | 1 | 9, 10 |  |
|  |  |  | Frequency range |  | 758 | - | 773 | -32 | 1 |  |  |
|  |  |  | Frequency range |  | 773 | - | 803 | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 9 |  |
| DC_30_n2 |  |  | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 41, 42, 48, 50, 51, 53, 66, 70, 71, 74, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 25 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  |  |  | E-UTRA Band 2 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  |  |  | E-UTRA Band 43, NR Band n77 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
| DC_30_n5 |  |  | E-UTRA Band 2, 4, 5, 7, 12, 13, 14, 17, 24, 25, 26, 29, 30, 38, 48, 66, 70, 71, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 41, 53 NR Band n77 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
| DC_30_n66 |  |  | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 27, 29, 30, 38, 41, 66, 70, 71 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 48, NR Band n77 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
| DC_30_n77 |  |  | E-UTRA Band 2, 4, 5, 7, 12, 13, 14, 17, 24, 25, 26, 27, 29, 30, 41, 53, 66, 70, 71, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
| DC_38_n1 |  |  |  | E-UTRA Band 1, 3, 5, 8, 20, 22, 27, 28, 31, 32, 34, 40, 42, 43, 50, 51, 65, 67, 68, 72, 74, 75, 76 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
| DC_38_n3 |  |  |  | E-UTRA Band 1, 5, 8, 20, 27, 28, 31, 32, 33, 34, 40, 43, 50, 51, 65, 67, 68, 72, 74, 75, 76 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  |  | E-UTRA Band 22, 42 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
| DC_38_n8 |  |  |  | E-UTRA Band 1, 2, 4, 12, 13, 14, 17 20, 27, 28, 29, 30, 31, 32, 33, 34, 39, 40, 45, 50, 51, 65, 66, 67, 68, 72, 73, 74, 75, 76, 85, 87, 88 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  |  | E-UTRA band 3, 22, 42, 43, 52 NR Band n77, n78, n79 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  |  | E-UTRA Band 8 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  |  |  |  | E-UTRA Band 11, 21 |  | FDL_low | - | FDL_high | -50 | 1 | 12 |  |
|  |  |  |  | Frequency range |  | 860 | - | 890 | -40 | 1 | 5, 12 |  |
|  |  |  |  | Frequency range |  | 1884.5 |  | 1915.7 | -41 | 0.3 | 3, 12 |  |
|  |  |  |  | Frequency range |  | 2620 | - | 2645 | -15.5 | 5 | 5, 7, 22 |  |
|  |  |  |  | Frequency range |  | 2645 | - | 2690 | -40 | 1 | 5, 22 |  |
| DC_38_n28 |  |  | E-UTRA Band 1, 4, 22, 32, 42, 43, 50, 51, 52, 65, 66, 74, 75, 76, NR Band n77, n78 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | Frequency range |  | 470 | - | 694 | -42 | 8 | 5, 17 |  |
|  |  |  | Frequency range |  | 470 | - | 710 | -26.2 | 6 | 14 |  |
|  |  |  | Frequency range |  | 662 | - | 694 | -26.2 | 6 | 5 |  |
|  |  |  | Frequency range |  | 758 | - | 773 | -32 | 1 | 5 |  |
|  |  |  | Frequency range |  | 773 | - | 803 | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 1, 4, 22, 32, 42, 43, 50, 51, 52, 65, 66, 74, 75, 76, NR Band n77, n78 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
| DC_38_n78 |  |  | N/A |  |  |  |  |  |  |  |  |
| DC_38_n79 |  |  | E-UTRA Band 1, 3, 5, 8, 28, 34, 40, 42, 65, 74 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 2620 | - | 2645 | -15.5 | 5 | 5, 7, 22 |  |
|  |  |  | Frequency range |  | 2645 | - | 2690 | -40 | 1 | 5, 22 |  |
| DC_39_n40 |  |  | E-UTRA Band 1, 8, 22, 26, 28, 34, 41, 42, 44, 45, 50, 51, 52, 73, 74 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | NR Band n77, n78, n79 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | Frequency range |  | 1805 |  | 1855 | -40 | 1 | 18 |  |
|  |  |  | Frequency range |  | 1855 |  | 1880 | -15.5 | 5 | 5, 7, 18 |  |
| DC_39_n41 |  |  | E-UTRA Band 1, 8, 26, 28, 34, 42, 44, 45, 50, 51, 74 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | NR Band n77, n78, n79 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | E-UTRA Band 40 |  | FDL_low | - | FDL_high | -40 | 1 |  |  |
|  |  |  | Frequency range |  | 1805 | - | 1855 | -40 | 1 | 5 |  |
|  |  |  | Frequency range |  | 1855 | - | 1880 | -15.5 | 5 | 5, 7, 19 |  |
| DC_39_n78 |  |  | E-UTRA Band 1, 8, 28, 34, 40, 41, 44, 45 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 1805 | - | 1855 | -40 | 1 | 18 |  |
|  |  |  | Frequency range |  | 1855 | - | 1880 | -15.5 | 5 | 18 |  |
| DC_39_n79 |  |  | E-UTRA Band 1, 8, 28, 34, 40, 41, 44, 45 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 1805 | - | 1855 | -40 | 1 | 18 |  |
|  |  |  | Frequency range |  | 1855 | - | 1880 | -15.5 | 5 | 18 |  |
| DC_40_n1 |  |  | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 22, 26, 27, 28, 31, 32, 38, 41, 42, 43, 44, 45, 50, 51, 52, 65, 67, 68, 69, 72, 73, 74, 75, 76 NR Band n78 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 34 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  |  |  | NR Band n77, n79 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
| DC_40_n41 |  |  | Bands 1, 3, 5, 8, 11, 18, 19, 21, 26, 27, 28, 34, 39, 42, 44, 45, 50, 51, 65, 73, 74, NR Band n77, n78 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | NR Band n79 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
| DC_40_n77 |  |  | N/A |  |  |  |  |  |  |  |  |
| DC_40_n78 |  |  | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 33, 34, 38, 39, 41, 44, 45, 50, 51, 65, 67, 68, 69, 72, 73, 74, 75, 76 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | NR Band n79 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
| DC_40_n79 |  |  | Bands 1, 3, 5, 8, 11, 18, 19, 21, 26, 28, 34, 39, 41, 42, 65, 74 NR band n78 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
| DC_41_n1 |  |  | E-UTRA Band 1, 5, 8, 11, 18, 19, 21, 26, 27, 28, 40, 42, 44, 45, 50, 51, 52, 65, 73, 74 NR Band n78 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 3, 34 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  |  |  | NR Band n77, n79 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | Frequency range |  | 1880 | - | 1895 | -40 | 1 | 5, 16 |  |
|  |  |  | Frequency range |  | 1895 | - | 1915 | -15.5 | 5 | 5, 7, 16 |  |
|  |  |  | Frequency range |  | 1915 | - | 1920 | +1.6 | 5 | 5, 7, 16 |  |
| DC_41_n3 |  |  | E-UTRA Band 1, 5, 8, 26, 27, 28, 34, 39, 44, 45, 50, 51, 65, 73, 74 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 3 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  |  |  | E-UTRA Band 42, 52 NR Band n77, n78, n79 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | E-UTRA Band 40 |  | FDL_low | - | FDL_high | -40 | 1 |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
| DC_41_n28 |  |  | E-UTRA Band 4, 14, 18, 19, 20, 26, 27, 39, 42, 43, 48, 50, 51, 52, 65, 66, 71, 73 NR Band n77, n78, n79 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | E-UTRA Band 1 |  | FDL_low | - | FDL_high | -50 | 1 | 9, 11 |  |
|  |  |  | E-UTRA Band 2, 3, 5, 8, 24, 25, 30, 31, 34, 70, 72 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 11, 21, 74, 75, 76 |  | FDL_low | - | FDL_high | -50 | 1 | 9, 10 |  |
|  |  |  | E-UTRA Band 40 |  | FDL_low | - | FDL_high | -40 | 1 |  |  |
|  |  |  | Frequency range |  | 470 | - | 694 | -42 | 8 | 5, 17 |  |
|  |  |  | Frequency range |  | 470 | - | 710 | -26.2 | 6 | 14 |  |
|  |  |  | Frequency range |  | 662 | - | 694 | -26.2 | 6 | 5 |  |
|  |  |  | Frequency range |  | 758 | - | 773 | -32 | 1 | 5 |  |
|  |  |  | Frequency range |  | 773 | - | 803 | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 9 |  |
| DC_41_n77 |  |  | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 26, 28, 33, 34, 39, 44, 45, 73, 74 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 40 |  | FDL_low | - | FDL_high | -40 | 1 |  |  |
|  |  |  | Frequency range |  | 1884.5 |  | 1915.7 | -41 | 0.3 | 3 |  |
| DC_41_n78 |  |  | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 26, 28, 34, 39, 44, 45, 74 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 40 |  | FDL_low | - | FDL_high | -40 | 1 |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
| DC_41_n79 |  |  | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 26, 28, 34, 42, 44, 45, 65, 74 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 40 |  | FDL_low | - | FDL_high | -40 | 1 |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
| DC_42_n1 |  |  | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 38, 40, 41, 44, 45, 50, 51, 65, 67, 68, 69, 72, 73, 74, 75, 76, NR Band n79 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 3, 34 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  |  |  | Frequency range |  | 1880 | - | 1895 | -40 | 1 | 5, 16 |  |
|  |  |  | Frequency range |  | 1895 | - | 1915 | -15.5 | 5 | 5, 7, 16 |  |
|  |  |  | Frequency range |  | 1915 | - | 1920 | +1.6 | 5 | 5, 7, 16 |  |
| DC_42_n3 |  |  | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 33, 34, 38, 40, 41, 44, 45, 50, 51, 65, 67, 68, 69, 72, 73, 74, 75, 76 NR Band n79 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 3 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
| DC_42_n51 |  |  | E-UTRA Band 3, 8, 20, 25, 30, 31, 34, 39, 41, 73 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 1, 2, 4, 5, 6, 7, 12, 13, 14, 17, 23, 24, 26, 27, 28, 29, 32, 38, 40, 44, 46, 65, 66, 67, 68, 70, 71 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
| DC_42_n77 |  |  | N/A |  |  |  |  |  |  |  |  |
| DC_42_n78 |  |  | N/A |  |  |  |  |  |  |  |  |
| DC_42_n79 |  |  | N/A |  |  |  |  |  |  |  |  |
| DC_48_n5 |  |  | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 50, 51, 66, 70, 71, 74, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 41 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |
| DC_48_n12 |  |  | E-UTRA Band 2, 5, 13, 14, 17, 24, 25, 26, 30, 41, 71, 74 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 4, 50, 51, 66, 70 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | E-UTRA Band 12, 85 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
| DC_48_n25 |  |  | E-UTRA Band 4, 5, 12, 13 , 14, 17, 24, 26, 29, 30, 41, 66, 70, 71, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 2, 25 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 |  |  |
|  |  |  | Frequency range |  | 1880 |  | 1895 | -40 | 1 | 5, 16 |  |
|  |  |  | Frequency range |  | 1895 |  | 1915 | -15.5 | 5 | 5, 7, 16 |  |
|  |  |  | Frequency range |  | 1915 |  | 1920 | +1.6 | 5 | 5, 7, 16 |  |
| DC_48_n66 |  |  | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 50, 51, 66, 70, 71, 74, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
| DC_48_n71 |  |  | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 30, 50, 51, 53, 66, 74, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 2, 25, 41, 70 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | E-UTRA Band 29 |  | FDL_low | - | FDL_high | -38 | 1 | 5 |  |
|  |  |  | E-UTRA Band 71 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
| DC_48_n77 |  |  |  | N/A |  |  |  |  |  |  |  |  |
| DC_66_n2 |  |  | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 41, 50, 51, 53, 66, 70, 71, 74, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 25 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  |  |  | E-UTRA Band 2 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  |  |  | E-UTRA Band 22, 42, 43, NR Band n77 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
| DC_66_n5 |  |  | E-UTRA Band 1, 2, 3, 4, 5, 6, 7, 8, 12, 13, 14, 17, 24, 25, 26, 28, 29, 30, 34, 38, 40, 43, 45, 50, 51, 65, 66, 70, 71, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 41, 42, 48, 52, NR Band n77 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
| DC_66_n7 |  |  | E-UTRA Band 2, 4, 5, 7, 12, 13, 14, 17, 26, 27, 28, 29, 30, 43, 50, 51, 66, 74, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 42 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | Frequency range |  | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |  |
|  |  |  | Frequency range |  | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |  |
|  |  |  | Frequency range |  | 2595 | - | 2620 | -40 | 1 | 5, 6 |  |
| DC_66_n12 |  |  | E-UTRA Band 2, 5, 13, 14, 17, 24, 25, 26, 27, 30, 41, 50, 53, 70, 71, 74 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 4, 51, 66, 48, NR Band n77 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | E-UTRA Band 12, 85 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
| DC_66_n25 |  |  | E-UTRA Band 4, 5, 7, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 38, 41, 50, 51, 53, 66, 70, 71, 74, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 42, 48, NR Band n77 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | E-UTRA Band 2 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  |  |  | E-UTRA Band 25 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  |  |  | E-UTRA Band 43 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
| DC_66_n28 |  |  | E-UTRA Band 2, 5, 7, 25, 26, 27, 38, 41 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 4, 10, 42, 43, 50, 51, 66, 74, NR band n77, n78 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | Frequency range |  | 470 | - | 694 | -42 | 8 | 5, 17 |  |
|  |  |  | Frequency range |  | 470 | - | 710 | -26.2 | 6 | 14 |  |
|  |  |  | Frequency range |  | 662 | - | 694 | -26.2 | 6 | 5 |  |
|  |  |  | Frequency range |  | 758 | - | 773 | -32 | 1 | 5 |  |
|  |  |  | Frequency range |  | 773 | - | 803 | -50 | 1 |  |  |
| DC_66_n30 |  |  | E-UTRA Band 2, 4, 5, 7, 12, 13, 14, 17, 24, 25, 26, 27, 29, 30, 38, 41, 53, 66, 70, 71, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 48, NR Band n77 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
| DC_66_n41 |  |  | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 27, 28, 29, 30, 43, 50, 51, 66, 70, 71, 74, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 42, 48, NR Band n77 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
| DC_66_n38 |  |  | EUTRA 2, 4, 5, 12, 13,14,17, 25, 27, 28, 29, 30, 43, 50, 51, 66, 74, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 42 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | Frequency range |  | 2620 | - | 2645 | -15.5 | 5 | 5, 7, 22 |  |
|  |  |  | Frequency range |  | 2645 | - | 2690 | -40 | 1 | 5, 22 |  |
| DC_66_n48 |  |  | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 50, 51, 66, 70, 71, 74, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
| DC_66_n71 |  |  | E-UTRA Band 4, 5, 13, 14, 17, 24, 26, 27, 29, 30, 43, 50, 51, 66, 74 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 2, 7, 22, 25, 41, 42, 48, 70, NR Band n77 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | E-UTRA Band 71 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
| DC_66_n77 |  |  | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 26, 29, 30, 41, 65, 66, 70, 71 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
| DC_66_n78, DC_66_n86_ULSUP-TDM_n78 |  |  | E-UTRA Band 1, 3, 5, 7, 8, 20, 26, 28, 34, 39, 40, 41, 65 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
| DC_71_n2 |  |  | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 30, 48, 66 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 25, 41, 70, NR Band n2, n77 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | E-UTRA band 71 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  |  |  | E-UTRA Band 29 |  | FDL_low | - | FDL_high | -38 | 1 | 5 |  |
| DC_71_n5 |  |  | E-UTRA Band 4, 12, 13, 14, 17, 24, 26, 30, 48, 66, 85 NR Band n5 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 2, 25, 41, 70, NR Band n77 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | E-UTRA Band 29 |  | FDL_low | - | FDL_high | -38 | 1 | 5 |  |
|  |  |  | E-UTRA Band 71 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
| DC_71_n38 |  |  | E-UTRA Band 4, 5, 12, 13, 14, 17, 30, 66, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 2 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | E-UTRA band 29 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
| DC_71_n41 |  |  | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 30, 48, 66, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA band 2, 25, 70 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | E-UTRA band 71 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
|  |  |  | E-UTRA Band 29 |  | FDL_low | - | FDL_high | -38 | 1 | 5 |  |
| DC_71_n48 |  |  | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 29, 30, 50, 51, 66, 71, 74, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 2, 25, 41, 70 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
| DC_71_n66 |  |  | E-UTRA Band 4, 5, 13, 14, 17, 24, 26, 27, 29, 30, 43, 50, 51, 66, 74 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 2, 7, 22, 25, 41, 42, 48, 70, NR Band n77 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
|  |  |  | E-UTRA Band 71 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |
| DC_71_n78 |  |  | E-UTRA Band 5, 26 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  |  |  | E-UTRA Band 41 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
| NOTE 1: FDL_low and FDL_high refer to each frequency band specified in Table 5.5-1 in TS 36.101 [4] or in Table 5.2-1 in TS 38.101-1 [2]. NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 in TS 36.101 [4] and Table 6.5.3.1-2 in TS 38.101-1 [2] are permitted for each assigned carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180 kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz NOTE 4: Void NOTE 5: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1, Table 6.6.3.1A-1 in TS 36.101 [4] or in Table 6.5.3.1-1 in TS 38.101-1 [2] from the edge of the channel bandwidth. NOTE 6: This requirement is applicable for any channel bandwidths within the range 2500 - 2570 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2560.5 - 2562.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2552 - 2560 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 7: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 8: This requirement is applicable for any channel bandwidths within the range 1920 - 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 - 1938 MHz the requirement is applicable only for an uplink NOTE 9: Applicable when the assigned E-UTRA or NR carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 10: As exceptions, measurements with a level up to the applicable requirement of -38 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 2nd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 2nd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 11: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 12: This requirement is applicable only for the following cases: A: for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 902.5 MHz ≤ Fc < 907.5 MHz with an uplink transmission bandwidth less than or equal to 20 RB; B: for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 907.5 MHz ≤ Fc ≤ 912.5 MHz without any restriction on uplink transmission bandwidth; C: for carriers of 10 MHz channel bandwidth when carrier centre frequency (Fc) is Fc = 910 MHz with an uplink transmission bandwidth less than or equal to 32 RB with RBstart > 3. NOTE 13: Void NOTE 14: This requirement is applicable for 5 and 10 MHz E-UTRA or NR channel bandwidth allocated within 718-728MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and RBstart < 48. NOTE 15: Void NOTE 16: This requirement is applicable for any channel bandwidths within the range 1920 - 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 - 1938 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 17: This requirement is applicable in the case of a 10 MHz E-UTRA or NR carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. NOTE 18: This requirement is only applicable for E-UTRA carriers with bandwidth confined within 1885 - 1920 MHz (requirement for carriers with at least 1RB confined within 1880 - 1885 MHz is not specified). This requirement applies for an uplink transmission bandwidth less than or equal to 54 RB for E-UTRA carriers of 15 MHz bandwidth when carrier center frequency is within the range 1892.5 - 1894.5 MHz and for E-UTRA carriers of 20 MHz bandwidth when carrier center frequency is within the range 1895 - 1903 MHz. NOTE 19: Void NOTE 20: Void. NOTE 21: Void NOTE 22: This requirement is applicable for power class 3 UE for any channel bandwidths within the range 2570 - 2615 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2605.5 - 2607.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2597 - 2605 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. |  |  |  |  |  |  |  |  |  |  |  |

NOTE:	To simplify the above Table, E-UTRA band numbers are listed for bands which are specified only for E-UTRA operation or both E-UTRA and NR operation. NR band numbers are listed for bands which are specified only for NR operation.

<< Next changes >>

7.3B.2.3	Inter-band EN-DC within FR1

7.3B.2.3.0	General

Reference sensitivity exceptions are specified for the condition when there is uplink transmission only in the aggressor band.

7.3B.2.3.1	Reference sensitivity exceptions due to UL harmonic interference for EN-DC in NR FR1

Sensitivity degradation is allowed for a band if it is impacted by UL harmonic interference from another band part of the same EN-DC configuration. Reference sensitivity exceptions for the victim band (high) are specified in Table 7.3B.2.3.1-1 with uplink configuration of the agressor band (low) specified in Table 7.3B.2.3.1-2.

Table 7.3B.2.3.1-1: Reference sensitivity exceptions (MSD) due to UL harmonic for EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| 1, 3 | n772,13 |  | 23.9 | 22.1 | 20.9 | 19.8 | 19.0 | 17.9 | 16.8 | 16.0 | 15.4 | 14.8 | 14.3 | 13.8 |
|  | n773 |  | 1.1 | 0.8 | 0.3 |  |  |  |  |  |  |  |  |  |
| 2 | n482,13 | 27.3 | 24.4 | 22.4 | 21.2 |  | 19.0 | 18 | 17.1 | 16.3 | 15.4 | 15 | 14.5 | 14 |
|  | n483 | 1.9 | 1.4 | 0.9 | 0.4 |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 2 | n772, 13 |  | 23.9 | 22.1 | 20.9 | 19.8 | 19.0 | 17.9 | 16.8 | 16.0 | 15.5 | 14.8 | 14.3 | 13.8 |
|  | n773 |  | 1.1 | 0.8 | 0.3 | 0.1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 2 | n782,13 |  | 23.9 | 22.1 | 20.9 | 19.8 | 19.0 | 17.9 | 16.8 | 16.0 | 15.4 | 14.8 | 14.3 | 13.8 |
|  | n783 |  | 1.1 | 0.8 | 0.3 |  |  |  |  |  |  |  |  |  |
| 3 | n782,13 |  | 23.9 | 22.1 | 20.9 | 19.8 | 19.0 | 17.9 | 16.8 | 16.0 | 15.4 | 14.8 | 14.3 | 13.8 |
|  | n783 |  | 1.1 | 0.8 | 0.3 |  |  |  |  |  |  |  |  |  |
| n3 | 422, 13 | 27.3 | 24.3 | 22.5 | 21.3 |  |  |  |  |  |  |  |  |  |
|  | 423 | 1.9 | 1.3 | 1.0 | 0.5 |  |  |  |  |  |  |  |  |  |
| 4 | n782,13 |  | 23.9 | 22.1 | 20.9 | 19.8 | 19.0 | 17.9 | 16.8 | 16.0 | 15.4 | 14.8 | 14.3 | 13.8 |
|  | n783 |  | 1.1 | 0.8 | 0.3 |  |  |  |  |  |  |  |  |  |
| 5 | n776, 7, 17 |  | 10.5 | 8.9 | 7.8 | 7.2 | 6.5 | 5.1 | 4.2 | 3.5 | 2.8 | 2.3 | 2.1 | 1.4 |
|  | n774, 5, 17 |  | 10.4 | 8.9 | 7.8 | 6.7 | 6 | 4.7 | 3.7 | 3 | 2.3 | 1.7 | 1.2 | 0.7 |
| 5 | n786,7 |  | 10.5 | 8.9 | 7.8 | 7.1 | 6.5 | 5.4 | 4.2 | 3.5 | 2.9 | 2.3 | 2.1 | 1.4 |
| 7 | n793 |  | 1.1 | 0.8 | 0.3 |  |  |  |  |  |  |  |  |  |
| 8 | n78,9 | 10 | 7.6 | 6.2 | 5.3 | 4.3 | 3.2 | 2.3 | 1.3 |  |  |  |  |  |
| 8 | n418,9 | N/A | 13 | 11.3 | 10.1 |  | 8.3 | 7.0 | 6.1 | 5.5 | 4.9 | 4.3 | 3.9 | 3.5 |
| 8 | n776,7 n786,7 |  | 10.8 | 9.1 | 8 | 7.2 | 6.5 | 5.1 | 4.2 | 3.5 | 2.9 | 2.3 | 2.1 | 1.4 |
| 8 | n794,5 |  |  |  |  |  |  | 6.8 | 6.2 | 5.6 |  | 4.9 |  | 4.4 |
| n8 | 314 | N/A | N/A | N/A | N/A |  |  |  |  |  |  |  |  |  |
| n8 | 78,9,10 | 10 | 7.6 | 6.2 | 5.3 |  |  |  |  |  |  |  |  |  |
| 12 | n668,9,10 | 10 | 7.5 | 6.2 | 5.5 | 4.5 | 3.7 | 2.4 |  |  |  |  |  |  |
| 12 | n774,5 |  | 10.4 | 8.9 | 7.8 | 6.7 | 5.7 | 4.7 | 3.7 | 3 | 2.3 | 1.7 | 1.2 | 0.7 |
| 12 | n784,5 |  | 10.4 | 8.9 | 7.8 | 7.1 | 6.5 | 4.7 | 3.7 | 3 | 2.3 | 1.7 | 1.2 | 0.7 |
| n12 | 484,5 | 13 | 10.4 | 8.9 | 7.8 |  |  |  |  |  |  |  |  |  |
| n12 | 668,9,10 | 10 | 7.5 | 6.2 | 5.5 |  |  |  |  |  |  |  |  |  |
| 13 | n774, 5 |  | 10.4 | 8.9 | 7.8 | 6.7 | 5.7 | 4.7 | 3.7 | 3 | 2.3 | 1.7 | 1.2 | 0.7 |
| 14 | n774, 5 |  | 10.4 | 8.9 | 7.8 | 6.7 | 5.7 | 4.7 | 3.7 | 3 | 2.3 | 1.7 | 1.2 | 0.7 |
| 18，19 | n774,5 n784,5 |  | 10.4 | 8.9 | 7.8 | 7.1 | 6.5 | 4.7 | 3.7 | 3 | 2.3 | 1.7 | 1.2 | 0.7 |
| 28 | n502,13 | 27.8 | 24.6 | 22.8 | 21.6 |  | 19.5 | 18.5 | 17.5 | 16.7 |  | 15.4 |  |  |
|  | n503 | 1.9 | 1.4 | 0.9 | 0.4 |  |  |  |  |  |  |  |  |  |
| 28 | n512,13 | 27.8 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n513 | 1.9 |  |  |  |  |  |  |  |  |  |  |  |  |
| 28 | n668,9,10 | 10.2 | 7.6 | 6.2 | 5.3 | 4.0 | 3.2 | 2 |  |  |  |  |  |  |
| 28 | n774,5 n784,5 |  | 10.4 | 8.9 | 7.8 | 7.1 | 6.5 | 4.7 | 3.7 | 3 | 2.3 | 1.7 | 1.2 | 0.7 |
| 20 | n388,9 | 12.9 | 10.3 | 8.4 | 7.4 | 6.7 | 6.1 | 5 |  |  |  |  |  |  |
| 20 | n41 | 12.9 | 10.3 | 8.4 | 7.4 |  | 6.1 | 5 | 4.3 | 3.9 | 3.5 | 3.1 | 2.7 | 2.1 |
| 20 | n776,7 n786,7 |  | 10.8 | 9.1 | 8 | 7.3 | 6.8 | 6 | 4.0 | 3.2 | 2.5 | 2.0 | 1.5 | 1.0 |
| n28 | 2116 | N/A | N/A | N/A |  |  |  |  |  |  |  |  |  |  |
| 25 | n772, 13 |  | 23.9 | 22.1 | 20.9 | 19.8 | 19.0 | 17.9 | 16.8 | 16.0 | 15.5 | 14.8 | 14.3 | 13.8 |
|  | n773 |  | 1.1 | 0.8 | 0.3 | 0.1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 25 | n782, 13 |  | 23.9 | 22.1 | 20.9 | 19.8 | 19.0 | 17.9 | 16.8 | 16.0 | 15.5 | 14.8 | 14.3 | 13.8 |
|  | n783 |  | 1.1 | 0.8 | 0.3 | 0.1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| n25 | 482,13 | 27.3 | 24.4 | 22.4 | 21.2 |  |  |  |  |  |  |  |  |  |
|  | 483 | 1.9 | 1.4 | 0.9 | 0.4 |  |  |  |  |  |  |  |  |  |
| 26 | n418,9 |  | 10.3 | 8.4 | 7.4 |  | 6.1 | 5 | 4.3 | 3.9 | 3.5 | 3.1 | 2.9 | 2.7 |
| 26 | n776,7 n786,7 |  | 10.8 | 9.1 | 8 | 7.3 | 6.8 | 6 | 4.0 | 3.2 | 2.5 | 2.0 | 1.5 | 1.0 |
| 28 | n18,9,10 | 10.2 | 7.6 | 6.2 | 5.3 | 4.0 | 3.2 | 2 | 1.0 |  |  |  |  |  |
| n28 | 18,9,10 | 10.2 | 7.6 | 6.2 | 5.3 |  |  |  |  |  |  |  |  |  |
| n28 | 48,9,10 | 10.2 | 7.6 | 6.2 | 5.3 |  |  |  |  |  |  |  |  |  |
| 28 | n75 | 28.1 | 25.3 | 24.0 | 22.8 | 21.6 | 20.4 | 19.2 | 18.0 |  |  |  |  |  |
| n28 | 112,10,13 | 24.8 | 21.8 |  |  |  |  |  |  |  |  |  |  |  |
| n28 | 424,5,10 | 14.1 | 10.4 | 8.9 | 7.9 |  |  |  |  |  |  |  |  |  |
| n28 | 668,9,10 | 10.2 | 7.6 | 6.2 | 5.3 |  |  |  |  |  |  |  |  |  |
| 71 | n211 | 4.6 | 1.0 | 0.7 | 0.6 |  |  |  |  |  |  |  |  |  |
|  | n212 | 1.7 | 1.0 | 0.7 | 0.6 |  |  |  |  |  |  |  |  |  |
| n71 | 211 | 4.6 | 1.0 | 0.7 | 0.6 |  |  |  |  |  |  |  |  |  |
|  | 212 | 1.7 | 1.0 | 0.7 | 0.6 |  |  |  |  |  |  |  |  |  |
| n71 | 76,7 | 14.6 | 11.7 | 10.1 | 9 |  |  |  |  |  |  |  |  |  |
| 71 | n416,7 |  | 10.8 | 9.1 | 8.0 |  | 6.1 | 5.1 | 4.2 | 3.5 | 2.7 | 2.3 | 2.1 | 1.4 |
| 66 | n482,13 | 27.3 | 24.4 | 22.4 | 21.2 |  | 19.0 | 18 | 17.1 | 16.3 | 15.4 | 15 | 14.5 | 14 |
|  | n483 | 1.9 | 1.4 | 0.9 | 0.4 |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 66 | n772, 13 |  | 23.9 | 22.1 | 20.9 | 19.8 | 19.0 | 17.9 | 16.8 | 16.0 | 15.3 | 14.8 | 14.3 | 13.8 |
|  | n773 |  | 1.1 | 0.8 | 0.3 | 0.1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 66 | n782,13 |  | 23.9 | 22.1 | 20.9 | 19.8 | 19.0 | 17.9 | 16.8 | 16.0 | 15.4 | 14.8 | 14.3 | 13.8 |
|  | n783 |  | 1.1 | 0.8 | 0.3 |  |  |  |  |  |  |  |  |  |
| n66 | 482,13 | 27.3 | 24.4 | 22.4 | 21.2 |  |  |  |  |  |  |  |  |  |
|  | 483 | 1.9 | 1.4 | 0.9 | 0.4 |  |  |  |  |  |  |  |  |  |
| 71 | n784,5 |  | 10.4 | 8.9 | 7.8 | 7.1 | 6.5 | 4.7 | 3.7 | 3 | 2.3 | 1.7 | 1.2 | 0.7 |
| NOTE 1: Void NOTE 2: The requirements should be verified for UL EARFCN or NR ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 3: The requirements are only applicable to channel bandwidths no larger than 20 MHz and with a carrier frequency at MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. NOTE 4: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 5th transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 5: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 6: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 4th transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 7: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 8: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 9 The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LBsuch that in MHz and with the carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the low band. NOTE 10: Applicable for the operations with 2 or 4 antenna ports supported in the band with carrier aggregation configured. NOTE 11: These requirements apply when the lower edge frequency of the 5 MHz uplink channel in Band 71 is located at or below 668 MHz and the downlink channel in Band 2 is located with its upper edge at 1990 MHz. NOTE 12: These requirements apply when the lower edge frequency of the 10 MHz, 15 MHz, or 20 MHz uplink channel in Band 71 is located at or below 668 MHz and the downlink channel in Band 2 is located with its upper edge at 1990 MHz. NOTE 13: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range ∆FHD above and below the edge of this downlink transmission bandwidth. The value ∆FHD depends on the EN-DC band combination: ∆FHD = 10 MHz for DC_1_n77, DC_2_n48, DC_2_n77, DC_42_n3, DC_48_n25, DC_48_n66, DC_66_n48, DC_66_n77, DC_3_n77, DC_3_n78, DC_11_n28 and DC_28_n50, DC_28_n51, DC_66_n78, DC_25_n77, DC_25_n78. NOTE 14: No requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the low band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of the high band. The reference sensitivity for all active downlink component carriers is only verified when this is not the case (the requirements specified in clause 7.3.1 from TS 36.101-1 apply unless otherwise specified). NOTE 15: MSD test point can be chosen according to supported BW and lowest SCS supported by the UE. NOTE 16: The frequency range in band n28 is restricted for this band combination to 728 - 738 MHz for the UL. This band is subject to 2nd harmonic fall in B21 also which MSD is not specified. NOTE 17: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 from TS 38.101-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.3.1-2: Uplink configuration for reference sensitivity exceptions due to UL harmonic interference for EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / UL RB allocation of the agressor band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 30 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 70 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| 1 | n77 | 15 |  | 25 | 36 | 50 | 64 | 80 | 100 | 100 | 100 | 100 | 100 | 100 | 100 |
| 2 | n48 | 15 | 12 | 25 | 36 | 50 |  | 80 | 100 | 100 | 100 | 100 | 100 | 100 | 100 |
| 2 | n77 | 15 |  | 25 | 36 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 |
| 2 | n78 | 15 |  | 25 | 36 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 |
| 3 | n77, n78 | 15 |  | 25 | 36 | 50 | 64 | 80 | 50 | 50 | 50 | 100 | 50 | 50 | 50 |
| n3 | 42 | 15 | 12 | 25 | 36 | 50 |  |  |  |  |  |  |  |  |  |
| 4 | n78 | 15 |  | 25 | 36 | 50 | 64 | 80 | 100 | 100 | 100 | 100 | 100 | 100 | 100 |
| 5 | n77 | 15 |  | 16 | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 |
| 5 | n78 | 15 | 8 | 16 | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 |
| 7 | n79 | 15 |  | 25 | 36 | 50 |  |  |  |  |  |  |  |  |  |
| 8 | n7 | 15 | 8 | 16 | 25 | 25 | 25 | 25 | 25 | 25 |  |  |  |  |  |
| 8 | n41 | 15 |  | 16 | 25 | 25 |  | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 |
| 8 | n77 n78 | 15 |  | 16 | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 |
| 8 | n79 | 15 |  |  |  |  |  |  | 25 | 25 | 25 |  | 25 |  | 25 |
| n8 | 7 | 15 | 8 | 16 | 25 | 25 |  |  |  |  |  |  |  |  |  |
| 12 | n66 | 15 | 8 | 16 | 20 | 20 | 20 | 20 | 20 |  |  |  |  |  |  |
| 12 | n77 | 15 |  | 10 | 15 | 20 | 20 | 20 | 20 | 20 | 20 | 20 | 20 | 20 | 20 |
| 12 | n78 | 15 |  | 10 | 15 | 20 | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 |
| n12 | 48 | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |
| n12 | 66 | 15 | 8 | 16 | 20 | 20 |  |  |  |  |  |  |  |  |  |
| 13 | n77 | 15 |  | 15 | 20 | 20 | 20 | 20 | 20 | 20 | 20 | 20 | 20 | 20 | 20 |
| 14 | n77 | 15 |  | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 |
| 18 | n77, n78 | 15 |  | 16 | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 |
| 19 | n77, n78 | 15 |  | 16 | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 |
| 20 | n38 | 15 | 8 | 16 | 25 | 25 | 25 | 25 | 25 |  |  |  |  |  |  |
| 20 | n41 | 15 | 8 | 16 | 25 | 25 |  | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 |
| 20 | n77, n78 | 15 |  | 16 | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 |
| 25 | n77 | 15 |  | 25 | 36 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 |
| 25 | n78 | 15 |  | 25 | 36 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 |
| 26 | n41 | 15 |  | 16 | 25 | 25 |  | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 |
| 26 | n77, n78 | 15 |  | 16 | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 |
| 28 | n1 | 15 | 8 | 16 | 25 | 25 | 25 | 25 | 25 | 25 |  |  |  |  |  |
| n28 | 1 | 15 | 8 | 16 | 25 | 25 |  |  |  |  |  |  |  |  |  |
| n28 | 4 | 15 | 8 | 16 | 25 | 25 |  |  |  |  |  |  |  |  |  |
| 28 | n75 | 15 | 8 | 16 | 25 | 25 | 25 | 25 | 25 | 25 |  |  |  |  |  |
| 28 | n50 | 15 | 12 | 25 | 25 | 25 |  | 25 | 25 | 25 | 25 |  | 25 |  |  |
| 28 | n51 | 15 | 12 |  |  |  |  |  |  |  |  |  |  |  |  |
| 28 | n66 | 15 | 8 | 16 | 25 | 25 | 25 | 25 | 25 |  |  |  |  |  |  |
| n28 | 11 | 15 | 12 | 25 |  |  |  |  |  |  |  |  |  |  |  |
| n28 | 42 | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |
| n28 | 66 | 15 | 8 | 16 | 25 | 25 |  |  |  |  |  |  |  |  |  |
| 28 | n77, n78 | 15 |  | 10 | 15 | 20 | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 |
| 66 | n48 | 15 | 12 | 25 | 36 | 50 |  | 80 | 100 | 100 | 100 | 100 | 100 | 100 | 100 |
| 66 | n77 | 15 |  | 25 | 36 | 50 | 64 | 80 | 100 | 100 | 100 | 100 | 100 | 100 | 100 |
| 66 | n78 | 15 |  | 25 | 36 | 50 | 64 | 80 | 100 | 100 | 100 | 100 | 100 | 100 | 100 |
| n66 | 48 | 15 | 12 | 25 | 36 | 50 |  |  |  |  |  |  |  |  |  |
| 71 | n2 | 15 | 254 85 | 254 85 | 204 85 | 204 85 |  |  |  |  |  |  |  |  |  |
| n71 | 2 | 15 | 254 85 | 254 85 | 204 85 | 204 85 |  |  |  |  |  |  |  |  |  |
| n71 | 7 | 15 | 8 | 16 | 25 | 25 |  |  |  |  |  |  |  |  |  |
| 71 | n41 | 15 |  | 16 | 25 | 25 |  | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 |
| 71 | n78 | 15 |  | 10 | 15 | 20 | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 |
| NOTE 1: The UL configuration applies regardless of the channel bandwidth of the UL band unless the UL resource blocks exceed that specified in Table 7.3.1-2 in TS 36.101 [4] or Table 7.3.2-3 in TS 38.101-1 [2] for the uplink bandwidth in which case the allocation according to Table 7.3.1-2 in TS 36.101 [4] or Table 7.3.2-3 in TS 38.101-1 [2] applies NOTE 2: Void NOTE 3: Unless stated otherwise, UL resource blocks shall be centred within the transmission bandwidth configuration for the channel bandwidth. NOTE 4: These requirements apply when the lower edge frequency of the 5 MHz uplink channel in Band 71 is located at or below 668 MHz and the downlink channel in Band 2 is located with its upper edge at 1990 MHz. NOTE 5: These requirements apply when the lower edge frequency of the 10 MHz, 15 MHz, or 20 MHz uplink channel in Band 71 is located at or below 668 MHz and the downlink channel in Band 2 is located with its upper edge at 1990 MHz. NOTE 6: If the aggressor band is NR band, the test SCS and UL RB can be adjusted according to supported BW and lowest SCS supported by the UE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.3.1-3: Reference sensitivity QPSK PREFSENS (EN-DC with n46)

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| 2 | n461 |  |  |  | N/A |  | N/A |  | N/A | N/A |  |  |
| n46 | 22,3 | 28 | 28 | 28 | 28 |  |  |  |  |  |  |  |
| n46 | 482,4 | 22.6 | 19.5 | 17.8 | 16.6 |  |  |  |  |  |  |  |
| 66 | n46 |  |  |  | N/A |  | N/A |  | N/A | N/A |  |  |
| NOTE 1: These requirements apply when there is at least one individual RE within the downlink (victim) transmission bandwidth which falls into the reference sensitivity exclusion region as specified in Table 6.x.1.7-2 and Table 6.x.1.7-3. NOTE 2: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (higher) band and when the frequency range of relative higher band’s uplink channel bandwidth or uplink 1st adjacent channel bandwidth is fully or partially overlapped with the downlink transmission bandwidth of a victim (lower) band. NOTE 3: The requirements for a victim (lower) band apply for UL EARFCN of the aggressor (higher) band (superscript HB) such that in MHz with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. Note 4: The requirements should be verified for UL NR-ARFCN of the aggressor (high) band (superscript HB) such that in MHz and with carrier frequency in the victim (lower) band in MHz and the channel bandwidth configured in the higher band. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.3.1-4: n46 Reference sensitivity measurement exclusion region in MHz

| Licensed Component Carriers / E-UTRA Band / Harmonic order / Channel BW in UL |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Band | Harmonic order | 5MHz | 10MHz | 15MHz | 20MHz |
| 2 | 3 | +/- 15 | +/- 23 | +/- 35 | +/- 45 |
| 66 | 3 | +/- 15 | +/- 23 | +/- 35 | +/- 45 |
| NOTE 1: Even though UL harmonic does not fall directly into n46 the exclusion region still applies. NOTE 2: The center of the exclusion region is obtained by multiplying the uplink channel center frequency by the harmonic order. |  |  |  |  |  |

Table 7.3B.2.3.1-5: Uplink configuration for reference sensitivity exceptions due to receiver harmonic mixing for EN-DC paring with n46

| E-UTRA or NR Band / SCS / Channel bandwidth of the affected DL band / UL RB allocation of the agressor band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| n46 | 2 | 15 | 25 | 50 | 75 | 100 |  |  |  |  |  |  |  |
| n46 | 48 | 15 | 12 | 25 | 36 | 50 |  |  |  |  |  |  |  |

7.3B.2.3.2	Reference sensitivity exceptions due to receiver harmonic mixing for EN-DC in NR FR1

Sensitivity degradation is allowed for a band if it is impacted by receiver harmonic mixing due to another band part of the same EN-DC configuration. Reference sensitivity exceptions for the victim band (low) are specified in Table 7.3B.2.3.2-1 with uplink configuration of the agressor band (high) specified in Table 7.3B.2.3.2-2.

Table 7.3B.2.3.2-1: Reference sensitivity exceptions (MSD) due to receiver harmonic mixing for EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| 1 | n714 | 26.8 | 23.6 | 21.2 | 15.6 |  |  |  |  |  |  |  |
| 2 | n714 | 26.8 | 23.6 | 21.2 | 15.6 |  |  |  |  |  |  |  |
| n2 | 714 | 26.8 | 23.6 | 21.2 | 15.6 |  |  |  |  |  |  |  |
| n38 | 59 | N/A | N/A |  |  |  |  |  |  |  |  |  |
| n40 | 284 | 37.8 | 34.8 | 33 | 30.3 |  |  |  |  |  |  |  |
| n41 | 189 | N/A | N/A | N/A |  |  |  |  |  |  |  |  |
| n41 | 264 | 24.3 | 24.3 | 22.5 | N/A |  |  |  |  |  |  |  |
| n77 | 2 | 6.1 | 5.0 | 4.0 | 3.7 |  |  |  |  |  |  |  |
| n77 | 3 | 5.7 | 4.0 | 3.0 | 2.7 |  |  |  |  |  |  |  |
| n78 | 3 | 5.7 | 4.0 | 3.0 | 2.7 |  |  |  |  |  |  |  |
| n77 | 78 | 10.4 | 10.4 | 10.4 | 10.4 |  |  |  |  |  |  |  |
| n77 | 122 | 31 | 28 |  |  |  |  |  |  |  |  |  |
| n77 | 132 | 31 | 28 |  |  |  |  |  |  |  |  |  |
| n77 | 142 | 31 | 28 |  |  |  |  |  |  |  |  |  |
| n77 | 418 | 10.4 | 10.4 | 10.4 | 10.4 |  |  |  |  |  |  |  |
| n77 | 25 | [6.1] | [5.0] | [4.0] | [3.7] |  |  |  |  |  |  |  |
| n77 | 282 | 28 | 25 | 23.2 | 22 |  |  |  |  |  |  |  |
| n7711 | 292 | 31 | 28 |  |  |  |  |  |  |  |  |  |
| n78 | 408 | 10.4 | 10.4 | 10.4 | 10.4 |  |  |  |  |  |  |  |
| n78 | 418 | 10.4 | 10.4 | 10.4 | 10.4 |  |  |  |  |  |  |  |
| n79 | 114 | 39.3 | 36.3 | 34.5 |  |  |  |  |  |  |  |  |
| n79 | 192 | 29.5 | 26.5 | 24.7 |  |  |  |  |  |  |  |  |
| n79 | 214 | 39.3 | 36.3 | 34.5 |  |  |  |  |  |  |  |  |
| n79 | 262 | 27 | 24 | 22.2 |  |  |  |  |  |  |  |  |
| NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (higher) band for which the mixing product due to harmonic of victim (lower) band LO with leakage of aggressor (higher) band is within the downlink transmission bandwidth of a victim (lower) band. NOTE 2: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. NOTE 3: Void. NOTE 4: The requirements should be verified for DL EARFCN or NR ARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. NOTE 5: Void NOTE 6: Void NOTE 7: Void NOTE 8: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that fDLLB=fULHB/0.150.1 with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. NOTE 9: No requirements apply for the case that there is at least one individual RE within the uplink transmission bandwidth of the relative higher band and when the frequency range of relative higher band’s uplink channel bandwidth or uplink 1st adjacent channel bandwidth is fully or partially overlapped with the 3 times of the frequency range of the relative lower band’s downlink channel bandwidth. The reference sensitivity is only verified when this is not the case. NOTE 10: MSD test point can be chosen according to supported BW and lowest SCS supported by the UE. NOTE 11: The MSD test points cannot be verified for the band combination in US due to the Band n77 frequency range restriction. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.3.2-1a: Reference sensitivity exceptions (MSD) due to receiver harmonic mixing for PC2 EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n77 | 141 | 34 | 31 |  |  |  |  |  |  |  |  |  |
| n77 | 121 | 34 | 31 |  |  |  |  |  |  |  |  |  |
| n772 | 291 | 34 | 31 |  |  |  |  |  |  |  |  |  |
| NOTE 1: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. NOTE 2: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 from TS 38.101-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.3.2-2: Uplink configuration for reference sensitivity exceptions due to receiver harmonic mixing for EN-DC in NR FR1

| E-UTRA or NR Band / SCS / Channel bandwidth of the affected DL band / UL RB allocation of the agressor band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| 1 | n71 | 15 | 25 | 50 | 75 | 100 |  |  |  |  |  |  |  |
| 2 | n71 | 15 | 25 | 50 | 50 | 50 |  |  |  |  |  |  |  |
| n2 | 71 | 15 | 25 | 50 | 50 | 50 |  |  |  |  |  |  |  |
| n40 | 28 | 15 | 25 | 50 | 75 | 100 |  |  |  |  |  |  |  |
| n41 | 26 | 15 | 25 | 50 | 75 |  |  |  |  |  |  |  |  |
| n77 | 2 | 15 | 25 | 50 | 75 | 100 |  |  |  |  |  |  |  |
| n77 | 3 | 15 | 25 | 50 | 75 | 100 |  |  |  |  |  |  |  |
| n78 | 3 | 15 | 25 | 50 | 75 | 100 |  |  |  |  |  |  |  |
| n77 | 7 | 15 | 12 | 25 | 36 | 50 |  |  |  |  |  |  |  |
| n77 | 12 | 15 | 25 | 50 |  |  |  |  |  |  |  |  |  |
| n77 | 13 | 15 | 25 | 50 |  |  |  |  |  |  |  |  |  |
| n77 | 14 | 15 | 25 | 50 |  |  |  |  |  |  |  |  |  |
| n77 | 25 | 15 | 25 | 50 | 75 | 100 |  |  |  |  |  |  |  |
| n77 | 28 | 15 | 25 | 50 | 75 | 100 |  |  |  |  |  |  |  |
| n77 | 29 | 15 | 25 | 50 |  |  |  |  |  |  |  |  |  |
| n77 | 41 | 15 | 12 | 25 | 36 | 50 |  |  |  |  |  |  |  |
| n78 | 29 | 15 | 25 | 50 |  |  |  |  |  |  |  |  |  |
| n78 | 40 | 15 | 12 | 25 | 36 | 50 |  |  |  |  |  |  |  |
| n78 | 41 | 15 | 12 | 25 | 36 | 50 |  |  |  |  |  |  |  |
| n79 | 11 | 15 | 25 | 50 | 75 |  |  |  |  |  |  |  |  |
| n79 | 19 | 15 | 25 | 50 | 75 |  |  |  |  |  |  |  |  |
| n79 | 21 | 15 | 25 | 50 | 75 |  |  |  |  |  |  |  |  |
| n79 | 26 | 15 | 25 | 50 | 75 |  |  |  |  |  |  |  |  |
| NOTE 1: Void NOTE 2: Void NOTE 3: The UL configuration applies regardless of the channel bandwidth of the UL band. UL resource blocks allocation in the table shall be further limited to that specified in Table 7.3.1-2 in TS 36.101 [4] or Table 7.3.2-3 in TS 38.101-1 [2]. NOTE 4: Unless otherwise stated, the UL resource blocks allocation is applied at the center of the channel bandwidth. The note applies to the entire table. NOTE 5: If the aggressor band is NR band, the test SCS and UL RB can be adjusted according to supported BW and lowest SCS supported by the UE. |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.3B.2.3.3	Void

7.3B.2.3.4	Reference sensitivity exceptions due to cross band isolation for EN-DC in NR FR1

Sensitivity degradation is allowed for a band if it is impacted by UL of another band part of the same EN-DC configuration due to cross band isolation issues. Reference sensitivity exceptions for the victim band are specified in Table 7.3B.2.3.4-1 and Table 7.3B.2.3.4-1a with uplink configuration of the agressor band specified in Table 7.3B.2.3.4-2.

Table 7.3B.2.3.4-1: Reference sensitivity exceptions (MSD) due to cross band isolation for PC3 EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n13 | 3 | 3 | 2.3 | 2 | 1.8 |  |  |  |  |  |  |  |  |  |
| n1 | 40 | 6.6 | 6.6 | 6.6 | 6.6 |  |  |  |  |  |  |  |  |  |
| n1 | 41 | 6.1 | 6.1 | 6.1 | 6.1 |  |  |  |  |  |  |  |  |  |
| 13 | n3 | 3 | 2.2 | 1.9 | 1.7 | 1.6 | 1.5 | 1.4 | 1.3 |  |  |  |  |  |
| 1 | n40 | 6.6 | 6.6 | 6.6 | 6.6 | 6.6 | 6.6 | 6.6 | 6.6 | 6.6 |  | 6.6 | 6.6 | 6.6 |
| 1 | n41 |  | 6.1 | 6.1 | 6.1 |  | 6.1 | 6.1 | 6.1 | 6.1 | 6.1 | 6.1 | 6.1 | 6.1 |
| n3 | 11 | 6.4 | 6.1 |  |  |  |  |  |  |  |  |  |  |  |
| 3 | n41 |  | 0.7 | 0.7 | 0.7 |  | 0.7 | 0.7 | 0.7 | 0.7 | 0.7 | 0.7 | 0.7 | 0.7 |
| 3 | n51 | 6.4 |  |  |  |  |  |  |  |  |  |  |  |  |
| 30 | n66 | 8.3 | 8.3 | 8.3 | 8.3 | 8.3 | 8.3 | 8.3 |  |  |  |  |  |  |
| n3 | 41 | 0.7 | 0.7 | 0.7 | 0.7 |  |  |  |  |  |  |  |  |  |
| n5 | 28 | 4.5 | 3 | 2.2 | 0.3 |  |  |  |  |  |  |  |  |  |
| 7 | n40 | 3.7 | 3.4 | 3.2 | 3.1 | 3.1 | 3.1 | 3.1 | 3.1 | 3.1 |  | 3.1 | 3.1 | 3.1 |
| 18 | n28x | 31.3 | 28.7 | 26.9 | 24.3 |  | 12.4 |  |  |  |  |  |  |  |
| n25 | 2 | 33 | 33 | 33 | 33 |  |  |  |  |  |  |  |  |  |
| n34 | 3 | 3 | 2.2 | 1.9 | 1.7 |  |  |  |  |  |  |  |  |  |
| n38 | 1 | 1.9 | 1.9 | 1.9 | 1.9 |  |  |  |  |  |  |  |  |  |
| n38 | 2 | 0.6 | 0.6 | 0.6 | 0.6 |  |  |  |  |  |  |  |  |  |
| n38 | 4 | 1.9 | 1.9 | 1.9 | 1.9 |  |  |  |  |  |  |  |  |  |
| n38 | 66 | 1.9 | 1.9 | 1.9 | 1.9 |  |  |  |  |  |  |  |  |  |
| 38 | n1 | 1.9 | 1.9 | 1.9 | 1.9 | 1.9 | 1.9 | 1.9 | 1.9 |  |  |  |  |  |
| n1 | 38 | 2.9 | 2.9 | 2.9 | 2.9 |  |  |  |  |  |  |  |  |  |
| n40 | 1 | 8.3 | 8.3 | 8.3 | 8.3 |  |  |  |  |  |  |  |  |  |
| n41 | 4 | 3.5 | 3.5 | 3.5 | 3.5 |  |  |  |  |  |  |  |  |  |
| 40 | n1 | 8.3 | 8.3 | 8.3 | 8.3 | 8.3 | 8.3 | 8.3 | 8.3 |  |  |  |  |  |
| n40 | 7 | 3.7 | 3.7 | 3.7 | 3.7 |  |  |  |  |  |  |  |  |  |
| n41 | 1 | 9.1 | 9.1 | 9.1 | 9.1 |  |  |  |  |  |  |  |  |  |
| n41 | 2 | 0.6 | 0.6 | 0.6 | 0.6 |  |  |  |  |  |  |  |  |  |
| n41 | 3 | 0.6 | 0.6 | 0.6 | 0.6 |  |  |  |  |  |  |  |  |  |
| 41 | n1 | 9.1 | 9.1 | 9.1 | 9.1 | 9.1 | 9.1 | 9.1 | 9.1 |  |  |  |  |  |
| 41 | n3 | 0.6 | 0.6 | 0.6 | 0.6 | 0.6 | 0.6 | 0.6 | 0.6 |  |  |  |  |  |
| n41 | 661 | 3.5 | 3.5 | 3.5 | 3.5 |  |  |  |  |  |  |  |  |  |
| n41 | 25 | 0.6 | 0.6 | 0.6 | 0.6 |  |  |  |  |  |  |  |  |  |
| n50 | 3 | 2.5 | 1.9 | 1.6 | 1.5 |  |  |  |  |  |  |  |  |  |
| n71 | 12 | 2.3 | 2.3 |  |  |  |  |  |  |  |  |  |  |  |
| n77 | 71 | 4.5 | 4.5 | 4.5 | 4.5 |  |  |  |  |  |  |  |  |  |
| n77 | 411 | 4.5 | 4.5 | 4.5 | 4.5 |  |  |  |  |  |  |  |  |  |
| 41 | n77 |  | 8.3 | 8.3 | 8.3 | 7.3 | 6.5 | 6.3 | 5.3 | 4.5 | 4.3 | 4.0 | 3.9 | 3.8 |
| n78 | 71 | 4.5 | 4.5 | 4.5 | 4.5 |  |  |  |  |  |  |  |  |  |
| n78 | 38 | 3.3 | 3.3 | 3.3 | 3.3 |  |  |  |  |  |  |  |  |  |
| n78 | 401 | 4.5 | 4.5 | 4.5 | 4.5 |  |  |  |  |  |  |  |  |  |
| n78 | 411 | 4.5 | 4.5 | 4.5 | 4.5 |  |  |  |  |  |  |  |  |  |
| n78 | 46 |  |  |  | 7 |  |  |  |  |  |  |  |  |  |
| 41 | n78 |  | 8.3 | 8.3 | 8.3 | 7.3 | 6.5 | 6.3 | 5.3 | 4.5 | 4.3 | 4.0 | 3.9 | 3.8 |
| n79 | 426 | 2.6 | 2.6 | 2.6 | 2.6 |  |  |  |  |  |  |  |  |  |
| n843 | 3 | 3 | 2.3 | 2 | 1.8 |  |  |  |  |  |  |  |  |  |
| 48 | n46 | - | - | - | 7 | - | - | 5.7 | - | 5.1 | - | 4.7 | - | - |
| n46 | 48 | 13.3 | 10.4 | 8.8 | 7.8 | - | - | 7.8 | 7 | 6.5 | - | 5.7 | 5.4 | 5.1 |
| NOTE 1: Applicable only when harmonic mixing MSD for this combination is not applied. NOTE 2: The B41 requirements are modified by -0.5dB when carrier frequency of the assigned E-UTRA channel bandwidth is within 2515 – 2690 MHz. NOTE 3: These requirements apply when the uplink is active in Band n1, n84 and the separation between the lower edge of the uplink channel in Band n1, n84 and the upper edge of the downlink channel in Band 3 is < 60 MHz. For each channel bandwidth in Band 3, the requirement applies regardless of channel bandwidth in Band n1, n84. NOTE 4: The DL victim band should be configured using the lowest SCS that is compatible with the highest CBW for which an MSD is specified. NOTE 5: MSD test point can be chosen according to supported BW and lowest SCS supported by the UE. NOTE 6: The requirements only apply for UEs supporting inter-band DC_42_n79 ENDC with simultaneous Rx/Tx capability. Simultaneous Rx/Tx capability does not apply for UEs supporting band 42 with a n77 implementation only. These restrictions are applicable to related higher order configurations. NOTE X: The MSD exceptions are applicable to the case that interference of UL band 3rd order IMD product falls into the affected DL channels. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.3.4-1a: Reference sensitivity exceptions (MSD) due to cross band isolation for PC2 EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| 3 | n41 |  | 0.7 | 0.7 | 0.7 |  | 0.7 | 0.7 | 0.7 | 0.7 | 0.7 | 0.7 | 0.7 | 0.7 |
| n41 | 3 | 2.3 | 2.3 | 2.3 | 2.3 |  |  |  |  |  |  |  |  |  |
| n78 | 7 | 6.4 | 6.4 | 6.4 | 6.4 |  |  |  |  |  |  |  |  |  |
| n41 | 2 | 1.6 | 1.6 | 1.6 | 1.6 |  |  |  |  |  |  |  |  |  |
| n41 | 66 | 5.4 | 5.4 | 5.4 | 5.4 |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.3.4-2: Uplink configuration for reference sensitivity exceptions due to cross band isolation for EN-DC in NR FR1

| E-UTRA or NR Band / SCS / Channel bandwidth of the affected DL band / UL RB allocation of the agressor band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 30 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 70 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| n1 | 3 | 15 | 25 | 25 | 25 | 25 |  |  |  |  |  |  |  |  |  |
| n1 | 40 | 15 | 25 | 50 | 75 | 100 |  |  |  |  |  |  |  |  |  |
| n1 | 41 | 15 | 100 | 100 | 100 | 100 |  |  |  |  |  |  |  |  |  |
| 1 | n3 | 15 | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 |  |  |  |  |  |
| 1 | n40 | 15 | 25 | 50 | 75 | 100 | 100 | 100 | 100 | 100 | 100 |  | 100 | 100 | 100 |
| 1 | n41 | 15 |  | 100 | 100 | 100 |  | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 |
| n3 | 11 | 15 | 25 | 50 |  |  |  |  |  |  |  |  |  |  |  |
| 3 | n41 | 15 |  | 50 | 50 | 50 |  | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 |
| 3 | n51 | 15 | 25 |  |  |  |  |  |  |  |  |  |  |  |  |
| 30 | n66 | 15 | 25 | 25 | 25 | 25 | 25 | 25 | 25 |  |  |  |  |  |  |
| n3 | 41 | 15 | 25 | 502 | 502 | 502 |  |  |  |  |  |  |  |  |  |
| n5 | 28 | 15 | 25 | 25 | 20 | 20 |  |  |  |  |  |  |  |  |  |
| 7 | n40 | 15 | 25 | 50 | 75 | 75 | 75 | 100 | 100 | 100 | 100 |  | 100 | 100 | 100 |
| 18 | n28x | 15 | 25 | 25 | 25 | 25 |  | 25 |  |  |  |  |  |  |  |
| n25 | 2 | 15 | 40 | 40 | 40 | 40 |  |  |  |  |  |  |  |  |  |
| n34 | 3 | 15 | 25 | 25 | 25 | 25 |  |  |  |  |  |  |  |  |  |
| n38 | 1 | 15 | 100 | 100 | 100 | 100 |  |  |  |  |  |  |  |  |  |
| n38 | 2 | 15 | 100 | 100 | 100 | 100 |  |  |  |  |  |  |  |  |  |
| n38 | 4 | 15 | 100 | 100 | 100 | 100 |  |  |  |  |  |  |  |  |  |
| n38 | 66 | 15 | 100 | 100 | 100 | 100 |  |  |  |  |  |  |  |  |  |
| 38 | n1 | 15 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 |  |  |  |  |  |
| n1 | 38 | 15 | 100 | 100 | 100 | 100 |  |  |  |  |  |  |  |  |  |
| n40 | 1 | 15 | 25 | 50 | 75 | 100 |  |  |  |  |  |  |  |  |  |
| n41 | 4 | 30 | 128 | 128 | 128 | 128 |  |  |  |  |  |  |  |  |  |
| 40 | n1 | 15 | 25 | 50 | 75 | 100 |  |  |  |  |  |  |  |  |  |
| n40 | 7 | 30 | 216 | 216 | 216 | 216 |  |  |  |  |  |  |  |  |  |
| n41 | 1 | 30 | 128 | 128 | 128 | 128 |  |  |  |  |  |  |  |  |  |
| n41 | 2 | 30 | 160 | 160 | 160 | 160 |  |  |  |  |  |  |  |  |  |
| n41 | 3 | 30 | 160 | 160 | 160 | 160 |  |  |  |  |  |  |  |  |  |
| 41 | n1 | 15 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 |  |  |  |  |  |
| 41 | n3 | 15 | 25 | 50 | 75 | 100 | 100 | 100 | 100 | 100 |  |  |  |  |  |
| n41 | 66 | 30 | 128 | 128 | 128 | 128 |  |  |  |  |  |  |  |  |  |
| n41 | 25 | 30 | 160 | 160 | 160 | 160 |  |  |  |  |  |  |  |  |  |
| n50 | 3 | 30 | 160 | 160 | 160 | 160 |  |  |  |  |  |  |  |  |  |
| n71 | 12 | 15 | 20 | 20 |  |  |  |  |  |  |  |  |  |  |  |
| n77 | 7 | 30 | 270 | 270 | 270 | 270 |  |  |  |  |  |  |  |  |  |
| n77 | 41 | 30 | 270 | 270 | 270 | 270 |  |  |  |  |  |  |  |  |  |
| 41 | n77 | 15 |  | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 |
| n78 | 7 | 30 | 270 | 270 | 270 | 270 |  |  |  |  |  |  |  |  |  |
| n78 | 38 | 30 | 270 | 270 | 270 | 270 |  |  |  |  |  |  |  |  |  |
| n78 | 40 | 30 | 270 | 270 | 270 | 270 |  |  |  |  |  |  |  |  |  |
| n78 | 41 | 30 | 270 | 270 | 270 | 270 |  |  |  |  |  |  |  |  |  |
| n78 | 46 | 30 |  |  |  | 270 |  |  |  |  |  |  |  |  |  |
| 41 | n78 | 15 |  | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 |
| n79 | 42 | 30 | 2705 | 2705 | 2705 | 2705 |  |  |  |  |  |  |  |  |  |
| n84 | 3 | 15 | 25 | 25 | 25 | 25 |  |  |  |  |  |  |  |  |  |
| 48 | n46 | 15 |  |  |  | 216 |  |  | 216 |  | 216 |  | 216 |  |  |
| n46 | 48 | 30 | 216 | 216 | 216 | 216 |  |  | 216 | 216 | 216 |  | 216 | 216 | 216 |
| NOTE 1: The UL configuration applies regardless of the channel bandwidth of the UL band. UL resource blocks allocation in the table shall be further limited to that specified in Table 7.3.1-2 in TS 36.101 [4] or Table 7.3.2-3 in TS 38.101-1 [2]. NOTE 2: The UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth. NOTE 3: When the maximum UL RB allocation “LCRB” value is less than the maximum transmission bandwidth configuration “NRB” defined in Table 5.3.2-1 in 38.101-1 [2] for the specified UL band SCS, the UL band should be configured using the lowest CBW that is compatible with the maximum specified LCRB value. NOTE 4: If the aggressor band is NR band, the test SCS and UL RB can be adjusted according to supported BW and lowest SCS supported by the UE. NOTE 5: The requirements only apply for UEs supporting inter-band ENDC with simultaneous Rx/Tx capability. Simultaneous Rx/Tx capability does not apply for UEs supporting band 42 with a n77 implementation only. These restrictions are applicable to related higher order configurations. NOTE X: The UL configuration are applicable to the case that interference of UL band 3rd order IMD product falls into the affected DL channels. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.3B.2.3.5	MSD for intermodulation interference due to dual uplink operation for EN-DC in NR FR1

7.3B.2.3.5.0	General

For EN-DC configurations in NR FR1 the UE may indicate capability of not supporting simultaneous dual uplink operation due to possible intermodulation interference overlapping in frequency to its own primary downlink channel bandwidth if

-	the intermodulation order is 2;

-	the intermodulation order is 3 when both operating bands are between 450 MHz – 960 MHz or between 1427 MHz – 2690 MHz

In the case for EN-DC configurations in NR FR1 for which the intermodulation products caused by dual uplink operation do not interfere with its own primary downlink channel bandwidth as defined in Annex I the UE is mandated to operate in dual and triple uplink mode.

For EN-DC configurations in NR FR1 with uplink and downlink assigned to E-UTRA and NR FR1 bands given in Table 7.3B.2.3.5.1-1, Table 7.3B.2.3.5.1-1a, Table 7.3B.2.3.5.2-0 and Table 7.3B.2.3.5.2-1 the reference sensitivity is defined only for the specific uplink and downlink test points specified in Table 7.3B.2.3.5.1-1, Table 7.3B.2.3.5.1-1a, Table 7.3B.2.3.5.2-0 and Table 7.3B.2.3.5.2-1. For these test points the reference sensitivity levels specified in clause 7.3.1 in TS 36.101 [4] and 7.3.2 of TS 38.101-1 [2] for the corresponding channel bandwidths or in clause 7.3.1 of TS 36.101 [4] are relaxed by the amount of the parameter MSD given in Table 7.3B.2.3.5.1-1, Table 7.3B.2.3.5.1-1a, Table 7.3B.2.3.5.2-0 and Table 7.3B.2.3.5.2-1.

The throughput on each of the CGs shall be ≥ 95% of the maximum throughput of the respective reference measurement channels as specified in Annex A of TS 38.101-1 [2] and Annex A of TS 36.101 [4], with parameters specified in Table 7.3B.2.3.5.1-1, Table 7.3B.2.3.5.1-1a, Table 7.3B.2.3.5.2-0 and Table 7.3B.2.3.5.2-1 with dual UL transmissions overlapping in time unless otherwise stated.

7.3B.2.3.5.1	MSD test points for intermodulation interference due to dual uplink operation for PC3 EN-DC in NR FR1 involving two bands

Table 7.3B.2.3.5.1-1: MSD test points for PCell due to dual uplink operation for EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_1_n3 | 1 | 1950 | 5 | 25 | 2140 | 23 | IMD3 |
|  | n3 | 1760 | 5 | 25 | 1855 | N/A | N/A |
| DC_1A-n5A | 1 | 1965 | 5 | 25 | 2155 | 6 | IMD4 |
|  | n5 | 836.5 | 5 | 25 | 876.5 | N/A | N/A |
| DC_1A_n8A | 1 | 1965 | 5 | 25 | 2155 | 6.0 | IMD4 |
|  | n8 | 887.5 | 5 | 25 | 932.5 | N/A | N/A |
| DC_1A_n71A DC_1A_n71B | 1 | 1958 | 5 | 25 | 2148 | N/A | N/A |
|  | n71 | 668 | 5 | 25 | 622 | 15.1 | IMD3 |
| DC_1A_n77A, DC_1A_SUL_n77A-n84A, DC_1A_n77(2A), | 1 | 1950 | 5 | 25 | 2140 | 29.8 | IMD23 |
|  |  |  |  |  |  | 32.54 |  |
|  | n77 | 4090 | 10 | 50 | 4090 | N/A | N/A |
| DC_1A_n77A, DC_1A_SUL_n77A-n84A, DC_1A_n77(2A), DC_1A_n77(3A), DC_1A_n78A, DC_1A_SUL_n78A-n84A, DC_1A_n78(2A) | 1 | 1950 | 5 | 25 | 2140 | 8.0 | IMD43 |
|  |  |  |  |  |  | 10.74 |  |
|  | n77, n78 | 3710 | 10 | 50 | 3710 | N/A | N/A |
| DC_2A_n46A | 2 | 1880 | 5 | 25 | 1960 | 12.0 | IMD3 |
|  | n46 | 5720 | 20 | 100 | 5720 | N/A | N/A |
| DC_2A_n48A | 2 | 1852.5 | 5 | 25 | 1932.5 | 12 | IMD4 |
|  | n48 | 3625 | 20 | 100 | 3625 | N/A | N/A |
| DC_2A_n66A, DC_2A-2A_n66A DC_2A_n66(2A) | 2 | 1855 | 5 | 25 | 1935 | 20 | IMD3 |
|  | n66 | 1775 | 5 | 25 | 2175 | N/A | N/A |
| DC_2A_n66A, DC_2A-2A_n66A DC_2A_n66(2A) | 2 | 1883.3 | 5 | 25 | 1963.3 | N/A | N/A |
|  | n66 | 1750 | 5 | 25 | 2150 | 4 | IMD5 |
| DC_2A_n77A DC_2A-2A_n77A | 2 | 1855 | 5 | 25 | 1935 | 26 | IMD2 |
|  |  |  |  |  |  | 28.74 |  |
|  | n77 | 3790 | 10 | 50 | 3790 | N/A | N/A |
|  | 2 | 1900 | 5 | 25 | 1980 | 8.0 | IMD4 |
|  |  |  |  |  |  | 10.74 |  |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | N/A |
|  | 2 | 1885 | 5 | 25 | 1965 | 5 | IMD5 |
|  |  |  |  |  |  | 7.74 |  |
|  | n77 | 3810 | 10 | 50 | 3810 | N/A | N/A |
| DC_2A_n78A DC_2A_n78(2A) DC_2A-2A_n78(2A) | 2 | 1855 | 5 | 25 | 1935 | 26 | IMD23 |
|  |  |  |  |  |  | 28.74 |  |
|  | n78 | 3790 | 10 | 50 | 3790 | N/A | N/A |
|  | 2 | 1885 | 5 | 25 | 1965 | 8.0 | IMD43 |
|  |  |  |  |  |  | 10.74 |  |
|  | n78 | 3690 | 10 | 50 | 3690 | N/A | N/A |
| DC_3_n1 | 3 | 1760 | 5 | 25 | 1855 | N/A | N/A |
|  | n1 | 1950 | 5 | 25 | 2140 | 23 | IMD3 |
| DC_3_n5 | 3 | 1771 | 10 | 50 | 1866 | 4 | IMD4 |
|  | n5 | 838 | 5 | 25 | 883 | N/A | N/A |
|  | 3 | 1721 | 10 | 50 | 1816 | N/A | N/A |
|  | n5 | 838 | 5 | 25 | 883 | 24 | IMD23 |
| DC_3A_n7A DC_3C_n7A | 3 | 1730 | 5 | 25 | 1825 | N/A | N/A |
|  | n7 | 2535 | 10 | 50 | 2655 | 10.2 | IMD4 |
| DC_3_n8 | n8 | 900 | 5 | 25 | 945 | 8 | IMD43 |
|  | 3 | 1755 | 10 | 50 | 1850 | N/A | N/A |
|  | n8 | 897.5 | 5 | 25 | 942.5 | N/A | N/A |
|  | 3 | 1747.5 | 10 | 50 | 1842.5 | 6.4 | IMD5 |
| DC_3A_n20A | 3 | 1775 | 5 | 25 | 1870 | 4 | IMD4 |
| DC_3C_n20A | n20 | 840 | 5 | 25 | 799 | N/A | N/A |
|  | 3 | 1735 | 5 | 25 | 1830 | N/A | N/A |
|  | n20 | 847 | 5 | 25 | 806 | 9 | IMD4 |
| DC_3A_n38A | 3 | 1713 | 5 | 25 | 1808 | 8.2 | IMD4 |
|  | n38 | 2617 | 5 | 25 | 2617 | N/A | N/A |
| DC_3A_n41A DC_3C_n41A DC_3A_SUL_n41A-n80A, DC_3C_SUL_n41A-n80A | 3 | 1740 | 5 | 25 | 1835 | 8.2 | IMD4 |
|  | n41 | 2657.5 | 10 | 50 | 2657.5 | N/A | N/A |
| DC_3A_n77A, DC_3A_n77(2A), DC_3A_n77(3A), DC_3A_SUL_n77A-n80A, DC_3A_n78A, DC_3A_SUL_n78A-n80A, DC_3A_n78(2A), DC_3C_n78A DC_3C_n78(2A) | 3 | 1740 | 5 | 25 | 1835 | 26 | IMD23 |
|  |  |  |  |  |  | 28.74 |  |
|  | n77, n78 | 3575 | 10 | 50 | 3575 | N/A | N/A |
| DC_3A_n77A, DC_3A_n77(2A), DC_3C_n77A, DC_3C_n77(2A), DC_3A_SUL_n77A-n80A, DC_3A_n78A, DC_3A_SUL_n78A-n80A, DC_3A_n78(2A), DC_3C_n78A DC_3C_n78(2A) | 3 | 1765 | 5 | 25 | 1860 | 8.0 | IMD43 |
|  |  |  |  |  |  | 10.74 |  |
|  | n77, n78 | 3435 | 10 | 50 | 3435 | N/A | N/A |
| DC_4A_n2A | 2 | 1860 | 20 | 502 | 1940 | 5 | IMD3 |
|  | 4 | 1752.5 | 5 | 25 | 2152.5 | N/A | N/A |
|  | 2 | 1868.3 | 5 | 25 | 1948.3 | N/A | N/A |
|  | 4 | 1735 | 5 | 25 | 2135 | 5 | IMD5 |
| DC_4A_n5A | n5 | 838 | 5 | 25 | 883 | 30 | IMD23 |
|  | 4 | 1721 | 5 | 25 | 2121 | N/A | N/A |
| DC_4A_n7A | 4 | 1730 | 5 | 25 | 2130 | N/A | N/A |
|  | n7 | 2535 | 10 | 50 | 2655 | 15 | IMD4 |
| DC_5A_n3A | 5 | 838 | 5 | 25 | 883 | N/A | N/A |
|  | n3 | 1771 | 10 | 50 | 1866 | 4 | IMD4 |
|  | 5 | 838 | 5 | 25 | 883 | 24 | IMD23 |
|  | n3 | 1721 | 10 | 50 | 1816 | N/A | N/A |
| DC_5_n7 | n7 | 2547 | 10 | 50 | 2667 | N/A | N/A |
|  | 5 | 834 | 5 | 25 | 879 | 12 | IMD33 |
| DC_5_n38 | 5 | 844 | 5 | 25 | 889 | 12 | IMD33 |
|  | n38 | 2577 | 10 | 50 | 2577 | N/A | N/A |
| DC_5A_n66A | 5 | 838 | 5 | 25 | 883 | 30 | IMD23 |
|  | n66 | 1721 | 5 | 25 | 2121 | N/A | N/A |
| DC_5A_n77A8 | 5 | 844 | 5 | 25 | 889 | 8.3 | IMD4 |
|  | n77 | 3421 | 10 | 50 | 3421 | N/A | N/A |
|  | 5 | 826.5 | 5 | 25 | 871.5 | 5.5 | IMD5 |
|  | n77 | 4177.5 | 10 | 50 | 4177.5 | N/A | N/A |
| DC_5A_n78A DC_5A_n78(2A) DC_5A_n78C | 5 | 844 | 5 | 25 | 889 | 8.3 | IMD4 |
|  | n78 | 3421 | 10 | 50 | 3421 | N/A | N/A |
| DC_7_n3 | 7 | 2535 | 10 | 50 | 2655 | 13 | IMD4 |
|  | n3 | 1730 | 5 | 25 | 1825 | N/A | N/A |
| DC_7_n5 | 7 | 2547 | 10 | 50 | 2667 | N/A | N/A |
|  | n5 | 834 | 5 | 25 | 879 | 12 | IMD33 |
| DC_7A_n20A | 7 | 2512 | 10 | 50 | 2632 | N/A | N/A |
|  | n20 | 851 | 5 | 25 | 810 | 12 | IMD33 |
| DC_7_n40 | 7 | 2510 | 5 | 25 | 2630 | 23 | IMD3 |
|  | n40 | 2390 | 5 | 25 | 2390 | N/A | N/A |
| DC_7A_n66A DC_7A-7A_n66A DC_7C_n66A | 7 | 2535 | 10 | 50 | 2655 | 15 | 4th IMD |
|  | n66 | 1730 | 5 | 25 | 2130 | N/A | N/A |
| DC_7A_n77A DC_7A-7A_n77(2A) DC_7A_n77(2A) DC_7C_n77A DC_7C_n77(2A) | 7 | 2540 | 5 | 25 | 2660 | 7.1 | IMD4 |
|  | n77 | 3870 | 10 | 50 | 3870 | N/A | N/A |
| DC_7_n79 | 7 | 2510 | 5 | 25 | 2630 | [8] | IMD4 |
|  | n79 | 4900 | 40 | 216 | 4900 | N/A | N/A |
| DC_8A_n1A | 8 | 887.5 | 5 | 25 | 932.5 | N/A | N/A |
|  | n1 | 1965 | 5 | 25 | 2155 | 6 | IMD4 |
| DC_8A_n3A | 8 | 900 | 5 | 25 | 945 | 8 | IMD43 |
|  | n3 | 1755 | 10 | 50 | 1850 | N/A | N/A |
|  | 8 | 897.5 | 5 | 25 | 942.5 | N/A | N/A |
|  | n3 | 1747.5 | 10 | 50 | 1842.5 | 6.4 | IMD5 |
| DC_8A_n20A | n20 | 849.5 | 5 | 25 | 808.5 | 25 | IMD33 |
|  | 8 | 890.5 | 5 | 25 | 935.5 | N/A | N/A |
|  | n20 | 847.5 | 5 | 25 | 806.5 | N/A | N/A |
|  | 8 | 892.5 | 5 | 25 | 937.5 | 25 | IMD33 |
| DC_8A_n41A DC_8A_SUL_n41A-n81A | 8 | 882.5 | 5 | 25 | 927.5 | 12.1 | IMD33 |
|  | n41 | 2685 | 10 | 50 | 2685 | N/A | N/A |
| DC_8A_n77A, DC_8A_n78A, DC_8A_n78(2A), DC_8A_n77(3A), DC_8A_SUL_n78A-n81A | 8 | 897.5 | 5 | 25 | 942.5 | 8.3 | IMD4 |
|  | n77, n78 | 3635 | 10 | 50 | 3635 | N/A | N/A |
| DC_8A_n79A, DC_8A-n79C, DC_8A_SUL_n79A-n81A | 8 | 897.5 | 5 | 25 | 942.5 | 4.8 | IMD5 |
|  | n79 | 4532.5 | 40 | 216 | 4532.5 | N/A | N/A |
| DC_11A_n28A | 11 | 1430.5 | 5 | 25 | 1478.5 | N/A | N/A |
|  | n28 | 743 | 5 | 25 | 798 | 10.4 | IMD4 |
| DC_12A_n77A | 12 | 702 | 5 | 20 | 732 | 5.5 | IMD5 |
|  | n77 | 3540 | 10 | 50 | 3540 | N/A | N/A |
| DC_12_n78 | 12 | 710 | 5 | 25 | 740 | 5.5 | IMD5 |
|  | n78 | 3580 | 10 | 50 | 3580 | N/A | N/A |
| DC_13_n5 | 13 | 783 | 5 | 25 | 752 | N/A | N/A |
|  | n5 | 828 | 5 | 25 | 873 | 25 | IMD3 |
| DC_13A_n7A DC_13A_n7(2A) | 13 | 784.5 | 5 | 25 | 753.5 | N/A | N/A |
|  | n7 | 2520 | 40 | 216 | 2640 | 2.5 | IMD5 |
| DC_13A_n77A | 13 | 784.5 | 5 | 20 | 753.5 | 5.5 | IMD5 |
|  | n77 | 3891.5 | 10 | 50 | 3891.5 | N/A | N/A |
| DC_14A_n5A | 14 | 791 | 5 | 25 | 761 | N/A | N/A |
|  | n5 | 836 | 5 | 25 | 881 | 25 | IMD3 |
|  | 14 | 795.5 | 5 | 25 | 765.5 | 25 | IMD3 |
|  | n5 | 826.5 | 5 | 25 | 871.5 | N/A | N/A |
| DC_14A_n77A | 14 | 795.5 | 5 | 15 | 765.5 | 5.5 | IMD5 |
|  | n77 | 3947.5 | 10 | 50 | 3947.5 | N/A | N/A |
| DC_18A_n3A | 18 | 823 | 5 | 25 | 868 | N/A | N/A |
|  | n3 | 1721 | 5 | 25 | 1816 | 4 | IMD4 |
| DC_18A_n77A DC_18A_n78A | 18 | N/A | N/A | N/A | N/A | N/A | IMD4 |
|  | n77, n78 | N/A | N/A | N/A | N/A | N/A | N/A |
| DC_19A_n78A | 19 | N/A | N/A | N/A | N/A | N/A | IMD4 |
|  | n78 | N/A | N/A | N/A | N/A | N/A | N/A |
| DC_20A_n3A | 20 | 840 | 5 | 25 | 799 | N/A | N/A |
|  | n3 | 1775 | 5 | 25 | 1870 | 4 | IMD4 |
|  | 20 | 847 | 5 | 25 | 806 | 9 | IMD4 |
|  | n3 | 1735 | 5 | 25 | 1830 | N/A | N/A |
| DC_20A_n38A | 20 | N/A | N/A | N/A | N/A | N/A | IMD5 |
|  | n38 | N/A | N/A | N/A | N/A | N/A | N/A |
| DC_20_n7 | 20 | 851 | 5 | 25 | 810 | 12 | IMD33 |
|  | n7 | 2512 | 10 | 50 | 2632 | N/A | N/A |
| DC_20A_n8A | 20 | 849.5 | 5 | 25 | 808.5 | 25 | IMD3 |
|  | n8 | 892.5 | 5 | 25 | 937.5 | 25 | IMD3 |
| DC_20_n41 | 20 | 851 | 5 | 25 | 810 | 12.1 | IMD3 |
|  | n41 | 2512 | 10 | 50 | 2512 | N/A | N/A |
| DC_20_n41 | 20 | 841 | 5 | 25 | 800 | 8.1 | IMD5 |
|  | n41 | 2564 | 10 | 50 | 2564 | N/A | N/A |
| DC_20A_n77A, DC_20A_n78A DC_20A_n78C7, DC_20A_n78(2A), DC_20A_SUL_n78A-n82A | 20 | 850 | 5 | 25 | 809 | 11 | IMD4 |
|  | n77, n78 | 3359 | 10 | 50 | 3359 | N/A | N/A |
| DC_20A_n77A | 20 | 840 | 5 | 25 | 799 | 6.5 | IMD5 |
|  | n77 | 4159 | 10 | 50 | 4159 | N/A | N/A |
| DC_21A_n28A7 | 21 | 1450.4 | 5 | 25 | 1498.4 | 2.5 | IMD5 |
|  | n28 | 735.5 | 5 | 25 | 790.5 | N/A | N/A |
| DC_21A_n79A | 21 | 1457.5 | 5 | 25 | 1505.5 | 18.4 | IMD3 |
|  | n79 | 4420.5 | 40 | 216 | 4420.5 | N/A | N/A |
| DC_25A_n77A DC_25A-25A_n77A | 25 | 1855 | 5 | 25 | 1935 | 26 | IMD2 |
|  | n77 | 3790 | 10 | 50 | 3790 | N/A | N/A |
|  | 25 | 1900 | 5 | 25 | 1980 | 8 | IMD4 |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | N/A |
|  | 25 | 1885 | 5 | 25 | 1965 | 5 | IMD5 |
|  | n77 | 3810 | 10 | 50 | 3810 | N/A | N/A |
| DC_25A_n78A DC_25A-25A_n78A | 25 | 1855 | 5 | 25 | 1935 | 26 | IMD2 |
|  | n78 | 3790 | 10 | 50 | 3790 | N/A | N/A |
|  | 25 | 1885 | 5 | 25 | 1965 | 8 | IMD4 |
|  | n78 | 3690 | 10 | 50 | 3690 | N/A | N/A |
|  | 25 | 1875 | 5 | 25 | 1955 | 5 | IMD5 |
|  | n78 | 3790 | 10 | 50 | 3790 | N/A | N/A |
| DC_26A_n41A | 26 | 839 | 5 | 25 | 884 | 15.6 | IMD33 |
|  | n41 | 2562 | 10 | 50 | 2562 | N/A | N/A |
| DC_28_n50 | 28 | 730 | 10 | 50 | 775 | 15.3 | IMD 2 |
|  | n50 | 1500 | 10 | 50 | 1500 | N/A | N/A |
|  | 28 | 740 | 10 | 50 | 785 | 6 | IMD 4 |
|  | n50 | 1500 | 10 | 50 | 1500 | N/A | N/A |
|  | 28 | 740 | 10 | 50 | 785 | 0.5 | IMD 5 |
|  | n50 | 1500 | 10 | 50 | 1500 | N/A | N/A |
| DC_28A_n51A | 28 | 742.3 | 5 | 25 | 797.3 | 5 | IMD4 |
|  | n51 | 1429.5 | 5 | 25 | 1429.5 | N/A | N/A |
| DC_26A_n77A, DC_26A_n78A | 26 | 836.5 | 5 | 25 | 881.5 | 11.1 | IMD4 |
|  | n77, n78 | 3391 | 10 | 50 | 3391 | N/A | N/A |
| DC_28A_n77A, DC_28A_n78A, DC_28A_n78(2A), DC_28A_SUL_n78A-n83A | 28 | 705.5 | 5 | 25 | 760.5 | 5.5 | IMD5 |
|  | n77, n78 | 3582.5 | 10 | 50 | 3582.5 | N/A | N/A |
| DC_30A_n77A | 30 | 2310 | 5 | 25 | 2355 | 8.0 | IMD4 |
|  | n77 | 3487.5 | 10 | 50 | 3487.5 | N/A | N/A |
| DC_38A_n3A | n3 | 1713 | 5 | 25 | 1808 | 8.2 | IMD4 |
|  | 38 | 2617 | 5 | 25 | 2617 | N/A | N/A |
| DC_38A_n8A | 38 | 2617.5 | 5 | 25 | 2617.5 | N/A | N/A |
|  | n8 | 887.5 | 5 | 25 | 932.5 | 8.1 | IMD5 |
| DC_41A_n3A DC_41C_n3A | n3 | 1740 | 5 | 25 | 1835 | 8.2 | IMD4 |
|  | 41 | 2657.5 | 5 | 25 | 2657.5 | N/A | N/A |
| DC_42_n3 | 42 | 3575 | 10 | 50 | 3575 | N/A | N/A |
|  | n3 | 1740 | 5 | 25 | 1835 | 26 | 2nd3 |
|  |  |  |  |  |  | 28.74 |  |
|  | 42 | 3435 | 10 | 50 | 3435 | N/A | N/A |
|  | n3 | 1765 | 5 | 25 | 1860 | 8.0 | 4th3 |
|  |  |  |  |  |  | 10.74 |  |
| DC_42_n28 | 42 | 3582.5 | 10 | 50 | 3582.5 | N/A | N/A |
|  | n28 | 705.5 | 5 | 25 | 760.5 | 5.5 | IMD5 |
| DC_48A_n12A | 48 | 3557.5 | 10 | 50 | 3557.5 | N/A | N/A |
|  | n12 | 705.5 | 5 | 25 | 735.5 | 5.5 | IMD5 |
| DC_48A_n25A DC_48C_n25A DC_48D_n25A | 48 | 3625 | 20 | 100 | 3625 | N/A | N/A |
|  | n25 | 1852.5 | 5 | 25 | 1932.5 | 12 | IMD4 |
| DC_48A_n66A DC_48C_n66A DC_48D_n66A | 48 | 3630 | 20 | 100 | 3630 | N/A | N/A |
|  | n66 | 1715 | 5 | 25 | 2115 | 4 | IMD5 |
| DC_66A_n2A, DC_66A-66A_n2A | 66 | 1775 | 5 | 25 | 2175 | N/A | N/A |
|  | n2 | 1855 | 5 | 25 | 1935 | 20 | IMD3 |
|  | 66 | 1750 | 5 | 25 | 2150 | 4 | IMD5 |
|  | n2 | 1883.3 | 5 | 25 | 1963.3 | N/A | N/A |
| DC_66A_n5A | n5 | 838 | 5 | 25 | 883 | 30 | IMD23 |
|  | 66 | 1721 | 5 | 25 | 2121 | N/A | N/A |
| DC_66A_n7A DC_66A-66A_n7A DC_66A_n7(2A) DC_66A-66A_n7(2A) | 66 | 1730 | 5 | 25 | 2130 | N/A | N/A |
|  | n7 | 2535 | 10 | 50 | 2655 | 15 | IMD4 |
| DC_66A_n25A | 66 | 1775 | 5 | 25 | 2175 | N/A | N/A |
|  | n25 | 1855 | 5 | 25 | 1935 | 20 | IMD3 |
|  | 66 | 1712.5 | 5 | 25 | 2112.5 | 23 | IMD3 |
|  | n25 | 1912.5 | 5 | 25 | 1992.5 | N/A | N/A |
|  | 66 | 1750 | 5 | 25 | 2150 | 4 | IMD5 |
|  | n25 | 1883.3 | 5 | 25 | 1963.3 | N/A | N/A |
| DC_66A_n46A | 66 | 1735 | 5 | 25 | 2135 | 12.0 | IMD3 |
|  | n46 | 5605 | 20 | 100 | 5605 | N/A | N/A |
| DC_66A_n48A | 66 | 1715 | 5 | 25 | 2115 | 4 | IMD5 |
|  | n48 | 3630 | 20 | 100 | 3630 | N/A | N/A |
| DC_66A_n71A | 66 | 1750 | 5 | 25 | 2150 | 5 | IMD4 |
|  | n71 | 675 | 5 | 25 | 629 | N/A | N/A |
| DC_66A_n77A DC_66-66_n77A DC_66-66-66_n77A | 66 | 1775 | 5 | 25 | 2175 | 31.0 | IMD2 |
|  | n77 | 3950 | 10 | 50 | 3950 | N/A | N/A |
|  | 66 | 1760 | 5 | 25 | 2160 | 5.0 | IMD5 |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | N/A |
| DC_66A_n78A | 66 | 1730 | 5 | 25 | 2150 | 5.0 | IMD5 |
|  | n78 | 3660 | 10 | 50 | 3660 | N/A | N/A |
| DC_71A_n38A | 71 | 665 | 5 | 25 | 619 | 11 | IMD4 |
|  | n38 | 2614 | 5 | 25 | 2614 | N/A | N/A |
| DC_71A_n41A | 71 | 666 | 5 | 25 | 620 | 11 | IMD4 |
|  | n41 | 2618 | 5 | 25 | 2618 | N/A | N/A |
| DC_71A_n66A | 71 | 675 | 5 | 25 | 629 | N/A | N/A |
|  | n66 | 1750 | 5 | 25 | 2150 | 5 | IMD4 |
| DC_71A_n78A | 71 | 681.5 | 5 | 25 | 635.5 | 5.5 | IMD5 |
| DC_71A_n78(2A) | n78 | 3361.5 | 10 | 50 | 3361.5 | N/A | N/A |
| NOTE 1: Both of the transmitters shall be set min(+20 dBm, PCMAX_L,c) as defined in clause 6.2.5A. NOTE 2: RBstart = 0 NOTE 3: This band is subject to IMD5 also which MSD is not specified. NOTE 4: Applicable only if operation with 4 antenna ports is supported in the band with EN-DC configured. NOTE 5: Void NOTE 6: For NR band, UL/DL BW and UL LCRB can be adjusted according to the supported BW and lowest SCS supported by the UE. NOTE 7: The frequency range in band n28 is restricted for this band combination to 728 - 738 MHz for the UL and 783 - 793 MHz for the DL. This band is subject to IMD2, IMD4 and IMD5 fall in n28 also which MSD is not specified. In addition, this band is subject to IMD4 fall in B21 also which MSD is not specified. NOTE 8: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 from TS 38.101-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |

Table 7.3B.2.3.5.1-1a: MSD test points for PCell due to dual uplink operation for PC2 EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_3A_n41A | 3 | 1740 | 5 | 25 | 1835 | 18.4 | IMD4 |
|  | n41 | 2657.5 | 10 | 50 | 2657.5 | N/A | N/A |
| DC_3A_n78A | 3 | 1740 | 5 | 25 | 1835 | 31.9 | IMD2 |
|  | n78 | 3575 | 10 | 50 | 3575 | N/A | N/A |
| DC_3A_n78A | 3 | 1765 | 5 | 25 | 1860 | 18.5 | IMD4 |
|  | n78 | 3435 | 10 | 50 | 3435 | N/A | N/A |
| DC_1A_n78A | 1 | 1950 | 5 | 25 | 2140 | 17.8 | IMD4 |
|  | n78 | 3710 | 10 | 50 | 3710 | N/A | N/A |
| DC_8A_n78A | 8 | 897.5 | 5 | 25 | 942.5 | 15.5 | IMD4 |
|  | n78 | 3635 | 10 | 50 | 3635 | N/A | N/A |
| DC_2A_n77A DC_2A-2A_n77A DC_2A_n77C DC_2A-2A_n77C | 2 | 1855 | 5 | 25 | 1935 | 32.10 | IMD2 |
|  |  |  |  |  |  | 34.852 |  |
|  | n77 | 3790 | 10 | 50 | 3790 | N/A | N/A |
|  | 2 | 1900 | 5 | 25 | 1980 | 19.10 | IMD41 |
|  |  |  |  |  |  | 21.852 |  |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | N/A |
| DC_5A_n77A3 DC_5A_n77C3 | 5 | 844 | 5 | 25 | 889 | 18.60 | IMD41 |
|  | n77 | 3421 | 10 | 50 | 3421 | N/A | N/A |
| DC_13A_n77A DC_13A_n77C | 13 | 782 | 5 | 20 | 751 | 15.37 | IMD5 |
|  | n77 | 3879 | 10 | 50 | 3879 | N/A | N/A |
| DC_66A_n77A DC_66A-66A_n77A DC_66A-66A-66A_n77A DC_66A_n77C DC_66A-66A_n77C DC_66A-66A-66A_n77C | 66 | 1775 | 5 | 25 | 2175 | 34.33 | IMD2 |
|  | n77 | 3950 | 10 | 50 | 3950 | N/A | N/A |
|  | 66 | 1760 | 5 | 25 | 2160 | 11.27 | IMD5 |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | N/A |
| DC_5A_n78A | 5 | 844 | 5 | 25 | 889 | 17.5 | IMD4 |
|  | n78 | 3421 | 10 | 52 | 3421 | N/A | N/A |
| DC_12A_n77A | 12 | 702 | 5 | 20 | 732 | 11.7 | IMD5 |
|  | n77 | 3540 | 10 | 50 | 3540 | N/A | N/A |
| DC_14A_n77A | 14 | 795.5 | 5 | 15 | 765.5 | 11.7 | IMD5 |
|  | n77 | 3947.5 | 10 | 50 | 3947.5 | N/A | N/A |
| DC_30A_n77A | 30 | 2310 | 5 | 25 | 2355 | 17.6 | IMD4 |
|  | n77 | 3487.5 | 10 | 50 | 3487.5 | N/A | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. NOTE 2: Applicable only if operation with 4 antenna ports is supported in the band with EN-DC configured. NOTE 3: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 from TS 38.101-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |

7.3B.2.3.5.2	MSD test points for intermodulation interference due to dual uplink operation for EN-DC in NR FR1 involving three bands

Table 7.3B.2.3.5.2-0: MSD test points for Pcell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_66A-(n)71AA | 66 | 1750 | 5 | 25 | 2150 | 5 | IMD4 |
|  | n71 | 678 | 10 | 10 (RBstart =0) | 632 | N/A | N/A |
| NOTE 1: For NR band, UL/DL BW and UL LCRB can be adjusted according to the supported BW and lowest SCS supported by the UE. |  |  |  |  |  |  |  |

<< Next changes >>

7.3B.3.3	Inter-band EN-DC within FR1

7.3B.3.3.1	ΔRIB,c for EN-DC in two bands

Table 7.3B.3.3.1-1: ΔRIB,c due to EN-DC(two bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1_n28 | n28 | 0.2 |
| DC_1_n51 | n51 | 0.1 |
| DC_1_n77 | 1 | 0.2 |
|  | n77 | 0.5 |
| DC_1_n78 | n78 | 0.5 |
| DC_2_n30 | 2 | 0.4 |
| DC_2-2_n30 | n30 | 0.5 |
| DC_2_n48 | 2 | 0.2 |
|  | n48 | 0.5 |
| DC_2_n66 | 2 | 0.3 |
| DC_2-2_n66 | n66 | 0.3 |
| DC_2_n77 DC_2-2_n77 | 2 | 0.2 |
|  | n77 | 0.5 |
| DC_2_n78, DC_2-2_n78 | 2 | 0.2 |
|  | n78 | 0.5 |
| DC_3_n41 | n41 | 03 |
|  |  | 0.54 |
| DC_3_n51 | 3 | 0.2 |
|  | n51 | 0.2 |
| DC_3_n77, DC_3-3_n77 | 3 | 0.2 |
|  | n77 | 0.5 |
| DC_3_n78, DC_3-3_n78 | 3 | 0.2 |
|  | n78 | 0.5 |
| DC_4_n2 | 4 | 0.3 |
|  | n2 | 0.3 |
| DC_4_n28 | n28 | 0.2 |
| DC_4_n38 | 4 | 0.5 |
|  | n38 | 0.5 |
| DC_4_n41 | 4 | 0.5 |
|  | n41 | 0.51 |
|  |  | 12 |
| DC_4_n78 | 4 | 0.2 |
|  | n78 | 0.5 |
| DC_5_n3 | 5 | 0.2 |
|  | n3 | 0.2 |
| DC_5_n12 | 5 | 0.5 |
|  | n12 | 0.3 |
| DC_5_n77 | 5 | 0.2 |
|  | n77 | 0.5 |
| DC_5_n78 | 5 | 0.2 |
|  | n78 | 0.5 |
| DC_7_n8, DC_7-7_n8 | n8 | 0.2 |
| DC_7_n40 | n40 | 0.5 |
| DC_7_n51 | n51 | 0.2 |
| DC_7_n66, DC_7-7_n66 | 7 | 0.5 |
|  | n66 | 0.5 |
| DC_7_n71 | n71 | 0.2 |
| DC_7_n77, DC_7-7_n77 | n77 | 0.5 |
| DC_7_n78, DC_7-7_n78 | n78 | 0.5 |
| DC_7_n79 | n79 | 0.5 |
| DC_8_n7 | 8 | 0.2 |
| DC_8_n28 | 8 | 0.2 |
|  | n28 | 0.1 |
| DC_8_n77 | 8 | 0.2 |
|  | n77 | 0.5 |
| DC_8_n78 | 8 | 0.2 |
|  | n78 | 0.5 |
| DC_11_n3 | 11 | 0.3 |
|  | n3 | 0.5 |
| DC_11_n28 | n28 | 0.2 |
| DC_11_n77 | n77 | 0.5 |
| DC_11_n78 | n78 | 0.5 |
| DC_12_n5 | 12 | 0.3 |
|  | n5 | 0.5 |
| DC_12_n66 | 12 | 0.5 |
| DC_12_n71 | 12 | 0.8 |
|  | n71 | 0.8 |
| DC_12_n77 | 12 | 0.2 |
|  | n77 | 0.5 |
| DC_12_n78 | 12 | 0.2 |
|  | n78 | 0.5 |
| DC_13_n7 | 13 | 0.5 |
|  | n7 | 0.5 |
| DC_13_n77 | 13 | 0.2 |
|  | n77 | 0.5 |
| DC_13_n78 | 13 | 0.2 |
|  | n78 | 0.5 |
| DC_14_n77 | 14 | 0.2 |
|  | n77 | 0.5 |
| DC_18_n41 | n41 | 03 |
| DC_18_n77 | n77 | 0.5 |
| DC_19_n77 | n77 | 0.5 |
| DC_19_n78 | n78 | 0.5 |
| DC_20_n38 | 20 | 0.2 |
| DC_20_n51 | n51 | 0.2 |
| DC_20_n77 | n77 | 0.5 |
| DC_20_n78 | n78 | 0.5 |
| DC_21_n77 | n77 | 0.5 |
| DC_21_n78 | n78 | 0.5 |
| DC_25_n41, DC_25-25_n41 | n41 | 01 |
|  |  | 0.52 |
| DC_25_n77 DC_25-25_n77 | 25 | 0.2 |
|  | n77 | 0.5 |
| DC_25_n78 DC_25-25_n78 | 25 | 0.2 |
|  | n78 | 0.5 |
| DC_26_n77 | n77 | 0.5 |
| DC_26_n78 | n78 | 0.5 |
| DC_28_n1 | 28 | 0.2 |
| DC_28_n8 | 28 | 0.1 |
|  | n8 | 0.2 |
| DC_28_n51 | n51 | 0.2 |
| DC_28_n66 | 28 | 0.2 |
| DC_28_n77 | 28 | 0.2 |
|  | n77 | 0.5 |
| DC_28_n78 | 28 | 0.2 |
|  | n78 | 0.5 |
| DC_30_n66 | 30 | 0.5 |
|  | n66 | 0.4 |
| DC_30_n77 | n77 | 0.5 |
| DC_38_n78 | 38 | 0.4 |
|  | n78 | 0.5 |
| DC_38_n79 | n79 | 0.5 |
| DC_39_n40 | 39 | 0.3 |
|  | n40 | 0.3 |
| DC_39_n41 | 39 | 0.2 |
|  | n41 | 0.2 |
| DC_39_n78 | n78 | 0.5 |
| DC_39_n79 | n79 | 0.5 |
| DC_40_n77 | 40 | 0.4 |
|  | n77 | 0.5 |
| DC_40_n78 | 40 | 0.45 |
|  | n78 | 0.55 |
| DC_40_n79 | n79 | 0.5 |
| DC_41_n3 | 41 | 03 |
|  |  | 0.54 |
| DC_41_n77 | n77 | 0.5 |
| DC_41_n78 | n78 | 0.5 |
| DC_41_n79 | n79 | 0.5 |
| DC_42_n1 | 42 | 0.5 |
| DC_42_n3 | 42 | 0.5 |
|  | n3 | 0.2 |
| DC_42_n28 | 42 | 0.2 |
|  | n28 | 0.5 |
| DC_42_n51 | n51 | 0.2 |
| DC_48_n25 | 48 | 0.5 |
|  | n25 | 0.2 |
| DC_48_n46 | 48 | 0.5 |
| DC_48_n66 | 48 | 0.5 |
|  | n66 | 0.2 |
| DC_66_n2 | 66 | 0.3 |
| DC_66-66_n2 DC_66-66-66_n2 | n2 | 0.3 |
| DC_66_n7 | 66 | 0.5 |
| DC_66-66_n7 | n7 | 0.5 |
| DC_66_n12 | 66 | 0.5 |
| DC_66_n25 | 66 | 0.3 |
|  | n25 | 0.3 |
| DC_66_n28 | n28 | 0.2 |
| DC_66_n30 | 66 | 0.5 |
| DC_66-66_n30 | n30 | 0.4 |
| DC_66_n38, | 66 | 0.5 |
| DC_66-66_n38 | n38 | 0.5 |
| DC_66_n41 | 66 | 0.5 |
|  | n41 | 0.51 |
|  |  | 12 |
| DC_66_n48, DC_66-66_n48 | 66 | 0.2 |
|  | n48 | 0.5 |
| DC_66_n77 DC_66-66_n77 DC_66-66-66_n77 | 66 | 0.2 |
|  | n77 | 0.5 |
| DC_66_n78, | 66 | 0.2 |
| DC_66-66_n78 | n78 | 0.5 |
| DC_71_n38 | 71 | 0.2 |
| DC_71_n41 | 71 | 0.2 |
| DC_71_n78 | 71 | 0.2 |
|  | n78 | 0.5 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545 – 2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496 – 2545 MHz. NOTE 3: Applicable for the frequency range of 2515 – 2690 MHz. NOTE 4: Applicable for the frequency range of 2496 – 2515 MHz. NOTE 5: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. |  |  |

<< End of changes >>