# 3GPP TSG-RAN WG4 Meeting # 102-e	R4-2203967

# Electronic Meeting, February 21 – March 3, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 0994 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Big CR on Introducing NR inter-band CA for 3DL Bands and 1UL band for 38.101-1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CA_R17_3BDL_1BUL |  |  |  |  |  | Date: |  |  | 2022-03-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In terms of the agreements captured in TR38.862 (R4-2119074): In RAN4 #102-e meeting, each rapporteur should use the new formats in their rapporteur big CR to reflect the approved TPs and endorsed CRs. Therefore, this big CR use the new formats to reflect the completed inter-band CA combinations are introduced into TS 38.101-1 from RAN4 #101bis-e and #102 meetings. This big CR is to merge all endorsed CR and to capture newly introduced band combinations in the respective TP for NR inter-band CA for 3DL/1UL and 3DL/2UL CA configurations(Only for table 5.2A.2.2). |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add operating bands in Table 5.2A.2.2-1 in section 5.2A.2.2. - Add configurations for these band combinations in Table 5.5A.3.2-1in section 5.5A.3.2. - Add ΔTIB,c in Table 6.2A.4.2.4-1 in section 6.2A.4.2.4. - Add ΔRIB,c in Table 7.3A.3.2.3-1 in section 7.3A.3.2.3. The 2UL/3DL CA/DC configurations approved in the following contributions are added: - R4-2200190, Draft CR for 38.101-1: support of n77(2A) in 2UL CA_n1A-n77-n79A, SoftBank Corp. - R4-2200351, BCS corrections for CA_n7-n66-n77 and CA_n66-n71-n77, Nokia, [Bell Mobility, Telus] - R4-2201048, Draft CR for 38.101-1 to introduce CA_n3A-n28A-n77(3A) and CA_n1A-n3A-n77(2A), Samsung, KDDI - R4-2201064, Draft CR for 38.101-1 to introduce new configurations to CA_n5-n25-n77 and CA_n5-n66-n77 with 2UL, Samsung, Telus, Bell Mobility - R4-2201087, draftCR to add n78(2A) to CA_n7A-n28A-n78 already in 38.101-1, Nokia, BT - R4-2201516, DraftCR for 38.101-1: Correction on CA_n7-n66-n78, Huawei, HiSilicon, Bell Mobility, Telus - R4-2201517, DraftCR for 38.101-1: CA_n25(2A)-n38A-n66(2A), Huawei, HiSilicon, Bell Mobility, Telus - R4-2201518, DraftCR for 38.101-1: additional combinations for CA_n7-n25-n66, Huawei, HiSilicon, Bell Mobility, Telus - R4-2201547, draft CR 38.101-1 to add new configurations for CA_n25-n41-n66, CA_n25-n41-n71, CA_n25-n66-n71, CA_n41-n66-n71, CA_n66-n71-n77 - R4-2200202, TP for TR 38.717-03-02: CA_n1-n3-n79, SoftBank Corp. - R4-2200204, TP for TR 38.717-03-02: CA_n1-n28-n41, SoftBank Corp. - R4-2200205, TP for TR 38.717-03-02: CA_n1-n28-n77, SoftBank Corp. - R4-2200206, TP for TR 38.717-03-02: CA_n1-n28-n79, SoftBank Corp. - R4-2200208, TP for TR 38.717-03-02: CA_n1-n41-n77, SoftBank Corp. - R4-2200355, TP for CA_n28-n78-n79 for TR 38.717-03-02, NTT DOCOMO, INC. MediaTek Inc. - R4-2201036, TP for TR 38.717-03-02: CA_n1A-n3A-n18A, Samsung, KDDI - R4-2201037, TP for TR 38.717-03-02: CA_n1A-n18A-n28A, Samsung, KDDI - R4-2201038, TP for TR 38.717-03-02: CA_n1A-n18A-n41A, Samsung, KDDI - R4-2201039, TP for TR 38.717-03-02: CA_n1A-n18A-n77A, Samsung, KDDI - R4-2201043, TP for TR 38.717-03-02: CA_n3A-n18A-n28A, Samsung, KDDI - R4-2201044, TP for TR 38.717-03-02: CA_n3A-n18A-n77A, Samsung, KDDI - R4-2201045, TP for TR 38.717-03-02: CA_n18A-n28A-n41A, Samsung, KDDI - R4-2201046, TP for TR 38.717-03-02: CA_n18A-n28A-n77A, Samsung, KDDI - R4-2201047, TP for TR 38.717-03-02: CA_n18A-n41A-n77A, Samsung, KDDI - R4-2201053, TP for TR 38.717-03-02: CA_n2-n5-n48, Samsung, Verizon - R4-2201054, TP for TR 38.717-03-02: CA_n2-n48-n66, Samsung, Verizon - R4-2201055, TP for TR 38.717-03-02: CA_n2-n48-n77, Samsung, Verizon - R4-2201056, TP for TR 38.717-03-02: CA_n5-n48-n66, Samsung, Verizon - R4-2201057, TP for TR 38.717-03-02: CA_n5-n48-n77, Samsung, Verizon - R4-2201114, TP to TR 38.717-03-02: Addition of CA_n12-n30-n66, Nokia, AT&T - R4-2201116, TP to TR 38.717-03-02: Addition of CA_n2-n12-n30, Nokia, AT&T - R4-2201117, TP to TR 38.717-03-02: Addition of CA_n2-n12-n66, Nokia, AT&T - R4-2201118, TP to TR 38.717-03-02: Addition of CA_n2-n29-n30, Nokia, AT&T - R4-2201119, TP to TR 38.717-03-02: Addition of CA_n2-n29-n66, Nokia, AT&T - R4-2201560, TP for TR 38.717-03-02: CA_n1-n3-n20, Ericsson, BT plc - R4-2201563,TP for TR 38.717-03-02: CA_n1-n3-n67, Ericsson, BT plc - R4-2201567,TP for TR 38.717-03-02: CA_n1-n20-n67, Ericsson, BT plc - R4-2201569, TP for TR 38.717-03-02: CA_n3-n20-n67, Ericsson, BT plc - R4-2202187, TP to TR 38.717-03-02 Addition of CADC_n7A-n46-n78A, Nokia, BT - R4-2202188, TP to TR 38.717-03-02: Addition of CA_n29-n30-n66，Nokia, AT&T - R4-2202190, TP for TR 38.717-03-02: CA_n41-n70-n78, Ericsson - R4-2204130, Draft CR for 38.101-1: support of DL n77(2A) in 2UL CA of CA_n1A-n28A-n77, SoftBank Corp. - R4-2205565, draftCR to add BCS for CA_n1A-n40A-n78A to 38.101-1, Nokia - R4-2205698, draft CR to add new configurations for CA_n3-n7-n78, Ericsson, BT plc - R4-2205699, draft CR to add new configurations for CA_n3-n7-n28 and CA_n3-n28-n78, Ericsson, BT plc - R4-2205694, TP for TR 38.717-03-02: CA_n66-n70-n78, Ericsson - R4-2206250, TP for TR38.717-03-02_CA_n28A-n40A-n41A, ZTE - R4-2206251, TP for TR 38.717-03-02: CA_n41-n66-n70, Ericsson - R4-2206383, TP to TR 38.717.03-02 for CA_n46-n48-n96 The 1UL/3DL CA/DC configurations approved in the following contributions are added: - R4-2201024 TP for TR 38.717-03-01 CA_n1-n3-n18 - R4-2201025 TP for TR 38.717-03-01 CA_n1-n18-n28 - R4-2201026 TP for TR 38.717-03-01 CA_n1-n18-n41 - R4-2201027 TP for TR 38.717-03-01 CA_n1-n18-n77 - R4-2201028 TP for TR 38.717-03-01 CA_n1-n28-n41 - R4-2201029 TP for TR 38.717-03-01 CA_n1-n28-n77 - R4-2201030 TP for TR 38.717-03-01 CA_n1-n41-n77 - R4-2201031 TP for TR 38.717-03-01 CA_n3-n18-n28 - R4-2201032 TP for TR 38.717-03-01 CA_n3-n18-n77 - R4-2201033 TP for TR 38.717-03-01 CA_n18-n28-n41 - R4-2201034 TP for TR 38.717-03-01 CA_n18-n28-n77 - R4-2201035 TP for TR 38.717-03-01 CA_n18-n41-n77 - R4-2201108 TP to TR 38.717-03-01 Addition of CA_n12-n30-n66 - R4-2201109 TP to TR 38.717-03-01 Addition of CA_n29-n30-n66 - R4-2201110 TP to TR 38.717-03-01 Addition of CA_n2-n12-n30 - R4-2201111 TP to TR 38.717-03-01 Addition of CA_n2-n12-n66 - R4-2201112 TP to TR 38.717-03-01 Addition of CA_n2-n29-n30 - R4-2201113 TP to TR 38.717-03-01 Addition of CA_n2-n29-n66 - R4-2201552 TP for TR 38.717-03-01 to include CA_n41-n70-n78 - R4-2201566 TP for TR 38.717-03-01 to include CA_n1-n20-n67 - R4-2201568 TP for TR 38.717-03-01 to include CA_n3-n20-n67 - R4-2201724 TP for TR 38.717-03-01 to include CA_n2-n71-n78 - R4-2201725 TP for TR 38.717-03-01 to include CA_n2-n66-n78 - R4-2200200 Draft CR_n3-n28-n41_tibrib - R4-2201063 Draft CR for 38.101-1 to introduce new configurations to CA_n5-n25-n77 and CA_n5-n66-n77 with 1UL - R4-2202184 draft CR to 38101-1-h40 improve note for CA_n20-n28 and higher order combos - R4-2202185 Draft CR for 38.101-1 to introduce new configurations to CA_n5-n48-n77 and CA_n48-n66-n77 with 1UL - R4-2205171 draft CR for 38.101-1 CA_n7-n25-n66 - R4-2205172 draft CR for 38.101-1 CA_n25-n38-n66 - R4-2206382 TP to TR TR38.717-03-01 for CA_n46-n48-n96 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | These band combinations would not be supported in Rel-17. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2A.2.2, 5.5A.3.2, 6.2A.4.2.4, 7.3A.3.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## << Start of change 1>>

#### 5.2A.2.2	Inter-band CA (three bands)

Table 5.2A.2.2-1: Inter-band CA operating bands involving FR1 (three bands)

| NR CA Band | NR Band (Table 5.2-1) | DL interruption allowed (Note 4) |
| --- | --- | --- |
| CA_n1-n3-n5 | n1, n3, n5 |  |
| CA_n1-n3-n7 | n1, n3, n7 |  |
| CA_n1-n3-n8 | n1, n3, n8 |  |
| CA_n1-n3-n18 | n1, n3, n18 |  |
| CA_n1-n3-n20 | n1, n3, n20 |  |
| CA_n1-n3-n28 | n1, n3, n28 |  |
| CA_n1-n3-n413 | n1, n3, n41 |  |
| CA_n1-n3-n77 | n1, n3, n77 |  |
| CA_n1-n3-n783 | n1, n3, n78 | No for CA_n1-n78, CA_n3-n78 |
| CA_n1-n3-n793 | n1, n3, n79 |  |
| CA_n1-n5-n7 | n1, n5, n7 |  |
| CA_n1-n5-n28 | n1, n5, n28 |  |
| CA_n1-n5-n78 | n1, n5, n78 |  |
| CA_n1-n7-n8 | n1, n7, n8 |  |
| CA_n1-n7-n28 | n1, n7, n28 |  |
| CA_n1-n7-n783 | n1，n7, n78 |  |
| CA_n1-n8-n28 | n1, n8, n28 |  |
| CA_n1-n8-n77 | n1, n8, n77 |  |
| CA_n1-n8-n783 | n1, n8, n78 |  |
| CA_n1-n8-n79 | n1, n8, n79 |  |
| CA_n1-n18-n28 | n1, n18, n28 |  |
| CA_n1-n18-n41 | n1, n18, n41 |  |
| CA_n1-n18-n77 | n1, n18, n77 |  |
| CA_n1-n20-n67 | n1, n20, n67 |  |
| CA_n1-n20-n78 | n1, n20, n78 |  |
| CA_n1A-n28A-n40 | n1, n28, n40 |  |
| CA_n1-n28-n413 | n1, n28, n41 |  |
| CA_n1-n28-n773 | n1, n28, n77 |  |
| CA_n1-n28-n793 | n1, n28, n79 |  |
| CA_n1-n28-n783 | n1, n28, n78 |  |
| CA_n1-n40-n78 | n1, n40, n78 |  |
| CA_n1-n41-n773 | n1, n41, n77 |  |
| CA_n1-n77-n79 | n1, n77, n79 |  |
| CA_n1-n78-n79 | n1, n78, n79 |  |
| CA_n2-n5-n30 | n2, n5, n30 |  |
| CA_n2-n5-n48 | n2, n5, n48 |  |
| CA_n2-n5-n66 | n2, n5, n66 |  |
| CA_n2-n5-n77 | n2, n5, n77 |  |
| CA_n2-n12-n30 | n2, n12, n30 |  |
| CA_n2-n12-n66 | n2, n12, n66 |  |
| CA_n2-n12-n77 | n2, n12, n77 |  |
| CA_n2-n14-n30 | n2, n14, n30 |  |
| CA_n2-n14-n66 | n2, n14, n66 |  |
| CA_n2-n14-n77 | n2, n14, n77 |  |
| CA_n2-n29-n30 | n2, n29, n30 |  |
| CA_n2-n29-n66 | n2, n29, n66 |  |
| CA_n2-n29-n77 | n2, n29, n77 |  |
| CA_n2-n30-n66 | n2, n30, n66 |  |
| CA_n2-n30-n77 | n2, n30, n77 |  |
| CA_n2-n48-n66 | n2, n48, n66 |  |
| CA_n2-n48-n77 | n2, n48, n77 |  |
| CA_n2-n66-n77 | n2, n66, n77 |  |
| CA_n2-n66-n78 | n2, n66, n78 |  |
| CA_n2-n71-n78 | n2, n71, n78 |  |
| CA_n3-n5-n7 | n3, n5, n7 |  |
| CA_n3-n5-n28 | n3, n5, n28 |  |
| CA_n3-n5-n78 | n3, n5, n78 |  |
| CA_n3-n7-n8 | n3, n7, n8 |  |
| CA_n3-n7-n28 | n3, n7, n28 |  |
| CA_n3-n7-n783 | n3, n7, n78 |  |
| CA_n3-n8-n28 | n3, n8, n28 |  |
| CA_n3-n8-n77 | n3, n8, n77 |  |
| CA_n3-n8-n783 | n3, n8, n78 |  |
| CA_n3-n18-n28 | n3, n18, n28 |  |
| CA_n3-n18-n41 | n3, n18, n41 |  |
| CA_n3-n18-n77 | n3, n18, n77 |  |
| CA_n3-n20-n67 | n3, n20, n67 |  |
| CA_n3-n20-n78 | n3, n20, n78 |  |
| CA_n3-n28-n413 | n3, n28, n41 |  |
| CA_n3-n28-n773 | n3, n28, n77 |  |
| CA_n3-n28-n783 | n3, n28, n78 |  |
| CA_n3-n28-n793 | n3, n28, n79 |  |
| CA_n3-n40-n41 | n3, n40, n41 | No for CA n3-n40, CA n3-n41 |
| CA_n3-n41-n77 | n3, n41, n77 |  |
| CA_n3-n41-n78 | n3, n41, n78 |  |
| CA_n3-n41-n793 | n3, n41, n79 | No |
| CA_n3-n77-n79 | n3, n77, n79 |  |
| CA_n5-n7-n28 | n5, n7, n28 |  |
| CA_n5-n7-n78 | n5, n7, n78 |  |
| CA_n5-n12-n77 | n5, n12, n77 |  |
| CA_n5-n14-n77 | n5, n14, n77 |  |
| CA_n5-n25-n66 | n5, n25, n66 |  |
| CA_n5-n25-n77 | n5, n25, n77 |  |
| CA_n5-n25-n78 | n5, n25, n78 |  |
| CA_n5-n29-n77 | n5, n29, n77 |  |
| CA_n5-n30-n66 | n5, n30, n66 |  |
| CA_n5-n30-n77 | n5, n30, n77 |  |
| CA_n5-n48-n77 | n5, n48, n77 |  |
| CA_n5-n48-n66 | n5, n48, n66 |  |
| CA_n5-n66-n77 | n5, n66, n77 |  |
| CA_n5-n66-n78 | n5, n66, n78 |  |
| CA_n7-n8-n28 | n7, n8, n28 |  |
| CA_n7-n8-n78 | n7, n8, n78 |  |
| CA_n7-n25-n66 | n7, n25, n66 |  |
| CA_n7-n25-n77 | n7, n25, n77 |  |
| CA_n7-n25-n78 | n7, n25, n78 |  |
| CA_n7-n28-n78 | n7, n28, n78 |  |
| CA_n7-n46-n78 | n7, n46, n78 |  |
| CA_n7-n66-n78 | n7, n66, n78 |  |
| CA_n7-n66-n77 | n7, n66, n77 |  |
| CA_n8-n28-n783 | n8, n28, n78 |  |
| CA_n8-n39-n41 | n8, n39, n41 | No for CA n8-n41, CA n39-n41 |
| CA_n8-n40-n41 | n8, n40, n41 |  |
| CA_n8-n41-n793 | n8, n41, n79 | No |
| CA_n8-n78-n79 | n8, n78, n79 |  |
| CA_n12-n30-n66 | n12, n30, n66 |  |
| CA_n12-n30-n77 | n12, n30, n77 |  |
| CA_n12-n66-n77 | n12, n66, n77 |  |
| CA_n13-n25-n66 | n13, n25, n66 |  |
| CA_n13-n25-n77 | n13, n25, n77 |  |
| CA_n13-n66-n77 | n13, n66, n77 |  |
| CA_n14-n30-n66 | n14, n30, n66 |  |
| CA_n14-n30-n77 | n14, n30, n77 |  |
| CA_n14-n66-n77 | n14, n66, n77 |  |
| CA_n18-n28-n41 | n18, n28, n41 |  |
| CA_n18-n28-n77 | n18, n28, n77 |  |
| CA_n18-n41-n77 | n18, n41, n77 |  |
| CA_n20-n28-n78 | n20, n28, n78 |  |
| CA_n24-n41-n48 | n24, n41, n48 |  |
| CA_n24-n41-n77 | n24, n41, n77 |  |
| CA_n24-n48-n77 | n24, n48, n77 |  |
| CA_n25-n41-n77 | n25, n41, n77 |  |
| CA_n25-n29-n66 | n25, n29, n66 |  |
| CA_n25-n38-n78 | n25, n38, n78 |  |
| CA_n25-n41-n66 | n25, n41, n66 |  |
| CA_n25-n41-n71 | n25, n41, n71 |  |
| CA_n25-n41-n77 | n25, n41, n77 | No for CA_n1-n78, CA_n3-n78 |
| CA_n25-n41-n78 | n25, n41, n78 |  |
| CA_n25-n48-n66 | n25, n48, n66 |  |
| CA_n25-n66-n71 | n25, n66, n71 |  |
| CA_n25-n66-n77 | n25, n66, n77 |  |
| CA_n25-n66-n78 | n25, n66, n78 |  |
| CA_n25-n71-n77 | n25, n71, n77 |  |
| CA_n25-n71-n78 | n25, n71, n78 |  |
| CA_n26-n66-n70 | n26, n66, n70 |  |
| CA_n28-n40-n41 | n28, n40, n41 |  |
| CA_n28-n40-n79 | n28, n40, n79 |  |
| CA_n28-n41-n793 | n28, n41, n79 |  |
| CA_n28-n46-n78 | n28, n46, n78 |  |
| CA_n28-n77-n79 | n28, n77, n79 |  |
| CA_n28-n78-n79 | n28, n78, n79 |  |
| CA_n28-n40-n78 | n28, n40, n78 |  |
| CA_n28-n41-n77 | n28, n41, n77 |  |
| CA_n28-n41-n78 | n28, n41, n78 |  |
| CA_n29-n30-n66 | n29, n30, n66 |  |
| CA_n29-n30-n77 | n29, n30, n77 |  |
| CA_n29-n66-n70 | n29, n66, n70 |  |
| CA_n29-n66-n77 | n29, n66, n77 |  |
| CA_n30-n66-n77 | n30, n66, n77 |  |
| CA_n38-n66-n78 | n38, n66, n78 |  |
| CA_n39-n40-n41 | n39, n40, n41 |  |
| CA_n39-n40-n79 | n39, n40, n79 |  |
| CA_n39-n41-n79 | n39, n41, n79 | No |
| CA_n40-n41-n791,2 | n40, n41, n79 | No for CA n40-n79, CA n41-n79 |
| CA_ n41-n66-n71 | n41, n66, n71 |  |
| CA_n41-n66-n78 | n41, n66, n78 |  |
| CA_ n41-n66-n77 | n41, n66, n77 |  |
| CA_n41-n70-n78 | n41, n70, n78 |  |
| CA_n41-n71-n77 | n41, n71, n77 |  |
| CA_n41-n71-n78 | n41, n71, n78 |  |
| CA_n48-n66-n70 | n48, n66, n70 |  |
| CA_n48-n66-n71 | n48, n66, n71 |  |
| CA_n48-n66-n77 | n48, n66, n77 |  |
| CA_n48-n70-n71 | n48, n70, n71 |  |
| CA_n66-n70-n71 | n66, n70, n71 |  |
| CA_n66-n71-n77 | n66, n71, n77 |  |
| CA_n66-n71-n78 | n66, n71, n78 |  |
| NOTE 1: The frequency range below 2506 MHz for Band n41 is not used in this band combination. NOTE 2: Applicable for frequency range above 4800 MHz for Band n79 in this band combination. NOTE 3: Applicable for UE supporting inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability NOTE 4: Applicable when dynamic Tx switching is conducted. The DL interruption requirement is specified in clause 8.2.2.2.10 of 38.133 [13]. |  |  |

## << End of change 1>>

## << Start of change 2>>

#### 5.5A.3.2	Configurations for inter-band CA (three bands)

Table 5.5A.3.2-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier6 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n1A-n3A-n5A | CA_n1A-n3A CA_n1A-n5A CA_n3A-n5A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n5 | 5, 10, 15, 20 |  |
| CA_n1A-n3A-n7A | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  | CA_n1A-n3A CA_n1A-n7A CA_n3A-n7A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n1 | 5, 10, 15, 20 | 2 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
| CA_n1A-n3A-n7B | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n7 | CA_n7B_BCS0 |  |
|  | CA_n1A-n3A CA_n1A-n7A CA_n3A-n7A CA_n7B | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n7 | CA_n7B_BCS0 |  |
| CA_n1A-n3A-n8A | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n8 | 5, 10, 15, 20 |  |
| CA_n1A-n3A-n18A |  |  |  |  |  | CA_n1A-n3A CA_n1A-n18A CA_n3A-n18A |  |  |  |  |  | n1 |  |  |  |  |  | 5, 10, 15, 20, 25, 30, 40, 50 |  |  |  |  |  | 0 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n3 |  |  |  |  |  | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n18 |  |  |  |  |  | 5, 10, 15 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n1A-n3A-n20A | - | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n20 | 5, 10, 15, 20 |  |
| CA_n1A-n3A-n28A | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n28 | 5, 10, 15, 202 |  |
|  | CA_n1A-n3A CA_n1A-n28A CA_n3A-n28A | n1 | 5, 10, 15, 20 | 1 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 2 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n28 | 5, 10, 15, 201, 301 |  |
| CA_n1A-n3A-n41A | CA_n1A-n3A CA_n1A-n41A CA_n3A-n41A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n41 |  |  |  |  |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n1A-n3A-n67A |  |  |  |  |  | CA_n1A-n3A |  |  |  |  |  | n1 |  |  |  |  |  | 5, 10, 15, 20, 25, 30, 40, 50 |  |  |  |  |  | 0 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n3 |  |  |  |  |  | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n67 |  |  |  |  |  | 5, 10, 15, 20 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n1A-n3A-n77A | CA_n1A-n3A CA_n1A-n77A CA_n3A-n77A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n1A-n3A-n77(2A) | CA_n1A-n3A CA_n1A-n77A CA_n3A-n77A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n1A-n3A-n78A | CA_n1A-n3A CA_n1A-n78A7 CA_n3A-n78A7A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n1 | 5, 10, 15, 20 | 2 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n3A-n78(2A) | CA_n1A-n3A CA_n1A-n78A CA_n3A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n1A-n3A-n79A | CA_n1A-n3A CA_n1A-n79A CA_n3A-n79A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n1A-n5A-n7A | CA_n1A-n5A CA_n1A-n7A CA_n5A-n7A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
| CA_n1A-n5A-n7B | CA_n1A-n5A CA_n1A-n7A CA_n5A-n7A CA_n7B | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n7 | CA_n7B_BCS0 |  |
| CA_n1A-n5A-n28A | - | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  | 28 | 5, 10, 15, 20, 30 |  |
| CA_n1A-n5A-n78A | CA_n1A-n5A CA_n1A-n78A CA_n5A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 704, 80, 90, 100 |  |
| CA_n1A-n7A-n8A | - | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |
|  |  | n8 | 5, 10, 15, 20, 35 |  |
| CA_n1A-n8A-n28A | - | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n8 | 5, 10, 15, 20, 35 |  |
|  |  | n28 | 10, 15, 20 |  |
| CA_n1A-n8A-n77A | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n1A-n8A-n77(2A) | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n1A-n7A-n28A | CA_n1A-n7A CA_n1A-n28A CA_n7A-n28A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
| CA_n1A-n7B-n28A | CA_n1A-n28A CA_n1A-n7A CA_n7A-n28A CA_n7B | n1 | 5, 10, 15, 20 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
| CA_n1A-n7A-n78A | CA_n1A-n7A CA_n1A-n78A CA_n7A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 901, 100 |  |
|  |  | n1 | 5, 10, 15, 20 | 1 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 901, 100 |  |
| CA_n1A-n7B-n78A | CA_n1A-n78A CA_n1A-n7A CA_n7A-n78A CA_n7B | n1 | 5, 10, 15, 20 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 704, 80, 90, 100 |  |
| CA_n1A-n7A-n78(2A) | CA_n1A-n7A CA_n1A-n78A CA_n7A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
|  |  | n1 | 5, 10, 15, 20 | 1 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n1A-n8A-n78A | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n1 | 5, 10, 15, 20 | 1 |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
| CA_n1A-n8A-n78(2A) | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78(2A)_BCS1 |  |
| CA_n1A-n8A-n79A | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n1A-n18A-n28A |  |  |  |  |  | CA_n1A-n18A CA_n1A-n28A CA_n18A-n28A |  |  |  |  |  | n1 |  |  |  |  |  | 5, 10, 15, 20, 25, 30, 40, 50 |  |  |  |  |  | 0 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n18 |  |  |  |  |  | 5, 10, 15 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n28 |  |  |  |  |  | 5, 10 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n1A-n18A-n41A | CA_n1A-n18A CA_n1A-n41A CA_n18A-n41A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n18 | 5, 10, 15 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
| CA_n1A-n18A-n77A | CA_n1A-n18A CA_n1A-n77A CA_n18A-n77A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n18 | 5, 10, 15 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n1A-n20A-n67A |  |  |  |  |  | CA_n1A-n20A |  |  |  |  |  | n1 |  |  |  |  |  | 5, 10, 15, 20, 25, 30, 40, 50 |  |  |  |  |  | 0 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n20 |  |  |  |  |  | 5, 10, 15, 20 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n67 |  |  |  |  |  | 5, 10, 15, 20 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n1A-n20A-n78A | - | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n20 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n28A-n40A | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  |
| CA_n1A-n28A-n40B | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n40 | CA_n40B_BCS0 |  |
| CA_n1A-n28A-n41A | CA_n1A-n28A CA_n1A-n41A CA_n28A-n41A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
| CA_n1A-n28A-n77A | CA_n1A-n28A CA_n1A-n77A CA_n28A-n77A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n1A-n28A-n77(2A) |  |  |  |  |  | CA_n1A-n28A CA_n1A-n77A CA_n28A-n77A |  |  |  |  |  | n1 |  |  |  |  |  | 5, 10, 15, 20 |  |  |  |  |  | 0 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n28 |  |  |  |  |  | 5, 10, 15, 20 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  | CA_n77(2A)_BCS0 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n1A-n28A-n78A | CA_n1A-n28A CA_n1A-n78A CA_n28A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 202 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n1 | 5, 10, 15, 20 | 1 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n28A-n78(2A) | CA_n1A-n28A CA_n1A-n78A CA_n28A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n1A-n28A-n79A | CA_n1A-n28A CA_n1A-n79A CA_n28A-n79A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n79 |  |  |  |  |  | 40, 50, 60, 80, 100 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n1A-n3A-n20A |  |  |  |  |  | CA_n1A-n3A CA_n1A-n20A CA_n3A-n20A |  |  |  |  |  | n1 |  |  |  |  |  | 5, 10, 15, 20, 25, 30, 40, 50 |  |  |  |  |  | 0 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n3 |  |  |  |  |  | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n20 |  |  |  |  |  | 5, 10, 15, 20 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n1A-n40A-n78A | CA_n1A-n40A CA_n1A-n78A CA_n40A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  | CA_n1A-n40A CA_n1A-n78A CA_n40A-n78A | n1 | 5, 10, 15, 20 | 1 |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | CA_n1A-n40A CA_n1A-n78A CA_n40A-n78A |  |  |  |  |  | n1 |  |  |  |  |  | 5, 10, 15, 20 |  |  |  |  |  | 2 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n40 |  |  |  |  |  | 5, 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n1A-n40B-n78A | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n40 | CA_n40B_BCS0 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n1A-n41A-n77A | CA_n1A-n41A CA_n1A-n77A CA_n41A-n77A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n1A-n77A-n79A4 | CA_n1A-n77A CA_n1A-n79A CA_n77A-n79A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n79 |  |  |  |  |  | 40, 50, 60, 80, 100 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n1A-n77(2A)-n79A4 |  |  |  |  |  | CA_n1A-n77A CA_n1A-n79A CA_n77A-n79A |  |  |  |  |  | n1 |  |  |  |  |  | 5, 10, 15, 20 |  |  |  |  |  | 0 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  | CA_n77(2A)_BCS0 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n79 |  |  |  |  |  | 40, 50, 60, 80, 100 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n1A-n78A-n79A5 |  |  |  |  |  | CA_n1A-n78A CA_n1A-n79A CA_n78A-n79A |  |  |  |  |  | n1 |  |  |  |  |  | 5, 10, 15, 20 |  |  |  |  |  | 0 |  |  |  |  |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n1 | 5, 10, 15, 20 | 1 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n1A-n78(2A)-n79A | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n78 | CA_n78(2A)_BCS1 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n2A-n5A-n30A | CA_n2A-n5A CA_n2A-n30A CA_n5A-n30A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n30 | 5, 10 |  |
| CA_n2A-n5A-n48A | CA_n2A-n5A CA_n2A-n48A CA_n5A-n48A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n5A-n48B | CA_n2A-n5A CA_n2A-n48A CA_n5A-n48A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n48 | CA_n48B_BCS0 |  |
|  |  | n2 | 5, 10, 15, 20 | 1 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n48 | CA_n48B_BCS1 |  |
|  |  | n2 | 5, 10, 15, 20 | 2 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n48 | CA_n48B_BCS2 |  |
| CA_n2A-n5A-n48(2A) | CA_n2A-n5A CA_n2A-n48A CA_n5A-n48A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n2 | 5, 10, 15, 20 | 1 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n48 | CA_n48(2A)_BCS1 |  |
| CA_n2A-n5A-n48(A-B) | CA_n2A-n5A CA_n2A-n48A CA_n5A-n48A | n2 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n5 | 5, 10, 15, 20, 251 |  |
|  |  | n48 | CA_n48(A-B)_BCS0 |  |
|  |  | n2 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n5 | 5, 10, 15, 20, 251 |  |
|  |  | n48 | CA_n48(A-B)_BCS1 |  |
| CA_n2(2A)-n5A-n30A | CA_n2A-n5A CA_n2A-n30A CA_n5A-n30A | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n30 | 5, 10 |  |
| CA_n2A-n5A-n66A | CA_n2A-n5A CA_n2A-n66A CA_n5A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n2(2A)-n5A-n66A | CA_n2A-n5A CA_n2A-n66A CA_n5A-n66A | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n2A-n5A-n66(2A) | CA_n2A-n5A CA_n2A-n66A CA_n5A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n66 | CA_n66(2A)_BCS0 |  |
| CA_n2A-n5A-n77A | n777 CA_n2A-n5A CA_n2A-n77A7 CA_n5A-n77A7 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n5A-n77C | CA_n2A-n5A CA_n2A-n77A CA_n5A-n77A | n2 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n5 | 5, 10, 15, 20, 251 |  |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n2 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n5 | 5, 10, 15, 20, 251 |  |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n2A-n5A-n77(2A) | CA_n2A-n5A CA_n2A-n77A CA_n5A-n77A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2(2A)-n5A-n77A | CA_n2A-n5A CA_n2A-n77A CA_n5A-n77A | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n12A-n30A |  |  |  |  |  | CA_n2A-n12A CA_n2A-n30A CA_n12A-n30A |  |  |  |  |  | n2 |  |  |  |  |  | 5, 10, 15, 20 |  |  |  |  |  | 0 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n12 |  |  |  |  |  | 5, 10, 15 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n30 |  |  |  |  |  | 5, 10 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n2(2A)-n12A-n30A | CA_n2A-n12A CA_n2A-n30A CA_n12A-n30A | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n30 | 5, 10 |  |
| CA_n2A-n12A-n66A | CA_n2A-n12A CA_n2A-n66A CA_n12A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n2(2A)-n12A-n66A | CA_n2A-n12A CA_n2A-n66A CA_n12A-n66A | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n2A-n12A-n66(2A) | CA_n2A-n12A CA_n2A-n66A CA_n12A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n2(2A)-n12A-n66(2A) | CA_n2A-n12A CA_n2A-n66A CA_n12A-n66A | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n2A-n12A-n66(3A) | CA_n2A-n12A CA_n2A-n66A CA_n12A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n66 | CA_n66(3A)_BCS0 |  |
| CA_n2A-n12A-n77A | n777 CA_n2A-n12A CA_n2A-n77A7 CA_n12A-n77A7 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2(2A)-n12A-n77A | CA_n2A-n12A CA_n2A-n77A CA_n12A-n77A | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n12A-n77(2A) | CA_n2A-n12A CA_n2A-n77A CA_n12A-n77A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2A-n14A-n30A | CA_n2A-n14A CA_n2A-n30A CA_n14A-n30A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n14 | 5, 10 |  |
|  |  | n30 | 5, 10 |  |
| CA_n2(2A)-n14A-n30A | CA_n2A-n14A CA_n2A-n30A CA_n14A-n30A | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n14 | 5, 10 |  |
|  |  | n30 | 5, 10 |  |
| CA_n2A-n14A-n66A | CA_n2A-n14A CA_n2A-n66A CA_n14A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n14 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n2(2A)-n14A-n66A | CA_n2A-n14A CA_n2A-n66A CA_n14A-n66A | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n14 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n2A-n14A-n66(2A) | CA_n2A-n14A CA_n2A-n66A CA_n14A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n14 | 5, 10 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n2A-n14A-n77A | n777 CA_n2A-n14A CA_n2A-n77A7 CA_n14A-n77A7 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n14 | 5, 10 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n14A-n77(2A) | CA_n2A-n14A CA_n2A-n77A CA_n14A-n77A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n14 | 5, 10 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2(2A)-n14A-n77A | CA_n2A-n14A CA_n2A-n77A CA_n14A-n77A | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n14 | 5, 10 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n29A-n30A |  |  |  |  |  | CA_n2A-n30A |  |  |  |  |  | n2 |  |  |  |  |  | 5, 10, 15, 20 |  |  |  |  |  | 0 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n29 |  |  |  |  |  | 5, 10 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n30 |  |  |  |  |  | 5, 10 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n2(2A)-n29A-n30A | CA_n2A-n30A | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n30 | 5, 10 |  |
| CA_n2A-n29A-n66A | CA_n2A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n2(2A)-n29A-n66A | CA_n2A-n66A | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n2A-n29A-n66(2A) | CA_n2A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n2(2A)-n29A-n66(2A) | CA_n2A-n66A | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n2A-n29A-n77A | CA_n2-n77 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2(2A)-n29A-n77A | CA_n2-n77 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n29A-n77(2A) | CA_n2-n77 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2A-n30A-n66A | CA_n2A-n30A CA_n30A-n66A CA_n2A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
| CA_n2(2A)-n30A-n66A | CA_n2A-n30A CA_n30A-n66A CA_n2A-n66A | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
| CA_n2A-n30A-n66(2A) | CA_n2A-n30A CA_n30A-n66A CA_n2A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | CA_n66(2A)_BCS0 |  |
| CA_n2A-n30A-n77A | n777 CA_n2A-n30A CA_n2A-n77A7 CA_n30A-n77A7 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n30A-n77(2A) | CA_n2A-n30A CA_n2A-n77A CA_n30A-n77A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2(2A)-n30A-n77A | CA_n2A-n30A CA_n2A-n77A CA_n30A-n77A | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n48A-n66A | CA_n2A-n48A CA_n2A-n66A CA_n48A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n2A-n48(A-B)-n66A | CA_n2A-n48A CA_n2A-n66A CA_n48A-n66A | n2 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n48 | CA_n48(A-B)_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n2 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n48 | CA_n48(A-B)_BCS1 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n2A-n48B-n66A | CA_n2A-n48A CA_n2A-n66A CA_n48A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48B_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n2 | 5, 10, 15, 20 | 1 |
|  |  | n48 | CA_n48B_BCS1 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n2 | 5, 10, 15, 20 | 2 |
|  |  | n48 | CA_n48B_BCS2 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n2A-n48(2A)-n66A | CA_n2A-n48A CA_n2A-n66A CA_n48A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n2 | 5, 10, 15, 20 | 1 |
|  |  | n48 | CA_n48(2A)_BCS1 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n2A-n48A-n77A | CA_n2A-n48A CA_n2A-n77A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n48A-n77C | CA_n2A-n48A CA_n2A-n77A CA_n77C | n2 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n2 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n2A-n48B-n77A | CA_n2A-n48A CA_n2A-n77A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48B_BCS0 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n2 | 5, 10, 15, 20 | 1 |
|  |  | n48 | CA_n48B_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n2 | 5, 10, 15, 20 | 2 |
|  |  | n48 | CA_n48B_BCS2 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n48(2A)-n77A | CA_n2A-n48A CA_n2A-n77A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n2 | 5, 10, 15, 20 | 1 |
|  |  | n48 | CA_n48(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n66A-n77A | CA_n2A-n66A CA_n66A-n77A CA_n2A-n77A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2(2A)-n66A-n77A | CA_n2A-n66A CA_n66A-n77A CA_n2A-n77A | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n66(2A)-n77A | CA_n2A-n66A CA_n66A-n77A CA_n2A-n77A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n66A-n77C | CA_n2A-n66A CA_n66A-n77A CA_n2A-n77A | n2 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n2 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n2A-n66A-n77(2A) | CA_n2A-n66A CA_n66A-n77A CA_n2A-n77A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2A-n66A-n78A |  |  |  |  |  | - |  |  |  |  |  | n2 |  |  |  |  |  | 5, 10, 15, 20 |  |  |  |  |  | 0 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n66 |  |  |  |  |  | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n2A-n66A-n78(2A) | - | n2 | 5, 10, 15, 20 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n2A-n71A-n78A |  |  |  |  |  | - |  |  |  |  |  | n2 |  |  |  |  |  | 5, 10, 15, 20 |  |  |  |  |  | 0 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n71 |  |  |  |  |  | 5, 10, 15, 20 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n2A-n71A-n78(2A) | - | n2 | 5, 10, 15, 20 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n3A-n5A-n7A | - | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40 |  |
|  | CA_n3A-n5A CA_n3A-n7A CA_n5A-n7A | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
| CA_n3A-n5A-n7B | - | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n7 | CA_n7B_BCS0 |  |
|  | CA_n3A-n5A CA_n3A-n7A CA_n5A-n7A CA_n7B | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n7 | CA_n7B_BCS0 |  |
| CA_n3A-n5A-n28A | - | n3 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  | n28 | 5, 10, 15, 20, 30 |  |
| CA_n3A-n5A-n78A | CA_n3A-n5A CA_n3A-n78A CA_n5A-n78A | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n7A-n8A | - | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |
|  |  | n8 | 5, 10, 15, 20, 35 |  |
| CA_n3A-n7A-n28A | - | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
|  | CA_n3A-n7A CA_n3A-n28A CA_n7A-n28A | n3 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 2 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
| CA_n3A-n7B-n28A | - | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
|  | CA_n3A-n7A CA_n3A-n28A CA_n7A-n28A CA_n7B | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
| CA_n3A-n7A-n78A | - | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  |  |  |  |  |  |  |  |  |  | n7 |  |  |  |  |  | 5, 10, 15, 20, 25, 30, 40, 50 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A |  |  |  |  |  | n3 |  |  |  |  |  | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |  | 1 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n7 |  |  |  |  |  | 5, 10, 15, 20, 25, 30, 40, 50 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  | 10, 15, 20, 25, 30, 40, 50, 60, 704, 80, 90, 100 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n7B-n78A | - | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A CA_n7B |  |  |  |  |  | n3 |  |  |  |  |  | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |  | 1 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n7 |  |  |  |  |  | CA_n7B_BCS0 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  | 10, 15, 20, 25, 30, 40, 50, 60, 704, 80, 90, 100 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n7A-n78(2A) |  |  |  |  |  | CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A |  |  |  |  |  | n3 |  |  |  |  |  | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |  | 0 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n7 |  |  |  |  |  | 5, 10, 15, 20, 25, 30, 40, 50 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  | CA_n78(2A)_BCS2 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n8A-n28A | - | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n8 | 5, 10, 15, 20, 35 |  |
|  |  | n28 | 5, 10, 15, 20, 30 |  |
| CA_n3A-n8A-n77A | - | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n3A-n8A-n77(2A) | - | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n3A-n8A-n78A | CA_n3A-n8A CA_3A-n78A CA_n8A-n78A | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n3A-n18A-n28A |  |  |  |  |  | CA_n3A-n18A CA_n3A-n28A CA_n18A-n28A |  |  |  |  |  | n3 |  |  |  |  |  | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |  | 0 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n18 |  |  |  |  |  | 5, 10, 15 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n28 |  |  |  |  |  | 5, 10 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n18A-n41A | CA_n3A-n41A CA_n3A-n18A CA_n18A-n41A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n18 | 5, 10, 15 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
| CA_n3A-n18A-n77A |  |  |  |  |  | CA_n3A-n18A CA_n3A-n77A CA_n18A-n77A |  |  |  |  |  | n3 |  |  |  |  |  | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |  | 0 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n18 |  |  |  |  |  | 5, 10, 15 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n20A-n67A |  |  |  |  |  | CA_n3A-n20A |  |  |  |  |  | n3 |  |  |  |  |  | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |  | 0 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n20 |  |  |  |  |  | 5, 10, 15, 20 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n67 |  |  |  |  |  | 5, 10, 15, 20 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n20A-n78A | - | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n20 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n28A-n41A | CA_n3A-n28A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n28 | 5, 10, 15, 20, 30 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
| CA_n3A-n28A-n77A | CA_n3A-n28A CA_n3A-n77A CA_n28A-n77A | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n28 | 5, 10, 15, 20, 30 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n3A-n28A-n77(2A) | CA_n3A-n28A CA_n3A-n77A CA_n28A-n77A | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  | CA_n77(2A) | n28 | 5, 10, 15, 20 |  |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n28 | 5, 10, 15, 20, 30 |  |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
| CA_n3A-n28A-n77(3A) | CA_n3A-n28A CA_n3A-n77A CA_n28A-n77A | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n77 | CA_n77(3A)_BCS0 |  |
| CA_n3A-n28A-n78A | CA_n3A-n28A CA_n3A-n78A CA_n28A-n78A | n3 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 202 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n28 | 5, 10, 15, 202 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 | 2 |
|  |  | n28 | 5, 10 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n3A-n28A-n78(2A) | CA_n3A-n28A CA_n3A-n78A CA_n28A-n78A | n3 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 202 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  |  |  |  |  |  |  |  |  |  |  | n28 |  |  |  |  |  | 5, 10 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  | CA_n78(2A)_BCS2 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n3 |  |  |  |  |  | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |  | 2 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n28 |  |  |  |  |  | 5, 10, 15, 20 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  | CA_n78(2A)_BCS2 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n28A-n79A | CA_n3A-n28A CA_n3A-n79A CA_n28A-n79A | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n79 | 40, 50, 80, 100 |  |
| CA_n3A-n77A-n79A4 | CA_n3A-n77A CA_n3A-n79A CA_n77A-n79A | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n3A-n77(2A)-n79A4 | CA_n3A-n77A CA_n3A-n79A CA_n77A-n79A | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n3A-n40A-n41A | CA_n3A-n40A CA_n3A-n41A CA_n40A-n41A | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n3A-n41A-n77A | CA_n3A-n41A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  | CA_n3A-n77A | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  | CA_n41A-n77A | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n41A-n77(2A) | CA_n3A-n41A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  | CA_n3A-n77A | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  | CA_n41A-n77A | n77 | CA_n77(2A)_BCS0 |  |
| CA_n3A-n41A-n78A | - | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | CA_n3A-n41A | n3 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  | CA_n3A-n78A | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  | CA_n41A-n78A | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n3A-n41A-n78(2A) | CA_n3A-n41A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  | CA_n3A-n78A | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  | CA_n41A-n78A | n78 | CA_n78(2A)_BCS2 |  |
| CA_n3A-n41A-n79A | - | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30 | 1 |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n5A-n7A-n28A | - | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  | n7 | 5, 10, 15, 25, 30, 35, 40, 50 |  |
|  |  | n28 | 5, 10, 15, 20, 30 |  |
| CA_n5A-n7A-n78A | - | n5 | 5, 10, 15, 20 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | CA_n5A-n7A CA_n5A-n78A CA_n7A-n78A | n5 | 5, 10, 15, 20 | 1 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 704, 80, 90, 100 |  |
| CA_n5A-n7B-n78A | - | n5 | 5, 10, 15, 20 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | CA_n5A-n7A CA_n5A-n78A CA_n7A-n78A CA_n7B | n5 | 5, 10, 15, 20 | 1 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 704, 80, 90, 100 |  |
| CA_n5A-n12A-n77A | n777 CA_n5A-n12A CA_n5A-n77A7 CA_n12A-n77A7 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n12A-n77(2A) | CA_n5A-n12A CA_n5A-n77A CA_n12A-n77A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n12 | 5, 10, 15 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n5A-n14A-n77A | n777 CA_n5A-n14A CA_n5A-n77A7 CA_n14A-n77A7 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n14 | 5, 10 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n14A-n77(2A) | CA_n5A-n14A CA_n5A-n77A CA_n14A-n77A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n14 | 5, 10 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n5A-n25A-n66A | CA_n5A-n25A CA_n5A-n66A CA_n25A-n66A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n5A-n25(2A)-n66A | CA_n5A-n25A CA_n5A-n66A CA_n25A-n66A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n5A-n25A-n66(2A) | CA_n5A-n25A CA_n5A-n66A CA_n25A-n66A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n5A-n25(2A)-n66(2A) | CA_n5A-n25A CA_n5A-n66A CA_n25A-n66A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n5A-n25A-n77A | CA_n5A-n25A | n5 | 5, 10, 15, 20 | 0 |
|  | CA_n5A-n77A | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  | CA_n25A-n77A | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n25(2A)-n77A |  |  |  |  |  | CA_n5A-n25A CA_n5A-n77A CA_n25A-n77A |  |  |  |  |  | n5 |  |  |  |  |  | 5, 10, 15, 20 |  |  |  |  |  | 0 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n25 |  |  |  |  |  | CA_n25(2A)_BCS0 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n5A-n25A-n77(2A) | CA_n5A-n25A CA_n5A-n77A CA_n25A-n77A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n5A-n25(2A)-n77(2A) | CA_n5A-n25A CA_n5A-n77A CA_n25A-n77A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n5A-n25A-n78A | CA_n5A-n25A CA_n5A-n78A CA_n25A-n78A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n25(2A)-n78A | CA_n5A-n25A CA_n5A-n78A CA_n25A-n78A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n25A-n78(2A) | CA_n5A-n25A CA_n5A-n78A CA_n25A-n78A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n5A-n25(2A)-n78(2A) | CA_n5A-n25A CA_n5A-n78A CA_n25A-n78A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n5A-n29A-n77A | CA_n5A-n77A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n29A-n77(2A) | CA_n5A-n77A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n5A-n30A-n66A | CA_n5A-n30A CA_n30A-n66A CA_n5A-n66A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
| CA_n5A-n30A-n66(2A) | CA_n5A-n30A CA_n30A-n66A CA_n5A-n66A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | CA_n66(2A)_BCS0 |  |
| CA_n5A-n30A-n77A | n777 CA_n5A-n30A CA_n5A-n77A7 CA_n30A-n77A7 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n30A-n77(2A) | CA_n5A-n30A CA_n5A-n77A CA_n30A-n77A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n5A-n48A-n66A | CA_n5A-n48A CA_n5A-n66A CA_n48A-n66A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n5A-n48(A-B)-n66A | CA_n5A-n48A CA_n5A-n66A CA_n48A-n66A | n5 | 5, 10, 15, 20, 251 | 0 |
|  |  | n48 | CA_n48(A-B)_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n5 | 5, 10, 15, 20, 251 | 1 |
|  |  | n48 | CA_n48(A-B)_BCS1 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n5A-n48B-n66A | CA_n5A-n48A CA_n5A-n66A CA_n48A-n66A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48B_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n5 | 5, 10, 15, 20 | 1 |
|  |  | n48 | CA_n48B_BCS1 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n5 | 5, 10, 15, 20 | 2 |
|  |  | n48 | CA_n48B_BCS2 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n5A-n48(2A)-n66A | CA_n5A-n48A CA_n5A-n66A CA_n48A-n66A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n5 | 5, 10, 15, 20 | 1 |
|  |  | n48 | CA_n48(2A)_BCS1 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n5A-n48A-n77A | CA_n5A-n48A CA_n5A-n77A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n48A-n77C | CA_n5A-n48A CA_n5A-n77A CA_n77C | n5 | 5, 10, 15, 20, 251 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n5 | 5, 10, 15, 20, 251 | 1 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n5A-n48B-n77A | CA_n5A-n48A CA_n5A-n77A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48B_BCS0 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n5 | 5, 10, 15, 20 | 1 |
|  |  | n48 | CA_n48B_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n5 | 5, 10, 15, 20 | 2 |
|  |  | n48 | CA_n48B_BCS2 |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n5A-n48B-n77C |  |  |  |  |  | CA_n5A-n48A CA_n5A-n77A |  |  |  |  |  | n5 |  |  |  |  |  | 5, 10, 15, 20 |  |  |  |  |  | 0 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n48 |  |  |  |  |  | CA_n48B_BCS0 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  | CA_n77C_BCS0 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n5 |  |  |  |  |  | 5, 10, 15, 20 |  |  |  |  |  | 1 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n48 |  |  |  |  |  | CA_n48B_BCS0 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  | CA_n77C BCS0 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n5 |  |  |  |  |  | 5, 10, 15, 20 |  |  |  |  |  | 2 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n48 |  |  |  |  |  | CA_n48B_BCS1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  | CA_n77C BCS0 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n5 |  |  |  |  |  | 5, 10, 15, 20 |  |  |  |  |  | 3 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n48 |  |  |  |  |  | CA_n48B_BCS1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  | CA_n77C BCS1 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n5A-n48(2A)-n77A |  |  |  |  |  | CA_n5A-n48A CA_n5A-n77A |  |  |  |  |  | n5 |  |  |  |  |  | 5, 10, 15, 20 |  |  |  |  |  | 0 |  |  |  |  |  |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n5 | 5, 10, 15, 20 | 1 |
|  |  | n48 | CA_n48(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n48(2A)-n77C |  |  |  |  |  | CA_n5A-n48A CA_n5A-n77A |  |  |  |  |  | n5 |  |  |  |  |  | 5, 10, 15, 20 |  |  |  |  |  | 0 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n48 |  |  |  |  |  | CA_n48(2A)_BCS0 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  | CA_n77C_BCS0 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n5 |  |  |  |  |  | 5, 10, 15, 20 |  |  |  |  |  | 1 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n48 |  |  |  |  |  | CA_n48(2A)_BCS0 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  | CA_n77C_BCS1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n5 |  |  |  |  |  | 5, 10, 15, 20 |  |  |  |  |  | 2 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n48 |  |  |  |  |  | CA_n48(2A)_BCS1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  | CA_n77C_BCS0 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n5 |  |  |  |  |  | 5, 10, 15, 20 |  |  |  |  |  | 3 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n48 |  |  |  |  |  | CA_n48(2A)_BCS1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  | CA_n77C_BCS1 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n5A-n66A-n77A |  |  |  |  |  | CA_n5A-n66A CA_n66A-n77A CA_n5A-n77A |  |  |  |  |  | n5 |  |  |  |  |  | 5, 10, 15, 20 |  |  |  |  |  | 0 |  |  |  |  |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n66(2A)-n77A | CA_n5A-n66A CA_n66A-n77A CA_n5A-n77A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n66(2A)-n77(2A) |  |  |  |  |  | CA_n5A-n66A CA_n5A-n77A CA_n66A-n77A |  |  |  |  |  | n5 |  |  |  |  |  | 5, 10, 15, 20 |  |  |  |  |  | 0 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n66 |  |  |  |  |  | CA_n66(2A)_BCS1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  | CA_n77(2A)_BCS1 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n5A-n66A-n77C | CA_n5A-n66A CA_n66A-n77A CA_n5A-n77A | n5 | 5, 10, 15, 20, 251 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n5 | 5, 10, 15, 20, 251 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n5A-n66A-n77(2A) | CA_n5A-n66A CA_n66A-n77A CA_n5A-n77A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n5A-n66A-n78A | CA_n5A-n66A CA_n5A-n78A CA_n66A-n78A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n5 | 5, 10, 15, 20 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n66(2A)-n78A | CA_n5A-n66A CA_n5A-n78A CA_n66A-n78A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n66A-n78(2A) | CA_n5A-n66A CA_n5A-n78A CA_n66A-n78A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n5A-n66(2A)-n78(2A) | CA_n5A-n66A CA_n5A-n78A CA_n66A-n78A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n7A-n8A-n28A | - | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 | 0 |
|  |  | n8 | 5, 10, 15, 20, 35 |  |
|  |  | n28 | 5, 10, 15, 20, 30 |  |
| CA_n7A-n8A-n78A | - | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 | 0 |
|  |  | n8 | 5, 10, 15, 20, 35 |  |
|  |  | n78 | 10, 15, 20, 30 |  |
| CA_n7A-n25A-n66A | CA_n7A-n25A CA_n7A-n66A CA_n25A-n66A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n7A-n25(2A)-n66A |  |  |  |  |  | CA_n7A-n25A CA_n7A-n66A CA_n25A-n66A |  |  |  |  |  | n7 |  |  |  |  |  | 5, 10, 15, 20, 25, 30, 40, 50 |  |  |  |  |  | 0 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n25 |  |  |  |  |  | CA_n25(2A)_BCS0 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n66 |  |  |  |  |  | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n7A-n25(2A)-n66(2A) |  |  |  |  |  | CA_n7A-n25A CA_n7A-n66A CA_n25A-n66A |  |  |  |  |  | n7 |  |  |  |  |  | 5, 10, 15, 20, 25, 30, 40, 50 |  |  |  |  |  | 0 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n25 |  |  |  |  |  | CA_n25(2A)_BCS0 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n66 |  |  |  |  |  | CA_n66(2A)_BCS1 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n7A-n25A-n66(2A) |  |  |  |  |  | CA_n7A-n25A CA_n7A-n66A CA_n25A-n66A |  |  |  |  |  | n7 |  |  |  |  |  | 5, 10, 15, 20, 25, 30, 40, 50 |  |  |  |  |  | 0 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n25 |  |  |  |  |  | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n66 |  |  |  |  |  | CA_n66(2A)_BCS1 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n7(2A)-n25A-n66A |  |  |  |  |  | CA_n7A-n25A CA_n7A-n66A CA_n25A-n66A |  |  |  |  |  | n7 |  |  |  |  |  | CA_n7(2A)_BCS0 |  |  |  |  |  | 0 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n25 |  |  |  |  |  | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n66 |  |  |  |  |  | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n7(2A)-n25(2A)-n66A |  |  |  |  |  | CA_n7A-n25A CA_n7A-n66A CA_n25A-n66A |  |  |  |  |  | n7 |  |  |  |  |  | CA_n7(2A)_BCS0 |  |  |  |  |  | 0 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n25 |  |  |  |  |  | CA_n25(2A)_BCS0 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n66 |  |  |  |  |  | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n7(2A)-n25A-n66(2A) |  |  |  |  |  | CA_n7A-n25A CA_n7A-n66A CA_n25A-n66A |  |  |  |  |  | n7 |  |  |  |  |  | CA_n7(2A)_BCS0 |  |  |  |  |  | 0 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n25 |  |  |  |  |  | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n66 |  |  |  |  |  | CA_n66(2A)_BCS1 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n7(2A)-n25(2A)-n66(2A) |  |  |  |  |  | CA_n7A-n25A CA_n7A-n66A CA_n25A-n66A |  |  |  |  |  | n7 |  |  |  |  |  | CA_n7(2A)_BCS0 |  |  |  |  |  | 0 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n25 |  |  |  |  |  | CA_n25(2A)_BCS0 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n66 |  |  |  |  |  | CA_n66(2A)_BCS1 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n7A-n25A-n77A | CA_n7A-n25A CA_n7A_n77A CA_n25A-n77A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7A-n25(2A)-n77A | CA_n7A-n25A CA_n7A_n77A CA_n25A-n77A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7A-n25A-n77(2A) | CA_n7A-n25A CA_n7A_n77A CA_n25A-n77A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n7A-n25(2A)-n77(2A) | CA_n7A-n25A CA_n7A_n77A CA_n25A-n77A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n7(2A)-n25A-n77A | CA_n7A-n25A CA_n7A_n77A CA_n25A-n77A | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7(2A)-n25(2A)-n77A | CA_n7A-n25A CA_n7A_n77A CA_n25A-n77A | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7(2A)-n25A-n77(2A) | CA_n7A-n25A CA_n7A_n77A CA_n25A-n77A | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n7(2A)-n25(2A)-n77(2A) | CA_n7A-n25A CA_n7A_n77A CA_n25A-n77A | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n7A-n25A-n78A | CA_n7A-n25A CA_n7A-n78A CA_n25A-n78A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 704, 80, 904, 100 |  |
| CA_n7A-n25A-n78(2A) | CA_n7A-n25A CA_n7A-n78A CA_n25A-n78A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  |  |  |  |  |  |  |  |  |  |  | n25 |  |  |  |  |  | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  | CA_n78(2A)_BCS2 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n7A-n28A-n78(2A) |  |  |  |  |  | CA_n7A-n28A CA_n7A-n78A CA_n28A-n78A |  |  |  |  |  | n7 |  |  |  |  |  | 5, 10, 15, 20, 25, 30, 40, 50 |  |  |  |  |  | 0 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n28 |  |  |  |  |  | 5, 10, 15, 20 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  | CA_n78(2A)_BCS2 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n7A-n28A-n78A | - | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
|  | CA_n7A-n28A CA_n7A-n78A CA_n28A-n78A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 704, 80, 90, 100 |  |
| CA_n7B-n28A-n78A | - | n7 | CA_n7B_BCS0 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
|  | CA_n7A-n28A CA_n7A-n78A CA_n28A-n78A CA_n7B | n7 | CA_n7B_BCS0 | 1 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 704, 80, 90, 100 |  |
| CA_n7A-n46A-n78A | CA_n7A-n46A CA_n7A-n78A CA_n46A-n78A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n46 | 20, 40, 60, 80 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7A-n46C-n78A | CA_n7A-n46A CA_n7A-n78A CA_n46A-n78A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n46 | CA_n46C_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7A-n46D-n78A | CA_n7A-n46A CA_n7A-n78A CA_n46A-n78A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n46 | CA_n46D_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7A-n66A-n77A | CA_n7A-n66A CA_n7A-n77A CA_n66A-n77A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7A-n66(2A)-n77A | CA_n7A-n66A CA_n7A-n77A CA_n66A-n77A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7A-n66A-n77(2A) | CA_n7A-n66A CA_n7A-n77A CA_n66A-n77A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n7A-n66(2A)-n77(2A) | CA_n7A-n66A CA_n7A-n77A CA_n66A-n77A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n7(2A)-n66A-n77A | CA_n7A-n66A CA_n7A-n77A CA_n66A-n77A | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7(2A)-n66(2A)-n77A | CA_n7A-n66A CA_n7A-n77A CA_n66A-n77A | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7(2A)-n66A-n77(2A) | CA_n7A-n66A CA_n7A-n77A CA_n66A-n77A | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n7(2A)-n66(2A)-n77(2A) | CA_n7A-n66A CA_n7A-n77A CA_n66A-n77A | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n7A-n66A-n78A | CA_n7A-n66A CA_n7A-n78A CA_n66A-n78A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7A-n66A-n78(2A) | CA_n7A-n66A CA_n7A-n78A CA_n66A-n78A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | CA_n78(2A)_BCS1 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n7(2A)-n66A-n78A | CA_n7A-n66A CA_n7A-n78A CA_n66A-n78A | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7A-n66(2A)-n78A | CA_n7A-n66A CA_n7A-n78A CA_n66A-n78A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7(2A)-n66(2A)-n78A | CA_n7A-n66A CA_n7A-n78A CA_n66A-n78A | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7(2A)-n66A-n78(2A) | CA_n7A-n66A CA_n7A-n78A CA_n66A-n78A | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n7(2A)-n66(2A)-n78(2A) | CA_n7A-n66A CA_n7A-n78A CA_n66A-n78A | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n8A-n28A-n78A | - | n8 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n8A-n39A-n41A | - | n8 | 5, 10, 15, 20 | 0 |
|  |  | n39 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 100 |  |
|  |  | n8 | 5, 10, 15, 20 | 1 |
|  |  | n39 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n41 | 10, 15, 20, 40, 50, 60 |  |
| CA_n8A-n40A-n41A | CA_n8A-n40A CA_n8A-n41A CA_n40A-n41A | n8 | 5, 10, 15, 20 | 0 |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n8A-n41A-n79A | - | n8 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n8 | 5, 10, 15, 20 | 1 |
|  |  | n41 | 10, 15, 20, 40, 50, 60 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n8A-n78A-n79A | - | n8 | 5, 10, 15, 20 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n8A-n78(2A)-n79A | - | n8 | 5, 10, 15, 20 | 0 |
|  |  | n78 | CA_n78(2A)_BCS1 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n12A-n30A-n66A |  |  |  |  |  | CA_n12A-n30A CA_n12A-n66A CA_n30A-n66A |  |  |  |  |  | n12 |  |  |  |  |  | 5, 10, 15 |  |  |  |  |  | 0 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n30 |  |  |  |  |  | 5, 10 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n66 |  |  |  |  |  | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n12A-n30A-n66(2A) |  |  |  |  |  | CA_n12A-n30A CA_n12A-n66A CA_n30A-n66A |  |  |  |  |  | n12 |  |  |  |  |  | 5, 10, 15 |  |  |  |  |  | 0 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n30 |  |  |  |  |  | 5, 10 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n66 |  |  |  |  |  | CA_n66(2A)_BCS1 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n12A-n30A-n66(3A) |  |  |  |  |  | CA_n12A-n30A CA_n12A-n66A CA_n30A-n66A |  |  |  |  |  | n12 |  |  |  |  |  | 5, 10, 15 |  |  |  |  |  | 0 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n30 |  |  |  |  |  | 5, 10 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n66 |  |  |  |  |  | CA_n66(3A)_BCS0 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n12A-n30A-n77A | CA_n12A-n30A, n777 CA_n12A-n77A7 CA_n30A-n77A7 | n12 | 5, 10 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n12A-n30A-n77(2A) | CA_n12A-n30A CA_n12A- n77A CA_n30A-n77A | n12 | 5, 10 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n12A-n66A-n77A | CA_n12A-n66A, n777 CA_n12A-n77A7 CA_n66A-n77A7 | n12 | 5, 10, 15 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n12A-n66(2A)-n77A | CA_n12A-n66A CA_n12A-n77A CA_n66A-n77A | n12 | 5, 10, 15 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n12A-n66A-n77(2A) | CA_n12A-n66A CA_n12A-n77A CA_n66A-n77A | n12 | 5, 10, 15 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n13A-n25A-n66A | CA_n13A-n25A CA_n13A-n66A CA_n25A-n66A | n13 | 5, 10 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n13A-n25A-n77A | - | n13 | 5, 10 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n13A-n66A-n77A | - | n13 | 5, 10 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n14A-n30A-n66A | CA_n14A-n30A CA_n14A-n66A CA_n30A-n66A | n14 | 5, 10 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n14A-n30A-n66(2A) | CA_n14A-n30A CA_n14A-n66A CA_n30A-n66A | n14 | 5, 10 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n14A-n30A-n77A | n777 CA_n14A-n30A CA_n14A-n77A7 CA_n30A-n77A7 | n14 | 5, 10 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n14A-n30A-n77(2A) | CA_n14A-n30A CA_n14A-n77A CA_n30A-n77A | n14 | 5, 10 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n14A-n66A-n77A | n777 CA_n14A-n66A CA_n14A-n77A7 CA_n66A-n77A7 | n14 | 5, 10 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n14A-n66(2A)-n77A | CA_n14A-n66A CA_n14A-n77A CA_n66A-n77A | n14 | 5, 10 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n14A-n66A-n77(2A) | CA_n14A-n66A CA_n14A-n77A CA_n66A-n77A | n14 | 5, 10 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n18A-n28A-n41A |  |  |  |  |  | CA_n18A-n28A CA_n18A-n41A CA_n28A-n41A |  |  |  |  |  | n18 |  |  |  |  |  | 5, 10, 15 |  |  |  |  |  | 0 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n28 |  |  |  |  |  | 5, 10 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n41 |  |  |  |  |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n18A-n28A-n77A | CA_n18A-n28A CA_n18A-n41A CA_n28A-n41A | n18 | 5, 10, 15 | 0 |
|  |  | n28 | 5, 10 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n18A-n41A-n77A | CA_n18A-n28A CA_n18A-n41A CA_n28A-n41A | n18 | 5, 10, 15 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n20A-n28A-n78A | - | n20 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
| CA_n24A-n41A-n48A | CA_n24A-n41A CA_n24A_n48A CA_n41_n48A | n24 | 5, 10 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n24A-n41(2A)-n48A | CA_n24A-n41A CA_n24A_n48A CA_n41_n48A | n24 | 5, 10 | 0 |
|  |  | n41 | CA_n41(2A) BCS1 |  |
|  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n24A-n41A-n48(2A) | CA_n24A-n41A CA_n24A_n48A CA_n41_n48A | n24 | 5, 10 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n48 | CA_n48(2A) BCS0 |  |
| CA_n24A-n41(2A)-n48(2A) | CA_n24A-n41A CA_n24A_n48A CA_n41_n48A | n24 | 5, 10 | 0 |
|  |  | n41 | CA_n41(2A) BCS1 |  |
|  |  | n48 | CA_n48(2A) BCS0 |  |
| CA_n24A-n41A-n77A | CA_n24A-n41A CA_n24A_n77A CA_n41_n77A | n24 | 5, 10 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n24A-n41(2A)-n77A | CA_n24A-n41A CA_n24A_n77A CA_n41_n77A | n24 | 5, 10 | 0 |
|  |  | n41 | CA_n41(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n24 | 5, 10 | 1 |
|  |  | n41 | CA_n41(2A) BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n24A-n41A-n77(2A) | CA_n24A-n41A CA_n24A_n77A CA_n41_n77A | n24 | 5, 10 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n24 | 5, 10 | 1 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | CA_n77(2A) BCS0 |  |
| CA_n24A-n41(2A)-n77(2A) | CA_n24A-n41A CA_n24A_n77A CA_n41_n77A | n24 | 5, 10 | 0 |
|  |  | n41 | CA_n41(2A)_BCS1 |  |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n24 | 5, 10 | 1 |
|  |  | n41 | CA_n41(2A) BCS1 |  |
|  |  | n77 | CA_n77(2A) BCS0 |  |
| CA_n24A-n48A-n77A |  | n24 | 5, 10 | 0 |
|  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n24A-n48(2A)-n77A |  | n24 | 5, 10 | 0 |
|  |  | n48 | CA_n48(2A) BCS0 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n24A-n48A-n77(2A) |  | n24 | 5, 10 | 0 |
|  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n77 | CA_n77(2A) BCS0 |  |
| CA_n24A-n48(2A)-n77(2A) |  | n24 | 5, 10 | 0 |
|  |  | n48 | CA_n48(2A) BCS0 |  |
|  |  | n77 | CA_n77(2A) BCS0 |  |
| CA_n25A-n29A-n66A | CA_n25A-n66A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n25A-n38A-n66A | CA_n25A-n38A CA_n25A-n66A CA_n38A-n66A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n25(2A)-n38A-n66A | CA_n25A-n38A CA_n25A-n66A CA_n38A-n66A | n25 | CA_n25(2A)_BCS0 | 0 |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n25(2A)-n38A-n66(2A) |  |  |  |  |  | CA_n25A-n38A CA_n25A-n66A CA_n38A-n66A |  |  |  |  |  | n25 |  |  |  |  |  | CA_n25(2A)_BCS0 |  |  |  |  |  | 0 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n38 |  |  |  |  |  | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n66 |  |  |  |  |  | CA_n66(2A)_BCS1 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n25A-n38A-n66(2A) | CA_n25A-n38A CA_n25A-n66A CA_n38A-n66A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n38 | 5, 10, 15, 20 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n25A-n38A-n78A | CA_n25A-n38A CA_n25A-n78A CA_n38A-n78A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n25A-n38A-n78(2A) | CA_n25A-n38A CA_n25A-n78A CA_n38A-n78A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n25(2A)-n38A-n78A | CA_n25A-n38A CA_n25A-n78A CA_n38A-n78A | n25 | CA_n25(2A)_BCS0 | 0 |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n25(2A)-n38A-n78(2A) | CA_n25A-n38A CA_n25A-n78A CA_n38A-n78A | n25 | CA_n25(2A)_BCS0 | 0 |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n25A-n41A-n66A | - | n25 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  | CA_n25A-n41A CA_n25A-n66A CA_n41A-n66A | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n25A-n41A-n66(2A) | - | n25 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n66 |  |  |  |  |  | CA_n66(2A)_BCS1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | CA_n25A-n41A CA_n25A-n66A CA_n41A-n66A |  |  |  |  |  | n25 |  |  |  |  |  | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |  | 1 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n41 |  |  |  |  |  | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n66 |  |  |  |  |  | CA_n66(2A)_BCS1 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n25A-n41C-n66A | - | n25 | 5, 10, 15, 20 | 0 |
|  |  | n41 | CA_n41C_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  | CA_n25A-n41A CA_n25A-n66A CA_n41A-n66A | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n41 | CA_n41C_BCS1 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n25A-n41(2A)-n66A | - | n25 | 5, 10, 15, 20 | 0 |
|  |  | n41 | CA_n41(2A)_BCS1 |  |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  | CA_n25A-n41A CA_n25A-n66A CA_n41A-n66A | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n41 | CA_n41(2A)_BCS1 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n25(2A)-n41A-n66A | - | n25 | CA_n25(2A)_BCS1 | 0 |
|  |  |  |  |  |  |  |  |  |  |  |  | n41 |  |  |  |  |  | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n66 |  |  |  |  |  | 5, 10, 15, 20, 40 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | CA_n25A-n41A CA_n25A-n66A CA_n41A-n66A |  |  |  |  |  | n25 |  |  |  |  |  | CA_n25(2A)_BCS1 |  |  |  |  |  | 1 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n41 |  |  |  |  |  | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n66 |  |  |  |  |  | 5, 10, 15, 20, 30, 40 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n25A-n41A-n71A | - | n25 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  | CA_n25A-n41A | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  | CA_n41A-n71A | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | CA_n25A-n71A | n71 | 5, 10, 15, 20 |  |
| CA_n25A-n41A-n71B | - | n25 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n71 |  |  |  |  |  | CA_n71B_BCS2 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | CA_n25A-n41A CA_n41A-n71A CA_n25A-n71A |  |  |  |  |  | n25 |  |  |  |  |  | 5, 10, 15, 20, 30, 40, 50 |  |  |  |  |  | 1 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n41 |  |  |  |  |  | 10, 15. 20, 30, 40, 50, 60, 80, 90, 100 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n71 |  |  |  |  |  | CA_n71B_BCS2 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n25A-n41A-n71(2A) | - | n25 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n71 | CA_n71(2A)_BCS0 |  |
|  | CA_n25A-n41A CA_n41A-n71A CA_n25A-n71A | n25 | 5, 10, 15, 20, 30, 40, 50 | 1 |
|  |  | n41 | 10, 15. 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n71 | CA_n71B_BCS0 |  |
| CA_n25A-n41(2A)-n71A | - | n25 | 5, 10, 15, 20 | 0 |
|  |  | n41 | CA_n41(2A)_BCS1 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  | CA_n25A-n41A CA_n41A-n71A CA_n25A-n71A | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n41 | CA_n41(2A)_BCS1 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n25A-n41C-n71A | - | n25 | 5, 10, 15, 20 | 0 |
|  |  | n41 | CA_n41C_BCS0 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  | CA_n25A-n41A CA_n41A-n71A CA_n25A-n71A | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n41 | CA_n41C_BCS1 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n25(2A)-n41A-n71A | - | n25 | CA_n25(2A)_BCS1 | 0 |
|  |  |  |  |  |  |  |  |  |  |  |  | n41 |  |  |  |  |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n71 |  |  |  |  |  | 5, 10, 15, 20 |  |  |  |  |  |  |  |  |  |  |  |
|  | CA_n25A-n41A CA_n41A-n71A CA_n25A-n71A | n25 | CA_n25(2A)_BCS1 | 1 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n25A-n41A-n77A | CA_n25A-n41A CA_n25A-n77A CA_n41A-n77A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n25A-n41(2A)-n77A | CA_n25A-n41A CA_n25A-n77A CA_n41A-n77A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n41 | CA_n41(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n41 | CA_n41(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
| CA_n25A-n41A-n77(2A) | CA_n25A-n41A CA_n25A-n77A CA_n41A-n77A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n25(2A)-n41A-n77A | CA_n25A-n41A CA_n25A-n77A CA_n41A-n77A | n25 | CA_n25(2A)_BCS1 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n25A-n41C-n77A | CA_n41C CA_n25A-n41A CA_n25A-n77A CA_n41A-n77A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n41 | CA_n41C_BCS0 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n41 | CA_n41C_BCS2 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n25A-n41A-n78A | CA_n25A-n41A CA_n25A-n78A CA_n41A-n78A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n25A-n41A-n78(2A) | CA_n25A-n41A CA_n25A-n78A CA_n41A-n78A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n25A-n48A-n66A | CA_n25A-n48A CA_n25A-n66A CA_n48A-n66A | n25 | 5, 10, 15, 20 | 0 |
|  |  | n48 | 5, 10, 15, 20, 40, 50 |  |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n25A-n48(2A)-n66A | CA_n25A-n48A CA_n25A-n66A CA_n48A-n66A | n25 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n25A-n48C-n66A | CA_n25A-n48A CA_n25A-n66A CA_n48A-n66A | n25 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48C_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n48 | CA_n48C_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n25A-n66A-n71A | - | n25 | 5, 10, 15, 20 | 0 |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  | CA_n25A-n66A CA_n25A-n71A CA_n66A-n71A | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n25A-n66A-n71B | CA_n25A-n66A CA_n25A-n71A CA_n66A-n71A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n71 | CA_n71B_BCS2 |  |
| CA_n25A-n66A-n71(2A) | CA_n25A-n66A CA_n25A-n71A CA_n66A-n71A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n71 | CA_n71(2A)_BCS0 |  |
| CA_n25A-n66(2A)-n71A | CA_n25A-n66A CA_n25A-n71A CA_n66A-n71A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n25(2A)-n66A-n71A | CA_n25A-n66A CA_n25A-n71A CA_n66A-n71A | n25 | CA_n25(2A)_BCS1 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n25A-n66A-n77A | CA_n25A-n66A CA_n25A-n77A CA_n66A-n77A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n25A-n66(2A)-n77A | CA_n25A-n66A CA_n25A-n77A CA_n66A-n77A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n25A-n66A-n77(2A) | CA_n25A-n66A CA_n25A-n77A CA_n66A-n77A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n25A-n66(2A)-n77(2A) | CA_n25A-n66A CA_n25A-n77A CA_n66A-n77A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n25(2A)-n66A-n77A | CA_n25A-n66A CA_n25A-n77A CA_n66A-n77A | n25 | CA_n25(2A)_BCS0 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n25(2A)-n66(2A)-n77A | CA_n25A-n66A CA_n25A-n77A CA_n66A-n77A | n25 | CA_n25(2A)_BCS0 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n25(2A)-n66A-n77(2A) | CA_n25A-n66A CA_n25A-n77A CA_n66A-n77A | n25 | CA_n25(2A)_BCS0 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n25(2A)-n66(2A)-n77(2A) | CA_n25A-n66A CA_n25A-n77A CA_n66A-n77A | n25 | CA_n25(2A)_BCS0 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n25A-n66A-n78A | CA_n25A-n66A CA_n25A-n78A CA_n66A-n78A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n25(2A)-n66A-n78A | CA_n25A-n66A CA_n25A-n78A CA_n66A-n78A | n25 | CA_n25(2A)_BCS0 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n25A-n66(2A)-n78A | CA_n25A-n66A CA_n25A-n78A CA_n66A-n78A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n25A-n66A-n78(2A) | CA_n25A-n66A CA_n25A-n78A CA_n66A-n78A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n25(2A)-n66(2A)-n78A | CA_n25A-n66A CA_n25A-n78A CA_n66A-n78A | n25 | CA_n25(2A)_BCS0 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n25(2A)-n66A-n78(2A) | CA_n25A-n66A CA_n25A-n78A CA_n66A-n78A | n25 | CA_n25(2A)_BCS0 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n25A-n66(2A)-n78(2A) | CA_n25A-n66A CA_n25A-n78A CA_n66A-n78A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n25(2A)-n66(2A)-n78(2A) | CA_n25A-n66A CA_n25A-n78A CA_n66A-n78A | n25 | CA_n25(2A)_BCS0 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n25A-n71A-n77A | CA_n25A-n71A CA_n25A-n77A CA_n71A-n77A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n25A-n71B-n77A | CA_n25A-n71A CA_n25A-n77A CA_n71A-n77A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n71 | CA_n71B_BCS2 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n25A-n71(2A)-n77A | CA_n25A-n71A CA_n25A-n77A CA_n71A-n77A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n71 | CA_n71(2A)_BCS0 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n25(2A)-n71A-n77A | CA_n25A-n71A CA_n25A-n77A CA_n71A-n77A | n25 | CA_n25(2A)_BCS1 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n25A-n71A-n78A | CA_n25A-n71A CA_n25A-n78A CA_n71A-n78A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n25A-n71A-n78(2A) | CA_n25A-n71A CA_n25A-n78A CA_n71A-n78A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n26A-n66A-n70A | CA_n26A-n66A CA_n26A-n70A | n26 | 5, 10, 15, 20 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n26A-n66(2A)-n70A | CA_n26A-n66A CA_n26A-n70A | n26 | 5, 10, 15, 20 | 0 |
|  |  | n66 | CA_n66(2A)_BCS0 |  |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n28A-n40A-n41A |  |  |  |  |  | CA_n28A-n40A CA_n28A-n41A CA_n40A-n41A |  |  |  |  |  | n28 |  |  |  |  |  | 5, 10, 15, 20, 30 |  |  |  |  |  | 0 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n40 |  |  |  |  |  | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n41 |  |  |  |  |  | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n28A-n40A-n78A | CA_n28A-n40A CA_n28A-n78A CA_n40A-n78A | n28 | 5, 10, 15, 20 | 0 |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  | CA_n28A-n40A CA_n28A-n78A CA_n40A-n78A | n28 | 5, 10, 15, 20 | 1 |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80, 100 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n28A-n40A-n79A | CA_n28A-n40A CA_n28A-n79A CA_n40A-n79A | n28 | 5, 10, 15, 20, 30 | 0 |
|  |  | n40 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n28A-n41A-n77A | CA_n28A-n41A | n28 | 5, 10, 15, 20, 30 | 0 |
|  | CA_n28A-n77A | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  | CA_n41A-n77A | n77 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n28A-n41A-n77(2A) | CA_n28A-n41A | n28 | 5, 10, 15, 20, 30 | 0 |
|  | CA_n28A-n77A | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  | CA_n41A-n77A | n77 | CA_n77(2A)_BCS0 |  |
| CA_n28A-n41A-n78A | CA_n28A-n41A CA_n41A-n78A CA_n28A-n78A | n28 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 90, 100 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
| CA_n28A-n41A-n78(2A) | CA_n78(2A) | n28 | 5, 10, 15, 20, 30 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n28A-n41A-n79A | CA_n28A-n41A CA_n28A-n79A CA_n41A-n79A | n28 | 5, 10, 15, 20, 30 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n28A-n46A-n78A | CA_n28A-n46A CA_n28A-n78A CA_n46A-n78A | n28 | 5, 10, 15, 20 | 0 |
|  |  | n46 | 20, 40, 60, 80 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n28A-n46C-n78A | CA_n28A-n46A CA_n28A-n78A CA_n46A-n78A | n28 | 5, 10, 15, 20 | 0 |
|  |  | n46 | CA_n46C_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n28A-n46D-n78A | CA_n28A-n46A CA_n28A-n78A CA_n46A-n78A | n28 | 5, 10, 15, 20 | 0 |
|  |  | n46 | CA_n46D_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n28A-n77A-n79A4 | CA_n28A-n77A CA_n28A-n79A CA_n77A-n79A | n28 | 5, 10, 15, 20 | 0 |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n28A-n77(2A)-n79A4 | CA_n28A-n77A CA_n28A-n79A CA_n77A-n79A | n28 | 5, 10, 15, 20 | 0 |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n28A-n78A-n79A | CA_n28A-n78A CA_n28A-n79A CA_n78A-n79A | n28 | 5, 10, 15, 20 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n29A-n30A-n66A |  |  |  |  |  | CA_n30A-n66A |  |  |  |  |  | n29 |  |  |  |  |  | 5, 10 |  |  |  |  |  | 0 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n30 |  |  |  |  |  | 5, 10 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n66 |  |  |  |  |  | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n29A-n30A-n66(2A) |  |  |  |  |  | CA_n30A-n66A |  |  |  |  |  | n29 |  |  |  |  |  | 5, 10 |  |  |  |  |  | 0 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n30 |  |  |  |  |  | 5, 10 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n66 |  |  |  |  |  | CA_n66(2A)_BCS1 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n29A-n30A-n77A | CA_n30A-n77A | n29 | 5, 10 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n29A-n30A-n77(2A) | CA_n30A-n77A | n29 | 5, 10 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n29A-n66A-n70A | - | n29 | 5, 10 | 0 |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n29A-n66B-n70A | - | n29 | 5, 10 | 0 |
|  |  | n66 | CA_n66B_BCS0. |  |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n29A-n66(2A)-n70A | - | n29 | 5, 10 | 0 |
|  |  | n66 | CA_n66(2A)_BCS0 |  |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n29A-n66A-n77A | CA_n66A-n77A | n29 | 5, 10 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n29A-n66(2A)-n77A | CA_n66A-n77A | n29 | 5, 10 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n29A-n66A-n77(2A) | CA_n66A-n77A | n29 | 5, 10 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n30A-n66A-n77A | n777 CA_n30A-n66A CA_n30A-n77A7 CA_n66A-n77A7 | n30 | 5, 10 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n30A-n66(2A)-n77A | CA_n30A-n66A CA_n30A-n77A CA_n66A-n77A | n30 | 5, 10 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n30A-n66A-n77(2A) | CA_n30A-n66A CA_n30A-n77A CA_n66A-n77A | n30 | 5, 10 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n38A-n66A-n78A | CA_n38A-n66A CA_n38A-n78A CA_n66A-n78A | n38 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n38A-n66A-n78(2A) | CA_n38A-n66A CA_n38A-n78A CA_n66A-n78A | n38 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n38A-n66(2A)-n78A | CA_n38A-n66A CA_n38A-n78A CA_n66A-n78A | n38 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n38A-n66(2A)-n78(2A) | CA_n38A-n66A CA_n38A-n78A CA_n66A-n78A | n38 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n39A-n40A-n41A | CA_n39A-n40A CA_n39A-n41A CA_n40A-n41A | n39 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n39A-n40A-n79A | CA_n39A-n40A CA_n40A-n79A CA_n39A-n79A | n39 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n39A-n41A-n79A | - | n39 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n39 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n41 | 10, 15, 20, 40, 50, 60 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n40A-n41A-n79A | CA_n40A-n41A CA_n40A-n79A CA_n41A-n79A | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 | 0 |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 100 |  |
|  |  | n79 | , 40, 50, 60, 80, 100 |  |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  |  |  |  |  |  |  |  |  |  |  | n41 |  |  |  |  |  | 10, 15, 20, 40, 50, 60 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n79 |  |  |  |  |  | , 40, 50, 60, 80, 100 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n41A-n66A-n70A |  |  |  |  |  | CA_n41A-n66A CA_n41A-n70A |  |  |  |  |  | n41 |  |  |  |  |  | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |  | 0 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n66 |  |  |  |  |  | 10, 15, 20, 25, 30, 40 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n70 |  |  |  |  |  | 5, 10, 15, 201, 251 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n41A-n66A-n71A | - | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  | CA_n41A-n71A CA_n66A-n71A CA_n41A-n66A | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n41A-n66A-n71B | CA_n41A-n66A CA_n41A-n71A CA_n66A-n71A | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n71 | CA_n71B_BCS2 |  |
| CA_n41A-n66A-n71(2A) | CA_n41A-n66A CA_n41A-n71A CA_n66A-n71A | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n71(2A)_BCS0 |  |
| CA_n41A-n66(2A)-n71A | CA_n41A-n66A CA_n66A-n71A CA_n41A-n71A | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n41(2A)-n66A-n71A | - | n41 | CA_n41(2A)_BCS1 | 0 |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  | CA_n41A-n71A CA_n66A-n71A CA_n41A-n66A | n41 | CA_n41(2A)_BCS1 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n41C-n66A-n71A | - | n41 | CA_n41C_BCS0 | 0 |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  | CA_n41A-n71A CA_n66A-n71A CA_n41A-n66A | n41 | CA_n41C_BCS1 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n41A-n66A-n77A | CA_n41A-n66A CA_n41A-n77A CA_n66A-n77A | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n41A-n66A-n77(2A) | CA_n41A-n71A CA_n66A-n71A CA_n41A-n66A | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n41A-n66(2A)-n77A | CA_n41A-n66A CA_n41A-n77A CA_n66A-n77A | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n41A-n66(2A)-n77(2A) | CA_n41A-n66A CA_n41A-n77A CA_n66A-n77A | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n41(2A)-n66A-n77A | CA_n41A-n66A CA_n41A-n77A CA_n66A-n77A | n41 | CA_n41(2A)_BCS1 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n41C-n66A-n77A | CA_41C CA_n41A-n66A CA_n41A-n77A CA_n66A-n77A | n41 | CA_n41C_BCS0 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n41A-n66A-n78A | CA_n41A-n66A CA_n41A-n78A CA_n66A-n78A | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n41A-n66A-n78(2A) | CA_n41A-n66A CA_n41A-n78A CA_n66A-n78A | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  | CA_n78(2A)_BCS2 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n41A-n70A-n78A |  |  |  |  |  | CA_n41A-n70A CA_n41A-n78A CA_n70A-n78A |  |  |  |  |  | n41 |  |  |  |  |  | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |  | 0 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n70 |  |  |  |  |  | 5, 10, 15, 201, 251 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n41A-n66(2A)-n78A | CA_n41A-n66A CA_n41A-n78A CA_n66A-n78A | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n41A-n66(2A)-n78(2A) | CA_n41A-n66A CA_n41A-n78A CA_n66A-n78A | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n41A-n70A-n78A |  |  |  |  |  | - |  |  |  |  |  | n41 |  |  |  |  |  | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |  | 0 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n70 |  |  |  |  |  | 5, 10, 15, 20, 25 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n41A-n71A-n77A | CA_n41A-n71A CA_n41A-n77A CA_n71A-n77A | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 1 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n41A-n71B-n77A | CA_n41A-n71A CA_n41A-n77A CA_n71A-n77A | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n71 | CA_n71B_BCS2 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n41A-n71(2A)-n77A | CA_n41A-n71A CA_n41A-n77A CA_n71A-n77A | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n71 | CA_n71(2A)_BCS0 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n41A-n71A-n77(2A) | CA_n41A-n71A CA_n41A-n77A CA_n71A-n77A | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n41(2A)-n71A-n77A | CA_n41A-n71A CA_n41A-n77A CA_n71A-n77A | n41 | CA_n41(2A)_BCS1 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n41C-n71A-n77A | CA_41C CA_n41A-n71A CA_n41A-n77A CA_n71A-n77A | n41 | CA_n41C_BCS0 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n41A-n71A-n78A | CA_n41A-n71A CA_n41A-n78A CA_n71A-n78A | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n41A-n71A-n78(2A) | CA_n41A-n71A CA_n41A-n78A CA_n71A-n78A | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n46A-n48A-n96A |  |  |  |  |  | CA_n46A-n48A CA_n48A-n96A |  |  |  |  |  | n46 |  |  |  |  |  | 10, 20, 40, 60, 80 |  |  |  |  |  | 0 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n48 |  |  |  |  |  | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n96 |  |  |  |  |  | 20, 40, 60, 80 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n46B-n48A-n96A |  |  |  |  |  | CA_n46A-n48A CA_n48A-n96A |  |  |  |  |  | n46 |  |  |  |  |  | CA_n46B_BCS0 |  |  |  |  |  | 0 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n48 |  |  |  |  |  | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n96 |  |  |  |  |  | 20, 40, 60, 80 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n46C-n48A-n96A |  |  |  |  |  | CA_n46A-n48A CA_n48A-n96A |  |  |  |  |  | n46 |  |  |  |  |  | CA_n46C_BCS0 |  |  |  |  |  | 0 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n48 |  |  |  |  |  | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n96 |  |  |  |  |  | 20, 40, 60, 80 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n46D-n48A-n96A |  |  |  |  |  | CA_n46A-n48A CA_n48A-n96A |  |  |  |  |  | n46 |  |  |  |  |  | CA_n46D_BCS0 |  |  |  |  |  | 0 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n48 |  |  |  |  |  | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n96 |  |  |  |  |  | 20, 40, 60, 80 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n46N-n48A-n96A |  |  |  |  |  | CA_n46A-n48A CA_n48A-n96A |  |  |  |  |  | n46 |  |  |  |  |  | CA_n46N_BCS0 |  |  |  |  |  | 0 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n48 |  |  |  |  |  | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n96 |  |  |  |  |  | 20, 40, 60, 80 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n46A-n48B-n96A |  |  |  |  |  | CA_n48B CA_n46A-n48A CA_n48A-n96A CA_n46A-n48B CA_n48B-n96A |  |  |  |  |  | n46 |  |  |  |  |  | 10, 20, 40, 60, 80 |  |  |  |  |  | 0 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n48 |  |  |  |  |  | CA_n48B_BCS0 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n96 |  |  |  |  |  | 20, 40, 60, 80 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n46B-n48B-n96A |  |  |  |  |  | CA_n48B CA_n46A-n48A CA_n48A-n96A CA_n46A-n48B CA_n48B-n96A |  |  |  |  |  | n46 |  |  |  |  |  | CA_n46B_BCS0 |  |  |  |  |  | 0 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n48 |  |  |  |  |  | CA_n48B_BCS0 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n96 |  |  |  |  |  | 20, 40, 60, 80 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n46C-n48B-n96A |  |  |  |  |  | CA_n48B CA_n46A-n48A CA_n48A-n96A CA_n46A-n48B CA_n48B-n96A |  |  |  |  |  | n46 |  |  |  |  |  | CA_n46C_BCS0 |  |  |  |  |  | 0 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n48 |  |  |  |  |  | CA_n48B_BCS0 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n96 |  |  |  |  |  | 20, 40, 60, 80 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n46D-n48B-n96A |  |  |  |  |  | CA_n48B CA_n46A-n48A CA_n48A-n96A CA_n46A-n48B CA_n48B-n96A |  |  |  |  |  | n46 |  |  |  |  |  | CA_n46D_BCS0 |  |  |  |  |  | 0 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n48 |  |  |  |  |  | CA_n48B_BCS0 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n96 |  |  |  |  |  | 20, 40, 60, 80 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n46N-n48B-n96A |  |  |  |  |  | CA_n48B CA_n46A-n48A CA_n48A-n96A CA_n46A-n48B CA_n48B-n96A |  |  |  |  |  | n46 |  |  |  |  |  | CA_n46N_BCS0 |  |  |  |  |  | 0 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n48 |  |  |  |  |  | CA_n48B_BCS0 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n96 |  |  |  |  |  | 20, 40, 60, 80 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n46A-n48C-n96A |  |  |  |  |  | CA_n48B CA_n46A-n48A CA_n48A-n96A CA_n46A-n48B CA_n48B-n96A |  |  |  |  |  | n46 |  |  |  |  |  | 10, 20, 40, 60, 80 |  |  |  |  |  | 0 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n48 |  |  |  |  |  | CA_n48C_BCS0 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n96 |  |  |  |  |  | 20, 40, 60, 80 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n46B-n48C-n96A |  |  |  |  |  | CA_n48B CA_n46A-n48A CA_n48A-n96A CA_n46A-n48B CA_n48B-n96A |  |  |  |  |  | n46 |  |  |  |  |  | CA_n46B_BCS0 |  |  |  |  |  | 0 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n48 |  |  |  |  |  | CA_n48C_BCS0 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n96 |  |  |  |  |  | 20, 40, 60, 80 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n46C-n48C-n96A |  |  |  |  |  | CA_n48B CA_n46A-n48A CA_n48A-n96A CA_n46A-n48B CA_n48B-n96A |  |  |  |  |  | n46 |  |  |  |  |  | CA_n46C_BCS0 |  |  |  |  |  | 0 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n48 |  |  |  |  |  | CA_n48C_BCS0 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n96 |  |  |  |  |  | 20, 40, 60, 80 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n46D-n48C-n96A |  |  |  |  |  | CA_n48B CA_n46A-n48A CA_n48A-n96A CA_n46A-n48B CA_n48B-n96A |  |  |  |  |  | n46 |  |  |  |  |  | CA_n46D_BCS0 |  |  |  |  |  | 0 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n48 |  |  |  |  |  | CA_n48C_BCS0 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n96 |  |  |  |  |  | 20, 40, 60, 80 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n46N-n48C-n96A |  |  |  |  |  | CA_n48B CA_n46A-n48A CA_n48A-n96A CA_n46A-n48B CA_n48B-n96A |  |  |  |  |  | n46 |  |  |  |  |  | CA_n46N_BCS0 |  |  |  |  |  | 0 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n48 |  |  |  |  |  | CA_n48C_BCS0 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n96 |  |  |  |  |  | 20, 40, 60, 80 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n46A-n48A-n96B |  |  |  |  |  | CA_n46A-n48A CA_n48A-n96A |  |  |  |  |  | n46 |  |  |  |  |  | 10, 20, 40, 60, 80 |  |  |  |  |  | 0 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n48 |  |  |  |  |  | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n96 |  |  |  |  |  | CA_n96B_BCS0 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n46B-n48A-n96B |  |  |  |  |  | CA_n46A-n48A CA_n48A-n96A |  |  |  |  |  | n46 |  |  |  |  |  | CA_n46B_BCS0 |  |  |  |  |  | 0 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n48 |  |  |  |  |  | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n96 |  |  |  |  |  | CA_n96B_BCS0 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n46C-n48A-n96B |  |  |  |  |  | CA_n46A-n48A CA_n48A-n96A |  |  |  |  |  | n46 |  |  |  |  |  | CA_n46C_BCS0 |  |  |  |  |  | 0 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n48 |  |  |  |  |  | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n96 |  |  |  |  |  | CA_n96B_BCS0 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n46D-n48A-n96B |  |  |  |  |  | CA_n46A-n48A CA_n48A-n96A CA |  |  |  |  |  | n46 |  |  |  |  |  | CA_n46D_BCS0 |  |  |  |  |  | 0 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n48 |  |  |  |  |  | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n96 |  |  |  |  |  | CA_n96B_BCS0 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n46N-n48A-n96B |  |  |  |  |  | CA_n46A-n48A CA_n48A-n96A |  |  |  |  |  | n46 |  |  |  |  |  | CA_n46N_BCS0 |  |  |  |  |  | 0 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n48 |  |  |  |  |  | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n96 |  |  |  |  |  | CA_n96B_BCS0 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n46A-n48B-n96C |  |  |  |  |  | CA_n48B CA_n46A-n48A CA_n48A-n96A CA_n46A-n48B CA_n48B-n96A |  |  |  |  |  | n46 |  |  |  |  |  | 10, 20, 40, 60, 80 |  |  |  |  |  | 0 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n48 |  |  |  |  |  | CA_n48B_BCS0 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n96 |  |  |  |  |  | CA_n96C_BCS0 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n46B-n48B-n96C |  |  |  |  |  | CA_n48B CA_n46A-n48A CA_n48A-n96A CA_n46A-n48B CA_n48B-n96A |  |  |  |  |  | n46 |  |  |  |  |  | CA_n46B_BCS0 |  |  |  |  |  | 0 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n48 |  |  |  |  |  | CA_n48B_BCS0 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n96 |  |  |  |  |  | CA_n96C_BCS0 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n46C-n48B-n96C |  |  |  |  |  | CA_n48B CA_n46A-n48A CA_n48A-n96A CA_n46A-n48B CA_n48B-n96A |  |  |  |  |  | n46 |  |  |  |  |  | CA_n46C_BCS0 |  |  |  |  |  | 0 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n48 |  |  |  |  |  | CA_n48B_BCS0 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n96 |  |  |  |  |  | CA_n96C_BCS0 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n46D-n48B-n96C |  |  |  |  |  | CA_n48B CA_n46A-n48A CA_n48A-n96A CA_n46A-n48B CA_n48B-n96A |  |  |  |  |  | n46 |  |  |  |  |  | CA_n46D_BCS0 |  |  |  |  |  | 0 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n48 |  |  |  |  |  | CA_n48B_BCS0 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n96 |  |  |  |  |  | CA_n96C_BCS0 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n46N-n48B-n96C |  |  |  |  |  | CA_n48B CA_n46A-n48A CA_n48A-n96A CA_n46A-n48B CA_n48B-n96A |  |  |  |  |  | n46 |  |  |  |  |  | CA_n46N_BCS0 |  |  |  |  |  | 0 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n48 |  |  |  |  |  | CA_n48B_BCS0 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n96 |  |  |  |  |  | CA_n96C_BCS0 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n46A-n48C-n96D |  |  |  |  |  | CA_n48B CA_n46A-n48A CA_n48A-n96A CA_n46A-n48B CA_n48B-n96A |  |  |  |  |  | n46 |  |  |  |  |  | 10, 20, 40, 60, 80 |  |  |  |  |  | 0 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n48 |  |  |  |  |  | CA_n48C_BCS0 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n96 |  |  |  |  |  | CA_n96D_BCS0 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n46B-n48C-n96D |  |  |  |  |  | CA_n48B CA_n46A-n48A CA_n48A-n96A CA_n46A-n48B CA_n48B-n96A |  |  |  |  |  | n46 |  |  |  |  |  | CA_n46B_BCS0 |  |  |  |  |  | 0 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n48 |  |  |  |  |  | CA_n48C_BCS0 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n96 |  |  |  |  |  | CA_n96D_BCS0 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n46C-n48C-n96D |  |  |  |  |  | CA_n48B CA_n46A-n48A CA_n48A-n96A CA_n46A-n48B CA_n48B-n96A |  |  |  |  |  | n46 |  |  |  |  |  | CA_n46C_BCS0 |  |  |  |  |  | 0 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n48 |  |  |  |  |  | CA_n48C_BCS0 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n96 |  |  |  |  |  | CA_n96D_BCS0 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n46D-n48C-n96D |  |  |  |  |  | CA_n48B CA_n46A-n48A CA_n48A-n96A CA_n46A-n48B CA_n48B-n96A |  |  |  |  |  | n46 |  |  |  |  |  | CA_n46D_BCS0 |  |  |  |  |  | 0 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n48 |  |  |  |  |  | CA_n48C_BCS0 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n96 |  |  |  |  |  | CA_n96D_BCS0 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n46N-n48C-n96D |  |  |  |  |  | CA_n48B CA_n46A-n48A CA_n48A-n96A CA_n46A-n48B CA_n48B-n96A |  |  |  |  |  | n46 |  |  |  |  |  | CA_n46N_BCS0 |  |  |  |  |  | 0 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n48 |  |  |  |  |  | CA_n48C_BCS0 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n96 |  |  |  |  |  | CA_n96D_BCS0 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n46A-n48C-n96E |  |  |  |  |  | CA_n48B CA_n46A-n48A CA_n48A-n96A CA_n46A-n48B CA_n48B-n96A |  |  |  |  |  | n46 |  |  |  |  |  | 10, 20, 40, 60, 80 |  |  |  |  |  | 0 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n48 |  |  |  |  |  | CA_n48C_BCS0 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n96 |  |  |  |  |  | CA_n96E_BCS0 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n46B-n48C-n96E |  |  |  |  |  | CA_n48B CA_n46A-n48A CA_n48A-n96A CA_n46A-n48B CA_n48B-n96A |  |  |  |  |  | n46 |  |  |  |  |  | CA_n46B_BCS0 |  |  |  |  |  | 0 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n48 |  |  |  |  |  | CA_n48C_BCS0 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n96 |  |  |  |  |  | CA_n96E_BCS0 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n46C-n48C-n96E |  |  |  |  |  | CA_n48B CA_n46A-n48A CA_n48A-n96A CA_n46A-n48B CA_n48B-n96A |  |  |  |  |  | n46 |  |  |  |  |  | CA_n46C_BCS0 |  |  |  |  |  | 0 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n48 |  |  |  |  |  | CA_n48C_BCS0 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n96 |  |  |  |  |  | CA_n96E_BCS0 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n46D-n48C-n96E |  |  |  |  |  | CA_n48B CA_n46A-n48A CA_n48A-n96A CA_n46A-n48B CA_n48B-n96A |  |  |  |  |  | n46 |  |  |  |  |  | CA_n46D_BCS0 |  |  |  |  |  | 0 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n48 |  |  |  |  |  | CA_n48C_BCS0 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n96 |  |  |  |  |  | CA_n96E_BCS0 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n46N-n48C-n96E |  |  |  |  |  | CA_n48B CA_n46A-n48A CA_n48A-n96A CA_n46A-n48B CA_n48B-n96A |  |  |  |  |  | n46 |  |  |  |  |  | CA_n46N_BCS0 |  |  |  |  |  | 0 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n48 |  |  |  |  |  | CA_n48C_BCS0 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n96 |  |  |  |  |  | CA_n96E_BCS0 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n46A-n48(2A)-n96A |  |  |  |  |  | CA_n46A-n48A CA_n48A-n96A |  |  |  |  |  | n46 |  |  |  |  |  | 10, 20, 40, 60, 80 |  |  |  |  |  | 0 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n48 |  |  |  |  |  | CA_n48(2A)_BCS0 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n96 |  |  |  |  |  | 20, 40, 60, 80 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n46B-n48(2A)-n96A |  |  |  |  |  | CA_n46A-n48A CA_n48A-n96A |  |  |  |  |  | n46 |  |  |  |  |  | CA_n46B_BCS0 |  |  |  |  |  | 0 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n48 |  |  |  |  |  | CA_n48(2A)_BCS0 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n96 |  |  |  |  |  | 20, 40, 60, 80 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n46C-n48(2A)-n96A |  |  |  |  |  | CA_n46A-n48A CA_n48A-n96A |  |  |  |  |  | n46 |  |  |  |  |  | CA_n46C_BCS0 |  |  |  |  |  | 0 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n48 |  |  |  |  |  | CA_n48(2A)_BCS0 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n96 |  |  |  |  |  | 20, 40, 60, 80 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n46D-n48(2A)-n96A |  |  |  |  |  | CA_n46A-n48A CA_n48A-n96A |  |  |  |  |  | n46 |  |  |  |  |  | CA_n46D_BCS0 |  |  |  |  |  | 0 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n48 |  |  |  |  |  | CA_n48(2A)_BCS0 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n96 |  |  |  |  |  | 20, 40, 60, 80 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n46N-n48(2A)-n96A |  |  |  |  |  | CA_n46A-n48A CA_n48A-n96A |  |  |  |  |  | n46 |  |  |  |  |  | CA_n46N_BCS0 |  |  |  |  |  | 0 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n48 |  |  |  |  |  | CA_n48(2A)_BCS0 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n96 |  |  |  |  |  | 20, 40, 60, 80 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n46A-n48(2A)-n96B |  |  |  |  |  | CA_n46A-n48A CA_n48A-n96A |  |  |  |  |  | n46 |  |  |  |  |  | 20, 40, 60, 80 |  |  |  |  |  | 0 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n48 |  |  |  |  |  | CA_n48(2A)_BCS0 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n96 |  |  |  |  |  | CA_n96B_BCS0 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n46B-n48(2A)-n96B |  |  |  |  |  | CA_n46A-n48A CA_n48A-n96A |  |  |  |  |  | n46 |  |  |  |  |  | CA_n46B_BCS0 |  |  |  |  |  | 0 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n48 |  |  |  |  |  | CA_n48(2A)_BCS0 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n96 |  |  |  |  |  | CA_n96B_BCS0 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n46C-n48(2A)-n96B |  |  |  |  |  | CA_n46A-n48A CA_n48A-n96A |  |  |  |  |  | n46 |  |  |  |  |  | CA_n46C_BCS0 |  |  |  |  |  | 0 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n48 |  |  |  |  |  | CA_n48(2A)_BCS0 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n96 |  |  |  |  |  | CA_n96B_BCS0 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n46D-n48(2A)-n96B |  |  |  |  |  | CA_n46A-n48A CA_n48A-n96A |  |  |  |  |  | n46 |  |  |  |  |  | CA_n46D_BCS0 |  |  |  |  |  | 0 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n48 |  |  |  |  |  | CA_n48(2A)_BCS0 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n96 |  |  |  |  |  | CA_n96B_BCS0 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n46N-n48(2A)-n96B |  |  |  |  |  | CA_n46A-n48A CA_n48A-n96A |  |  |  |  |  | n46 |  |  |  |  |  | CA_n46N_BCS0 |  |  |  |  |  | 0 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n48 |  |  |  |  |  | CA_n48(2A)_BCS0 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n96 |  |  |  |  |  | CA_n96B_BCS0 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n46A-n48(2A)-n96C |  |  |  |  |  | CA_n46A-n48A CA_n48A-n96A |  |  |  |  |  | n46 |  |  |  |  |  | 10, 20, 40, 60, 80 |  |  |  |  |  | 0 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n48 |  |  |  |  |  | CA_n48(2A)_BCS0 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n96 | CA_n96C_BCS0 |  |
| CA_n46B-n48(2A)-n96C | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46B_BCS0 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n96 | CA_n96C_BCS0 |  |
| CA_n46C-n48(2A)-n96C | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46C_BCS0 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n96 | CA_n96C_BCS0 |  |
| CA_n46D-n48(2A)-n96C | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46D_BCS0 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n96 | CA_n96C_BCS0 |  |
| CA_n46N-n48(2A)-n96C | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46N_BCS0 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n96 | CA_n96C_BCS0 |  |
| CA_n46A-n48(2A)-n96D | CA_n46A-n48A CA_n48A-n96A | n46 | 10, 20, 40, 60, 80 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n96 | CA_n96D_BCS0 |  |
| CA_n46B-n48(2A)-n96D | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46B_BCS0 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n96 | CA_n96D_BCS0 |  |
| CA_n46C-n48(2A)-n96D | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46C_BCS0 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n96 | CA_n96D_BCS0 |  |
| CA_n46D-n48(2A)-n96D | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46D_BCS0 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n96 | CA_n96D_BCS0 |  |
| CA_n46N-n48(2A)-n96D | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46N_BCS0 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n96 | CA_n96D_BCS0 |  |
| CA_n46A-n48(2A)-n96E | CA_n46A-n48A CA_n48A-n96A | n46 | 10, 20, 40, 60, 80 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n96 | CA_n96E_BCS0 |  |
| CA_n46B-n48(2A)-n96E | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46B_BCS0 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n96 | CA_n96E_BCS0 |  |
| CA_n46C-n48(2A)-n96E | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46C_BCS0 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n96 | CA_n96E_BCS0 |  |
| CA_n46D-n48(2A)-n96E | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46D_BCS0 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n96 | CA_n96E_BCS0 |  |
| CA_n46N-n48(2A)-n96E | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46N_BCS0 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n96 | CA_n96E_BCS0 |  |
| CA_n46A-n48(3A)-n96A | CA_n46A-n48A CA_n48A-n96A | n46 | 10, 20, 40, 60, 80 | 0 |
|  |  | n48 | CA_n48(3A)_BCS0 |  |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n46B-n48(3A)-n96A | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46B_BCS0 | 0 |
|  |  | n48 | CA_n48(3A)_BCS0 |  |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n46C-n48(3A)-n96A | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46C_BCS0 | 0 |
|  |  | n48 | CA_n48(3A)_BCS0 |  |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n46D-n48(3A)-n96A | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46D_BCS0 | 0 |
|  |  | n48 | CA_n48(3A)_BCS0 |  |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n46N-n48(3A)-n96A | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46N_BCS0 | 0 |
|  |  | n48 | CA_n48(3A)_BCS0 |  |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n46A-n48(3A)-n96B | CA_n46A-n48A CA_n48A-n96A | n46 | 10, 20, 40, 60, 80 | 0 |
|  |  | n48 | CA_n48(3A)_BCS0 |  |
|  |  | n96 | CA_n96B_BCS0 |  |
| CA_n46B-n48(3A)-n96B | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46B_BCS0 | 0 |
|  |  | n48 | CA_n48(3A)_BCS0 |  |
|  |  | n96 | CA_n96B_BCS0 |  |
| CA_n46C-n48(3A)-n96B | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46C_BCS0 | 0 |
|  |  | n48 | CA_n48(3A)_BCS0 |  |
|  |  | n96 | CA_n96B_BCS0 |  |
| CA_n46D-n48(3A)-n96B | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46D_BCS0 | 0 |
|  |  | n48 | CA_n48(3A)_BCS0 |  |
|  |  | n96 | CA_n96B_BCS0 |  |
| CA_n46N-n48(3A)-n96B | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46N_BCS0 | 0 |
|  |  | n48 | CA_n48(3A)_BCS0 |  |
|  |  | n96 | CA_n96B_BCS0 |  |
| CA_n46A-n48(3A)-n96C | CA_n46A-n48A CA_n48A-n96A | n46 | 10, 20, 40, 60, 80 | 0 |
|  |  | n48 | CA_n48(3A)_BCS0 |  |
|  |  | n96 | CA_n96C_BCS0 |  |
| CA_n46B-n48(3A)-n96C | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46B_BCS0 | 0 |
|  |  | n48 | CA_n48(3A)_BCS0 |  |
|  |  | n96 | CA_n96C_BCS0 |  |
| CA_n46C-n48(3A)-n96C | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46C_BCS0 | 0 |
|  |  | n48 | CA_n48(3A)_BCS0 |  |
|  |  | n96 | CA_n96C_BCS0 |  |
| CA_n46D-n48(3A)-n96C | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46D_BCS0 | 0 |
|  |  | n48 | CA_n48(3A)_BCS0 |  |
|  |  | n96 | CA_n96C_BCS0 |  |
| CA_n46N-n48(3A)-n96C | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46N_BCS0 | 0 |
|  |  | n48 | CA_n48(3A)_BCS0 |  |
|  |  | n96 | CA_n96C_BCS0 |  |
| CA_n46A-n48(3A)-n96D | CA_n46A-n48A CA_n48A-n96A | n46 | 10, 20, 40, 60, 80 | 0 |
|  |  | n48 | CA_n48(3A)_BCS0 |  |
|  |  | n96 | CA_n96D_BCS0 |  |
| CA_n46B-n48(3A)-n96D | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46B_BCS0 | 0 |
|  |  | n48 | CA_n48(3A)_BCS0 |  |
|  |  | n96 | CA_n96D_BCS0 |  |
| CA_n46C-n48(3A)-n96D | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46C_BCS0 | 0 |
|  |  | n48 | CA_n48(3A)_BCS0 |  |
|  |  | n96 | CA_n96D_BCS0 |  |
| CA_n46D-n48(3A)-n96D | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46D_BCS0 | 0 |
|  |  | n48 | CA_n48(3A)_BCS0 |  |
|  |  | n96 | CA_n96D_BCS0 |  |
| CA_n46N-n48(3A)-n96D | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46N_BCS0 | 0 |
|  |  | n48 | CA_n48(3A)_BCS0 |  |
|  |  | n96 | CA_n96D_BCS0 |  |
| CA_n46A-n48(3A)-n96E | CA_n46A-n48A CA_n48A-n96A | n46 | 10, 20, 40, 60, 80 | 0 |
|  |  | n48 | CA_n48(3A)_BCS0 |  |
|  |  | n96 | CA_n96E_BCS0 |  |
| CA_n46B-n48(3A)-n96E | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46B_BCS0 | 0 |
|  |  | n48 | CA_n48(3A)_BCS0 |  |
|  |  | n96 | CA_n96E_BCS0 |  |
| CA_n46C-n48(3A)-n96E | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46C_BCS0 | 0 |
|  |  | n48 | CA_n48(3A)_BCS0 |  |
|  |  | n96 | CA_n96E_BCS0 |  |
| CA_n46D-n48(3A)-n96E | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46D_BCS0 | 0 |
|  |  | n48 | CA_n48(3A)_BCS0 |  |
|  |  | n96 | CA_n96E_BCS0 |  |
| CA_n46N-n48(3A)-n96E | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46N_BCS0 | 0 |
|  |  | n48 | CA_n48(4A)_BCS0 |  |
|  |  | n96 | CA_n96E_BCS0 |  |
| CA_n46A-n48(4A)-n96A | CA_n46A-n48A CA_n48A-n96A | n46 | 10, 20, 40, 60, 80 | 0 |
|  |  | n48 | CA_n48(4A)_BCS0 |  |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n46B-n48(4A)-n96A | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46B_BCS0 | 0 |
|  |  | n48 | CA_n48(4A)_BCS0 |  |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n46C-n48(4A)-n96A | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46C_BCS0 | 0 |
|  |  | n48 | CA_n48(4A)_BCS0 |  |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n46D-n48(4A)-n96A | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46D_BCS0 | 0 |
|  |  | n48 | CA_n48(4A)_BCS0 |  |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n46N-n48(4A)-n96A | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46N_BCS0 | 0 |
|  |  | n48 | CA_n48(4A)_BCS0 |  |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n46A-n48(4A)-n96B | CA_n46A-n48A CA_n48A-n96A | n46 | 10, 20, 40, 60, 80 | 0 |
|  |  | n48 | CA_n48(4A)_BCS0 |  |
|  |  | n96 | CA_n96B_BCS0 |  |
| CA_n46B-n48(4A)-n96B | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46B_BCS0 | 0 |
|  |  | n48 | CA_n48(4A)_BCS0 |  |
|  |  | n96 | CA_n96B_BCS0 |  |
| CA_n46C-n48(4A)-n96B | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46C_BCS0 | 0 |
|  |  | n48 | CA_n48(4A)_BCS0 |  |
|  |  | n96 | CA_n96B_BCS0 |  |
| CA_n46D-n48(4A)-n96B | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46D_BCS0 | 0 |
|  |  | n48 | CA_n48(4A)_BCS0 |  |
|  |  | n96 | CA_n96B_BCS0 |  |
| CA_n46N-n48(4A)-n96B | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46N_BCS0 | 0 |
|  |  | n48 | CA_n48(4A)_BCS0 |  |
|  |  | n96 | CA_n96B_BCS0 |  |
| CA_n46A-n48(4A)-n96C | CA_n46A-n48A CA_n48A-n96A | n46 | 10, 20, 40, 60, 80 | 0 |
|  |  | n48 | CA_n48(4A)_BCS0 |  |
|  |  | n96 | CA_n96C_BCS0 |  |
| CA_n46B-n48(4A)-n96C | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46B_BCS0 | 0 |
|  |  | n48 | CA_n48(4A)_BCS0 |  |
|  |  | n96 | CA_n96C_BCS0 |  |
| CA_n46C-n48(4A)-n96C | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46C_BCS0 | 0 |
|  |  | n48 | CA_n48(4A)_BCS0 |  |
|  |  | n96 | CA_n96C_BCS0 |  |
| CA_n46D-n48(4A)-n96C | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46D_BCS0 | 0 |
|  |  | n48 | CA_n48(4A)_BCS0 |  |
|  |  | n96 | CA_n96C_BCS0 |  |
| CA_n46N-n48(4A)-n96C | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46N_BCS0 | 0 |
|  |  | n48 | CA_n48(4A)_BCS0 |  |
|  |  | n96 | CA_n96C_BCS0 |  |
| CA_n46A-n48(4A)-n96D | CA_n46A-n48A CA_n48A-n96A | n46 | 10, 20, 40, 60, 80 | 0 |
|  |  | n48 | CA_n48(4A)_BCS0 |  |
|  |  | n96 | CA_n96D_BCS0 |  |
| CA_n46B-n48(4A)-n96D | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46B_BCS0 | 0 |
|  |  | n48 | CA_n48(4A)_BCS0 |  |
|  |  | n96 | CA_n96D_BCS0 |  |
| CA_n46C-n48(4A)-n96D | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46C_BCS0 | 0 |
|  |  | n48 | CA_n48(4A)_BCS0 |  |
|  |  | n96 | CA_n96D_BCS0 |  |
| CA_n46D-n48(4A)-n96D | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46D_BCS0 | 0 |
|  |  | n48 | CA_n48(4A)_BCS0 |  |
|  |  | n96 | CA_n96D_BCS0 |  |
| CA_n46N-n48(4A)-n96D | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46N_BCS0 | 0 |
|  |  | n48 | CA_n48(4A)_BCS0 |  |
|  |  | n96 | CA_n96D_BCS0 |  |
| CA_n46A-n48(4A)-n96E | CA_n46A-n48A CA_n48A-n96A | n46 | 10, 20, 40, 60, 80 | 0 |
|  |  | n48 | CA_n48(4A)_BCS0 |  |
|  |  | n96 | CA_n96E_BCS0 |  |
| CA_n46B-n48(4A)-n96E | CA_n46A-n48A CA_n48A-n96A | n46 | CA_n46B_BCS0 | 0 |
|  |  | n48 | CA_n48(4A)_BCS0 |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n96 |  |  |  |  |  | CA_n96E_BCS0 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n46C-n48(4A)-n96E |  |  |  |  |  | CA_n46A-n48A CA_n48A-n96A |  |  |  |  |  | n46 |  |  |  |  |  | CA_n46C_BCS0 |  |  |  |  |  | 0 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n48 |  |  |  |  |  | CA_n48(4A)_BCS0 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n96 |  |  |  |  |  | CA_n96E_BCS0 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n46D-n48(4A)-n96E |  |  |  |  |  | CA_n46A-n48A CA_n48A-n96A |  |  |  |  |  | n46 |  |  |  |  |  | CA_n46D_BCS0 |  |  |  |  |  | 0 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n48 |  |  |  |  |  | CA_n48(4A)_BCS0 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n96 |  |  |  |  |  | CA_n96E_BCS0 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n46N-n48(4A)-n96E |  |  |  |  |  | CA_n46A-n48A CA_n48A-n96A |  |  |  |  |  | n46 |  |  |  |  |  | CA_n46N_BCS0 |  |  |  |  |  | 0 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n48 |  |  |  |  |  | CA_n48(4A)_BCS0 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n96 | CA_n96E_BCS0 |  |
| CA_n48A-n66A-n70A | CA_n48A-n66A CA_n48-n70A | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n48A-n66(2A)-n70A | CA_n48A-n66A CA_n48-n70A | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n66 | CA_n66(2A)_BCS0 |  |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n48(2A)-n66A-n70A | CA_n48A-n66A CA_n48-n70A | n48 | CA_n48(2A)_BCS1 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n48B-n66A-n70A | CA_n48A-n66A CA_n48-n70A | n48 | CA_n48B_BCS2 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n48A-n66A-n71A | CA_n48A-n71A CA_n66A-n71A CA_n48A-n66A | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n48A-n66(2A)-n71A | CA_n48A-n71A CA_n66A-n71A CA_n48A-n66A | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n66 | CA_n66(2A)_BCS0 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n48(2A)-n66A-n71A | CA_n48A-n71A CA_n66A-n71A CA_n48A-n66A | n48 | CA_n48(2A)_BCS1 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n48B-n66A-n71A | CA_n48A-n71A CA_n66A-n71A CA_n48A-n66A | n48 | CA_n48B_BCS2 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n48A-n66A-n71(2A) | CA_n48A-n71A CA_n66A-n71A CA_n48A-n66A | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n71 | CA_n71(2A)_BCS0 |  |
| CA_n48A-n66A-n77A | CA_n48A-n66A CA_n66A-n77A | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n48A-n66A-n77C | CA_n48A-n66A CA_n66A-n77A CA_n77C | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n48B-n66A-n77A | CA_n48A-n66A CA_n66A-n77A | n48 | CA_n48B_BCS0 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n48 | CA_n48B_BCS1 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n48 | CA_n48B_BCS2 | 2 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n48(2A)-n66A-n77A | CA_n48A-n66A CA_n66A-n77A | n48 | CA_n48(2A)_BCS0 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n48 | CA_n48(2A)_BCS1 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n48(2A)-n66A-n77C | CA_n48A-n66A CA_n66A-n77A | n48 | CA_n48(2A)_BCS0 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n48 | CA_n48(2A)_BCS0 | 1 |
|  |  |  |  |  |  |  |  |  |  |  |  | n66 |  |  |  |  |  | 5, 10, 15, 20, 25, 30, 40 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  | CA_n77C_BCS1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n48 |  |  |  |  |  | CA_n48(2A)_BCS1 |  |  |  |  |  | 2 |  |  |  |  |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n48 | CA_n48(2A)_BCS1 | 3 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n48A-n70A-n71A | CA_n48A-n71A CA_n70A-n71A CA_n48A-n70A | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n48(2A)-n70A-n71A | CA_n48A-n71A CA_n70A-n71A CA_n48A-n70A | n48 | CA_n48(2A)_BCS1 | 0 |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n48B-n70A-n71A | CA_n48A-n71A CA_n70A-n71A CA_n48A-n70A | n48 | CA_n48B_BCS2 | 0 |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n48A-n70A-n71(2A) | CA_n48A-n71A CA_n70A-n71A CA_n48A-n70A | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
|  |  | n71 | CA_n71(2A)_BCS0 |  |
| CA_n66A-n70A-n71A | CA_n66A-n71A CA_n70A-n71A | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n71 |  |  |  |  |  | 5, 10, 15, 20 |  |  |  |  |  |  |  |  |  |  |  |
| _CA_n66A-n70A-n78A |  |  |  |  |  | CA_n66A-n78A CA_n70A-n78A |  |  |  |  |  | n66 |  |  |  |  |  | 10, 15, 20, 25, 30, 40 |  |  |  |  |  | 0 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n70 |  |  |  |  |  | 5, 10, 15, 201, 251 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n66A-n70A-n71(2A) | CA_n66A-n71A CA_n70A-n71A | n66 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
|  |  | n71 | CA_n71(2A)_BCS0 |  |
| CA_n66B-n70A-n71A | CA_n66A-n71A CA_n70A-n71A | n66 | CA_n66B_BCS0. | 0 |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n66(2A)-n70A-n71A | CA_n66A-n71A CA_n70A-n71A | n66 | CA_n66(2A)_BCS0 | 0 |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n66A-n71A-n77A | CA_n66A-n71A CA_n66A-n77A CA_n71A-n77A | n66 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n66A-n71B-n77A | CA_n66A-n71A CA_n66A-n77A CA_n71A-n77A | n66 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n71 | CA_n71B_BCS2 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n66A-n71(2A)-n77A | CA_n66A-n71A CA_n66A-n77A CA_n71A-n77A | n66 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n71 | CA_n71(2A)_BCS0 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n66(2A)-n71A-n77A | CA_n66A-n71A, CA_n66A-n77A, CA_n71A-n77A | n66 | CA_n66(2A)_BCS1 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n66A-n71A-n77(2A) | CA_n66A-n71A, CA_n66A-n77A, CA_n71A-n77A | n66 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n66(2A)-n71A-n77(2A) | CA_n66A-n71A, CA_n66A-n77A, CA_n71A-n77A | n66 | CA_n66(2A)_BCS1 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n66A-n71A-n78A | CA_n66A-n78A CA_n66A-n71A CA_n71A-n78A | n66 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n66A-n71A-n78(2A) | CA_n66A-n78A CA_n66A-n71A CA_n71A-n78A | n66 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n66(2A)-n71A-n78A | CA_n66A-n78A CA_n66A-n71A CA_n71A-n78A | n66 | CA_n66(2A)_BCS1 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n66(2A)-n71A-n78(2A) | CA_n66A-n78A CA_n66A-n71A CA_n71A-n78A | n66 | CA_n66(2A)_BCS1 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| NOTE 1: This UE channel bandwidth is applicable only to downlink NOTE 2: For the 20 MHz bandwidth, the minimum requirements are specified for NR UL carrier frequencies confined to either 713-723 MHz or 728-738 MHz. NOTE 3: The SCS of each channel bandwidth for NR band refers to Table 5.3.5-1. NOTE 4: The minimum requirements only apply for non-simultaneous Tx/Rx between all carriers for TDD combinations. NOTE 5: Simultaneous Rx/Tx capability for TDD combinations does not apply for UEs supporting band n78 with an n77 implementation. NOTE 6: Only single uplink carriers with power class other than PC3 are listed. NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination |  |  |  |  |

## << End of change 2>>

## << Start of change 3>>

##### 6.2A.4.2.4	ΔTIB,c for Inter-band CA (three bands)

Table 6.2A.4.2.4-1: ΔTIB,c due to NR CA (three bands)

| Inter-band CA combination | NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| CA_n1-n3-n5 | n1 | 0.3 |
|  | n3 | 0.3 |
|  | n5 | 0.3 |
| CA_n1-n3-n7 | n1 | 0.6 |
|  | n3 | 0.6 |
|  | n7 | 0.6 |
| CA_n1-n3-n8 | n1 | 0.3 |
|  | n3 | 0.3 |
|  | n8 | 0.3 |
| CA_n1-n3-n18 | n1 | 0.3 |
|  | n3 | 0.3 |
|  | n18 | 0.3 |
| CA_n1-n3-n20 | n1 | 0.3 |
|  | n3 | 0.3 |
|  | n20 | 0.3 |
| CA_n1-n3-n28 | n1 | 0.3 |
|  | n3 | 0.3 |
|  | n28 | 0.6 |
| CA_n1-n3-n41 | n1 | 0.5 |
|  | n3 | 0.5 |
|  | n41 | 0.35 |
|  |  | 0.86 |
| CA_n1-n3-n77 | n1 | 0.6 |
|  | n3 | 0.6 |
|  | n77 | 0.8 |
| CA_n1-n3-n78 | n1 | 0.6 |
|  | n3 | 0.6 |
|  | n78 | 0.8 |
| CA_n1-n3-n79 | n1 | 0.3 |
|  | n3 | 0.3 |
|  | n79 | 0.8 |
| CA_n1-n5-n7 | n1 | 0.5 |
|  | n5 | 0.3 |
|  | n7 | 0.6 |
| CA_n1-n5-n28 | n1 | 0.3 |
|  | n5 | 0.6 |
|  | n28 | 0.6 |
| CA_n1-n5-n78 | n1 | 0.6 |
|  | n5 | 0.6 |
|  | n78 | 0.8 |
| CA_n1-n7-n8 | n1 | 0.5 |
|  | n7 | 0.6 |
|  | n8 | 0.6 |
| CA_n1-n7-n28 | n1 | 0.5 |
|  | n7 | 0.6 |
|  | n28 | 0.6 |
| CA_n1-n7-n78 | n1 | 0.6 |
|  | n7 | 0.6 |
|  | n78 | 0.8 |
| CA_n1-n8-n28 | n1 | 0.3 |
|  | n8 | 0.6 |
|  | n28 | 0.6 |
| CA_n1-n8-n77 | n1 | 0.3 |
|  | n8 | 0.6 |
|  | n77 | 0.8 |
| CA_n1-n8-n78 | n1 | 0.3 |
|  | n8 | 0.6 |
|  | n78 | 0.8 |
| CA_n1-n8-n79 | n1 | 0.3 |
|  | n8 | 0.6 |
|  | n79 | 0.8 |
| CA_n1-n18-n28 | n1 | 0.3 |
|  | n18 | 0.5 |
|  | n28 | 0.5 |
| CA_n1-n18-n41 | n1 | 0.5 |
|  | n18 | 0.3 |
|  | n41 | 0.5 |
| CA_n1-n18-n77 | n1 | 0.3 |
|  | n18 | 0.3 |
|  | n77 | 0.8 |
| CA_n1-n20-n67 | n1 | 0.5 |
|  | n20 | 0.6 |
|  | n67 | 0.8 |
| CA_n1-n20-n78 | n1 | 0.3 |
|  | n20 | 0.6 |
|  | n78 | 0.8 |
| CA_n1-n28-n40 | n1 | 0.6 |
|  | n28 | 0.3 |
|  | n40 | 0.5 |
| CA_n1-n28-n41 | n1 | 0.5 |
|  | n28 | 0.6 |
|  | n41 | 0.6 |
| CA_n1-n28-n77 | n1 | 0.6 |
|  | n28 | 0.6 |
|  | n77 | 0.8 |
| CA_n1-n28-n78 | n1 | 0.3 |
|  | n28 | 0.6 |
|  | n78 | 0.8 |
| CA_n1-n28-n79 | n1 | 0 |
|  | n28 | 0.2 |
|  | n79 | 0.5 |
| CA_n1-n40-n78 | n1 | 0.3 |
|  | n40 | 0.5 |
|  | n78 | 0.8 |
| CA_n1-n41-n77 | n1 | 0.5 |
|  | n41 | 0.5 |
|  | n77 | 0.8 |
| CA_n1-n77-n79 | n1 | 0.6 |
|  | n77 | 0.8 |
|  | n79 | 0.5 |
| CA_n1-n78-n79 | n1 | 0.3 |
|  | n78 | 0.8 |
|  |  | 1.57 |
|  | n79 | 0.5 |
|  |  | 1.57 |
| CA_n2-n5-n30 | n2 | 0.5 |
|  | n5 | 0.3 |
|  | n30 | 0.3 |
| CA_n2-n5-n48 | n2 | 0.6 |
|  | n5 | 0.3 |
|  | n48 | 0.8 |
| CA_n2-n5-n66 | n2 | 0.5 |
|  | n5 | 0.3 |
|  | n66 | 0.5 |
| CA_n2-n5-n77 | n2 | 0.6 |
|  | n5 | 0.8 |
|  | n77 | 0.8 |
| CA_n2-n12-n30 | n2 | 0.5 |
|  | n12 | 0.3 |
|  | n30 | 0.3 |
| CA_n2-n12-n66 | n2 | 0.5 |
|  | n12 | 0.8 |
|  | n66 | 0.5 |
| CA_n2-n12-n77 | n2 | 0.6 |
|  | n12 | 0.3 |
|  | n77 | 0.8 |
| CA_n2-n14-n30 | n2 | 0.5 |
|  | n14 | 0.3 |
|  | n30 | 0.5 |
| CA_n2-n14-n66 | n2 | 0.5 |
|  | n14 | 0.3 |
|  | n66 | 0.5 |
| CA_n2-n14-n77 | n2 | 0.5 |
|  | n14 | 0.3 |
|  | n77 | 0.8 |
| CA_n2-n29-n30 | n2 | 0.5 |
|  | n30 | 0.3 |
| CA_n2-n29-n66 | n2 | 0.5 |
|  | n66 | 0.5 |
| CA_n2-n29-n77 | n2 | 0.6 |
|  | n77 | 0.8 |
| CA_n2-n30-n66 | n2 | 0.5 |
|  | n30 | 0.3 |
|  | n66 | 0.5 |
| CA_n2-n30-n77 | n2 | 0.6 |
|  | n30 | 0.3 |
|  | n77 | 0.8 |
| CA_n2-n48-n66 | n2 | 0.6 |
|  | n48 | 0.8 |
|  | n66 | 0.6 |
| CA_n2-n48-n77 | n2 | 0.6 |
|  | n48 | 0.8 |
|  | n77 | 0.8 |
| CA_n2-n66-n77 | n2 | 0.6 |
|  | n66 | 0.6 |
|  | n77 | 0.8 |
| CA_n2-n66-n78 | n2 | 0.6 |
|  | n66 | 0.6 |
|  | n78 | 0.8 |
| CA_n2-n71-n78 | n2 | 0.6 |
|  | n71 | 0.6 |
|  | n78 | 0.8 |
| CA_n3-n5-n7 | n3 | 0.5 |
|  | n5 | 0.3 |
|  | n7 | 0.5 |
| CA_n3-n5-n28 | n3 | 0.3 |
|  | n5 | 0.6 |
|  | n28 | 0.5 |
| CA_n3-n5-n78 | n3 | 0.6 |
|  | n5 | 0.6 |
|  | n78 | 0.8 |
| CA_n3-n7-n8 | n3 | 0.5 |
|  | n7 | 0.5 |
|  | n8 | 0.6 |
| CA_n3-n7-n28 | n3 | 0.5 |
|  | n7 | 0.5 |
|  | n28 | 0.3 |
| CA_n3-n7-n78 | n3 | 0.6 |
|  | n7 | 0.6 |
|  | n78 | 0.8 |
| CA_n3-n8-n28 | n3 | 0.3 |
|  | n8 | 0.6 |
|  | n28 | 0.5 |
| CA_n3-n8-n77 | n3 | 0.6 |
|  | n8 | 0.6 |
|  | n77 | 0.8 |
| CA_n3-n8-n78 | n3 | 0.6 |
|  | n8 | 0.6 |
|  | n78 | 0.8 |
| CA_n3-n18-n28 | n3 | 0.3 |
|  | n18 | 0.5 |
|  | n28 | 0.3 |
| CA_n3-n18-n41 | n3 | 0.5 |
|  | n18 | 0.3 |
|  | n41 | 0.31/0.82 |
| CA_n3-n18-n77 | n3 | 0.6 |
|  | n18 | 0.3 |
|  | n77 | 0.8 |
| CA_n3-n20-n67 | n3 | 0.3 |
|  | n20 | 0.5 |
|  | n67 | 0.5 |
| CA_n3-n20-n78 | n3 | 0.6 |
|  | n20 | 0.6 |
|  | n78 | 0.8 |
| CA_n3-n28-n41 | n3 | 0.5 |
|  | n28 | 0.3 |
|  | n41 | 0.31/0.82 |
| CA_n3-n28-n77 | n3 | 0.6 |
|  | n28 | 0.5 |
|  | n77 | 0.8 |
| CA_n3-n28-n78 | n3 | 0.5 |
|  | n28 | 0.3 |
|  | n78 | 0.8 |
| CA_n3-n28-n79 | n3 | 0.3 |
|  | n28 | 0.5 |
|  | n79 | 0.8 |
| CA_n3-n77-n79 | n3 | 0.6 |
|  | n77 | 0.8 |
|  | n79 | 0 |
| CA_n3-n40-n41 | n3 | 0.5 |
|  | n40 | 0.5 |
|  | n41 | 0.51,3 |
|  |  | 0.82,3 |
| CA_n3-n41-n77 | n3 | 0.6 |
|  | n41 | 0.31/0.82 |
|  | n77 | 0.8 |
| CA_n3-n41-n78 | n3 | 0.6 |
|  | n41 | 0.31/0.82 |
|  | n78 | 0.8 |
| CA_n3-n41-n79 | n3 | 0.3 |
|  | n41 | 0.31 |
|  |  | 0.82 |
|  | n79 | 0.8 |
| CA_n5-n7-n28 | n5 | 0.5 |
|  | n7 | 0.3 |
|  | n28 | 0.6 |
| CA_n5-n7-n78 | n5 | 0.6 |
|  | n7 | 0.6 |
|  | n78 | 0.8 |
| CA_n5-n12-n77 | n5 | 0.8 |
|  | n12 | 0.4 |
|  | n77 | 0.5 |
| CA_n5-n14-n77 | n5 | 0.5 |
|  | n14 | 0.3 |
|  | n77 | 0.8 |
| CA_n5-n25-n66 | n5 | 0.3 |
|  | n25 | 0.5 |
|  | n66 | 0.5 |
| CA_n5-n25-n77 | n5 | 0.6 |
|  | n25 | 0.6 |
|  | n77 | 0.8 |
| CA_n5-n25-n78 | n5 | 0.6 |
|  | n25 | 0.6 |
|  | n78 | 0.8 |
| CA_n5-n29-n77 | n5 | 0.8 |
|  | n77 | 0.5 |
| CA_n5-n30-n66 | n5 | 0.3 |
|  | n30 | 0.3 |
|  | n66 | 0.5 |
| CA_n5-n30-n77 | n5 | 0.6 |
|  | n30 | 0.3 |
|  | n77 | 0.8 |
| CA_n5-n48-n66 | n5 | 0.3 |
|  | n48 | 0.8 |
|  | n66 | 0.6 |
| CA_n5-n48-n77 | n5 | 0.6 |
|  | n48 | 0.8 |
|  | n77 | 0.8 |
| CA_n5-n66-n77 | n5 | 0.6 |
|  | n66 | 0.6 |
|  | n77 | 0.8 |
| CA_n5_n66-n78 | n5 | 0.6 |
|  | n66 | 0.6 |
|  | n78 | 0.8 |
| CA_n7-n8-n28 | n7 | 0.3 |
|  | n8 | 0.6 |
|  | n28 | 0.5 |
| CA_n7-n8-n78 | n7 | 0.5 |
|  | n8 | 0.6 |
|  | n78 | 0.8 |
| CA_n7_n25-n66 | n7 | 0.5 |
|  | n25 | 0.5 |
|  | n66 | 0.5 |
| CA_n7-n25-n77 | n7 | 0.5 |
|  | n25 | 0.6 |
|  | n77 | 0.8 |
| CA_n7-n25-n78 | n7 | 0.5 |
|  | n25 | 0.6 |
|  | n78 | 0.8 |
| CA_n7_n28-n78 | n7 | 0.3 |
|  | n28 | 0.3 |
|  | n78 | 0.8 |
| CA_n7-n46-n78 | n7 | 0.5 |
|  | n46 | 0 |
|  | n78 | 0.8 |
| CA_n7-n66-n77 | n7 | 0.5 |
|  | n66 | 0.6 |
|  | n77 | 0.8 |
| CA_n7_n66-n78 | n7 | 0.5 |
|  | n66 | 0.6 |
|  | n78 | 0.8 |
| CA_n8-n28-n78 | n8 | 0.6 |
|  | n28 | 0.5 |
|  | n78 | 0.8 |
| CA_n8-n39-n41 | n8 | 0.6 |
|  | n39 | 0.54 |
|  | n41 | 0.54 |
| CA_n8-n40-n41 | n8 | 0.3 |
|  | n40 | 0.33 |
|  | n41 | 0.33 |
| CA_n8-n41-n79 | n8 | 0.6 |
|  | n41 | 0.3 |
|  | n79 | 0.8 |
| CA_n8-n78-n79 | n8 | 0.6 |
|  | n78 | 0.8 |
|  | n79 | 0.8 |
| CA_n12-n30-n66 | n12 | 0.8 |
|  | n30 | 0.3 |
|  | n66 | 0.5 |
| CA_n12-n30-n77 | n12 | 0.5 |
|  | n30 | 0.3 |
|  | n77 | 0.5 |
| CA_n12-n66-n77 | n12 | 0.8 |
|  | n66 | 0.6 |
|  | n77 | 0.8 |
| CA_n13-n25-n66 | n13 | 0.3 |
|  | n25 | 0.5 |
|  | n66 | 0.5 |
| CA_n13-n25-n77 | n13 | 0.3 |
|  | n25 | 0.6 |
|  | n77 | 0.8 |
| CA_n13-n66-n77 | n13 | 0.5 |
|  | n66 | 0.6 |
|  | n77 | 0.8 |
| CA_n14-n30-n66 | n14 | 0.3 |
|  | n30 | 0.3 |
|  | n66 | 0.5 |
| CA_n14-n30-n77 | n14 | 0.5 |
|  | n30 | 0.3 |
|  | n77 | 0.8 |
| CA_n14-n66-n77 | n14 | 0.6 |
|  | n66 | 0.6 |
|  | n77 | 0.8 |
| CA_n18-n28-n41 | n18 | 0.4 |
|  | n28 | 0.4 |
|  | n41 | 0.3 |
| CA_n18-n28-n77 | n18 | 0.5 |
|  | n28 | 0.5 |
|  | n77 | 0.8 |
| CA_n18-n41-n77 | n18 | 0.3 |
|  | n41 | 0.3 |
|  | n77 | 0.8 |
| CA_n20-n28-n78 | n20 | 0.6 |
|  | n28 | 0.5 |
|  | n78 | 0.8 |
| CA_n24-n41-n48 | n24 | 0.6 |
|  | n41 | 0.41 |
|  |  | 0.92 |
|  | n48 | 0.8 |
| CA_n24-n41-n77 | n24 | 0.6 |
|  | n41 | 0.45 |
|  |  | 0.96 |
|  | n77 | 0.8 |
| CA_n24-n48-n77 | n24 | 0.6 |
|  | n48 | 0.8 |
|  | n77 | 0.8 |
| CA_n25-n29-n66 | n25 | 0.5 |
|  | n29 | 0 |
|  | n66 | 0.5 |
| CA_n25-n38-n78 | n25 | 0.5 |
|  | n38 | 0.4 |
|  | n78 | 0.8 |
| CA_n25-n41-n66 | n25 | 0.5 |
|  | n41 | 0.85 |
|  |  | 1.36 |
|  | n66 | 0.5 |
| CA_n25-n41-n71 | n25 | 0.5 |
|  | n41 | 0.5 |
|  | n71 | 0.6 |
| CA_n25-n41-n77 | n25 | 0.5 |
|  | n41 | 0.5 |
|  | n77 | 0.6 |
| CA_n25-n41-n78 | n25 | 0.6 |
|  | n41 | 0.5 |
|  | n78 | 0.8 |
| CA_n25-n48-n66 | n25 | 0.6 |
|  | n48 | 0.8 |
|  | n66 | 0.6 |
| CA_n25-n66-n71 | n25 | 0.5 |
|  | n66 | 0.5 |
|  | n71 | 0.6 |
| CA_n25-n66-n77 | n25 | 0.6 |
|  | n66 | 0.6 |
|  | n77 | 0.8 |
| CA_n25-n66-n78 | n25 | 0.6 |
|  | n66 | 0.6 |
|  | n78 | 0.8 |
| CA_n25-n71-n77 | n25 | 0.6 |
|  | n71 | 0.6 |
|  | n77 | 0.8 |
| CA_n25-n71-n78 | n25 | 0.6 |
|  | n71 | 0.6 |
|  | n78 | 0.8 |
| CA_n26-n66-n70 | n26 | 0.3 |
|  | n66 | 0.5 |
|  | n70 | 0.5 |
| CA_n28-n40-n41 | n28 | 0.3 |
|  | n40 | 0.5 |
|  | n41 | 0.5 |
| CA_n28-n40-n78 | n28 | 0.5 |
|  | n40 | 0.3 |
|  | n78 | 0.8 |
| CA_n28-n40-n79 | n28 | 0.5 |
|  | n40 | 0.3 |
|  | n79 | 0.8 |
| CA_n28-n41-n79 | n28 | 0.5 |
|  | n41 | 0.3 |
|  | n79 | 0.8 |
| CA_n28-n41-n77 | n28 | 0.5 |
|  | n41 | 0.3 |
|  | n77 | 0.8 |
| CA_n28-n41-n78 | n28 | 0.5 |
|  | n41 | 0.3 |
|  | n78 | 0.8 |
| CA_n28-n46-n78 | n28 | 0.5 |
|  | n46 | 0 |
|  | n78 | 0.8 |
| CA_n28-n77-n79 | n28 | 0.5 |
|  | n77 | 0.8 |
|  | n79 | 0.5 |
| CA_n28-n78-n79 | n28 | 0.5 |
|  | n78 | 0.8 |
|  |  | 1.57 |
|  | n79 | 0.5 |
|  |  | 1.57 |
| CA_n29-n30-n66 | n30 | 0.3 |
|  | n66 | 0.5 |
| CA_n29-n30-n77 | n30 | 0.3 |
|  | n77 | 0.5 |
| CA_n29-n66-n70 | n29 | 0 |
|  | n66 | 0.5 |
|  | n70 | 0.5 |
| CA_n29-n66-n77 | n66 | 0.6 |
|  | n77 | 0.8 |
| CA_n30-n66-n77 | n30 | 0.3 |
|  | n66 | 0.6 |
|  | n77 | 0.8 |
| CA_n38-n66-n78 | n38 | 0.5 |
|  | n66 | 0.5 |
|  | n78 | 0.8 |
| CA_n39-n40-n41 | n39 | 0.3 |
|  | n40 | 0.3 |
|  | n41 | 0.3 |
| CA_n39-n40-n79 | n39 | 0.3 |
|  | n40 | 0 |
|  | n79 | 0.8 |
| CA_n39-n41-n79 | n39 | 0.3 |
|  | n41 | 0.34 |
|  | n79 | 0.84 |
| CA_n40-n41-n79 | n40 | 0.53 |
|  | n41 | 0.53 |
|  | n79 | 0.8 |
| CA_n41-n66-n71 | n41 | 0.85 |
|  |  | 1.36 |
|  | n66 | 0.5 |
|  | n71 | 0.3 |
| CA_n41-n66-n77 | n41 | 0.5 |
|  | n66 | 0.6 |
|  | n77 | 0.8 |
| CA_n41-n66-n78 | n41 | 0.5 |
|  | n66 | 0.6 |
|  | n78 | 0.8 |
| CA_n41-n70-n78 | n41 | 0.6 |
|  | n70 | 0.6 |
|  | n78 | 0.8 |
| CA_n41-n71-n77 | n41 | 0.3 |
|  | n71 | 0.5 |
|  | n77 | 0.8 |
| CA_n41-n71-n78 | n41 | 0.3 |
|  | n71 | 0.5 |
|  | n78 | 0.8 |
| CA_n46-n48-n96 | n46 | 0.5 |
|  | n48 | 0.8 |
|  | n96 | 0.6 |
| CA_n48-n66-n70 | n48 | 0.8 |
|  | n66 | 0.6 |
|  | n70 | 0.6 |
| CA_n48-n66-n71 | n48 | 0.5 |
|  | n66 | 0.5 |
|  | n71 | 0.3 |
| CA_n48-n66-n77 | n48 | 0.8 |
|  | n66 | 0.6 |
|  | n77 | 0.8 |
| CA_n48-n70-n71 | n48 | 0.5 |
|  | n70 | 0.5 |
|  | n71 | 0.3 |
| CA_n66-n70-n71 | n66 | 0.5 |
|  | n70 | 0.5 |
|  | n71 | 0.6 |
| CA_n66-n71-n77 | n66 | 0.6 |
|  | n71 | 0.6 |
|  | n77 | 0.8 |
| CA_n66-n71-n78 | n66 | 0.6 |
|  | n71 | 0.5 |
|  | n78 | 0.8 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2515-2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2515 MHz. NOTE 3: Only applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx among band 40 and 41. NOTE 4: Applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx between n39 and n41. NOTE 5: The requirement is applied for UE transmitting on the frequency range of 2545 - 2690 MHz. NOTE 6: The requirement is applied for UE transmitting on the frequency range of 2496 - 2545 MHz. NOTE 7: The requirements only apply for UE supporting inter-band carrier aggregation with simultaneous Rx/Tx capability, and NR UL carrier frequencies are confined to 3700 MHz-3800MHz for n78 and 4400 MHz-4500MHz for n79. Simultaneous Rx/Tx capability does not apply for UEs supporting band n78 with a n77 implementation. |  |  |

## << End of change 3>>

## << Start of change 4>>

##### 7.3A.3.2.3	ΔRIB,c for three bands

Table 7.3A.3.2.3-1: ΔRIB,c due to CA (three bands)

| Inter-band CA combination | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| CA_n1-n3-n5 | n1 | 0 |
|  | n3 | 0 |
|  | n5 | 0 |
| CA_n1-n3-n8 | n1 | 0.2 |
|  | n3 | 0.2 |
|  | n8 | 0.5 |
| CA_n1-n3-n18 | n1 | 0 |
|  | n3 | 0 |
|  | n18 | 0 |
| CA_n1-n3-n20 | n1 | 0 |
|  | n3 | 0 |
|  | n20 | 0 |
| CA_n1-n3-n28 | n28 | 0.2 |
| CA_n1-n3-n41 | n41 | 05 |
|  |  | 0.56 |
| CA_n1-n3-n78 | n1 | 0.2 |
|  | n3 | 0.2 |
|  | n78 | 0.5 |
| CA_n1-n3-n77 | n1 | 0.2 |
|  | n3 | 0.2 |
|  | n77 | 0.5 |
| CA_n1-n3-n79 | n1 | 0 |
|  | n3 | 0 |
|  | n79 | 0.5 |
| CA_n1-n5-n7 | n1 | 0 |
|  | n5 | 0 |
|  | n7 | 0 |
| CA_n1-n5-n28 | n5 | 0.2 |
|  | n28 | 0.2 |
| CA_n1-n5-n78 | n1 | 0.2 |
|  | n5 | 0.2 |
|  | n78 | 0.5 |
| CA_n1-n7-n8 | n8 | 0.2 |
| CA_n1-n7-n28 | n28 | 0.2 |
| CA_n1-n7-n78 | n1 | 0.2 |
|  | n7 | 0.2 |
|  | n78 | 0.5 |
| CA_n1-n8-n28 | n8 | 0.2 |
|  | n28 | 0.2 |
| CA_n1-n8-n77 | n1 | 0 |
|  | n8 | 0.2 |
|  | n77 | 0.5 |
| CA_n1-n8-n78 | n8 | 0.2 |
|  | n78 | 0.5 |
| CA_n1-n8-n79 | n8 | 0.2 |
|  | n79 | 0.5 |
| CA_n1-n18-n28 | n1 | 0 |
|  | n18 | 0 |
|  | n28 | 0 |
| CA_n1-n18-n41 | n1 | 0 |
|  | n18 | 0 |
|  | n41 | 0 |
| CA_n1-n18-n77 | n1 | 0 |
|  | n18 | 0 |
|  | n77 | 0.5 |
| CA_n1-n20-n67 | n1 | 0 |
|  | n20 | 0.2 |
|  | n67 | 0.2 |
| CA_n1-n20-n78 | n1 | 0 |
|  | n20 | 0 |
|  | n78 | 0.5 |
| CA_n1-n28-n40 | n1 | 0 |
|  | n28 | 0.2 |
|  | n40 | 0 |
| CA_n1-n28-n41 | n1 | 0 |
|  | n28 | 0.2 |
|  | n41 | 0 |
| CA_n1-n28-n77 | n1 | 0.2 |
|  | n28 | 0.2 |
|  | n77 | 0.5 |
| CA_n1-n28-n78 | n28 | 0.2 |
|  | n78 | 0.5 |
| CA_n1-n40-n78 | n78 | 0.5 |
| CA_n1-n41-n77 | n1 | 0 |
|  | n41 | 0 |
|  | n77 | 0.5 |
| CA_n1-n77-n79 | n1 | 0.2 |
|  | n77 | 0.5 |
| CA_n1-n78-n79 | n78 | 0.5 |
| CA_n2-n5-n30 | n2 | 0.4 |
|  | n5 | 0 |
|  | n30 | 0.5 |
| CA_n2-n5-n48 | n2 | 0.2 |
|  | n5 | 0..0 |
|  | n48 | 0.5 |
| CA_n2-n5-n66 | n2 | 0.3 |
|  | n5 | 0 |
|  | n66 | 0.3 |
| CA_n2-n5-n77 | n2 | 0.2 |
|  | n5 | 0.5 |
|  | n77 | 0.5 |
| CA_n2-n12-n30 | n2 | 0.4 |
|  | n12 | 0 |
|  | n30 | 0.5 |
| CA_n2-n12-n66 | n2 | 0.3 |
|  | n12 | 0.5 |
|  | n66 | 0.3 |
| CA_n2-n12-n77 | n2 | 0.2 |
|  | n12 | 0.2 |
|  | n77 | 0.5 |
| CA_n2-n14-n30 | n2 | 0.3 |
|  | n14 | 0 |
|  | n30 | 0.3 |
| CA_n2-n14-n66 | n2 | 0.3 |
|  | n14 | 0 |
|  | n66 | 0.3 |
| CA_n2-n14-n77 | n2 | 0.2 |
|  | n14 | 0.2 |
|  | n77 | 0.5 |
| CA_n2-n29-n30 | n2 | 0.3 |
|  | n29 | 0 |
|  | n30 | 0.3 |
| CA_n2-n29-n66 | n2 | 0.3 |
|  | n29 | 0 |
|  | n66 | 0.3 |
| CA_n2-n29-n77 | n2 | 0.2 |
|  | n29 | 0.2 |
|  | n77 | 0.5 |
| CA_n2-n30-n66 | n2 | 0.4 |
|  | n30 | 0.5 |
|  | n66 | 0.4 |
| CA_n2-n30-n77 | n2 | 0.2 |
|  | n30 | 0 |
|  | n77 | 0.5 |
| CA_n2-n48-n66 | n2 | 0.3 |
|  | n48 | 0.5 |
|  | n66 | 0.3 |
| CA_n2-n48-n77 | n2 | 0.2 |
|  | n48 | 0.5 |
|  | n77 | 0.5 |
| CA_n2-n66-n77 | n2 | 0.2 |
|  | n66 | 0.2 |
|  | n77 | 0.5 |
| CA_n2-n66-n77 | n2 | 0.2 |
|  | n66 | 0.2 |
|  | n77 | 0.5 |
| CA_n2-n66-n78 | n2 | 0.3 |
|  | n66 | 0.3 |
|  | n78 | 0.5 |
| CA_n3-n5-n28 | n5 | 0.2 |
|  | n28 | 0.1 |
| CA_n3-n7-n8 | n8 | 0.2 |
| CA_n3-n7-n78 | n3 | 0.2 |
|  | n7 | 0.2 |
|  | n78 | 0.5 |
| CA_n3-n8-n28 | n8 | 0.2 |
|  | n28 | 0.1 |
| CA_n3-n8-n77 | n3 | 0.2 |
|  | n8 | 0.2 |
|  | n77 | 0.5 |
| CA_n3-n5-n78 | n3 | 0.2 |
|  | n5 | 0.2 |
|  | n78 | 0.5 |
| CA_n3-n8-n78 | n3 | 0.2 |
|  | n8 | 0.2 |
|  | n78 | 0.5 |
| CA_n3-n18-n28 | n3 | 0 |
|  | n18 | 0 |
|  | n28 | 0 |
| CA_n3-n18-n41 | n41 | 01/0.52 |
| CA_n3-n18-n77 | n3 | 0.2 |
|  | n18 | 0 |
|  | n77 | 0.5 |
| CA_n3-n20-n67 | n3 | 0 |
|  | n20 | 0.1 |
|  | n67 | 0.1 |
| CA_n3-n20-n78 | n3 | 0.2 |
|  | n20 | 0 |
|  | n78 | 0.5 |
| CA_n3-n28-n41 | n41 | 01/0.52 |
| CA_n3-n28-n77 | n3 | 0.2 |
|  | n28 | 0.2 |
|  | n77 | 0.5 |
| CA_n3-n28-n78 | n28 | 0.2 |
|  | n78 | 0.5 |
| CA_n3-n28-n79 | n3 | 0 |
|  | n28 | 0.2 |
|  | n79 | 0.5 |
| CA_n3-n77-n79 | n3 | 0.2 |
|  | n77 | 0.5 |
|  | n79 | 0 |
| CA_n3-n40-n41 | n41 | 01,3 |
|  |  | 0.52,3 |
| CA_n3-n41-n77 | n3 | 0.2 |
|  | n41 | 01/0.52 |
|  | n77 | 0.5 |
| CA_n3-n41-n78 | n3 | 0.2 |
|  | n41 | 01/0.52 |
|  | n78 | 0.5 |
| CA_n3-n41-n79 | n41 | 0.5 |
|  | n79 | 0.5 |
| CA_n5-n7-n28 | n28 | 0.2 |
| CA_n5-n7-n78 | n5 | 0.2 |
|  | n7 | 0.2 |
|  | n78 | 0.5 |
| CA_n5-n12-n77 | n5 | 0.5 |
|  | n12 | 0.3 |
|  | n77 | 0.5 |
| CA_n5-n14-n77 | n5 | 0.2 |
|  | n14 | 0.2 |
|  | n77 | 0.5 |
| CA_n5-n25-n77 | n5 | 0.2 |
|  | n25 | 0.2 |
|  | n77 | 0.5 |
| CA_n5-n25-n78 | n5 | 0.2 |
|  | n25 | 0.2 |
|  | n78 | 0.5 |
| CA_n5-n29-n77 | n5 | 0.5 |
|  | n29 | 0.3 |
|  | n77 | 0.5 |
| CA_n5-n30-n66 | n5 | 0 |
|  | n30 | 0.5 |
|  | n66 | 0.4 |
| CA_n5-n30-n77 | n5 | 0.2 |
|  | n30 | 0 |
|  | n77 | 0.5 |
| CA_n5-n48-n66 | n5 | 0 |
|  | n48 | 0.5 |
|  | n66 | 0.2 |
| CA_n5-n48-n77 | n5 | 0.2 |
|  | n48 | 0.5 |
|  | n77 | 0.5 |
| CA_n5-n66-n77 | n5 | 0.2 |
|  | n66 | 0.2 |
|  | n77 | 0.5 |
| CA_n5-n66-n78 | n5 | 0.5 |
|  | n66 | 0.2 |
|  | n78 | 0.5 |
| CA_n7-n8-n28 | n8 | 0.2 |
|  | n28 | 0.1 |
| CA_n7-n8-n78 | n8 | 0.2 |
|  | n78 | 0.5 |
| CA_n7-n25-n66 | n7 | 0.5 |
|  | n25 | 0.3 |
|  | n66 | 0.5 |
| CA_n7-n25-n77 | n7 | 0.5 |
|  | n25 | 0.2 |
|  | n77 | 0.5 |
| CA_n7-n25-n78 | n7 | 0.5 |
|  | n25 | 0.2 |
|  | n78 | 0.5 |
| CA_n7-n28-n78 | n78 | 0.5 |
| CA_n7-n46-n78 | n7 | 0.5 |
|  | n46 | 0 |
|  | n78 | 0.5 |
| CA_n7-n66-n77 | n7 | 0.5 |
|  | n66 | 0.5 |
|  | n77 | 0.5 |
| CA_n7-n66-n78 | n7 | 0.5 |
|  | n66 | 0.5 |
|  | n78 | 0.5 |
| CA_n8-n28-n78 | n8 | 0.2 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |
| CA_n8-n39-n41 | n39 | 0.24 |
|  | n41 | 0.24 |
| CA_n8-n41-n79 | n41 | 0.5 |
|  | n79 | 0.5 |
| CA_n8-n78-n79 | n8 | 0.2 |
|  | n78 | 0.5 |
|  | n79 | 0.5 |
| CA_n12-n30-n66 | n12 | 0.5 |
|  | n30 | 0.5 |
|  | n66 | 0.4 |
| CA_n12-n30-n77 | n12 | 0.2 |
|  | n30 | 0 |
|  | n77 | 0.5 |
| CA_n12-n66-n77 | n12 | 0.5 |
|  | n66 | 0.5 |
|  | n77 | 0.5 |
| CA_n13-n25-n66 | n25 | 0.3 |
|  | n66 | 0.3 |
| CA_n13-n25-n77 | n13 | 0 |
|  | n25 | 0.2 |
|  | n77 | 0.5 |
| CA_n13-n66-n77 | n13 | 0.3 |
|  | n66 | 0.3 |
|  | n77 | 0.5 |
| CA_n14-n30-n66 | n14 | 0 |
|  | n30 | 0.5 |
|  | n66 | 0.4 |
| CA_n14-n30-n77 | n14 | 0.2 |
|  | n30 | 0 |
|  | n77 | 0.5 |
| CA_n14-n66-n77 | n14 | 0.2 |
|  | n66 | 0.5 |
|  | n77 | 0.5 |
| CA_n18-n28-n41 | n18 | 0 |
|  | n28 | 0 |
|  | n41 | 0 |
| CA_n18-n28-n77 | n18 | 0 |
|  | n28 | 0 |
|  | n77 | 0.5 |
| CA_n18-n41-n77 | n18 | 0 |
|  | n41 | 0 |
|  | n77 | 0.5 |
| CA_n20-n28-n78 | n28 | 0.2 |
|  | n78 | 0.5 |
| CA_n24-n41-n48 | n24 | 0.0 |
|  | n41 | 0.0 |
|  | n48 | 0.5 |
| CA_n24-n41-n77 | n24 | 0.2 |
|  | n41 | 0.0 |
|  | n77 | 0.5 |
| CA_n24-n48-n77 | n24 | 0.2 |
|  | n48 | 0.5 |
|  | n77 | 0.5 |
| CA_n25-n29-n66 | n25 | 0.3 |
|  | n66 | 0.3 |
| CA_n25-n38-n78 | n25 | 0.2 |
|  | n38 | 0.4 |
|  | n78 | 0.5 |
| CA_n25-n41-n66 | n25 | 0.3 |
|  | n41 | 0.55 |
|  |  | 16 |
|  | n66 | 0.3 |
| CA_n25-n41-n71 | n71 | 0.2 |
| CA_n25-n41-n78 | n25 | 0.2 |
|  | n41 | 0.5 |
|  | n78 | 0.5 |
| CA_n25-n48-n66 | n25 | 0.3 |
|  | n48 | 0.5 |
|  | n66 | 0.3 |
| CA_n25-n66-n71 | n25 | 0.3 |
|  | n66 | 0.3 |
|  | n71 | 0.3 |
| CA_n25-n66-n78 | n25 | 0.3 |
|  | n66 | 0.3 |
|  | n78 | 0.5 |
| CA_n25-n66-n77 | n25 | 0.3 |
|  | n66 | 0.3 |
|  | n77 | 0.5 |
| CA_n25-n71-n77 | n25 | 0.2 |
|  | n71 | 0.2 |
|  | n77 | 0.5 |
| CA_n25-n71-n78 | n25 | 0.2 |
|  | n71 | 0.3 |
|  | n78 | 0.5 |
| CA_n26-n66-n70 | n26 | 0 |
|  | n66 | 0 |
|  | n70 | 0 |
| CA_n28-n40-n41 | n28 | 0 |
|  | n40 | 0 |
|  | n41 | 0 |
| CA_n28-n40-n78 | n78 | 0.5 |
| CA_n28-n40-n79 | n28 | 0.2 |
|  | n40 | 0 |
|  | n79 | 0.5 |
| CA_n28-n41-n77 | n28 | 0.2 |
|  | n77 | 0.5 |
| CA_n28-n41-n78 | n28 | 0.2 |
|  | n78 | 0.5 |
| CA_n28-n41-n79 | n28 | 0.2 |
|  | n41 | 0.5 |
|  | n79 | 0.5 |
| CA_n28-n46-n78 | n28 | 0.2 |
|  | n46 | 0 |
|  | n78 | 0.5 |
| CA_n28-n77-n79 | n28 | 0.2 |
|  | n77 | 0.5 |
| CA_n28-n78-n79 | n28 | 0.2 |
|  | n78 | 0.5 |
| CA_n29-n30-n66 | n29 | 0 |
|  | n30 | 0.5 |
|  | n66 | 0.4 |
| CA_n29-n30-n77 | n29 | 0.2 |
|  | n30 | 0 |
|  | n77 | 0.5 |
| CA_n29-n66-n77 | n29 | 0.5 |
|  | n66 | 0.5 |
|  | n77 | 0.5 |
| CA_n30-n66-n77 | n30 | 0.5 |
|  | n66 | 0.4 |
|  | n77 | 0.5 |
| CA_n39-n40-n79 | n39 | 0.3 |
|  | n40 | 0.3 |
|  | n79 | 0.5 |
| CA_n39-n41-n79 | n39 | 0.34 |
|  | n41 | 0.34 |
|  | n79 | 0.8 |
| CA_n40-n41-n79 | n40 | 08 |
|  | n41 | 0.58 |
|  | n79 | 0.5 |
| CA_n41-n66-n71 | n41 | 0.51 |
|  |  | 12 |
|  | n66 | 0.5 |
| CA_n41-n66-n77 | n41 | 0.2 |
|  | n66 | 0.2 |
|  | n77 | 0.5 |
| CA_n41-n66-n78 | n41 | 0.2 |
|  | n66 | 0.2 |
|  | n78 | 0.5 |
| CA_n41-n70-n78 | n41 | 0.2 |
|  | n70 | 0.2 |
|  | n78 | 0.5 |
| CA_n41-n71-n77 | n71 | 0.2 |
|  | n77 | 0.5 |
| CA_n41-n71-n78 | n41 | 0 |
|  | n71 | 0.2 |
|  | n78 | 0.5 |
| CA_n46-n48-n96 | n46 | 0.5 |
|  | n48 | 0.5 |
|  | n96 | 0.6 |
| CA_n48-n66-n70 | n48 | 0.5 |
|  | n66 | 0.2 |
|  | n70 | 0.2 |
| CA_n48-n66-n71 | n48 | 0.2 |
|  | n66 | 0.2 |
|  | n71 | 0.2 |
| CA_n48-n66-n77 | n48 | 0.5 |
|  | n66 | 0.2 |
|  | n77 | 0.5 |
| CA_n48-n70-n71 | n48 | 0.2 |
|  | n70 | 0.2 |
|  | n71 | 0.2 |
| CA_n66-n71-n77 | n66 | 0.2 |
|  | n71 | 0.2 |
|  | n77 | 0.5 |
| CA_n66-n71-n78 | n66 | 0.2 |
|  | n71 | 0.2 |
|  | n78 | 0.5 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. NOTE 3: Only applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx among band 40 and 41. NOTE 4: Applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx between n39 and n41. NOTE 5: The requirement is applied for UE transmitting on the frequency range of 2545 - 2690 MHz. NOTE 6: The requirement is applied for UE transmitting on the frequency range of 2496 - 2545 MHz. NOTE 7: Void. NOTE 8: Void. |  |  |

## << End of change 4>>