3GPP TSG-RAN WG4 Meeting #102e	R4-2206538

# Electronic Meeting, February 21 - March 3, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 1034 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on UE RF requirements for DMRS bundling in TS 38.101-1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_cov_enh-Core |  |  |  |  |  | Date: |  |  | 2022-02-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To support DMRS bundling for PUSCH and PUCCH in Rel-17 coverage enhancements WI, phase continuity and power consistency need to be maintained with certain tolerances. For power consistency, the existing requirement for aggregate power tolerance applies and no dedicated new requirements are to be introduced. For phase continuity, new requirements should be introduced. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce the phase continuity requirements for DMRS bundling for PUSCH and PUCCH. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | DMRS bundling for PUSCH and PUCCH cannot be supported in Rel-17. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New sub-clauses: 6.4.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< unchanged text omitted >

6.4.2.5	Phase continuity requirements for DMRS bundling

For bands that UE indicates the support of DMRS bundling, the maximum allowable difference between the measured phase value in any slot p-1 and slot p, or slot 0 and any slot p for each antenna connector shall satisfy the requirements as listed in Table 6.4.2.5-1 for the measurement conditions defined in Table 6.4.2.5-2, within a measurement time window limited by the UE capability of maximum duration for DMRS bundling [maxDMRS-BundlingDuration], and defined for each frequency band separately. The phase value for each slot is measured as shown in Annex F.9. These requirements apply to PUCCH and PUSCH transmissions with DFT-s-OFDM and CP-OFDM waveforms.

Table 6.4.2.5-1: Maximum allowable phase difference for DMRS bundling

| UL channel | Modulation order | Phase difference between any slot p-1 and slot p (NOTE 2) | Phase difference between slot 0 and any slot p (NOTE 3) |
| --- | --- | --- | --- |
| PUSCH | Pi/2 BPSK, QPSK | [25] degrees | [30] degrees |
| PUCCH | Pi/2 BPSK, BPSK, QPSK |  |  |
| NOTE 1: The UE capability of the length of maximum duration refers to the maximum time duration during which UE is able to meet the phase continuity requirements, assuming no phase consistency violating events defined in TS 38.214 in between. NOTE 2: This requirement applies for FDD and TDD bands, for supported DMRS bundling configurations ≤ 8 slots. NOTE 3: This requirement applies only for FDD bands, for supported DMRS bundling configurations of 16 slots. |  |  |  |

The above requirements are applicable when all the following conditions are met within the measurement time window:

-	RB allocation in terms of length and frequency position does not change, and intra-slot and inter-slot frequency hopping is not activated.

-	Modulation order does not change.

-	No network commanded TA takes effect.

-	The TPMI precoder does not change.

-	There is no change in UE transmission power level, and no change in the level of P-MPR applied by the UE.

-	UE is not scheduled with uplink transmission of other physical channel/signal in-between the PUSCH or PUCCH transmissions.

-	For TDD, no downlink slot(s) or downlink symbol(s) or flexible symbol(s) with/without DL monitoring occasion configured in-between the PUSCH or PUCCH transmissions.

Table 6.4.2.5-2: Measurement conditions for the maximum allowable phase difference

| Parameter | Unit | Level |
| --- | --- | --- |
| UE Output Power | dBm | PCMAX,f,c in clause 6.2.4, P-MPR = 0 |
| UE downlink received power |  | Not change |
| Operating conditions |  | Normal conditions |
| Transmission bandwidth |  | Confined within FUL_low + [4] MHz and FUL_high – [4] MHz |
| DL signal frequency |  | Not change before and during the measurement window |
| DL signal timing |  | Maintained constant before and during the measurement window |
| UL slots for testing |  | Tested on consecutive UL slots |

< unchanged text omitted >