3GPP RAN TSG Meeting #95e		RP-220451

Electronic meeting, March 17 - 23, 2022

## Status Report to TSG

Agenda item:			9.2.5

| WI / SI Name | Study on enhanced test methods for FR2 NR UEs |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: Yes | Core part: No | Performance part: No |  | Testing part: No |
| Acronym | FS_FR2_enhTestMethods |  |  |  |  |
| Unique ID | 850071 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-211600 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: Mar / 2022 | Core part: | Performance part: | Testing part: |  |
| Overall Completion level | Study Item: 90% | Core part: | Performance Part: | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | TSG RAN WG4 |
| --- | --- | --- |
| Rapporteur | Name | Anatoliy IOFFE, Ruixin WANG, Aida Vera LOPEZ |
|  | Company | Apple Inc., vivo, Intel Corporation |
|  | Email | aioffe@apple.com, ruixin.wang@vivo.com, aida.l.vera.lopez@intel.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | Yes |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

One additional quarter is requested to conclude the remaining open issues.

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.4	RAN4

#### 2.4.1	Agreements

RAN4 #101-bis-e (January 2022, Electronic)

General

-	Summary of the electronic meeting email discussions was provided in [11]

-	A text proposal on on release independence applicability of test method enhancements was approved in [1]

-	Version 1.3.0 of TR38.884 was approved in [12] during the email approval period

Objective 1 (test methodology for high DL power and low UL power test cases)

-	N/A

Objective 2 (solutions to minimize the impact of polarization basis mismatch)

-	N/A

Objective 3 (enhancements to support the verification of RF requirements for inter-band FR2+FR2 CA)

-	N/A

Objective 4 (enhancements to support extreme temperature conditions for all applicable FR2 UE RF test cases)

-	N/A

Objective 5 (enhancements to reduce test time)

-	N/A

Objective 6 (testability aspects for the introduction of the new band n262)

-	N/A

Objective 7 (study and define test methods for FR2-2)

-	A text proposal on the extension of NR test methods to FR2-2 was approved in [9]

-	The following agreements were captured in the agreed Way Forward [10]

-	Vehicular UE

-	Avoid developing a standardized ground plane for FR2 vehicular UEs as part of this SI.

-	Consider the optional ground plane designed and manufactured by the OEM an integral part of the FR2 vehicular UE DUT submitted for conformance testing.

-	Consider battery and AC/DC powered operation acceptable for FR2 vehicular UEs and request manufacturers to provide proper guidance on cable routings.

-	Handheld UE

-	The worst-case antenna assumption for testability and MU assessment of handheld UEs in FR2-2 is [8 x2]. Single UE antenna element pattern parameters, similar to Table 5.2.3.3-1, need to be finalized in RAN4#102-e clear conclusions.

-	Test methodology for UE RF

-	Unless otherwise stated, FR2-2 will follow the baseline UE RF methodology detailed in TR 38.810. MU assessment will be revised to reflect proper frequency-dependent parameters and worst-case array size.

-	Enhanced test methodology

-	Applicability of methodology enhancements of three methods in Objective 5 can be extended to FR2-2.

-	Objective 3 discussions should be postponed until core requirements are discussed.

-	Further discuss the remaining objectives

-	Extend applicability of Objective 2 and Objective 5 solutions to FR2-2

-	Test methodology for RRM

-	Reusing FR2-1 RRM test methodology

-	All test methods and measurement setup for FR2 RRM methodology defined in TR 38.810 Clause 6 and TS 38.508-1 are applicable for FR2-2, except for Noc derivation (TR 38.810 - Clause 6.2.1.4.3) and Maximum SNR derivation (TR 38.810 - Annex B.2)

-	Noc level derivation

-	For FR2-2 RRM testing the Noc level is derived similar to the Noc level for UE demodulation test methods

-	Assessment of testable RRM DL SNR range

-	Perform an informative assessment of testable RRM DL SNR range for FR2-2 for maximum frequency (~71GHz) using TR38.810 methodology as starting point

-	Test methodology for UE demodulation and CSI

-	Multi-path fading channel model

-	Consider TDL-A channel model with RMS delay spread as in range of 5-20ns and with 3km/h UE mobility as starting point for the analysis

-	Path delay grid

-	Max applicable channel bandwidth:

-	Option 1: 2GHz

-	Option 2: Smaller than 2GHz

-	Sampling frequency:

-	Option 1: 2GHz

-	Option 2: 800/400MHz

-	Noc level derivation

-	Define Noc levels for FR2-2 UE demodulation testing only based on the following definition:

Noc (PC_X, Band_Y) = RESFENSPCX, BandY -10log10(SCSREFSENS x PRBREFSENS x 12) - SNRREFSENS + ∆thermal

-	Consider parameters in table below for Noc level definition

| Parameter | Clarification/Value |
| --- | --- |
| REFSENSPCX, BandY | The REFSENS value in dBm specified for Power Class X UE in band Y |
| CBW for REFSENS | 100 MHz |
| SCSREFSENS | 120 kHz |
| PRBREFSENS | NRB associated with subcarrier spacing SCSREFSENS for channel bandwidth used for REFSENS calcualtion |
| SNRREFSENS | -1 dB |
| ∆thermal | An amount of dB that the wanted noise is set above UE thermal noise, giving a rise in total noise of ∆BB. ∆thermal = 6dB, giving a rise in total noise of 1dB |

-	FFS on ∆thermal value revision to cope with potential low max DL SNR

-	Assessment of testable Demodulation DL SNR range

-	Prioritize IFF method for the testable DL SNR assessment

-	Assume backoff from 1dB compression as [] following the latest conclusion in RAN5 for FR2-1 as starting point value for further assessment

-	Assume reference range length as [0.725m] for IFF method.

-	Provide the following remaining test system parameters next meeting: TE amplifier 1dB compression, cable loss, free space path loss, TE DL absolute power setting uncertainty, probe antenna gain.

-	Add the parameter "Connector insertion loss," to the TR 38.810 DL SNR assessment methodology for FR2-2 assessment with the value to be further checked by the next meeting

RAN4 #102-e (February 2022, Electronic)

General

-	Summary of the electronic meeting email discussions was provided in [23]

-	Version 1.4.0 of TR38.884 was approved in [27] during the email approval period

Objective 1 (test methodology for high DL power and low UL power test cases)

-	N/A

Objective 2 (solutions to minimize the impact of polarization basis mismatch)

-	The following agreements were captured in the agreed Way Forward [22]:

-	MIMO EVM measurement

-	For two-layer uplink MIMO in FR2, define the zero-forcing receiver as the inverse of the effective channel matrix so that 
GZF=H-1

-	Agree to the text proposal for Section 5.2.3.1.1.2 of TR38.884-130 in the Appendix

-	Include relevant text proposal content in R4-2207202

Objective 3 (enhancements to support the verification of RF requirements for inter-band FR2+FR2 CA)

-	N/A

Objective 4 (enhancements to support extreme temperature conditions for all applicable FR2 UE RF test cases)

-	N/A

Objective 5 (enhancements to reduce test time)

-	N/A

Objective 6 (testability aspects for the introduction of the new band n262)

-	N/A

Objective 7 (study and define test methods for FR2-2)

-	A text proposal on NR test methods extension to FR2-2 was approved in [23]

-	A text proposal on minimum SNR for RRM test cases for band n263 was approved in [24]

-	A text proposal on minimum SNR for demodulation test cases for band n263 was approved in [25]

-	The following agreements were captured in the agreed Way Forward [22]:

-	Remaining work of SI

-	RAN4 should conclude the preliminary MU assessments for UE RF, RRM, and demodulation based on the agreed assumption on the number of UE antenna elements.

-	RAN4 should conclude the max achievable SNR for demodulation and for RRM in the beam peak direction.

-	RAN4 should conclude the applicability of Objectives 1-5 to FR2-2 based on the agreements below and capture the related agreements in TR38.884.

-	As a starting point, the same High DL power and low UL power test cases for which NF based solutions (i.e. CFFNF, CFFDNF, and CFFdeltaNF) are applicable in FR2-1, can be considered for NF based solutions applicability in FR2-2. In case relaxations are needed for IFF/DFF methods for a given test case, it is up to RAN5 to confirm applicability of NF based solutions

-	At least, RSRPB based Rx beam peak search, Single link polarization measurement and Fast Spherical Coverage Method can be applied to 52.6-71GHz directly

-	General aspects

-	Testing and calibration

-	RAN4 should confirm if the testing and calibration aspects detailed in Clause 5.2.1.3 of TR 38.810 can be extended to FR2-2

-	Testing and calibration aspects can be extended to FR2-2. Relevant text is already included in R4-2207202

-	Radiating aperture

-	Reuse D = 5cm for PC3

-	Additional information can be included into TR for the background if needed

-	UE types

-	Single-element antenna assumption

-	Single UE antenna element pattern parameters can be reused as Table G.1.1-1 in TR38.810, with below exceptions:

-	half-power beamwidth: [80º/60º] as starting point

-	Gain: [5dBi]

-	Worst-case antenna array configuration (MxN) for PC1 and PC2

-	PC1: [144(12*12)]

-	PC2: [40 (10*4)]

-	Beam steering assumptions of PC3, PC1 and PC2

-	Reuse the beam steering assumptions in TR38.810 for PC3

-	Test methodology for RRM

-	Perform informative assessment of testable RRM DL SNR range for FR2-2 for the first and second scenario of RRM requirements and for both types of RRM requirements

-	Assume [7] dB gain between fine and rough beams for FR2-2

-	Above assumption has no impact on RRM core requirements discussion

-	Test methodology for Demodulation and CSI

-	Further work on the TP based on the following ways to change test methodology

-	Acceptable ∆thermal decrease

-	Option 1: 1 dB (similar to FR2-1)

-	Option 2: 2 dB

-	Other options are not precluded

-	Adjustment of TE

-	Acceptable restriction of allocation size within CBW

-	Note: ∆thermal decrease, and allocation size within CBW are included in the side condition for demodulation and CSI requirements.

-	Path delay grid

-	Limit maximum applicable CBW and minimum sampling frequency by [400MHz]

#### 2.4.2	Remaining Open issues

-	Objective 1 (test methodology for high DL power and low UL power test cases)

-	None

-	Objective 2 (solutions to minimize the impact of polarization basis mismatch)

-	None

-	Objective 3 (enhancements to support the verification of RF requirements for inter-band FR2+FR2 CA)

-	None

-	Objective 4 (extreme temperature conditions for all applicable FR2 UE RF test cases)

-	None

-	Objective 5 (testability enhancements to reduce test time)

-	None

-	Objective 6 (testability aspects for the introduction of the new band n262)

-	None

-	Objective 7 (testability aspects for the extension to FR2-2)

-	Conclude the measurement grid analysis and the related uncertainty contribution

-	Conclude testable SNR and path delay grid for the demodulation methods

-	Capture the preliminary MU assessment for the extension of RF, RRM, and demodulation permitted methods in TR38.810 to FR2-2

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

N/A

## 4.	References

RAN4 #101-bis-e (January 2022, Electronic)

- R4-2200452	TP to TR 38.884 on release independence applicability of test method enhancements	Apple

- R4-2200907	On permitted test methods for demodulation in FR2-2	Apple

- R4-2201873	TP to TR 38.884 on extension of NR test methods to FR2-2	Intel Corporation

- R4-2201874	NR FR2-2 OTA test methods for RRM	Intel Corporation

- R4-2201875	NR FR2-2 OTA test methods for UE demodulation	Intel Corporation

- R4-2201921	On Vehicular UEs	Keysight Technologies UK Ltd

- R4-2201927	UE types and permitted methods for FR2-2	Intel Corporation

- R4-2201990	Issues with MIMO EVM Measurement Using the Pseudo-Inverse	Lenovo, Motorola Mobility

- R4-2203078	TP to TR 38.884 on extension of NR test methods to FR2-2	Intel Corporation

- R4-2203079	WF on OTA test methods for FR2-2	Intel Corporation

- R4-2203120	Email discussion summary for [101-bis-e][327] FR2_enhTestMethods	Moderator (Intel)

- R4-2203128	Draft TR 38.884 v1.3.0: Study on enhanced test methods for FR2 NR Ues	Apple, vivo, Intel

RAN4 #102-e (February 2022, Electronic)

- R4-2203636	On FR2-2 Antenna Assumptions	Keysight Technologies UK Ltd

- R4-2203706	Proposals to conclude the enhanced test methods study item	Apple

- R4-2204386	FR2-2 OTA test methods for UE demodulation	Intel Corporation

- R4-2204964	TP to TR38.884 on applicability extension of test methods for band FR2-2	vivo

- R4-2204965	Discussion on test methods for FR2-2	vivo

- R4-2205007	Discussion on FR2-2 OTA test methods	Huawei,HiSilicon

- R4-2205915	FR2-2 OTA test methods for UE RRM	Intel Corporation

- R4-2206091	On general aspects and UE testing methodology for FR2-2	Intel Corporation

- R4-2206116	MIMO EVM Measurement for FR2	Lenovo

- R4-2207201	WF on FR2-2 OTA test methods	Intel Corporation

- R4-2207202	TP for TR 38.884 on NR test methods extension to FR2-2	Intel Corporation

- R4-2207203	TP to TR38.884 on minimum SNR for RRM test cases for band n263	Apple

- R4-2207459	TP to TR38.884 on minimum SNR for demodulation test cases for band n263	Apple

- R4-2207452	Email discussion summary for [102-e][337] FR2_enhTestMethods_Part1	Moderator (Intel)

- R4-2207512	Draft TR 38.884 v1.4.0: Study on enhanced test methods for FR2 NR Ues	Apple, vivo, Intel

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template