3GPP TSG-RAN Meeting #95e		RP-220465
Electronic meeting, March 17 – 23, 2022

# Title:	draft LS to ISED-Canada to clarify the RSS-195 requirement related to wireless communication service equipment operating in 2305-2320 MHz / 2345-2360MHz frequency range

# Response to:

Source:	Apple (TSG-RAN)

To:	ISED-Canada

Cc:	3GPP RAN4

Contact Person:

Name:	Anatoliy Ioffe

E-mail Address:	aioffe(at)apple(dot)com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None

1. Overall Description:

TSG RAN would like to inform ISED-Canada of the current understanding in 3GPP of the RSS-195 regulatory requirements related to Wireless Communication Service (WCS) equipment operating in the Bands 2305-2320 MHz and 2345-2360 MHz and the implementation of these requirements in the 3GPP specifications.

Band n30 (2305 - 2315 MHz UL / 2350 - 2360 MHz DL) was introduced to the NR specification in Rel-16, with A-MPR and NS_21 specified according to the US regulatory FCC 27.53 requirements.  After analyzing both sets of regulatory requirements, 3GPP has arrived at the following conclusion:

-	FCC 27.53 requirements state that a narrower resolution bandwidth (RBW) is permissible if RBW is >= 1% of the channel BW. Consequently, the specification can be -13dBm / (1% channel BW) for 0-1MHz from channel edge. Only if RBW < 1% of channel BW, the power is integrated over 1MHz.

-	RSS-195 requirements state that RBW must be >= 1% of the channel BW (cannot be < 1%), and power must be integrated over 1MHz. Consequently, the specification is -13dBm/1MHz for 0-1MHz from the channel edge.

The figure below illustrates this comparison in emission limits:

TSG RAN would like to kindly invite ISED-Canada to review this understanding of the RSS-195 requirements and, if necessary, to provide any additional guidance or clarifications.

In the meanwhile, TSG RAN will introduce a new network signalling NS_61 with the associated requirements and A-MPR for Canadian WCS for band n30.

2. Actions:

TSG RAN asks ISED-Canada to take the above information into account in their future work.

3. Date of Next TSG-RAN Meetings:

TSG-RAN Meeting #96		6. – 9. June 2022	Budapest, TBC

TSG-RAN Meeting #97		12. – 15. September 2022	USA, TBC