3GPP TSG-RANP Meeting #95-e	RP-220471

Online, March 17~23, 2022

Agenda item:	9.3.3.1

Source:	Qualcomm Incorporated

Title:	Clarification to SON/MDT WID

Document for:	Discussion and Decision

# Introduction

In this contribution, we propose to clarify the objective of the SON/MDT work item description [1] related to MDT inter-RAT mobility.

# Discussion

In RAN#94-e meeting, a new WID for SON/MDT was agreed [1] which included the following objective:

- Support of signaling based logged MDT override protection to address the scenario where the signaling based MDT configured in LTE, the UE comes to connected in NR [RAN2, RAN3]

We believe that this objective would benefit from a sharper definition, both of the scenario, as well as of the actual objective. For example:

- There are at least two different scenarios related to logged MDT handling (i.e., depending on whether the timer T330 has expired or not), and it is not clear whether either or both are covered

- In either case, the scenario description should be specific regarding the fact that the logged MDT file configured in LTE has not been uploaded

- It may be beneficial also to avoid the use of the expression “override protection” which can be open to interpretation – instead it may be better to spell out that the objective is to minimize or avoid the loss of logged data

A possible re-write of the objective with this in mind could be as below:

- For signalling based logged MDT, specify mechanisms to minimize loss of logged data in the following scenarios [RAN2, RAN3]: - UE reselects to NR while T330 is running (after logged MDT configuration in LTE) - UE reselects to NR after T330 expires and before uploading the logged MDT file.

We therefore recommend the following:

Proposal: 	Clarify objective related to inter-RAT logged MDT handling in [1], by replacing it with a formulation as provided in this document, or similar.

# Conclusion

Based on the above discussion, we recommend that RAN adopts the following proposal:

Proposal: 	Clarify objective related to inter-RAT logged MDT handling in [1], by replacing it with a formulation as provided in this document, or similar.

# References

- RP-213553, New WID on further enhancement of data collection for SON (Self-Organising Networks)/MDT (Minimization of Drive Tests) in NR and EN-DC, Dec 2021.