3GPP TSG-RAN Meeting #95-e	RP-220517

Online, 17 – 23 Mar, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.443 | CR | 0131 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of new bands and bandwidth allocation for LTE-based 5G terrestrial broadcast |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Qualcomm Inc. |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_terr_bcast_bands_part1-Core |  |  |  |  |  | Date: |  |  | 2022-03-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | RAN1 CRs for the WI were approved by RAN#94e in RP-212975. RAN2 CR for RRC parameter was agreed in R2-2203633. This CR adds the corresponding changes to M2AP specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Following changes are introduced: - Introduce signaling to configure the PMCH bandwidth of an MBSFN area |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | New bands and bandwidth allocation for LTE-based 5G terrestrial broadcast cannot be supported. RAN3 specification remains incomplete and misaligned with RAN1 and RAN2 specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.1.27, 9.2.1.x (new), 9.3.5, 9.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Change

#### 9.2.1.27	MCCH related BCCH Extended Configuration Item

This information element provides MCCH related BCCH configuration information to the eNB in line with either the MBSFN-AreaInfo-r16 IE or the MBSFN-AreaInfo-r17 IE as defined in TS 36.331 [11].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| MBSFN Area Id | M |  | 9.2.1.14 |  | - | - |
| Repetition Period Expanded | M |  | ENUMERATED (rf1, rf2, rf4, rf8, rf16, rf32, rf64, rf128, rf256, …) | The same encoding as the mcch-RepetitionPeriod-r16 IE in the mcch-Config-r16 IE as specified in TS 36.331 [11]. | - |  |
| Offset | M |  | INTEGER (0..10) | The same encoding as the mcch-Offset-r16 IE in the mcch-Config-r16 IE as specified in TS 36.331 [11]. | - | - |
| Modification Period Expanded | M |  | ENUMERATED (rf1, rf2, rf4, rf8, rf16, rf32, rf64, rf128, rf256, rf512, rf1024, …) | The same encoding as the mcch-ModificationPeriod-r16 IE in the mcch-Config-r16 IE as specified in TS 36.331 [11]. | - | - |
| Subframe Allocation Info Expanded | M |  | BIT STRING (SIZE(10)) | The same encoding as the sf-AllocInfo-r16 IE in the mcch-Config-r16 IE as specified in TS 36.331 [11]. | - | - |
| Modulation and Coding Scheme | M |  | ENUMERATED (n2, n7, n13, n19) | The same encoding as the signallingMCS-r16 IE in the mcch-Config-r16 IE as specified in TS 36.331 [11]. | - | - |
| Subcarrier Spacing MBMS Expanded | M |  | ENUMERATED (khz-7dot5, khz-2dot5, khz-1dot25, khz-0dot37, …, kHz15) | The same encoding as the subcarrierSpacingMBMS-r16 IE in the mcch-Config-r16 IE as specified in TS 36.331 [11]. The value “kHz15” is not used unless the Additional Configuration Parameters IE is present. | - | - |
| Time separation | O |  | ENUMERATED (sl2, sl4, … ) | The same encoding as the timeSeparation-r16 IE in the mcch-Config-r16 IE as specified in TS 36.331 [11]. | - | - |
| Cell Information List |  | 0..1 |  |  |  |  |
| >Cell Information |  | 1 to <maxnoofCells> |  |  |  |  |
| >>E-UTRAN CGI | M |  | 9.2.1.11 |  | - | - |
| >>Cell Reservation Info | M |  | ENUMERATED (reservedCell, nonReservedCell, …) |  | - | - |
| Additional Configuration Parameters | O |  | 9.2.1.X | This IE is included for a configuration corresponding to the MBSFN-AreaInfoList-r17 as specified in TS 36.331 [11]. | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofCells | Maximum no. of cells that may be served by an eNB. The value for maxnoofCells is 256. |

#### 9.2.1.X	Additional Configuration Parameters

This information element provides additional MCCH related BCCH configuration information to the eNB in line with the MBSFN-AreaInfo-r17 IE as defined in TS 36.331 [11].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PMCH Bandwidth | M |  | ENUMERATED (n40, n35, n30, …) | The same encoding as the pmch-Bandwidth-r17 IE as specified in TS 36.331 [11]. |

Next Change

### 9.3.5	Information Element definitions

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

M2AP-Ies {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) m2ap (4) version1 (1) m2ap-Ies (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-MCH-Scheduling-PeriodExtended,

id-MCH-Scheduling-PeriodExtended2,

id-Modification-PeriodExtended,

id-Modulation-Coding-Scheme2,

id-Repetition-PeriodExtended,

id-Subcarrier-SpacingMBMS,

id-SubframeAllocationExtended,

id-SubframeAllocationFurtherExtension,

id-AdditionalConfigParameters,

maxnoofMBSFNareas,

maxnoofPMCHsperMBSFNarea,

maxnoofCells,

maxnoofMBMSServiceAreasPerCell,

maxnoofSessionsPerPMCH,

maxnooferrors,

maxnoofCellsforMBMS

FROM M2AP-Constants

Criticality,

ProcedureCode,

ProtocolIE-ID,

TriggeringMessage

FROM M2AP-CommonDataTypes

ProtocolExtensionContainer{},

ProtocolIE-Single-Container{},

M2AP-PROTOCOL-EXTENSION,

M2AP-PROTOCOL-IES

FROM M2AP-Containers;

#### -- A

AdditionalConfigParameters ::= SEQUENCE {

pmch-Bandwidth	PMCH-Bandwidth,

iE-Extensions				ProtocolExtensionContainer { {AdditionalConfigParameters-ExtIEs} } OPTIONAL

}

AdditionalConfigParameters-ExtIEs M2AP-PROTOCOL-EXTENSION ::= {

...

}

AllocatedSubframesEnd ::= INTEGER (0..1535)

AllocationAndRetentionPriority ::= SEQUENCE {

priorityLevel				PriorityLevel,

pre-emptionCapability		Pre-emptionCapability,

pre-emptionVulnerability	Pre-emptionVulnerability,

iE-Extensions				ProtocolExtensionContainer { {AllocationAndRetentionPriority-ExtIEs} } OPTIONAL

}

AllocationAndRetentionPriority-ExtIEs M2AP-PROTOCOL-EXTENSION ::= {

...

}

#### -- B

BitRate ::= INTEGER (0..10000000000)

#### -- C

Cause ::= CHOICE {

radioNetwork		CauseRadioNetwork,

transport			CauseTransport,

nAS					CauseNAS,

protocol			CauseProtocol,

misc				CauseMisc,

...

}

CauseMisc ::= ENUMERATED {

control-processing-overload,

hardware-failure,

om-intervention,

unspecified,

...

}

CauseNAS ::= ENUMERATED {

unspecified,

...

}

CauseProtocol ::= ENUMERATED {

transfer-syntax-error,

abstract-syntax-error-reject,

abstract-syntax-error-ignore-and-notify,

message-not-compatible-with-receiver-state,

semantic-error,

abstract-syntax-error-falsely-constructed-message,

unspecified,

...

}

CauseRadioNetwork ::= ENUMERATED {

unknown-or-already-allocated-MCE-MBMS-M2AP-ID,

unknown-or-already-allocated-eNB-MBMS-M2AP-ID,

unknown-or-inconsistent-pair-of-MBMS-M2AP-IDs,

radio-resources-not-available,

interaction-with-other-procedure,

unspecified,

...,

invalid-QoS-combination,

not-supported-QCI-value

}

CauseTransport ::= ENUMERATED {

transport-resource-unavailable,

unspecified,

...

}

Cell-Information ::= SEQUENCE {

eCGI							ECGI,

cellReservationInfo			ENUMERATED {reservedCell, nonReservedCell, ...},

iE-Extensions					ProtocolExtensionContainer { { Cell-Information-ExtIEs} } OPTIONAL,

...

}

Cell-Information-ExtIEs M2AP-PROTOCOL-EXTENSION ::= {

...

}

Cell-Information-List ::= SEQUENCE (SIZE(1..maxnoofCells)) OF Cell-Information

CriticalityDiagnostics ::= SEQUENCE {

procedureCode					ProcedureCode					OPTIONAL,

triggeringMessage				TriggeringMessage				OPTIONAL,

procedureCriticality			Criticality						OPTIONAL,

iEsCriticalityDiagnostics		CriticalityDiagnostics-IE-List	OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {CriticalityDiagnostics-ExtIEs} }	OPTIONAL,

...

}

CriticalityDiagnostics-ExtIEs M2AP-PROTOCOL-EXTENSION ::= {

...

}

CriticalityDiagnostics-IE-List ::= SEQUENCE (SIZE (1..maxnooferrors)) OF

SEQUENCE {

iECriticality			Criticality,

iE-ID					ProtocolIE-ID,

typeOfError				TypeOfError,

iE-Extensions			ProtocolExtensionContainer { {CriticalityDiagnostics-IE-List-ExtIEs} } OPTIONAL,

...

}

CriticalityDiagnostics-IE-List-ExtIEs M2AP-PROTOCOL-EXTENSION ::= {

...

}

#### -- D

#### -- E

ECGI ::= SEQUENCE {

pLMN-Identity				PLMN-Identity,

eUTRANcellIdentifier		EUTRANCellIdentifier,

iE-Extensions				ProtocolExtensionContainer { {ECGI-ExtIEs} } OPTIONAL,

...

}

ECGI-ExtIEs M2AP-PROTOCOL-EXTENSION ::= {

...

}

ENB-ID ::= CHOICE {

macro-eNB-ID	BIT STRING (SIZE (20)),

... ,

short-Macro-eNB-ID		BIT STRING (SIZE(18)),

long-Macro-eNB-ID		BIT STRING (SIZE(21))

}

ENB-MBMS-Configuration-data-Item ::= SEQUENCE {

eCGI							ECGI,

mbsfnSynchronisationArea		MBSFN-SynchronisationArea-ID,

mbmsServiceAreaList				MBMS-Service-Area-ID-List,

iE-Extensions					ProtocolExtensionContainer { { ENB-MBMS-Configuration-data-Item-ExtIEs} } OPTIONAL,

...

}

ENB-MBMS-Configuration-data-Item-ExtIEs M2AP-PROTOCOL-EXTENSION ::= {

...

}

ENB-MBMS-Configuration-data-ConfigUpdate-Item ::= CHOICE {

mBMSConfigData					ENB-MBMS-Configuration-data-Item,

eCGI							ECGI,

...

}

ENB-MBMS-M2AP-ID ::= INTEGER (0..65535)

ENBname ::= PrintableString (SIZE (1..150,...))

EUTRANCellIdentifier ::= BIT STRING (SIZE (28))

#### -- F

#### -- G

GBR-QosInformation ::= SEQUENCE {

mBMS-E-RAB-MaximumBitrateDL			BitRate,

mBMS-E-RAB-GuaranteedBitrateDL		BitRate,

iE-Extensions						ProtocolExtensionContainer { { GBR-QosInformation-ExtIEs} } OPTIONAL,

...

}

GBR-QosInformation-ExtIEs M2AP-PROTOCOL-EXTENSION ::= {

...

}

GlobalENB-ID ::= SEQUENCE {

pLMN-Identity			PLMN-Identity,

eNB-ID					ENB-ID,

iE-Extensions			ProtocolExtensionContainer { {GlobalENB-ID-ExtIEs} } OPTIONAL,

...

}

GlobalENB-ID-ExtIEs M2AP-PROTOCOL-EXTENSION ::= {

...

}

GlobalMCE-ID ::= SEQUENCE {

pLMN-Identity			PLMN-Identity,

mCE-ID					MCE-ID,

iE-Extensions			ProtocolExtensionContainer { {GlobalMCE-ID-ExtIEs} } OPTIONAL,

...

}

GlobalMCE-ID-ExtIEs M2AP-PROTOCOL-EXTENSION ::= {

...

}

GTP-TEID ::= OCTET STRING (SIZE (4))

#### -- H

#### -- I

IPAddress ::= OCTET STRING (SIZE(4..16))

#### -- J

#### -- K

#### -- L

LCID ::= INTEGER (0..28)

#### -- M

MBMS-Cell-List ::= SEQUENCE (SIZE(1.. maxnoofCellsforMBMS)) OF ECGI

MBMS-E-RAB-QoS-Parameters ::= SEQUENCE {

qCI								QCI,

gbrQosInformation				GBR-QosInformation		OPTIONAL,

allocationAndRetentionPriority	AllocationAndRetentionPriority,

iE-Extensions					ProtocolExtensionContainer { { MBMS-E-RAB-QoS-Parameters-ExtIEs} }	OPTIONAL,

...

}

MBMS-E-RAB-QoS-Parameters-ExtIEs M2AP-PROTOCOL-EXTENSION ::= {

...

}

MBMS-Service-associatedLogicalM2-ConnectionItem ::= SEQUENCE {

eNB-MBMS-M2AP-ID				ENB-MBMS-M2AP-ID OPTIONAL,

mCE-MBMS-M2AP-ID				MCE-MBMS-M2AP-ID OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { MBMS-Service-associatedLogicalM2-ConnectionItemExtIEs} } OPTIONAL,

...

}

MBMS-Service-associatedLogicalM2-ConnectionItemExtIEs M2AP-PROTOCOL-EXTENSION ::= {

...

}

MBMS-Service-Area ::= OCTET STRING

MBMS-Service-Area-ID-List ::= SEQUENCE (SIZE(1..maxnoofMBMSServiceAreasPerCell)) OF MBMS-Service-Area

MBMS-Session-ID ::= OCTET STRING (SIZE (1))

MBMSsessionListPerPMCH-Item ::= SEQUENCE (SIZE(1..maxnoofSessionsPerPMCH)) OF SEQUENCE {

tmgi							TMGI,

lcid							LCID,

iE-Extensions					ProtocolExtensionContainer { { MBMSsessionListPerPMCH-Item-ExtIEs} } OPTIONAL,

...

}

MBMSsessionListPerPMCH-Item-ExtIEs M2AP-PROTOCOL-EXTENSION ::= {

...

}

MBMSsessionsToBeSuspendedListPerPMCH-Item ::= SEQUENCE (SIZE(1..maxnoofSessionsPerPMCH)) OF SEQUENCE {

tmgi							TMGI,

iE-Extensions					ProtocolExtensionContainer { { MBMSsessionsToBeSuspendedListPerPMCH-Item-ExtIEs} } OPTIONAL,

...

}

MBMSsessionsToBeSuspendedListPerPMCH-Item-ExtIEs M2AP-PROTOCOL-EXTENSION ::= {

...

}

MBSFN-Area-ID ::= INTEGER (0..255)

MBSFN-SynchronisationArea-ID ::= INTEGER (0..65535)

MBSFN-Subframe-Configuration ::= SEQUENCE {

radioframeAllocationPeriod		ENUMERATED {n1, n2, n4, n8, n16, n32},

radioframeAllocationOffset		INTEGER (0..7),

subframeAllocation				CHOICE {

oneFrame						BIT STRING (SIZE (6)  ),

fourFrames						BIT STRING (SIZE (24) ) },

iE-Extensions					ProtocolExtensionContainer { { MBSFN-Subframe-Configuration-ExtIEs} } OPTIONAL,

...

}

MBSFN-Subframe-Configuration-ExtIEs M2AP-PROTOCOL-EXTENSION ::= {

{ID id-SubframeAllocationExtended			CRITICALITY reject	EXTENSION SubframeAllocationExtended			PRESENCE optional}|

{ID id-SubframeAllocationFurtherExtension	CRITICALITY reject	EXTENSION SubframeAllocationFurtherExtension		PRESENCE optional},

...

}

MCCH-Update-Time ::= INTEGER (0..255)

MCCHrelatedBCCH-ConfigPerMBSFNArea-Item ::= SEQUENCE {

mbsfnArea						MBSFN-Area-ID,

pdcchLength						ENUMERATED {s1, s2, ...},

repetitionPeriod				ENUMERATED {rf32, rf64, rf128, rf256},

offset							INTEGER (0..10),

modificationPeriod				ENUMERATED {rf512, rf1024},

subframeAllocationInfo			BIT STRING (SIZE(6)),

modulationAndCodingScheme		ENUMERATED {n2, n7, n13, n19},

cellInformationList				Cell-Information-List	OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { MCCHrelatedBCCH-ConfigPerMBSFNArea-Item-ExtIEs} } OPTIONAL,

...

}

MCCHrelatedBCCH-ConfigPerMBSFNArea-Item-ExtIEs M2AP-PROTOCOL-EXTENSION ::= {

-- Extension for Rel-14 to support MCCH repetition period values –-

{ID id-Repetition-PeriodExtended			CRITICALITY reject	EXTENSION Repetition-PeriodExtended	PRESENCE optional}|

-- Extension for Rel-14 to support MCCH modification period values –-

{ID id-Modification-PeriodExtended			CRITICALITY reject	EXTENSION Modification-PeriodExtended	PRESENCE optional}|

{ID id-Subcarrier-SpacingMBMS				CRITICALITY reject	EXTENSION Subcarrier-SpacingMBMS		PRESENCE optional},

...

}

MCCHrelatedBCCH-ExtConfigPerMBSFNArea-Item ::= SEQUENCE {

mbsfnArea						MBSFN-Area-ID,

repetitionPeriodExpanded		ENUMERATED {rf1, rf2, rf4, rf8, rf16, rf32, rf64, rf128, rf256, ...},

offset							INTEGER (0..10),

modificationPeriodExpanded		ENUMERATED {rf1, rf2, rf4, rf8, rf16, rf32, rf64, rf128, rf256, rf512, rf1024, ...},

subframeAllocationInfoExpanded	BIT STRING (SIZE(10)),

modulationAndCodingScheme		ENUMERATED {n2, n7, n13, n19},

subcarrier-SpacingMBMSExpanded	ENUMERATED {khz-7dot5, khz-2dot5, khz-1dot25, khz-0dot37, ..., kHz15},

timeSeparation					ENUMERATED {sl2, sl4, ...}		OPTIONAL,

cellInformationList				Cell-Information-List			OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { MCCHrelatedBCCH-ExtConfigPerMBSFNArea-Item-ExtIEs} } OPTIONAL,

...

}

MCCHrelatedBCCH-ExtConfigPerMBSFNArea-Item-ExtIEs M2AP-PROTOCOL-EXTENSION ::= {

-- Extension for Rel-17 to support PMCH Bandwidth values –-

{ID id-AdditionalConfigParameters	CRITICALITY reject	EXTENSION AdditionalConfigParameters	PRESENCE optional},

...

}

MCE-ID ::= OCTET STRING (SIZE(2))

MCE-MBMS-M2AP-ID ::= INTEGER (0.. 16777215)

MCEname ::= PrintableString (SIZE (1..150,...))

MCH-Scheduling-Period ::= ENUMERATED {rf8, rf16, rf32, rf64, rf128, rf256, rf512, rf1024}

MCH-Scheduling-PeriodExtended ::= ENUMERATED {rf4, ...}

MCH-Scheduling-PeriodExtended2 ::= ENUMERATED {rf1, rf2, ...}

Modulation-Coding-Scheme2 ::= INTEGER (0..27)

Modification-PeriodExtended ::= ENUMERATED {rf1, rf2, rf4, rf8, rf16, rf32, rf64, rf128, rf256, ...}

#### -- N

#### -- O

#### -- P

PLMN-Identity ::= OCTET STRING (SIZE(3))

PMCH-Bandwidth ::= ENUMERATED {n40, n35, n30, ...}

PMCH-Configuration ::= SEQUENCE {

allocatedSubframesEnd			AllocatedSubframesEnd,

dataMCS							INTEGER (0..28),

mchSchedulingPeriod				MCH-Scheduling-Period,

iE-Extensions					ProtocolExtensionContainer { {PMCH-Configuration-ExtIEs} } OPTIONAL,

...

}

PMCH-Configuration-ExtIEs M2AP-PROTOCOL-EXTENSION ::= {

-- Extension for Rel-12 to support 256QAM for MTCH –

{ID id-Modulation-Coding-Scheme2			CRITICALITY reject	EXTENSION Modulation-Coding-Scheme2		PRESENCE optional}|

-- Extension for Rel-12 to support shorter MCH scheduling period –

{ID id-MCH-Scheduling-PeriodExtended		CRITICALITY reject	EXTENSION MCH-Scheduling-PeriodExtended	PRESENCE optional}|

-- Extension for Rel-14 to support shorter MCH scheduling period values –

{ID id-MCH-Scheduling-PeriodExtended2		CRITICALITY	reject	EXTENSION MCH-Scheduling-PeriodExtended2	PRESENCE optional},

...

}

Common-Subframe-Allocation-Period ::= ENUMERATED {rf4, rf8, rf16, rf32, rf64, rf128, rf256}

Pre-emptionCapability ::= ENUMERATED {

shall-not-trigger-pre-emption,

may-trigger-pre-emption

}

Pre-emptionVulnerability ::= ENUMERATED {

not-pre-emptable,

pre-emptable

}

PriorityLevel ::= INTEGER { spare (0), highest (1), lowest (14), no-priority (15) } (0..15)

#### -- Q

#### QCI ::= INTEGER (0..255)

#### -- R

#### Repetition-PeriodExtended ::= ENUMERATED {rf1, rf2, rf4, rf8, rf16, ...}

#### -- S

SC-PTM-Information ::= SEQUENCE {

mbmsCellList					MBMS-Cell-List,

mbms-E-RAB-QoS-Parameters		MBMS-E-RAB-QoS-Parameters,

iE-Extensions					ProtocolExtensionContainer { {SC-PTM-Information-ExtIEs} } OPTIONAL,

...

}

SC-PTM-Information-ExtIEs M2AP-PROTOCOL-EXTENSION ::= {

...

}

SFN ::= INTEGER (0..1023)

Subcarrier-SpacingMBMS ::= ENUMERATED {khz-7dot5, khz-1dot25, ...}

SubframeAllocationExtended	::=	CHOICE {

oneFrameExtension				BIT STRING (SIZE(2)),

fourFrameExtension				BIT STRING (SIZE(8)),

choice-extension				ProtocolIE-Single-Container { { SubframeAllocationExtended-ExtIEs} },

...

}

SubframeAllocationExtended-ExtIEs M2AP-PROTOCOL-IES ::= {	...

}

SubframeAllocationFurtherExtension	::=	CHOICE {

oneFrameFurtherExtension				BIT STRING (SIZE(2)),

fourFrameFurtherExtension				BIT STRING (SIZE(8)),

choice-extension				ProtocolIE-Single-Container { { SubframeAllocationFurtherExtension-ExtIEs} },

...

}

SubframeAllocationFurtherExtension-ExtIEs M2AP-PROTOCOL-IES ::= {	...

}

#### -- T

TimeToWait ::= ENUMERATED {v1s, v2s, v5s, v10s, v20s, v60s, ...}

TMGI ::= SEQUENCE {

pLMNidentity			PLMN-Identity,

serviceID				OCTET STRING (SIZE (3)),

iE-Extensions			ProtocolExtensionContainer { {TMGI-ExtIEs} } OPTIONAL,

...

}

TMGI-ExtIEs M2AP-PROTOCOL-EXTENSION ::= {

...

}

TNL-Information ::= SEQUENCE {

iPMCAddress						IPAddress,

iPSourceAddress					IPAddress,

gTP-TEID						GTP-TEID,

iE-Extensions					ProtocolExtensionContainer { {TNL-Information-ExtIEs} } OPTIONAL,

...

}

TNL-Information-ExtIEs M2AP-PROTOCOL-EXTENSION ::= {

...

}

TypeOfError ::= ENUMERATED {

not-understood,

missing,

...

}

#### -- U

#### -- V

#### -- W

#### -- X

#### -- Y

#### -- Z

END

Next Change

### 9.3.7	Constant definitions

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

M2AP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) m2ap (4) version1 (1) m2ap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM M2AP-CommonDataTypes;

-- **************************************************************

--

#### -- Elementary Procedures

--

-- **************************************************************

id-sessionStart													ProcedureCode ::= 0

id-sessionStop													ProcedureCode ::= 1

id-mbmsSchedulingInformation									ProcedureCode ::= 2

id-errorIndication												ProcedureCode ::= 3

id-reset														ProcedureCode ::= 4

id-m2Setup														ProcedureCode ::= 5

id-eNBConfigurationUpdate										ProcedureCode ::= 6

id-mCEConfigurationUpdate										ProcedureCode ::= 7

id-privateMessage												ProcedureCode ::= 8

id-sessionUpdate												ProcedureCode ::= 9

id-mbmsServiceCounting											ProcedureCode ::= 10

id-mbmsServiceCountingResultsReport								ProcedureCode ::= 11

id-mbmsOverloadNotification										ProcedureCode ::= 12

-- **************************************************************

--

#### -- Lists

--

-- **************************************************************

maxnoofMBSFNareas							INTEGER ::= 256

maxnoofMBSFN-Allocations					INTEGER ::= 8

maxnoofPMCHsperMBSFNarea					INTEGER ::= 15

maxnoofCells								INTEGER ::= 256

maxnoofMBMSServiceAreasPerCell				INTEGER ::= 256

maxnoofSessionsPerPMCH						INTEGER ::= 29

maxnooferrors								INTEGER ::= 256

maxNrOfIndividualM2ConnectionsToReset		INTEGER ::= 256

maxnoofCountingService						INTEGER ::= 16

maxnoofCellsforMBMS							INTEGER ::= 4096

-- **************************************************************

--

#### -- Ies

--

-- **************************************************************

id-MCE-MBMS-M2AP-ID															ProtocolIE-ID ::= 0

id-ENB-MBMS-M2AP-ID															ProtocolIE-ID ::= 1

id-TMGI																		ProtocolIE-ID ::= 2

id-MBMS-Session-ID															ProtocolIE-ID ::= 3

id-MBMS-Service-Area														ProtocolIE-ID ::= 6

id-TNL-Information															ProtocolIE-ID ::= 7

id-CriticalityDiagnostics													ProtocolIE-ID ::= 8

id-Cause																	ProtocolIE-ID ::= 9

id-MBSFN-Area-Configuration-List											ProtocolIE-ID ::= 10

id-PMCH-Configuration-List													ProtocolIE-ID ::= 11

id-PMCH-Configuration-Item													ProtocolIE-ID ::= 12

id-GlobalENB-ID																ProtocolIE-ID ::= 13

id-ENBname																	ProtocolIE-ID ::= 14

id-ENB-MBMS-Configuration-data-List											ProtocolIE-ID ::= 15

id-ENB-MBMS-Configuration-data-Item											ProtocolIE-ID ::= 16

id-GlobalMCE-ID																ProtocolIE-ID ::= 17

id-MCEname																	ProtocolIE-ID ::= 18

id-MCCHrelatedBCCH-ConfigPerMBSFNArea										ProtocolIE-ID ::= 19

id-MCCHrelatedBCCH-ConfigPerMBSFNArea-Item									ProtocolIE-ID ::= 20

id-TimeToWait																ProtocolIE-ID ::= 21

id-MBSFN-Subframe-Configuration-List										ProtocolIE-ID ::= 22

id-MBSFN-Subframe-Configuration-Item										ProtocolIE-ID ::= 23

id-Common-Subframe-Allocation-Period										ProtocolIE-ID ::= 24

id-MCCH-Update-Time															ProtocolIE-ID ::= 25

id-ENB-MBMS-Configuration-data-List-ConfigUpdate							ProtocolIE-ID ::= 26

id-ENB-MBMS-Configuration-data-ConfigUpdate-Item							ProtocolIE-ID ::= 27

id-MBMS-Service-associatedLogicalM2-ConnectionItem							ProtocolIE-ID ::= 28

id-MBSFN-Area-ID															ProtocolIE-ID ::= 29

id-ResetType																ProtocolIE-ID ::= 30

id-MBMS-Service-associatedLogicalM2-ConnectionListResAck					ProtocolIE-ID ::= 31

id-MBMS-Counting-Request-Session											ProtocolIE-ID ::= 32

id-MBMS-Counting-Request-Session-Item										ProtocolIE-ID ::= 33

id-MBMS-Counting-Result-List												ProtocolIE-ID ::= 34

id-MBMS-Counting-Result-Item												ProtocolIE-ID ::= 35

id-Modulation-Coding-Scheme2												ProtocolIE-ID ::= 36

id-MCH-Scheduling-PeriodExtended											ProtocolIE-ID ::= 37

id-Alternative-TNL-Information												ProtocolIE-ID ::= 38

id-Overload-Status-Per-PMCH-List											ProtocolIE-ID ::= 39

id-PMCH-Overload-Status														ProtocolIE-ID ::= 41

id-Active-MBMS-Session-List													ProtocolIE-ID ::= 42

id-MBMS-Suspension-Notification-List 										ProtocolIE-ID ::= 43

id-MBMS-Suspension-Notification-Item										ProtocolIE-ID ::= 44

id-SC-PTM-Information														ProtocolIE-ID ::= 45

id-Modification-PeriodExtended												ProtocolIE-ID ::= 46

id-Repetition-PeriodExtended												ProtocolIE-ID ::= 47

id-MCH-Scheduling-PeriodExtended2											ProtocolIE-ID ::= 48

id-Subcarrier-SpacingMBMS													ProtocolIE-ID ::= 49

id-SubframeAllocationExtended												ProtocolIE-ID ::= 50

id-MCCHrelatedBCCH-ExtConfigPerMBSFNArea-Item 								ProtocolIE-ID ::= 51

id-MCCHrelatedBCCH-ExtConfigPerMBSFNArea									ProtocolIE-ID ::= 52

id-SubframeAllocationFurtherExtension										ProtocolIE-ID ::= 53

id-AdditionalConfigParameters												ProtocolIE-ID ::= XX

END

End of Changes