3GPP TSG-RAN WG Meeting #95 Electronic	RP-220548

Online, March, 17-23, 2022

Agenda item:	9.3.2.1

Source:	Xiaomi Communications

Title:	Discussion on Rel-18 WID: Further NR mobility enhancements

Document for:	Discussion and Decision

# 1 	Introduction

In RAN#94-e, a new WID [1] for mobility enhancement was approved. According to the objectives of the mobility WID, there are still some points which are unclear or not determined. In this contribution, we provide our understandings and analysis on the Rel-18 WID: Further NR mobility enhancements.

# 2	Discussions

Views on Objective 3:

According to Note 5 in the Objective 3, we need to check whether CHO including target MCG and target SCG has been supported in Rel-17. According to the Rel-17 RAN3 work, RAN3 specified CHO including target MCG and target SCG in TEI17. In RAN3#115-e, the three CRs [2] [3] [4] of supporting CHO including target MCG and target SCG were approved/endorsed. Two container IEs are added in the Addition and Modification procedures respectively, which indicate the Addition or Modification related to a conditional mobility supporting CHO including target MCG and target SCG.

Observation 1: According to the CRs approved in RAN3#115-e, CHO including target MCG and target SCG is supported in RAN3 in Rel-17.

According to the 3GPP TS 38.331, when a CHO is triggered, the UE applies an RRCReconfiguration message for handover, regardless of whether the RRCReconfiguration message includes the target SCG or not. RAN2 specification can support CHO including target MCG and target SCG without extra enhancements.

Observation 2: RAN2 specification can support CHO including target MCG and target SCG without extra enhancements in Rel-17.

According to the Observation 1 and Observation 2 given above, we think that the Objective 3 of the mobility work item can be removed

Proposal 1: Objective 3 of CHO including target MCG and target SCG can be removed.

Views on Objective 4:

In Objective 4 of CHO including target MCG and candidate SCGs for CPC/CPA, the applicable scenarios for MR-DC are still not clear. In R16, CHO (MCG) can work together with MR-DC (i.e. either via LTE or via NR). The CPC/CPA for EN-DC and NR-DC has been supported, and CHO with MR-DC has been discussed in Rel-17. Hence, we think that MR-DC supporting should be added for Objective 4, especially to support EN-DC and NR-DC.

Observation 3: The CPC/CPA for EN-DC and NR-DC has been supported.

Proposal 2: MR-DC support including EN-DC and NR-DC should be indicated in Objective 4 of CHO including target MCG and candidate SCGs for CPC/CPA.

According to the analysis given above, we consider that the mobility WID can be updated according to the above Proposal 1 and Proposal 2:

Proposal 3: To update the Rel-18 WID: Further NR mobility enhancements as follows:

- To specify CHO including target MCG and candidate SCGs for CPC/CPA for EN-DC and NR-DC [RAN3, RAN2] - CHO including target MCG and target SCG is used as the baseline

# 3	Conclusion

According to the analysis given above, we have the following observations and proposals:

Observation 1: According to the CRs approved in RAN3#115-e, CHO including target MCG and target SCG is supported in RAN3 in Rel-17.

Observation 2: RAN2 specification can support CHO including target MCG and target SCG without extra enhancements in Rel-17.

Observation 3: The CPC/CPA for EN-DC and NR-DC has been supported.

Proposal 1: Objective 3 of CHO including target MCG and target SCG can be removed.

Proposal 2: MR-DC support including EN-DC and NR-DC should be indicated in Objective 4 of CHO including target MCG and candidate SCGs for CPC/CPA.

Proposal 3: To update the Rel-18 WID: Further NR mobility enhancements as follows:

- To specify CHO including target MCG and candidate SCGs for CPC/CPA for EN-DC and NR-DC [RAN3, RAN2] - CHO including target MCG and target SCG is used as the baseline

# 4 	References

[1] RP-213565 New WI: Further NR mobility enhancements, MediaTek

[2] R3-222838 Enabling CHO with SCG configuration [CHOwithDCkept], Nokia, Nokia Shanghai Bell, CMCC, Vodafone, CR0580r3, TS 38.423 v16.8.0, Rel-17

[3] R3-222839 Enabling CHO with SCG configuration [CHOwithDCkept], Nokia, Nokia Shanghai Bell, CMCC, Vodafone, CR1590r3, TS 36.423 v16.8.0, Rel-17

[4] R3-222821 Support of CHO with SCG configuration, Huawei, China Telecom, China Unicom, draftCR