3GPP TSG RAN meeting #95e										       RP-220563

Electronic Meeting, March 17 - 23, 2022

Status Report to TSG

Agenda item:			9.5.1.5

| WI / SI Name | NR coverage enhancements |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_cov_enh |  |  |  |  |
| Unique ID | 900061 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-211566 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: NA | Core part: 03/2022 | Performance part: 09/2022 | Testing part: NA |  |
| Overall Completion level | Study Item: NA | Core part: 100% | Performance Part: 35% | Testing part: NA |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN WG1 |
| --- | --- | --- |
| Rapporteur | Name | Jianchi Zhu |
|  | Company | China Telecom |
|  | Email | zhujc@chinatelecom.cn |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

2.1	RAN1

2.1.1	Agreements

RAN1 #107b-e

Enhancements on PUSCH repetition type A:

Conclusion:

No consensus to introduce pusch-AggregationFactor-r17.

Agreement

- Remove the notes from “Per (UE, cell, TRP, …)” and “Comment” columns of the existing AvailableSlotCounting in the consolidated RRC parameter list.

- If separate FGs are defined for DG-PUSCH and CG-PUSCH, add another AvailableSlotCounting to the consolidated RRC parameter list, with the following contents.

| WI code | Sub-feature group | RAN1 specification | Section | RAN2 Parant IE | RAN2 ASN.1 name | Parameter name in the spec | New or existing? | Parameter name in the text | Description | Value range | Default value aspect | Per (UE, cell, TRP, …) | UE-specific or Cell-specific | Specification | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR_cov_enh-Core | Enhancement on PUSCH repetition Type A |  |  |  |  | AvailableSlotCounting | new |  | Enabling PUSCH repetitions counted on the basis of available slots | ENUMERATED {enabled, disable } |  | in ConfiguredGrantConf | UE-specific | 38.331 | Agreement: • Each available slot identified by the UE is considered as a transmission occasion for PUSCH repetition. o RV is cycled across transmission occasions, irrespective of whether PUSCH transmission in the transmission occasion is further omitted or not. |

Conclusion:

- The cancellation of LP PUSCH (introduced in Rel-17 eIIoT/URLLC WI) is applied in Step 2 of the previously agreed 2-step procedure of Rel-17 PUSCH repetitions counted on the basis of available slots (i.e., Option 1-B).

  - No specification impact is expected.

Conclusion

- The CovEnh discussion on the available slot counting for inter-cell mTRPs is deferred until further progress on the collision handling between UL channels/signals and multiple SSBs for inter-cell mTRPs is made in feMIMO session.

TB processing over multi-slot PUSCH:

Conclusion

- There is no consensus in RAN1 on whether the index of the starting coded bit in the circular buffer should be expressed as function of the lifting size Zc.

Agreement

- The Rel-16 per-slot transmission occasion definition is re-used for transmission power determination for TBoMS.

Conclusion

- Configuration and/or indication of priority of TBoMS transmission is up to gNB.

- No new TBoMS-specific collision handling and dropping rules are introduced.

Agreement

The following text proposal for TS 38.213, Clause 9.2.6, is adopted.

9.2.6 PUCCH repetition procedure <omitted text> If a UE would transmit a PUCCH over a first number of slots and the UE would transmit a PUSCH with repetition Type A or a TB processing over multiple slots over a second number of slots, and the PUCCH transmission would overlap with the PUSCH transmission in one or more slots, and the conditions in clause 9.2.5 for multiplexing the UCI in the PUSCH are satisfied in the overlapping slots, the UE transmits the PUCCH and does not transmit the PUSCH in the overlapping slots. <omitted text>

Conclusion

- Existing rules can be reused for UCI multiplexing on PUSCH in case of TBoMS and UL CA scenario.

Agreement

- A UE that supports TBoMS supports all values of N defined for TBoMS, and a UE that supports TBoMS repetition supports all values of M defined for TBoMS repetition.

Agreement

- The use of TBoMS for HD-FDD UE with counting on available slot is supported.

- Note: existing mechanism as in AI8.8.1.1 should be applied for this case

Agreement

- For CG-PUSCH transmissions of TBoMS, the UE is not expected to be configured with the time duration for the N*M transmissions larger than the time duration derived by the periodicity P.

Joint channel estimation for PUSCH:

Conclusion:

- It is not expected to redefine transmission occasion for PUSCH/PUCCH for DMRS bundling in Rel-17.

Agreement:

- The value range of PUSCH-TimeDomainWindowLength is INTEGER (2..[32]).

- The value range of PUCCH-TimeDomainWindowLength is INTEGER (2..[8]).

- Note: the value shall not exceed the maximum duration.

Agreement:

Adopt the following TP to TS 38.214

6.1.7 UE procedure for determining time domain windows for bundling DM-RS < unchanged text omitted> - For PUCCH transmissions of PUCCH repetition, a dropping or cancellation of a PUCCH transmission according to clause 9, clause 9.2.6 and clause 11.1 of [6, TS 38.213]. < unchanged text omitted>

Agreement:

Send an LS to RAN4 asking the following question

- For extended CP, is 11-symbol the maximum length for the non-zero un-scheduled gap in-between the PUSCH transmission or PUCCH repetition, when UE is required to maintain power consistency and phase continuity?

Endorsement

[DRAFT] LS on DMRS bundling for PUSCH and PUCCH is endorsed in principle.

Agreement

Final LS R1-2200773 on DMRS bundling for PUSCH and PUCCH is endorsed.

Agreement:

- If DMRS bundling and UL beam switching for multi-TRP operation are configured simultaneously, UL beam switching for multi-TRP operation is regarded as a semi-static event.

Agreement:

Update the description of the RRC parameters PUSCH-Window-Restart and PUCCH-Window-Restart as follows.

- UE bundles PUSCH DM-RS remaining in a nominal time domain window after event(s) triggered by DCI or MAC-CE that violate power consistency and phase continuity requirements

- UE bundles PUCCH DM-RS remaining in a nominal time domain window after event(s) triggered by DCI or MAC-CE that violate power consistency and phase continuity requirements

Note: Events which are triggered by DCI or MAC CE, but regarded as semi-static events, e.g. frequency hopping, UL beam switching for multi-TRP operation, or other if defined, are excluded.

PUCCH enhancements:

Agreement

In column J of RRC parameter “PUCCH-nrofSlots-r17”, add a note as the following:

- Note: a PUCCH resource not configured with PUCCH-nrofSlots-r17 can attain the value of 1 according to when the Rel-15/16 parameter nrofSlots is not configured.

Agreement

For PUCCH repetitions and PUSCH repetitions with DMRS bundling, introduce the following two RRC parameters for frequency hopping interval configuration.

- PUCCH-Frequencyhopping-Interval

- PUSCH-Frequencyhopping-Interval

Note: finalize the details (such as value range, parent IE, etc…) of these two RRC parameters in RAN1 107#bis-e.

Agreement

PUCCH repetitions with different sets of power control parameters in multi-TRP operation should be regarded as a [semi-static] event that causes power consistency and phase continuity not to be maintained across PUCCH repetitions.

Agreement

- The RRC parameter “PUCCH-DMRS-Bundling” is per UL BWP, and the RRC parameter “PUCCH-TimeDomainWindowLength” is per UL BWP.

  - PUCCH DMRS bundling is not supported for PUCCH format 0/2.

Agreement

Adopt the following table for the RRC parameters PUCCH-nrofSlots-r17, PUCCH-DMRS-Bundling, PUCCH-TimeDomainWindowLength, PUCCH-Window-Restart, PUCCH-Frequencyhopping-Interval, and PUSCH-Frequencyhopping-Interval.

| WI code | Sub-feature group | Parameter name in the spec | New or existing? | Description | Value range | Default value aspect | Per (UE, cell, TRP, …) | UE-specific or Cell-specific |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR_cov_enh-Core | PUCCH enhancements | PUCCH-nrofSlots-r17 | new | A new repetition parameter corresponding to Rel-17 dynamic PUCCH repetition factor indication. The new repetition parameter is configured per PUCCH resource and should be in PUCCH-Resource. Note: a PUCCH resource not configured with PUCCH-nrofSlots-r17 can attain the value of 1 according to when the Rel-15/16 parameter nrofSlots is not configured. | ENUMERATED {2, 4, 8} |  | In PUCCH-resource | UE-specific |
| NR_cov_enh-Core | DM-RS bundling for PUCCH | PUCCH-DMRS-Bundling | new | Enabling/disabling of DM-RS bundling and time domain window for PUCCH repetitions. | ENUMERATED {enabled, disable } |  | in PUCCH-Config Note: The RRC parameter “PUCCH-DMRS-Bundling” is per UL BWP. PUCCH DMRS Bundling is not supported for PUCCH format 0/2 | UE-specific |
| NR_cov_enh-Core | DM-RS bundling for PUCCH | PUCCH-TimeDomainWindowLength | new | Length of a [nominal] time domain window in slots for DMRS bundling for PUCCH. | INTEGER (2..[8]) Note: the value shall not exceed the maximum duration |  | in PUCCH-Config Note: The RRC parameter “PUCCH-TimeDomainWindowLength” is per UL BWP. PUCCH DMRS Bundling is not supported for PUCCH format 0/2 | UE-specific |
| NR_cov_enh-Core | DM-RS bundling for PUCCH | PUCCH-Window-Restart | new | UE bundles PUCCH DM-RS slots remaining in a bundling nominal time domain window after a slot for which events violate power consistency and phase continuity requirements UE bundles PUCCH DM-RS remaining in a nominal time domain window after dynamic event(s) triggered by DCI or MAC-CE that violate power consistency and phase continuity requirements Note: Events, which are triggered by DCI or MAC CE, but regarded as semi-static events, e.g. frequency hopping, UL beam switching for multi-TRP operation, or other if defined, are excluded. | ENUMERATED {enabled, disable } |  | in PUCCH-Config | UE-specific |
| NR_cov_enh-Core | DM-RS bundling for PUCCH | PUCCH-Frequencyhopping-Interval | New | Number of consecutive slots for UE to perform Rel-17 inter-slot frequency hopping with inter-slot bundling for PUCCH Note: When DMRS bundling for PUCCH is enabled by PUCCH-DMRS-Bundling, PUCCH frequency hopping interval is only determined by the configuration of PUCCH hopping interval if PUCCH hopping interval is configured. | FFS | PUCCH-TimeDomainWindowLength | in PUCCH-Config | UE-specific |
| NR_cov_enh-Core | DM-RS bundling for PUSCH | PUSCH-Frequencyhopping-Interval | New | Number of consecutive slots for UE to perform Rel-17 inter-slot frequency hopping with inter-slot bundling for PUSCH. Note 1: This RRC parameter is shared for both DG-PUSCH and CG-PUSCH Note 2: When DMRS bundling for PUSCH is enabled by PUSCH-DMRS-Bundling, PUSCH frequency hopping interval is only determined by the configuration of PUSCH hopping interval if PUSCH hopping interval is configured. | FFS | PUSCH-TimeDomainWindowLength | in PUSCH-Config | UE-specific |

Agreement

The same mechanism supporting inter-slot frequency hopping with inter-slot bundling for DMRS bundling for type-A PUSCH repetitions is reused for TBoMS with/without repetitions.

- No optimization of FH mechanism specific to TBoMS with repetitions

Type A PUSCH repetitions for Msg3:

Agreement

Regarding how a UE should interpret MCS information field for indication of the number of repetitions for the case of CBRA, Option 1 is supported.

- When a UE requests Msg3 repetition, the repurposed MCS information field is applied. gNB schedules Msg3 with or without repetition for the UE requesting Msg3 repetition.

  - Repetition factor K=1 is included in the four candidate repetition factors used for repetition indication.

- When the UE doesn’t request Msg3 repetition (including legacy UE), the legacy MCS information field is applied. gNB schedules Msg3 without repetition for the UE not requesting Msg3 repetition.

Agreement

The 3 LSB bits of MCS information field in DCI format 0_0 with CRC scrambled by the TC-RNTI is used to indicate one value from 8 candidate MCS indexes for Msg3 retransmission.

  - The 8 candidate MCS indexes can be configured by SIB1, MCS 0~7 are applied if the configuration is absent. The first 4 indexes of the 8 candidate MCS indexes are used for initial PUSCH transmission scheduled by RAR UL grant.

Agreement

For the number of repetitions configured by numberOfMsg3Repetitions, support {1, 2, 3, 4, 7, 8, 12, 16}.

Conclusion

For Rel-17 CE WI, Issue (4~7) in Section 2.6 of R1-2200712 will not be discussed in RAN1 in future meetings, and issue (1~3) in Section 2.6 of R1-2200712 can only be discussed in RAN1 if requested by other WGs.

Agreement

All slots are considered as available slots for Msg3 repetition for both FD-FDD UEs and HD-FDD RedCap UEs.

Agreement

Introduce the following the RRC parameter for indication of the number of repetitions for PUSCH repetition scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI.

| WI code | Sub-feature group | Parameter name in the spec | New or existing? | Description | Value range | Default value aspect | Per (UE, cell, TRP, …) | UE-specific or Cell-specific | Specification | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR_cov_enh-Core | Type A PUSCH repetitions for Msg3 | numberOfMsg3Repetitions | new | The number of repetitions for PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI | SEQUENCE (SIZE (4)) OF INTEGER (1,2, 3, 4, 7, 8, 12, 16) | {1, 2, 3, 4} | RACH-ConfigCommon | Cell-specific | 38.331 | Working Assumption Down-select only one from the following methods for indication of the number of repetitions of Msg3 initial transmission. - Alt 1: If TDRA information field is chosen, Option 2 is supported. - The candidate values for repetition factor could be chosen from {[1], 2, 3, 4, 7, 8, [12], [16]} - Alt 2: If MCS information field is chosen, repurpose the MCS information field as follows. - 2 MSB bits of the MCS information field are used for selecting one repetition factor from a SIB1 configured set with 4 candidate values. - The set of candidate values for repetition factor could be chosen from {[1], 2, 3, 4, 7, 8, [12], [16]} Note: Whether ‘1’ is included depends on the outcome of interpretation of the selected information field. Agreements - For indication of the number of repetitions of Msg3 initial transmission, Alt 2 (i.e., using MCS information field) is adopted. - Four candidate MCS indexes can be configured by SIB1 for Msg3 initial transmission. MCS 0~3 are applied if the configuration is absent. If the four candidate repetition factors are not configured, the default values are {1, 2, 3, 4}. Agreement For repetition indication for Msg3 re-transmission, Option 1 (i.e., use the same mechanism as supported for Msg3 initial transmission) is adopted. FFS: [8] MCS index to be used for Msg3 re-transmission Working assumption : support repetition for a PUSCH scheduled by RAR UL grant, including both Msg3 PUSCH and CFRA PUSCH. - Use the same mechanism of Msg3 PUSCH repetition, when applicable, for CFRA PUSCH with repetitions. - No separate CFRA preamble/RO for repetition of CFRA PUSCH is introduced. - No additional optimization specific for CFRA PUSCH is considered for CFRA PUSCH with repetition. - No additional RAN1 specification impact Note: UE reports Msg3 repetition capability after initial access. Note: The working assumption can be confirmed only if no additional RAN1 specification impact nor optimization specific for CFRA PUSCH. Agreement Regarding how a UE should interpret MCS information field for indication of the number of repetitions for the case of CBRA, Option 1 is supported. - When a UE requests Msg3 repetition, the repurposed MCS information field is applied. gNB schedules Msg3 with or without repetition for the UE requesting Msg3 repetition. - Repetition factor K=1 is included in the four candidate repetition factors used for repetition indication. When the UE doesn’t request Msg3 repetition (including legacy UE), the legacy MCS information field is applied. gNB schedules Msg3 without repetition for the UE not requesting Msg3 repetition. Agreement For the number of repetitions configured by numberOfMsg3Repetitions, support {1, 2, 3, 4, 7, 8, 12, 16}. |

Agreement

Introduce the following the RRC parameter for indication of the MCS index for PUSCH repetition scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI.

| WI code | Sub-feature group | Parameter name in the spec | New or existing? | Description | Value range | Default value aspect | Per (UE, cell, TRP, …) | UE-specific or Cell-specific | Specification | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR_cov_enh-Core | Type A PUSCH repetitions for Msg3 | mcs-Msg3Repetition | new | Configure eight candidate MCS indexes for PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI. Only the first 4 configured or default MCS indexes are used for PUSCH transmission scheduled by RAR UL grant. | SEQUENCE (SIZE (8)) OF INTEGER (0..31) | {0, 1, 2, 3, 4, 5, 6, 7} | RACH-ConfigCommon | Cell-specific | 38.331 | Agreements - For indication of the number of repetitions of Msg3 initial transmission, Alt 2 (i.e., using MCS information field) is adopted. - Four candidate MCS indexes can be configured by SIB1 for Msg3 initial transmission. MCS 0~3 are applied if the configuration is absent. If the four candidate repetition factors are not configured, the default values are {1, 2, 3, 4}. Agreement For repetition indication for Msg3 re-transmission, Option 1 (i.e., use the same mechanism as supported for Msg3 initial transmission) is adopted. FFS: [8] MCS index to be used for Msg3 re-transmission Working assumption : support repetition for a PUSCH scheduled by RAR UL grant, including both Msg3 PUSCH and CFRA PUSCH. - Use the same mechanism of Msg3 PUSCH repetition, when applicable, for CFRA PUSCH with repetitions. - No separate CFRA preamble/RO for repetition of CFRA PUSCH is introduced. - No additional optimization specific for CFRA PUSCH is considered for CFRA PUSCH with repetition. - No additional RAN1 specification impact Note: UE reports Msg3 repetition capability after initial access. Note: The working assumption can be confirmed only if no additional RAN1 specification impact nor optimization specific for CFRA PUSCH. Agreement The 3 LSB bits of MCS information field in DCI format 0_0 with CRC scrambled by the TC-RNTI is used to indicate one value from 8 candidate MCS indexes for Msg3 retransmission. The 8 candidate MCS indexes can be configured by SIB1, MCS 0~7 are applied if the configuration is absent. The first 4 indexes of the 8 candidate MCS indexes are used for initial PUSCH transmission scheduled by RAR UL grant. |

Conclusion

- Same Tx beam is applied for all repetitions of Msg3 initial transmission scheduled by a RAR UL grant.

  - It is up to UE implementation how the Tx beam forms.

- Same Tx beam is applied for all repetitions of Msg3 re-transmission scheduled by a DCI scrambled with TC-RNTI.

  - It is up to UE implementation how the Tx beam forms.

Agreement

Adopt the following text proposal for Clause 7.3.1.1.1 in TS 38.212 h00.

7.3.1.1.1 Format 0_0 **Unchanged parts are omitted** The following information is transmitted by means of the DCI format 0_0 with CRC scrambled by TC-RNTI: - Identifier for DCI formats – 1 bit - The value of this bit field is always set to 0, indicating an UL DCI format - Frequency domain resource assignment – number of bits determined by the following: - bits if the higher layer parameter useInterlacePUCCH-PUSCH in BWP-UplinkCommon is not configured, where - is the size of the initial UL bandwidth part. - For PUSCH hopping with resource allocation type 1: - MSB bits are used to indicate the frequency offset according to Table 8.3-1 in Clause 8.3 of [5, TS 38.213], where if and otherwise - bits provide the frequency domain resource allocation according to Clause 6.1.2.2.2 of [6, TS 38.214] - For non-PUSCH hopping with resource allocation type 1: - bits provide the frequency domain resource allocation according to Clause 6.1.2.2.2 of [6, TS 38.214] - If the higher layer parameter useInterlacePUCCH-PUSCH in BWP-UplinkCommon is configured - 5 bits provide the frequency domain resource allocation according to Clause 6.1.2.2.3 of [6, TS 38.214] if the subcarrier spacing for the active UL bandwidth part is 30 kHz - 6 bits provide the frequency domain resource allocation according to Clause 6.1.2.2.3 of [6, TS 38.214] if the subcarrier spacing for the active UL bandwidth part is 15 kHz - Time domain resource assignment – 4 bits as defined in Clause 6.1.2.1 of [6, TS 38.214] - Frequency hopping flag – 1 bit according to Table 7.3.1.1.1-3, as defined in Clause 6.3 of [6, TS 38.214] - Modulation and coding scheme – 5 bits as defined in Clause 6.1.4.1 of [6, TS 38.214] - If the UE requests repetition of PUSCH scheduled by RAR UL grant [11, TS 38.321], 5 bits as defined in Clause 6.1.2.1 and Clause 6.1.4.1 of [6, TS 38.214], - else, 5 bits as defined in Clause 6.1.4.1 of [6, TS 38.214]. - New data indicator – 1 bit, reserved - Redundancy version – 2 bits as defined in Table 7.3.1.1.1-2 - HARQ process number – 4 bits, reserved - TPC command for scheduled PUSCH – 2 bits as defined in Clause 7.1.1 of [5, TS 38.213] - ChannelAccess-CPext – 2 bits indicating combinations of channel access type and CP extension as defined in Table 7.3.1.1.1-4, or Table 7.3.1.1.1-4A if ChannelAccessMode-r16 = "semistatic" is provided, for operation in a cell with shared spectrum channel access; 0 bit otherwise - Padding bits, if required. - UL/SUL indicator – 1 bit if the cell has two ULs and the number of bits for DCI format 1_0 before padding is larger than the number of bits for DCI format 0_0 before padding; 0 bit otherwise. The UL/SUL indicator, if present, locates in the last bit position of DCI format 0_0, after the padding bit(s). - If 1 bit, reserved, and the corresponding PUSCH is always on the same UL carrier as the previous transmission of the same TB **Unchanged parts are omitted**

Agreement

Adopt the following text proposal for Clause 6.1.4.1 in TS 38.214 h00

#### 6.1.4.1 Modulation order and target code rate determination For a PUSCH scheduled by RAR UL grant or for a PUSCH scheduled by a fallbackRAR UL grant or for a MsgA PUSCH transmission, or for a PUSCH scheduled by a DCI format 0_0 with CRC scrambled by C-RNTI, MCS-C-RNTI, TC-RNTI, CS-RNTI, or for a PUSCH scheduled by a DCI format 0_1 or DCI format 0_2 with CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, SP-CSI-RNTI, or for a PUSCH with configured grant using CS-RNTI, and if transform precoding is disabled for this PUSCH transmission according to Clause 6.1.3 - if mcs-TableDCI-0-2 in pusch-Config is set to 'qam256', and PUSCH is scheduled by a PDCCH with DCI format 0_2 with CRC scrambled by C-RNTI or SP-CSI-RNTI, - the UE shall use IMCS and Table 5.1.3.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel; - elseif the UE is not configured with MCS-C-RNTI, mcs-TableDCI-0-2 in pusch-Config is set to 'qam64LowSE', and the PUSCH is scheduled by a PDCCH by a PDCCH with DCI format 0_2 with CRC scrambled by C-RNTI or SP-CSI-RNTI, - the UE shall use IMCS and Table 5.1.3.1-3 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel. - elseif mcs-Table in pusch-Config is set to 'qam256', and PUSCH is scheduled by a PDCCH with DCI format 0_1 with CRC scrambled by C-RNTI or SP-CSI-RNTI, - the UE shall use IMCS and Table 5.1.3.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel. - elseif the UE is not configured with MCS-C-RNTI, mcs-Table in pusch-Config is set to 'qam64LowSE', and the PUSCH is scheduled by a PDCCH with a DCI format other than DCI format 0_2 in a UE-specific search space with CRC scrambled by C-RNTI or SP-CSI-RNTI, - the UE shall use IMCS and Table 5.1.3.1-3 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel. - elseif the UE is configured with MCS-C-RNTI, and the PUSCH is scheduled by a PDCCH with CRC scrambled by MCS-C-RNTI, - the UE shall use IMCS and Table 5.1.3.1-3 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel. - elseif mcs-Table in configuredGrantConfig is set to 'qam256', - if PUSCH is scheduled by a PDCCH with CRC scrambled by CS-RNTI or - if PUSCH is transmitted with configured grant - the UE shall use IMCS and Table 5.1.3.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel. - elseif mcs-Table in configuredGrantConfig is set to 'qam64LowSE', - if PUSCH is scheduled by a PDCCH with CRC scrambled by CS-RNTI or - if PUSCH is transmitted with configured grant, - the UE shall use IMCS and Table 5.1.3.1-3 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel. - elseif for a MsgA PUSCH transmission, - the UE shall use higher layer parameter msgA-MCS for IMCS and Table 5.1.3.1-1 to determine the Target code rate (R) used in the physical uplink shared channel. - elseif for PUSCH repetition type A the UE requests repetition of PUSCH scheduled by RAR UL grant [11, TS 38.321], when transmitting PUSCH scheduled by RAR UL grant, - the 2 LSBs of the MCS information field of the RAR UL grant provide a codepoint to determine the MCS index IMCS according to Table 6.1.4.1-3, based on whether or not the higher layer parameter mcs-Msg3Repetition is configured. The UE shall use the determined IMCS and Table 5.1.3.1-1 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel. - elseif the UE requests repetition of PUSCH scheduled by RAR UL grant [11, TS 38.321], when transmitting PUSCH scheduled by DCI format 0_0 with CRC scrambled by the TC-RNTI, - the 3 LSBs of the MCS information field of the DCI format 0_0 with CRC scrambled by the TC-RNTI provide a codepoint to determine the MCS index IMCS according to Table 6.1.4.1-4, based on whether or not the higher layer parameter mcs-Msg3Repetition is configured. The UE shall use the determined IMCS and Table 5.1.3.1-1 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel. - else - the UE shall use IMCS and Table 5.1.3.1-1 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel. else - if mcs-TableTransformPrecoderDCI-0-2 in pusch-Config is set to 'qam256', and PUSCH is scheduled by a PDCCH with DCI format 0_2 with CRC scrambled by C-RNTI or SP-CSI-RNTI, - the UE shall use IMCS and Table 5.1.3.1.-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel. - elseif the UE is not configured with MCS-C-RNTI, mcs-TableTransformPrecoderDCI-0-2 in pusch-Config is set to 'qam64LowSE', and the PUSCH is scheduled by a PDCCH with DCI format 0_2 with CRC scrambled by C-RNTI or SP-CSI-RNTI, - the UE shall use IMCS and Table 6.1.4.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel. - elseif mcs-TableTransformPrecoder in pusch-Config is set to 'qam256', and PUSCH is scheduled by a PDCCH with DCI format 0_1 with CRC scrambled by C-RNTI or SP-CSI-RNTI, - the UE shall use IMCS and Table 5.1.3.1.-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel. - elseif the UE is not configured with MCS-C-RNTI, mcs-TableTransformPrecoder in pusch-Config is set to 'qam64LowSE', and the PUSCH is scheduled by a PDCCH with a DCI format other than DCI format 0_2 in a UE-specific search space with CRC scrambled by C-RNTI or SP-CSI-RNTI, - the UE shall use IMCS and Table 6.1.4.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel. - elseif the UE is configured with MCS-C-RNTI, and the PUSCH is scheduled by a PDCCH with CRC scrambled by MCS-C-RNTI, - the UE shall use IMCS and Table 6.1.4.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel. - elseif mcs-TableTransformPrecoder in configuredGrantConfig is set to 'qam256', - if PUSCH is scheduled by a PDCCH with CRC scrambled by CS-RNTI or - if PUSCH is transmitted with configured grant, - the UE shall use IMCS and Table 5.1.3.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel. - elseif mcs-TableTransformPrecoder in configuredGrantConfig is set to 'qam64LowSE', - if PUSCH is scheduled by a PDCCH with CRC scrambled by CS-RNTI or - if PUSCH is transmitted with configured grant, - the UE shall use IMCS and Table 6.1.4.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel. - elseif for a MsgA PUSCH transmission, - the UE shall use higher layer parameter MsgA-MCS for IMCS and Table 6.1.4.1-1 to determine the Target code rate (R) used in the physical uplink shared channel. - the UE shall use q=2 for determining modulation order Qm in Table 6.1.4.1-1. - elseif for PUSCH repetition type A the UE requests repetition of PUSCH scheduled by RAR UL grant [11, TS 38.321], when transmitting PUSCH scheduled by RAR UL grant, - the 2 LSBs of the MCS information field of the RAR UL grant provide a codepoint to determine the MCS index IMCS according to Table 6.1.4.1-3, based on whether or not the higher layer parameter mcs-Msg3Repetition is configured. The UE shall use the determined IMCS and Table 6.1.4.1-1 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel. - elseif the UE requests repetition of PUSCH scheduled by RAR UL grant [11, TS 38.321], when transmitting PUSCH scheduled by DCI format 0_0 with CRC scrambled by the TC-RNTI, - the 3 LSBs of the MCS information field of the DCI format 0_0 with CRC scrambled by the TC-RNTI provide a codepoint to determine the MCS index IMCS according to Table 6.1.4.1-4, based on whether or not the higher layer parameter mcs-Msg3Repetition is configured. The UE shall use the determined IMCS and Table 6.1.4.1-1 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel. - else - the UE shall use IMCS and Table 6.1.4.1-1to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel. end <Unchanged parts are omitted> Table 6.1.4.1-3: MCS index IMCS as a function of 2 LSBs of MCS information field in RAR UL grant mcs-Msg3Repetition is configured; mcs-Msg3Repetitions is not configured. Codepoint; IMCS; Codepoint; IMCS. 00; First value of mcs-Msg3Repetition; 00; 0. 01; Second value of mcs-Msg3Repetition; 01; 1. 10; Third value of mcs-Msg3Repetition; 10; 2. 11; Fourth value of mcs-Msg3Repetition; 11; 3. Table 6.1.4.1-4: MCS index IMCS as a function of 3 LSBs of MCS information field in DCI format 0_0 with CRC scrambled by the TC-RNTI mcs-Msg3Repetition is configured; mcs-Msg3Repetitions is not configured. Codepoint; IMCS; Codepoint; IMCS. 000; First value of mcs-Msg3Repetition; 000; 0. 001; Second value of mcs-Msg3Repetition; 001; 1. 010; Third value of mcs-Msg3Repetition; 010; 2. 011; Fourth value of mcs-Msg3Repetition; 011; 3. 100; Fifth value of mcs-Msg3Repetition; 100; 4. 101; Sixth value of mcs-Msg3Repetition; 101; 5. 110; Seventh value of mcs-Msg3Repetition; 110; 6. 111; Eighth value of mcs-Msg3Repetition; 111; 7. <Unchanged parts are omitted>

Agreement

Adopt the following text proposal for Clause 8.3 in TS38.213 h00

8.3 PUSCH scheduled by RAR UL grant <Unchanged parts are omitted> A UE can be provided in RACH-ConfigCommon a set of numbers of repetitions for a PUSCH transmission with PUSCH repetition Type A that is scheduled by a RAR UL grant or by a DCI format 0_0 with CRC scrambled by a TC-RNTI. If the UE requests repetition of PUSCH scheduled by RAR UL grant [11, TS 38.321], Tthe UE transmits repeats the PUSCH transmission over NPUSCHrepeat slots, where NPUSCHrepeat is indicated by the 2 MSBs of the MCS field in the RAR UL grant or in the DCI format 0_0 with CRC scrambled by the TC-RNTI, and determines a redundancy version and RBs for each repetition as described in [6, TS 38.214]. For unpaired spectrum operation, the UE determines the NPUSCHrepeat slots as the first NPUSCHrepeat slots starting from slot n+k2+? where a repetition of the PUSCH transmission does not include a symbol indicated as downlink by tdd-UL-DL-ConfigurationCommon or indicated as a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst. <Unchanged parts are omitted>

RAN1 #108-e

Agreement

Final LS R1-2202867 is endorsed.

Enhancements on PUSCH repetition type A:

Agreement

The TP#B’’ (in R1-2202567) to TS38.214 is endorsed

Agreement

- For DG-PUSCH repetition Type A scheduled by DCI format 0_1 or 0_2, the legacy counting method applies when K=1, regardless of whether AvailableSlotCounting is enabled or not

  - Note: The legacy assumption on the K2 offset is applied, i.e., no RAN1 spec impact on the K2 offset is expected.

- For CG-PUSCH repetition Type A,  the legacy counting method applies when K=1, regardless of whether AvailableSlotCounting is enabled or not.

  - Note: No RAN1 spec impact is expected.

  - Note: No additional restriction on CG periodicity configurations, compared to Rel-16, is considered for CG-PUSCH with K=1.

- For CG-PUSCH repetition Type A, when AvailableSlotCounting is enabled, and for K>1, a UE assumes that the slot (i.e., the first configured slot in a CG period) determined in 38.321 Section 5.8.2 can be the slot which is not counted in K available slots.

  - Note: No RAN1 spec impact is expected.

TB processing over multi-slot PUSCH:

Agreement:

For the bit selection of TBoMS:

- G in Clause 5.4.2.1 [and/or Clause 6.2.6] of TS 38.212 is redefined for TBoMS as the total number of coded bits available for transmission of the transport block in a slot.

- Note: no intention to change the legacy behavior

For determining the index of the starting coded bit in the circular buffer for the n-th slot of a single TBoMS, RAN1 to down-select one of the following two options during RAN1 #108-e.

Option 1 The index of the starting coded bit in the circular buffer for the n-th slot of a single TBoMS, i.e., sn, is calculated as sn=k0n=0(sn-1+H+τn-1)mod Ncbn=1, …,N-1 Where: - ko is given by Table 5.4.2.1-2. - N is the number of slots allocated for TBoMS - Ncb is the length of circular buffer - H is the total number of coded bits available for transmission of the TB in a slot allocated for TBoMS, assuming no UCI multiplexing - τn-1 is the number of filler bits that would be skipped in the bit selection step, assuming no UCI multiplexing, in the n-1-th slot allocated for TBoMS, if any. Note: this equation describes the logic of the bit-selection for TBoMS; decision on where and how to capture this in TS 38.212 is up to the Editor.

Option 2 Adopt the following TP for TS 38.212 #### 5.4.2.1 Bit selection <Unchanged parts omitted> Denote by the redundancy version number for this transmission ( = 0, 1, 2 or 3), the rate matching output bit sequence , , is generated as follows, where is given by Table 5.4.2.1-2 according to the value of and LDPC base graph if numberOfSlotsTBoMS is not present in the resource allocation table or the value of numberOfSlotsTBoMS in the row indicated by the Time domain resource assignment field in DCI is 1, otherwise for the first allocated slot, that is the 0-th slot, is given by Table 5.4.2.1-2 according to the value of and LDPC base graph and [] for n-th slot, n=1,2,…numberOfSlotsTBoMS-1, is the bit next to the last selected bit by the bit selection in the previous slot assuming the UCI is not multiplexed:

Agreement:

The index of the starting coded bit in the circular buffer for the n-th slot of a single TBoMS, i.e., sn, is calculated as

sn=k0n=0(sn-1+H+τn-1)mod Ncbn=1, …,N-1

Where:

- k0 is given by Table 5.4.2.1-2.

- N is the number of slots allocated for TBoMS

- Ncb is the length of circular buffer

- H is the total number of coded bits available for transmission of the TB in a slot allocated for TBoMS, assuming no UCI multiplexing

- τn-1 is the number of filler bits that would be skipped in the bit selection step, assuming no UCI multiplexing, in the n-1-th slot allocated for TBoMS, if any.

Note: this equation describes the logic of the determining the starting coded bit in bit selection for TBoMS slot; decision on where and how to capture this or wording that is equivalent in TS 38.212 is up to the Editor.

Conclusion:

There is no consensus in RAN1 to support TBoMS for CG-Type 1.

Conclusion:

The decision on whether to modify the definition of G in Clause 6.2.6 of TS 38.212 such that this parameter is described as the total number of coded bits available for transmission of the transport block in a slot is left to the Editor’s discretion.

Joint channel estimation for PUSCH:

Agreement:

- For HD-FDD RedCap UEs configured with DMRS bundling, an event is constituted for a case where a dropping or cancellation of a PUSCH transmission according dropping rules in [17.2, TS 38.213].

Agreement:

- For HD-FDD RedCap UEs configured with DMRS bundling, an event is constituted for a case where the gap between two consecutive PUSCH transmissions overlaps with any symbol of downlink reception or downlink monitoring even if neither of the repetitions overlaps with it.

Conclusion:

   PUSCH repetitions with different sets of power control parameters in multi-TRP operation is regarded as a semi-static event.

Note: No additional specification impact is expected

Agreement:

- Gap between two PUSCH/PUCCH transmissions exceeds 11 symbols for extended CP is regarded as an event.

Agreement:

- The value range of PUSCH-TimeDomainWindowLength is INTEGER (2 .. 32).

- The value range of PUCCH-TimeDomainWindowLength is INTEGER (2 .. 8).

- Note: the value shall not exceed the maximum duration.

PUCCH enhancements:

Agreement:

Inter-slot frequency hopping pattern for PUSCH repetitions with DMRS bundling is determined based on physical slot index

Agreement:

Inter-slot frequency hopping pattern for PUCCH repetitions with DMRS bundling is determined based on down-selection (in RAN1#108e) option 2

- Option 2: relative slot index – option A

Note: Option A/B for relative slot index are defined as the following

- Option A: frequency hopping pattern is determined based on relative slot index. Relative slot 0 is the slot where PUCCH/PUSCH repetition starts. Each of the subsequent slots are counted and relative slot index increase by one regardless the slot is used to transmit PUCCH/PUSCH or not.

- Option B: frequency hopping pattern is determined based on relative slot index. Relative slot 0 is the slot where PUCCH/PUSCH repetition starts. One of the subsequent slots are counted and the slot index increase by one only if the slot is used to transmit PUCCH/PUSCH.

Conclusion:

For frequency hopping for PUCCH/PUSCH repetitions with DMRS bundling, in Rel-17, there is no consensus to increase the number of frequency offset over what are supported in Rel-15/16.

Agreement (Excerpted from RAN1#107bis-e):

PUCCH repetitions with different sets of power control parameters in multi-TRP operation should be regarded as a [semi-static] event that causes power consistency and phase continuity not to be maintained across PUCCH repetitions.

Agreement:

Update the above agreement made in RAN1 107bis-e as below

PUCCH repetitions with different sets of power control parameters in multi-TRP operation should be regarded as a semi-static event that causes power consistency and phase continuity not to be maintained across PUCCH repetitions.

Agreement:

- Values for PUSCH-Frequencyhopping-Interval are “{2, 4, 5, 6, 8, 10, 12, 14, 16, 20}”

Note: For unpaired spectrum, UE is not expected to be configured the value of 6, 8, 12, 14, 16

Agreement:

- Values for PUCCH-Frequencyhopping-Interval are “{2, 4, 5, 10}”

Agreement:

When both inter-slot frequency hopping and DMRS bundling are enabled for PUCCH/PUSCH repetitions, UE is expected to be configured with at least one of frequency hopping interval and TDW length.

Note: Conclusion is to added to RRC parameter agreement and sent to RAN2 in LS.

Type A PUSCH repetitions for Msg3:

Agreement (Excerpted from RAN1#107bis-e):

Regarding how a UE should interpret MCS information field for indication of the number of repetitions for the case of CBRA, Option 1 is supported.

- When a UE requests Msg3 repetition, the repurposed MCS information field is applied. gNB schedules Msg3 with or without repetition for the UE requesting Msg3 repetition.

  - Repetition factor K=1 is included in the four candidate repetition factors used for repetition indication.

- When the UE doesn’t request Msg3 repetition (including legacy UE), the legacy MCS information field is applied. gNB schedules Msg3 without repetition for the UE not requesting Msg3 repetition.

Agreement:

The above agreement (Excerpted from RAN1#107bis-e) is updated as follows.

Regarding how a UE should interpret MCS information field for indication of the number of repetitions for the case of CBRA, Option 1 is supported. - When a UE requests Msg3 repetition, the repurposed MCS information field is applied. gNB schedules Msg3 with or without repetition for the UE requesting Msg3 repetition. - Whether Rrepetition factor K=1 is included in the four candidate repetition factors used for repetition indication is up to gNB configuration. - When the UE doesn’t request Msg3 repetition (including legacy UE), the legacy MCS information field is applied. gNB schedules Msg3 without repetition for the UE not requesting Msg3 repetition.

Agreement:

The Rel-17 collision rules among SSB and Msg3 PUSCH transmission for HD-FDD UEs, which is subject to the discussion on collision handling for Msg3 transmission in AI 8.6.1.2, can be reused for Msg3 PUSCH repetition for HD-FDD UEs.

Agreement:

- RAN1 confirms to support separate RO for requesting of Msg3 PUSCH repetition in a BWP configured with ROs not for requesting of Msg3 PUSCH repetition.

- Note: It expects RAN2 will continue the work on RACH partitioning based on separate RO for Rel-17 CE, and no further action in RAN1 on this aspect is pursued unless triggered by RAN2.

Agreement:

[Draft] Reply LS on UL BWP with PRACH resources only for RACH with Msg3 repetition, R1-220xxxx (Reply LS_v02), is endorsed in principle

Agreement:

Final LS on UL BWP with PRACH resources only for RACH with Msg3 repetition, R1-2202829, is endorsed.

Agreement:

The following TP for Clause 6.1.2.1 of TS 38.214 h00 is endorsed.

6.1.2.1 Resource allocation in time domain <Unchanged parts are omitted> For PUSCH repetition Type A, when transmitting PUSCH scheduled by DCI format 0_1 or 0_2 in PDCCH with CRC scrambled with C-RNTI, MCS-C-RNTI, or CS-RNTI with NDI=1, the number of repetitions K is determined as - if numberOfRepetitions is present in the resource allocation table, the number of repetitions K is equal to numberOfRepetitions; - elseif the UE is configured with pusch-AggregationFactor, the number of repetitions K is equal to pusch-AggregationFactor; - otherwise K=1. - the number of slots used for TBS determination N is equal to 1. For PUSCH repetition type A, when transmitting PUSCH scheduled by RAR UL grant, the 2 MSBs of the MCS information field of the RAR UL grant provide a codepoint to determine the number of repetitions K according to Table 6.1.2.1-1A, based on whether or not the higher layer parameter numberOfMsg3Repetitions is configured. The number of slots used for TBS determination N is equal to 1. For PUSCH repetition type A, when transmitting PUSCH scheduled by DCI format 0_0 with CRC scrambled by TC-RNTI, the 2 MSBs of the MCS information field of the DCI format 0_0 with CRC scrambled by TC-RNTI provide a codepoint to determine the number of repetitions K according to Table 6.1.2.1-1A, based on whether or not the higher layer parameter numberOfMsg3Repetitions is configured. The number of slots used for TBS determination N is equal to 1. <Unchanged parts are omitted>

2.1.2	Remaining Open issues

2.2	RAN2

2.2.1	Agreements

RAN2#116bis-e

- Non-CE to CE fallback is not supported in Rel-17.

- UE cannot fallback from CFRA or 2-step RACH to CE-RACH.

- CE-capable UEs use the legacy threshold, rsrp-ThresholdSSB-SUL, in its selection of UL carrier for RACH.

- Msg3 repetition is modelled in the same way as dynamically scheduled bundles in the MAC spec.

- From CE’s perspective, it does not matter whether UE first selects RA type or CE when initiating a RACH procedure.

- From CE’s perspective, it is confirmed that the eligibility criteria for CE is determined before the selection of RACH partition.

- From CE’s perspective, the RSRP threshold for requesting Msg3 repetition can be configured per BWP on both NUL and SUL.

- When CE is configured in RACH partitions, the configuration granularity for the RSRP threshold for requesting Msg3 repetition should be decided by the common RACH session.

- From CE’s perspective, CE RACH can be configured with a separate RSRP threshold for SSB selection and this threshold can be configured per BWP.

- When CE is configured in RACH partitions, the configuration granularity for the RSRP threshold for SSB selection in a CE RACH procedure should be decided by the common RACH session.

- For now there is no consensus in RAN2 on whether to support CFRA with msg3 repetition or not, based on the assumption that RAN1 should discuss this first.

- LS on BWP with only CE RACH resources R2-2201982 is approved.

- MAC running CR R2-2201966 is noted and will be used as a baseline for discussion at RAN2#117-e.

- RRC running CR R2-2201964 is noted and will be used as a baseline for discussion at RAN2#117-e.

- Stage 2 running CR R2-2201963 is endorsed and attached in the LS R2-2201784 sent to RAN1 for RAN1’s check.

Working assumption:

- From RAN2’s perspective, a dedicated UL BWP can be configured with only CE RACH resources. Its feasibility is to be confirmed by RAN1.

RAN2#117-e

- On ra-ContentionResolutionTimer, the text proposal in current MAC CR is agreeable， further update is not needed.

- If RAN1 confirms the feasibility of dedicated BWP configured with only CE RACH resources, in this case, the RSRP threshold for requesting Msg3 repetition will not be configured, so RedCap UEs can only trigger CE RACH in this BWP, no need to perform CE vs non-CE selection.

- For Proposal 2, to capture the configuration restriction in field description of CE selection RSRP threshold. E.g. “the field is only present if both CE and non-CE RACH resources are configured for the BWP” (will be implemented in RIP RRC CR).

- During a measurement gap, the MAC entity can transmit all repetitions of the Msg3 transmission on UL-SCH (no spec impact).

2.2.2	Remaining Open issues

2.3	RAN3

2.3.1	Agreements

2.3.2	Remaining Open issues

2.4	RAN4

2.4.1	Agreements

The progress in RAN4 #101e-bis meeting is summarized below:

RAN4 UE RF part:

- The email discussion summary for [101-bis-e][130] NR_cov_enh was provided in R4-2202330.

- The RAN4 RF work plan for NR coverage enhancements WI was updated in R4-2200020.

- The reply LS on Maximum duration for DMRS bundling was approved in R4-2202368.

- The WF on phase continuity and power consistency for PUCCH and PUSCH transmissions was approved in R4-2202418.

RAN4 BS demodulation part:

Following Way Forwards are approved:

- WF on PUSCH demodulation performance of Rel-17 NR coverage enhancement was approved in R4-2203030.

- WF on PUCCH demodulation performance of Rel-17 NR coverage enhancement was approved in R4-2203031.

Following agreements are reached regarding the general test scope:

- Agree to define BS demodulation requirements for PUSCH TBoMS.

- Agree to define BS PUSCH demodulation requirements with JCE, while the detailed parameters should be set following RAN1 and RAN4 RF agreement.

- Agree not to define BS demodulation requirements for dynamic indication of PUCCH repetition.

The progress in RAN4 #102e meeting is summarized below:

RAN4 UE RF part:

- The email discussion summary for [102-e][136] NR_cov_enh was provided in R4-2206436.

- The RAN4 RF work plan for NR coverage enhancements WI was updated in R4-2203818.

- Two reply LSs on Maximum duration for DMRS bundling were approved in R4-2206537 and R4-2206580.

- CRs on UE RF requirements for DMRS bundling for TS 38.101-1 and TS 38.101-2 were agreed in R4-2206538 and R4-2206539 respectively.

- CRs on measurement for DMRS bundling for TS 38.101-1 and TS 38.101-2 were endorsed in R4-2206541 and R4-2206542 respectively, and agreed in R4-2206640 and R4-2206641 respectively.

- WF on issues for maintenance of NR coverage enhancements was approved R4-2206593.

RAN4 BS demodulation part:

Following Way Forwards are approved:

- WF on PUSCH demodulation performance of Rel-17 NR coverage enhancement was approved in R4-2207210.

- WF on PUCCH demodulation performance of Rel-17 NR coverage enhancement was approved in R4-2207211.

Following agreements are reached regarding the general test scope:

- Agree not to define BS demodulation requirements for PUSCH repetition type A with 32 repetitions.

- Agree not to consider type A PUSCH repetition for Msg 3 with inter-slot frequency hopping requirement.

- Agree not to define new BS demodulation requirement for NR coverage enhancement for Redcap UE.

- Agree to introduce BS PUCCH demodulation requirements for NR coverage enhancement with JCE with 2Rx only (As baseline assumption).

2.4.2	Remaining Open issues

- Finalization of BS demodulation and conformance testing requirements.

2.5	RAN5

2.5.1	Agreements

2.5.2	Remaining Open issues

2.5.3	Remaining Open issues with cross-WG dependencies

2.6	RAN6

2.6.1	Agreements

2.6.2	Remaining Open issues

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

3.1	SAx/CTs

3.1.1	Agreements with cross-TSG impacts

3.1.2	Remaining Open issues with cross-TSG impacts

4.	References

RAN1 #107b-e:

- R1-2200051	Discussion on coverage enhancements for PUSCH repetition type A	Huawei, HiSilicon

- R1-2200052	Discussion on TB processing over multi-slot PUSCH	Huawei, HiSilicon

- R1-2200053	Discussion on joint channel estimation for PUSCH	Huawei, HiSilicon

- R1-2200054	Discussion on PUCCH coverage enhancement	Huawei, HiSilicon

- R1-2200055	Discussion on Msg3 repetition for coverage enhancement	Huawei, HiSilicon

- R1-2200086	Remaining issues on enhancement for PUSCH repetition type A	vivo

- R1-2200087	Remaining issues on PUSCH TB processing over multiple slots	vivo

- R1-2200088	Remaining issues on joint channel estimation for PUSCH	vivo

- R1-2200089	Remaining issues on PUCCH enhancements	vivo

- R1-2200090	Remaining issues on Type A PUSCH repetitions for Msg3	vivo

- R1-2200111	Discussion on remaining issues for enhanced PUSCH repetition type A	ZTE

- R1-2200112	Discussion on remaining issues for TB processing over multi-slot PUSCH	ZTE

- R1-2200113	Discussion on remaining issues for joint channel estimation for PUSCH	ZTE

- R1-2200114	Discussion on remaining issues for coverage enhancements for PUCCH	ZTE

- R1-2200115	Discussion on remaining issues for Type A PUSCH repetitions for Msg3	ZTE

- R1-2200116	Discussion on RRC parameters for coverage enhancement	ZTE

- R1-2200152	Remaining issues on TB processing over multi-slot PUSCH	CATT

- R1-2200153	Remaining issues on PUCCH enhancements	CATT

- R1-2200154	Remaining issues on Type A PUSCH repetitions for Msg3	CATT

- R1-2200160	Enhancements on PUSCH repetition type A	Nokia, Nokia Shanghai Bell

- R1-2200161	Transport block processing for PUSCH coverage enhancements	Nokia, Nokia Shanghai Bell

- R1-2200162	Joint channel estimation for PUSCH coverage enhancements	Nokia, Nokia Shanghai Bell

- R1-2200163	PUCCH coverage enhancements	Nokia, Nokia Shanghai Bell

- R1-2200164	Approaches and solutions for Type A PUSCH repetitions for Msg3	Nokia, Nokia Shanghai Bell

- R1-2200165	Performance of Type A PUSCH repetitions for Msg3 for different MCS index configurations	Nokia, Nokia Shanghai Bell

- R1-2200205	Enhancements on PUSCH repetition type A	Samsung

- R1-2200206	TB processing over multi-slot PUSCH	Samsung

- R1-2200207	Joint channel estimation for PUSCH	Samsung

- R1-2200208	PUCCH enhancements	Samsung

- R1-2200209	Type A PUSCH repetitions for Msg3	Samsung

- R1-2200237	TB processing over multi-slot PUSCH	NTT DOCOMO, INC.

- R1-2200238	Joint channel estimation for PUSCH	NTT DOCOMO, INC.

- R1-2200239	PUCCH enhancements for coverage enhancement	NTT DOCOMO, INC.

- R1-2200240	Type A PUSCH repetitions for Msg3	NTT DOCOMO, INC.

- R1-2200268	Discussion on joint channel estimation for PUSCH	Panasonic Corporation

- R1-2200279	Discussion on joint channel estimation for PUSCH	Spreadtrum Communications

- R1-2200280	Discussion on PUCCH enhancements	Spreadtrum Communications

- R1-2200301	Enhancements on PUSCH Repetition Type A	Qualcomm Incorporated

- R1-2200302	TB processing over multi-slot PUSCH	Qualcomm Incorporated

- R1-2200303	Joint channel estimation for PUSCH	Qualcomm Incorporated

- R1-2200304	PUCCH enhancements	Qualcomm Incorporated

- R1-2200305	Type A PUSCH repetition for Msg3	Qualcomm Incorporated

- R1-2200321	Discussion on TB processing over multi-slot PUSCH	Panasonic Corporation

- R1-2200322	Discussion on the interaction between inter-slot frequency hopping and DMRS bundling	Panasonic Corporation

- R1-2200323	Discussion on Type A PUSCH repetitions for Msg.3	Panasonic Corporation

- R1-2200334	Enhancements on PUSCH repetition type A	OPPO

- R1-2200335	Further considerations for TB over multi-slot PUSCH	OPPO

- R1-2200336	Consideration on Joint channel estimation for PUSCH	OPPO

- R1-2200337	PUCCH enhancements for coverage	OPPO

- R1-2200338	Type A PUSCH repetitions for Msg3 coverage	OPPO

- R1-2200379	Remaining details of enhancements on PUSCH repetition type A	Intel Corporation

- R1-2200380	Remaining details on TB processing over multi-slot PUSCH	Intel Corporation

- R1-2200381	Remaining details on joint channel estimation for PUSCH	Intel Corporation

- R1-2200382	Remaining details on PUCCH enhancements	Intel Corporation

- R1-2200383	Remaining details on Msg3 PUSCH repetition	Intel Corporation

- R1-2200420	Discussion on PUSCH repetition type A enhancement	Apple

- R1-2200421	Discussion on TB processing over multi-slot PUSCH	Apple

- R1-2200422	Discussion on joint channel estimation for PUSCH	Apple

- R1-2200423	Further discussion on PUCCH coverage enhancement	Apple

- R1-2200424	On remaining issues for Msg3 Coverage Enhancement	Apple

- R1-2200466	Discussion on TB processing over multi-slot PUSCH	xiaomi

- R1-2200467	Discussion on joint channel estimation for PUSCH	xiaomi

- R1-2200468	Discussion on PUCCH enhancements	xiaomi

- R1-2200469	Discussion on Type A PUSCH repetition for Msg3	xiaomi

- R1-2200470	Other considerations for coverage enhancement	xiaomi

- R1-2200486	Remaining issues on joint channel estimation	China Telecom

- R1-2200487	FL Summary of joint channel estimation for PUSCH	Moderator (China Telecom)

- R1-2200488	Remaining issues on inter-slot frequency hopping with inter-slot bundling for PUCCH and PUSCH	China Telecom

- R1-2200489	Remaining issues on type A PUSCH repetitions for Msg3	China Telecom

- R1-2200499	Joint channel estimation for PUSCH	Sharp

- R1-2200500	Enhancements on PUSCH repetition type A	Sharp

- R1-2200501	TB processing over multi-slot PUSCH	Sharp

- R1-2200502	PUCCH coverage enhancement	Sharp

- R1-2200503	Type A PUSCH repetitions for Msg3	Sharp

- R1-2200509	Discussion on remaining issue of TBoMS	NEC

- R1-2200518	Type-A PUSCH repetition for coverage enhancement	InterDigital, Inc.

- R1-2200519	TB processing over multiple slots	InterDigital, Inc.

- R1-2200520	Discussions on joint channel estimation	InterDigital, Inc.

- R1-2200521	Discussions on PUCCH enhancements	InterDigital, Inc.

- R1-2200522	Type A PUSCH repetitions for Msg3	InterDigital, Inc.

- R1-2200553	Discussion on Joint channel estimation over multi-slot	MediaTek Inc.

- R1-2200563	Remaining issues on RRC parameters for NR coverage enhancement	vivo

- R1-2200588	Remaining issues on PUSCH repetition type A enhancement	CMCC

- R1-2200589	Remaining issues on TB processing over multi-slot PUSCH	CMCC

- R1-2200590	Remaining issues on joint channel estimation for PUSCH	CMCC

- R1-2200591	Remaining issues on PUCCH enhancements	CMCC

- R1-2200592	Remaining issues on type A PUSCH repetitions for Msg3	CMCC

- R1-2200604	Discussion on TBoMS PUSCH	TCL Communication Ltd.

- R1-2200613	Discussions on joint channel estimation for PUSCH	LG Electronics

- R1-2200614	Discussions on coverage enhancement for PUCCH	LG Electronics

- R1-2200615	Discussion on coverage enhancement for Msg3 PUSCH	LG Electronics

- R1-2200636	Remaining issues on PUCCH enhancements for coverage enhancement	WILUS Inc.

- R1-2200637	Remaining issues on Type A PUSCH repetitions for Msg3	WILUS Inc.

- R1-2200653	Other considerations for coverage enhancement	Huawei, HiSilicon

- R1-2200655	Remaining Issues for PUSCH Repetition Type A Enhancement	Ericsson

- R1-2200656	Remaining Issues for TB Processing over Multi-Slot PUSCH	Ericsson

- R1-2200657	Remaining Issues for Joint Channel Estimation for PUSCH	Ericsson

- R1-2200658	Remaining Issues for PUCCH Dynamic Repetition and DMRS Bundling	Ericsson

- R1-2200659	Remaining Issues for Type A PUSCH Repetition for Msg3	Ericsson

- R1-2200660	Frequency Hopping for TBoMS	Ericsson

- R1-2200676	Joint channel estimation for PUSCH	Samsung

- R1-2200680	FL summary of TB processing over multi-slot PUSCH (AI 8.8.1.2)	Moderator (Nokia)

- R1-2200685	FL Summary #1 on Enhancements on PUSCH repetition type A	Moderator (Sharp)

- R1-2200686	FL Summary #2 on Enhancements on PUSCH repetition type A	Moderator (Sharp)

- R1-2200687	FL Summary #3 on Enhancements on PUSCH repetition type A	Moderator (Sharp)

- R1-2200708	FL summary #1 of PUCCH coverage enhancement	Moderator (Qualcomm)

- R1-2200712	Feature lead summary #1 on support of Type A PUSCH repetitions for Msg3	Moderator (ZTE)

- R1-2200713	Feature lead summary #2 on support of Type A PUSCH repetitions for Msg3	Moderator (ZTE)

- R1-2200714	Feature lead summary #3 on support of Type A PUSCH repetitions for Msg3	Moderator (ZTE)

- R1-2200735	FL Summary#2 of joint channel estimation for PUSCH	Moderator (China Telecom)

- R1-2200736	FL Summary#3 of joint channel estimation for PUSCH	Moderator (China Telecom)

- R1-2200750	FL summary #2 of TB processing over multi-slot PUSCH (AI 8.8.1.2)	Moderator (Nokia/Nokia Shanghai Bell)

- R1-2200751	FL summary #3 of TB processing over multi-slot PUSCH (AI 8.8.1.2)	Moderator (Nokia/Nokia Shanghai Bell)

- R1-2200752	Final FL summary of TB processing over multi-slot PUSCH (AI 8.8.1.2)	Moderator (Nokia/Nokia Shanghai Bell)

- R1-2200754	FL summary #2 of PUCCH coverage enhancement	Moderator (Qualcomm)

- R1-2200773	LS on DMRS bundling for PUSCH and PUCCH	RAN1, China Telecom

- R1-2200774	FL summary #3 of PUCCH coverage enhancement	Moderator (Qualcomm)

- R1-2200789	FL Summary #4 on Enhancements on PUSCH repetition type A	Moderator (Sharp)

- R1-2200790	[107-e-NR-R17-CovEnh-03] Summary of email discussion on joint channel estimation for PUSCH	Moderator (China Telecom)

- R1-2200802	Feature lead summary #4 on support of Type A PUSCH repetitions for Msg3	Moderator (ZTE)

- R1-2200806	Final FL summary of PUCCH coverage enhancement	Moderator (Qualcomm)

RAN1 #108-e:

- R1-2200966	Discussion on coverage enhancements for PUSCH repetition type A	Huawei, HiSilicon

- R1-2200967	Discussion on TB processing over multi-slot PUSCH	Huawei, HiSilicon

- R1-2200968	Discussion on joint channel estimation for PUSCH	Huawei, HiSilicon

- R1-2200969	Discussion on PUCCH coverage enhancement	Huawei, HiSilicon

- R1-2200970	Discussion on Msg3 repetition for coverage enhancement	Huawei, HiSilicon

- R1-2201012	Enhancements on PUSCH repetition type A	Nokia, Nokia Shanghai Bell

- R1-2201013	Transport block processing for PUSCH coverage enhancements	Nokia, Nokia Shanghai Bell

- R1-2201014	Joint channel estimation for PUSCH coverage enhancements	Nokia, Nokia Shanghai Bell

- R1-2201015	PUCCH coverage enhancements	Nokia, Nokia Shanghai Bell

- R1-2201016	Approaches and solutions for Type A PUSCH repetitions for Msg3	Nokia, Nokia Shanghai Bell

- R1-2201104	Remaining issues on enhancement for PUSCH repetition type A	vivo

- R1-2201105	Remaining issues on PUSCH TB processing over multiple slots	vivo

- R1-2201106	Remaining issues on joint channel estimation for PUSCH	vivo

- R1-2201107	Remaining issues on PUCCH enhancements	vivo

- R1-2201108	Remaining issues on Type A PUSCH repetitions for Msg3	vivo

- R1-2201164	Discussion on remaining issues for enhanced PUSCH repetition type A	ZTE

- R1-2201165	Discussion on remaining issues for TB processing over multi-slot PUSCH	ZTE

- R1-2201166	Discussion on remaining issues for joint channel estimation for PUSCH	ZTE

- R1-2201167	Discussion on remaining issues for coverage enhancements for PUCCH	ZTE

- R1-2201168	Discussion on remaining issues for Type A PUSCH repetitions for Msg3	ZTE

- R1-2201169	Discussion on RRC parameters for coverage enhancement	ZTE

- R1-2201283	Enhancements on PUSCH repetition type A	OPPO

- R1-2201284	Further considerations for TB over multi-slot PUSCH	OPPO

- R1-2201285	Consideration on Joint channel estimation for PUSCH	OPPO

- R1-2201286	PUCCH enhancements for coverage	OPPO

- R1-2201287	Type A PUSCH repetitions for Msg3 coverage	OPPO

- R1-2201373	Remaining issues on enhancements on PUSCH repetition type A	CATT

- R1-2201374	Remaining issues on TB processing over multi-slot PUSCH	CATT

- R1-2201375	Remaining issues on joint channel estimation for PUSCH	CATT

- R1-2201376	Remaining issues on PUCCH enhancement	CATT

- R1-2201377	Remaining issues on Type A PUSCH repetitions for Msg3	CATT

- R1-2201380	Discussion on enhancements on PUSCH repetition Type A	Panasonic Corporation

- R1-2201381	Discussion on TB processing over multi-slot PUSCH	Panasonic Corporation

- R1-2201382	Discussion on the interaction between inter-slot frequency hopping and DMRS bundling	Panasonic Corporation

- R1-2201434	Discussion on joint channel estimation for PUSCH	Panasonic Corporation

- R1-2201442	Remaining issues on PUSCH repetition type A enhancements	China Telecom

- R1-2201443	Remaining issues on joint channel estimation	China Telecom

- R1-2201444	FL Summary of joint channel estimation for PUSCH	Moderator (China Telecom)

- R1-2201445	Remaining issues on inter-slot frequency hopping with inter-slot bundling	China Telecom

- R1-2201446	Remaining issues on type A PUSCH repetitions for Msg3	China Telecom

- R1-2201487	Remaining issues on enhancements on PUSCH repetition type A	NTT DOCOMO, INC.

- R1-2201488	Remaining issues on TB processing over multi-slot PUSCH	NTT DOCOMO, INC.

- R1-2201489	Remaining issues on joint channel estimation for PUSCH	NTT DOCOMO, INC.

- R1-2201490	Remaining issues on PUCCH enhancement	NTT DOCOMO, INC.

- R1-2201491	Remaining issues on type A PUSCH repetitions for Msg3	NTT DOCOMO, INC.

- R1-2201554	Discussion on enhancements for PUSCH repetition Type A	Spreadtrum Communications

- R1-2201555	Discussion on joint channel estimation for PUSCH	Spreadtrum Communications

- R1-2201556	Discussion on PUCCH enhancements	Spreadtrum Communications

- R1-2201657	Type-A PUSCH repetition for coverage enhancement	InterDigital, Inc.

- R1-2201658	TB processing over multiple slots	InterDigital, Inc.

- R1-2201659	Discussion on joint channel estimation	InterDigital, Inc.

- R1-2201660	Discussion on PUCCH enhancements	InterDigital, Inc.

- R1-2201661	Type A PUSCH repetitions for Msg3	InterDigital, Inc.

- R1-2201708	Remaining details of enhancements on PUSCH repetition type A	Intel Corporation

- R1-2201709	Remaining details on TB processing over multi-slot PUSCH	Intel Corporation

- R1-2201710	Remaining details on joint channel estimation for PUSCH	Intel Corporation

- R1-2201711	Remaining details on PUCCH enhancements	Intel Corporation

- R1-2201712	Remaining details on Msg3 PUSCH repetition	Intel Corporation

- R1-2201780	Remaining issues on PUSCH repetition type A enhancement	Apple

- R1-2201781	Remaining issues on TB processing over multi-slot PUSCH	Apple

- R1-2201782	Remaining issues on cross-slot channel estimation for PUSCH	Apple

- R1-2201783	Remaining issues on PUCCH coverage enhancement	Apple

- R1-2201868	Remaining issues on PUSCH repetition type A enhancement	CMCC

- R1-2201869	Remaining issues on TB processing over multi-slot PUSCH	CMCC

- R1-2201870	Remaining issues on joint channel estimation for PUSCH	CMCC

- R1-2201871	Remaining issues on PUCCH enhancements	CMCC

- R1-2201872	Remaining issues on type A PUSCH repetitions for Msg3	CMCC

- R1-2201905	Discussion on remaining issue of TBoMS	NEC

- R1-2201912	Remaining issues on joint channel estimation for PUSCH	Xiaomi

- R1-2201913	Remaining issues on PUCCH enhancements	Xiaomi

- R1-2201925	Remaining issues on TB processing over multi-slot PUSCH	Xiaomi

- R1-2201926	Remaining issues on Type A PUSCH repetition for Msg3	Xiaomi

- R1-2201961	Remaining Issues for PUSCH Repetition Type A Enhancement	Ericsson

- R1-2201962	Remaining Issues for TB Processing over Multi-Slot PUSCH	Ericsson

- R1-2201963	Remaining Issues for Joint Channel Estimation for PUSCH	Ericsson

- R1-2201964	Remaining Issues for PUCCH Dynamic Repetition and DMRS Bundling	Ericsson

- R1-2201965	Remaining Issues for Type A PUSCH Repetition for Msg3	Ericsson

- R1-2201966	Frequency Hopping for TBoMS	Ericsson

- R1-2202026	Enhancements on PUSCH repetition type A	Samsung

- R1-2202027	TB processing over multi-slot PUSCH	Samsung

- R1-2202028	Joint channel estimation for PUSCH	Samsung

- R1-2202029	PUCCH enhancements	Samsung

- R1-2202030	Type A PUSCH repetitions for Msg3	Samsung

- R1-2202085	Discussion on Joint channel estimation over multi-slot	MediaTek Inc.

- R1-2202151	Enhancements on PUSCH Repetition Type A	Qualcomm Incorporated

- R1-2202152	TB processing over multi-slot PUSCH	Qualcomm Incorporated

- R1-2202153	Joint channel estimation for PUSCH	Qualcomm Incorporated

- R1-2202154	PUCCH enhancements	Qualcomm Incorporated

- R1-2202196	Enhancements on PUSCH repetition type A	Sharp

- R1-2202197	TB processing over multi-slot PUSCH	Sharp

- R1-2202198	Joint channel estimation for PUSCH	Sharp

- R1-2202199	PUCCH coverage enhancement	Sharp

- R1-2202200	Type A PUSCH repetitions for Msg3	Sharp

- R1-2202237	Discussion on joint channel estimation for PUSCH	TCL Communication Ltd.

- R1-2202238	Discussion on PUCCH enhancements	TCL Communication Ltd.

- R1-2202239	Discussion on PUSCH repetition type A enhancement	TCL Communication Ltd.

- R1-2202240	Discussion on TBoMS PUSCH	TCL Communication Ltd.

- R1-2202299	Discussions on PUSCH repetition type A enhancements	LG Electronics

- R1-2202300	Discussions on TB processing over multi-slot PUSCH	LG Electronics

- R1-2202301	Discussions on joint channel estimation for PUSCH	LG Electronics

- R1-2202302	Discussions on coverage enhancement for PUCCH	LG Electronics

- R1-2202303	Discussion on coverage enhancement for Msg3 PUSCH	LG Electronics

- R1-2202448	On futher potential coverage enhancements	Huawei, HiSilicon

- R1-2202486	Remaining issues on enhancements for PUSCH repetition Type A	WILUS Inc.

- R1-2202487	Remaining issues for TB processing over multi-slot PUSCH	WILUS Inc.

- R1-2202488	Remaining issues on PUCCH enhancements for coverage enhancement	WILUS Inc.

- R1-2202527	FL summary of TB processing over multi-slot PUSCH (AI 8.8.1.2)	Moderator (Nokia)

- R1-2202555	FL summary #1 of PUCCH coverage enhancement	Moderator (Qualcomm)

- R1-2202566	FL Summary #1 on Enhancements on PUSCH repetition type A	Moderator (Sharp)

- R1-2202567	FL Summary #2 on Enhancements on PUSCH repetition type A	Moderator (Sharp)

- R1-2202568	FL Summary #3 on Enhancements on PUSCH repetition type A	Moderator (Sharp)

- R1-2202569	FL Summary #4 on Enhancements on PUSCH repetition type A	Moderator (Sharp)

- R1-2202592	Feature lead summary #1 on support of Type A PUSCH repetitions for Msg3	Moderator (ZTE)

- R1-2202593	Feature lead summary #2 on support of Type A PUSCH repetitions for Msg3	Moderator (ZTE)

- R1-2202594	Feature lead summary #3 on support of Type A PUSCH repetitions for Msg3	Moderator (ZTE)

- R1-2202616	FL Summary#2 of joint channel estimation for PUSCH	Moderator (China Telecom)

- R1-2202617	FL Summary#3 of joint channel estimation for PUSCH	Moderator (China Telecom)

- R1-2202638	FL summary #2 of TB processing over multi-slot PUSCH (AI 8.8.1.2)	Moderator (Nokia)

- R1-2202639	FL summary #3 of TB processing over multi-slot PUSCH (AI 8.8.1.2)	Moderator (Nokia)

- R1-2202640	Final FL summary of TB processing over multi-slot PUSCH (AI 8.8.1.2)	Moderator (Nokia)

- R1-2202676	FL summary #2 of PUCCH coverage enhancement	Moderator (Qualcomm)

- R1-2202753	FL summary #3 of PUCCH coverage enhancement	Moderator (Qualcomm)

- R1-2202869	FL Summary#4 of joint channel estimation for PUSCH	Moderator (China Telecom)

- R1-2202870	[108-e-R17-CovEnh-03] Summary of email discussion on joint channel estimation for PUSCH	Moderator (China Telecom)

RAN2 #116bis-e:

- R2-2200095	LS on updated Rel-17 LTE and NR higher-layers parameter list	Ericsson

- R2-2200192	Issues on coverage enhancements	Qualcomm Incorporated

- R2-2200251	Discussion on CE’s impact on UL carrier selection	OPPO

- R2-2200269	Considerations on requesting Msg3 repetition	NEC Corporation

- R2-2200272	Remaining issues related to coverage enhancement	Xiaomi

- R2-2200421	Consideration on RAN2 impacts of Msg3 repetition	CATT

- R2-2200515	Running 38300 CR for NR coverage enhancements	China Telecom

- R2-2200602	Running 38321 CR for NR coverage enhancements	ZTE Corporation

- R2-2200603	Remaining issues on Msg3 repetition in CE	ZTE Corporation, Sanechips

- R2-2201177	Further Discussion on RAN2 Impacts of Msg3 Repetition	vivo

- R2-2201426	Remaining issues for supporting Msg3 repetition	LG Electronics Inc.

- R2-2201590	RAN2 aspects for Coverage Enhancement	Nokia, Nokia Shanghai Bell

- R2-2201598	On Type A PUSCH repetitions for Msg3	Ericsson

- R2-2201616	RRC running CR for CE	Huawei, HiSilicon

- R2-2201617	Remaining issues on RAN2 support of Msg3 PUSCH repetition	Huawei, HiSilicon

- R2-2201747	[offline-111] CovEnh general aspects	Qualcomm

- R2-2201758	[offline-111] CovEnh general aspects - second round	Qualcomm

- R2-2201784	LS on Stage 2 description for  coverage enhancements	RAN2, China Telecom

- R2-2201963	Running 38300 CR for NR coverage enhancements	China Telecom

- R2-2201964	RRC running CR for CE	Huawei, HiSilicon

- R2-2201966	Running 38.321 CR for NR coverage enhancements	ZTE Corporation

- R2-2201982	LS on BWP with only CE RACH resources     RAN2, Qualcomm

RAN2 #117-e:

- R2-2202153	Reply LS on Maximum duration for DMRS bundling (R4-2202368; contact: Qualcomm)	RAN4

- R2-2202652	TS 38.321 CR for Rel-17 Coverage enhancement	ZTE Corporation, Sanechips

- R2-2202695	Remaining issues for Msg3 repetition	CATT

- R2-2202831	TS 38.300 CR for Rel-17 NR coverage enhancements	China Telecom

- R2-2202981	Discussion on CFRA PUSCH with Repetition	vivo

- R2-2203007	Minor connection to the stage-2 running CR	OPPO

- R2-2203031	Discussion on Msg3 repetition for CFRA	Qualcomm Incorporated

- R2-2203127	Introduction of NR coverage enhancements in RRC	Huawei, HiSilicon

- R2-2203128	On measurement gap handling for Msg3 repetitions	Huawei, HiSilicon

- R2-2203168	Further issues on msg3 repetitions	Ericsson

- R2-2203284	BWP with only CR-RACH resources	Nokia, Nokia Shanghai Bell

- R2-2203553	TS 38.321 CR for Rel-17 Coverage enhancement	ZTE Corporation, Sanechips

- R2-2203554	Introduction of NR coverage enhancements in RRC	Huawei, HiSilicon

- R2-2203996	Reply LS on Maximum duration for DMRS bundling (R4-2206537; contact: Qualcomm)	RAN4

- R2-2204034	[AT117-e][111][CovEnh] MAC CR (ZTE)	ZTE Corporation

- R2-2204036	TS 38.300 CR for Rel-17 NR coverage enhancements	China Telecom

- R2-2204037	Introduction of NR coverage enhancements in RRC	Huawei, HiSilicon

- R2-2204118	Reply LS on UL BWP with PRACH resources only for RACH with Msg3 repetition (R1-2202829; contact: ZTE)	RAN1

- R2-2204130	Reply LS on Stage 2 description for Coverage Enhancements (R1-2202867; contact: China Telecom)	RAN1

RAN4 #101bis-e:

- R4-2202330, Email discussion summary for [101-bis-e][130] NR_cov_enh, Moderator (China Telecom)

- R4-2202368, Reply LS on Maximum duration for DMRS bundling, Qualcomm

- R4-2202418, WF on phase continuity and power consistency for PUCCH and PUSCH transmissions, Huawei, HiSilicon

- R4-2200020, Updated RAN4 RF work plan for NR coverage enhancements WI, China Telecom

- R4-2201706, simulation updated results for phase tolerance for PUSCH repetition, Ericsson

- R4-2201986, Some remaining open issues on coverage enhancements, MediaTek (Chengdu) Inc.

- R4-2200021, RF requirements for the non-zero gap in between PUSCH/PUCCH transmissions, China Telecom

- R4-2200343, OFF power requirement for the gap in TX on case, Qualcomm Incorporated

- R4-2201705, RF impact on non-scheduled gap, Ericsson

- R4-2200022, On phase continuity and power consistency tolerance, China Telecom

- R4-2200338, Discussion on UE requirement for JCE, Qualcomm Incorporated

- R4-2200471, Views on phase continuity and power consistency for PUSCH and PUCCH repetition, Sony

- R4-2200926, Discussion on testability of coverage enhancement requirements, ROHDE & SCHWARZ

- R4-2201704, On JCE phase continuity and power consistency tolerance for PUCCH and PUSCH, Ericsson

- R4-2201840, Feasibility of proposed test methods for phase continuity and power consistency tolerance measurements, Anritsu Limited

- R4-2201958, On phase continuity for multiple transmissions, Huawei, HiSilicon

- R4-2200023, On maximum duration for joint channel estimation, China Telecom

- R4-2201707, LS reply On maximum duration of phase continuity and power consistency for PUCCH and PUSCH repetition, Ericsson

- R4-2200024, On UE autonomous adjustment and DL reception in-between transmission, China Telecom

- R4-2200344, Maximum duration handling for JCE, Qualcomm Incorporated

- R4-2201703, On measurement of the TX coherent transmission, Ericsson

- R4-2203117, Email discussion summary for [101-bis-e][319] NR_cov_enh_Demod_NWM, Moderator (China Telecom)

- R4-2203030, WF on PUSCH demodulation performance of Rel-17 NR coverage enhancement, China Telecom

- R4-2203031, WF on PUCCH demodulation performance of Rel-17 NR coverage enhancement, Nokia

- R4-2200025, BS demodulation requirements for NR coverage enhancements, China Telecom

- R4-2200408, BS demodulation requirements for NR coverage enhancements, Nokia, Nokia Shanghai Bell

- R4-2200477, Discussion on BS demodulation requirements for NR coverage enhancement, Ericsson

- R4-2200756, View on demodulation requirement for Rel-17 coverage enhancement, Samsung

- R4-2201017, Discussion on BS coverage enhancement demod, Huawei,HiSilicon

- R4-2201604, Discussion on scope of BS demodulation requirements for NR coverage enhancements WI, Intel Corporation

RAN4 #102-e:

- R4-2206436, Email discussion summary for [102-e][136] NR_cov_enh, Moderator (China Telecom)

- R4-2206537, Reply LS on Maximum duration for DMRS bundling Qualcomm

- R4-2206538, CR on UE RF requirements for DMRS bundling in TS 38.101-1 Huawei, HiSilicon

- R4-2206539, CR on UE RF requirements for DMRS bundling in TS 38.101-2 Huawei, HiSilicon

- R4-2206593, WF on issues for maintenance of NR coverage enhancements, Ericsson

- R4-2206580, Reply LS on Length of Maximum duration, China Telecom

- R4-2203818, Updated RAN4 RF work plan for NR coverage enhancements WI China Telecom

- R4-2203822, 38.101-1 CR: UE RF requirements for DMRS bundling, China Telecom

- R4-2203823, 38.101-2 CR: UE RF requirements for DMRS bundling China Telecom

- R4-2204820, Draft CR on UE RF requirements for DMRS bundling Huawei, HiSilicon

- R4-2204821, Draft CR on UE RF requirements for DMRS bundling, Huawei, HiSilicon

- R4-2205531, simulation updated results for phase tolerance for PUSCH repetition, Ericsson

- R4-2205532, LS reply on DMRS bundling for PUSCH and PUCCH, Ericsson

- R4-2206131, Coverage enhancements – remaining UE RF requirements aspects, MediaTek (Chengdu) Inc.

- R4-2203819, Non-zero un-scheduled gap in between PUSCH/PUCCH transmissions, China Telecom

- R4-2205530, RF impact on non-scheduled gap, Ericsson

- R4-2203820, On phase continuity tolerance for DMRS bundling, China Telecom

- R4-2205529, On JCE phase continuity and power consistency tolerance for PUCCH and PUSCH, Ericsson

- R4-2205535, CR on phase tolerance for DMRS bundling in TS 38.101-1, Ericsson

- R4-2205536, CR on phase tolerance for DMRS bundling in TS 38.101-2, Ericsson

- R4-2205882, Discussion on UE phase discontinuity requirement, Qualcomm Incorporated

- R4-2206014, On phase continuity requirement with coverage enhancement, Apple

- R4-2203821, On maximum duration for DMRS bundling, China Telecom

- R4-2204818, On phase continuity for multiple transmissions, Huawei, HiSilicon

- R4-2205537, LS reply On maximum duration of phase continuity and power consistency for PUCCH and PUSCH repetition, Ericsson

- R4-2204819, On maximum length for the non-zero un-scheduled gap, Huawei, HiSilicon

- R4-2205528, On measurement of the TX coherent transmission, Ericsson

- R4-2206541, CR on measurement for DMRS bundling in TS 38.101-1, Ericsson

- R4-2206542, CR on measurement for DMRS bundling in TS 38.101-2, Ericsson

- R4-2207449, Email discussion summary for [102-e][329] NR_cov_enh_Demod, Moderator (China Telecom)

- R4-2207210, WF on PUSCH demodulation performance of Rel-17 NR coverage enhancement, China Telecom

- R4-2207211, WF on PUCCH demodulation performance of Rel-17 NR coverage enhancement, Nokia

- R4-2206132, JCE BS demodulation requirements, MediaTek (Chengdu) Inc

- R4-2203552, View on PUSCH demodulation requirement for Rel-17 coverage enhancement, Samsung

- R4-2205489, PUSCH demodulation performance of Rel-17 NR coverage enhancements, Nokia, Nokia Shanghai Bell

- R4-2205500, On BS PUSCH demodulation requirements for NR coverage enhancements, China Telecom

- R4-2205772, Discussion on BS coverage enhancement demod PUSCH, Huawei, HiSilicon

- R4-2205817, Discussion on PUSCH demodulation requirements for NR coverage enhancements WI, Intel Corporation

- R4-2204033, Discussion on NR coverage enhancement PUSCH demodulation, Ericsson

- R4-2203553, View on PUCCH demodulation requirement for Rel-17 coverage enhancement, Samsung

- R4-2205490, PUCCH demodulation performance of Rel-17 NR coverage enhancements, Nokia, Nokia Shanghai Bell

- R4-2205501, On BS PUCCH demodulation requirements for NR coverage enhancements, China Telecom

- R4-2205773, Discussion on BS coverage enhancement demod PUCCH, Huawei, HiSilicon

- R4-2205818, Discussion on PUCCH demodulation requirements for NR coverage enhancements WI, Intel Corporation

- R4-2204034, Discussion on NR coverage enhancement PUCCH demodulation, Ericsson

- R4-2206632, CR on measurement for DMRS bundling in TS 38.101-1	 Ericsson

- R4-2206633, CR on measurement for DMRS bundling in TS 38.101-2	 Ericsson