3GPP TSG-RANP Meeting #95-e	RP-220610

Online, March 17~23, 2022

Agenda item:	9.2.7

Source:	Qualcomm Incorporated

Title:	NCD-SSB in RRC Idle/Inactive for Rel-18 eRedCap UEs

Document for:	Discussion and Decision

# Introduction

In this contribution, we first discuss the benefits of making NCD-SSB available to Rel-18 eRedCap UEs in RRC Idle/Inactive. We then propose to add its discussion and specification as an objective in the Rel-18 eRedCap WID.

# Discussion

In RAN1#107-e meeting, RAN1 agreed that NCD-SSB can be configured in RedCap-specific BWPs when RedCap UE is in RRC Connected. In addition, RAN1 made the working assumption that NCD-SSB can be configured in RedCap-specific initial BWP if it is configured for paging.

- For FR1, - For a separate initial DL BWP (if it does not include CD-SSB and the entire CORESET#0) from RAN1 perspective, - If it is configured for random access while not for paging in idle/inactive mode, RedCap UE does NOT expect it to contain SSB/CORESET#0/SIB. - Note: RAN1 assumes REDCAP UE performing Random access in the separate DL BWP does not need to monitor paging in a BWP containing CORESET#0 - Working assumption: If it is configured for paging, RedCap UE expects it to contain NCD-SSB for serving cell but not CORESET#0/SIB from RAN1 perspective - For an RRC-configured active DL BWP in connected mode (if it does not include CD-SSB and the entire CORESET#0) from RAN1 perspective, - A RedCap UE supporting mandatory FG 6-1 (but not optional FG 6-1a) expects it to contain NCD-SSB for serving cell but not CORESET#0/SIB - A RedCap UE can indicate the following as optional capability: - Not need NCD-SSB: A RedCap UE can in addition optionally support relevant operation (except for standalone use for RRM measurement) based on for CSI-RS (working assumption) and/or FG 6-1a by reporting optional capabilities.

Observation 1. 	RAN1 has already made the working assumption that NCD-SSB can be configured in RedCap-specific initial BWP if it is configured for paging.

However, in the RANP#94-e meeting, companies agreed to deprioritize NCD-SSB in RRC Idle/Inactive in Rel-17, out of concern that with the limited time left in Rel-17 WI, RAN2 may not have enough time to discuss and specify the necessary enhancements.

- Scheme 1 (i.e. UE in IDLE and INACTIVE monitors paging in an initial BWP associated with CD-SSB) is adopted for further work in Rel-17. Scheme 2 (i.e. UE in IDLE and INACTIVE monitors paging in an initial BWP associated with NCD-SSB) is not considered further in Rel-17.

Observation 2. 	Use of NCD-SSB in RRC Idle/Inactive was deprioritized in Rel-17 due to the concern that its study may risk on-time completion of the WI in RAN2.

Rel-18 eRedCap SI includes the study on narrower max UE bandwidth. If agreed, network would more likely configure an eRedCap-specific initial BWP, not only for RACH but for paging as well. And the use of NCD-SSB will be more useful eRedCap UE’s idle mode procedures.

Observation 3. 	If Rel-18 eRedCap introduces narrower max UE bandwidth, use of NCD-SSB in eRedCap specific initial BWP will be more useful in supporting their idle mode procedures.

Since RAN1 already discussed NCS-SSB in RRC Idle/Inactive and made a working assumption, we think it is not necessary to study this feature in the Rel-18 SI phase. Instead, it can be directly included as one of the objectives in the Rel-18 eRedCap WID.

Proposal. 	Include the discussion and specification of NCD-SSB in RRC Idle/Inactive in the Rel-18 eRedCap WID.

# Conclusion

Based on the above discussion, we’d recommend the plenary to discuss and adopt the following proposal:

Observation 1. 	RAN1 has already made the working assumption that NCD-SSB can be configured in RedCap-specific initial BWP if it is configured for paging.

Observation 2. 	Use of NCD-SSB in RRC Idle/Inactive was deprioritized in Rel-17 due to the concern that its study may risk on-time completion of the WI in RAN2.

Observation 3. 	If Rel-18 eRedCap introduces narrower max UE bandwidth, use of NCD-SSB in eRedCap specific initial BWP will be more useful in supporting their idle mode procedures.

Proposal. 	Include the discussion and specification of NCD-SSB in RRC Idle/Inactive in the Rel-18 eRedCap WID.

# References

- RAN1#107-e meeting report, Nov 2021.

- RP-213689, Summary for 94e-39-R17-RedCap-WI, Dec 2021.