3GPP TSG RAN Meeting#95-e	RP-220615

Electronic Meeting, March 17 – 23, 2022

Source:	NTT DOCOMO, INC

Title:	Views on NR NTN enhancements

Document for: 	Discussion and Decision

Agenda Item:	9.3.2.7

Introduction

In RAN#94-e meeting, a new WI: NR NTN (Non-Terrestrial Networks) enhancements [1] was approved. One of the objectives of NTN enhancement is Network verified UE location, which is shown as follows.

4.1.3 Network verified UE location Have a 1-TU 6-month study phase focusing on the following (to derive clear & limited scope): - Study detailed regulatory requirement for network-verified UE location, e.g. accuracy requirement (at RAN plenary, from RAN#95 to RAN#96). [RAN] - Including further clarification on network verified UE location and its relationship to network-based positioning [RAN] - Study and evaluate, if needed, solutions for network to verify UE reported location information [RAN2, RAN1, RAN3] RAN to determine by RAN#98 whether the study has identified any need for Network verified UE location specification support in Rel-18.

According to the WID, detailed regulatory requirement for network-verified UE location including further clarification on network verified UE location needs to be studied at RAN plenary. In this contribution, our views on network verified UE location are provided.

Discussion

During RAN#94-e meeting, the objective of network verified UE location was discussed a lot. From companies’ comments in the email discussion, it is observed that there were two main different understandings on the motivation of “network verified UE location”, which are summarized as below.

- Motivation A: network can utilize UE location to provide specific services, such as emergency call, public warning, etc. Accordingly, solutions for estimation of UE location at network side need to be studied.

- Motivation B: NW can check the accuracy of UE reported location, where the UE reported location can be the location information obtained by UE itself via GNSS, and NW can terminate the service for the UE when the accuracy of UE reported location is quite low, e.g., if fake GNSS reporting is identified. Accordingly, solutions for UE reporting location information need to be studied. Meanwhile, solutions for estimation of UE location at network side need to be studied as well, so that NW can check the accuracy of UE reported location, i.e., check the difference between UE reported location and NW estimated location.

To clarify the scope of network verified UE location, we think which of the above motivations is the main target of this objective needs to be discussed and decided. From our perspective, the Motivation A, i.e., network can utilize UE location to provide specific services, such as emergency call, public warning, etc., could be more important to make the same service available in both TN and NTN. Hence, solutions for estimation of UE location at network side need to be studied in WI.

Observation:

- From companies’ comments in the email discussion, it is observed that there were two main different understandings on the motivation of network verified UE location:

  - Motivation A: network can utilize UE location to provide specific services, e.g., emergency call, public warning, etc.

  - Motivation B: NW can check the accuracy of UE reported location and terminate the service for the UE when the accuracy of UE reported location is quite low, e.g., if fake GNSS reporting is identified.

Proposal 1:

- NW utilizing UE location to provide specific services, e.g., emergency call, public warning, should be the main motivation of “network verified UE location”.

Proposal 2:

- For network verified UE location, solutions for estimation of UE location at network side need to be studied.

Another objective at RAN plenary is to study detailed regulatory requirement for network-verified UE location. As discussed above, there can be different understandings on the motivation, and they will lead to different requirement of accuracy. Thus, the motivation should be clarified first, and the requirement should be defined accordingly. From our perspective, the above Motivation A could be more important, thus, the requirement should be defined targeting the motivation that NW can utilize UE location to provide specific services, e.g., emergency call, public warning. Moreover, regulatory requirements in Rel-16 NR positioning can be taken as baseline for the initial step, which are listed as below.

-	Horizontal positioning error <= 50m for 80% of UEs

-	Vertical positioning error <5 m for 80% of UEs

-	End to end latency and TTFF < 30 seconds

Proposal 3:

- Requirement should be defined targeting the motivation that NW can utilize UE location to provide specific services, e.g., emergency call, public warning. Regulatory requirements in Rel-16 NR positioning can be taken as baseline for the initial step.

Conclusion

Observation:

- From companies’ comments in the email discussion, it is observed that there were two main different understandings on the motivation of network verified UE location:

  - Motivation A: network can utilize UE location to provide specific services, e.g., emergency call, public warning, etc.

  - Motivation B: NW can check the accuracy of UE reported location and terminate the service for the UE when the accuracy of UE reported location is quite low, e.g., if fake GNSS reporting is identified.

Proposal 1:

- NW utilizing UE location to provide specific services, e.g., emergency call, public warning, should be the main motivation of “network verified UE location”.

Proposal 2:

- For network verified UE location, solutions for estimation of UE location at network side need to be studied.

Proposal 3:

- Requirement should be defined targeting the motivation that NW can utilize UE location to provide specific services, e.g., emergency call, public warning. Regulatory requirements in Rel-16 NR positioning can be taken as baseline for the initial step.

- 4. References

[1] RP-213690, RAN Vice-Chair (AT&T), Thales, “New WI: NR NTN (Non-Terrestrial Networks) enhancements”

[2] RP-213680, Moderator's summary of discussion [94e-17-R18-NTN-NR]