3GPP TSG RAN Meeting #95-e																			RP-220665

Electronic Meeting, March 17 - 23, 2022

Source: 	ZTE

Title: 	Discussion for LS RP-220030 on Location Services for Drones

Agenda item:	14

Document for:	Discussion/Decision

Introduction

In RAN2#116-e meeting, RAN2 received an LS from ETSI TC LI in R2-2110295 as shown in section 4. Based on 3GPP TR 36.777 ”Study on Enhanced LTE Support for Aerial Vehicles”, the LS raises the doubt whether LTE/NR positioning mechanisms can meet regulatory requirements in use cases where the UE is in a drone. The LS points out some potential scenarios including the case that drones may be served on the back lobes of cell sites.

Then, a reply LS RP-213674 was agreed in RAN#94-e meeting as shown in section 5. RAN clarifies that the 3GPP positioning features for both LTE and NR can meet the regulatory and commercial requirements described in 3GPP TR 38.855 although the positioning features was not explicitly considered for drones.

However, the reply LS seems not clear enough. So TC LI further sent an LS RP-220030 with some clarification questions as below.

- TC LI understands that the TR 38.855 is not usable as a reference in the other related work because it is an 800 series technical report and meant to be for 3GPP internal use. Therefore, TC LI kindly requests clarification on when the normative work would be done and where it would be specified. The regulatory requirements apply to any service that is offered.

In this contribution, we provide our views on how to reply the LS RP-220030.

Discussion

In NR Release 16 and LTE Release 13, the positioning methods are studied and further specified. As discussed in RAN#94-e meeting, it is clear that the 3GPP positioning features for both LTE and NR can meet the regulatory and commercial requirements. Even though the study of the positioning features is not dedicated for the UE in a drone, the positioning methods defined in NR/LTE are still applicable to the UE in a drone as the NR specifications do not have restriction for this type of UE.

In addition, in the 3GPP UAV WID for Release 18, the enhancements on measurement reports will be specified including reporting of height, location and speed in measurement report. But this enhancement will be just to facilitate gNB scheduling without any positioning feature enhancement. Hence, the feature will not be for location service purpose and nothing related to the issue raised in the LSs.

So, to TC LI’s question, we think it should clarify that 3GPP positioning features defined in both LTE and NR can meet the regulatory and commercial requirements, and are applicable to UE in a drone.

- TR 38.855 is just a 3GPP technical report which captures outcome of the study item for NR positioning features include the NR positioning requirements, deployment scenarios, evaluation methodologies, positioning technologies, evaluation results, specification impacts and conclusion.

- The outcome of the normative work is captured in 3GPP technical specifications (TSs) rather than in TRs.

  - For NR positioning, the normative work is done from Release 16, the corresponding TSs in RAN include 38.305, 37.355, 38.455, etc.

  - For LTE positioning, the normative work was done in Release 13 for improving vertical and horizontal accuracy to sufficiently support the regulatory mandate in a variety of deployment options, the corresponding TSs in RAN include 36.305, 36.355, 36.455, etc.

Conclusion

Based on the discussion in section 2, we provide our draft reply LS as follows

1. Overall Description: TSG RAN thanks ETSI TC LI for their further liaison. TSG RAN thinks 3GPP positioning features defined in both LTE and NR can meet the regulatory and commercial requirements, and are applicable to the UE in a drone. - TR 38.855 is just a 3GPP technical report which captures the outcome of the study item. - The outcome of the normative work is captured in 3GPP technical specifications (TSs). - For NR positioning, the normative work is done from Release 16, the corresponding TSs in RAN include 38.305, 37.355, 38.455, etc. - For LTE positioning, the normative work was done in Release 13 for improving vertical and horizontal accuracy to sufficiently support the regulatory mandate in a variety of deployment options, the corresponding TSs in RAN include 36.305, 36.355, 36.455, etc. 2. Actions: TSG RAN asks ETSI TC LI to take the above information into account in their future work.

LS from ETSI TC LI to RAN2 (R2-2110295)

- Introduction: ETSI TC LI kindly asks for clarification on report 3GPP TR 36.777 Study on Enhanced LTE Support for Aerial Vehicles. While the report does discuss the implication of such a service impacting the existing spectrum, it failed to address the location determination functions used in RRC for cell measurements and or Location Reporting Control required for regulatory purposes such as Emergency Call or Lawful Access support and the use of Location Reporting primitives used by Location Services. One might reply that Remote ID1 and its reporting solves these location issues, however the following 2 use cases are not supported. Emergency call from a person's UE in a Drone; that UE does not have access to the Drones location information. While GPS may be accurate, the Location Reporting may be impaired if the drone is served on the back lobes of cell sites based on its height above these said cell sites. We note that the UE in emergency Call will provide its GPS data, we also note that the network will verify that location, and we now see potential issues in the reporting and validation of that location information, where GPS will report one location and the Location Reporting function may report a different location based on a back lobe serving the drone and or UE. We question what information will be provided to public authorities noting financial penalties for failing to accurately report location in some jurisdictions and or the imposing of restrictions to these future services. Location services for Asset tracking or other location services that use an LCS server to provide services to MNO and third parties, any UE transported in Drones, becomes problematic based on comments in the use case above and depending on the measurement service used. Lawful Access, especially if a Drone taxi is used, requires location information as per national regulations of the individual in the drone using periodic location or LALS (Lawful Access Location Services)i We note that Remote ID provides information to different regulatory bodies and may not be available to an LCS server nor to Law Enforcement Agencies or with proper foresight. (Note that regulatory restrictions, i.e. lawful interception/access requires a warrant, while support for Navigation of drones may be a Terms and Conditions (T&C) with local governments Transportation agencies and each handled through different network functions). - Actions ETSI TC LI request RAN to provide guidance on this issue and any proposed solutions and timing. Work Item “Study on enhanced Support for Aerial Vehicles” RP-171050 (http://3gpp.org/ftp/tsg_ran/TSG_RAN/TSGR_76/Docs/RP-171050.zip ) 3GPP 36.777 “Enhanced LTE support for aerial vehicles” (https://portal.3gpp.org/desktopmodules/Specifications/SpecificationDetails.aspx?specificationId=3231 ) Work Item “Enhanced LTE Support for Aerial Vehicles” RP-172826 (http://3gpp.org/ftp/tsg_ran/TSG_RAN/TSGR_78/Docs/RP-172826.zip) Study Item “Remote Identification of Unmanned Aerial Systems” SP-180172 (http://www.3gpp.org/ftp/tsg_sa/TSG_SA/TSGS_79/Docs//SP-180172.zip) 3GPP 22.825 “Study on Remote Identification of Unmanned Aerial Systems” (https://portal.3gpp.org/desktopmodules/Specifications/SpecificationDetails.aspx?specificationId=3527) Work Item “Remote Identification of Unmanned Aerial Systems” SP-180771 (http://3gpp.org/ftp/tsg_sa/TSG_SA/TSGS_81/Docs/SP-180771.zip) Study Item “Enhancements for UAVs” SP-180909 (http://3gpp.org/ftp/tsg_sa/TSG_SA/TSGS_81/Docs/SP-180909.zip) i 3GPP TS 33.127 / 3GPP TS 33.128.

Reply LS to ETSI TC LI (RP-213674)

TSG RAN thanks ETSI TC LI for their liaison titled " Location Services: Drones". TSG RAN acknowledges that positioning was not considered explicitly in the development of 3GPP TR 36.777. However, RAN would like to clarify that the 3GPP positioning features for both LTE and NR can meet the regulatory and commercial requirements described in 3GPP TR 38.855.