3GPP TSG RAN meeting #95-e									     RP-220747

Electronic Meeting, March 17 - 23, 2022

## Status Report to TSG

Agenda item: 9.5.2.3 UE power saving enhancements for NR [RAN2 WI: NR_UE_pow_sav_enh]

| WI / SI Name | UE power saving enhancements for NR |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: - | Core part: Yes | Performance part: Yes |  | Testing part: - |
| Acronym | NR_UE_pow_sav_enh |  |  |  |  |
| Unique ID | 860047 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-212630 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: - | Core part: 03/2022 | Performance part: 09/2022 | Testing part: - |  |
| Overall Completion level | Study Item: - | Core part: Overall: 100% | Performance Part: Overall: 35% | Testing part: - |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN2 |
| --- | --- | --- |
| Rapporteur | Name | Weide Wu (RAN2, RAN1, RAN4) |
|  | Company | MediaTek Inc. |
|  | Email | weide.wu@mediatek.com |
| Rapporteur | Name | Dapeng Li (RAN3) |
|  | Company | ZTE |
|  | Email | dapeng@zte.com.cn |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table: N/A

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

Relating to scope item 1) – a): - Specify enhancements for idle/inactive-mode UE power saving, considering system performance aspects [RAN2, RAN1, RAN3] - Study and specify paging enhancement(s) to reduce unnecessary UE paging receptions, subject to no impact to legacy UEs [RAN2, RAN1, RAN3] - NOTE: RAN1 to check and update, if needed, evaluation methodology in RAN1 #102-e meeting The following agreements are achieved:

RAN1 #107-bis-e Meeting Agreement A new PEI-RNTI is supported for DCI format 2_7. The following text proposals are adopted: TS 38.213 section 10.1: <Unchanged text is omitted> - a Type2A-PDCCH CSS set configured by peiSearchSpace in DownlinkConfigCommonSIB for a DCI format 2_7 with CRC scrambled by a PEI-RNTI on the primary cell of the MCG <Unchanged text is omitted>. TS 38.213 section 10.4B: <Unchanged text is omitted> A UE in RRC_IDLE state or RRC_INACTIVE state can be provided by TRS-ResourceSetConfig a set of TRS occasions [6, TS 38.214]. If TRS-ResourceSetConfig is provided, a DCI format 2_7 with CRC scrambled by PEI-RNTI or a DCI format 1_0 with CRC scrambled by P-RNTI includes a TRS availability indication field [4, TS 38.212] that provides a bitmap to groups of TRS resource sets where the configuration of each TRS resource set includes an association to a bit of the bitmap. <Unchanged text is omitted>. Agreement For whether and how to accommodate PEI-O location determination for the case POnumPerPEI is smaller than Ns, decide one of the following alternatives - Alt-2: It is supported, and UE applies the single value in PEI-F_offset for the frame-level offset and the (flooris/POnumPerPEI+1) -th value out of Ns/POnumPerPEI configured values in firstPDCCH-MonitoringOccasionOfPEI-O for the symbol-level offset Note: The number of PO mapping to one PEI should be multiple of Ns when POnumPerPEI is larger than Ns Agreement For PEI-O location determination: - Range of PEI-F_offset, in unit of frame, is 0, 1,2, …, 16 - Range of firstPDCCH-MonitoringOccasionOfPEI-O, in unit of symbol, is decided as one of the following alternatives: - Alt-1: The same value range as firstPDCCH-MonitoringOccasionOfPO Agreement For the value PEI-RNTI, decide one of the following: - Alt-1: PEI-RNTI is of fixed value, and value design is up to RAN2. Conclusion For a UE supporting R17 paging enhancement feature, it is up to UE implementation whether the UE monitors the MO(s) for a PEI. - If UE decides to not to monitor PEI, it has to monitor UE’s PO as defined in 38.304. - Note: No specification impact Agreement - The PDCCH monitoring occasions defined by peiSearchSpace colliding with UL symbols are omitted from the determination of the PEI monitoring occasions. The PDCCH monitoring occasions for PEI which do not overlap with UL symbols (determined according to tdd-UL-DL-ConfigurationCommon) are sequentially numbered from zero starting from the first PDCCH monitoring occasion for PEI in the PEI-O. Agreement The transmission power of PEI is based on the same determination method as paging PDCCH, and the following TP is adopted. .............................................................. Text Proposal for 38.213.............................................................................. 4.1 Cell search Cell search is the procedure for a UE to acquire time and frequency synchronization with a cell and to detect the physical layer Cell ID of the cell. A UE receives the following synchronization signals (SS) in order to perform cell search: the primary synchronization signal (PSS) and secondary synchronization signal (SSS) as defined in [4, TS 38.211]. A UE assumes that reception occasions of a physical broadcast channel (PBCH), PSS, and SSS are in consecutive symbols, as defined in [4, TS 38.211], and form a SS/PBCH block. The UE assumes that SSS, PBCH DM-RS, and PBCH data have same EPRE. The UE may assume that the ratio of PSS EPRE to SSS EPRE in a SS/PBCH block is either 0 dB or 3 dB. If the UE has not been provided dedicated higher layer parameters, the UE may assume that the ratio of PDCCH DMRS EPRE to SSS EPRE is within -8 dB and 8 dB when the UE monitors PDCCHs for a DCI format 1_0 with CRC scrambled by SI-RNTI, P-RNTI, or RA-RNTI, and DCI format 2_7 with CRC scrambled by PEI-RNTI. ............................................................... Text Proposal for 38.213................................................................................... Agreement UE expects at most one PEI outcome from each PDCCH monitoring occasion of PEI-O, and the following text proposal is adopted. Section 10.1 of TS 38. 213 *** Unchanged text is omitted *** If a UE is provided - one or more search space sets by corresponding one or more ofsearchSpaceZero, searchSpaceSIB1, searchSpaceOtherSystemInformation,pagingSearchSpace, peiSearchSpace, ra-SearchSpace, or a CSS set byPDCCH-Config, and - a SI-RNTI, a P-RNTI, a PEI-RNTI, a RA-RNTI, a MsgB-RNTI, a SFI-RNTI, an INT-RNTI, a TPC-PUSCH-RNTI, a TPC-PUCCH-RNTI, or a TPC-SRS-RNTI then, for a RNTI from any of these RNTIs, the UE does not expect to process information from more than one DCI format with CRC scrambled with the RNTI per slot. *** Unchanged text is omitted ***. Agreement If one PEI-O is associated with POs of 2 PFs, - The two PFs are consecutive PFs configured in SIB - FFS: two PFs are consecutive PFs within the same paging cycle - Note: As an example, SFN of the first PF of the PF(s) associated with the PEI-O can be obtained by: (SFN of UE’s PF) - iPO/Ns⋅T/N Agreement - LS to RAN2 for sharing agreements with potential RAN2 specification impacts - Draft LS in R1-2200788 Conclusion - For Rel-17, inclusion of SI change indication or ETWS/CMAS notification in DCI format 2_7 is not supported. Conclusion For Rel-17 DCI format 2_7, - Explicit starting position configuration for paging indication field or TRS availability indication field is not supported. - Paging indication field starts from the first bit, and TRS availability indication field starts from the next bit after the end of the paging indication field - UE does not expect the total size of paging indication filed and TRS availability indication field is larger than the DCI payload size Working assumption - SearchSpaceId = 0 can be configured forpeiSearchSpace for the case of CORESET multiplexing pattern 2 or 3 Agreement - Draft LS R1-2200788 is endorsed in principle - Include above working assumption in the LS on paging enhancement to RAN2 Agreement - Final LS R1-2200800 is endorsed.

RAN1 #108-e meeting Conclusion For Rel-17, the following for paging early indication design are not supported: - Multipe PEI-Os for one PO - UE assistance information for preferred offset between PEI-O and PO - Different PEI configurations for different devices types Agreement The following TP for 38.202-h00, subclause 6 to reflect reception type with PEI-RNTI is endorsed. <Begin TP for 38.202-h00, subclause 6.2> Table 6.2-1: Downlink "Reception Types" "Reception Type"; Physical Channel(s); Monitored RNTI; Associated Transport Channel; Comment. <unchanged rows omitted for brevity>. P; PDCCH; CI-RNTI; N/A. Q; PDCCH; PEI-RNTI; N/A; Note1. Table 6.2-2: Downlink "Reception Type" combinations Supported Combinations; Comment. PCell; PSCell; SCell. 1. RRC_IDLE. A + (B and/or (C1 or Q) and/or D0) + F0; Note 1. 2. RRC_INACTIVE. A + (B and/or (C1 or Q) and/or D0) + F0; Note 1. < unchanged rows omitted for brevity>. <End TP for 38.202-h00, subclause 6.2> Agreement For the alignment with the following RAN2#116bis-e agreements,  Both subgroupNumPerPO and Nsg-UEID range from 1 to 8.  If network supports PEI but not subgrouping, the wholeSubgroupConfig-r17 is absent. The parameter subgroupsNumPerPO is mandatory present if subgroupConfig-r17 is configured.. - Adopt following TP to TS38.213 Section 10.4: TS38.213v17.0.0] 10.4 A PDCCH monitoring for early indication of paging A UE can be provided the following for detection of a DCI format 2_7 in RRC_IDLE state or in RRC_INACTIVE state [12, TS 38.331] - a search space set, by peiSearchSpace, to monitor PDCCH for detection of DCI format 2_7 according to a Type2A-PDCCH CSS set as described in clause 10.1 - a number of frames, by PEI-F_offset, from the start of a first paging frame of paging frames associated with a number of PDCCH monitoring occasions for DCI format 2_7 [17, TS 38.304] to the start of a frame - a number of symbols, by firstPDCCH-MonitoringOccasionOfPEI-O, from the start of the frame to the start of the first PDCCH monitoring occasion for DCI format 2_7 - a size, by payloadSizeDCI_format2_7 - a number of subgroups per paging occasion, , by subgroupsNumPerPO - a number of paging occasions associated with the number of PDCCH monitoring occasions for DCI format 2_7, , by PONumPerPEI A paging indication field of DCI format 2_7 includes segments of bits, where if and if is not providedconfigured or . For a subgroup index , , a UE determines a value for the bit in the paging indication field, where is a paging occasion index, and , , , and are defined in [17, TS 38.304]. When the value is '1', the UE monitors a paging occasion determined according to [17, TS 38.304]; otherwise, the UE is not required to monitor the paging occasion.. - Adopt following TP to TS38.212 Section 7.3.1.3.8: [TS38.212v17.0.0] 7.3.1.3.8 Format 2_7 DCI format 2_7 is used for notifying the paging early indication and TRS availability indication for one or more UEs. The following information is transmitted by means of the DCI format 2_7 with CRC scrambled by PEI-RNTI: - Paging indication field – bit(s), where - is the number of paging occasions configured by higher layer parameter PONumPerPEI as defined in Clause 10.4A in [5, TS 38.213]; - is the number of sub-groups of a paging occasion configured by higher layer parametersubgroupsNumPerPO, if subgroupsNumPerPO is configured and not set to 0; otherwise is set to 1. - Each bit in the field indicates one UE subgroup of a paging occasion if subgroupsNumPerPOsubgroupsNumPerPO is configured and not set to 0; otherwise each bit in the field indicates the UE group of a paging occasion. - TRS availability indication – 1, 2, 3, 4, 5, or 6 bits if TRS-ResourceSetConfig is configured; 0 bits otherwise. The size of DCI format 2_7 is indicated by the higher layer parameter payloadSizeDCI_format2_7, according to Clause 10.4A of [5, TS 38.213]. If the number of information bits in format 2_7 is less than the size of format 2_7, the remaining bits are reserved.. Agreement Adopt the following TP to Section 10 of TS 38.213 ## 10 UE procedure for receiving control information *** Unchanged text is omitted *** If the UE is configured with a SCG, the UE shall apply the procedures described in this clause for both MCG and SCG except for PDCCH monitoring in Type0/0A/2/2A-PDCCH CSS sets where the UE is not required to apply the procedures in this clause for the SCG - When the procedures are applied for MCG, the terms 'secondary cell', 'secondary cells' , 'serving cell', 'serving cells' in this clause refer to secondary cell, secondary cells, serving cell, serving cells belonging to the MCG respectively. - When the procedures are applied for SCG, the terms 'secondary cell', 'secondary cells', 'serving cell', 'serving cells' in this clause refer to secondary cell, secondary cells (not including PSCell), serving cell, serving cells belonging to the SCG respectively. The term 'primary cell' in this clause refers to the PSCell of the SCG. *** Unchanged text is omitted ***. Conclusion SFN of the first PF of the PF(s) associated with the PEI-O is provided by (SFN of UE’s PF) - Agreement - Confirm the following working assumption: Working assumption SearchSpaceId = 0 can be configured for peiSearchSpace for the case of CORESET multiplexing pattern 2 or 3. - When SearchSpaceId = 0 is configured for peiSearchSpace, subject to SS/PBCH block and CORESET multiplexing pattern 2 or 3, the PDCCH monitoring occasions for PEI-O are same as for RMSI as defined in clause 13 in TS 38.213. - UE determines first PDCCH MO for PEI-O based on PEI-F_offset and firstPDCCH-MonitoringOccasionOfPEI-O, as previously agreed for the case with SearchSpaceId > 0. - Note: UE expects the first PDCCH MO for PEI-O determined from PEI-F_offset and firstPDCCH-MonitoringOccasionOfPEI-O is aligned with the MOs for RMSI as defined in clause 13 in TS 38.213.

Relating to scope item 1) – b): - Specify enhancements for idle/inactive-mode UE power saving, considering system performance aspects [RAN2, RAN1] - Specify means to provide potential TRS/CSI-RS occasion(s) available in connected mode to idle/inactive-mode UEs, minimizing system overhead impact [RAN1] - NOTE: Always-on TRS/CSI-RS transmission by gNodeB is not required The following agreements are achieved:

RAN1 #107-bis-e Meeting Agreement Support a configuration parameter for the number of, X, TRS resources for a TRS resource set at least for FR2 - Applicable values for X: {2, 4} - For FR1, X is based on configuration parameter with applicable value {2, 4} Note: Configuration follows restriction specified in sub-clause 5.1.6.1.1 in TS38.214 for connected mode TRS Agreement One or more scrambling IDs is configured for a TRS resource set. - If a single scrambling ID is configured, it applies to all the TRS resources. - Otherwise, each TRS resource is provided with a scrambling ID. Agreement The applicable values:{64, 128, 256, 512} are supported for the validity duration configured by higher layer, - Note: If UE is provided a configuration of validity duration longer than 10.24s, and the UE does not support eDRX, it is up to UE implementation whether to assume the validity duration length is no larger than 10.24s Agreement UE can receive L1 based signaling for TRS availability indication before the expiration/end of validity duration associated with previous L1 based signaling for TRS availability indication - For each bit indicated as ‘1’ in the availability indication field of the current L1 based signaling TRS availability indication, the UE assumes the corresponding TRS resource set(s) are available from the reference point until the end of the validity duration associated with the current L1 based signaling indication. - For each bit indicated as ‘0’ in the availability indication field of the current L1 based signaling, the UE keeps the existing current assumption on the availability or unavailability of the corresponding TRS resource set(s). Note: the validity duration for different group of TRS resources sets correspond to different bits in the availability indication field can be different and are maintained independently. Note: CATT has concern on the power saving gain based on this agreement. Agreement Confirm the following working assumption Working Assumption If TRS resource is configured in SIB, L1 based availability indication is always enabled based on the configuration. Agreement The parameter of periodicityAndOffset is used to determine the location of the first slot of TRS resource set. Agreement If SIB configures TRS resource, TRS availability indication field is present in DCI format 2_7 (if configured) with CRC scrambled by PEI-RNTI and DCI format 1_0 with CRC scrambled by P-RNTI Note: Huawei, MTK and ZTE have concern on the agreement.

RAN1 #108-e Meeting Agreement The following TP to TS 38.213 is endorsed 10.4B Indication of TRS resources A UE in RRC_IDLE state or RRC_INACTIVE state can be provided by TRS-ResourceSetConfig a set of TRS occasions [6, TS 38.214]. If TRS-ResourceSetConfig is provided, a DCI format 2_7 if configured or and a DCI format 1_0 with CRC scrambled by P-RNTI includes a the TRS availability indication field [4, TS 38.212] that provides a bitmap to groups of TRS resource sets where the configuration of each TRS resource set includes an association to a bit of the bitmap. The UE can be additionally provided a multiple, by validityDuration, for a number of frames provided bydefaultPagingCycle for TRS resource sets with indicated presence; if validityDuration is not provided, the multiple is equal to 2. A value of '1' for a bit of the bitmap indicates presence of associated TRS resource sets for the multiple of the number of frames, starting from a SFN determined from [17, TS 38.304] that corresponds to the frame that includes a PDCCH providing the DCI format 2_7, or the DCI format 1_0 with CRC scrambled by P-RNTI, with the TRS availability indication fieldindicating the TRS resource sets, where is provided by defaultPagingCycle. A value of '0' for a bit of the bitmap is ignored by the UE indicates that the UE keeps existing assumption of the availability or unavailability of associated TRS resource sets. A UE can receive first and second PDCCHs that provide DCI format 2_7 or DCI format 1_0 with CRC scrambled by P-RNTI that indicate presence of TRS resource sets for the multiple of the number of frames, where the second PDCCH reception after the first PDCCH reception by a time that is smaller than themultiple of the number of frames. *** Unchanged text is omitted ***. Agreement The following TP to TS 38.214 is endorsed 5.1.6.1.1 CSI-RS for tracking <omitted text> Each NZP CSI-RS resource, defined in Clause 7.4.1.5.3 of [4, TS 38.211], is configured by the higher layer parameter TRS-ResourceSet with the following restrictions for a UE in RRC_IDLE or RRC_INACTIVE: - the number of periodic NZP CSI-RS resources configured by a TRS-ResourceSet is given by numberOfresources - the time-domain locations of the two CSI-RS resources in a slot, or of the four CSI-RS resources in two consecutive slots (which are the same across two consecutive slots), is one of - , , or for frequency range 1 and frequency range 2, - , , , , , or for frequency range 2. - where the first symbol location in a slot is indicated by firstOFDMSymbolInTimeDomainin the TRS-ResourceSet and the second symbol location in a slot isfirstOFDMSymbolInTimeDomain + 4 - a single port CSI-RS resource with density given by Table 7.4.1.5.3-1 from [4, TS 38.211]. - the bandwidth and the frequency location of the NZP CSI-RS resource, is given by the higher layer parameter nrofRBs, startingRB and frequencyDomainAllocation in a TRS-ResourceSet and applies to all resources in a TRS-ResourceSet. The frequencyDomainAllocation configuration is Bandwidth, nrofRBs, and the initial CRB index, startingRB, of the NZP CSI-RS resource configured by TRS-ResourceSet are not restricted by initial DL BWP. - UE is not required to receive TRS occasions outside the initial DL BWP. - the periodicity for periodic NZP CSI-RS resources, is given by the higher layer parameterperiodicityAndOffset configured by a TRS-ResourceSet, is one of slots where 10, 20, 40, or 80 and where µ is defined in Clause 4.3 of [4, TS 38.211], applies to all resources in a TRS-ResourceSet. The slot offset given by the higher layer parameter periodicityAndOffset configured by a TRS-ResourceSet provides the location of the first slot containing the periodic NZP CSI-RS resources configured by by a TRS-ResourceSet. - the UE does not expect the TRS-ResourceSet to be configured with the periodicity of slots if the bandwidth of NZP CSI-RS resource is larger than 52 resource blocks. - the UE may assume the sub-carrier spacing of the NZP CSI-RS resources configured byTRS-ResourceSet to be same as the sub-carrier spacing of the initial DL BWP. - powerControlOffsetSS given by a TRS-ResourceSet applies to all resources in a TRS-ResourceSet. - the QCL information for periodic NZP CSI-RS resources, is given by the higher layer parameter ssb-Index configured by a TRS-ResourceSet, is a SS/PBCH block, applies to all resources in a TRS-ResourceSet. - One or more scramblingIDs where if a single scramblingID is configured, it applies to all NZP-CSI-RS resources in the resource set, otherwise, each NZP-CSI-RS resource is provided with a scramblingID. - the UE may assume the following quasi co-location type(s): - 'typeC' with an SS/PBCH block and, when applicable, 'typeD' with the same SS/PBCH block. For each TRS-ResourceSet the index of the associated bit in TRS availability indication field [5, TS 38.212], is given by the higher layer parameter indBitID. <omitted text>. Conclusion No consensus to support - PEI DCI provides L1 availability indication information only for RS resources with QCL references to be the same as for the L1 availability indication occasion Conclusion A UE can receive different TRS availability indication content from PEI and associated paging PDCCH. - Note: It is up to UE implementation whether UE receives both PEI and the associated paging PDCCH to receive TRS availability indication Conclusion No consensus to support - For PEI based availability indication, the reference time of the start of validity duration follows the reference time for associated monitoring occasion for the paging DCI. Agreement Down-select one of the TPs to clarify the current default DRX cycle used for determining the reference point for start of validity duration for an availability indication - Alt1: Adopt TP#1 - Alt2: Adopt TP#2 - Note: TRS availability indication transmitted in all PDCCH monitoring occasions of the same PEI-O or PO is same. - Note: Reference point for TRS availability indication in a given DCI is identical for all UEs regardless of UE specific DRX value. ============================= start of TP#1========================================== ## 10.4B Indication of TRS resources ## <Unchanged text omitted> A value of '1' for a bit of the bitmap indicates presence of associated TRS resource sets for the multiple of the number of frames, starting from a SFN determined from SFN+PF_offsetmodT=0 [17, TS 38.304] that corresponds to the frame within the DRX cycle that includes a the PDCCH providing the DCI format 2_7, or the DCI format 1_0 with CRC scrambled by P-RNTI, with the TRS availability indication field indicating the TRS resource sets, where T is provided by defaultPagingCycle. A value of '0' for a bit of the bitmap is ignored by the UE. A UE can receive first and second PDCCHs that provide DCI format 2_7 or DCI format 1_0 with CRC scrambled by P-RNTI that indicate presence of TRS resource sets for the multiple of the number of frames, where the second PDCCH reception after the first PDCCH reception by a time that is smaller than the multiple of the number of frames. ## <Unchanged text omitted> ============================= end of TP#1========================================== ============================= start of TP#2========================================== ## 10.4B Indication of TRS resources ## <Unchanged text omitted> A value of '1' for a bit of the bitmap indicates presence of associated TRS resource sets for the multiple of the number of frames, starting from a SFN determined from SFN+PF_offsetmodT=0 [17, TS 38.304] that corresponds to the frame of the DRX cycle associated with the first PDCCH MO of the PEI-O or PO that includes a the PDCCH providing the DCI format 2_7, or the DCI format 1_0 with CRC scrambled by P-RNTI, with the TRS availability indication field indicating the TRS resource sets, where T is provided by defaultPagingCycle. A value of '0' for a bit of the bitmap is ignored by the UE. A UE can receive first and second PDCCHs that provide DCI format 2_7 or DCI format 1_0 with CRC scrambled by P-RNTI that indicate presence of TRS resource sets for the multiple of the number of frames, where the second PDCCH reception after the first PDCCH reception by a time that is smaller than the multiple of the number of frames. ## <Unchanged text omitted> ============================= end of TP#2==========================================. Agreement TRS-ResourceSetConfig IE is not accounted in determination of available resource elements for PDSCH.

Relating to scope item 2) – a): - Study and specify, if agreed, enhancements on power saving techniques for connected-mode UE, subject to minimized system performance impact [RAN1, RAN4] - Study and specify, if agreed, extension(s) to Rel-16 DCI-based power saving adaptation during DRX Active Time for an active BWP, including PDCCH monitoring reduction when C-DRX is configured [RAN1] - NOTE: Rel-15 and Rel-16 available power saving solutions should be supported by the UE and included in the evaluation. RAN1 will ask the confirmation from RAN2 that Rel-15 and Rel-16 available power saving solutions are properly utilized. The following agreements are achieved:

RAN1 #107-bis-e Meeting Agreement Remove the bracket from following agreement made in RAN1#106bis-E Agreement (extracted from RAN1#106bis-E) - For Case 3 (i.e., 3 SSSG switching) , the following is supported - 2-bit in scheduling DCI is supported to indicate PDCCH monitoring adaptation UE behaviors - ‘00’ is Beh 2 - ‘01’ is Beh 2A - ‘10’ is Beh 2B - ‘11’ is reserved Agreement Confirm the following working assumption (extracted from RAN1#107-bis agreements) Agreement - For value X in Beh 1A, candidate skipping values are - Up to [100ms] length is supported, - The X is configured and indicated in the unit of slot. - Working assumption for candidate values for X - {1,2,3,…,20,30, 40, 50, 60, 80, 100} for 15 kHz SCS, - {1,2,3,…,40, 60, 80, 100, 120,160,200} for 30 kHz SCS, - {1,2,3,…,80, 120, 160, 200, 240, 320,400} for 60kHz SCS, - {1,2,3,…,160, 240, 320,400, 480, 640,800} for 120kHz SCS - FFS: Equal to or longer than the applicable minimum scheduling offset - FFS: additional symbol level / PDCCH monitoring period level skipping duration Agreement - The initial timer value for switching from SSSG#2 to SSSG#0 and from SSSG#1 to SSSG#0 is common and configured per BWP. - The PDCCHSkippingDurationList-r17 is configured per BWP. Working Assumption - The bit length of the candidate skipping values and SSSG switching initial timer values in slots for 480kHz and 960kHz SCS are assumed to be the same as that for 120KHz SCS Agreement PDCCH monitoring adaptation is only applied to the individual scheduling cell (including self-scheduling and cross-carrier scheduling) Conclusion For Rel-17 PDCCH monitoring adaptation, the case where single DCI indicates monitoring adaptation for multiple cells is not supported. Agreement For PDCCH monitoring adaptation case 1, support at most M = 3 for PDCCH skipping with 2 bits indication. Agreement The configuration of PDCCH monitoring adaptation is common for DCI format x-1 and x-2, neither Alt 1 nor Alt 2 is agreed, - Alt 1: Support RRC parameter configured for the PDCCH monitoring adaptation indication field presence/absence in DCI format x-2. - Alt 2: The configuration of PDCCH monitoring adaptation is common for DCI format x-1 and x-2, except skipping duration for DCI format x-2 (i.e., PDCCHSkippingDurationList-r17-x-2 for DCI format x-2). The PDCCH monitoring adaptation indication field for DCI format x-2 is dependent on PDCCHSkippingDurationList-r17-x-2 if configured. Conclusion No consensus to introduce non-scheduling DCI based PDCCH monitoring adaptation for Rel-17.

RAN1 #108 -e Meeting Agreement - The 480kHz and 960kHz SCS are also supported for Rel-17 PDCCH monitoring adaptation. Agreement - It is up to B52.6GHz to deicide the values for candidate skipping values and SSSG switching initial timer values. Agreement Answer 1 for R1-2200884 (R2-2201960): Yes, PDCCH skipping can be applied to C-RNTI(s) and MCS-C-RNTI(s) in at least Type3 PDCCH CSS and USS monitored during RAR/MsgB window for RAR/MsgB reception based on RAN1 agreements till RAN1#107bis-e. Agreement Answer 2 for R1-2200884 (R2-2201960): RAN1 would discuss and conclude how to capture the above RAN2 agreements in RAN1 specification. o It is RAN1 understanding PDCCH skipping is not applied to perform PDCCH monitoring during RAR window/MsgB window/contention resolution timer or when SR is pending. - FFS: If the UE is indicated skipping PDCCH monitoring for a duration and at the first slot after the last OFDM symbol of a positive SR transmission in PUCCH, the UE can stops PDCCH skipping and monitor PDCCH for UL grant (i.e., PDCCH skipping is not activated ). Agreement Answer 4 for R1-2200884 (R2-2201960):  It is RAN1’s understanding that transmission of CSI/SRS is not affected by PDCCH skipping i.e. CSI/SRS is still transmitted during the skipping duration. Agreement  It is RAN1 understanding that for DRX operation, - As DCP is monitored only outside active time, it cannot be missed due to PDCCH skipping as skipping applies only in active time. Note: RAN1 is discussing whether PDCCH skipping duration can apply to outside active time. Agreement [Draft] LS reply on PDCCH skipping, R1-2202896 is endorsed in principle. Agreement Final LS reply on PDCCH skipping, R1-2202905 is endorsed Agreement For DCI format 0_1, DCI format 0_2, DCI format 1_1 and DCI format 1_2, - 1 or 2 bits, if PDCCHSkippingDurationList is not configured and if searchSpaceGroupIdList-r17 is configured - 1 bit if the UE is configured by searchSpaceGroupIdList-r17 with search space set(s) with group index 0 and search space set(s) with group index 1, and if the UE is not configured by searchSpaceGroupIdList-r17 with any search space set with group index 2; - 2 bits if the UE is configured by searchSpaceGroupIdList-r17 with search space set(s) with group index 0, search space set(s) with group index 1 and search space set(s) with group index 2; Note: Same as in existing specifications, specifications allow gNB to configure a search-space set with zero PDCCH candidates. Agreement If a UE is provided group indexes for a Type3-PDCCH CSS set or a USS set by searchSpaceGroupIdList-r17 and a timer value by searchSpaceSwitchTimer-r17 for PDCCH monitoring on a serving cell and the timer is running, the UE - Resets the timer after a slot of the active DL BWP of the serving cell when the UE detects a DCI format in a PDCCH reception in the slot with CRC scrambled by - Working Assumption: C-RNTI/CS-RNTI/MCS-C-RNTI - Otherwise, decrease the timer value by one after each slot. - Note: There is no timer for SSSG switching is running for UE in SSSG#0 Working Assumption - Upon detecting a scheduling DCI format 1-1/1-2/0-1/0-2 indicating SSSG switching (i.e., Beh 2/2A/2B), - the UE applies SSSG switching on an active BWP of the serving cell at a first slot that is at least Pswitch symbols after the last symbol of the PDCCH reception - FFS: a minimum applicable scheduling offset is configured in the BWP - Note: Pswitch is defined in Table 10.4-1 in TS38.213 Agreement - Upon detecting a scheduling DCI format 1-1/1-2/0-1/0-2 indicating PDCCH skipping (i.e., Beh 1A), select one of the following schemes - Alt 1a: the UE applies Beh 1A on an active BWP of the serving cell at the first slot after the last OFDM symbol of the PDCCH reception - FFS: a minimum applicable scheduling offset is configured in the BWP - FFS: whether the UE will monitor PDCCH when DRX Retransmission timer is running Agreement - When the timer expires, the UE monitors PDCCH on the serving cell according to search space sets with group index 0 in a slot - no earlier than [application delay] after the slot where timer expires and - if PDCCH skipping is being applied, no earlier than in slot where PDCCH skipping duration expires. Working assumptions: - When the UE receives DCI format 0_1 /1_1/0_2/1_2 with a ‘PDCCH monitoring adaptation indication' field in slot n, the UE does not expect to receive DCI format 0_1 /1_1/0_2/1_2 with a ‘PDCCH monitoring adaptation indication’ field indicating change to another SSSG or skipping for the same active BWP of the scheduling cell before slot n+X of the scheduling cell, where X is the value of the application delay. - If SSSG timer expires in slot n, the UE does not expect to receive DCI format 0_1 /1_1/0_2/1_2 with a ‘PDCCH monitoring adaptation indication’ field indicating change to another SSSG or skipping for the same active BWP of the scheduling cell before slot n+X of the scheduling cell, where X is the value of the application delay.

#### 2.1.2	Remaining open issues

RAN1 processed maintenance work in RAN1 #107-bis-e and #108-e meetings for resolving identified issues and generating necessary corrections to the RAN1 related specifications. RAN1 will continue maintenance work with particular focus on cross-WG and cross-feature related issue(s), if identified.

## 2.2	RAN2

#### 2.2.1	Agreements

Relating to scope item 1) - a) - Specify enhancements for idle/inactive-mode UE power saving, considering system performance aspects [RAN2, RAN1, RAN3] - Study and specify paging enhancement(s) to reduce unnecessary UE paging receptions, subject to no impact to legacy UEs [RAN2, RAN1, RAN3] - NOTE: RAN1 to check and update, if needed, evaluation methodology in RAN1 #102-e meeting The following agreements are achieved:

- RAN2 #116bis-e Meeting RAN configuration (of subgrouping) includes the two parameters Nsg-UEID (number of UEID-based subgroups) and subgroupsNumPerPO (total number of subgroups in a PO): - - If only CN-assigned subgrouping is used, subgroupsNumPerPO is present (the value then equals to the number of CN-assigned subgroups), and Nsg-UEID is absent. - - If only UEID-based subgrouping is used, subgroupsNumPerPO and Nsg-UEID are present, and Nsg-UEID has the same value as subgroupsNumPerPO. - - If both subgrouping methods are used, both subgroupsNumPerPO and Nsg-UEID are present, and 0 < Nsg-UEID < subgroupsNumPerPO. RAN2 aims to Support PEI and subgrouping with eDRX. FFS the impact. RAN2 assumes that there is no particular impact to Uu signalling to support RAN sharing. It is further assumed that Core Networks must have consistent policy if subgrouping is used by multiple Core Networks. RAN2 assumes that PEI can be used “without” subgrouping. FFS whether the bits in the PEI for subgrouping then need to have any particular meaning, or whether this would be done by just having one subgroup. RAN2 assumes that PEI monitoring can not be specifically enabled/disabled for individual UEs. PEI subgroup indices are allocated to CN-assigned subgroups first. The 1st bit in the PEI bitmap corresponds to the CN-assigned subgroup #1, 2nd bit in the bitmap corresponds to the CN-assigned subgroup #2, and so on. After CN-assigned subgroups, the subsequent bits, if any, correspond to the UEID-based subgroup #1, #2, and so on. Both subgroupNumPerPO and Nsg-UEID range from 1 to 8. If network supports PEI but not subgrouping, the whole SubgroupConfig-r17 is absent. The parameter subgroupsNumPerPO is mandatory present if subgroupConfig-r17 is configured. UE is configured to monitor PEI, either only in the last used cell or any other cells (after cell reselection). FFS how the configuration is provided in [SI, RRCRelease, or NAS message]. If a cell supports both UE identity based and CN assigned subgrouping, for UEID based paging subgrouping, UE belongs to k-th paging subgroup, where - - k = [floor (UE Identity/(N*Ns)) mod Nsg-UEID] + Nsg-CN, - - N is the number of Paging frames, - - Ns is the number of POs per paging frame, - - Nsg-UEID is the number of UEID-based paging subgroups, and - - Nsg-CN is the number of CN assigned paging subgroups (= subgroupNumPerPO - Nsg-UEID). [058] Paging enhancement capability(-ies) (e.g. PEI capability, UEID based subgrouping capability or the combined capability of PEI and UEID based subgrouping) are ‘optional with capability signalling’ as gNB needs to know the paging enhancement capability(-ies) to page the UE [058] Paging enhancement capability(-ies) can be included into the UERadioPagingInfo IE in the UECapabilityInformation message as agreed in RAN2#116 (i.e. Introduce a UERadioPagingInfo IE in the UECapabilityInformation message in NR in Rel-17) [058] gNB interprets UE’s reported UECapabilityInformation, copies the UERadioPagingInfo IE out and includes it as a container UE-RadioPagingInfo IE in the UERadioPagingInformation inter-node message to AMF [058] Separate indications for UE capability of CN based subgrouping and UEID based subgrouping (confirms earlier assumption) [058] UE’s capability of supporting the UE ID based subgrouping is reported to RAN by AS UE capability signalling while UE’s capability of supporting the CN-assigned subgrouping is reported to CN by NAS signalling. (confirms earlier assumption).

- RAN2 #117-e Meeting Network indicates whether UE monitors PEI in last used cell in system information. A PEI-capable UE stores its “last used cell” information. FFS on how to capture this in the specifications. Do not introduce an associated timer for the “last used cell” information stored by UE. The “lastUsedCellOnly” indication is a cell-level configuration and there is no per-subgroup indication. Introduce a one-bit indication of lastUsedCellOnly in PEI-Config. RAN2 clarifies the meaning of “last used cell only”: When a cell broadcasts “last used cell only”, a UE monitors PEI only if its last connection was released by this cell. PEI + UEID subgrouping is one capability A PEI-capable UE must support at least UEID-based subgrouping method. RAN2 confirms that “PEI without subgrouping” can be implemented by configuring PEI plus UEID subgrouping with one subgroup. “PEI without subgrouping” can be configured by only one method. When PEI is applied with eDRX, the UEID for UEID-based subgrouping is determined by 5G-S-TMSI mod 32768. No special handling or configuration is introduced for PEI monitoring with PTW (i.e., PEI is applicable to each PO within PTW) Send an informative LS to RAN3/SA2/CT1 for RAN2 agreements about PEI and paging subgrouping and ask question about mismatched understanding of “last cell” between network and UE. If a UE cannot find its subgroup ID with the PEI configurations in a cell, it monitors legacy paging. Agree the easy agreement proposals (removing whether from P1). - A PEI-capable UE stores its “last used cell” information. FFS on whether/how to capture this in the specifications. - Do not introduce an associated timer for the “last used cell” information stored by UE. - The “lastUsedCellOnly” indication is a cell-level configuration and there is no per-subgroup indication. - Introduce a one-bit indication of lastUsedCellOnly in PEI-Config. - RAN2 clarifies the meaning of “last used cell only”: When a cell broadcasts “last used cell only”, a UE monitors PEI only if its last connection was released by this cell. - A PEI-capable UE must support at least UEID-based subgrouping method. - RAN2 confirms that “PEI without subgrouping” can be implemented by configuring PEI plus UEID subgrouping with one subgroup. - “PEI without subgrouping” can be configured by only one method. - When PEI is applied with eDRX, the UEID for UEID-based subgrouping is determined by 5G-S-TMSI mod 32768. - No special handling or configuration is introduced for PEI monitoring with PTW (i.e., PEI is applicable to each PO within PTW) - Send an informative LS to RAN3/SA2/CT1 for RAN2 agreements about PEI and paging subgrouping and ask question about mismatched understanding of “last cell” between network and UE. - If a UE cannot find its subgroup ID with the PEI configurations in a cell, it monitors legacy paging. On P4 send an LS to RAN3 cc SA2, asking about the issue, can also indicate tentative solution and invite for feedback. Ask R3 also on P13, to what extent this may be a problem. A UE with CN-assigned subgroup ID should derive UEID-based subgroup ID in a cell supporting only UEID-based subgrouping. The LS out in R2-2204240 is approved.

Relating to scope item 1) – b): - Specify enhancements for idle/inactive-mode UE power saving, considering system performance aspects [RAN2, RAN1] - Specify means to provide potential TRS/CSI-RS occasion(s) available in connected mode to idle/inactive-mode UEs, minimizing system overhead impact [RAN1] - NOTE: Always-on TRS/CSI-RS transmission by gNodeB is not required The following agreements are achieved:

- RAN2 #116bis-e Meeting The number of bits N in the bitmap used for L1 availability indication is derived implicitly from the number of different values of indBitID. There is no need for an explicit parameter. RAN2 confirm TRS/CSI-RS can be applied to eDRX UEs. Confirm that there will be no particular mechanism for availability indication based on SIB (beyond the presence of the RS configuration) A UE which acquired SIB-X with a TRS/CSI-RS configuration but didn’t yet receive an associated L1-based availability indication considers the configured TRS/CSI-RS as FFS: “unavailable” or “available”. R2 doesn't send an LS to R1 on SIB segmentation - [055] OFFLINE [055] Indicating the TRS/CSI-RS availability in Idle/Inactive when releasing the UE to Idle/Inactive in the RRCRelease message is not pursued. [055] RAN2 follows RAN1 agreement that if TRS resource is configured in SIB, L1 based availability indication is always enabled based on that configuration. [055] RAN2 waits for RAN1 to finalize the contents of SIB-X before finalizing aspects on SIB-X sizing, segmentation etc [058] Postpone the discussion of UE AS capabilities for RLM/BFD relaxation to next meeting.

- RAN2 #117-e Meeting gNB does not need to know the UE capability for TRS/CSI-RS in idle and inactive mode. Introduce R1 29-2 as optional without capability signalling A UE which acquired SIB-X with a TRS/CSI-RS configuration but didn’t yet receive an associated L1-based availability indication considers the configured TRS/CSI-RS as “unavailable”. RAN2 reuses the existing mechanism used for SIB12 for implementing the SIBX segmentation Not sufficient support to introduce any special functionality for UEs in eDRX. Rely on UE determination on whether he has up to date information or not. Can think about whether clarification is needed.

Relating to scope item 2) – a): - Study and specify, if agreed, enhancements on power saving techniques for connected-mode UE, subject to minimized system performance impact [RAN1, RAN4] - Study and specify, if agreed, extension(s) to Rel-16 DCI-based power saving adaptation during DRX Active Time for an active BWP, including PDCCH monitoring reduction when C-DRX is configured [RAN1] - NOTE: Rel-15 and Rel-16 available power saving solutions should be supported by the UE and included in the evaluation. RAN1 will ask the confirmation from RAN2 that Rel-15 and Rel-16 available power saving solutions are properly utilized. The following agreements are achieved:

- RAN2 #116bis-e Meeting From RAN2 point of view, UE ignores PDCCH skipping while the SR is pending. From RAN2 point of view, if PDCCH skipping is applied to RNTI(s) monitored during RAR/MsgB window, the UE ignores PDCCH skipping during the RAR/MsgB window. From RAN2 point of view, UE ignores PDCCH skipping while contention resolution timer is running. If DCP can not be monitored due to PDCCH skipping, FFS whether to a) reuse the ps-Wakeup or b) PHY indicate DCP as 1 to MAC. No specification change is expected for either a) and b). Send LS to RAN1 - include agreed proposals 1, 2, 3 - ask RAN1 to take agreed proposals into account - ask RAN1 whether a) Physical layer of UE reports a value of 1 for Wake-up indication bit to higher layer or b) Physical layer of UE does not report Wake-up indication bit to higher layer, in case UE cannot monitor DCP due to PDCCH skipping. - Take comments above into account. The LS out in R2-2201960 is approved. [058] For UE capabilities of PDCCH monitoring adaptation, implement it as part of the UE capability rapporteur mega CRs from the R1 feature list

- RAN2 #117-e Meeting RAN2 clarifications to earlier RAN2 decisions: P1: UE ignores PDCCH skipping on all serving cells of the corresponding CG while SR is pending (FFS if “all” can be further restricted). P2: If PDCCH skipping is applied to RNTI(s) monitored during RAR/MsgB window, UE ignores PDCCH skipping on SpCell. P3: UE ignores PDCCH skipping on SpCell while contention resolution timer is running.

Relating to scope item 2) - b) - Study and specify, if agreed, enhancements on power saving techniques for connected-mode UE, subject to minimized system performance impact [RAN1, RAN4] - Study the feasibility and performance impact of relaxing UE measurements for RLM and/or BFD, particularly for low mobility UE with short DRX periodicity/cycle, and specify, if agreed, relaxation in the corresponding requirements [RAN4] - NOTE: Supplementary RAN2 work, if needed, can be triggered by RAN4 LS The following agreements are achieved:

- RAN2 #116bis-e Meeting BFD relaxation is enable/disable per serving cell (i.e. separately between Pcell/PScell and Scell). FFS on stage-3 details. RLM relaxation is enable/disable per-CG (i.e. separately between Pcell and PScell). FFS on stage-3 details, FFS if enable/disable is by the UE or by the network. Parameters of SSearchDeltaP and TSearchDeltaP for low mobility criterion is configured in dedicated signaling. FFS on stage-3 details (i.e. value range of parameters, in which IE). RAN2 assumes that the criteria for RLM/BFD relaxation will be captured in RAN2 TS, can ask R4 RAN2 to send an LS to RAN4 for RLM/BFD relaxation including the below aspects: - RAN2 conclusions on RLM/BFD relaxation - Specification split on RLM/BFD relaxation The LS out in R2-2201989 is approved. - [056] OFFLINE [056] RLM relaxation and BFD relaxation are enabled/disabled separately. [056][Post116bis-e][000] Postpone the discussion on how to enable/disable RLM relaxation per-CG, and how to enable/disable BFD relaxation per-serving cell to wait for RAN4 conclusions on the configuration of criteria [056] RAN2 assume the criteria configuration for RLM relaxation and BFD relaxation are configured separately. FFS Which criteria configuration(s) could be configured separately (e.g. serving cell quality). RAN2 can come back on this based on RAN4 conclusion. [056] Postpone the discussion on how to provide the criteria configuration for RLM relaxation and BFD relaxation for low mobility criterion to wait for progress from RAN4. [056] Postpone the discussion on how to provide the criteria configuration for RLM relaxation and BFD relaxation for serving cell quality criterion to wait for progress from RAN4. [056] Postpone the discussion on how to evaluate the low mobility criterion for RLM/BFD relaxation to wait for progress from RAN4. [056] Postpone the discussion on how to evaluate the serving cell quality criterion for RLM/BFD relaxation to wait for progress from RAN4. [056] BWP switch doesn’t impact evaluation of BFD relaxation or ongoing relaxation of BFD measurement. [056] if UE report on fulfillment or not (entry/exit) to network for RLM/BFD relaxation is agreeable, UAI is used to provide the report. [056] RAN2 assumes the configurations for RLM/BFD relaxation should be captured in RAN2 specification, while the relaxation requirements/approaches should be captured in RAN4 specification. [058] Postpone the discussion of UE AS capabilities for RLM/BFD relaxation to next meeting.

- RAN2 #117-e Meeting Introduce 2 separate capability bits for RLM relaxation feature and for BFD relaxation feature The capability bit(s) for RLM and BFD relaxation shall be per UE with FR differentiation - Working Agreement: UE can start/stop RLM/BFD relaxation by itself if it meets/fails the relaxation criteria. The feature is configured by RRC dedicated signalling, this is the only enable disable function that is supported. The configuration for RLM relaxation feature and BFD relaxation feature are provided in SpCellConfig and SpCellConfig/ScellConfig, respectively. Low mobility criterion is configured in NR Pcell for the case of NR SA/ NR CA/ NE-DC/NR-DC, and in the NR PSCell for the case of EN-DC. Introduce optional parameter(s) of offset on Qx (Qin for RLM relaxation and FFS for BFD relaxation) for good serving cell quality criterion in dedicated signaling. If the offset is absent, a pre-defined value is used (e.g. FFS [0dB]). FFS on stage-3 details (i.e. value range of parameters). Serving cell quality criterion is configured provided in PCell/PScell for RLM relaxation and in PCell/PScell for BFD relaxation (regarding Scell waiting for R4 input) Reply LS to RAN4 is in R2- 2204238

#### 2.2.2	Remaining open issues

RAN2 has accomplished core-part design and will enter maintenance phase for the core-part design and resolve critical issue(s), if identified. Regarding cross-WG work, including detailed RRC signalling design, RAN2 will take into account the LS(s)/agreements(s) from the relevant WGs for each feature of Rel-17 UE power saving enhancements for NR.

## 2.3	RAN3

#### 2.3.1	Agreements:

Relating to scope item 1) – a): - Specify enhancements for idle/inactive-mode UE power saving, considering system performance aspects [RAN2, RAN1, RAN3] - Study and specify paging enhancement(s) to reduce unnecessary UE paging receptions, subject to no impact to legacy UEs [RAN2, RAN1, RAN3] - NOTE: RAN1 to check and update, if needed, evaluation methodology in RAN1 #102-e meeting The following agreements are achieved:

- RAN3 #114bis-e Meeting Agreements: RAN3 BL CR allocated by rapporteur as below: Specification Baseline CR rapporteurs TS 38.413 Nokia TS 38.423 Huawei TS 38.473 ZTE TS 38.300 Ericsson TS 38.470 Qualcomm TS 38.410 CATT The CN provides subgroup ID included in subgroup information. Include UE paging subgroup information into the NG Paging message for CN paging. Introduce UE subgroup information into NGAP Core Network Assistance Information. Introduce UE paging subgroup information into F1 Paging message for paging a UE in RRC_INACTIVE/RRC_IDLE. Adopt the terminology "PEIPS Assistance Information" as the IE name to align with SA2 terminology, containing a CN subgroup ID. The CN assigned subgroup ID is encoded as 0…7. (note: further checking maybe needed) Add some clarification on stage 2 behaviour related PEIPS assistance information for RRC INACTIVE for TS38.300. Working Assumption: NG-RAN node can know the total number of subgroups supported by CN via OAM.

- RAN3 #115-e Meeting Remove the Editor Note in Xn BL CR. Introduce a new F1 only UE Capability e.g. F1 UE Paging Capability IE, that could be populated with IEs as needed in the future. (the first would be UEID subgroup support). Turn the "WA: NG-RAN node can know the total number of subgroups supported by CN via OAM." into agreement. Add some description related to PEIPS assistance information on path switch procedure for 38.413. Reply LS to RAN2 (check details). Other issues not covered by first round, if needed. Provide TPs based on the agreements, agree stage2/stage3 TPs. Add a "last used cell indication" IE in the item of Paging cell List IE in F1 paging message to indicate which cell is the last used cell. CU does not need to know the last cell restriction information broadcast by the DU. The gNB-DU System Information need to be extended to include SIBx, and sent to the gNB-CU. New TP to TS 38.300 BL CR: Addition of PEIPS over Xn in R3-222522 Agreed New TP to TS 38.470 BL CR: (TP for Powersaving BLCR for TS 38.470) Introduction of SIBx on TRS resources in R3-222681 Agreed Reply LS to RAN2 on paging subgrouping and PEI in R3-222848 rev in R3-222874 Agreed R3-221578; Support of UE Power Saving Enhancements (Nokia, Nokia Shanghai Bell, Ericsson, Qualcomm Incorporated); CR0725r2, TS 38.413 v16.8.0, Rel-17, Cat. B Endorsed as BL CR. R3-221582; Addition of PEIPS Assistance Information (Ericsson, Qualcomm Inc., Nokia, Nokia Shanghai Bell, Huawei, ZTE, Samsung, CATT); DraftCR Endorsed as BL CR. R3-221583; CR to TS38.473 for UE paging subgroup (ZTE, MediaTek, China Unicom, Nokia, Nokia Shanghai Bell, Ericsson, Huawei, Samsung, Qualcomm Incorporated, CATT); CR0855r2, TS 38.473 v16.8.0, Rel-17, Cat. B - Change the Tdoc number as "R3-221583" in the CR cover page Rev in R3-222548 Endorsed as BL CR. R3-221593; Supporting UE Power Saving Enhancements (Huawei, Nokia, Nokia Shanghai Bell, Samsung, ZTE, Ericsson, Qualcomm Incorporated, CATT); CR0732r2, TS 38.423 v16.8.0, Rel-17, Cat. B - Add "other specs" in cover page Rev in R3-222549 Endorsed as BL CR. R3-221594; (BL CR to TS 38.410) Support for ePowerSaving (CATT, Nokia, Nokia Shanghai Bell, Ericsson, ZTE, Qualcomm Incorporated, Samsung); CR0037r1, TS 38.410 v16.4.0, Rel-17, Cat. B Endorsed as BL CR. R3-221596; (BL CR to TS 38.470) Support for UE Power Saving Enhancements (Qualcomm Incorporated, Nokia, Nokia Shanghai Bell, Samsung, CATT, Huawei, Ericsson, ZTE); CR0080r1, TS 38.470 v16.5.0, Rel-17, Cat. B Endorsed as BL CR. R3-222201; (TP for TS 38.413) Support for UE power saving (CATT); Other Rev in R3-222850 Agreed. R3-222251; (TP to TS38.473 and TS38.423) paging subgroup (ZTE Corporation, MediaTek Inc.); Other Rev in R3-222849 Agreed. R3-222316; (TP to BL CR on TS38.473) Further discussion on UE power saving (Samsung); Other Rev in R3-222772 Agreed.

#### 2.3.2	Remaining Open issues:

For the support of UE power saving enhancements in Rel17, RAN3 has accomplished the core-part design, including introduction of PEIPS Assistance Information into PAGING message and Core Network Assistance Information for RRC INACTIVE IE over NGAP, and PEIPS Assistance Information into Xn/F1 paging message. The UEID based subgrouping support information and last used cell information are also added into F1AP paging message, and the corresponding stage2 and stage3 work in RAN3 are completed. RAN3 will enter maintenance phase for the core-part design and resolve critical issue(s), if identified.

## 2.4	RAN4

#### 2.4.1	Agreements

Relating to scope item 2) - b) - Study and specify, if agreed, enhancements on power saving techniques for connected-mode UE, subject to minimized system performance impact [RAN1, RAN4] - Study the feasibility and performance impact of relaxing UE measurements for RLM and/or BFD, particularly for low mobility UE with short DRX periodicity/cycle, and specify, if agreed, relaxation in the corresponding requirements [RAN4] - NOTE: Supplementary RAN2 work, if needed, can be triggered by RAN4 LS The following agreements are achieved:

RAN4 #101-bis-e Meeting Approved: WF on RLM/BFD relaxation for UE Power Saving enhancements (R4-2202640): #### Issue 1-1-1: Feature for RLM/BFD relaxation in Rel-17 feature table - RAN4 to introduce a UE capability to indicate the support of RLM/BFD relaxation in general in Rel-17 feature table. #### Issue 1-2-1&Issue 1-2-2: Enable the RLM/BFD relaxation feature - RLM/BFD relaxation is enabled by explicit signaling. - FFS the applicability conditions that UE is allowed to apply relaxed requirement. #### Issue 2-1: L3 CSI-RS to be used for Low mobility criteria - Option 1: L3 CSI-RS can be used for low mobility criteria evaluation for Ues supports CSI-RS based L3 measurements as well. - Option 2: The existing agreement to use SSB based L3-RSRP measurement of the serving cell to evaluate the low mobility criterion is sufficient. #### Issue 2-2: Accuracy and delay requirements for low mobility criteria - The RRM measurements used for low mobility evaluation shall fulfil the accuracy requirements defined in TS 38.133 section 10. Issue 2-2-1: the specific SSB to be measured for the per-UE low mobility criterion evaluation. FFS - Option 1a: The intra-frequency L3 RSRP measurement of serving cell based on SSB is derived as the intra-frequency SS-RSRP measured over single SSB, and shall fulfil the measurement requirement in TS 38.133 section 9.2.5.2 and the performance requirement in section 10.1. (Nokia) - Option 1b: Network needs to configure the specific SSB to be measured for the per-UE low mobility criterion evaluation. #### Issue 2-4: Low mobility criteria configuration type - Low mobility criterion is configured on per-UE basis. #### Issue 3-1: Good serving cell quality criteria for RLM/BFD - Agreement - The good serving cell quality criteria for RLM/BFD is based on an offset X dB and Qx, while Qx is derived from PDCCH transmission parameters. - Qx = Qin for RLM - Qx = [Qin] for BFD - Note: definition of Qin for BFD needs to be clarified - The offset X can be configured from a set of 4 values - Exact values are FFS - One pre-defined value is used for evaluation if the offset is not configured - Pre-defined value X = [0] dB - Signalling details are up to RAN2 #### Issue 3-4: Cell quality criteria configuration type - Option 1: per-serving cell basis - Option 2: per-UE basis - Option 3: per-CG basis #### Issue 5-1: Lower bound of relaxed evaluation period - The lower bound of relaxed evaluation period is NOT relaxed by K, if K > 2 is applying. - The lower bound of relaxed evaluation period is relaxed by K, if K <=2 and K>1 is applying. #### Issue 5-2: relaxation factors - Confirm K0, FR2, SSB = 1 for 80 ms < MAX(TDRX, TSSB) ≤ 160 ms. Issue 5-2-2: The relaxation factor for FR1: - Option 1: - K1, FR1 =4 for 40 ms < MAX(TDRX, TRS) ≤ 80 ms - K2, FR1 =4 for MAX(TDRX, TRS) ≤ 40 ms - Option 2 - For FR1 RLM: (consider only DRx <= 80ms) - K = 2 when DRx > 40ms *or* T310 <= 640ms; - K = 4 when DRx <= 40ms *and * T310>640ms, - For FR1 BFD: K = 2 Issue 5-2-3: The relaxation factor for FR2 SSB - Option 1: K1, FR2, SSB= 2 for MAX(TDRX, TSSB) ≤ 80 ms - Option 2: - K=1.5 for 60 ms ≤ MAX(TDRX, TSSB) ≤ 80 ms. - K=2 for MAX(TDRX, TSSB) ≤ 60 ms #### Issue 6-1-1: Relaxation criteria for multiple RLM-RS/BFD-RS - Option 1: - For entering condition: the radio link quality of at least one RS resource is better than the entering threshold. - For exit condition: the radio link quality for all the RS resources is worse than the exiting threshold. - Option 2 - The UE is allowed to operate RLM/BFD in relaxed mode for a certain cell (SpCell or SCell) when the radio link quality is better than the threshold (Qout + X1) for all RLM-RS resource. - The UE shall exit the relaxed mode when the radio link quality is worse than the threshold (Qout + X2) for any the RLM-RS resources. - The values of X1, X2 can be same as those discussed for good serving cell quality. #### Issue 6-3: Specification section for relaxation criteria FFS Capture the relaxation criteria in the separate sub-section, while RAN4 refers the definition of relaxation criteria to RAN1 or RAN2 specification LS on signalings for enabling RLM and BFD relaxation in R17 UE power saving (R4-2202769)

RAN4 #102-e Meeting Approved: WF on RLM/BFD relaxation for UE Power Saving enhancements (R4-2206909): #### Issue 2-1-1: The cases that UE is allowed to apply the relaxed RLM/BFD requirement UE is allowed to apply the relaxed RLM/BFD requirement, - provided UE is configured the explicit signalling to enable RLM/BFD relaxation and UE has fulfilled good serving cell criterion, if low mobility criteria is NOT configured, or - provided UE is configured the explicit signalling to enable RLM/BFD relaxation and UE has fulfilled both good serving cell criterion and low mobility criterion if low mobility criteria is configured - The following note shall be further clarified in the maintenance phase - Note: if the explicit signalling is configured to enable the RLM/BFD relaxation and fulfilment of the low mobility criteria, or if it means low mobility state of the UE may be implicitly inferred. #### Issue 2-1-3: whether to introduce explicit relaxation command for network to allow UE to apply the relaxed RLM/BFD requirements, irrespective evaluation result of the relaxation criteria - In R17 UE power saving, not to introduce explicit relaxation command for network to allow UE to apply the relaxed RLM/BFD requirements, irrespective evaluation result of the relaxation criteria. #### Issue 2-2-1: L3 CSI-RS to be used for Low mobility criteria - The existing agreement to use SSB based L3-RSRP measurement of the serving cell to evaluate the low mobility criterion is sufficient #### Issue 2-2-2: Clarification on the specific SSB to be measured for the low mobility criterion evaluation. - For R17 low mobility criterion, the existing definition of L3 SS-RSRP is used and there is no need to indicate specific SSB for low mobility criterion. #### Issue 2-2-3: Additional Low mobility criteria - In R17 UE power saving, no additional low mobility criterion is needed besides R16 low mobility criterion. #### Issue 2-2-4: Clarifications for Low mobility criteria evaluation - The low mobility criteria is evaluated on the - NR PCell for the case of NR single carrier, NR CA, NE-DC - NR PSCell for the case of EN-DC - NR PCell for the case of NR-DC - the low mobility criteria is evaluated on SpCell (s), regardless whether BFD is configured in Scell or not #### Issue 2-2-5: Clarification on L3 filtering for intra-frequency L3 RSRP measurement of serving cell is used for low mobility criterion - No RRM requirement impact with respect to the L3 filtering for intra-frequency L3 RSRP measurement of serving cell is used for low mobility criterion. Issue 2-3-1: For RLM, the predefined offset X dB - For RLM, confirm the predefined offset value X is 0 dB #### Issue 2-3-2: For RLM, other configurable values of offset X dB - For RLM, the offset X dB can be configured from a set of [2, 4, 6, 8] dB. #### Issue 2-3-3: For BFD, the reference threshold Qx and the predefined offset X - For BFD, confirm Qx = Qin and the predefined offset value X is 0 dB. #### Issue 2-3-4: For BFD, other configurable values of offset X dB - For BFD, the offset X dB can be configured from a set of [2, 4, 6, 8] dB. #### Issue 2-3-5: Configuration type of offset for the cell quality criteria - The offset X dB for the cell quality criteria is configured per UE. Separate values can be configured per FR. - The offset X dB can be configured separately for RLM and BFD #### Issue 2-5-1: Relaxation factors - The relaxation factor for FR1: - K1, FR1 = 2 for 40 ms < MAX(TDRX, TRS) ≤ 80 ms - K2, FR1 = 4 for MAX(TDRX, TRS) ≤ 40 ms - The relaxation factor for FR2 SSB: - K1, FR2, SSB= 2 for MAX(TDRX, TSSB) ≤ 80 ms. #### Issue 2-5-3: Additional N310/N311 values for relaxation mode - In R17 UE power saving, additional values of the RLF parameters, e.g. T310/N310/N311, for the relaxed operation are not supported. #### Issue 2-6-1: Specification for relaxation criteria - Capture the relaxation criteria for RLM/BFD relaxation in the RAN2 specifications Issue 2-6-2: Clarification on multiple RLM-RS/BFD-RS - The following can be clarified in the maintenance phase - For entering condition, - Option 1: the radio link quality of at least one RLM-RS is better than the entering threshold. (Huawei, Qualcomm, Xiaomi, Apple, vivo. OPPO) - Option 2: The UE is allowed to operate RLM/BFD in relaxed mode for a certain cell (SpCell or SCell) when the radio link quality is better than the threshold (e.g. Qout + X1) for all RLM-RS resource. (CMCC, Ericsson, CATT, Nokia, ZTE) - Option 3: pending by other issues (Intel) - For exiting condition, - Option 1: the radio link quality for all the RLM-RS resources are worse than the exiting threshold. (Huawei, Qualcomm, Apple, OPPO) - Option 2: The UE shall exit the relaxed mode when the radio link quality is worse than the threshold (e.g. Qout + X2) for any the RLM-RS resources. (CMCC, Ericsson, CATT, Nokia, ZTE) - Option 3: The UE behaviour on checking the exiting condition of cell quality criterion regarding multiple RLM-RSs/BFD-RSs is not specified. (vivo, Xiaomi) - Option 4: pending by other issues (Intel) #### Issue 2-6-4: Set P values to one - Reuse the legacy P factor when relaxed RLM/BFD requirements are applied. LS on RLM and BFD relaxation in R17 UE power saving (R4-2206787) Reply LS to RAN2 on RLM/BFD relaxation for ePowSav (R4-2207087)

Relating to objective of performance part Specify necessary RRM and UE demodulation performance requirements for the specified enhancements The following agreements are achieved:

RAN4 #102-e Meeting Approved: WF on RLM/BFD relaxation for UE Power Saving enhancements (R4-2206909): #### Issue 3-1: Performance requirements - No RRM requirement for R17 idle mode UE power saving enhancement. - FFS No additional accuracy requirement for serving cell quality criterion - FFS No need to define radio link monitoring in-sync test cases for RLM/BFD measurement relaxation #### Issue 3-2: General test configuration - FFS - Proposal 1: Design all test cases when both low mobility criterion and good serving cell quality criterion are configured and fulfils. Do not design the test cases for other cases such as low mobility criterion is not configured. (CATT, QC) #### Issue 3-3: DRX period setting - FFS - Option 1: Different DRX period can be configured for FR1 and FR2 test cases. (CMCC) - The example DRX values can be 40ms and 80ms. - Option 2: DRX period are the same for FR1 and FR2 test cases. (CATT, vivo) - DRX period is 40 ms. #### Issue 3-4: Test case list - The below test cases are provided for information. - For NR-SA No.; Test case; Note. RLM; 1; Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX=TBDms. 2; Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX= TBDms; Related to Issue 3-1. 3; Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX= TBDms. 4; Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX= TBDms; Related to Issue 3-1. 5; Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX= TBDms. 6; Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX= TBDms; Related to Issue 3-1. 7; Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX= TBDms. 8; Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX= TBDms; Related to Issue 3-1. BFD; 1; Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX= TBDms. 2; Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX= TBDms. 3; Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in DRX= TBDms. 4; Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in DRX= TBDms. - For EN-DC No.; Test case; Note. RLM; 1; Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with SSB-based RLM RS in DRX=TBDms. 2; Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX= TBDms; Related to Issue 3-1. 3; Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with CSI-RS-based RLM in DRX= TBDms. 4; Radio Link Monitoring In-sync Test for FR1 PSCell configured with CSI-RS-based RLM in DRX= TBDms; Related to Issue 3-1. 5; Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with SSB-based RLM RS in DRX= TBDms. 6; Radio Link Monitoring In-sync Test for FR2 PSCell configured with SSB-based RLM RS in DRX= TBDms; Related to Issue 3-1. 7; Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with CSI-RS-based RLM in DRX= TBDms. 8; Radio Link Monitoring In-sync Test for FR2 PSCell configured with CSI-RS-based RLM in DRX= TBDms; Related to Issue 3-1. BFD; 1; Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in DRX= TBDms. 2; Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with CSI-RS-based BFD and LR in DRX= TBDms. 3; Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with SSB-based BFD and LR in DRX= TBDms. 4; Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with CSI-RS-based BFD and LR in DRX= TBDms. #### Issue 3-5: Detailed test configuration - FFS - Proposal 1: reusing the corresponding legacy test with the following modifications: (Qualcomm) - RLM Out-of-sync SSB based non-DRx in FR1 in EN-DC - Configure offset to Qin for entering condition = 0dB to keep the SINR variation setting in the legacy test - Change D1 as KSSB, FR1 * 20 (T_SSB) * 2 (P) * 10 + 20 (T_SSB) * 2 (P) = 400 KSSB, FR1 + 40 (ms) - BFD CSI-RS based DRx in FR2 in NR-SA - Configure offset to Qin for entering condition = 0dB and set SNR1>Qin - Extend T3 by the additional delay allowed for BFD evaluation - Proposal 2: The number of time duration depends on exit threshold. (CMCC) - If exit threshold is Qout+ZdB, then at least 4 time durations should be set up. - For T1, the SINR is higher than Qin; For T2, the SINR is higher than Qout+ZdB and lower than Qin; for T3, the SINR is higher than Qout and lower than Qout+ZdB; For T4, the SINR is lower than Qout. - If exit threshold is Qout, then at least 3 time durations should be set up. - For T1, the SINR is higher than Qin; For T2, the SINR is higher than Qout and lower than Qin; for T3, the SINR is lower than Qout. - Proposal 3: The length of duration of D1 depends on exit threshold (CMCC) - If exit threshold is Qout+ZdB, the current D1 value can be reused. - If exit threshold is Qout, the D1 value should be relaxed. Approved Way Forward on demodulation requirements for power saving enhancement (R4-2207225): Issue 1: Whether to define requirements for paging early indication in idle/inactive-mode? - Agreement: Do not need to define requirements for paging early indication in idle/inactive-mode. Issue 2: Whether to define requirements for TRS occasion in idle/inactive-mode? - Agreement: Do not need to define requirements for TRS occasion in idle/inactive-mode. Issue 3: Whether to define requirements for PDCCH monitoring reduction in connected-mode? - Agreement: Do not need to define requirements for PDCCH monitoring reduction in connected-mode. Agreed WF: - Way Forward on demodulation requirements for power saving enhancement (R4-2207225)

#### 2.4.2	Remaining open issues

RAN4 has accomplished core-part design and will enter maintenance phase for the core-part design and resolve critical issue(s), if identified. RAN4 also started performance part of work and decided no demodulation requirement for Rel-17 UE power saving enhancements. RAN4 will continue performance-part work for the remaining details on RRM performance requirements for RLM/BFD relaxation.

## 2.5	RAN5

#### 2.5.1	Agreements: N/A (RAN5 is not involved in the WI)

#### 2.5.2	Remaining Open issues: N/A

#### 2.5.3	Remaining Open issues with cross-WG dependencies: N/A

## 2.6	RAN6

#### 2.6.1	Agreements: N/A (RAN6 is not involved in the WI)

#### 2.6.2	Remaining Open issues: N/A

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts: N/A

#### 3.1.2	Remaining Open issues with cross-TSG impacts: N/A

## 4.	References

- Previous status report(s) RP-213348 Status report for WI_UE Power Saving Enhancements for NR Rapporteur (MediaTek) RP-212612 Status report for WI_UE Power Saving Enhancements for NR Rapporteur (MediaTek) RP-211452 Status report for WI_UE Power Saving Enhancements for NR Rapporteur (MediaTek) RP-210733 Status report for WI_UE Power Saving Enhancements for NR Rapporteur (MediaTek) RP-202685 Status report for WI_UE Power Saving Enhancements for NR Rapporteur (MediaTek) RP-201701 Status report for WI_UE Power Saving Enhancements for NR Rapporteur (MediaTek)

RAN1 Contributions

RAN1 #107-bis-e Meeting - General: R1-2200809 Session notes for 8.7 (UE Power Saving Enhancements) Ad-hoc Chair (CMCC) (rev of R1-2200793) R1-2200834 Summary of email discussion on Rel-17 RRC parameters for UE power saving Moderator (MediaTek Inc.) R1-2200817 Collection of RAN1-related agreements for UE power saving enhancements for NR Rapporteur (MediaTek) R1-2200800 LS on Paging Enhancement RAN1 R1-2200788 [Draft] LS on Paging Enhancement MediaTek Inc. - Potential paging enhancements: R1-2200801 Summary#5 of Maintenance on Paging Enhancement Moderator (MediaTek Inc.) R1-2200787 Summary#4 of Maintenance on Paging Enhancement Moderator (MediaTek Inc.) R1-2200759 Summary#3 of Maintenance on Paging Enhancement Moderator (MediaTek Inc.) R1-2200743 Summary#2 of Maintenance on Paging Enhancement Moderator (MediaTek Inc.) R1-2200605 Summary#1 of Maintenance on Paging Enhancement Moderator (MediaTek Inc.) R1-2200032 Remaining issues on Paging enhancements for UE power saving in IDLE/inactive mode Huawei, HiSilicon R1-2200066 Remaining issues of power saving enhancements for paging ZTE, Sanechips R1-2200083 Remaining issues on paging enhancements for idle/inactive mode UE power saving vivo R1-2200149 Paging enhancement for UE power saving CATT R1-2200155 Remaining issues on paging enhancements for idle/inactive mode UE power saving TCL Communication Ltd. R1-2200201 Maintenance on paging enhancements Samsung R1-2200234 Discussion on paging enhancement NTT DOCOMO, INC. R1-2200276 Discussion on potential paging enhancements for UE power saving Spreadtrum Communications R1-2200298 Paging enhancements for idle and inactive UE power saving Qualcomm Incorporated R1-2200331 Further discussion on Paging enhancements for power saving OPPO R1-2200362 Open items on paging enhancements for UE power saving Nokia, Nokia Shanghai Bell R1-2200376 Discussion on the remaining aspects of paging enhancements Intel Corporation R1-2200397 On paging enhancement Panasonic R1-2200417 Remaining issues on paging enhancements for idle/inactive UEs Apple R1-2200463 Remaining issues on paging enhancement for power saving xiaomi R1-2200478 Remaining aspects of Paging Enhancements Ericsson R1-2200532 Paging enhancement for UE power saving Lenovo, Motorola Mobility R1-2200560 Discussion on PEI Design Transsion Holdings R1-2200575 Discussion on potential paging enhancements LG Electronics R1-2200585 Remaining issues on paging early indication CMCC R1-2200606 Maintenance on Design of Paging Early Indication MediaTek Inc. R1-2200609 On paging early indication Nordic Semiconductor ASA R1-2200618 Remaining issues on paging enhancements InterDigital, Inc. - TRS/CSI-RS occasion(s) for idle/inactive UEs R1-2200782 Moderator Summary #4 on TRS/CSI-RS occasion(s) for idle/inactive UEs Moderator (Samsung) R1-2200726 Moderator Summary #3 on TRS/CSI-RS occasion(s) for idle/inactive UEs Moderator (Samsung) R1-2200727 Moderator Summary #2 on TRS/CSI-RS occasion(s) for idle/inactive UEs Moderator (Samsung) R1-2200203 Moderator Summary #1 on TRS/CSI-RS occasion(s) for idle/inactive UEs Moderator (Samsung) R1-2200033 Remaining issue on assistance TRS occasions for IDLE/inactive mode Huawei, HiSilicon R1-2200067 Remaining issues of TRS for RRC idle and inactive UEs ZTE, Sanechips R1-2200084 Remaining issues on TRS/CSI-RS occasion(s) for idle/inactive UEs vivo R1-2200150 Remaining issues of TRS/CSI-RS for paging enhancement CATT R1-2200156 Remaining issues of TRS/CSI-RS occasion(s) for idle/inactive UEs TCL Communication Ltd. R1-2200202 Maintenance on TRS/CSI-RS occasion(s) for idle/inactive UEs Samsung R1-2200235 Discussion on TRS/CSI-RS occasion for idle/inactive UEs NTT DOCOMO, INC. R1-2200277 Discussion on TRS/CSI-RS occasion(s) for idle/inactive UEs Spreadtrum Communications R1-2200299 TRS and CSI-RS for idle and inactive UE power saving Qualcomm Incorporated R1-2200332 Further discussion on RS occasion for idle/inactive UEs OPPO R1-2200363 Open issues on n TRS information for IDLE/INACTIVE mode UEs Nokia, Nokia Shanghai Bell R1-2200377 Discussion on the remaining details of TRS/CSI-RS occasions in idle/inactive time Intel Corporation R1-2200398 Potential enhancements for TRS/CSI-RS occasion(s) for idle/inactive UEs Panasonic R1-2200418 Remaining issues on indication of TRS configurations for idle/inactive UEs Apple R1-2200464 Remaining issues on TRS/CSI-RS configuration and indication for idle/inactive UEs xiaomi R1-2200479 Remaining aspects of TRS occasion provisioning for Idle/Inactive UEs Ericsson R1-2200498 Remaining issues on TRS/CSI-RS occasions for idle/inactive UEs Sharp R1-2200533 Provision of TRS/CSI-RS for idle/inactive UEs Lenovo, Motorola Mobility R1-2200576 Discussion on TRS/CSI-RS occasion(s) for idle/inactive UEs LG Electronics R1-2200586 Remaining issues on TRS/CSI-RS occasion(s) for idle/inactive UEs CMCC R1-2200607 Maintenance on Design of TRS/CSI-RS Information for idle/inactive UEs MediaTek Inc. R1-2200610 On TRS design for idle/inactive UEs Nordic Semiconductor ASA R1-2200619 Remaining issues on TRS/CSI-RS occasions for idle/inactive UEs InterDigital, Inc. - Potential extension(s) to Rel-16 DCI-based power saving adaptation during DRX Active Time R1-2200758 FL summary#3 of DCI-based power saving adaptation Moderator (vivo) R1-2200697 FL summary#2 of DCI-based power saving adaptation Moderator (vivo) R1-2200696 FL summary#1 of DCI-based power saving adaptation Moderator (vivo) R1-2200034 Remaining issues on the extensions to Rel-16 DCI-based power saving adaptation for an active BWP Huawei, HiSilicon R1-2200069 Remaining issues of PDCCH monitoring adaptation during DRX Active Time ZTE, Sanechips R1-2200085 Remaining issues on DCI-based power saving adaptation in connected mode vivo R1-2200151 PDCCH monitoring adaptation CATT R1-2200167 Discussion on DCI-based power saving adaptation NEC R1-2200204 Maintenance on DCI-based power saving techniques Samsung R1-2200236 Discussion on extension to DCI-based power saving adaptation NTT DOCOMO, INC. R1-2200278 Discussion on power saving techniques for connected-mode UEs Spreadtrum Communications R1-2200300 DCI-based power saving adaptation during DRX ActiveTime Qualcomm Incorporated R1-2200333 DCI-based power saving adaptation solutions OPPO R1-2200364 Open issues on PDCCH monitoring adaptation for UE power saving Nokia, Nokia Shanghai Bell R1-2200378 Discussion on remaining aspects of power saving in active time Intel Corporation R1-2200399 Potential extension(s) to Rel-16 DCI-based power saving adaptation during DRX ActiveTime Panasonic R1-2200419 Remaining issues on enhanced DCI-based power saving adaptation Apple R1-2200465 Remaining issues on PDCCH monitoring adaptation for power saving xiaomi R1-2200481 Remaining aspects of active time power savings mechanisms Ericsson R1-2200534 Enhanced DCI based power saving adaptation Lenovo, Motorola Mobility R1-2200577 Discussion on DCI-based power saving adaptation during DRX ActiveTime LG Electronics R1-2200587 Remaining issues on PDCCH monitoring reduction during DRX active time CMCC R1-2200608 Maintenance on Enhancement of DCI-based PDCCH Monitoring Adaptation MediaTek Inc. R1-2200611 On PDCCH monitoring adaptation Nordic Semiconductor ASA R1-2200620 Remaining issues on DCI-based power saving adaptation InterDigital, Inc. - Others R1-2200068 Simulation results of UE power saving in RRC idle and inactive state ZTE, Sanechips R1-2200480 Modeling of NR gNB power consumption Ericsson R1-2200649 Analysis on power consumption for IDLE mode UE Huawei, HiSilicon R1-2200070 Further discussion on power saving for RRC connected UEs ZTE, Sanechips R1-2200482 Evaluation results for UE power saving schemes Ericsson R1-2200537 On the evaluation methodology of NR network power consumption MediaTek Inc.

RAN1 #108-bis-e Meeting - General: R1-2202787 Session notes for 8.7 (Maintenance on UE Power Saving Enhancements) Ad-hoc Chair (CMCC) R1-2202905 LS reply on PDCCH skipping RAN1 R1-2202896 [Draft] LS reply on PDCCH skipping RAN1 - Potential paging enhancements: R1-2202067 Summary#1 for Maintenance on Paging Enhancements MediaTek Inc. R1-2200944 Remaining issues on Paging enhancements for UE power saving in IDLE/inactive mode Huawei, HiSilicon R1-2201101 Remaining issues on paging enhancements for idle/inactive mode UE power saving vivo R1-2201130 Remaining issues of power saving enhancements for paging ZTE, Sanechips R1-2201280 Further discussion on Paging enhancements for power saving OPPO R1-2201370 Remaining issues of Paging enhancement for UE power saving CATT R1-2201484 Discussion on paging enhancement NTT DOCOMO, INC. R1-2201551 Discussion on potential paging enhancements for UE power saving Spreadtrum Communications R1-2201641 On remaining issues of paging enhancement Panasonic R1-2201705 Discussion on remaining aspects of the design of paging enhancements Intel Corporation R1-2201777 Remaining issues on paging enhancements for idle/inactive UEs Apple R1-2201865 Remaining issues on paging early indication CMCC R1-2201917 Remaining issues on paging enhancement for power saving Xiaomi R1-2202022 Maintenance on paging enhancements Samsung R1-2202068 Maintenance on Paging Enhancements MediaTek Inc. R1-2202148 Remaining issues on paging early indication design Qualcomm Incorporated R1-2202216 Maintenance for Paging Enhancements Ericsson R1-2202236 Discussion on PEI Design Transsion Holdings R1-2202247 Remaining issues on paging enhancements InterDigital, Inc. R1-2202328 Open items on paging enhancements for UE power saving Nokia, Nokia Shanghai Bell R1-2202346 Discussion on potential paging enhancements LG Electronics R1-2202384 On paging early indication Nordic Semiconductor ASA - TRS/CSI-RS occasion(s) for idle/inactive UEs R1-2202651 Moderator summary#3 on TRS/CSI-RS occasion(s) for idle/inactive UEs Moderator (Samsung) R1-2202650 Moderator summary#2 on TRS/CSI-RS occasion(s) for idle/inactive UEs Moderator (Samsung) R1-2202024 Moderator summary#1 on TRS/CSI-RS occasion(s) for idle/inactive UEs Moderator (Samsung) R1-2200945 Remaining issue on assistance TRS occasions for IDLE/inactive mode Huawei, HiSilicon R1-2201102 Remaining issues on TRS/CSI-RS occasion(s) for idle/inactive UEs vivo R1-2201131 Remaining issues of TRS for RRC idle and inactive UEs ZTE, Sanechips R1-2201281 Further discussion on RS occasion for idle/inactive UEs OPPO R1-2201371 Remaining issues of TRS/CSI-RS for paging enhancement CATT R1-2201485 Discussion on TRS/CSI-RS occasion for idle/inactive Ues NTT DOCOMO, INC. R1-2201552 Discussion on TRS/CSI-RS occasion(s) for idle/inactive UEs Spreadtrum Communications R1-2201642 Remaining issues on enhancements for TRS/CSI-RS occasion(s) for idle/inactive UEs Panasonic R1-2201706 Discussion on the remaining details of TRS/CSI-RS occasions in idle/inactive time Intel Corporation R1-2201778 Remaining issues on indication of TRS configurations for idle/inactive UEs Apple R1-2201866 Remaining issues on TRS/CSI-RS occasion(s) for idle/inactive UEs CMCC R1-2201918 Remaining issues on TRS/CSI-RS configuration and indication for idle/inactive UEs Xiaomi R1-2202023 Maintenance on TRS/CSI-RS occasion(s) for idle/inactive UEs Samsung R1-2202069 Maintenance on TRS/CSI-RS Information for idle/inactive UEs MediaTek Inc. R1-2202149 Remaining issues on idle and inactive TRS Qualcomm Incorporated R1-2202195 Remaining issues on TRS/CSI-RS occasions for idle/inactive UEs Sharp R1-2202217 Maintenance for TRS occasion provisioning for Idle/Inactive UEs Ericsson R1-2202248 Remaining issues on TRS/CSI-RS occasions for idle/inactive UEs InterDigital, Inc. R1-2202329 Open issues on n TRS information forto IDLE/INACTIVE mode UEs Nokia, Nokia Shanghai Bell R1-2202347 Discussion on TRS/CSI-RS occasion(s) for idle/inactive UEs LG Electronics R1-2202385 On TRS design for idle/inactive UEs Nordic Semiconductor ASA - Potential extension(s) to Rel-16 DCI-based power saving adaptation during DRX Active Time R1-2202894 FL summary#5 of DCI-based power saving adaptation Moderator (vivo) R1-2202739 FL summary#4 of DCI-based power saving adaptation Moderator (vivo) R1-2202738 FL summary#3 of DCI-based power saving adaptation Moderator (vivo) R1-2202574 FL summary#2 of DCI-based power saving adaptation Moderator (vivo) R1-2202573 FL summary#1 of DCI-based power saving adaptation Moderator (vivo) R1-2200946 Remaining issues on the extensions to Rel-16 DCI-based power saving adaptation for an active BWP Huawei, HiSilicon R1-2201103 Remaining issues on DCI-based power saving adaptation in connected mode vivo R1-2201133 Remaining issues of PDCCH monitoring adaptation during DRX Active Time ZTE, Sanechips R1-2201282 Remaining issues for DCI-based power saving adaptation OPPO R1-2201372 Remaining issues of PDCCH monitoring adaptation CATT R1-2201486 Discussion on extension to DCI-based power saving adaptation NTT DOCOMO, INC. R1-2201553 Discussion on power saving techniques for connected-mode UEs Spreadtrum Communications R1-2201613 Remaining issues on DCI-based power saving adaptation during DRX active time ETRI R1-2201643 Remaining issues for extensions to Rel-16 DCI-based power saving adaptation during DRX Active Time Panasonic R1-2201707 Discussion on remaining aspects of power saving in active time Intel Corporation R1-2201779 Remaining details on enhanced DCI-based power saving adaptation Apple R1-2201867 Remaining issues on PDCCH monitoring reduction during DRX active time CMCC R1-2201919 Remaining issues on PDCCH monitoring adaptation for power saving Xiaomi R1-2202025 Maintenance on DCI-based power saving techniques Samsung R1-2202070 Maintenance on DCI-based PDCCH Monitoring Adaptation MediaTek Inc. R1-2202094 Enhanced DCI based power saving adaptation Lenovo R1-2202150 DCI-based power saving adaptation during DRX ActiveTime Qualcomm Incorporated R1-2202219 Maintenance for active time power savings mechanisms Ericsson R1-2202249 Remaining issues on DCI-based power saving adaptation InterDigital, Inc. R1-2202330 Open issues on PDCCH monitoring adaptation for UE power saving Nokia, Nokia Shanghai Bell R1-2202348 Discussion on DCI-based power saving adaptation during DRX ActiveTime LG Electronics R1-2202365 Remaining issues on DCI-based power saving adaptation NEC R1-2202386 On PDCCH monitoring adaptation Nordic Semiconductor ASA - Others R1-2201132 Simulation results of UE power saving in RRC idle and inactive state ZTE, Sanechips R1-2202218 Modeling of NR gNB power consumption Ericsson R1-2202426 Evaluation of power saving gain for IDLE mode power saving enhancements in Rel-17 Huawei, HiSilicon R1-2201893 Discussion on power saving for RRC connected UE ZTE, Sanechips R1-2202071 Discussion on Rel-18 network energy savings MediaTek Inc. R1-2202220 Evaluation results for UE power saving schemes Ericsson R1-2202476 Discussion on UE assumption of TRS availability Huawei, HiSilicon

RAN2 Contributions

RAN2 #116-bis-e Meeting Organizational LS in R2-2200130 LS on further agreements on RLM and BFD relaxation for UE Power Saving enhancements (R4-2120314; contact: vivo, MediaTek) RAN4 LS out R2-2201960 LS on PDCCH Skipping in RRC_CONNECTED RAN2 To:RAN1 R2-2201989 LS to RAN4 on RLM/BFD relaxation for ePowSav RAN2 To:RAN4 Cc:RAN1 CRs R2-2200591 38.304 Running CR for ePowSav vivo draftCR Rel-17 38.304 16.7.0 NR_UE_pow_sav_enh-Core R2-2201157 38.300 running CR for introduction of UE power saving enhancements Huawei, HiSilicon draftCR Rel-17 38.300 16.8.0 NR_UE_pow_sav_enh-Core R2-2111491 R2-2201268 Update of 38.331 running CR for ePowSav with RAN1#107-e inputs CATT draftCR Rel-17 38.331 16.7.0 B NR_UE_pow_sav_enh-Core R2-2111657 Other R2-2201476 [Draft] LS on network control over the use of PEI Futurewei Technologies LS out Rel-17 NR_UE_pow_sav_enh-Core To:SA2 Cc:RAN3 Open Issues Paging Sub-grouping and Paging Early Indication R2-2201675 [Pre116bis][005][ePowSav] Summary of 8.9.2.1 Paging Sub-grouping and Paging Early Indication (MediaTek) MediaTek R2-2201916 Summary of [AT116bis-e][054][ePowSav] Subgrouping and PEI MediaTek Inc. R2-2200197 UE Identity based Paging Subgrouping Aspects Samsung Electronics R2-2200198 UE Identity for paging subgrouping with eDRX Samsung Electronics R2-2200199 Simultaneous support of UE Identity based and CN assigned Paging Subgrouping Samsung Electronics R2-2200239 Discussion on paging subgrouping OPPO R2-2200315 Open Issues for PEI and UE Paging Subgrouping MediaTek Inc. R2-2200455 Remaining open issues on subgrouping Intel Corporation R2-2200464 Discussing on Paging Sub-grouping and Paging Early Indication Beijing Xiaomi Mobile Software R2-2200592 Discussion on remaining issues on PEI and sub-grouping vivo R2-2200898 Considerations on remaining issues for paging subgrouping CMCC R2-2200899 Further considerations on UE assistance information CMCC R2-2200910 Discussion on paging subgrouping enhancements for idle/inactive-mode UE power saving Sony R2-2201102 On some remaining issues in 38.304 running CR for ePowSav Futurewei Technologies R2-2201153 Remaining issues on CN controlled subgrouping Huawei, HiSilicon, CMCC R2-2201155 PEI configuration and monitoring Huawei, HiSilicon R2-2201219 Further Consideration on Paging Subgrouping ZTE Corporation,Sanechips R2-2201221 Consideration on the UE capability for Paging Enhancement ZTE Corporation,Sanechips R2-2201269 Consideration on Paging Sub-grouping CATT R2-2201289 Discussion on coexistence of paging subgroup and multicast paging LG Electronics R2-2201290 Remaining issues on paging subgrouping LG Electronics R2-2201332 PEI monitoring area DENSO CORPORATION R2-2201339 Remaining details on subgrouping Nokia, Nokia Shanghai Bell R2-2201463 On network control over the use of PEI Futurewei Technologies R2-2201541 On the co-existence of UE-ID and CN assigned subgroups Interdigital, Inc. R2-2201542 UE assistance for CN assigned subgroups Interdigital, Inc. R2-2201555 PEI in last used cell Ericsson R2-2201557 Paging Early Indication and Subgroups Ericsson R2-2201543 Subgroup determination Interdigital, Inc. TRS/CSI-RS for idle/inactive R2-2201677 Summary of 8.9.2.2 TRS/CSI-RS for idle/inactive (CATT) CATT R2-2201918 Report of [AT116bis-e][055][ePowSav] TRS/CSI-RS for idle/inactive CATT R2-2200240 Discussion on TRS/CSI-RS applicability for eDRX UEs OPPO R2-2200466 Discussion on TRS CSI-RS for RRC-IDLE and RRC-INACTIVE State UE Beijing Xiaomi Mobile Software R2-2200593 Discussion on TRS CSI-RS in idle inactive mode vivo R2-2201204 R17 NR UE Power Save SIB-X sizing aspects Apple R2-2201220 Further Consideration on TRS for Idle and Inactive UE ZTE Corporation,Sanechips= R2-2201240 Discussion on TRS/CSI-RS and eDRX Sharp R2-2201270 TRS/CSI-RS for idle/inactive: leftover issues CATT R2-2201307 Discussion on TRS/CSI-RS for idle/inactive LG Electronics Finland R2-2201497 Potential TRS/CSI-RS occasion(s) Nokia, Nokia Shanghai Bell R2-2201556 TRS exposure Ericsson RLM/BFD relaxation R2-2201684 Summary of 8.9.2.3 RLM BFD relaxation vivo R2-2201941 Summary of [AT116bis-e][056][ePowSav] RLM/BFD relaxation (vivo) vivo R2-2200186 Issues on RLM-BFD relaxations Qualcomm Incorporated R2-2200241 Discussion on RAN2’s impact of RLM/BFD relaxation OPPO R2-2200381 Discussion on RLM_BFD measurement relaxation NEC Europe Ltd R2-2200451 Further considerations for RLM/BFD relaxation Intel Corporation R2-2200465 Discussion on RLM_BFD measurement relaxation Beijing Xiaomi Mobile Software R2-2200594 Discussion on configurations of RLM/BFD relaxation for power saving vivo R2-2201156 Discussion on RLM/BFD relaxation and DCI-based power saving adaptation Huawei, HiSilicon R2-2201271 Consideration on RLM and BFD relaxation CATT R2-2201544 RLM/BFD Relaxation Reporting Interdigital, Inc. R2-2201578 Discussion on RLM/BFD Relaxation LG Electronics Finland R2-2201614 On RLM/BFD relaxation Nokia, Nokia Shanghai Bell PDCCH skip R2-2200200 PDCCH Skipping in RRC_CONNECTED Samsung Electronics Co., Ltd R2-2200187 Enhancements for adaptive PDCCH monitoring Qualcomm Incorporated R2-2201222 Initial Discussion on DCI based Power Saving ZTE Corporation,Sanechips Further Enhancements R2-2200188 Subgrouping among paging occasions Qualcomm Incorporated UE Capabilities R2-2201681 Summary of AI 8.9.3: UE capabilities Intel R2-2200242 Discussion on UE capabilities OPPO R2-2200452 UE capability for Rel-17 UE power saving Intel Corporation R2-2200453 Draft running CR to 38331 on UE capabilities for Rel-17 UE power saving Intel Corporation draftCR Rel-17 38.331 16.7.0 B NR_UE_pow_sav_enh-Core R2-2200454 Draft running CR to 38306 on UE capabilities for Rel-17 UE power saving Intel Corporation draftCR Rel-17 38.306 16.7.0 B NR_UE_pow_sav_enh-Core R2-2200463 Discussing on UE capability for Paging enhancement Beijing Xiaomi Mobile Software R2-2200595 Discussion on capabilities for ePowSav vivo R2-2201154 UE capability design for paging subgrouping Huawei, HiSilicon R2-2201205 R17 NR UE Power Save UE capability aspects Apple R2-2201340 RAN2 impact on connected mode power saving Nokia, Nokia Shanghai Bell

RAN2 #117-e Meeting General LS in: R2-2202112 LS on UE capability for paging enhancement (R1-2200768; contact: Ericsson) RAN1 LS in Rel-17 To:RAN2 R2-2202115 LS on Paging Enhancement (R1-2200800; contact: MediaTek) RAN1 LS in Rel-17 To:RAN2 R2-2202168 LS on signalings for enabling RLM and BFD relaxation in R17 UE power saving (R4-2202769; contact: vivo) RAN4 LS in Rel-17 To:RAN2 Cc:RAN1 R2-2202306 Discussion on reply LS on signaling for RLM BFD relaxation vivo discussion Rel-17 NR_UE_pow_sav_enh-Core LS out: R2-2204240 LS out on PEI and UE Subgrouping RAN2 To:RAN3 Cc:SA2, CT1 R2-2204238 Reply LS to RAN4 on RLM/BFD relaxation for ePowSav RAN2 To:RAN4 CRs and Rapporteur Resolutions: R2-2202307 Introduction of ePowSav in TS 38.304 vivo (Rapporteur) CR Rel-17 38.304 16.7.0 0227 - B NR_UE_pow_sav_enh-Core R2-2202308 Discussion on type-3 open issues in TS 38.304 (Rapporteur resolutions) vivo discussion Rel-17 NR_UE_pow_sav_enh-Core R2-2203058 Introduction of ePowSav in TS 38.331 CATT CR Rel-17 38.331 16.7.0 2924 - B NR_UE_pow_sav_enh-Core R2-2203232 Introduction of UE power saving enhancements In 38.300 Huawei, HiSilicon CR Rel-17 38.300 16.8.0 0417 - B NR_UE_pow_sav_enh-Core Open issues Pre-discussions: R2-2202309 Summary of [Pre117-e][006][ePowSav] RLM BFD relaxation (vivo) vivo discussion Rel-17 NR_UE_pow_sav_enh-Core Late R2-2202664 Summary report of [Pre117-e][007][ePowSav] UE capabilities Intel Corporation discussion Rel-17 NR_UE_pow_sav_enh-Core Late R2-2202769 Summary of [Pre117-e][004][ePowSav] PEI and paging subgrouping (MediaTek) MediaTek Inc. discussion Late R2-2203059 Summary of [Pre117-e[005][ePowSav] TRS / CSI-RS Open Issues Input (CATT) CATT discussion Rel-17 NR_UE_pow_sav_enh-Core Late PEI and paging subgrouping: R2-2203720 Summary of 8.9.3.2.1 PEI and Paging Subgrouping MediaTek Inc. R2-2202279 Open issues for PEI and paging subgrouping NEC Europe Ltd R2-2202285 Open Issues for PEI and paging subgrouping Samsung Electronics Co., Ltd R2-2202286 UE Identity for paging subgrouping with eDRX Samsung Electronics Co., Ltd R2-2202310 Discussion on remaining open issues on PEI and subgrouping vivo R2-2202353 Discussing on PEI and paging subgrouping Xiaomi Communications R2-2202519 Open Issues in Enhanced NR UE Power Save PEI / Paging Subgrouping Apple R2-2202771 Open Issues for PEI and Paging Subgrouping MediaTek Inc. R2-2202881 PEI and subgrouping remaining issues Nokia, Nokia Shanghai Bell R2-2202882 Impact of subgrouping on other WGs Nokia, Nokia Shanghai Bell R2-2202993 Discussion on PEI and paging subgrouping OPPO R2-2203036 R2-22xxxxx Remaining issues on PEI LG Electronics Inc R2-2203229 Remaining issues on CN controlled subgrouping Huawei, HiSilicon R2-2203231 PEI with eDRX Huawei, HiSilicon R2-2203243 Considerations on PEI without Subgrouping Configuration ZTE Corporation,Sanechips R2-2203244 Considerations on PEI and Subgrouping Information in Xn and NG interface ZTE Corporation,Sanechips R2-2203245 Considerations on Open Issues of PEI and Subgrouping ZTE Corporation,Sanechips R2-2203252 PEI and paging subgrouping Ericsson R2-2203292 (OI 1.4) Considerations on support of PEI with eDRX Interdigital, Inc. R2-2203305 Remaining issue on PEI mobility Intel Corporation R2-2203474 Handling of gNB not supporting CN-assigned subgrouping Futurewei Technologies R2-2203478 On supporting PEI with eDRX Futurewei Technologies PDCCH Skip: R2-2203708 [Pre117-e][024][ePowSav] Summary of AI 8.9.3.2.2 PDCCH Skip (Samsung) Samsung Electronics Co., Ltd R2-2202287 PDCCH Skipping in RRC_CONNECTED Samsung Electronics Co., Ltd R2-2202311 Discussion on PDCCH Skipping in RRC_CONNECTED vivo R2-2202883 UL PUSCH transmission impact on PDCCH skipping Nokia, Nokia Shanghai Bell R2-2202994 Discussion on PDCCH skipping OPPO R2-2203230 PDCCH skipping while UL reTx timer is running Huawei, HiSilicon R2-2203253 DCI-based power saving adaptation during DRX Active Time Ericsson UE capabilities R2-2202355 Discussing on remaining issues of UE capability for paging enhancement Xiaomi Communications Other R2-2202312 Discussion on TRS availability when SI change vivo R2-2202354 Discussion on remaining issues on UE power saving Xiaomi Communications R2-2202779 Further considerations on UE assistance information CMCC R2-2202995 Discussion on PEI indication determination in RRC INACTIVE OPPO R2-2203068 TRS/CSI-RS configuration in RRC_CONNECTED DENSO CORPORATION R2-2203254 TRS exposure Ericsson

RAN3 Contributions

RAN3 #114-bis-e Meeting R3-220102 LS on paging subgrouping and PEI RAN2 R3-220235 Support of paging subgrouping for UE power saving (F1AP) CATT R3-220415 Support of paging subgrouping for UE power saving (F1AP) CATT R3-220236 Support of paging subgrouping for UE power saving (NGAP) CATT R3-220416 Support of paging subgrouping for UE power saving (NGAP) CATT R3-220564 Discussion on UE power saving Samsung R3-220565 CR to support UE power saving over F1 Samsung R3-220675 (TP for TS 38.413 and TS 38.473) Supporting UE Power Saving Enhancements Huawei R3-220767 Work plan for UE power saving enhancements WI ZTE,MediaTek Inc. R3-220897 Discussion on UE paging subgroup ZTE Corporation, MediaTek R3-220898 CR to TS38.413 for UE paging subgroup ZTE Corporation, MediaTek, China Unicom R3-220900 CR to TS38.423 for UE paging subgroup ZTE Corporation, MediaTek R3-220365 Support of UE Power Saving Enhancements Nokia, Nokia Shanghai Bell R3-220366 Support of UE Power Saving Enhancements Nokia, Nokia Shanghai Bell R3-221133 Support of UE Power Saving Enhancements Nokia, Nokia Shanghai Bell, Ericsson, Qualcomm Incorporated R3-220286 Addition of PEIPS Assistance Information Qualcomm Incorporated, Ericsson R3-220287 Addition of PEIPS Assistance Information Qualcomm Incorporated, Ericsson R3-220307 Addition of UE Subgrouping for paging Ericsson, Qualcomm Incorporated R3-220308 Addition of PEIPS Assistance Information Ericsson, Qualcomm Incorporated R3-221153 Addition of PEIPS Assistance Information Ericsson, Qualcomm Inc., Nokia, Nokia Shanghai Bell, Huawei, ZTE, Samsung, CATT R3-220899 CR to TS38.473 for UE paging subgroup ZTE Corporation, MediaTek, China Unicom R3-221156 CR to TS38.473 for UE paging subgroup ZTE, MediaTek, China Unicom, Nokia, Nokia Shanghai Bell, Ericsson, Huawei, Samsung, Qualcomm Incorporated, CATT R3-220676 Supporting UE Power Saving Enhancements Huawei R3-221261 Supporting UE Power Saving Enhancements Huawei R3-221268 (BL CR to TS 38.410) Support for ePowerSaving CATT, Nokia, Nokia Shanghai Bell, Ericsson, ZTE, Qualcomm Incorporated, Samsung R3-221315 (BL CR to TS 38.470) Support for UE Power Saving Enhancements Qualcomm Incorporated, Nokia, Nokia Shanghai Bell, Samsung, CATT, Huawei, Ericsson, ZTE R3-221010 CB: # 18_UEPowerSaving - Summary of email discussion ZTE - moderator R3-221147 CB: # 18_UEPowerSaving - Summary of email discussion ZTE - moderator R3-221428 CB: # 18_UEPowerSaving - Summary of email discussion ZTE - moderator

RAN3 #115-e Meeting R3-221594 (BL CR to TS 38.410) Support for ePowerSaving CATT, Nokia, Nokia Shanghai Bell, Ericsson, ZTE, Qualcomm Incorporated, Samsung R3-221745 (TP for XnAP BL CR on UE Power Savings) EN Removal on RAN Paging Qualcomm Incorporated R3-221822 (TP for TS 38.470 & TS 38.300) Stage 2 conclusions for UE Power Saving Enhancements Nokia, Nokia Shanghai Bell R3-221823 (TP for TS 38.473) Stage 3 conclusions for UE Power Saving Enhancements Nokia, Nokia Shanghai Bell R3-221971 (TP for Powersaving BLCR for TS 38.423) Supporting UE Power Saving Enhancements Huawei R3-221972 (TP for Powersaving BLCR for TS 38.473 and TS 38.413) Supporting UE Power Saving Enhancements Huawei R3-222196 Work plan for UE power saving enhancements WI ZTE,MediaTek Inc. R3-222250 Left issues on UE power saving enhancement ZTE Corporation, MediaTek Inc. R3-222522 TP to TS 38.300 BL CR: Addition of PEIPS over Xn Ericsson R3-222412 CB: # 20_UEPowerSaving - Summary of email discussion ZTE - moderator R3-222567 CB: # 20_UEPowerSaving - Summary of email discussion ZTE - moderator R3-222681 (TP for Powersaving BLCR for TS 38.470) Introduction of SIBx on TRS resources Huawei R3-222316 (TP to BL CR on TS38.473) Further discussion on UE power saving Samsung R3-222772 (TP to BL CR on TS38.473) Further discussion on UE power saving Samsung R3-222251 (TP to TS38.473 and TS38.423) paging subgroup ZTE Corporation, MediaTek Inc. R3-222849 (TP to TS38.423) paging subgroup ZTE Corporation, MediaTek Inc., Qualcomm Incorporated, Huawei R3-222201 (TP for TS 38.413) Support for UE power saving CATT R3-222850 (TP for TS 38.413) Support for UE power saving CATT, ZTE R3-222848 Reply LS on paging subgrouping and PEI ZTE R3-222874 Reply LS on paging subgrouping and PEI ZTE R3-221582 Addition of PEIPS Assistance Information Ericsson, Qualcomm Inc., Nokia, Nokia Shanghai Bell, Huawei, ZTE, Samsung, CATT R3-222964 Addition of PEIPS Assistance Information Ericsson, Qualcomm Inc., Nokia, Nokia Shanghai Bell, Huawei, ZTE, Samsung, CATT R3-221578 Support of UE Power Saving Enhancements Nokia, Nokia Shanghai Bell, Ericsson, Qualcomm Incorporated R3-222965 Support of UE Power Saving Enhancements Nokia, Nokia Shanghai Bell, Ericsson, Qualcomm Incorporated R3-221593 Supporting UE Power Saving Enhancements Huawei, Nokia, Nokia Shanghai Bell, Samsung, ZTE, Ericsson, Qualcomm Incorporated, CATT R3-222549 Supporting UE Power Saving Enhancements Huawei, Nokia, Nokia Shanghai Bell, Samsung, ZTE, Ericsson, Qualcomm Incorporated, CATT R3-222966 Supporting UE Power Saving Enhancements Huawei, Nokia, Nokia Shanghai Bell, Samsung, ZTE, Ericsson, Qualcomm Incorporated, CATT R3-221596 (BL CR to TS 38.470) Support for UE Power Saving Enhancements Qualcomm Incorporated, Nokia, Nokia Shanghai Bell, Samsung, CATT, Huawei, Ericsson, ZTE R3-222967 (BL CR to TS 38.470) Support for UE Power Saving Enhancements Qualcomm Incorporated, Nokia, Nokia Shanghai Bell, Samsung, CATT, Huawei, Ericsson, ZTE R3-221583 CR to TS38.473 for UE paging subgroup ZTE, MediaTek, China Unicom, Nokia, Nokia Shanghai Bell, Ericsson, Huawei, Samsung, Qualcomm Incorporated, CATT R3-222548 CR to TS38.473 for UE paging subgroup ZTE, MediaTek, China Unicom, Nokia, Nokia Shanghai Bell, Ericsson, Huawei, Samsung, Qualcomm Incorporated, CATT R3-222968 CR to TS38.473 for UE paging subgroup ZTE, MediaTek, China Unicom, Nokia, Nokia Shanghai Bell, Ericsson, Huawei, Samsung, Qualcomm Incorporated, CATT

RAN4 Contributions

RAN4 #101-bis-e Meeting General: R4-2202640 WF on RLM/BFD relaxation for UE Power Saving enhancements MediaTek Inc. R4-2202642 CR on TS38.133 for applicability of RLM measurement relaxation MediaTek inc. R4-2202643 DraftCR on SSB based relaxed RLM requirements Huawei, Hisilicon R4-2202644 draft CR on CSI-RS RLM requirements relaxation for R17 UE power saving vivo R4-2202647 Draft CR for TS 38.133 Minimum requirement for SSB based BFD for UE configured with relaxed measurement criterion CMCC R4-2202648 Draft CR Minimum requirement for CSI-RS based beam failure detection for UE configured with relaxed measurement criterion Xiaomi R4-2202649 Draft CR on relaxed measurement criteria for BFD CATT R4-2202759 Draft Big CR: RRM requirements Rel-17 NR UE Power Saving Enhancements MediaTek R4-2200599 LS on signalings for enabling RLM and BFD relaxation in R17 UE power saving vivo R4-2202641 LS on signalings for enabling RLM and BFD relaxation in R17 UE power saving Vivo, Mediatek R4-2202769 LS on signaling for enabling RLM and BFD relaxation in R17 UE power saving vivo RRM core requirements: R4-2200325 On Power Saving RRM Requirement Qualcomm, Inc. R4-2200530 Discussion on NR UE power saving for RLM and BM Intel Corporation UE measurements relaxation for RLM and/or BFD: R4-2200105 Further discussion on RLM/BFD relaxation for UE power saving enhancement CATT R4-2200106 Draft CR on relaxed measurement criteria for BFD CATT R4-2200258 UE measurements relaxation for RLM and/or BFD Apple R4-2200600 Discussion on RLM and BFD relaxation for NR UE power saving vivo R4-2200601 draft CR on CSI-RS RLM requirements relaxation for R17 UE power saving vivo R4-2200685 Further discussion on UE measurements relaxation for RLM and/or BFD Xiaomi R4-2200686 Draft CR Minimum requirement for CSI-RS based beam failure detection for UE configured with relaxed measurement criterion Xiaomi R4-2200797 Draft CR for TS 38.133 Minimum requirement for SSB based BFD for UE configured with relaxed measurement criterion CMCC R4-2200806 Discussion on RLM/BFD relaxation for NR power saving enhancement CMCC R4-2200896 Discussion about RLM/BFD measurement relaxation Nokia, Nokia Shanghai Bell R4-2200897 38.133 draft CR on RLM relaxation criteria Nokia, Nokia Shanghai Bell R4-2201143 Discussion on RRM requirements for R17 RLM/BFD relaxation OPPO R4-2201406 RLM and RLF relaxation for UE power saving ZTE Corporation R4-2201611 Discussion on UE measurement relaxation for RLM/BFD Huawei, Hisilicon R4-2201612 DraftCR on SSB based relaxed RLM requirements Huawei, Hisilicon R4-2201867 Discussions on UE power saving for RLM and BFD Ericsson R4-2201868 Draft CR: Applicability rule for relaxed BFD requirements Ericsson R4-2201962 Discussion on Rel-17 RLM/BFD measurement relaxation MediaTek inc. R4-2201963 CR on TS38.133 for applicability of RLM measurement relaxation MediaTek inc. Email discussion summary: R4-2202565 Email discussion summary for [101-bis-e][214] NR_UE_pow_sav_enh Moderator (MediaTek) R4-2202731 Email discussion summary for [101-bis-e][214] NR_UE_pow_sav_enh Moderator (MediaTek)

RAN4 #102-e Meeting RRM core requirements – WF/LS approved/endorsed: R4-2206790 LS on RLM/BFD relaxation for NR UE power saving enhancements MediaTek Inc. R4-2206909 WF on RLM/BFD relaxation for UE Power Saving enhancements MediaTek Inc R4-2206910 Draft CR Minimum requirement for CSI-RS based beam failure detection for UE configured with relaxed measurement criterion Xiaomi R4-2207087 Reply LS to RAN2 on RLM/BFD relaxation for ePowSav vivo RRM core requirements – 1st email discussion conclusions: R4-2206787 LS on RLM and BFD relaxation in R17 UE power saving MediaTek Inc R4-2205661 CR on TS38.133 for applicability of RLM measurement relaxation MediaTek inc, Ericsson R4-2205332 DraftCR on SSB based relaxed RLM requirements Huawei, HiSilicon R4-2204338 draft CR on CSI-RS RLM requirements relaxation for R17 UE power saving vivo R4-2204707 38.133 draft CR on RLM relaxation criteria Nokia, Nokia Shanghai Bell R4-2205636 Draft CR to TS 38.133: Applicability of relaxed BFD requirements Ericsson, MediaTek Inc. R4-2204533 Draft CR for TS 38.133 Minimum requirement for SSB based BFD for UE configured with relaxed measurement criterion CMCC R4-2203904 Draft CR on relaxed measurement criteria for BFD CATT R4-2205850 Clause title change on big CR Qualcomm communications-France RRM core requirements - 2nd email discussion conclusions: R4-2206909 WF on RLM/BFD relaxation for UE Power Saving enhancements MediaTek Inc R4-2206910 Draft CR Minimum requirement for CSI-RS based beam failure detection for UE configured with relaxed measurement criterion Xiaomi R4-2206911 CR on TS38.133 for applicability of RLM measurement relaxation MediaTek inc, Ericsson R4-2206912 DraftCR on SSB based relaxed RLM requirements Huawei, HiSilicon R4-2206913 draft CR on CSI-RS RLM requirements relaxation for R17 UE power saving vivo R4-2206914 Draft CR to TS 38.133: Applicability of relaxed BFD requirements Ericsson, MediaTek Inc. R4-2206915 Draft CR for TS 38.133 Minimum requirement for SSB based BFD for UE configured with relaxed measurement criterion CMCC R4-2205850 Clause title change on big CR Qualcomm communications-France RRM performance requirements: R4-2203722 On Power Saving RRM Performance Requirement Scope Qualcomm, Inc. R4-2203758 UE power saving enhancement: RRM performance requirements Apple R4-2203905 Discussion on RRM test cases for UE power saving enhancement CATT R4-2204534 Discussion on test cases for RLM/BFD measurement relaxation CMCC Demodulation performance requirements: R4-2207225 Way Forward on demodulation requirements for power saving enhancement MediaTek R4-2203759 UE power saving enhancement: demod performance requirements Apple R4-2204535 Discussion on demodulation for UE power saving enhancement CMCC R4-2205099 UE demodulation requirements for UE power saving enhancement Ericsson R4-2205770 Discussion on for power saving enhancement demod Huawei,HiSilicon R4-2205912 Views on demodulation requirements for power saving enhancements MediaTek inc.