3GPP TSG-WG1 Meeting #95-e	RP-220781

e-meeting, March 17 – 23, 2022

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | xxx | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Candidate resource reporting within SL DRX active time based on UE implementation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh-Core |  |  |  |  |  | Date: |  |  | 2022-03-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The following agreement is made in RAN1#107-e but UE behaviour is not captured in the specification. Agreement When SL DRX active time of Rx-UE is provided by the higher layer for candidate resource selection (including resource (re)selection and re-evaluation/pre-emption checking), the following working assumption is confirmed with option 2 as agreement (with modification in RED) Working Assumption (RAN1#106bis-e) When PHY layer is indicated with an active time of RX UE from MAC layer for candidate resource selection, a restriction is applied in PHY layer so that at least a subset of candidate resources reported to MAC layer is located within the indicated active time of the RX UE. The following options will be further discussed in RAN1 to restrict resources for candidate resource selection taking into account the indicated active time from MAC layer: - Option 2: PHY layer selects and reports candidate resources in which at least a subset of the candidate resources is within the indicated active time of the RX UE - FFS: Details on when the number of subsets of candidate resource is less than the threshold - FFS: The subset of candidate resource outside of the active time should consider each inactive time period - FFS: UE selection of resource selection window to overlap with indicated RX UE active time - FFS: Whether it is up to UE implementation to report candidate resources only within the indicated active time of the RX UE |  |  |  |  |  |  |  |  |
| Summary of change: |  | After step 7), if there is no candidate resource remained within the provided SL DRX active time of the set SA, it is up to UE implementation to select and include at least one candidate resource for reporting to higher layer. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unspecified UE behavior for reporting a subset candiate resources within SL DRX active time of RX UE |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 8.1.4	UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2

In resource allocation mode 2, the higher layer can request the UE to determine a subset of resources from which the higher layer will select resources for PSSCH/PSCCH transmission. To trigger this procedure, in slot n, the higher layer provides the following parameters for this PSSCH/PSCCH transmission:

-	the resource pool from which the resources are to be reported;

-	L1 priority, prioTX;

-	the remaining packet delay budget;

-	the number of sub-channels to be used for the PSSCH/PSCCH transmission in a slot, LsubCH;

-	optionally, the resource reservation interval, Prsvp_TX, in units of msec.

-	if the higher layer requests the UE to determine a subset of resources from which the higher layer will select resources for PSSCH/PSCCH transmission as part of re-evaluation or pre-emption procedure, the higher layer provides a set of resources (r0,r1,r2,…) which may be subject to re-evaluation and a set of resources (r0',r1',r2',…) which may be subject to pre-emption.

-	it is up to UE implementation to determine the subset of resources as requested by higher layers before or after the slot ri'' - T3, where ri'' is the slot with the smallest slot index among (r0,r1,r2,…) and (r0',r1',r2',…) , and T3 is equal to Tproc,1SL, where Tproc,1SL  is defined in slots in Table 8.1.4-2 where μSL is the SCS configuration of the SL BWP.

-	Optionally, the indication of resource selection mechanism(s), as allowedResourceSelectionConfig, which may comprise of full sensing only, partial sensing only, random resource selection only, or any combination(s) thereof.

The following higher layer parameters affect this procedure:

-	sl-SelectionWindowList: internal parameter T2min is set to the corresponding value from higher layer parameter sl-SelectionWindowList for the given value of prioTX.

-	sl-Thres-RSRP-List: this higher layer parameter provides an RSRP threshold for each combination pi, pj, where pi is the value of the priority field in a received SCI format 1-A and pj is the priority of the transmission of the UE selecting resources; for a given invocation of this procedure, pj = prioTX.

-	sl-RS-ForSensing selects if the UE uses the PSSCH-RSRP or PSCCH-RSRP measurement, as defined in clause 8.4.2.1.

-	sl-ResourceReservePeriodList

-	sl-SensingWindow: internal parameter T0 is defined as the number of slots corresponding to sl-SensingWindow msec

-	sl-TxPercentageList: internal parameter X for a given prioTX is defined as sl-TxPercentageList (prioTX) converted from percentage to ratio

-	sl-PreemptionEnable: if sl-PreemptionEnable is provided, and if it is not equal to 'enabled', internal parameter priopre is set to the higher layer provided parameter sl-PreemptionEnable.

-	Optionally, minimum number of Y slots as Y_min (minNumCandidateSlots), which indicates the minimum number of Y slots that are included in the resources corresponding to periodic-based partial sensing.

-	Optionally, minimum number of Y' slots as Y'min   ([TBD]), which indicates the minimum number of Y' slots that are included in the resources corresponding to contiguous partial sensing.

-	Optionally, sensing occasion as periodicSensingOccasionReservePeriodList, which indicates the subset of periodicity values from sl-ResourceReservePeriodList used to determine periodic sensing occasions in periodic-based partial sensing. If not configured, all periodicity values from sl-ResourceReservePeriodList are used to determine periodic sensing occasions in periodic-based partial sensing.

-	Optionally, additional sensing occasions as additionalPeriodicSensingOccasion, which indicates that UE additionally monitors periodic sensing occasions that correspond to a set of values. The possible values of the set at least includes the most recent sensing occasion before the first slot of the candidate slots for a given reservation periodicity and the last periodic sensing occasion prior to the most recent one for the given reservation periodicity. If not configured, the UE monitors the most recent sensing occasion before the first slot of the candidate slots for the given periodicity used to determine periodic sensing occasions in periodic-based partial sensing.

-	Optionally, indication of the size in logical slots of contiguous partial sensing window as contiguousSensingWindowPeriodic.

The resource reservation interval, Prsvp_TX, if provided, is converted from units of msec to units of logical slots, resulting in Prsvp_TX' according to clause 8.1.7.

When the resource pool is (pre-)configured with allowedResourceSelectionConfig including full sensing, and full sensing is (pre-)configured in the UE by higher layers, the UE performs full sensing.

When periodic reservation for another TB (sl-MultiReserveResource) is enabled for the resource pool, the resource pool is (pre-)configured with allowedResourceSelectionConfig including partial sensing, and partial sensing is (pre-) configured in the UE by higher layer, the UE performs periodic-based partial sensing.

When a UE is triggered by higher layer to report resources for resource (re-)selection in a mode 2 Tx pool, the resource pool is (pre-)configured with allowedResourceSelectionConfig including partial sensing, and partial sensing is configured by higher layer, the UE may perform contiguous partial sensing.

Notation:

t'0SL, t'1SL, t'2SL,… denotes the set of slots which belongs to the sidelink resource pool and is defined in Clause 8.

The following steps are used:

1)	A candidate single-slot resource for transmission Rx,y is defined as a set of LsubCH contiguous sub-channels with sub-channel x+j in slot t'ySL where j=0,...,LsubCH-1. The UE shall assume that any set of LsubCH contiguous sub-channels included in the corresponding resource pool within the time interval [n+T1,n+T2] correspond to one candidate single-slot resource for UE performing full sensing, in a set of Y candidate slots within the time interval n+T1,n+T2 for UE performing periodic-based partial sensing correspond to one candidate single-slot resource, or in a set of Y' candidate slots within the time interval [n+T1,n+T2] for UE performing contiguous partial sensing if Prsvp_TX=0, correspond to one candidate single-slot resource, where

-	selection of T1 is up to UE implementation under 0 ≤ T1≤ Tproc,1SL  , where Tproc,1SL  is defined in slots in Table 8.1.4-2 where μSL is the SCS configuration of the SL BWP;

-	if  T2min is shorter than the remaining packet delay budget (in slots) then T2 is up to UE implementation subject to  T2min ≤ T2 ≤ remaining packet delay budget (in slots); otherwise T2 is set to the remaining packet delay budget (in slots).

-	Y is selected by UE where Y≥Ymin.

-	Y' is selected by UE where Y'≥Ymin'. When the UE performs contiguous partial sensing and if Prsvp_TX=0, if the number of candidate single-slot resources Y' is smaller than Ymin', it is up to UE implementation to include other candidate slots.

The total number of candidate single-slot resources is denoted by Mtotal.

2)	The sensing window is defined by the range of slots [n –T0,n–Tproc,0SL), when the UE performs full sensing, where T0 is defined above and Tproc,0SL is defined in slots in Table 8.1.4-1 where μSL is the SCS configuration of the SL BWP. The UE shall monitor slots which belongs to a sidelink resource pool within the sensing window except for those in which its own transmissions occur. The UE shall perform the behaviour in the following steps based on PSCCH decoded and RSRP measured in these slots.

When the UE performs periodic-based partial sensing, the UE shall monitor slots at ty-k×PreserveSL, where tySL is a slot of the selected candidate slots. The UE shall perform the behaviour in the following steps based on PSCCH decoded and RSRP measured in these slots.

The value of Preserve corresponds to periodicSensingOccasionReservePeriodList if configured, otherwise, the values correspond to all periodicity from sl-ResourceReservePeriodList.

The UE monitors k sensing occasions determined by additionalPeriodicSensingOccasion, as previously described, and not earlier than n –T0. For a given periodicity Preserve, the values of k correspond to the most recent sensing occasion earlier than ty0SL-(Tproc,0SL+Tproc,1SL ) if additionalPeriodicSensingOccasion is not (pre-)configured, and additionally includes the value of k corresponding to the last periodic sensing occasion prior to the most recent one if additionalPeriodicSensingOccasion is (pre-)configured. ty0SL is the first slot of the selected Y candidate slots of PBPS.

When the UE performs periodic-based partial sensing and contiguous partial sensing with periodic reservation for another TB (sl-MultiReserveResource) enabled, the sensing window is defined by the range of slots [n+TA, n+TB]. n+TA is M consecutive logical slots earlier than slot ty0SL, and n+TB is Tproc,0SL+Tproc,1SL slots earlier than ty0SL, where ty0SL is the first slot of the selected Y candidate slots of PBPS, and Tproc,0SL, Tproc,1SL are in units of physical time/slots. If Prsvp_TX≠0  the value of M is (pre-)configured with the contiguousSensingWindowPeriodic. If contiguousSensingWindowPeriodic is not (pre-)configured, M equals to 31. When the minimum M slots for CPS cannot be guaranteed and when Prsvp_TX=0, it is up to UE implementation to either continue with step 3) or perform random selection.

When the UE performs contiguous partial sensing with periodic reservation for another TB (sl-MultiReserveResource) disabled and if Prsvp_TX=0, the sensing window is defined by the range of slots [n+TA, n+TB]. TA and TB are both selected such that the UE has sensing results starting at M consecutive logical slots before ty0SL and ending at Tproc,0SL+Tproc,1SL slots earlier than ty0SL. When the minimum M slots for CPS cannot be guaranteed and when Prsvp_TX=0, it is up to UE implementation to either continue with step 3) or perform random selection.

3)	The internal parameter Th(pi,pj)  is set to the corresponding value of RSRP threshold indicated by the i-th field in sl-Thres-RSRP-List, where i=pi+pj-1*8.

4)	The set SA is initialized to the set of all the candidate single-slot resources.

5)	The UE shall exclude any candidate single-slot resource Rx,y from the set SA if it meets all the following conditions:

-	the UE has not monitored slot t'mSL in Step 2.

-	for any periodicity value allowed by the higher layer parameter sl-ResourceReservePeriodList and a hypothetical SCI format 1-A received in slot t'mSL with 'Resource reservation period' field set to that periodicity value and indicating all subchannels of the resource pool in this slot, condition c in step 6 would be met.

5a)	If the number of candidate single-slot resources Rx,y remaining in the set SA is smaller than X⋅Mtotal, the set SA is initialized to the set of all the candidate single-slot resources as in step 4.

6)	The UE shall exclude any candidate single-slot resource Rx,y from the set SA if it meets all the following conditions:

a)	the UE receives an SCI format 1-A in slot t'mSL, and 'Resource reservation period' field, if present, and 'Priority' field in the received SCI format 1-A indicate the values Prsvp_RX and prioRX, respectively according to Clause 16.4 in [6, TS 38.213];

b)	the RSRP measurement performed, according to clause 8.4.2.1 for the received SCI format 1-A, is higher than ThprioRX,prioTX;

c)	the SCI format received in slot t'mSL or the same SCI format which, if and only if the 'Resource reservation period' field is present in the received SCI format 1-A, is assumed to be received in slot(s) t'm+q×Prsvp_RX'SL determines according to clause 8.1.5 the set of resource blocks and slots which overlaps with Rx,y+j×Prsvp_TX' for q=1, 2, …, Q and j=0, 1, …, Cresel-1. Here, Prsvp_RX' is Prsvp_RX converted to units of logical slots according to clause 8.1.7, Q=TscalPrsvp_RX  if Prsvp_RX< Tscal and  n'-m≤Prsvp_RX', where t'n'SL = n if slot n belongs to the set t'0SL,t'1SL,...,t'T'max-1SL, otherwise slot t'n'SL is the first slot after slot n belonging to the set t'0SL,t'1SL,...,t'T'max-1SL; otherwise Q=1. Tscal is set to selection window size T2 converted to units of msec.

6a)	This step is executed only if the procedure in clause 8.1.4A is triggered.

7)	If the number of candidate single-slot resources remaining in the set SA is smaller than X⋅Mtotal, then Th(pi,pj) is increased by 3 dB for each priority value Th(pi,pj) and the procedure continues with step 4.

7a)	If sidelink DRX active time of RX UE is provided by the higher layer and there is no candidate single-slot resource remained within the sidelink DRX active time in the set SA, the UE based on its implementation selects and includes at least one candidate single-slot resources within the sidelink DRX active time in the set SA.

The UE shall report set SA to higher layers.

========================Unchanged parts are omitted ========================