3GPP TSG-RAN Meeting #95e		RP-221009
Electronic meeting, March 17 – 23, 2022

# Title:	draft LS to ISED-Canada to clarify the RSS-195 requirement related to wireless communication service equipment operating in 2305-2320 MHz / 2345-2360 MHz frequency range

# Response to:

Source:	Telus (TSG-RAN)

To:	ISED-Canada

Cc:	3GPP RAN4

Contact Person:

Name:	Ivo Maljevic

E-mail Address:	ivo(dot)maljevic(at)telus(dot)com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None

1. Overall Description:

TSG RAN would like to kindly ask ISED-Canada for clarification regarding the RSS-195 regulatory requirements related to Wireless Communication Service (WCS) equipment operating in the Bands 2305-2320 MHz and 2345-2360 MHz.

The band (2305 - 2315 MHz UL / 2350 - 2360 MHz DL) was introduced by 3GPP to the LTE specification as Band 30 in Rel-12 and to the NR specification as Band n30 in Rel-16 specified according to the US regulatory FCC 27.53 limits (with A-MPR and NS_21 defined in the specification).  After analyzing both the NS_21 and ISED emission limits, 3GPP has arrived at the following differences between these

-	NS_21 limits state that a narrower resolution bandwidth (RBW) is permissible if RBW is >= 1% of the channel BW (outside channel blocks). Consequently, the specification can be -13dBm / (1% channel BW) for 0-1MHz from channel edge.

-	RSS-195 requirements state that RBW must be >= 1% of the channel BW (cannot be < 1%), and power must be integrated over 1MHz. Consequently, the specification is -13dBm/1MHz for 0-1MHz from the channel edge.

The fundamental difference is whether the emission requirement of -13 dBm is applied over 1% channel BW or over 1 MHz as illustrated in the figure below

This means that (existing) devices compatible with the 3GPP NS_21 limits for Band 30/n30 may not be compatible with the those of RSS-195. It is the RAN understanding that the limits in other bands (outside the first MHz) of RSS-195 are identical to the NS_21 limits. In view of this, RAN is seeking clarification as to the difference from the NS_21 for the first MHz from the viewpoint of compatibility with other services.

TSG RAN would like to kindly invite ISED-Canada to provide clarification and any additional guidance of the RSS-195 requirements.

Additionally, 3GPP would like to kindly ask ISED-Canada to consider providing feedback by TSG-RAN Meeting #96, to account for the Rel-17 timeline.

2. Actions:

TSG RAN kindly asks ISED-Canada for clarification and feedback as described above.

3. Date of Next TSG-RAN Meetings:

TSG-RAN Meeting #96		6. – 9. June 2022	Budapest, TBC

TSG-RAN Meeting #97e		12. – 15. September 2022	Electronic Meeting, TBC