3GPP TSG-RAN Meeting #95e		RP-221011
Electronic meeting, March 17 – 23, 2022

# Title:	draft LS to ISED-Canada to clarify the RSS-195 requirement related to wireless communication service equipment operating in 2305-2320 MHz / 2345-2360 MHz frequency range

# Response to:

Source:	Telus (TSG-RAN)

To:	ISED-Canada

Cc:	3GPP RAN4

Contact Person:

Name:	Ivo Maljevic

E-mail Address:	ivo(dot)maljevic(at)telus(dot)com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None

1. Overall Description:

TSG RAN would like to kindly ask ISED-Canada for clarification regarding the RSS-195 regulatory requirements related to Wireless Communication Service (WCS) equipment operating in the Bands 2305-2320 MHz and 2345-2360 MHz.

The band (2305 - 2315 MHz UL / 2350 - 2360 MHz DL) was introduced by 3GPP to the LTE specification as Band 30 in Rel-12 and to the NR specification as Band n30 in Rel-16 specified according to the US regulatory FCC 27.53 limits (with A-MPR and NS_21 defined in the specification).  After analyzing both the FCC 27.53 and ISED emission limits, 3GPP has arrived at the following differences between these:

-	FCC 27.53 limits state that a narrower resolution bandwidth (RBW) is permissible if RBW is >= 1% of the channel BW (outside channel blocks). Consequently, the specification can be -13dBm / (1% channel BW) for 0-1MHz from channel edge.

-	RSS-195 requirements state that RBW must be >= 1% of the channel BW (cannot be < 1%), and power must be integrated over 1MHz. Consequently, the specification is -13dBm/1MHz for 0-1MHz from the channel edge.

The fundamental difference is whether the emission requirement of -13 dBm is applied over 1% channel BW or over 1 MHz as illustrated in the figure below

This means that (existing) devices compatible with the FCC 27.53 limits for Band 30/n30 may not be compatible with the those of RSS-195. It is the RAN understanding that the limits in other bands (outside the first MHz) of RSS-195 are identical to the FCC 27.53 limits. In view of this, RAN is seeking clarification as to the difference from the FCC 27.53 for the first MHz from the viewpoint of compatibility with other services.

TSG RAN would like to kindly invite ISED-Canada to provide clarification and any additional guidance of the RSS-195 requirements.

Additionally, 3GPP would like to kindly ask ISED-Canada to consider providing feedback by TSG-RAN Meeting #96, to account for the Rel-17 timeline.

2. Actions:

TSG RAN kindly asks ISED-Canada for clarification and feedback as described above.

3. Date of Next TSG-RAN Meetings:

TSG-RAN Meeting #96		6. – 9. June 2022	Budapest, TBC

TSG-RAN Meeting #97e		12. – 15. September 2022	Electronic Meeting, TBC