|  | ASIA-PACIFIC TELECOMMUNITY |  |
| --- | --- | --- |
|  | The 29th Meeting of the APT Wireless Group (AWG-29) |  |
|  | 21 – 29 March 2022, Virtual/Online Meeting | 29 March 2022 |

Source: AWG-29/OUT-02

APT Wireless Group

Reply liaison statement to 3GPP RAN, 3GPP RAN4

| Title of the Liaison Statement: | FREQUENCY ARRANGEMENTs FOR IMT in the BAND 470 – 703 MHz |
| --- | --- |
| Objective: | For Information and comments |
| Origin: | APT Wireless Group (AWG) |
| Contacts: | Mr. Le Van Tuan, Chair, AWG |

The AWG would like to thank 3GPP TSG RAN for its liaison statement (RP-212629) and is pleased that 3GPP is continuing to consider the APT 600 MHz band plan.

AWG-29 has considered the information and issues from the 3GPP liaison statement regarding the development of the APT 600 MHz band plan. AWG would like to provide the following normative regulatory information as requested in the liaison statement:

- Frequency plan for allowed operation in the band

AWG has considered the options B1 and B2 and would like to make the following recommendations to 3GPP:

- That B1 is the preferred option for APT and be referred to as the ‘APT 600 MHz’ band.

- That AWG invites 3GPP to immediately start work on the technical specifications to support Option B1.

- That B2 may be considered as an option for later standardization should it be required (in a 35 MHz + 35 MHz configuration).

- Out of band emission limits

APT/AWG is not considering any band specific emission limit other than the general emission limits already specified 3GPP TS 38.101-1 and TS 38.104. However, it is not precluded additional emission limit may be introduced by each administration to protect the other services. We suggest that out of band emission limits for both categories A and B should be defined.

- Emission limits outside of the allocated channel, or emission mask, as applicable

APT/AWG is not considering any band specific emission limit other than the general emission limits already specified in 3GPP TS 38.101-1 and TS 38.104. From the network side, NS_35 is intended for US specific requirements and may not apply in all APT countries.  From the UE side, the device should ideally support both the APT band plan as well as Band n71 for roaming purposes.  As such, the UE should support NS_35 for operation in Band n71 networks.

The choice between categories A and B for specifying base station unwanted emissions is for each Administration.

- Protected services and associated emission limits

The AWG would like to maximise spectrum efficiency and minimise frequency separations (guard bands) between DTT and Mobile.

APT/AWG observes that 3GPP already studied them and summarized in TR 38.860 in the following.

- It was concluded that due to various DTV channel arrangements across national regulations, the frequency separation is considered to be decided by administrators. APT countries use 6, 7 and 8 MHz TV channels. We have undertaken simulations to determine guard band requirements for DTV interference to UE blocking.

- The AWG has undertaken a study on a 4 MHz frequency separation (guard band). The results of the study found that 4 MHz of frequency separation (guard band) is adequate for mobile user equipment to operate in the presence of DTT transmitters without interference (see details in the attached document). This means that option B1 would not impact any additional DTT channels on the 8 MHz and 6 MHz DTT channel spacing / raster compared with option B2.

- For all the frequency arrangements considered in this study item, it was agreed that no BS spurious emissions limits be specified for protection of RAS services. Any possible protections measured required for the RAS services protection are considered to be under the responsibility of each administration considering aspects such as distance separation site engineering solutions, etc.

The AWG invite 3GPP to consider the attached study when it is undertaking its work on the APT 600 MHz band plan.

- Blocking requirements, if applicable

APT/AWG is not considering any band specific blocking requirement other than the general blocking requirement already specified 3GPP TS 38.101-1 and TS 38.104.

- Maximum transmit power

APT/AWG is not considering any specific requirement other than the already available UE power classes in 3GPP TS 38.101-1 and base station classes in TS 38.104 for other NR bands.

Considering the level of urgency, the AWG would invite this work to be completed by December 2022 if feasible.

AWG would appreciate 3GPP to provide any feedback before the 30th AWG (5 -9 September 2022) and would also keep 3GPP informed of the progress work in future AWG meetings.

For technical queries please contact:

Dr Mansoor Shafi

Spark Fellow Wireless

Spark New Zealand Ltd

+64 274515186

Email: Mansoor.shafi@spark.co.nz

Mr. Le Van Tuan

Chair

APT Wireless Group