3GPP TSG-RAN5 Meeting #95-e	R5-222142

# Online, , 9th May 2022 - 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-1 | CR | 0253 | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New MCVideo Off-network Message Defaults 5.5.14 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NIST |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCenhUEConTest |  |  |  |  |  | Date: |  |  | 2022-04-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Default messages for MCVideo off-network signalling messages did not exist |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added new clause for MCVideo off-network message defaults |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test cases would not have accurate message contents which could lead to false failures of devices |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.14 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<START OF CHANGES>

### 5.5.14	Default MCVideo Call Control Off-network Messages and Other Information Elements

#### 5.5.14.1	GROUP CALL PROBE

Table 5.5.14.1-1: GROUP CALL PROBE from the UE to Other UEs

| Derivation Path: TS 24.281 [86] Table 17.1.2.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Group call probe message identity | "10000001" |  |  |
| MCVideo group ID | px_MCVideo_Group_A_ID |  |  |

#### 5.5.14.2	GROUP CALL ANNOUNCEMENT

Table 5.5.14.2-1: GROUP CALL ANNOUNCEMENT from the UE to other UEs

| Derivation Path: TS 24.281 [86] Table 17.1.3.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Group call announcement message Identity | "10000010" |  |  |
| Call identifier | a random number uniformly distributed between (0, 65535) generated at the beginning of a call establishment |  |  |
| Call type | "00000001" | Basic Group Call |  |
|  | "00000011" |  | EMERGENCY-CALL |
|  | "00000100" |  | IMMPERIL-CALL |
| Refresh interval | 10000 | The Refresh interval contains a number denoting the minimum time interval (milliseconds) between two successive periodic announcements. NOTE: TS 24.281 [26] clause 9.3.2.4.3.1 states that the refresh interval of the call is fixed to 10 seconds (10000 ms) |  |
| Call start time | The Call start time value is an unsigned integer containing UTC time of the time when a call was started, in seconds since midnight UTC of January 1, 1970 (not counting leap seconds). |  |  |
| Last call type change time | The Last call type change time value is an unsigned integer containing UTC time of the time when a call priority was changed, in seconds since midnight UTC of January 1, 1970 (not counting leap seconds). |  |  |
| MCVideo group ID | px_MCVideo_Group_A_ID |  |  |
| SDP | As described in TS36.579-1, Table 5.5.3.1.3-2 |  |  |
| Originating MCPTT user ID | px_MCVideo_ID_User_A | pre-set MCVideo user ID |  |
| Last user to change call type | The ID of the last user to change contents |  |  |
| Confirm mode indication | Present |  |  |
| Probe response | Not Present |  |  |

#### 5.5.14.3	GROUP CALL ACCEPT

Table 5.5.14.3-1: GROUP CALL ACCEPT from the UE to other UEs

| Derivation Path: TS 24.281 [86] Table 17.1.4.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Group call accept message identity | "10000011" |  |  |
| Call identifier | a random number uniformly distributed between (0, 65536) generated at the beginning of a call establishment |  |  |
| Call type | "00000001" | Basic Group Call |  |
|  | "00000011" |  | EMERGENCY-CALL |
|  | "00000100" |  | IMMPERIL-CALL |
| MCVideo group ID | px_MCVideo_Group_A_ID |  |  |
| Sending MCVideo user ID | px_MCVideo_ID_User_A |  |  |

#### 5.5.14.4	GROUP CALL EMERGENCY END

Table 5.5.14.4-1: GROUP CALL EMERGENCY END from the UE to other UEs

| Derivation Path: TS 24.281 [86] Table 17.1.13.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Group call emergency end message identity | "10000100" |  |  |
| Call identifier | a random number uniformly distributed between (0, 65536) generated at the beginning of a call establishment |  |  |
| Last call type change time | The Last call type change time value is an unsigned integer containing UTC time of the time when a call priority was changed, in seconds since midnight UTC of January 1, 1970 (not counting leap seconds). |  |  |
| Last user to change call type | px_MCVideo_ID_User_A | The ID of the last user to change contents |  |
| MCVideo group ID | px_MCVideo_Group_A_ID |  |  |
| Originating MCVideo user ID | px_MCVideo_ID_User_A |  |  |

#### 5.5.14.5	GROUP CALL IMMINENT PERIL END

Table 5.5.14.5-1: GROUP CALL IMMINENT PERIL END from the UE to other UEs

| Derivation Path: TS 24.281 [86] Table 17.1.12.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Group call imminent peril end message identity | "10000101" |  |  |
| Call identifier | a random number uniformly distributed between (0, 65536) generated at the beginning of a call establishment |  |  |
| Last call type change time | The Last call type change time value is an unsigned integer containing UTC time of the time when a call priority was changed, in seconds since midnight UTC of January 1, 1970 (not counting leap seconds). |  |  |
| Last user to change call type | px_MCVideo_ID_User_A | The ID of the last user to change contents |  |
| MCVideo group ID | px_MCVideo_Group_A_ID |  |  |
| Originating MCVideo user ID | px_MCVideo_ID_User_A |  |  |

#### 5.5.14.6	GROUP CALL BROADCAST

Table 5.5.14.6-1: GROUP CALL BROADCAST from the UE to other UEs

| Derivation Path: TS 24.281 [86] Table 17.1.18.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Group call broadcast message identity | "10000110" |  |  |
| Call identifier | a random number uniformly distributed between (0, 65536) generated at the beginning of a call establishment |  |  |
| Call type | "00000010" | Broadcast Group Call |  |
| Originating MCVideo user ID | px_MCVideo_ID_User_A |  |  |
| MCVideo group ID | px_MCVideo_Group_A_ID |  |  |
| SDP | As described in TS36.579-1, Table 5.5.3.1.3-2 |  |  |

#### 5.5.14.7	GROUP CALL BROADCAST END

Table 5.5.14.7.1-1: GROUP CALL BROADCAST END from the UE to other UEs

| Derivation Path: TS 24.281 [86] Table 17.1.19.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Group Call Broadcast end message identity | "10000111" |  |  |
| Call identifier | a random number uniformly distributed between (0, 65536) generated at the beginning of a call establishment |  |  |
| MCVideo group ID | px_MCVideo_Group_A_ID |  |  |
| Originating MCVideo user ID | px_MCVideo_ID_User_A |  |  |

#### 5.5.14.8	PRIVATE CALL SETUP REQUEST

Table 5.5.14.8-1: PRIVATE CALL SETUP REQUEST from the UE to another UE

| Derivation Path: TS 24.281 [86] Table 17.1.5.1-1. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Private call setup request message identity | "10001000" |  |  |
| Call identifier | a random number uniformly distributed between (0, 65536) generated at the beginning of a call establishment |  |  |
| Commencement mode | "00000000" | Automatic Commencement Mode |  |
| Call type | "00000101" | Private Call |  |
| MCVideo user ID of the caller | px_MCVideo_ID_User_A |  |  |
| MCVideo user ID of the callee | px_MCVideo_ID_User_B |  |  |
| SDP offer | As described in TS36.579-1, Table 5.5.3.1.3-2 with condition PRIVATE_CALL |  |  |
| User location | Not Present |  |  |

#### 5.5.14.9	PRIVATE CALL RINGING

Table 5.5.14.9-1: PRIVATE CALL RINGING from the UE to another UE

| Derivation Path: TS 24.281 [86] Table 17.1.6.1-1. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Private call ringing message identity | "10001001" |  |  |
| Call identifier | a random number uniformly distributed between (0, 65536) generated at the beginning of a call establishment |  |  |
| MCVideo user ID of the caller | px_MCVideo_ID_User_A |  |  |
| MCVideo user ID of the callee | px_MCVideo_ID_User_B |  |  |

#### 5.5.14.10	PRIVATE CALL ACCEPT

Table 5.5.14.10-1: PRIVATE CALL ACCEPT from the UE to another UE

| Derivation Path: TS 24.281 [86] Table 17.1.7.1-1. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Private call accept message identity | "10001010" |  |  |
| Call identifier | a random number uniformly distributed between (0, 65536) generated at the beginning of a call establishment |  |  |
| MCVideo user ID of the caller | px_MCVideo_ID_User_A |  |  |
| MCVideo user ID of the callee | px_MCVideo_ID_User_B |  |  |
| SDP answer | As described in TS36.579-1, Table 5.5.3.1.3-2 with condition PRIVATE_CALL |  |  |

#### 5.5.14.11	PRIVATE CALL REJECT

Table 5.5.5.11.1-1: PRIVATE CALL REJECT from the UE to another UE

| Derivation Path: TS 24.281 [86] Table 17.1.8.1-1. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Private call reject message identity | "10001011" |  |  |
| Call identifier | a random number uniformly distributed between (0, 65536) generated at the beginning of a call establishment |  |  |
| Reason | "00000000" | 00000000 = REJECT; 00000001 = MEDIA FAILURE; 00000010 = BUSY; 00000011 = E2E SECURITY CONTEXT FAILURE; 00000100 = FAILED |  |
| MCVideo user ID of the caller | px_MCVideo_ID_User_A |  |  |
| MCVideo user ID of the callee | px_MCVideo_ID_User_B |  |  |

#### 5.5.14.12	PRIVATE CALL RELEASE

Table 5.5.14.12-1: PRIVATE CALL RELEASE from the UE to another UE

| Derivation Path: TS 24.281 [86] Table 17.1.9.1-1. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Private call release message identity | "10001100" |  |  |
| Call identifier | a random number uniformly distributed between (0, 65536) generated at the beginning of a call establishment |  |  |
| MCVideo user ID of the caller | px_MCVideo_ID_User_A |  |  |
| MCVideo user ID of the callee | px_MCVideo_ID_User_B |  |  |

#### 5.5.14.13	PRIVATE CALL RELEASE ACK

Table 5.5.14.13-1: PRIVATE CALL RELEASE ACK from the UE to another UE

| Derivation Path: TS 24.281 [86] Table 17.1.10.1-1. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Private call release ack message identity | "10001101" |  |  |
| Call identifier | a random number uniformly distributed between (0, 65536) generated at the beginning of a call establishment |  |  |
| MCVideo user ID of the caller | px_MCVideo_ID_User_A |  |  |
| MCVideo user ID of the callee | px_MCVideo_ID_User_B |  |  |

#### 5.5.14.14	PRIVATE CALL ACCEPT ACK

Table 5.5.14.14-1: PRIVATE CALL ACCEPT ACK from the UE to another UE

| Derivation Path: TS 24.281 [86] Table 17.1.11.1-1. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Private call accept ack message identity | "10001110" |  |  |
| Call identifier | a random number uniformly distributed between (0, 65536) generated at the beginning of a call establishment |  |  |
| MCVideo user ID of the caller | px_MCVideo_ID_User_A |  |  |
| MCVideo user ID of the callee | px_MCVideo_ID_User_B |  |  |

#### 5.5.14.15	GROUP EMERGENCY ALERT

Table 5.5.14.15.1-1: GROUP EMERGENCY ALERT from the UE to other UEs

| Derivation Path: TS 24.281 [86] Table 17.1.14.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Group emergency alert message identity | "10001111" |  |  |
| MCVideo group ID | px_MCVideo_Group_A_ID |  |  |
| Originating MCVideo user ID | px_MCVideo_ID_User_A |  |  |
| Organization name | px_MCX_DomainName_Organization_A |  |  |
| User location | Not Present |  |  |
| User location |  |  | USER_LOC |
| Latitude | any allowed value |  |  |
| Longitude | any allowed value |  |  |
| Altitude | Not present, or any allowed value | Optional IE |  |
| Accuracy | any allowed value |  |  |
| Timestamp | any allowed value |  |  |

| Condition | Explanation |
| --- | --- |
| USER_LOC | If requested, shall set the location IE with UE (MCPVideo Client) current location |

#### 5.5.14.16	GROUP EMERGENCY ALERT ACK

Table 5.5.14.16.1-1: GROUP EMERGENCY ALERT ACK from the UE to other UEs

| Derivation Path: TS 24.281 [86] Table 17.1.15.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Group emergency alert ack message identity | "10010000" |  |  |
| MCVideo group ID | px_MCVideo_Group_A_ID |  |  |
| Originating MCVideo user ID | px_MCVideo_ID_User_B |  |  |
| Sending MCVideo user ID | px_MCVideo_ID_User_A |  |  |

#### 5.5.14.17	GROUP EMERGENCY ALERT CANCEL

Table 5.5.14.17.1-1: GROUP EMERGENCY ALERT CANCEL from the UE to other UEs

| Derivation Path: TS 24.281 [86] Table 17.1.16.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Group emergency alert cancel message identity | "10010001" |  |  |
| MCVideo group ID | px_MCVideo_Group_A_ID |  |  |
| Originating MCVideo user ID | px_MCVideo_ID_User_A |  |  |
| Sending MCVideo user ID | px_MCVideo_ID_User_A |  |  |

#### 5.5.14.18	GROUP EMERGENCY ALERT CANCEL ACK message

Table 5.5.14.18.1-1: GROUP EMERGENCY ALERT CANCEL ACK from the UE to other UEs

| Derivation Path: TS 24.281 [86] Table 17.1.17.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Group emergency alert cancel ack message identity | "10010010" |  |  |
| MCVideo group ID | px_MCVideo_Group_A_ID |  |  |
| Originating MCVideo user ID | px_MCVideo_ID_User_A |  |  |
| Sending MCVideo user ID | px_MCVideo_ID_User_B |  |  |

#### 5.5.14.19	PRIVATE REMOTE VIDEO PUSH REQUEST message

Table 5.5.14.19-1: PRIVATE REMOTE VIDEO PUSH REQUEST from the UE to another UE

| Derivation Path: TS 24.381 [86] Table 17.1.20.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Remote video push setup request message identity | "10010011" |  |  |
| Call identifier | a random number uniformly distributed between (0, 65535) generated at the beginning of a call establishment |  |  |
| MCVideo remote push requester | px_MCVideo_ID_User_A | TS 24.281, Section 13.3.2.2.1 |  |
| MCVideo remote push call originator | px_MCVideo_ID_User_A | The stored caller ID |  |
| MCVideo remote push call recipient | px_MCVideo_ID_User_B | The stored callee ID |  |
| Video Information | The Video Information IE is used to indicate the source (user/group) of the video being pushed. | TS 24.281, Sections 13.3.2.2.1 and 17.2.17, Figure 17.2.17-1, Tables 17.2.17-1 and 17.2.17-2. |  |
| Source ID type | "00000000" | user ID |  |
| Length of Source ID contents |  |  |  |
| Source ID | px_MCVideo_ID_User_A |  |  |

#### 5.5.14.20	GROUP REMOTE VIDEO PUSH REQUEST message

Table 5.5.14.20-1: GROUP REMOTE VIDEO PUSH REQUEST from the UE to another UE

| Derivation Path: TS 24.281 [86] Table 17.1.21.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Remote video push setup request message identity | "10010100" |  |  |
| Call identifier | a random number uniformly distributed between (0, 65535) generated at the beginning of a call establishment |  |  |
| MCVideo remote push requester | px_MCVideo_ID_User_A |  |  |
| MCVideo remote push call originator | px_MCVideo_ID_User_A | The stored caller ID |  |
| MCVideo remote push call recipient | px_MCVideo_Group_A_ID | The stored group recipient ID |  |
| Video Information |  | The Video Information IE is used to indicate the source (user/group) of the video being pushed. |  |
| Source ID type | "00000001" | group ID |  |
| Length of Source ID contents |  |  |  |
| Source ID | px_MCVideo_Group_A_ID |  |  |

#### 5.5.14.21	VIDEO PUSH TRYING RESPONSE message

Table 5.5.14.21-1: VIDEO PUSH TRYING RESPONSE from UE to other UE

| Derivation Path: TS 24.281 [86] Table 17.1.22.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Remote video push trying response message identity | "10010101" |  |  |
| Call identifier | a random number uniformly distributed between (0, 65535) generated at the beginning of a call establishment |  |  |

#### 5.5.14.22	NOTIFY VIDEO PUSH message

Table 5.5.14.22-1: NOTIFY VIDEO PUSH message content

| Derivation Path: TS 24.281 [86] Table 17.1.23.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Remote video push notification message identity | "10010110" |  |  |
| Call identifier | a random number uniformly distributed between (0, 65535) generated at the beginning of a call establishment |  |  |
| Result | "00000000" | 00000000 = SUCCESS 00000001 =FAILURE |  |
| MCVideo remote push request notifier | px_MCVideo_ID_User_A | TS 24.281, section 13.3.2.2.6 |  |
| MCVideo remote push request notification recipient | px_MCVideo_ID_User_B |  |  |
| MCVideo remote push call recipient user | Not present |  |  |
|  | px_MCVideo_ID_User_A |  | PRIVATE-CALL |
| MCVideo remote push call recipient group | Not present |  |  |
|  | px_MCVideo_Group_A_ID |  | GROUP-CALL |
| Reason | Not present |  |  |

<END OF CHANGES>