3GPP TSG-RAN5 Meeting #95-e	R5-222364

# Electronic Meeting, 9th – 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-2 | CR | 0294 | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of test case 6.1.4.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCenhUEConTest |  |  |  |  |  | Date: |  |  | 2022-04-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - MCPTT user and group ids are sensitive information and therefore need to be ciphered - According to 24.379 clause 10.1.4.3.1 the originating participating function shall set the <mcptt-calling-user-id> in the mcptt-info to the MCPTT ID of the calling user and neither the controlling function nor the terminating participating function removes it again. - In general the are no MCPTT IDs in SIP header fields but their corresponding public user id is used. - According to 24.379 clause 10.1.4.4 the controlling function shall set g.3gpp.mcptt and g.3gpp.icsi-ref in the Accept-Contact header field of the SIP message - Editorial issues |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarification added to Table 6.1.4.1.3.3-2 and Table 6.1.4.1.3.3-5 that MCPTT user and group ids are ciphered - <mcptt-calling-user-id> added to Table 6.1.4.1.3.3-5 - px_MCPTT_ID_User_C replaced by px_MCX_SIP_PublicUserId_C in the P-Asserted-Identity in Table 6.1.4.1.3.3-4 - Condition ACCEPT-CONTACT-WITH-MEDIA-FEATURE-TAG added to Table 6.1.4.1.3.3-4 - Editorial corrections |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Invalid, inconsistent test specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 6.1.4.1	On-network / Remote Change of Selected Group / Selected Group Change of Targeted User / Client Originated (CO)

6.1.4.1.1	Test Purpose (TP)

(1)

with { UE (MCPTT Client) registered and authorised for MCPTT Service }

ensure that {

when { the MCPTT User requests to send a group selection change request to change the selected group of a targeted MCPTT user to a specific MCPTT group }

then { UE (MCPTT Client) sends a SIP MESSAGE message and responds to a SIP MESSAGE message with a SIP 200 (OK) message }

}

6.1.4.1.2	Conformance Requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.379 clause 10.1.4.2.1. Unless otherwise stated these are Rel-15 requirements.

[TS 24.379, clause 10.1.4.2.1]

Upon receiving a request from the MCPTT user to send a group selection change request to change the selected group of a targeted MCPTT user to a specific MCPTT group, the MCPTT client:

1)	if:

a)	the <RemoteGroupSelectionURIList> element does not exist in the MCPTT user profile document with one or more <entry> elements (see the MCPTT user profile document in 3GPP TS 24.484 [50]); or

b)	the <RemoteGroupSelectionURIList> element exists in the MCPTT user profile document and the MCPTT ID of the targeted MCPTT user does not match with one of the <entry> elements of the <RemoteGroupSelectionURIList> element of the MCPTT user profile document (see the MCPTT user profile document in 3GPP TS 24.484 [50]);

then:

a)	should indicate to the requesting MCPTT user that they are not authorised to change the selected MCPTT group of the targeted MCPTT user; and

b)	shall skip the rest of the steps of the present clause;

2)	shall generate a SIP MESSAGE request in accordance with 3GPP TS 24.229 [4] and IETF RFC 3428 [33] with the following clarifications:

a)	shall include the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcptt" (coded as specified in 3GPP TS 24.229 [4]), in a P-Preferred-Service header field according to IETF RFC 6050 [9] in the SIP MESSAGE request;

b)	shall include an Accept-Contact header field with the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcptt" along with the "require" and "explicit" header field parameters according to IETF RFC 3841 [6];

c)	may include a P-Preferred-Identity header field in the SIP MESSAGE request containing a public user identity as specified in 3GPP TS 24.229 [4]; and

d)	shall include an application/vnd.3gpp.mcptt-info+xml MIME body as specified in clause F.1 with the <mcpttinfo> element containing the <mcptt-Params> element with the <anyExt> element containing:

i)	the <mcptt-request-uri>  set to the MCPTT group identity to be selected by the targeted MCPTT user; and

ii)	the <request-type> element set to a value of "group-selection-change-request"; and

e)	shall insert in the SIP MESSAGE request a MIME resource-lists body with the MCPTT ID of the targeted MCPTT user, according to rules and procedures of IETF RFC 5366 [20];

3)	shall set the Request-URI to the public service identity identifying the participating MCPTT function serving the MCPTT user; and

4)	shall send the SIP MESSAGE request according to rules and procedures of 3GPP TS 24.229 [4].

Upon receipt of a SIP 4xx, 5xx or 6xx response to the SIP MESSAGE request, should indicate to the MCPTT user the failure of the sent group selection change request and not continue with the rest of the steps.

Upon receiving a "SIP MESSAGE request for group selection change response for terminating client", the MCPTT client:

1)	shall determine the success or failure of the sent group selection change request from the value of the <selected-group-change-outcome> element contained in the <anyExt> element of the <mcptt-Params> element  of the <mcpttinfo> element of the application/vnd.3gpp.mcptt-info+xml MIME body included in the received SIP MESSAGE request; and

2)	should indicate to the MCPTT user the success or failure of the sent group selection change request.

6.1.4.1.3	Test description

6.1.4.1.3.1	Pre-test conditions

System Simulator:

-	SS (MCPTT server)

-	For the underlying "transport bearer" over which the SS and the UE will communicate Parameters are set to the default parameters for the basic E-UTRA Single cell network scenarios, as defined in TS 36.508 [24] clause 4.4. The simulated Cell 1 shall belong to PLMN1 (the PLMN specified for MCPTT operation in the MCPTT configuration document).

IUT:

-	UE (MCPTT client)

-	The test USIM set as defined in TS 36.579-1 [2], subclause 5.5.10, is inserted.

Preamble:

-	The UE has performed the Generic Test Procedure for MCPTT UE registration as specified in TS 36.579-1 [2], subclause 5.4.2.

-	The MCPTT User performs the procedure for MCPTT Authorization/Configuration and Key Generation as specified in TS 36.579-1 [2], subclause 5.3.2.

-	UE States at the end of the preamble

-	The UE is in E-UTRA Registered, Idle Mode state.

-	The MCPTT Client Application has been activated and User has registered-in as the MCPTT User with the Server as active user at the Client.

6.1.4.1.3.2	Test procedure sequence

Table 6.1.4.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Make the MCPTT User request to send a group selection change request to change the selected group of a targeted MCPTT user to a specific MCPTT group. (NOTE 1) | - | - | - | - |
| 2 | Check: Does the UE (MCPTT Client) correctly perform procedure for MCX SIP MESSAGE Request - Accept CO as described in TS 36.579-1 [2] Table 5.3.30.3-1 to change the selected group of a targeted MCPTT user to a specific MCPTT group? | - | - | 1 | - |
| NOTE 1: This is expected to be done via a suitable implementation dependent MMI. |  |  |  |  |  |

6.1.4.1.3.3	Specific message contents

Table 6.1.4.1.3.3-1: SIP MESSAGE (Step 2, Table 6.1.4.1.3.2-1;
Step 2, TS 36.579-1 [2] Table 5.3.30.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.1-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCPTT Info | TS 24.379 [9] clause F.1 |  |
| MIME-part-body | MCPTT-Info as described in Table 6.1.4.1.3.3-2 |  |  |  |
| MIME body part |  | Resource lists |  |  |
| MIME-part-body | Resource-lists as described in Table 6.1.4.1.3.3-3 |  |  |  |

Table 6.1.4.1.3.3-2: MCPTT-Info in SIP MESSAGE (Table 6.1.4.1.3.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.1-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcpttinfo |  |  |  |  |
| mcptt-Params |  |  |  |  |
| anyExt |  |  |  |  |
| mcptt-request-uri | Encrypted (NOTE 1) <mcptt-request-uri> with mcpttURI set to px_MCPTT_Group_B_ID | The MCPTT group identity to be selected by the targeted MCPTT user | TS 24.379 [9] clause 10.1.4.2.1 |  |
| request-type | "group-selection-change-request" |  | TS 24.379 [9] clause 10.1.4.2.1 |  |
| NOTE 1: Encrypted element as described in TS 36.579-1 [2] Table 5.5.3.2.1-1A. |  |  |  |  |

Table 6.1.4.1.3.3-3: Resource lists in SIP MESSAGE (Table 6.1.4.1.3.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.3.1-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| resource-lists |  |  |  |  |
| list |  |  |  |  |
| entry[1] |  |  |  |  |
| uri attribute | px_MCPTT_ID_User_C | The targeted MCPTT user | TS 24.379 [9] clause 10.1.4.2.1 |  |

Table 6.1.4.1.3.3-4: SIP MESSAGE (Step 2, Table 6.1.4.1.3.2-1;
Step 4, TS 36.579-1 [2] Table 5.3.30.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.2-1 condition ACCEPT-CONTACT-WITH-MEDIA-FEATURE-TAG |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| P-Asserted-Identity |  |  |  |  |
| name-addr | px_MCX_SIP_PublicUserId_C |  | TS 24.379 [9] clause 10.1.4.4 |  |
| Message-body |  |  |  |  |
| MIME body part |  | MCPTT Info |  |  |
| MIME-part-body | MCPTT-Info as described in Table 6.1.4.1.3.3-5 |  |  |  |

Table 6.1.4.1.3.3-5: MCPTT-Info in SIP MESSAGE (Table 6.1.4.1.3.3-4)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.2-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcpttinfo |  |  |  |  |
| mcptt-Params |  |  |  |  |
| mcptt-calling-user-id | Encrypted (NOTE 1) <mcptt-calling-user-id> with mcpttURI set to px_MCPTT_ID_User_C |  |  |  |
| mcptt-calling-group-id | Encrypted (NOTE 1) <mcptt-calling-group-id> with mcpttURI set to px_MCPTT_Group_B_ID |  |  |  |
| anyExt |  |  |  |  |
| mcptt-request-uri | Encrypted (NOTE 1) <mcptt-request-uri> with mcpttURI set to px_MCPTT_Group_B_ID | The MCPTT group identity to be selected by the targeted MCPTT user | TS 24.379 [9] clause 10.1.4.4 |  |
| response-type | "group-selection-change-response" |  | TS 24.379 [9] clause 10.1.4.4 |  |
| selected-group-change-outcome | "success" |  |  |  |
| NOTE 1: Encrypted element as described in TS 36.579-1 [2] Table 5.5.3.2.1-1A. |  |  |  |  |