3GPP TSG-RAN5 Meeting #95-e	R5-222153

# Online, , 9th May 2022 - 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-6 | CR | 0051 | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New MCVIDEO OFF-NETWORK TC 7.1.1.5 Off-network Imminent Peril Call CO |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NIST |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCenhUEConTest |  |  |  |  |  | Date: |  |  | 2022-04-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | No test case existed for an off-network CO imminent peril call |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added new test case 7.1.1.5 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | An off-network Imminent Peril CO call would not be tested |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Specific message contents include changes proposed in CR R5-222142 - New MCVideo Off-network Message Defaults 5.5.14 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<START OF CHANGES>

### 7.1.1.5	Off-network / Group Call / Imminent Peril Call / Client Originated (CO)

7.1.1.5.1	Test Purpose (TP)

(1)

with { UE (MCVideo Client) registered and authorised for MCVideo Service, including authorised to initiate/cancel imminent peril group calls in off-network environment, and the UE (MCVideo Client) is in an off-network environment }

ensure that {

when { the MCVideo User requests the establishment of an MCVideo imminent peril group call }

then { UE (MCVideo Client) requests an MCPTT imminent peril group call by sending a GROUP CALL ANNOUNCEMENT message, and respects the transmission control imposed by the transmission control entity/arbitrator }

}

7.1.1.5.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in:

TS 24.281 clauses 9.3.2.4.2.1, 9.3.2.4.2.2, 9.3.2.4.3.1, TS 24.581 clauses 7.2.3.2.2, 7.2.3.5.2, 7.2.3.5.5. Unless otherwise stated these are Rel-15 requirements.

[TS 24.281, clause 9.3.2.4.2.1]

When in the "S1: start-stop" state, upon an indication from an MCVideo user to initiate a group call for an MCVideo group ID, the MCVideo client:

1)	shall store the MCVideo group ID as the MCVideo group ID of the call;

2)	shall create a call type control state machine as described in clause 9.3.3.2;

3)	shall generate a GROUP CALL PROBE message as specified in clause 17.1.2. In the GROUP CALL PROBE message, the MCVideo client:

a)	shall set the MCVideo group ID IE to the stored MCVideo group ID of the call;

4)	shall send the GROUP CALL PROBE message as specified in clause 9.3.1.1.1;

5)	shall start timer TFG3 (call probe retransmission);

6)	shall start timer TFG1 (wait for call announcement); and

7)	shall enter the "S2: waiting for call announcement" state.

[TS 24.281, clause 9.3.2.4.2.2]

When in the "S2: waiting for call announcement" state, upon expiration of TFG3 (call probe retransmission), the MCVideo client:

1)	shall generate a GROUP CALL PROBE message as specified in clause 17.1.2. In the GROUP CALL PROBE message, the MCVideo client:

a)	shall set the MCVideo group ID IE to the stored MCVideo group ID of the call;

2)	shall send the GROUP CALL PROBE message as specified in clause 9.3.1.1.1;

3)	shall start timer TFG3 (call probe retransmission); and

4)	shall remain in the "S2: waiting for call announcement" state.

[TS 24.281, clause 9.3.2.4.3.1]

When in the "S2: waiting for call announcement" state, upon expiry of timer TFG1 (wait for call announcement), the MCVideo client:

1)	shall stop timer TFG3 (call probe retransmission), if running;

2)	shall generate an SDP body as specified in clause 9.3.1.1.2 and store it as the SDP body of the call;

3)	shall generate a random number with uniform distribution between 0 and 65535 and store it as the call identifier of the call;

4)	shall select refresh interval value and store it as the refresh interval of the call;

5)	shall store own MCVideo user ID as the originating MCVideo user ID of the call;

6)	shall store the current UTC time as the call start time of the call;

7)	shall generate a GROUP CALL ANNOUNCEMENT message as specified in clause 17.1.3. In the GROUP CALL ANNOUNCEMENT message, the MCVideo client:

a)	shall set the Call identifier IE to the stored call identifier of the call;

b)	shall set the Call type IE to the stored current call type associated with the call type control state machine;

c)	shall set the Refresh interval IE to the stored refresh interval of the call;

d)	shall set the SDP IE to the stored SDP body of the call;

e)	shall set the Originating MCVideo user ID IE to the stored originating MCVideo user ID of the call;

f)	shall set the MCVideo group ID IE to the stored MCVideo group ID of the call;

g)	shall set the Call start time IE to the stored call start time of the call;

h)	shall set the Last call type change time IE to the stored last call type change time of the call associated with call type control state machine;

i)	shall set the Last user to change call type IE to last user to change call type associated with call type control state machine; and

j)	may include the Confirm mode indication IE;

8)	shall send the GROUP CALL ANNOUNCEMENT message as specified in clause 9.3.1.1.1;

9)	shall establish a media session based on the stored SDP body of the call;

10)	shall start transmission control as originating transmission participant as specified in clause a.b in 3GPP TS 24.581 [5];

11)	shall start timer TFG6 (max duration) with value as specified in clause 9.3.2.4.1.2;

12)	shall start timer TFG2 (call announcement) with value as specified in clause 9.3.2.4.1.1.1; and

13)	shall enter the "S3: part of ongoing call" state.

Note: In this release of the specification, the refresh interval of the call is fixed to 10 seconds.

[TS 24.581, clause 7.2.3.2.2]

When an MCVideo call is established with session announcement including an explicit transmission request, the originating transmission participant:

1.	shall create an instance of a transmission participant state transition diagram for basic operation state machine;

2.	shall send Transmission Granted message towards other transmission participants. The Transmission Granted message:

a.	shall include the granted priority in the Transmission priority field;

b.	shall include the MCVideo user's own MCVideo ID in the User ID field; and

c.	if the transmission request is a broadcast group call, system call, emergency call or an imminent peril call, shall include a Transmission Indicator field indicating the relevant call types; and

3.	shall enter 'O: transmission arbitration' state.

[TS 24.581, clause 7.2.3.5.2]

Upon receiving encoded media from the user or if encoded media is already buffered the transmission participant:

1.	shall start timer T206 (Stop talking warning);

2.	shall request the MCVideo client to start sending RTP media packets towards other MCVideo clients; and

3.	shall remain in 'O: transmission arbitration' state.

[TS 24.581, clause 7.2.3.5.5]

Upon receiving an indication from the MCVideo user to release permission to send RTP media and there is no transmitter in transmitter list, the transmission participant:

1.	shall stop timer T206 (Stop talking warning), if running;

2.	shall stop timer T207 (Stop talking), if running;

3.	shall send a Transmission Arbitration Release message towards other transmission participants. The Transmission Arbitration Release message:

a.	shall include the MCVideo ID of the MCVideo user in the User ID field; and

b.	if the session is not initiated as a broadcast group call with the B-bit set to '1' (Broadcast group call), shall include a Transmission Indicator field set to '0' (normal call);

4.	shall start timer T230 (Inactivity); and

5.	shall enter 'O: silence' state.

7.1.1.5.3	Test description

7.1.1.5.3.1	Pre-test conditions

System Simulator:

-	SS-UE1 (MCVideo client)

-	For the underlying "transport bearer" over which the SS and the UE will communicate, the SS is behaving as SS-UE1 as defined in TS 36.508 [24], configured for and operating as ProSe Direct Communication transmitting and receiving device.

-	GNSS simulator to simulate a location and provide a timing reference for the assistance of E-UTRAN off-network testing.

NOTE 1:	For operation in off-network environment, it needs be ensured that after the UE is powered up it considers the Geographical area #1 that is simulated by the GNSS simulator as being one of the geographical areas set in the USIM for operation when UE is "not served by E-UTRAN".

-	SS-NW (MCVideo server)

-	For the underlying "transport bearer" over which the SS and the UE will communicate Parameters are set to the default parameters for the basic E-UTRA Single cell network scenarios, as defined in TS 36.508 [24] clause 4.4. The simulated Cell 1 shall belong to PLMN1 (the PLMN specified for MCVideo operation in the MCVideo configuration document).

NOTE 2:	The SS operation as NW (MCVideo server) is needed only for the Preamble if the UE has to perform the Generic Test Procedure for MCVideo Authorization/Configuration and Key Generation as specified in TS 36.579-1 [2], subclause 5.3.2.

IUT:

-	UE (MCVideo client)

-	The test USIM set as defined in TS 36.579-1 [2], subclause 5.5.10 is inserted.

-	For the underlying "transport bearer" over which the SS and the UE will communicate, the UE is behaving as a ProSe enabled UE as defined in TS 36.508 [24], configured for and operating as ProSe Direct Communication transmitting and receiving device.

Preamble:

--	The UE has performed the Generic Test Procedure for MCVideo UE registration as specified in TS 36.579-1 [2], subclause 5.4.2A.

-	The MCVideo User performs the Generic Test Procedure for MCVideo Authorization/Configuration and Key Generation as specified in TS 36.579-1 [2], subclause 5.3.2.

-	The GNSS simulator is configured to simulate a location in the centre of Geographical area #1 and provide a timing reference, as defined in TS 36.508 [24] Table 4.11.2-2 scenario #1.

-	The UE is switched-off.

-	UE States at the end of the preamble

-	The UE is in state 'switched-off'.

7.1.1.5.3.2	Test procedure sequence

Table 7.1.1.5.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Power up the UE | - | - | - | - |
| 2 | Trigger the UE to reset UTC time and location. NOTE: The UTC time and location reset may be performed by MMI or AT command (+CUTCR). | - | - | - | - |
| 3 | Activate the MCVideo Client Application and register User A as the MCVideo User (TS 36.579-5 [5], px_MCX_User_A_username, px_MCX_User_A_password). (NOTE 1) | - | - | - | - |
| 4 | Make the UE (MCVideo Client) initiate an off-network imminent peril group call with an explicit transmission request. (NOTE 1) | - | - | - | - |
| - | EXCEPTION: The E-UTRA/EPC actions which are related to the MCVideo off-network call establishment are described in TS 36.579-1 [2], subclause 5.4.11 'Generic Test Procedure for MCPTT CO communication over ProSe direct one-to-many communication out of E-UTRA coverage / Monitoring/Discoverer procedure for group member discovery / One-to-many communication'. The test sequence below shows only the MCVideo relevant messages exchanged. | - | - | - | - |
| 5 | Check: Does the UE (MCVideo Client) send a GROUP CALL PROBE message to determine the current call status of the group? (NOTE 2) | --> | GROUP CALL PROBE | 1 | P |
| - | EXCEPTION: Step 6 is executed a total of 3 times |  |  |  |  |
| 6 | Check: At the expiration of timer TFG3 (call probe retransmission), does the UE (MCVideo Client) send a retransmission of the GROUP CALL PROBE sent in step 5? | --> | GROUP CALL PROBE | 1 | P |
| 7 | Check: At the expiration of timer TFG1 (wait for call announcement), and after receiving no response to the GROUP CALL PROBE, does the UE (MCVideo Client) send a GROUP CALL ANNOUNCEMENT message to initiate an off-network imminent peril group call? | --> | GROUP CALL ANNOUNCEMENT | 1 | P |
| 8 | The SS-UE1 (MCVideo Client) sends a GROUP CALL ACCEPT accepting the GROUP CALL ANNOUNCEMENT. | <-- | GROUP CALL ACCEPT | - | - |
| 9 | Check: Does the UE (MCVideo Client) send a Transmission Granted message towards the other transmission participants? | --> | Transmission Granted | 1 | P |
| 10 | Make the MCVideo User release transmission control. (NOTE 1), (NOTE 3) | - | - | - | - |
| 11 | Check: Does the UE (MCVideo Client) send a Transmission Arbitration Release message to the other transmission participants? | --> | Transmission Arbitration Release | 1 | P |
| 12 | Make the UE (MCVideo Client) release the imminent peril group call. (NOTE 1) | - | - | - | - |
| - | EXCEPTION: UE releases the E-UTRA connection. The E-UTRA/EPC actions which are related to the MCVideo call release are described in TS 36.579-1 [2], subclause 5.4.8, 'Generic Test Procedure for MCPTT communication over ProSe direct one-to-one communication out of E-UTRA coverage - release by the UE'. | - | - | - | - |
| NOTE 1: This is expected to be done via a suitable implementation dependent MMI. NOTE 2: Timer TFG3 (call probe retransmission)=40ms as defined in TS 36.579-1 [2], Table 5.5.8.1-1 and TFG1 (wait for call announcement)=150ms as defined in TS 36.579-1 [2], Table 5.5.8.1-1 are started upon the sending of the GROUP CALL PROBE message NOTE 3: If the MCVideo User does not release transmission control before timer T207 (Stop talking) expires, then the MCVideo Client will enter the 'O: silence' state per TS 24.581 [27] and the remaining steps will not be valid. Timer T206 (Stop talking warning) is started upon the sending of the Transmission Granted message. Timer T207 (Stop talking) starts upon the expiration of Timer T206 (Stop talking warning). Default value of Timer T206 (Stop talking warning)=27s according to TS 24.581 [27]. Default value of T207 (Stop talking) is 3s according to TS 24.581 [27]. |  |  |  |  |  |

7.1.1.5.3.3	Specific message contents

Table 7.1.1.5.3.3-1: GROUP CALL ANNOUNCEMENT (step 7, Table 7.1.1.5.3.2-1)

Derivation Path: TS 36.579-1 [2], Table 5.5.14.2-1, condition IMMPERIL-CALL

Table 7.1.1.5.3.3-2: GROUP CALL ACCEPT (step 8, Table 7.1.1.5.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.14.3-1, condition IMMPERIL-CALL |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Sending MCVideo user ID | px_MCVideo_ID_User_B |  |  |

Table 7.1.1.5.3.3-3: Transmission Granted (step 9, Table 7.1.1.5.3.2-1)

Derivation Path: TS 36.579-1 [2], Table 5.5.11.2.1-1, condition OFF-NETWORK, IMMPERIL-CALL

Table 7.1.1.5.3.3-4: Transmission Arbitration Release (step 11, Table 7.1.1.5.3.2-1)

Derivation Path: TS 36.579-1 [2], Table 5.5.11.2.4-1, condition OFF-NETWORK, IMMPERIL-CALL

<END OF CHANGES>