3GPP TSG-RAN5 Meeting #95-e	R5-222154

# Online, , 9th May 2022 - 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-6 | CR | 0052 | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New MCVIDEO OFF-NETWORK TC 7.1.1.6 Off-network Imminent Peril Call CT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NIST |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCenhUEConTest |  |  |  |  |  | Date: |  |  | 2022-04-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | No test case existed for an Off-network CT Imminent Peril Call |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added new test case 7.1.1.6 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | An off-network CT Imminent Peril Call would not be tested |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Specific message contents include changes proposed in CR R5-222142 - New MCVideo Off-network Message Defaults 5.5.14 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<START OF CHANGES>

### 7.1.1.6	Off-network / Group Call / Imminent Peril Call / Client Terminated (CT)

7.1.1.6.1	Test Purpose (TP)

(1)

with { UE (MCVideo Client) registered and authorised for MCVideo Service, including authorised to receive imminent peril group calls in off-network environment, and the UE (MCVideo Client) is in an off-network environment }

ensure that {

when { UE (MCVideo Client) receives a GROUP CALL ANNOUNCEMENT message for an imminent peril group call with Confirm mode indication present }

then { UE (MCVideo Client) sends a GROUP CALL ACCEPT message and respects the transmission control imposed by the transmission control entity/arbitrator }

}

7.1.1.6.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.281 clauses 9.3.2.4.3.3, 9.3.3.4.5, TS 24.581 clauses 7.2.3.2.3, 7.2.3.3.4, 7.2.3.4.2, 7.2.3.7.3. Unless otherwise stated these are Rel-15 requirements.

[TS 24.281, clause 9.3.2.4.3.3]

When in the "S1: start-stop" state, upon receiving a GROUP CALL ANNOUNCEMENT message with the MCVideo group ID IE not matching MCVideo group ID of the call stored for other state machines, the MCVideo client:

1)	shall store the value of the SDP IE of the GROUP CALL ANNOUNCEMENT message as the SDP body of the call;

2)	shall store the value of the Call identifier IE of the GROUP CALL ANNOUNCEMENT message as the call identifier of the call;

3)	shall store the value of the Originating MCVideo user ID IE of the GROUP CALL ANNOUNCEMENT message as the originating MCVideo user ID of the call;

4)	shall store the value of the Refresh interval IE of the GROUP CALL ANNOUNCEMENT message as the refresh interval of the call;

5)	shall store the value of the MCVideo group ID IE of the GROUP CALL ANNOUNCEMENT message as the MCVideo group ID of the call;

6)	shall store the value of the Call start time IE of the GROUP CALL ANNOUNCEMENT message as the call start time of the call;

7)	shall create a call type control state machine as described in clause 9.3.3.2;

8)	if the terminating UE is configured that the terminating MCVideo user acknowledgement is required upon a terminating call request reception:

a)	shall start timer TFG4 (waiting for the user);

b)	if the GROUP CALL ANNOUNCEMENT message contains the Confirm mode indication IE, shall enter the "S5: pending user action with confirm indication" state; and

c)	if the GROUP CALL ANNOUNCEMENT message does not contains the Confirm mode indication IE, shall enter the "S4: pending user action without confirm indication" state; and

9)	if the terminating UE is configured that the terminating MCVideo user acknowledgement is not required upon a terminating call request reception:

a)	shall establish a media session based on the stored SDP body of the call;

b)	shall start transmission control as terminating transmission participant as specified in clause a.b in 3GPP TS 24.581 [5];

c)	if the GROUP CALL ANNOUNCEMENT message contains the Confirm mode indication IE:

i)	shall generate a GROUP CALL ACCEPT message as specified in clause 17.1.4. In the GROUP CALL ACCEPT message, the MCVideo client:

A)	shall set the Call identifier IE to the stored call identifier of the call;

B)	shall set the Sending MCVideo user ID IE to own MCVideo user id;

C)	shall set the Call type IE to the stored current call type associated with the call type control state machine; and

D)	shall set the MCVideo group ID IE to the stored MCVideo group ID of the call; and

ii)	shall send the GROUP CALL ACCEPT message as specified in clause 9.3.1.1.1;

d)	shall start timer TFG6 (max duration) with value as specified in clause 9.3.2.4.1.2;

e)	shall start timer TFG2 (call announcement) with value as specified in clause 9.3.2.4.1.1.1; and

f)	shall enter the "S3: part of ongoing call" state.

[TS 24.281, clause 9.3.3.4.5]

When in the "T0: waiting for the call to establish" state, upon receipt of a GROUP CALL ANNOUNCEMENT message by an idle MCVideo client when MCVideo user acknowledgement is not required, the MCVideo client:

1)	shall set the stored last call type change time to the Last call type change time IE of the GROUP CALL ANNOUNCEMENT message;

2)	shall set the last user to change call type to the Last user to change call type IE of the GROUP CALL ANNOUNCEMENT message;

3)	if the Call type IE of the received GROUP CALL ANNOUNCEMENT message is set to "EMERGENCY GROUP CALL":

a)	shall set the stored current call type to "EMERGENCY GROUP CALL";

b)	shall set the stored current ProSe per-packet priority to value corresponding to MCVideo off-network emergency group call as described in 3GPP TS 24.483 [4];

c)	shall start timer TFG13 (implicit downgrade emergency) with value as specified in clause 9.3.3.4.1.1; and

d)	shall enter "T1: in-progress emergency group call" state;

4)	if the Call type IE of the received GROUP CALL ANNOUNCEMENT message is set to "IMMINENT PERIL GROUP CALL":

a)	shall set the stored current call type to "IMMINENT PERIL GROUP CALL";

b)	shall set the stored current ProSe per-packet priority to value corresponding to MCVideo off-network imminent peril group call as described in3GPP TS 24.483 [4];

c)	shall start timer TFG14 (implicit downgrade imminent peril) with value as specified in clause 9.3.3.4.1.2; and

d)	shall enter "T3: in-progress imminent peril group call" state; and

5)	if the Call type IE of the received GROUP CALL ANNOUNCEMENT message is set to "BASIC GROUP CALL":

a)	shall set the stored current call type to "BASIC GROUP CALL";

b)	shall set the stored current ProSe per-packet priority to value corresponding to MCVideo off-network basic group call as described in 3GPP TS 24.483 [4]; and

c)	shall enter "T2: in-progress basic group call" state.

[TS 24.581, clause 7.2.3.2.3]

When an MCVideo call is established the terminating transmission participant:

1.	shall create an instance of a transmission participant state transition diagram for basic operation state machine;

2.	shall start timer T230 (Inactivity); and

3.	shall enter 'O: silence' state.

[TS 24.581, clause 7.2.3.3.4]

When a Transmission Granted message is received and if the User ID in the Transmission Granted message does not match its own User ID, the transmission participant:

1.	may provide a transmission taken notification to the MCVideo user;

2.	if the Transmission Indicator field is included and the B-bit is set to '1' (Broadcast group call), shall provide a notification to the user indicating that this is a broadcast group call;

3.	shall stop timer T230 (Inactivity);

4.	shall start timer T203 (End of RTP media) and store the identity of the user, to whom the transmission was granted in the Transmission Granted message, in transmitter list; and

5.	shall enter 'O: has no permission' state.

[TS 24.581, clause 7.2.3.4.2]

If the transmission participant receives an indication from the MCVideo user that the MCVideo user wants to send media, the transmission participant:

1.	shall send the Transmission Request message to other clients. The Transmission Request message:

a.	if a priority different than the default transmission priority is required, shall include the Transmission Priority field with the requested priority in the <Transmission Priority> element;

b.	shall include the MCVideo ID of the MCVideo user in the User ID field; and

c.	if the transmission request is a broadcast group call, system call, emergency call or an imminent peril call, shall include a Transmission Indicator field indicating the relevant call types;

2.	shall initialise the counter C201 (Transmission request) with value set to 1;

3.	shall start timer T201 (Transmission Request); and

4.	shall enter 'O: pending request' state.

[TS 24.581, clause 7.2.3.7.3]

Upon receiving Transmission Rejected message, if the <User ID> value in the User ID field matches its own MCVideo ID and User ID of transmission participant sending the Transmission Rejected message matches the stored User ID of current transmission arbitrator, the transmission participant:

Editor's Note:	How a new participant obtains the identity of the transmission arbitrator is FFS.

1.	shall stop the timer T201 (Transmission Request);

2.	shall provide transmission deny notification to the user;

3.	may display the transmission deny reason to the user using information in the Reject Cause field; and

4.	shall enter 'O: has no permission' state.

Otherwise, if the <User ID> value in the User ID field matches its own MCVideo ID and there is no stored the current transmission arbitrator, the transmission participant:

1.	shall stop the timer T201 (Transmission Request);

2.	shall set the stored User ID of the current transmission arbitrator to the value in the User ID of transmission control server field as received in the Transmission Rejected message;

3.	shall provide transmission deny notification to the user;

4.	may display the transmission deny reason to the user using information in the Reject Cause field; and

5.	shall enter 'O: has no permission' state.

7.1.1.6.3	Test description

7.1.1.6.3.1	Pre-test conditions

System Simulator:

-	SS-UE1 (MCVideo client)

-	For the underlying "transport bearer" over which the SS and the UE will communicate, the SS is behaving as SS-UE1 as defined in TS 36.508 [24], configured for and operating as ProSe Direct Communication transmitting and receiving device.

-	GNSS simulator to simulate a location and provide a timing reference for the assistance of E-UTRAN off-network testing.

NOTE 1:	For operation in off-network environment, it needs be ensured that after the UE is powered up it considers the Geographical area #1 that is simulated by the GNSS simulator as being one of the geographical areas set in the USIM for operation when UE is "not served by E-UTRAN".

-	SS-NW (MCVideo server)

--	For the underlying "transport bearer" over which the SS and the UE will communicate Parameters are set to the default parameters for the basic E-UTRA Single cell network scenarios, as defined in TS 36.508 [24] clause 4.4. The simulated Cell 1 shall belong to PLMN1 (the PLMN specified for MCVideo operation in the MCVideo configuration document).

NOTE 2:	The SS operation as NW (MCVideo server) is needed only for the Preamble if the UE has to perform the MCVideo User performs the Generic Test Procedure for MCVideo Authorization/Configuration and Key Generation as specified in TS 36.579-1 [2], subclause 5.3.2.

IUT:

-	UE (MCVideo client)

-	The test USIM set as defined in TS 36.579-1 [2], subclause 5.5.10 is inserted.

-	For the underlying "transport bearer" over which the SS and the UE will communicate, the UE is behaving as a ProSe enabled UE as defined in TS 36.508 [24], configured for and operating as ProSe Direct Communication transmitting and receiving device.

Preamble:

-	The UE has performed the Generic Test Procedure for MCVideo UE registration as specified in TS 36.579-1 [2], subclause 5.4.2A.

-	The MCVideo User performs the Generic Test Procedure for MCVideo Authorization/Configuration and Key Generation as specified in TS 36.579-1 [2], subclause 5.3.2.

-	The GNSS simulator is configured to simulate a location in the centre of Geographical area #1 and provide a timing reference, as defined in TS 36.508 [24] Table 4.11.2-2 scenario #1.

-	The UE is switched-off.

-	UE States at the end of the preamble

-	The UE is in state 'switched-off'.

7.1.1.6.3.2	Test procedure sequence

Table 7.1.1.6.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Power up the UE. | - | - | - | - |
| 2 | Trigger the UE to reset UTC time and location. NOTE: The UTC time and location reset may be performed by MMI or AT command (+CUTCR). | - | - | - | - |
| 3 | Activate the MCVideo Client Application and register User A as the MCVideo User (TS 36.579-5 [5], px_MCX_User_A_username, px_MCX_User_A_password). (NOTE 1) | - | - | - | - |
| - | EXCEPTION: The E-UTRA/EPC actions which are related to the MCVideo call establishment are described in TS 36.579-1 [2], subclause 5.4.10 'Generic Test Procedure for MCPTT CT communication over ProSe direct one-to-many communication out of E-UTRA coverage / Announcing/Discoveree procedure for group member discovery'. The test sequence below shows only the MCVideo relevant messages exchanged. | - | - | - | - |
| 4 | SS-UE1 (MCVideo client) sends a GROUP CALL ANNOUNCEMENT for an imminent peril group call. | <-- | GROUP CALL ANNOUNCEMENT | - | - |
| 5 | Check: Does the UE (MCVideo Client) send a GROUP CALL ACCEPT message? | --> | GROUP CALL ACCEPT | 1 | P |
| 6 | SS-UE1 (MCVideo client) sends a Transmission Granted message notifying the recipients that it now has transmission control. | <-- | Transmission Granted | - | - |
| 7 | Make the MCVideo User request to send media. (NOTE 1) | - | - | - | - |
| 8 | Check: Does the UE (MCVideo Client) send a Transmission Request message? | --> | Transmission Request | 1 | P |
| 9 | SS-UE1 (MCVideo client) sends a Floor Granted message notifying the recipients that it now has transmission control. | <-- | Transmission Rejected | - | - |
| 10 | Check: Does the UE (MCVideo Client) provide transmission deny notification to the MCVideo User? (NOTE 1) |  |  | 1 | P |
| - | EXCEPTION: SS releases the E-UTRA connection. The E-UTRA/EPC actions which are related to the MCVideo call release are described in TS 36.579-1 [2], subclause 5.4.7, 'Generic Test Procedure for MCPTT communication over ProSe direct one-to-one communication out of E-UTRA coverage - release by the SS'. | - | - | - | - |
| NOTE 1: This is expected to be done via a suitable implementation dependent MMI. |  |  |  |  |  |

7.1.1.6.3.3	Specific message contents

Table 7.1.1.6.3.3-1: GROUP CALL ANNOUNCEMENT (step 4, Table 7.1.1.6.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.14.2-1, condition IMMPERIL-CALL |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SDP | As described in TS36.579-1 [2], Table 5.5.3.1.4-2 |  |  |
| Originating MCPTT user ID | px_MCVideo_ID_User_B |  |  |

Table 7.1.1.6.3.3-1: GROUP CALL ACCEPT (step 5, Table 7.1.1.6.3.2-1)

Derivation Path: TS 36.579-1 [2], Table 5.5.14.3-1, condition IMMPERIL-CALL

Table 7.1.1.6.3.3-2: Transmission Granted (step 6, Table 7.1.1.6.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.11.2.1-1, condition OFF-NETWORK, IMMPERIL-CALL |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| User ID |  |  |  |
| User ID | px_MCVideo_ID_User_B |  |  |

Table 7.1.1.6.3.3-3: Transmission Request (step 8, Table 7.1.1.6.3.2-1)

Derivation Path: TS 36.579-1 [2], Table 5.5.11.1.1-1, condition OFF-NETWORK, IMMPERIL-CALL

Table 7.1.1.6.3.3-4: Transmission Rejected (step 9, Table 7.1.1.6.3.2-1)

Derivation Path: TS 36.579-1 [2], Table 5.5.11.2.2-1, condition OFF-NETWORK, IMMPERIL-CALL

<END OF CHANGES>