3GPP TSG-RAN5 Meeting #95-e	R5-222156

# Online, , 9th May 2022 - 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-6 | CR | 0054 | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New MCVIDEO OFF-NETWORK TC 7.1.2.2 Off-network Broadcast Call CT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NIST |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCenhUEConTest |  |  |  |  |  | Date: |  |  | 2022-04-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | No test case existed for an off-network CT broadcast group call |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added new test case 7.1.2.2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | An off-network CT broadcast group call would not be tested |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Specific message contents include changes proposed in CR R5-222142 - New MCVideo Off-network Message Defaults 5.5.14 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<START OF CHANGES>

#### 7.1.2.2	Off-network / Group Call / Broadcast Group Call / Originator Releases Call / Client Terminated (CT)

7.1.2.2.1	Test Purpose (TP)

(1)

with { UE (MCVideo Client) registered and authorised for MCVideo Service, including authorised to receive broadcast calls in off-network environment, and the UE (MCVideo Client) is in an off-network environment, and the UE (MCVideo Client) has received a GROUP CALL BROADCAST message }

ensure that {

when { UE (MCVideo Client) receives a Transmission Granted message with the Transmission Indicator field included and with the B-bit set to '1' (Broadcast group call) }

then { UE (MCVideo Client) provides a notification to the MCVideo User indicating that this is a broadcast group call }

}

(2)

with { UE (MCVideo Client) in an ongoing off-network broadcast group call }

ensure that {

when { MCVideo User requests to send media }

then { UE (MCVideo Client) does not send a Transmission Request message }

}

7.1.2.2.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.281 clauses 9.4.2.4.2, 9.4.2.4.3, 9.4.2.4.8, TS 24.581 clauses 7.2.3.2.9, 7.2.3.4.6. Unless otherwise stated these are Rel-15 requirements.

[TS 24.281, clause 9.4.2.4.2]

When in the "B1: start-stop" state, upon receiving a GROUP CALL BROADCAST message with the Call identifier IE not matching any in-progress broadcast group call, the MCVideo client:

1)	shall store the value of the Call identifier IE of the GROUP CALL BROADCAST message as the call identifier of the call;

2)	shall store the value of the Call type IE of the GROUP CALL BROADCAST message as the received current call type;

3)	shall store the value of the SDP IE of the GROUP CALL BROADCAST message as the SDP body of the call;

4)	shall store the value of the Originating MCVideo user ID IE of the GROUP CALL BROADCAST message as the originating MCVideo user ID of the call;

5)	shall store the value of the MCVideo group ID IE of the GROUP CALL BROADCAST message as the MCVideo group ID of the call;

6)	if the terminating UE is configured that the terminating MCVideo user acknowledgement is required upon a terminating call request reception:

i)	shall start timer TFB3 (waiting for the user); and

ii)	shall enter the "B3: pending user action" state; and

7)	if the terminating UE is configured that the terminating MCVideo user acknowledgement is not required upon a terminating call request reception:

i)	shall establish a media session based on the stored SDP body of the call;

ii) shall start transmission control as terminating transmission participant as specified in clause a.b in 3GPP TS 24.581 [5];

iii)	shall start timer TFB1 (max duration); and

iv)	shall enter the "B2: in-progress broadcast group call" state.

[TS 24.281, clause 9.4.2.4.3]

When in the "B3: pending user action" state, upon indication from the MCVideo user to accept the incoming broadcast group call, the MCVideo client:

1)	shall establish a media session based on the stored SDP body of the call;

2)	shall start transmission control as terminating transmission participant as described specified in clause a.b in 3GPP TS 24.581 [5];

3)	shall stop timer TFB3 (waiting for the user);

4)	shall start timer TFB1 (max duration); and

5)	shall enter the "B2: in-progress broadcast group call" state.

[TS 24.281, clause 9.4.2.4.8]

When in the "B2: in-progress broadcast group call" state or "B4: ignoring same call ID" state, upon receiving GROUP CALL BROADCAST END message with the same Call identifier IE as the stored call identifier, the MCVideo client:

1)	shall release media session;

2)	shall stop transmission control, if running; and

3)	shall enter the "B1: start-stop" state.

[TS 24.581, clause 7.2.3.2.9]

When an MCVideo broadcast call is established the terminating transmission participant:

1.	shall create an instance of a transmission participant state transition diagram for basic operation state machine;

2.	shall start timer T203 (End of RTP media); and

3.	shall enter 'O: has no permission' state.

NOTE:	In MCVideo broadcast call, only originating MCVideo user is allowed to request transmission and transmit media. A Transmission Request message is locally denied to terminating MCVideo user, if requested.

[TS 24.581, clause 7.2.3.4.6]

When a Transmission Granted message is received and if the <User ID> value in the User ID field does not match its own MCVideo ID, the transmission participant:

1.	shall start timer T203 (End of RTP media) for the User ID;

2.	shall store the user to whom the transmission was granted in the Transmission Granted message in transmitter list;

3.	may provide a transmission taken notification to the MCVideo user;

4.	if the Transmission Indicator field is included with the B-bit set to '1' (Broadcast group call), shall provide a notification to the user indicating that this is a broadcast group call; and

5.	shall remain in the 'O: has no permission' state.

7.1.2.2.3	Test description

7.1.2.2.3.1	Pre-test conditions

System Simulator:

-	SS-UE1 (MCVideo Client)

-	For the underlying "transport bearer" over which the SS and the UE will communicate, the SS is behaving as SS-UE1 as defined in TS 36.508 [24], configured for and operating as ProSe Direct Communication transmitting and receiving device.

-	GNSS simulator to simulate a location and provide a timing reference for the assistance of E-UTRAN off-network testing.

NOTE 1:	For operation in off-network environment, it needs be ensured that after the UE is powered up it considers the Geographical area #1 that is simulated by the GNSS simulator as being one of the geographical areas set in the USIM for operation when UE is "not served by E-UTRAN".

-	SS-NW (MCVideo server)

-	For the underlying "transport bearer" over which the SS and the UE will communicate Parameters are set to the default parameters for the basic E-UTRA Single cell network scenarios, as defined in TS 36.508 [24] clause 4.4. The simulated Cell 1 shall belong to PLMN1 (the PLMN specified for MCVideo operation in the MCVideo configuration document).

NOTE 2:	The SS operation as NW (MCVideo server) is needed only for the Preamble if the UE has to perform the MCVideo User performs the Generic Test Procedure for MCVideo Authorization/Configuration and Key Generation as specified in TS 36.579-1 [2], subclause 5.3.2.

IUT:

-	UE (MCVideo Client)

-	The test USIM set as defined in TS 36.579-1 [2], subclause 5.5.10 is inserted.

-	For the underlying "transport bearer" over which the SS and the UE will communicate, the UE is behaving as a ProSe enabled UE as defined in TS 36.508 [24], configured for and operating as ProSe Direct Communication transmitting and receiving device.

Preamble:

-	The UE has performed the Generic Test Procedure for MCVideo UE registration as specified in TS 36.579-1 [2], subclause 5.4.2A.

-	The MCVideo User performs the Generic Test Procedure for MCVideo Authorization/Configuration and Key Generation as specified in TS 36.579-1 [2], subclause 5.3.2.

-	The GNSS simulator is configured to simulate a location in the centre of Geographical area #1 and provide a timing reference, as defined in TS 36.508 [24] Table 4.11.2-2 scenario #1.

-	The UE is switched-off.

-	UE States at the end of the preamble

-	The UE is in state 'switched-off'.

7.1.2.2.3.2	Test procedure sequence

Table 7.1.2.2.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Power up the UE | - | - | - | - |
| 2 | Trigger the UE to reset UTC time and location. NOTE: The UTC time and location reset may be performed by MMI or AT command (+CUTCR). | - | - | - | - |
| 3 | Activate the MCVideo Client Application and register User A as the MCVideo User (TS 36.579-5 [5], px_MCX_User_A_username, px_MCX_User_A_password). (NOTE 1) | - | - | - | - |
| - | EXCEPTION: The E-UTRA/EPC actions which are related to the MCVideo call establishment are described in TS 36.579-1 [2], subclause 5.4.10 'Generic Test Procedure for MCPTT CT communication over ProSe direct one-to-many communication out of E-UTRA coverage / Announcing/Discoveree procedure for group member discovery'. The test sequence below shows only the MCVideo relevant messages exchanged. | - | - | - | - |
| 4 | SS-UE1 (MCVideo Client) sends a GROUP CALL BROADCAST (NOTE 2) | <-- | GROUP CALL BROADCAST | - | - |
| - | EXCEPTION: Step 5a1- describes behaviour that depends on the UE configuration; the "lower case letter" identifies a step sequence that take place if the UE is configured that the terminating MCVideo user acknowledgement is required upon a terminating call request reception. | - | - | - | - |
| 5a1 | Make the MCVideo User accept the terminating call. | - | - | - | - |
| 6 | SS-UE1 (MCVideo Client) sends a Transmission Granted message. | <-- | Transmission Granted | - | - |
| 7 | Check: Does the UE (MCVideo Client provide a notification to the MCVideo User indicating that this is a broadcast group call? | - | - | 1 | P |
| 8 | Make the UE (MCVideo User) request to send media. (NOTE 1), (NOTE 3) | - | - | - | - |
| 9 | Check: Does the UE (MCVideo Client) send in the next 5 seconds a Transmission Request message to the transmission participants? | --> | Transmission Request | 2 | F |
| 10 | Wait 5 seconds for the Transmission Request message | - | - | - | - |
| 11 | SS-UE1 (MCVideo Client) sends a GROUP CALL BROADCAST END (NOTE 4) | <-- | GROUP CALL BROADCAST END | - | - |
| - | EXCEPTION: SS releases the E-UTRA connection. The E-UTRA/EPC actions which are related to the MCVideo call release are described in TS 36.579-1 [2], subclause 5.4.7, 'Generic Test Procedure for MCPTT communication over ProSe direct one-to-one communication out of E-UTRA coverage - release by the SS'. | - | - | - | - |
| NOTE 1: This is expected to be done via a suitable implementation dependent MMI. NOTE 2: Timer TFB1 (max duration) is started by the MCVideo Client upon the receiving of the GROUP CALL BROADCAST message. Timer TFB1 (max duration)=300s, as defined in TS 36.579-1 [2], Table 5.5.8.1-1. At the expire of TFB1 (max duration), the MCVideo Client will terminate participation in the broadcast call. NOTE 3: It is possible that the MCVideo Client may not allow the MCVideo User to request the floor. NOTE 4: Timer TFB1 (max duration) is stopped by the MCVideo Client upon the receiving of the GROUP CALL BROADCAST END message. |  |  |  |  |  |

7.1.2.2.3.3	Specific message contents

Table 7.1.2.2.3.3-2: GROUP CALL BROADCAST (step 4, Table 7.1.2.2.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.14.6-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Originating MCVideo user ID | px_MCVideo_ID_User_B |  |  |
| SDP | As described in TS 36.579-1 [2], Table 5.5.3.1.4-2 |  |  |

Table 7.1.2.2.3.3-2: Transmission Granted (step 6, Table 7.1.2.2.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.11.2.1-1, condition OFF-NETWORK |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| User ID |  |  |  |
| User ID | px_MCVideo_ID_User_B |  |  |
| Transmission Indicator |  |  |  |
| Transmission Indicator | "0100000000000000" | Broadcast group call |  |

Table 7.1.2.2.3.3-3: GROUP CALL BROADCAST End (step 11, Table 7.1.2.2.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.14.7.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Originating MCVideo user ID | px_MCVideo_ID_User_B |  |  |

<END OF CHANGES>