3GPP TSG-RAN5 Meeting #95-e	R5-222161

# Online, , 9th May 2022 - 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-6 | CR | 0059 | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New MCVIDEO OFF-NETWORK TC 7.3.1 Off-network Emergency Alert CO |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NIST |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCenhUEConTest |  |  |  |  |  | Date: |  |  | 2022-04-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | No test case existed for off-network emergency alerts CO |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added new test case 7.3.1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Off-network emergency alerts CO would not be tested |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3, 7.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Specific message contents include changes proposed in CR R5-222142 - New MCVideo Off-network Message Defaults 5.5.14 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<START OF CHANGES>

## 7.3	Emergency Alert

### 7.3.1	Off-network / Emergency Alert / Client Originated (CO)

7.3.1.1	Test Purpose (TP)

(1)

with { UE (MCVideo Client) registered and authorized for MCVideo Service, including authorized to initiate/cancel private calls with automatic commencement in off-network environment, and, the UE is in an off-network environment }

ensure that {

when { the MCVideo User requests to to transmit an emergency alert for an MCVideo group ID }

then { UE (MCVideo Client) sends a GROUP EMERGENCY ALERT message, and, starts timer TFE2 (emergency alert retransmission) }

}

(2)

with { UE (MCVideo Client) having sent a GROUP EMERGENCY ALERT message }

ensure that {

when { TFE2 (emergency alert retransmission) timer expires }

then { UE (MCVideo Client) sends a GROUP EMERGENCY ALERT message, and, starts timer TFE2 (emergency alert retransmission) }

}

(3)

with { UE (MCVideo Client) having sent a GROUP EMERGENCY ALERT message }

ensure that {

when { MCVideo User cancels the Emergency Alert }

then { UE (MCVideo Client) sends a GROUP EMERGENCY ALERT CANCEL message, and, stops timer TFE2 (emergency alert retransmission) }

}

7.3.1.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.281 clauses 11.3.3.1, 11.3.3.2, 11.3.3.5. Unless otherwise stated these are Rel-15 requirements.

[TS 24.281, clause 11.3.3.1]

When in state "E1: Not in emergency state", upon receiving an indication from the MCVideo user to transmit an emergency alert for an MCVideo group ID and the value of "/<x>/<x>/Common/AllowedActivateAlert" leaf node present in the user profile as specified in 3GPP TS 24.483 [4] is set to "true", the MCVideo client:

1)	shall set the stored emergency state as "true";

2)	shall set the stored MCVideo group ID to the indicated MCVideo group ID;

3)	shall generate a GROUP EMERGENCY ALERT message as specified in clause 17.1.14. In the GROUP EMERGENCY ALERT message, the MCVideo client:

a)	shall set the MCVideo group ID IE to the stored MCVideo group ID;

b)	shall set the Originating MCVideo user ID IE to own MCVideo user ID;

c)	shall set the Organization name IE to own organization name; and

d)	may set the User location IE with client's current location, if requested;

4)	shall send the GROUP EMERGENCY ALERT message as specified in clause 9.3.1.1.1;

5)	shall start timer TFE2 (emergency alert retransmission); and

6)	shall enter "E2: Emergency state" state.

[TS 24.281, clause 11.3.3.2]

When in state "E2: Emergency state", upon expiry of timer TFE2 (emergency alert retransmission), the MCVideo client:

1)	shall generate a GROUP EMERGENCY ALERT message as specified in clause 17.1.14. In the GROUP EMERGENCY ALERT message, the MCVideo client:

a)	shall set the MCVideo group ID IE to the stored MCVideo group ID;

b)	shall set the originating MCVideo user ID IE to own MCVideo user ID;

c)	shall set the Organization name IE to own organization name; and

d)	may set the Location IE with client's current location, if requested; and

2)	shall send the GROUP EMERGENCY ALERT message as specified in clause 9.3.1.1.1;

3)	shall start the timer TFE2 (emergency alert retransmission); and

4)	shall remain in the current state.

[TS 24.281, clause 11.3.3.5]

When in "E2: Emergency state", upon receiving an indication from the MCVideo user to cancel an emergency alert and the value of "/<x>/<x>/Common/AllowedCancelAlert" leaf node present in the user profile as specified in 3GPP TS 24.483 [4] is set to "true", the MCVideo client:

1)	shall set the stored emergency state as "false";

2)	shall generate a GROUP EMERGENCY ALERT CANCEL message as specified in clause 17.1.16. In the GROUP EMERGENCY ALERT CANCEL message, the MCVideo client:

a)	shall set the MCVideo group ID IE to the stored MCVideo group ID; and

b)	shall set the Originating MCVideo user ID IE to own MCVideo user ID; and

3)	shall send the GROUP EMERGENCY ALERT CANCEL message as specified in clause 9.3.1.1.1;

4)	shall stop timer TFE2 (emergency alert retransmission); and

5)	shall enter "E1: Not in emergency state" state.

7.3.1.3	Test description

7.3.1.3.1	Pre-test conditions

System Simulator:

-	SS-UE1 (MCVideo Client)

-	For the underlying "transport bearer" over which the SS and the UE will communicate, the SS is behaving as SS-UE1 as defined in TS 36.508 [24], configured for and operating as ProSe Direct Communication transmitting and receiving device.

-	GNSS simulator to simulate a location and provide a timing reference for the assistance of E-UTRAN off-network testing.

NOTE 1:	For operation in off-network environment, it needs be ensured that after the UE is powered up it considers the Geographical area #1 that is simulated by the GNSS simulator as being one of the geographical areas set in the USIM for operation when UE is "not served by E-UTRAN".

-	SS-NW (MCVideo server)

-	For the underlying "transport bearer" over which the SS and the UE will communicate Parameters are set to the default parameters for the basic E-UTRA Single cell network scenarios, as defined in TS 36.508 [24] clause 4.4. The simulated Cell 1 shall belong to PLMN1 (the PLMN specified for MCVideo operation in the MCVideo configuration document).

NOTE 2:	The SS operation as NW (MCVideo server) is needed only for the Preamble if the UE has to perform the Generic Test Procedure for MCVideo Authorization/Configuration and Key Generation as specified in TS 36.579-1 [2], subclause 5.3.2.

IUT:

-	UE (MCVideo Client)

-	The test USIM set as defined in TS 36.579-1 [2], subclause 5.5.10 is inserted.

-	For the underlying "transport bearer" over which the SS and the UE will communicate, the UE is behaving as a ProSe enabled UE as defined in TS 36.508 [24], configured for and operating as ProSe Direct Communication transmitting and receiving device.

Preamble:

--	The UE has performed the Generic Test Procedure for MCVideo UE registration as specified in TS 36.579-1 [2], subclause 5.4.2A.

-	The MCVideo User performs the Generic Test Procedure for MCVideo Authorization/Configuration and Key Generation as specified in TS 36.579-1 [2], subclause 5.3.2.

-	The GNSS simulator is configured to simulate a location in the centre of Geographical area #1 and provide a timing reference, as defined in TS 36.508 [24] Table 4.11.2-2 scenario #1.

-	The UE is switched-off.

-	UE States at the end of the preamble

-	The UE is in state 'switched-off'.

7.3.1.3.2	Test procedure sequence

Table 7.3.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Power up the UE. | - | - | - | - |
| 2 | Trigger the UE to reset UTC time and location. NOTE: The UTC time and location reset may be performed by MMI or AT command (+CUTCR). | - | - | - | - |
| 3 | Activate the MCVideo Client Application and register User A as the MCVideo User (TS 36.579-5 [5], px_MCX_User_A_username, px_MCX_User_A_password). (NOTE 1) | - | - | - | - |
| 4 | Make the MCVideo User request to transmit an emergency alert for Group A. (NOTE 1) | - | - | - | - |
| - | EXCEPTION: The E-UTRA/EPC actions which are related to the MCVideo call establishment are described in TS 36.579-1 [2], subclause 5.4.11 'Generic Test Procedure for MCPTT CO communication over ProSe direct one-to-many communication out of E-UTRA coverage / Monitoring/Discoverer procedure for group member discovery / One-to-many communication'. The test sequence below shows only the MCVideo relevant messages exchanged. | - | - | - | - |
| - | EXCEPTION: Steps 5a1-5b1 describe behaviour that depends on UE implementation; the "lower case letter" identifies a step sequence that takes place when the UE (MCVideo Client) includes user location information in the GROUP EMRGENCY ALERT message or not. | - | - | - | - |
| 5a1 | Check: Does the UE (MCVideo Client) send a GROUP EMERGENCY ALERT message without user location? NOTE: It is expected that the UE shall start timer TFE2 (emergency alert retransmission). | --> | GROUP EMERGENCY ALERT | 1 | P |
| 5b1 | Check: Does the UE (MCVideo Client) send a GROUP EMERGENCY ALERT message with user location? NOTE: It is expected that the UE shall start timer TFE2 (emergency alert retransmission). | --> | GROUP EMERGENCY ALERT | 1 | P |
| 6 | Start TFE2 (emergency alert retransmission) 10 seconds as defined in 36.579-1 [2] Table 5.5.8.1-1. | - | - | - | - |
| 7 | SS-UE1 (MCVideo Client) sends a GROUP EMERGENCY ALERT ACK message. | <-- | GROUP EMERGENCY ALERT ACK | - | - |
| 8 | TFE2 expires. | - | - | - | - |
| - | EXCEPTION: Steps 9a1-9b1 describe behaviour that depends on UE implementation; the "lower case letter" identifies a step sequence that takes place when the UE (MCVideo Client) includes user location information in the GROUP EMRGENCY ALERT message or not. |  |  |  |  |
| 9a1 | Check: Does the UE (MCVideo Client) send a GROUP EMERGENCY ALERT message without user location? NOTE: It is expected that the UE shall start timer TFE2 (emergency alert retransmission). | --> | GROUP EMERGENCY ALERT | 2 | P |
| 9b1 | Check: Does the UE (MCVideo Client) send a GROUP EMERGENCY ALERT message with user location? NOTE: It is expected that the UE shall start timer TFE2 (emergency alert retransmission). | --> | GROUP EMERGENCY ALERT | 2 | P |
| 10 | Before timer TFE2 expires, make the MCVideo User request to cancel the emergency alert for Group A. (NOTE 1) | - | - | - | - |
| 11 | Check: Does the UE (MCVideo Client) send a GROUP EMERGENCY ALERT CANCEL message? NOTE: It is expected that the UE shall stop timer TFE2 (emergency alert retransmission). | --> | GROUP EMERGENCY ALERT CANCEL | 3 | P |
| 12 | SS-UE1 (MCVideo Client) sends a GROUP EMERGENCY ALERT CANCEL ACK message. | <-- | GROUP EMERGENCY ALERT CANCEL ACK | - | - |
| - | EXCEPTION: UE releases the E-UTRA connection. The E-UTRA/EPC actions which are related to the MCVideo call release are described in TS 36.579-1 [2], subclause 5.4.8, 'Generic Test Procedure for MCPTT communication over ProSe direct one-to-one communication out of E-UTRA coverage - release by the UE'. | - | - | - | - |
| NOTE 1: This is expected to be done via a suitable implementation dependent MMI. |  |  |  |  |  |

7.3.1.3.3	Specific message contents

Table 7.3.1.3.3-1: GROUP EMERGENCY ALERT (steps 5b1, 9b1, Table 7.3.1.3.2-1)

Derivation Path: TS 36.579-1 [2], Table 5.5.14.15.1-1, condition USER_LOC

<END OF CHANGES>