3GPP TSG-RAN5 Meeting #95-e	R5-222147

# Online, , 9th May 2022 - 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-7 | CR | 0026 | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New MCData Test Case 7.2.1 Off-network Enhanced Status CO |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NIST |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCenhUEConTest |  |  |  |  |  | Date: |  |  | 2022-04-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | No test case existed for an off-network Enhanced Status CO call |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added test case 7.2.1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | An off-network Enhanced Status CO call would not be tested |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Specific message contents include changes proposed in CR R5-222141 - New MCData off-network signalling messages in 5.5.3.8 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<START OF CHANGES>

## 7.2	Enhanced Status (ES)

### 7.2.1	Off-network / Enhanced Status (ES) / Client Originated (CO)

7.2.1.1	Test Purpose (TP)

(1)

with { UE (MCData Client) registered and authorized for MCData Service, including authorized for MCData Service in off-network environment, and, the UE is in an off-network environment }

ensure that {

when { the MCDATA User requests to send an Enhanced Status with a disposition of only Delivery }

then { UE (MCDATA Client) sends an Enhanced Status with a disposition request of only Delivery via the SDS OFF-NETWORK MESSAGE message, and, initiates counter CFS1 (SDS retransmission) to 1 and starts timer TFS1 (SDS retransmission) }

}

(2)

with { UE (MCData Client) having sent a SDS OFF-NETWORK MESSAGE message and started timer TFS1 (SDS retransmission) }

ensure that {

when { timer TFS1 (SDS retransmission) expires }

then { UE (MCData Client) retransmits the SDS OFF-NETWORK MESSAGE message and, stops re-transmitting if the counter CFS1 (SDS retransmission) has reached its maximum value and TFS1 (SDS retransmission) has expired }

}

7.2.1.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in:

TS 24.282 clauses 14.3.1, 9.3.2.2, 9.3.2.3. Unless otherwise stated these are Rel-15 requirements.

[TS 24.282, clause 14.3.1]

Upon receiving request from MCData user to share enhanced for selected group:

1)	if the value of "/<x>/<x>/Common/MCData/AllowedEnhSvc" leaf node present in the group configuration as specified in 3GPP TS 24.483 [4] is set to "true" for the MCData group, the MCData client:

a)	shall use "/<x>/<x>/Common/MCData/EnhSvcOpValues/<x>/EnhSvcOpID" leaf node associated with user selected enhanced status operation value present in the group configuration as specified in 3GPP TS 24.483 [4] to generate a group standalone SDS message by following the procedure described in clause 9.3.2.2.

[TS 24.282, clause 9.3.2.2]

Upon receiving an indication to send an SDS message, the MCData client:

1)	if the request to send the SDS message is for a MCData group, shall check if the value of "/<x>/<x>/Common/MCData/AllowedSDS" leaf node, present in the group configuration as specified in 3GPP TS 24.483 [42], is set to "false". It the value is set to "false", shall reject the request to send the SDS message and not continue with the remaining procedures in this clause;

2)	if:

a)	a one-to-one SDS message is to be sent then, shall store the MCData user ID of the intended recipient as the target MCData user ID; or

b)	a group SDS message is to be sent then, shall store the MCData group ID as the target MCData group ID;

3)	may set the stored SDS disposition request type as:

a)	"DELIVERY", if only delivery disposition is requested;

b)	"READ", if only read disposition is requested; or

c)	"DELIVERY AND READ", if both delivery and read dispositions are requested;

4)	if an existing conversation is indicated then, shall store the conversation identifier of the indicated conversation as SDS conversation ID. Otherwise, shall generate an UUID as described in IETF RFC 4122 [14] and store SDS conversation ID;

5)	shall generate an UUID as described in IETF RFC 4122 [14] and store as the SDS message ID;

6)	if indicated that the SDS message is in reply to another SDS message then, shall store the message identifier of the indicated message as SDS reply ID;

7)	if indicated that the target recipient of the SDS message is an application then, shall store the application ID of the indicated application as the SDS application ID or as the SDS extended application ID;

8)	shall store the received payload as the SDS payload;

9)	shall store the received payload type as the SDS payload type;

10)	shall store the current UTC time as the SDS transmission time;

11)	shall generate a SDS OFF-NETWORK MESSAGE message as specified in clause 15.1.7. In the SDS OFF-NETWORK MESSAGE message, the MCData client:

a)	shall set the Sender MCData user ID IE to its own MCData user ID;

b)	if:

i)	a one-to-one SDS message is to be sent then shall set the Recipient MCData user ID IE to the stored target MCData user ID as specified in clause 15.2.15; or

ii)	a group SDS message is to be sent then, shall set the MCData group ID IE to the stored target MCData group ID as specified in clause 15.2.14;

c)	may set the SDS disposition request type IE to the stored the SDS disposition request type as specified in clause 15.2.3;

d)	shall set the Conversation ID IE to the stored conversation ID as specified in clause 15.2.9;

e)	shall set the Message ID IE to the stored SDS message ID as specified in clause 15.2.10;

f)	shall set the Date and time IE to the stored SDS transmission time as specified in clause 15.2.8;

g)	may include the InReplyTo message ID IE set to the stored SDS reply ID as specified in clause 15.2.11;

h)	may include:

i)	the Application ID IE set to the stored SDS application ID as specified in clause 15.2.7; or

ii)	the Extended application ID IE set to the stored SDS extended application ID as specified in clause 15.2.24;

i)	if end-to-end security is required for a one-to-one communication and the security context does not exist or if the existing security context has expired, shall include the Security parameters and Payload IE with security parameters as described in 3GPP TS 33.180 [26];

j)	if

i)	end-to-end security is not required for a one-to-one communication, or

ii)	sending the SDS OFF-NETWORK MESSAGE message to a MCData group;

may include the Payload IE as specified in clause 15.2.13 with:

i)	the Payload content type to the stored SDS payload type; and

ii)	the Payload data set to the stored SDS payload;

12)	if:

a)	a one-to-one SDS message is to be sent then, shall send the SDS OFF-NETWORK MESSAGE message as specified in clause 9.3.1.1; or

b)	a group SDS message is to be sent then, shall send the SDS OFF-NETWORK MESSAGE message as specified in clause 9.3.1.2;

13)	shall initialise the counter CFS1 (SDS retransmission) with the value set to 1; and

14)	shall start timer TFS1 (SDS retransmission).

[TS 24.282, clause 9.3.2.3]

Upon expiry of timer TFS1 (SDS retransmission), the MCData client:

1)	shall generate a SDS OFF-NETWORK MESSAGE message as specified in clause 15.1.7. In the SDS OFF-NETWORK MESSAGE message, the MCData client:

a)	shall set the Sender MCData user ID IE to its own MCData user ID;

b)	if:

i)	a one-to-one SDS message is to be sent then, shall set the Recipient MCData user ID IE to the stored target MCData user ID; or

ii)	a group SDS message is to be sent then, shall set the MCData group ID IE to the stored target MCData group ID;

c)	may set the SDS disposition request type IE to the stored the SDS disposition request type as specified in clause 15.2.3;

d)	shall set the Conversation ID IE to the stored conversation ID as specified in clause 15.2.9;

e)	shall set the Message ID IE to the stored SDS message ID as specified in clause 15.2.10;

f)	shall set the Date and time IE to the stored the SDS transmission time as specified in clause 15.2.8;

g)	may include the InReplyTo message ID IE set to the stored SDS reply ID as specified in clause 15.2.11;

h)	may include:

i)	the Application ID IE set to the stored SDS application ID as specified in clause 15.2.7; or

ii)	the Extended application ID IE set to the stored SDS extended application ID as specified in clause 15.2.24;

i)	if end-to-end security is required for a one-to-one communication and the security context does not exist or if the existing security context has expired, shall include the Security parameters IE with security parameters as described in 3GPP TS 33.180 [26]; and

j)	if:

i)	end-to-end security is not required for a one-to-one communication, or

ii)	sending the SDS OFF-NETWORK MESSAGE message to a MCData group;

may include the Payload IE as specified in clause 15.2.13 with:

i)	the Payload content type to the stored SDS payload type; and

ii)	the Payload data set to the stored SDS payload;

2)	if:

a)	a one-to-one SDS message was sent then, shall send the SDS OFF-NETWORK MESSAGE message as specified in clause 9.3.1.1; or

b)	a group SDS message was sent then, shall send the SDS OFF-NETWORK MESSAGE message as specified in clause 9.3.1.2;

3)	shall increment the counter CFS1(SDS retransmission) by 1; and

4)	shall start timer TFS1 (SDS retransmission) if the associated counter CFS1 (SDS retransmission) has not reached its upper limit.

7.2.1.3	Test description

7.2.1.3.1	Pre-test conditions

System Simulator:

-	SS-UE1 (MCData Client)

-	For the underlying "transport bearer" over which the SS and the UE will communicate, the SS is behaving as SS-UE1 as defined in TS 36.508 [24], configured for and operating as ProSe Direct Communication transmitting and receiving device.

-	GNSS simulator to simulate a location and provide a timing reference for the assistance of E-UTRAN off-network testing.

NOTE 1:	For operation in off-network environment, it needs be ensured that after the UE is powered up it considers the Geographical area #1 that is simulated by the GNSS simulator as being one of the geographical areas set in the USIM for operation when UE is "not served by E-UTRAN".

-	SS-NW (MCData server)

-	For the underlying "transport bearer" over which the SS and the UE will communicate Parameters are set to the default parameters for the basic E-UTRA Single cell network scenarios, as defined in TS 36.508 [24] clause 4.4. The simulated Cell 1 shall belong to PLMN1 (the PLMN specified for MCVideo operation in the MCVideo configuration document).

NOTE 2:	The SS operation as NW (MCVideo server) is needed only for the Preamble if the UE has to perform the Generic Test Procedure for MCVideo Authorization/Configuration and Key Generation as specified in TS 36.579-1 [2], subclause 5.3.2.

IUT:

-	UE (MCData Client)

-	The test USIM set as defined in TS 36.579-1 [2], subclause 5.5.10 is inserted.

-	For the underlying "transport bearer" over which the SS and the UE will communicate, the UE is behaving as a ProSe enabled UE as defined in TS 36.508 [24], configured for and operating as ProSe Direct Communication transmitting and receiving device.

-	CFS1 (SDS retransmission) is set to the default value of 5.

-	TFS1 (SDS retransmission) is set to the default value of 40 ms.

Preamble:

--	The UE has performed the Generic Test Procedure for MCData UE registration as specified in TS 36.579-1 [2], subclause 5.4.2B.

-	The MCData User performs the Generic Test Procedure for MCData Authorization/Configuration and Key Generation as specified in TS 36.579-1 [2], subclause 5.3.2.

-	The GNSS simulator is configured to simulate a location in the centre of Geographical area #1 and provide a timing reference, as defined in TS 36.508 [24] Table 4.11.2-2 scenario #1.

-	The UE is switched-off.

-	UE States at the end of the preamble

-	The UE is in state 'switched-off'.

7.2.1.3.2	Test procedure sequence

Table 7.2.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Power up the UE. | - | - | - | - |
| 2 | Trigger the UE to reset UTC time and location. NOTE: The UTC time and location reset may be performed by MMI or AT command (+CUTCR). | - | - | - | - |
| 3 | Activate the MCData Client Application and register User A as the MCData User (TS 36.579-5 [5], px_MCX_User_A_username, px_MCX_User_A_password). (NOTE 1) | - | - | - | - |
| 4 | Make the MCDATA User request to send an enhanced status to Group A using Enhanced Status Id "1" with a disposition request type of "DELIVERY". (NOTE 1) | - | - | - | - |
| - | EXCEPTION: The E-UTRA/EPC actions which are related to the MCData call establishment are described in TS 36.579-1 [2], subclause 5.4.11 'Generic Test Procedure for MCPTT CO communication over ProSe direct one-to-many communication out of E-UTRA coverage / Monitoring/Discoverer procedure for group member discovery / One-to-many communication'. The test sequence below shows only the MCData relevant messages exchanged. | - | - | - | - |
| - | EXCEPTION: Steps 5-7 are repeated CFS1=5 times (CFS1 defined in 24.282 [31] Table G.3.1-1) | - | - | - | - |
| 5 | Check: Does the UE (MCData Client) send an enhanced status via the SDS OFF-NETWORK MESSAGE message with a disposition request type of DELIVERY? NOTE: It is expected that the UE - shall initialize the counter CFS1 (SDS retransmission) with the value set to 1 on the first transmission, and, increase it by 1 with each re-transmission. - shall start timer TFS1 (SDS retransmission) | --> | SDS OFF-NETWORK MESSAGE | 1,2 | P |
| 6 | Start TFS1 (SDS retransmission) 40 milliseconds as defined in 24.282 [31] Table F.3.1-1. | - | - | - | - |
| 7 | TFS1 expires. | - | - | - | - |
| - | EXCEPTION: UE releases the E-UTRA connection. The E-UTRA/EPC actions which are related to the MCData call release are described in TS 36.579-1 [2], subclause 5.4.8, 'Generic Test Procedure for MCPTT communication over ProSe direct one-to-one communication out of E-UTRA coverage - release by the UE'. | - | - | - | - |
| - | EXCEPTION: The E-UTRA/EPC actions which are related to the MCData call establishment are described in TS 36.579-1 [2], subclause 5.4.10 'Generic Test Procedure for MCPTT CT communication over ProSe direct one-to-many communication out of E-UTRA coverage / Announcing/Discoveree procedure for group member discovery'. The test sequence below shows only the MCData relevant messages exchanged. | - | - | - | - |
| - | EXCEPTION: Steps 8-10 are repeated 5 times. | - | - | - | - |
| 8 | SS-UE1 (MCData Client) sends a SDS OFF-NETWORK NOTIFICATION message with disposition notification type of DELIVERED. | <-- | SDS OFF-NETWORK NOTIFICATION | - | - |
| 9 | Start 40 millisecond timer. | - | - | - | - |
| 10 | 40 millisecond timer expires | - | - | - | - |
| - | EXCEPTION: SS releases the E-UTRA connection. The E-UTRA/EPC actions which are related to the MCVideo call release are described in TS 36.579-1 [2], subclause 5.4.7, 'Generic Test Procedure for MCPTT communication over ProSe direct one-to-one communication out of E-UTRA coverage - release by the SS'. | - | - | - | - |
| NOTE 1: This is expected to be done via a suitable implementation dependent MMI. |  |  |  |  |  |

7.2.1.3.3	Specific message contents

Table 7.2.1.3.3-1: SDS OFF-NETWORK MESSAGE (step 5, Table 7.2.1.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.8.9-1, condition DELIVERED, MCD_grp |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |  |
| Payload | Payload as described in Table 7.2.1.3.3-2 |  |  |  |  |

Table 7.2.1.3.3-2: Payload in the SDS OFF-NETWORK MESSAGE (Table 7.2.1.3.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.8.9-3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Data payload message identity | '00000011'B | Data payload | TS 24.282 [87] clause 15.2.2 |  |
| Number of payloads | 1 | 1 payload | TS 24.282 [87] clause 15.2.12 |  |
| Payload |  |  | TS 24.282 [87] clause 15.2.13 |  |
| Payload IEI | '78'O |  |  |  |
| Length of Payload | length of the content |  |  |  |
| Payload content type | '00000110'B | ENHANCED STATUS |  |  |
| Payload data | "1" | The id as defined in the MCData Group Configuration Document | TS 36.579-1 [2], Table 5.5.7.3-1 |  |

Table 7.2.1.3.3-3: SDS OFF-NETWORK NOTIFICATION (step 8, Table 7.2.1.3.2-1)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.8.12-1, condition DELIVERED

<END OF CHANGES>