3GPP TSG-RAN5 Meeting #95-e	R5-222173

# Electronic Meeting, May 9 - 20, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2296 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of test frequencies for CA_n77C BCS0 and BCS1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R16-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Test frequencies for CA_77C BCS0 and BCS1 need to be added to enable testing of NR CA configuration CA_n77C. The supported channel bandwidth combinations for CA_n77C is specified in TS 38.101-1, Table 5.5A.1-1 as: The supported CBWs vs SCS for NR band 77 are: |  |  |  |  |  |  |  |  |
| Summary of change: |  | Test frequencies for SCS=30kHz and supported CBW combinations of CA_n77C BCS0 and BCS1 added to new clause 4.3.1.1.3.77. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Testing of CA_n77C not possible |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1.1.3.77 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | This CR also affects TEI15_Test, 5GS_NR_LTE-UEConTest as the CR also covers Rel-15 changes for CA_n77C BCS0 (witout UL CA) but as the Rel-15 and Rel-16 changes impacts and overlaps the same table it was not possible to split inte separate CRs. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

4.3.1.1.3.67 – 4.3.1.1.3.76	FFS

4.3.1.1.3.77	NR Intra-band contiguous configurations CA_n77

4.3.1.1.3.77.1	CA_n77C

Editor's note: Test frequencies for CA_n78C with mixed numerology with SCS CC1=15kHz and SCS CC2=30 kHz or 60kHz; and SCS CC1=30kHz and SCS CC2=15 kHz or 60 kHz is FFS.

Table 4.3.1.1.3.77.1-1: NR Intra-Band contiguous CA configuration CA_n77C (PCC=CC1 and SCC=CC2), SCS 30 kHz

| CBW combination (BCS) | CC Note 2 | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |  | SS block SCS [kHz] | GSCN |  | absoluteFrequencySSB [ARFCN] |  |  | Offset carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10+100 | CC1 | 10 | 30 | 24 | Downlink | Low | 3305,01 | 620334 | 3300,69 | 620046 | 0 | 30 |  | 7711 | 620352 |  | 18 |  | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3699,99 | 646666 | 3658,95 | 643930 | 102 |  |  | 7985 | 646656 |  | 14 |  | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 4095,39 | 673026 | 3909,63 | 660642 | 504 |  |  | 8260 | 673056 |  | 6 |  | 0 | 3 (3) | 1014 |
|  | Channel spacing CC1-CC2=54,6 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 30 | 273 | Downlink | Low | 3359,61 | 623974 | 3310,47 | 620698 | 0 | 30 |  | 7718 | 621024 |  | 14 |  | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3754,59 | 650306 | 3668,73 | 644582 | 102 |  |  | 7992 | 647328 |  | 10 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4149,99 | 676666 | 3919,41 | 661294 | 504 |  |  | 8266 | 673632 |  | 2 |  | 0 | 0 (0) | 1008 |
| 15+90 | CC1 | 15 | 30 | 38 | Downlink | Low | 3307,5 | 620500 | 3300,66 | 620044 | 0 | 30 |  | 7711 | 620352 |  | 20 |  | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3705 | 647000 | 3661,44 | 644096 | 102 |  |  | 7987 | 646848 |  | 16 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4102,92 | 673528 | 3914,64 | 660976 | 504 |  |  | 8263 | 673344 |  | 8 |  | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=52,08 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 90 | 30 | 245 | Downlink | Low | 3359,58 | 623972 | 3315,48 | 621032 | 0 | 30 |  | 7721 | 621312 |  | 16 |  | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 3757,08 | 650472 | 3676,26 | 645084 | 102 |  |  | 7997 | 647808 |  | 12 |  | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 4155 | 677000 | 3929,46 | 661964 | 504 |  |  | 8273 | 674304 |  | 4 |  | 0 | 0 (0) | 1008 |
| 15+100 | CC1 | 15 | 30 | 38 | Downlink | Low | 3307,5 | 620500 | 3300,66 | 620044 | 0 | 30 |  | 7711 | 620352 |  | 20 |  | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3699,99 | 646666 | 3656,43 | 643762 | 102 |  |  | 7983 | 646464 |  | 14 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4092,87 | 672858 | 3904,59 | 660306 | 504 |  |  | 8256 | 672672 |  | 6 |  | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=57,12 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 30 | 273 | Downlink | Low | 3364,62 | 624308 | 3315,48 | 621032 | 0 | 30 |  | 7721 | 621312 |  | 16 |  | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 3757,11 | 650474 | 3671,25 | 644750 | 102 |  |  | 7994 | 647520 |  | 10 |  | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 4149,99 | 676666 | 3919,41 | 661294 | 504 |  |  | 8266 | 673632 |  | 2 |  | 0 | 0 (0) | 1008 |
| 20+90 | CC1 | 20 | 30 | 51 | Downlink | Low | 3310,02 | 620668 | 3300,84 | 620056 | 0 | 30 |  | 7711 | 620352 |  | 8 |  | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3705 | 647000 | 3659,1 | 643940 | 102 |  |  | 7985 | 646656 |  | 4 |  | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 4100,1 | 673340 | 3909,48 | 660632 | 504 |  |  | 8260 | 673056 |  | 16 |  | 0 | 3 (3) | 1014 |
|  | Channel spacing CC1-CC2=54,9 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 90 | 30 | 245 | Downlink | Low | 3364,92 | 624328 | 3320,82 | 621388 | 0 | 30 |  | 7725 | 621696 |  | 20 |  | 0 | 2 (2) | 4 |
|  |  |  |  |  | & | Mid | 3759,9 | 650660 | 3679,08 | 645272 | 102 |  |  | 7999 | 648000 |  | 16 |  | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 4155 | 677000 | 3929,46 | 661964 | 504 |  |  | 8273 | 674304 |  | 4 |  | 0 | 0 (0) | 1008 |
| 20+100 | CC1 | 20 | 30 | 51 | Downlink | Low | 3310,02 | 620668 | 3300,84 | 620056 | 0 | 30 |  | 7711 | 620352 |  | 8 |  | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3699,99 | 646666 | 3654,09 | 643606 | 102 |  |  | 7982 | 646368 |  | 2 |  | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 4090,05 | 672670 | 3899,43 | 659962 | 504 |  |  | 8253 | 672384 |  | 14 |  | 0 | 3 (3) | 1014 |
|  | Channel spacing CC1-CC2=59,94 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 30 | 273 | Downlink | Low | 3369,96 | 624664 | 3320,82 | 621388 | 0 | 30 |  | 7725 | 621696 |  | 20 |  | 0 | 2 (2) | 4 |
|  |  |  |  |  | & | Mid | 3759,93 | 650662 | 3674,07 | 644938 | 102 |  |  | 7996 | 647712 |  | 14 |  | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 4149,99 | 676666 | 3919,41 | 661294 | 504 |  |  | 8266 | 673632 |  | 2 |  | 0 | 0 (0) | 1008 |
| 25+80 | CC1 | 25 | 30 | 65 | Downlink | Low | 3312,51 | 620834 | 3300,81 | 620054 | 0 | 30 |  | 7711 | 620352 |  | 10 |  | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3710,01 | 647334 | 3661,59 | 644106 | 102 |  |  | 7987 | 646848 |  | 6 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4107,66 | 673844 | 3914,52 | 660968 | 504 |  |  | 8263 | 673344 |  | 16 |  | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=52,32 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 80 | 30 | 217 | Downlink | Low | 3364,83 | 624322 | 3325,77 | 621718 | 0 | 30 |  | 7728 | 621984 |  | 2 |  | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 3762,33 | 650822 | 3686,55 | 645770 | 102 |  |  | 8004 | 648480 |  | 22 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4159,98 | 677332 | 3939,48 | 662632 | 504 |  |  | 8280 | 674976 |  | 8 |  | 0 | 0 (0) | 1008 |
| 25+90 | CC1 | 25 | 30 | 65 | Downlink | Low | 3312,51 | 620834 | 3300,81 | 620054 | 0 | 30 |  | 7711 | 620352 |  | 10 |  | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3705 | 647000 | 3656,58 | 643772 | 102 |  |  | 7983 | 646464 |  | 4 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4097,64 | 673176 | 3904,5 | 660300 | 504 |  |  | 8256 | 672672 |  | 12 |  | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=57,36 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 90 | 30 | 245 | Downlink | Low | 3369,87 | 624658 | 3325,77 | 621718 | 0 | 30 |  | 7728 | 621984 |  | 2 |  | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 3762,36 | 650824 | 3681,54 | 645436 | 102 |  |  | 8001 | 648192 |  | 20 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4155 | 677000 | 3929,46 | 661964 | 504 |  |  | 8273 | 674304 |  | 4 |  | 0 | 0 (0) | 1008 |
| 25+100 | CC1 | 25 | 30 | 65 | Downlink | Low | 3312,51 | 620834 | 3300,81 | 620054 | 0 | 30 |  | 7711 | 620352 |  | 10 |  | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3699,99 | 646666 | 3651,57 | 643438 | 102 |  |  | 7980 | 646176 |  | 2 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4087,59 | 672506 | 3894,45 | 659630 | 504 |  |  | 8249 | 672000 |  | 10 |  | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=62,4 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 30 | 273 | Downlink | Low | 3374,91 | 624994 | 3325,77 | 621718 | 0 | 30 |  | 7728 | 621984 |  | 2 |  | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 3762,39 | 650826 | 3676,53 | 645102 | 102 |  |  | 7997 | 647808 |  | 18 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4149,99 | 676666 | 3919,41 | 661294 | 504 |  |  | 8266 | 673632 |  | 2 |  | 0 | 0 (0) | 1008 |
| 30+80 | CC1 | 30 | 30 | 78 | Downlink | Low | 3315 | 621000 | 3300,96 | 620064 | 0 | 30 |  | 7711 | 620352 |  | 0 |  | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3710,01 | 647334 | 3659,25 | 643950 | 102 |  |  | 7985 | 646656 |  | 18 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4105,14 | 673676 | 3909,66 | 660644 | 504 |  |  | 8260 | 673056 |  | 4 |  | 0 | 3 (3) | 1014 |
|  | Channel spacing CC1-CC2=54,84 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 80 | 30 | 217 | Downlink | Low | 3369,84 | 624656 | 3330,78 | 622052 | 0 | 30 |  | 7732 | 622368 |  | 4 |  | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3764,85 | 650990 | 3689,07 | 645938 | 102 |  |  | 8006 | 648672 |  | 22 |  | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 4159,98 | 677332 | 3939,48 | 662632 | 504 |  |  | 8280 | 674976 |  | 8 |  | 0 | 0 (0) | 1008 |
| 30+90 | CC1 | 30 | 30 | 78 | Downlink | Low | 3315 | 621000 | 3300,96 | 620064 | 0 | 30 |  | 7711 | 620352 |  | 0 |  | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3705 | 647000 | 3654,24 | 643616 | 102 |  |  | 7982 | 646368 |  | 16 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4095,12 | 673008 | 3899,64 | 659976 | 504 |  |  | 8253 | 672384 |  | 0 |  | 0 | 3 (3) | 1014 |
|  | Channel spacing CC1-CC2=59,88 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 90 | 30 | 245 | Downlink | Low | 3374,88 | 624992 | 3330,78 | 622052 | 0 | 30 |  | 7732 | 622368 |  | 4 |  | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3764,88 | 650992 | 3684,06 | 645604 | 102 |  |  | 8003 | 648384 |  | 20 |  | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 4155 | 677000 | 3929,46 | 661964 | 504 |  |  | 8273 | 674304 |  | 4 |  | 0 | 0 (0) | 1008 |
| 30+100 | CC1 | 30 | 30 | 78 | Downlink | Low | 3315 | 621000 | 3300,96 | 620064 | 0 | 30 |  | 7711 | 620352 |  | 0 |  | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3699,99 | 646666 | 3649,23 | 643282 | 102 |  |  | 7978 | 645984 |  | 14 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4085,07 | 672338 | 3889,59 | 659306 | 504 |  |  | 8246 | 671712 |  | 22 |  | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=64,92 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 30 | 273 | Downlink | Low | 3379,92 | 625328 | 3330,78 | 622052 | 0 | 30 |  | 7732 | 622368 |  | 4 |  | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3764,91 | 650994 | 3679,05 | 645270 | 102 |  |  | 7999 | 648000 |  | 18 |  | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 4149,99 | 676666 | 3919,41 | 661294 | 504 |  |  | 8266 | 673632 |  | 2 |  | 0 | 0 (0) | 1008 |
| 40+70 | CC1 | 40 | 30 | 106 | Downlink | Low | 3320,01 | 621334 | 3300,93 | 620062 | 0 | 30 |  | 7711 | 620352 |  | 2 |  | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3714,99 | 647666 | 3659,19 | 643946 | 102 |  |  | 7985 | 646656 |  | 22 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4110,15 | 674010 | 3909,63 | 660642 | 504 |  |  | 8260 | 673056 |  | 6 |  | 0 | 3 (3) | 1014 |
|  | Channel spacing CC1-CC2=54,84 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 70 | 30 | 189 | Downlink | Low | 3374,85 | 624990 | 3340,83 | 622722 | 0 | 30 |  | 7739 | 623040 |  | 6 |  | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3769,83 | 651322 | 3699,09 | 646606 | 102 |  |  | 8013 | 649344 |  | 2 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4164,99 | 677666 | 3949,53 | 663302 | 504 |  |  | 8287 | 675648 |  | 10 |  | 0 | 0 (0) | 1008 |
| 40+80 | CC1 | 40 | 30 | 106 | Downlink | Low | 3320,01 | 621334 | 3300,93 | 620062 | 0 | 30 |  | 7711 | 620352 |  | 2 |  | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3710,01 | 647334 | 3654,21 | 643614 | 102 |  |  | 7982 | 646368 |  | 18 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4100,16 | 673344 | 3899,64 | 659976 | 504 |  |  | 8253 | 672384 |  | 0 |  | 0 | 3 (3) | 1014 |
|  | Channel spacing CC1-CC2=59,82 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 80 | 30 | 217 | Downlink | Low | 3379,83 | 625322 | 3340,77 | 622718 | 0 | 30 |  | 7739 | 623040 |  | 10 |  | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3769,83 | 651322 | 3694,05 | 646270 | 102 |  |  | 8009 | 648960 |  | 2 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4159,98 | 677332 | 3939,48 | 662632 | 504 |  |  | 8280 | 674976 |  | 8 |  | 0 | 0 (0) | 1008 |
| 40+90 | CC1 | 40 | 30 | 106 | Downlink | Low | 3320,01 | 621334 | 3300,93 | 620062 | 0 | 30 |  | 7711 | 620352 |  | 2 |  | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3705 | 647000 | 3649,2 | 643280 | 102 |  |  | 7978 | 645984 |  | 16 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4090,2 | 672680 | 3889,68 | 659312 | 504 |  |  | 8246 | 671712 |  | 16 |  | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=64,8 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 90 | 30 | 245 | Downlink | Low | 3384,81 | 625654 | 3340,71 | 622714 | 0 | 30 |  | 7739 | 623040 |  | 14 |  | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3769,8 | 651320 | 3688,98 | 645932 | 102 |  |  | 8006 | 648672 |  | 4 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4155 | 677000 | 3929,46 | 661964 | 504 |  |  | 8273 | 674304 |  | 4 |  | 0 | 0 (0) | 1008 |
| 40+100 | CC1 | 40 | 30 | 106 | Downlink | Low | 3320,01 | 621334 | 3300,93 | 620062 | 0 | 30 |  | 7711 | 620352 |  | 2 |  | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3699,99 | 646666 | 3644,19 | 642946 | 102 |  |  | 7975 | 645696 |  | 14 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4080,27 | 672018 | 3879,75 | 658650 | 504 |  |  | 8239 | 671040 |  | 6 |  | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=69,72 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 30 | 273 | Downlink | Low | 3389,73 | 625982 | 3340,59 | 622706 | 0 | 30 |  | 7739 | 623040 |  | 22 |  | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3769,71 | 651314 | 3683,85 | 645590 | 102 |  |  | 8002 | 648288 |  | 10 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4149,99 | 676666 | 3919,41 | 661294 | 504 |  |  | 8266 | 673632 |  | 2 |  | 0 | 0 (0) | 1008 |
| 50+60 | CC1 | 50 | 30 | 133 | Downlink | Low | 3325,02 | 621668 | 3301,08 | 620072 | 0 | 30 |  | 7711 | 620352 |  | 16 |  | 0 | 1 (1) | 2 |
| (0,1) |  |  |  |  | & | Mid | 3720 | 648000 | 3659,34 | 643956 | 102 |  |  | 7985 | 646656 |  | 12 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4115,04 | 674336 | 3909,66 | 660644 | 504 |  |  | 8260 | 673056 |  | 4 |  | 0 | 3 (3) | 1014 |
|  | Channel spacing CC1-CC2=54,96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 60 | 30 | 162 | Downlink | Low | 3379,98 | 625332 | 3350,82 | 623388 | 0 | 30 |  | 7746 | 623712 |  | 12 |  | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3774,96 | 651664 | 3709,08 | 647272 | 102 |  |  | 8020 | 650016 |  | 8 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4170 | 678000 | 3959,4 | 663960 | 504 |  |  | 8294 | 676320 |  | 0 |  | 0 | 1 (1) | 1010 |
| 50+70 | CC1 | 50 | 30 | 133 | Downlink | Low | 3325,02 | 621668 | 3301,08 | 620072 | 0 | 30 |  | 7711 | 620352 |  | 16 |  | 0 | 1 (1) | 2 |
| (1) |  |  |  |  | & | Mid | 3714,99 | 647666 | 3654,33 | 643622 | 102 |  |  | 7982 | 646368 |  | 10 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4105,11 | 673674 | 3899,73 | 659982 | 504 |  |  | 8253 | 672384 |  | 18 |  | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=59,88 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 70 | 30 | 189 | Downlink | Low | 3384,9 | 625660 | 3350,88 | 623392 | 0 | 30 |  | 7746 | 623712 |  | 8 |  | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3774,87 | 651658 | 3704,13 | 646942 | 102 |  |  | 8016 | 649632 |  | 2 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4164,99 | 677666 | 3949,53 | 663302 | 504 |  |  | 8287 | 675648 |  | 10 |  | 0 | 0 (0) | 1008 |
| 50+80 | CC1 | 50 | 30 | 133 | Downlink | Low | 3325,02 | 621668 | 3301,08 | 620072 | 0 | 30 |  | 7711 | 620352 |  | 16 |  | 0 | 1 (1) | 2 |
| (0,1) |  |  |  |  | & | Mid | 3710,01 | 647334 | 3649,35 | 643290 | 102 |  |  | 7978 | 645984 |  | 6 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4095,12 | 673008 | 3889,74 | 659316 | 504 |  |  | 8246 | 671712 |  | 12 |  | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=64,86 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 80 | 30 | 217 | Downlink | Low | 3389,88 | 625992 | 3350,82 | 623388 | 0 | 30 |  | 7746 | 623712 |  | 12 |  | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3774,87 | 651658 | 3699,09 | 646606 | 102 |  |  | 8013 | 649344 |  | 2 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4159,98 | 677332 | 3939,48 | 662632 | 504 |  |  | 8280 | 674976 |  | 8 |  | 0 | 0 (0) | 1008 |
| 50+90 | CC1 | 50 | 30 | 133 | Downlink | Low | 3325,02 | 621668 | 3301,08 | 620072 | 0 | 30 |  | 7711 | 620352 |  | 16 |  | 0 | 1 (1) | 2 |
| (1) |  |  |  |  | & | Mid | 3705 | 647000 | 3644,34 | 642956 | 102 |  |  | 7975 | 645696 |  | 4 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4085,16 | 672344 | 3879,78 | 658652 | 504 |  |  | 8239 | 671040 |  | 4 |  | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=69,84 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 90 | 30 | 245 | Downlink | Low | 3394,86 | 626324 | 3350,76 | 623384 | 0 | 30 |  | 7746 | 623712 |  | 16 |  | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3774,84 | 651656 | 3694,02 | 646268 | 102 |  |  | 8009 | 648960 |  | 4 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4155 | 677000 | 3929,46 | 661964 | 504 |  |  | 8273 | 674304 |  | 4 |  | 0 | 0 (0) | 1008 |
| 50+100 | CC1 | 50 | 30 | 133 | Downlink | Low | 3325,02 | 621668 | 3301,08 | 620072 | 0 | 30 |  | 7711 | 620352 |  | 16 |  | 0 | 1 (1) | 2 |
| (0,1) |  |  |  |  | & | Mid | 3699,99 | 646666 | 3639,33 | 642622 | 102 |  |  | 7971 | 645312 |  | 2 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4075,23 | 671682 | 3869,85 | 657990 | 504 |  |  | 8232 | 670368 |  | 18 |  | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=74,76 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 30 | 273 | Downlink | Low | 3399,78 | 626652 | 3350,64 | 623376 | 0 | 30 |  | 7745 | 623616 |  | 0 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 3774,75 | 651650 | 3688,89 | 645926 | 102 |  |  | 8006 | 648672 |  | 10 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4149,99 | 676666 | 3919,41 | 661294 | 504 |  |  | 8266 | 673632 |  | 2 |  | 0 | 0 (0) | 1008 |
| 60+60 | CC1 | 60 | 30 | 162 | Downlink | Low | 3330 | 622000 | 3300,84 | 620056 | 0 | 30 |  | 7711 | 620352 |  | 8 |  | 0 | 2 (2) | 4 |
| (0,1) |  |  |  |  | & | Mid | 3720 | 648000 | 3654,12 | 643608 | 102 |  |  | 7982 | 646368 |  | 0 |  | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 4110 | 674000 | 3899,4 | 659960 | 504 |  |  | 8253 | 672384 |  | 16 |  | 0 | 3 (3) | 1014 |
|  | Channel spacing CC1-CC2=60 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 60 | 30 | 162 | Downlink | Low | 3390 | 626000 | 3360,84 | 624056 | 0 | 30 |  | 7753 | 624384 |  | 16 |  | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3780 | 652000 | 3714,12 | 647608 | 102 |  |  | 8023 | 650304 |  | 8 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4170 | 678000 | 3959,4 | 663960 | 504 |  |  | 8294 | 676320 |  | 0 |  | 0 | 1 (1) | 1010 |
| 60+70 | CC1 | 60 | 30 | 162 | Downlink | Low | 3330 | 622000 | 3300,84 | 620056 | 0 | 30 |  | 7711 | 620352 |  | 8 |  | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3714,99 | 647666 | 3649,11 | 643274 | 102 |  |  | 7978 | 645984 |  | 22 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4100,07 | 673338 | 3889,47 | 659298 | 504 |  |  | 8246 | 671712 |  | 6 |  | 0 | 3 (3) | 1014 |
|  | Channel spacing CC1-CC2=64,92 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 70 | 30 | 189 | Downlink | Low | 3394,92 | 626328 | 3360,9 | 624060 | 0 | 30 |  | 7753 | 624384 |  | 12 |  | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3779,91 | 651994 | 3709,17 | 647278 | 102 |  |  | 8020 | 650016 |  | 2 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4164,99 | 677666 | 3949,53 | 663302 | 504 |  |  | 8287 | 675648 |  | 10 |  | 0 | 0 (0) | 1008 |
| 60+80 | CC1 | 60 | 30 | 162 | Downlink | Low | 3330 | 622000 | 3300,84 | 620056 | 0 | 30 |  | 7711 | 620352 |  | 8 |  | 0 | 2 (2) | 4 |
| (0,1) |  |  |  |  | & | Mid | 3710,01 | 647334 | 3644,13 | 642942 | 102 |  |  | 7975 | 645696 |  | 18 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4090,08 | 672672 | 3879,48 | 658632 | 504 |  |  | 8239 | 671040 |  | 0 |  | 0 | 3 (3) | 1014 |
|  | Channel spacing CC1-CC2=69,9 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 80 | 30 | 217 | Downlink | Low | 3399,9 | 626660 | 3360,84 | 624056 | 0 | 30 |  | 7753 | 624384 |  | 16 |  | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3779,91 | 651994 | 3704,13 | 646942 | 102 |  |  | 8016 | 649632 |  | 2 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4159,98 | 677332 | 3939,48 | 662632 | 504 |  |  | 8280 | 674976 |  | 8 |  | 0 | 0 (0) | 1008 |
| 60+90 | CC1 | 60 | 30 | 162 | Downlink | Low | 3330 | 622000 | 3300,84 | 620056 | 0 | 30 |  | 7711 | 620352 |  | 8 |  | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3705 | 647000 | 3639,12 | 642608 | 102 |  |  | 7971 | 645312 |  | 16 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4080,12 | 672008 | 3869,52 | 657968 | 504 |  |  | 8232 | 670368 |  | 16 |  | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=74,88 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 90 | 30 | 245 | Downlink | Low | 3404,88 | 626992 | 3360,78 | 624052 | 0 | 30 |  | 7753 | 624384 |  | 20 |  | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3779,88 | 651992 | 3699,06 | 646604 | 102 |  |  | 8013 | 649344 |  | 4 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4155 | 677000 | 3929,46 | 661964 | 504 |  |  | 8273 | 674304 |  | 4 |  | 0 | 0 (0) | 1008 |
| 60+100 | CC1 | 60 | 30 | 162 | Downlink | Low | 3330 | 622000 | 3300,84 | 620056 | 0 | 30 |  | 7711 | 620352 |  | 8 |  | 0 | 2 (2) | 4 |
| (0,1) |  |  |  |  | & | Mid | 3699,99 | 646666 | 3634,11 | 642274 | 102 |  |  | 7968 | 645024 |  | 14 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4070,19 | 671346 | 3859,59 | 657306 | 504 |  |  | 8225 | 669696 |  | 6 |  | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=79,8 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 30 | 273 | Downlink | Low | 3409,8 | 627320 | 3360,66 | 624044 | 0 | 30 |  | 7752 | 624288 |  | 4 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 3779,79 | 651986 | 3693,93 | 646262 | 102 |  |  | 8009 | 648960 |  | 10 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4149,99 | 676666 | 3919,41 | 661294 | 504 |  |  | 8266 | 673632 |  | 2 |  | 0 | 0 (0) | 1008 |
| 70+60 | CC1 | 70 | 30 | 189 | Downlink | Low | 3335,01 | 622334 | 3300,99 | 620066 | 0 | 30 |  | 7711 | 620352 |  | 22 |  | 0 | 1 (1) | 2 |
| (1) |  |  |  |  | & | Mid | 3720 | 648000 | 3649,26 | 643284 | 102 |  |  | 7978 | 645984 |  | 12 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4105,08 | 673672 | 3889,62 | 659308 | 504 |  |  | 8246 | 671712 |  | 20 |  | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=64,92 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 60 | 30 | 162 | Downlink | Low | 3399,93 | 626662 | 3370,77 | 624718 | 0 | 30 |  | 7759 | 624960 |  | 2 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 3784,92 | 652328 | 3719,04 | 647936 | 102 |  |  | 8027 | 650688 |  | 16 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4170 | 678000 | 3959,4 | 663960 | 504 |  |  | 8294 | 676320 |  | 0 |  | 0 | 1 (1) | 1010 |
| 70+70 | CC1 | 70 | 30 | 189 | Downlink | Low | 3335,01 | 622334 | 3300,99 | 620066 | 0 | 30 |  | 7711 | 620352 |  | 22 |  | 0 | 1 (1) | 2 |
| (1) |  |  |  |  | & | Mid | 3714,99 | 647666 | 3644,25 | 642950 | 102 |  |  | 7975 | 645696 |  | 10 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4095,03 | 673002 | 3879,57 | 658638 | 504 |  |  | 8239 | 671040 |  | 18 |  | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=69,96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 70 | 30 | 189 | Downlink | Low | 3404,97 | 626998 | 3370,95 | 624730 | 0 | 30 |  | 7760 | 625056 |  | 14 |  | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3784,95 | 652330 | 3714,21 | 647614 | 102 |  |  | 8023 | 650304 |  | 2 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4164,99 | 677666 | 3949,53 | 663302 | 504 |  |  | 8287 | 675648 |  | 10 |  | 0 | 0 (0) | 1008 |
| 70+80 | CC1 | 70 | 30 | 189 | Downlink | Low | 3335,01 | 622334 | 3300,99 | 620066 | 0 | 30 |  | 7711 | 620352 |  | 22 |  | 0 | 1 (1) | 2 |
| (1) |  |  |  |  | & | Mid | 3710,01 | 647334 | 3639,27 | 642618 | 102 |  |  | 7971 | 645312 |  | 6 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4085,04 | 672336 | 3869,58 | 657972 | 504 |  |  | 8232 | 670368 |  | 12 |  | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=74,94 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 80 | 30 | 217 | Downlink | Low | 3409,95 | 627330 | 3370,89 | 624726 | 0 | 30 |  | 7760 | 625056 |  | 18 |  | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3784,95 | 652330 | 3709,17 | 647278 | 102 |  |  | 8020 | 650016 |  | 2 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4159,98 | 677332 | 3939,48 | 662632 | 504 |  |  | 8280 | 674976 |  | 8 |  | 0 | 0 (0) | 1008 |
| 70+90 | CC1 | 70 | 30 | 189 | Downlink | Low | 3335,01 | 622334 | 3300,99 | 620066 | 0 | 30 |  | 7711 | 620352 |  | 22 |  | 0 | 1 (1) | 2 |
| (1) |  |  |  |  | & | Mid | 3705 | 647000 | 3634,26 | 642284 | 102 |  |  | 7968 | 645024 |  | 4 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4075,08 | 671672 | 3859,62 | 657308 | 504 |  |  | 8225 | 669696 |  | 4 |  | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=79,92 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 90 | 30 | 245 | Downlink | Low | 3414,93 | 627662 | 3370,83 | 624722 | 0 | 30 |  | 7760 | 625056 |  | 22 |  | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3784,92 | 652328 | 3704,1 | 646940 | 102 |  |  | 8016 | 649632 |  | 4 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4155 | 677000 | 3929,46 | 661964 | 504 |  |  | 8273 | 674304 |  | 4 |  | 0 | 0 (0) | 1008 |
| 70+100 | CC1 | 70 | 30 | 189 | Downlink | Low | 3335,01 | 622334 | 3300,99 | 620066 | 0 | 30 |  | 7711 | 620352 |  | 22 |  | 0 | 1 (1) | 2 |
| (1) |  |  |  |  | & | Mid | 3699,99 | 646666 | 3629,25 | 641950 | 102 |  |  | 7964 | 644640 |  | 2 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4065,15 | 671010 | 3849,69 | 656646 | 504 |  |  | 8218 | 669024 |  | 18 |  | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=84,84 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 30 | 273 | Downlink | Low | 3419,85 | 627990 | 3370,71 | 624714 | 0 | 30 |  | 7759 | 624960 |  | 6 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 3784,83 | 652322 | 3698,97 | 646598 | 102 |  |  | 8013 | 649344 |  | 10 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4149,99 | 676666 | 3919,41 | 661294 | 504 |  |  | 8266 | 673632 |  | 2 |  | 0 | 0 (0) | 1008 |
| 80+60 | CC1 | 80 | 30 | 217 | Downlink | Low | 3340,02 | 622668 | 3300,96 | 620064 | 0 | 30 |  | 7711 | 620352 |  | 0 |  | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3720 | 648000 | 3644,22 | 642948 | 102 |  |  | 7975 | 645696 |  | 12 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4100,1 | 673340 | 3879,6 | 658640 | 504 |  |  | 8239 | 671040 |  | 16 |  | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=69,9 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 60 | 30 | 162 | Downlink | Low | 3409,92 | 627328 | 3380,76 | 625384 | 0 | 30 |  | 7766 | 625632 |  | 8 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 3789,9 | 652660 | 3724,02 | 648268 | 102 |  |  | 8030 | 650976 |  | 20 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4170 | 678000 | 3959,4 | 663960 | 504 |  |  | 8294 | 676320 |  | 0 |  | 0 | 1 (1) | 1010 |
| 80+70 | CC1 | 80 | 30 | 217 | Downlink | Low | 3340,02 | 622668 | 3300,96 | 620064 | 0 | 30 |  | 7711 | 620352 |  | 0 |  | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3714,99 | 647666 | 3639,21 | 642614 | 102 |  |  | 7971 | 645312 |  | 10 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4090,05 | 672670 | 3869,55 | 657970 | 504 |  |  | 8232 | 670368 |  | 14 |  | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=74,94 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 70 | 30 | 189 | Downlink | Low | 3414,96 | 627664 | 3380,94 | 625396 | 0 | 30 |  | 7767 | 625728 |  | 20 |  | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3789,93 | 652662 | 3719,19 | 647946 | 102 |  |  | 8027 | 650688 |  | 6 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4164,99 | 677666 | 3949,53 | 663302 | 504 |  |  | 8287 | 675648 |  | 10 |  | 0 | 0 (0) | 1008 |
| 80+80 | CC1 | 80 | 30 | 217 | Downlink | Low | 3340,02 | 622668 | 3300,96 | 620064 | 0 | 30 |  | 7711 | 620352 |  | 0 |  | 0 | 2 (2) | 4 |
| (0,1) |  |  |  |  | & | Mid | 3710,01 | 647334 | 3634,23 | 642282 | 102 |  |  | 7968 | 645024 |  | 6 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4080 | 672000 | 3859,5 | 657300 | 504 |  |  | 8225 | 669696 |  | 12 |  | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=79,98 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 80 | 30 | 217 | Downlink | Low | 3420 | 628000 | 3380,94 | 625396 | 0 | 30 |  | 7767 | 625728 |  | 20 |  | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3789,99 | 652666 | 3714,21 | 647614 | 102 |  |  | 8023 | 650304 |  | 2 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4159,98 | 677332 | 3939,48 | 662632 | 504 |  |  | 8280 | 674976 |  | 8 |  | 0 | 0 (0) | 1008 |
| 80+90 | CC1 | 80 | 30 | 217 | Downlink | Low | 3340,02 | 622668 | 3300,96 | 620064 | 0 | 30 |  | 7711 | 620352 |  | 0 |  | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3705 | 647000 | 3629,22 | 641948 | 102 |  |  | 7964 | 644640 |  | 4 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4070,04 | 671336 | 3849,54 | 656636 | 504 |  |  | 8218 | 669024 |  | 4 |  | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=84,96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 90 | 30 | 245 | Downlink | Low | 3424,98 | 628332 | 3380,88 | 625392 | 0 | 30 |  | 7766 | 625632 |  | 0 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 3789,96 | 652664 | 3709,14 | 647276 | 102 |  |  | 8020 | 650016 |  | 4 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4155 | 677000 | 3929,46 | 661964 | 504 |  |  | 8273 | 674304 |  | 4 |  | 0 | 0 (0) | 1008 |
| 80+100 | CC1 | 80 | 30 | 217 | Downlink | Low | 3340,02 | 622668 | 3300,96 | 620064 | 0 | 30 |  | 7711 | 620352 |  | 0 |  | 0 | 2 (2) | 4 |
| (0,1) |  |  |  |  | & | Mid | 3699,99 | 646666 | 3624,21 | 641614 | 102 |  |  | 7961 | 644352 |  | 2 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4060,11 | 670674 | 3839,61 | 655974 | 504 |  |  | 8211 | 668352 |  | 18 |  | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=89,88 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 30 | 273 | Downlink | Low | 3429,9 | 628660 | 3380,76 | 625384 | 0 | 30 |  | 7766 | 625632 |  | 8 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 3789,87 | 652658 | 3704,01 | 646934 | 102 |  |  | 8016 | 649632 |  | 10 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4149,99 | 676666 | 3919,41 | 661294 | 504 |  |  | 8266 | 673632 |  | 2 |  | 0 | 0 (0) | 1008 |
| 90+60 | CC1 | 90 | 30 | 245 | Downlink | Low | 3345 | 623000 | 3300,9 | 620060 | 0 | 30 |  | 7711 | 620352 |  | 4 |  | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3720 | 648000 | 3639,18 | 642612 | 102 |  |  | 7971 | 645312 |  | 12 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4095,12 | 673008 | 3869,58 | 657972 | 504 |  |  | 8232 | 670368 |  | 12 |  | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=74,88 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 60 | 30 | 162 | Downlink | Low | 3419,88 | 627992 | 3390,72 | 626048 | 0 | 30 |  | 7773 | 626304 |  | 16 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 3794,88 | 652992 | 3729 | 648600 | 102 |  |  | 8034 | 651360 |  | 0 |  | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 4170 | 678000 | 3959,4 | 663960 | 504 |  |  | 8294 | 676320 |  | 0 |  | 0 | 1 (1) | 1010 |
| 90+70 | CC1 | 90 | 30 | 245 | Downlink | Low | 3345 | 623000 | 3300,9 | 620060 | 0 | 30 |  | 7711 | 620352 |  | 4 |  | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3714,99 | 647666 | 3634,17 | 642278 | 102 |  |  | 7968 | 645024 |  | 10 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4085,07 | 672338 | 3859,53 | 657302 | 504 |  |  | 8225 | 669696 |  | 10 |  | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=79,92 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 70 | 30 | 189 | Downlink | Low | 3424,92 | 628328 | 3390,9 | 626060 | 0 | 30 |  | 7773 | 626304 |  | 4 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 3794,91 | 652994 | 3724,17 | 648278 | 102 |  |  | 8030 | 650976 |  | 10 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4164,99 | 677666 | 3949,53 | 663302 | 504 |  |  | 8287 | 675648 |  | 10 |  | 0 | 0 (0) | 1008 |
| 90+80 | CC1 | 90 | 30 | 245 | Downlink | Low | 3345 | 623000 | 3300,9 | 620060 | 0 | 30 |  | 7711 | 620352 |  | 4 |  | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3710,01 | 647334 | 3629,19 | 641946 | 102 |  |  | 7964 | 644640 |  | 6 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4075,02 | 671668 | 3849,48 | 656632 | 504 |  |  | 8218 | 669024 |  | 8 |  | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=84,96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 80 | 30 | 217 | Downlink | Low | 3429,96 | 628664 | 3390,9 | 626060 | 0 | 30 |  | 7773 | 626304 |  | 4 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 3794,97 | 652998 | 3719,19 | 647946 | 102 |  |  | 8027 | 650688 |  | 6 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4159,98 | 677332 | 3939,48 | 662632 | 504 |  |  | 8280 | 674976 |  | 8 |  | 0 | 0 (0) | 1008 |
| 90+90 | CC1 | 90 | 30 | 245 | Downlink | Low | 3345 | 623000 | 3300,9 | 620060 | 0 | 30 |  | 7711 | 620352 |  | 4 |  | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3705 | 647000 | 3624,18 | 641612 | 102 |  |  | 7961 | 644352 |  | 4 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4065 | 671000 | 3839,46 | 655964 | 504 |  |  | 8211 | 668352 |  | 4 |  | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=90 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 90 | 30 | 245 | Downlink | Low | 3435 | 629000 | 3390,9 | 626060 | 0 | 30 |  | 7773 | 626304 |  | 4 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 3795 | 653000 | 3714,18 | 647612 | 102 |  |  | 8023 | 650304 |  | 4 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4155 | 677000 | 3929,46 | 661964 | 504 |  |  | 8273 | 674304 |  | 4 |  | 0 | 0 (0) | 1008 |
| 90+100 | CC1 | 90 | 30 | 245 | Downlink | Low | 3345 | 623000 | 3300,9 | 620060 | 0 | 30 |  | 7711 | 620352 |  | 4 |  | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3699,99 | 646666 | 3619,17 | 641278 | 102 |  |  | 7957 | 643968 |  | 2 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4055,07 | 670338 | 3829,53 | 655302 | 504 |  |  | 8204 | 667680 |  | 18 |  | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=94,92 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 30 | 273 | Downlink | Low | 3439,92 | 629328 | 3390,78 | 626052 | 0 | 30 |  | 7773 | 626304 |  | 12 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 3794,91 | 652994 | 3709,05 | 647270 | 102 |  |  | 8020 | 650016 |  | 10 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4149,99 | 676666 | 3919,41 | 661294 | 504 |  |  | 8266 | 673632 |  | 2 |  | 0 | 0 (0) | 1008 |
| 100+60 | CC1 | 100 | 30 | 273 | Downlink | Low | 3350,01 | 623334 | 3300,87 | 620058 | 0 | 30 |  | 7711 | 620352 |  | 6 |  | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3720 | 648000 | 3634,14 | 642276 | 102 |  |  | 7968 | 645024 |  | 12 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4090,2 | 672680 | 3859,62 | 657308 | 504 |  |  | 8225 | 669696 |  | 4 |  | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=79,8 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 60 | 30 | 162 | Downlink | Low | 3429,81 | 628654 | 3400,65 | 626710 | 0 | 30 |  | 7780 | 626976 |  | 2 |  | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 3799,8 | 653320 | 3733,92 | 648928 | 102 |  |  | 8037 | 651648 |  | 8 |  | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 4170 | 678000 | 3959,4 | 663960 | 504 |  |  | 8294 | 676320 |  | 0 |  | 0 | 1 (1) | 1010 |
| 100+70 | CC1 | 100 | 30 | 273 | Downlink | Low | 3350,01 | 623334 | 3300,87 | 620058 | 0 | 30 |  | 7711 | 620352 |  | 6 |  | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3714,99 | 647666 | 3629,13 | 641942 | 102 |  |  | 7964 | 644640 |  | 10 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4080,15 | 672010 | 3849,57 | 656638 | 504 |  |  | 8218 | 669024 |  | 2 |  | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=84,84 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 70 | 30 | 189 | Downlink | Low | 3434,85 | 628990 | 3400,83 | 626722 | 0 | 30 |  | 7780 | 626976 |  | 14 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 3799,83 | 653322 | 3729,09 | 648606 | 102 |  |  | 8034 | 651360 |  | 18 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4164,99 | 677666 | 3949,53 | 663302 | 504 |  |  | 8287 | 675648 |  | 10 |  | 0 | 0 (0) | 1008 |
| 100+80 | CC1 | 100 | 30 | 273 | Downlink | Low | 3350,01 | 623334 | 3300,87 | 620058 | 0 | 30 |  | 7711 | 620352 |  | 6 |  | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3710,01 | 647334 | 3624,15 | 641610 | 102 |  |  | 7961 | 644352 |  | 6 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4070,1 | 671340 | 3839,52 | 655968 | 504 |  |  | 8211 | 668352 |  | 0 |  | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=89,88 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 80 | 30 | 217 | Downlink | Low | 3439,89 | 629326 | 3400,83 | 626722 | 0 | 30 |  | 7780 | 626976 |  | 14 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 3799,89 | 653326 | 3724,11 | 648274 | 102 |  |  | 8030 | 650976 |  | 14 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4159,98 | 677332 | 3939,48 | 662632 | 504 |  |  | 8280 | 674976 |  | 8 |  | 0 | 0 (0) | 1008 |
| 100+90 | CC1 | 100 | 30 | 273 | Downlink | Low | 3350,01 | 623334 | 3300,87 | 620058 | 0 | 30 |  | 7711 | 620352 |  | 6 |  | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3705 | 647000 | 3619,14 | 641276 | 102 |  |  | 7957 | 643968 |  | 4 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4060,08 | 670672 | 3829,5 | 655300 | 504 |  |  | 8204 | 667680 |  | 20 |  | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=94,92 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 90 | 30 | 245 | Downlink | Low | 3444,93 | 629662 | 3400,83 | 626722 | 0 | 30 |  | 7780 | 626976 |  | 14 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 3799,92 | 653328 | 3719,1 | 647940 | 102 |  |  | 8027 | 650688 |  | 12 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4155 | 677000 | 3929,46 | 661964 | 504 |  |  | 8273 | 674304 |  | 4 |  | 0 | 0 (0) | 1008 |
| 100 +100 | CC1 | 100 | 30 | 273 | Downlink | Low | 3350,01 | 623334 | 3300,87 | 620058 | 0 | 30 |  | 7711 | 620352 |  | 6 |  | 0 | 2 (2) | 4 |
| (0,1) |  |  |  |  | & | Mid | 3699,99 | 646666 | 3614,13 | 640942 | 102 |  |  | 7954 | 643680 |  | 2 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4050,03 | 670002 | 3819,45 | 654630 | 504 |  |  | 8197 | 667008 |  | 18 |  | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=99,96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 30 | 273 | Downlink | Low | 3449,97 | 629998 | 3400,83 | 626722 | 0 | 30 |  | 7780 | 626976 |  | 14 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 3799,95 | 653330 | 3714,09 | 647606 | 102 |  |  | 8023 | 650304 |  | 10 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4149,99 | 676666 | 3919,41 | 661294 | 504 |  |  | 8266 | 673632 |  | 2 |  | 0 | 0 (0) | 1008 |
| Note 1: Corresponds to nominal channel spacing in accordance with TS 38.101-1 [7], clause 5.4A.1 for the CC1 and CC2 channel bandwidth combination. Note 2: CCs are specified in increasing frequency order. CC1 is used as PCell if nothing else is specified in the test case. Note 3: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 4: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-4 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

4.3.1.1.3.78	NR Intra-band contiguous configurations CA_n78