3GPP TSG-RAN5 Meeting #95-e	R5-222543

# Electronic Meeting, 9th – 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-3 | CR | 1365 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of Refsens CA test case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R16-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | MSD due to dual uplink for DC_1A_n5A has been removed in TS 38.101-3 in the last meeting but is still present in TS 38.521-3. MSD due to dual uplink for DC_2A_n78A in Table 7.3B.2.0.3.5.1 is not aligned to TS 38.101-3. |  |  |  |  |  |  |  |  |
| Summary of change: |  | MSD due to dual uplink for DC_1A_n5A has been removed. Test configuration for DC_1A_n5A in Table 7.3B.2.3.4.2.1-6 has been removed. Test requirement for DC_1A_n5A in Table 7.3B.2.3.5-4 has been deleted. DL Fc for DC_2A_n78A has been aligned to core specification. The IMD2 entry for DC_2A_n78A has been moved down and merged with the IMD4 entry. Corrected typo in test configuration for DC_2A_n78A in Table 7.3B.2.3.4.2.1-6. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Refsens for DC_1A_n5A is tested not according to core requirement. Refsens for DC_2A_n78A cannot be tested. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3B.2.0.3.5.1, 7.3B.2.3.4.2.1, 7.3B.2.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

7.3B.2.0.3.5.1	MSD test points for intermodulation interference due to dual uplink operation for PC3 EN-DC in NR FR1 involving two bands

Table 7.3B.2.0.3.5.1-1: MSD test points for PCell due to dual uplink operation for PC3 EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration |  | EUTRA or NR band |  | UL Fc (MHz) |  | UL/DL BW (MHz) |  | UL LCRB |  | DL Fc (MHz) |  | MSD (dB) |  | IMD order |
| DC_1_n3 |  | 1 |  | 1950 |  | 5 |  | 25 |  | 2140 |  | 23 |  | IMD3 |
|  |  | n3 |  | 1760 |  | 5 |  | 25 |  | 1855 |  | N/A |  | N/A |
| DC_1A_n77A |  | 1 |  | 1950 |  | 5 |  | 25 |  | 2140 |  | 29.8 |  | IMD23 |
| DC_1A_SUL_n77A- |  |  |  |  |  |  |  |  |  |  |  | 32.54 |  |  |
| n84A |  | n77, n78 |  | 4090 |  | 10 |  | 50 |  | 4090 |  | N/A |  | N/A |
| DC_1A_n77A |  | 1 |  | 1950 |  | 5 |  | 25 |  | 2140 |  | 8.0 |  | IMD43 |
| DC_1A-SUL_n77A-n84A |  |  |  |  |  |  |  |  |  |  |  | 10.74 |  |  |
| DC_1A_n78A, DC_1A_SUL_n78A-n84A |  | n77, n78 |  | 3710 |  | 10 |  | 50 |  | 3710 |  | N/A |  | N/A |
| DC_1A_n78A, |  | 1 |  | 1950 |  | 5 |  | 25 |  | 2140 |  | 8.0 |  | IMD43 |
| DC_1A_SUL_n78A- |  |  |  |  |  |  |  |  |  |  |  | 10.74 |  |  |
| n84A |  | n78 |  | 3710 |  | 10 |  | 25 |  | 3710 |  | N/A |  |  |
| DC_2A_n66A |  | 2 |  | 1855 |  | 5 |  | 25 |  | 1935 |  | 20 |  | IMD3 |
| DC_2A-2A_n66A |  | n66 |  | 1775 |  | 5 |  | 25 |  | 2175 |  | N/A |  | N/A |
| DC_2A_n66A |  | 2 |  | 1883.3 |  | 5 |  | 25 |  | 1963.3 |  | N/A |  | N/A |
| DC_2A-2A_n66A |  | n66 |  | 1750 |  | 5 |  | 25 |  | 2150 |  | 4 |  | IMD5 |
| DC_2A_n77A DC_2A-2A_n77A |  | 2 |  | 1855 |  | 5 |  | 25 |  | 1935 |  | 26 |  | IMD2 |
|  |  |  |  |  |  |  |  |  |  |  |  | 28.74 |  |  |
|  |  | n77 |  | 3790 |  | 10 |  | 50 |  | 3790 |  | N/A |  | N/A |
|  |  | 2 |  | 1900 |  | 5 |  | 25 |  | 1980 |  | 8.0 |  | IMD4 |
|  |  |  |  |  |  |  |  |  |  |  |  | 10.74 |  |  |
|  |  | n77 |  | 3720 |  | 10 |  | 50 |  | 3720 |  | N/A |  | N/A |
|  |  | 2 |  | 1885 |  | 5 |  | 25 |  | 1965 |  | 5 |  | IMD5 |
|  |  |  |  |  |  |  |  |  |  |  |  | 7.74 |  |  |
|  |  | n77 |  | 3810 |  | 10 |  | 50 |  | 3810 |  | N/A |  | N/A |
| DC_2A_n78A |  | 2 |  | 1855 |  | 5 |  | 25 |  | 1935 |  | 26 |  | IMD23 |
|  |  |  |  |  |  |  |  |  |  |  |  | 28.74 |  |  |
|  |  | n78 |  | 3790 |  | 10 |  | 50 |  | 3790 |  | N/A |  | N/A |
|  |  | 2 |  | 1885 |  | 5 |  | 25 |  | 1965 |  | 8.0 |  | IMD43 |
|  |  |  |  |  |  |  |  |  |  |  |  | 10.74 |  |  |
|  |  | n78 |  | 3690 |  | 10 |  | 50 |  | 3690 |  | N/A |  | N/A |
| DC_3_n1 |  | 3 |  | 1760 |  | 5 |  | 25 |  | 1855 |  | N/A |  | N/A |
|  |  | n1 |  | 1950 |  | 5 |  | 25 |  | 2140 |  | 23 |  | IMD3 |
|  |  | 3 |  | 1771 |  | 10 |  | 50 |  | 1866 |  | 4 |  | IMD4 |
| DC_3_n5 |  | n5 |  | 838 |  | 5 |  | 25 |  | 883 |  | N/A |  | N/A |
|  |  | 3 |  | 1721 |  | 10 |  | 50 |  | 1816 |  | N/A |  | N/A |
|  |  | n5 |  | 838 |  | 5 |  | 25 |  | 883 |  | 24 |  | IMD23 |
| DC_3A_n7A |  | 3 |  | 1730 |  | 5 |  | 25 |  | 1825 |  | N/A |  | N/A |
| DC_3C_n7A |  | n7 |  | 2535 |  | 10 |  | 50 |  | 2655 |  | 10.25 |  | IMD4 |
|  |  | 3 |  | 1775 |  | 5 |  | 25 |  | 1870 |  | 4 |  | IMD4 |
| DC_3A_n20A |  | n20 |  | 840 |  | 5 |  | 25 |  | 799 |  | N/A |  | N/A |
|  |  | 3 |  | 1735 |  | 5 |  | 25 |  | 1830 |  | N/A |  | N/A |
|  |  | n20 |  | 847 |  | 5 |  | 25 |  | 806 |  | 9 |  | IMD4 |
| DC_3A_n41A |  | 3 |  | 1740 |  | 5 |  | 25 |  | 1835 |  | 8.2 |  | IMD4 |
| DC_3C_n41A |  | n41 |  | 2657.5 |  | 10 |  | 50 |  | 2657.5 |  | N/A |  | IMD4 |
| DC_3A_SUL_n41A-n80A, |  | 3 |  | 1740 |  | 5 |  | 25 |  | 1835 |  | 8.2 |  | IMD4 |
| DC_3C_SUL_n41A-n80A |  | n41 |  | 2657.5 |  | 10 |  | 52 |  | 2657.5 |  | N/A |  | N/A |
| DC_3A_n77A |  | 3 |  | 1740 |  | 5 |  | 25 |  | 1835 |  | 26 |  | IMD23 |
| DC_3A_SUL_n77A-n80A DC_3A_n78A |  |  |  |  |  |  |  |  |  |  |  | 28.74 |  |  |
| DC_3A-SUL_n78A-n80A, DC_3C_n78A |  | n77, n78 |  | 3575 |  | 10 |  | 50 |  | 3575 |  | N/A |  | N/A |
| DC_3A_n77A |  | 3 |  | 1765 |  | 5 |  | 25 |  | 1860 |  | 8.0 |  | IMD43 |
| DC_3A_SUL_n77A-n80A, DC_3A_n78A |  |  |  |  |  |  |  |  |  |  |  | 10.74 |  |  |
| DC_3A-SUL_n78A-n80A, DC_3C_n78A |  | n77, n78 |  | 3435 |  | 10 |  | 50 |  | 3435 |  | N/A |  | N/A |
| DC_5A_n66A |  | 5 |  | 838 |  | 5 |  | 25 |  | 883 |  | 30 |  | IMD23 |
|  |  | n66 |  | 1721 |  | 5 |  | 25 |  | 2121 |  | N/A |  | N/A |
| DC_5A_n78A |  | 5 |  | 844 |  | 5 |  | 25 |  | 889 |  | 8.3 |  | IMD4 |
|  |  | n78 |  | 3421 |  | 10 |  | 50 |  | 3421 |  | N/A |  | N/A |
| DC_5A_n77A8 |  | 5 |  | 844 |  | 5 |  | 25 |  | 889 |  | 8.3 |  | IMD4 |
|  |  | n77 |  | 3421 |  | 10 |  | 50 |  | 3421 |  | N/A |  | N/A |
|  |  | 5 |  | 826.5 |  | 5 |  | 25 |  | 871.5 |  | 5.5 |  | IMD5 |
|  |  | n77 |  | 4177.5 |  | 10 |  | 50 |  | 4177.5 |  | N/A |  | N/A |
| DC_7_n3 |  | 7 |  | 2535 |  | 10 |  | 50 |  | 2655 |  | 13 |  | IMD4 |
|  |  | n3 |  | 1730 |  | 5 |  | 25 |  | 1825 |  | N/A |  | N/A |
| DC_7_n5 |  | 7 |  | 2547 |  | 10 |  | 50 |  | 2667 |  | N/A |  | N/A |
|  |  | n5 |  | 834 |  | 5 |  | 25 |  | 879 |  | 12 |  | IMD33 |
| DC_7A_n77A |  | 7 |  | 2540 |  | 5 |  | 25 |  | 2660 |  | 7.1 |  | IMD4 |
|  |  | n77 |  | 3870 |  | 10 |  | 50 |  | 3870 |  | N/A |  | N/A |
| DC_7A_n66A |  | 7 |  | 2535 |  | 10 |  | 50 |  | 2655 |  | 15 |  | 4th IMD |
| DC_7A-7A_n66A DC_7C_n66A |  | n66 |  | 1730 |  | 5 |  | 25 |  | 2130 |  | N/A |  | N/A |
| DC_8A_n1A |  | 8 |  | 887.5 |  | 5 |  | 25 |  | 932.5 |  | N/A |  | N/A |
|  |  | n1 |  | 1965 |  | 5 |  | 25 |  | 2155 |  | 6 |  | IMD4 |
|  |  | 8 |  | 900 |  | 5 |  | 25 |  | 945 |  | 8 |  | IMD43 |
| DC_8A_n3A |  | n3 |  | 1755 |  | 10 |  | 50 |  | 1850 |  | N/A |  | N/A |
|  |  | 8 |  | 897.5 |  | 5 |  | 25 |  | 942.5 |  | N/A |  | N/A |
|  |  | n3 |  | 1747.5 |  | 10 |  | 50 |  | 1842.5 |  | 6.4 |  | IMD5 |
| DC_8A_n20A | n20 |  | 849.5 |  | 5 |  | 25 |  | 808.5 |  | 25 |  | IMD33 |  |
|  | 8 |  | 890.5 |  | 5 |  | 25 |  | 935.5 |  | N/A |  | N/A |  |
|  | n20 |  | 847.5 |  | 5 |  | 25 |  | 806.5 |  | N/A |  | N/A |  |
|  | 8 |  | 892.5 |  | 5 |  | 25 |  | 937.5 |  | 25 |  | IMD33 |  |
| DC_8A_n41A |  | 8 |  | 882.5 |  | 5 |  | 25 |  | 927.5 |  | 12.1 |  | IMD33 |
|  |  | n41 |  | 2685 |  | 10 |  | 50 |  | 2685 |  | N/A |  | N/A |
| DC_8A_SUL_n41A- |  | 8 |  | 882.5 |  | 5 |  | 25 |  | 927.5 |  | 12.1 |  | IMD33 |
| n81A |  | n41 |  | 2685 |  | 10 |  | 52 |  | 2685 |  | N/A |  | N/A |
| DC_8A_n77A DC_8A_n78A |  | 8 |  | 897.5 |  | 5 |  | 25 |  | 942.5 |  | 8.3 |  | IMD4 |
| DC_8A-n79C, DC_8A-SUL_n78A-n81A |  | n77, n78 |  | 3635 |  | 10 |  | 50 |  | 3635 |  | N/A |  | H4 |
| DC_12_n78 |  | 12 |  | 710 |  | 5 |  | 25 |  | 740 |  | 5.5 |  | IMD5 |
|  |  | n78 |  | 3580 |  | 10 |  | 50 |  | 3580 |  | N/A |  | N/A |
| DC_13A_n77A |  | 13 |  | 784.5 |  | 5 |  | 20 |  | 753.5 |  | 5.5 |  | IMD5 |
|  |  | n77 |  | 3891.5 |  | 10 |  | 50 |  | 3891.5 |  | N/A |  | N/A |
|  |  | 20 |  | 840 |  | 5 |  | 25 |  | 799 |  | N/A |  | N/A |
| DC_20A_n3A |  | n3 |  | 1775 |  | 5 |  | 25 |  | 1870 |  | 4 |  | IMD4 |
|  |  | 20 |  | 847 |  | 5 |  | 25 |  | 806 |  | 9 |  | IMD4 |
|  |  | n3 |  | 1735 |  | 5 |  | 25 |  | 1830 |  | N/A |  | N/A |
| DC_8A_n79A |  | 8 |  | 897.5 |  | 5 |  | 25 |  | 942.5 |  | 4.8 |  | IMD5 |
| DC_8A-SUL_n79A-n81A |  | n79 |  | 4532.5 |  | 40 |  | 216 |  | 4532.5 |  | N/A |  | N/A |
| DC_18A_n77A DC_18A_n78A |  | 18 |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | IMD4 |
|  |  | n77, n78 |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |
| DC_12A_n77A |  | 12 |  | 702 |  | 5 |  | 20 |  | 732 |  | 5.5 |  | IMD5 |
|  |  | n77 |  | 3540 |  | 10 |  | 50 |  | 3540 |  | N/A |  | N/A |
| DC_19A_n78A |  | 19 |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | IMD4 |
|  |  | n78 |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |
| DC_20_n7 | 20 |  | 851 |  | 5 |  | 25 |  | 810 |  | 12 |  | IMD33 |  |
|  | n7 |  | 2512 |  | 10 |  | 50 |  | 2632 |  | N/A |  | N/A |  |
| DC_20A_n8A |  | 20 |  | 849.5 |  | 5 |  | 25 |  | 808.5 |  | 21 |  | IMD3 |
|  |  | n8 |  | 892.5 |  | 5 |  | 25 |  | 937.5 |  | 21 |  | IMD3 |
| DC_20A_n77A DC_20A_n78A, |  | 20 |  | 850 |  | 5 |  | 25 |  | 809 |  | 11 |  | IMD4 |
| DC_20A-SUL_n78A-n82A |  | n77, n78 |  | 3359 |  | 10 |  | 50 |  | 3359 |  | N/A |  | N/A |
| DC_20A_n77A |  | 20 |  | 840 |  | 5 |  | 25 |  | 799 |  | 6.5 |  | IMD54 |
|  |  | n77 |  | 4159 |  | 10 |  | 50 |  | 4159 |  | N/A |  | N/A |
| DC_21A_n79A |  | 21 |  | 1457.5 |  | 5 |  | 25 |  | 1505.5 |  | 18.4 |  | IMD3 |
|  |  | n79 |  | 4420.5 |  | 40 |  | 216 |  | 4420.5 |  | N/A |  | N/A |
| DC_26A_n41A |  | 26 |  | 839 |  | 5 |  | 25 |  | 884 |  | 15.6 |  | IMD3 |
|  |  | n41 |  | 2562 |  | 10 |  | 50 |  | 2562 |  | N/A |  | N/A |
|  |  | 28 |  | 730 |  | 10 |  | 50 |  | 775 |  | 15.3 |  | IMD 2 |
|  |  | n50 |  | 1500 |  | 10 |  | 50 |  | 1500 |  | N/A |  | N/A |
| DC_28_n50 |  | 28 |  | 740 |  | 10 |  | 50 |  | 785 |  | 6 |  | IMD 4 |
|  |  | n50 |  | 1500 |  | 10 |  | 50 |  | 1500 |  | N/A |  | N/A |
|  |  | 28 |  | 740 |  | 10 |  | 50 |  | 785 |  | 0.5 |  | IMD 5 |
|  |  | n50 |  | 1500 |  | 10 |  | 50 |  | 1500 |  | N/A |  | N/A |
| DC_28A_n51A |  | 28 |  | 742.3 |  | 5 |  | 25 |  | 797.3 |  | 5 |  | IMD 4 |
|  |  | n51 |  | 1429.5 |  | 5 |  | 25 |  | 1429.5 |  | N/A |  | N/A |
| DC_25A_n77A DC_25A-25A_n77A |  | 25 |  | 1855 |  | 5 |  | 25 |  | 1935 |  | 26 |  | IMD2 |
|  |  | n77 |  | 3790 |  | 10 |  | 50 |  | 3790 |  | N/A |  | N/A |
|  |  | 25 |  | 1900 |  | 5 |  | 25 |  | 1980 |  | 8 |  | IMD4 |
|  |  | n77 |  | 3720 |  | 10 |  | 50 |  | 3720 |  | N/A |  | N/A |
|  |  | 25 |  | 1885 |  | 5 |  | 25 |  | 1965 |  | 5 |  | IMD5 |
|  |  | n77 |  | 3810 |  | 10 |  | 50 |  | 3810 |  | N/A |  | N/A |
| DC_26A_n77A, |  | 26 |  | 836.5 |  | 5 |  | 25 |  | 881.5 |  | 11.1 |  | IMD4 |
| DC_26A_n78A |  | n77, n78 |  | 3391 |  | 10 |  | 50 |  | 3391 |  | N/A |  | N/A |
| DC_28A_n77A, |  | 28 |  | 705.5 |  | 5 |  | 25 |  | 760.5 |  | 5.5 |  | IMD5 |
| DC_28A_n78A, DC_28A-SUL_n78A-n83A |  | n77, n78 |  | 3582.5 |  | 10 |  | 50 |  | 3582.5 |  | N/A |  | N/A |
| DC_48A_n66A |  | 48 |  | 3630 |  | 20 |  | 100 |  | 3630 |  | N/A |  | N/A |
|  |  | n66 |  | 1715 |  | 5 |  | 25 |  | 2115 |  | 4 |  | IMD5 |
|  |  | 66 |  | 1775 |  | 5 |  | 25 |  | 2175 |  | N/A |  | N/A |
| DC_66A_n2A |  | n2 |  | 1855 |  | 5 |  | 25 |  | 1935 |  | 20 |  | IMD3 |
|  |  | 66 |  | 1750 |  | 5 |  | 25 |  | 2150 |  | 4 |  | IMD5 |
|  |  | n2 |  | 1883.3 |  | 5 |  | 25 |  | 1963.3 |  | N/A |  | N/A |
| DC_66A_n5A |  | n5 |  | 838 |  | 5 |  | 25 |  | 883 |  | 30 |  | IMD23 |
|  |  | 66 |  | 1721 |  | 5 |  | 25 |  | 2121 |  | N/A |  | N/A |
|  |  | 66 |  | 1775 |  | 5 |  | 25 |  | 2175 |  | N/A |  | N/A |
|  |  | n25 |  | 1855 |  | 5 |  | 25 |  | 1935 |  | 20 |  | IMD3 |
| DC_66A_n25A |  | 66 |  | 1712.5 |  | 5 |  | 25 |  | 2112.5 |  | 23 |  | IMD3 |
|  |  | n25 |  | 1912.5 |  | 5 |  | 25 |  | 1992.5 |  | N/A |  | N/A |
|  |  | 66 |  | 1750 |  | 5 |  | 25 |  | 2150 |  | 4 |  | IMD5 |
|  |  | n25 |  | 1883.3 |  | 5 |  | 25 |  | 1963.3 |  | N/A |  | N/A |
| DC_66A_n71A |  | 66 |  | 1750 |  | 5 |  | 25 |  | 2150 |  | 5 |  | IMD4 |
|  |  | n71 |  | 675 |  | 5 |  | 25 |  | 629 |  | N/A |  | N/A |
| DC_66A_n77A DC_66-66_n77A DC_66-66-66_n77A |  | 66 |  | 1775 |  | 5 |  | 25 |  | 2175 |  | 31.0 |  | IMD2 |
|  |  | n77 |  | 3950 |  | 10 |  | 50 |  | 3950 |  | N/A |  | N/A |
|  |  | 66 |  | 1760 |  | 5 |  | 25 |  | 2160 |  | 5.0 |  | IMD5 |
|  |  | n77 |  | 3720 |  | 10 |  | 50 |  | 3720 |  | N/A |  | N/A |
| DC_66A_n78A |  | 66 |  | 1730 |  | 5 |  | 25 |  | 2130 |  | 5.0 |  | IMD5 |
|  |  | n78 |  | 3660 |  | 5 |  | 50 |  | 3660 |  | N/A |  | N/A |
| NOTE 1: E-UTRA carrier shall be set to min(+20 dBm, PCMAX_L_E-UTRA,c) and NR carrier shall be set to min(+20 dBm, PCMAX_L,f,c,NR) as defined in clause 6.2B.4.1.3. NOTE 2: RBSTART = 0 NOTE 3: This band is subject to IMD5 also which MSD is not specified. NOTE 4: Applicable only if operation with 4 antenna ports is supported in the band with EN-DC configured. NOTE 5: Void. NOTE 6: For NR band, UL/DL BW and UL LCRB can be adjusted according to the supported BW and lowest SCS supported by the UE. NOTE 7: The frequency range in band n28 is restricted for this band combination to 728 - 738 MHz for the UL and 783 - 793 MHz for the DL. This band is subject to IMD2, IMD4 and IMD5 fall in n28 also which MSD is not specified. In addition, this band is subject to IMD4 fall in B21 also which MSD is not specified. NOTE 8: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 from TS 38.101-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

7.3B.2.3.4.2.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths based on EN-DC operating bands specified in clause 5.5B, channel bandwidths and sub-carrier spacings for the NR cell specified in TS 38.521-1 [8] clause 5.3 and channel bandwidth for the E-UTRA cell are specified in TS 36.521-1 [10] clause 5.4.2. All of these configurations shall be tested with applicable test parameters for each EN-DC configuration, and are shown in Table 7.3B.2.3.4.2.1-0 to Table 7.3B.2.3.4.2.1-6.

The details of the uplink and downlink reference measurement channels (RMCs) are specified in TS 36.521-1 [10] Annex A.2 and A.3 for E-UTRA RMC, and in TS 38.521-1 [8] Annex A.2 and A.3 for NR RMC respectively. The details of the OCNG patterns used are specified in TS 36.521-1 [10] Annex A.5 and in TS 38.521-1 [8] Annex A.5 for E-UTRA CG and NR CG, respectively. Configurations of PDSCH and PDCCH before measurement are specified in TS 36.521-1 [10] Annex C.2 and in TS 38.521-1 [8] Annex C.2 for E-UTRA CG and NR CG, respectively.

For configurations without any reference sensitivity exception, in a FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected. For configurations with reference sensitivity exception, in an E-UTRA band or FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected.

< Unchanged tables omitted >

Table 7.3B.2.3.4.2.1-6: Test Configuration Table for EN-DC configurations affected by Reference sensitivity exceptions (two bands)

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |
| NR Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1, E-UTRA Test Frequencies as specified in TS 36.508 [11] clause 4.3.1 |  |  |  |  |  | For test frequencies refer to “Range” columns. |  |  |  |
| Test EN-DC channel bandwidth as specified in TS 36.508 [11] clause 4.3.1 and TS 38.508-1 [6] clause 4.3.1. |  |  |  |  |  | Refer to "NR NRB"and "E-UTRA NRB " columns |  |  |  |
| NR Test SCS as specified in Table 5.3.5-1 in TS 38.521-1 [8] |  |  |  |  |  | Lowest supported SCS |  |  |  |
| Network signalling value |  |  |  |  |  | NS_01 by default Unless given by Table 7.3.3-3 in TS 36.521-1 [10] for the E-UTRA band and Table 7.3.2.3-4 in TS 38.521-1 [8] for the NR band. |  |  |  |
| Test Parameters for DC Configurations |  |  |  |  |  |  |  |  |  |
| ID | PCC – E-UTRA |  |  |  | SCG -NR |  |  |  |  |
|  | Band | Range | NRB |  | Band |  | Range | NRB |  |
|  | UL MOD | DL MOD | CH BW | DLalloc / UL alloc | UL MOD |  | DL MOD | UL/DL Ch BW | DLalloc / UL alloc |
| Test Settings for a DC_1A_n3A Configuration |  |  |  |  |  |  |  |  |  |
| 12,11 | 1 | Mid |  |  | n3 |  | Mid |  |  |
|  | QPSK | QPSK | 20 MHz | All RBs / REFSENS_ENDC_3 | N/A |  | CP-OFDM QPSK | 40 MHz | All RBs / 0 |
| 24,10 | 1 | UL 1950 / DL 2140 |  |  | n3 |  | UL 1760 / DL 1855 |  |  |
|  | QPSK | QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 |
| 36 | 1 | Low |  |  | n3 |  | High |  |  |
|  | QPSK | QPSK | 20 MHz | All RBs / REFSENS_ENDC_3 | N/A |  | CP-OFDM QPSK | 40 MHz | All RBs / 0 |
| Test Settings for a DC_1A_n78A Configuration |  |  |  |  |  |  |  |  |  |
| 14 | 1 | UL 1950 / DL 2140 |  |  | n78 |  | 3710 |  |  |
|  | QPSK | QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 10 MHz | All RBs / REFSENS_ENDC_4 |
| < Unchanged rows omitted > |  |  |  |  |  |  |  |  |  |  |
| Test Settings for a DC_2A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |
| 13 | 2 | UL 1870 MHZ |  |  | n78 |  | 3740 MHZ |  |  |  |
|  | QPSK | QPSK | 20 MHz | All RBs / REFSENS_ENDC_1 | N/A |  | CP-OFDM QPSK | 100 MHz | All RBs / 0 |  |
| 23 | 2 | UL 1885 MHz |  |  | n78 |  | 3740 MHz |  |  |  |
|  | QPSK | QPSK | 20 MHz | All RBs / REFSENS_ENDC_1 | N/A |  | CP-OFDM QPSK | 20 MHz | All RBs / 0 |  |
| 34 | 2 | UL 1855 MHz/ DL 1935 MHz |  |  | n78 |  | UL/DL 3790 MHz |  |  |  |
|  | QPSK | QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 10 MHz | All RBs / REFSENS_ENDC_4 |  |
| 44 | 2 | UL 1885 MHz/ DL 1965 MHz |  |  | n78 |  | UL/DL 3690 MHz |  |  |  |
|  | QPSK | QPSK | 5 MHz | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 10 MHz | All RBs / REFSENS_ENDC_4 |  |
| < Unchanged rows omitted > |  |  |  |  |  |  |  |  |  |
| NOTE 1: REFSENS_LTE refers to the single carrier Uplink RB allocation for reference sensitivity according to table 7.3.3-2 of TS 36.521-1 [10]. REFSENS_NR refers to the single carrier Uplink RB allocation for reference sensitivity according to table 7.3.2.4.1-3 of TS 38.521-1 [8]. REFSENS_ENDC_1 refers to the Uplink RB allocation for reference sensitivity exceptions due to UL harmonic interference according to table 7.3B.2.0.3.1-2. REFSENS_ENDC_2 refers to the Uplink RB allocation for reference sensitivity exceptions due to receiver harmonic mixing according to table 7.3B.2.0.3.2-2. REFSENS_ENDC_3 refers to the Uplink RB allocation for reference sensitivity exceptions due to cross band isolation according to table 7.3B.2.0.3.4-2. REFSENS_ENDC_4 refers to the Uplink RB allocation for reference sensitivity exceptions due to dual uplink operation for ENDC according to table 7.3B.2.0.3.5.1-1 for PC3 and table 7.3B.2.0.3.5.1-1a for PC2. NOTE 2: Void NOTE 3: Test ID with UL harmonic exception NOTE 4: Test ID with 2UL intermodulation exception NOTE 5: Test ID with UL receiver harmonic mixing NOTE 6: Test ID with UL cross band isolation NOTE 7: Void NOTE 8: Test ID with UL harmonic exception avoided NOTE 9: Test ID with UL receiving harmonic mixing exception avoided NOTE 10: Only applicable to UEs not supporting UE capability singleUL-Transmission. NOTE 11: Test ID with Cross band isolation exception avoided, which is only applicable to DC_1_n3, DC_3_n1 and DC_3_n84. NOTE 12: In an E-UTRA band or FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected. NOTE 13: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 from TS 38.101-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

7.3B.2.3.5	Test requirement

For inter-band EN-DC configurations affected by reference sensitivity exceptions, when test points without NOTE 8, 9 and 11 in Table 7.3B.2.3.4.2.1-6 are tested, the throughput of each CG shall be ≥ 95% of the maximum throughput for the reference receive power level specified in Table 7.3B.2.3.5-1, Table 7.3B.2.3.5-2, Table 7.3B.2.3.5-3, Table 7.3B.2.3.5-3a, Table 7.3B.2.3.5-4 and Table 7.3B.2.3.5-5 for MSDs due to uplink harmonic, harmonic mixing, cross band isolation for PC3 EN-DC, cross band isolation for PC2 EN-DC, dual uplinks for PC3 EN-DC and dual uplink operation for PC2 EN-DC, respectively. For a given EN-DC combo, if more than one category of MSD applies, UE shall pass all requirement.

For test points with NOTE 8, 9 or 11 in Table 7.3B.2.3.4.2.1-6, reference sensitivity test requirements are specified in Table 7.3.2.5-1 in TS 38.521-1 [8] for the NR CC, and Table 7.3.5-1 in TS 36.521-1 [10] for E-UTRA CC.

Reference sensitivity test requirements for test points in Table 7.3B.2.3.4.2.1-0 are specified in Table 7.3.2.5-1 in TS 38.521-1 [8].

For the UE which supports inter-band EN-DC, the minimum requirement for reference sensitivity in Table 7.3.2.5-1 of TS 38.521-1 [8] for NR band and Table 7.3.5-1 of TS 36.521-1 [10] for EUTRA band, shall be increased by the amount given in ΔRIB,c defined in clause 7.3B.3.3 for the applicable two, three, four and five bands operation.

< Unchanged tables omitted >

Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1, are specified in Table 7.3B.2.3.5-4 with uplink configuration specified in Table 7.3B.2.3.4.2.1-6.

Table 7.3B.2.3.5-4: Reference sensitivity exceptions due to dual uplink operation for PC3 EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 40 MHz (dBm) | IMD order | Duplex mode |
| DC_1A_n3A | 1 | N/A | [-76.3] | - | - | - | - | IMD3 | FDD |
|  | n3 | 15 | REFSENS |  | - | - | - | N/A | TDD |
| DC_1A_n77A | 1 | N/A | -69.5 | - | - | - | - | IMD23 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| < Unchanged rows omitted > |  |  |  |  |  |  |  |  |  |
| NOTE 1: E-UTRA carrier shall be set to min(+20 dBm, PCMAX_L_E-UTRA,c) and NR carrier shall be set to min(+20 dBm, PCMAX_L,f,c,NR) as defined in clause 6.2B.4.1.3. NOTE 2: RBSTART = 0 NOTE 3: This band is subject to IMD5 also which MSD is not specified. NOTE 4: The symbol "REFSENS" in this table refers to the reference sensitivity values for single carrier specified in Table 7.3.5-1 of TS 36.521-1 [10] for 2 antenna port E-UTRA band, Table 7.3_1.5-1 of TS 36.521-1 [10] for 4 antenna port E-UTRA band, Table 7.3.2.5-1 for 2 antenna port NR band and Table 7.3.2.5-2 for 4 antenna port NR band. NOTE 5: Void NOTE 6: TT is the same as defined in Table 7.3B.2.3.5-1a. NOTE 7: Applicable only if operation with 4 antenna ports is supported in the band with EN-DC configured. NOTE 8: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 from TS 38.101-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |  |  |

< End of changes >