3GPP TSG-RAN5 Meeting #95-e	R5-223697
Electronic Meeting, 9th May – 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 1666 | rev | 1 | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections in message exceptions and test points for FR1 test case 6.3A.4.1.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_FR1-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This test case reuses the test requirements from the single carrier absolute power control test case 6.3.4.2. However, that is not possible if value set for ss-PBCH-BlockPower IE is quite different because that affects to the UE path losses estimation. The power control is also affected by the RB allocation. In currect test definition, Inner_Full allocation is set for the case of intra-band contiguous. However, as per definition made in table 6.1A-1a, RB allocation varies a lot depending on the specific aggregated BW configuration what means for every single case the expected output power would be different, making the test definition complex, hard to maintain and no way to reuse test requirements from single CC test as it is currently doing. Proposed to use Outer_Full allocation instead. Test point analysis updated accordingly. To ensure MPR does not affect, p0-NominalWithGrant should be lowered by 8dB in Test Point 2 for intra-band contiguous. Despite smaller reduction is needed for intra-band non-contiguous, the same reduction is proposed in both cases in order to facilitate test definition and maintenance. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Value set for ss-PBCH-BlockPower IE as defined in test 6.3.4.2. Set Outer_Full allocation also for intra-band contiguous. Set p0-NominalWithGrant = -108dBm (8dB lower) for Test Point 2. Test requirements for Test Point 2 updated accordingly. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test specification will remain inconsistent. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3A.4.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TR 38.905 CR 0603 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision 1: -Modified p0-NominalWithGrant for Test Point 2 to ensure MPR will not affect. |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES >>>

##### 6.3A.4.1.1	Absolute power tolerance for CA (2UL CA)

Editor’s Note:

This test case in incomplete when signalling is absent for dualPA-Architecture IE due to lack of core requirements.

6.3A.4.1.1.1	Test purpose

To verify the ability of the UE transmitter for 2UL CA to set its initial output power to a specific value at the start of a contiguous transmission or non-contiguous transmission on each active component carrier with a long transmission gap, i.e. transmission gap is larger than 20ms.

6.3A.4.1.1.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that support intra-band contiguous and non-contiguous 2UL CA.

6.3A.4.1.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.3A.4.1.0.

6.3A.4.1.1.4	Test description

6.3A.4.1.1.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR CA configurations specified in 5.5A. All of these configurations shall be tested with applicable test parameters for each CA configuration, and are shown in table 6.3A.4.1.1.4.1-1 for intra-band contiguous CA and table 6.3A.4.1.1.4.1-2 for intra-band non-contiguous CA. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2 and A.3. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.3A.4.1.1.4.1-1: Test Configuration Table for intra-band contiguous CA

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Mid range |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Lowest NRB_agg, Highest NRB_agg |  |  |
| Test SCS as specified in Table 5.3.5-1 |  | Lowest, Highest |  |  |
| Test Parameters |  |  |  |  |
| Test ID | Downlink Configuration for PCC & SCC | Uplink Configuration |  |  |
|  |  | Modulation for all CCs | RB allocation (NOTE 1) |  |
|  |  |  | PCC | SCC |
| 1 | N/A for this test | CP-OFDM QPSK | Outer_Full | Outer_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in 6.1A-1a for contiguous RB allocation. NOTE 2: Test Channel Bandwidths and Test SCS are checked separately for each NR CA band combination, which applicable channel bandwidths and SCS are specified in Table 5.5A.1-1. NOTE 3: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg, only the combination with the highest NRB_PCC is tested. |  |  |  |  |

Table 6.3A.4.1.1.4.1-2: Test Configuration Table for intra-band non-contiguous CA

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  | Normal |  |  |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  | For test frequencies refer to “Range” columns |  |  |  |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  | Refer to “PCC NRB”and “SCC NRB ” columns |  |  |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | Lowest, Highest |  |  |  |  |  |  |
| Test Parameters |  |  |  |  |  |  |  |  |  |  |  |
| ID | CA config / CBW |  |  |  |  |  |  | DL config | UL config |  |  |
|  | PCC |  | SCC |  |  |  |  |  | CC MOD | RB allocation (NOTE 1) |  |
|  | Band | Range | Band | Range | PCC NRB | Wgap | SCC NRB |  |  | PCC | SCC |
| 2 | nX | CC1 | nX | CC2 | Highest NRB | Max (NOTE 4) | Highest NRB | N/A | CP-OFDM QPSK | Outer_Full | Outer_Full |
| NOTE 1: The RB allocation is defined in table 6.1-1 for each CC. NOTE 2: Test Channel Bandwidths and Test SCS are checked separately for each NR CA band combination, which applicable channel bandwidths and SCS are specified in Table 5.5A.2-1. NOTE 3: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg, only the combination with the highest NRB_PCC is tested. NOTE 4: The Wgap is defined to be widest possible on band based on the PCC and SCC configuration |  |  |  |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.3 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement Channel is set according to Table 6.3A.1.1.4.1-1 and Table 6.3A.1.1.4.1-2 as appropriate.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.3A.4.1.1.4.3. Note that PDCCH DCI format 0_1 sent after resetting uplink power with RRC Connection Reconfiguration, should have TPC command 0dB.

6.3A.4.1.1.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1, C.2 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 38.508-1 [5] clause5.5.1. Message contents are defined in clause 6.3A.4.1.1.4.3. Any PDCCH DCI format 0_1 sent to the UE during the configuration should have TPC command 0dB.

3.	SS activates SCC by sending the activation MAC CE (Refer TS 38.321 [18], clauses 5.9, 6.1.3.10). Wait for at least 2 seconds (Refer TS 38.133 [19], clause 9.3).

4.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 with TPC command 0dB for C_RNTI to schedule the UL RMC according to Table 6.3A.4.1.1.4.1-1 on both PCC and SCC. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

5.	Measure the initial output power of the first subframe of UE PUSCH first transmission for each component carrier.

6.	Repeat for the two test points as indicated in section 6.3A.4.1.1.4.3. The timing of the execution between the two test points shall be larger than 20ms.

6.3A.4.1.1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 with the following exceptions:

Table 6.3A.4.1.1.4.3-0: ServingCellConfigCommon

| Derivation Path: 38.508-1[5], Table 4.6.3-168 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommon ::= SEQUENCE { |  |  |  |
| ss-PBCH-BlockPower | 18 |  | SCS_15kHz |
|  | 21 |  | SCS_30kHz |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SCS_15kHz | SCS=15kHz for SS/PBCH block |
| SCS_30kHz | SCS=30kHz for SS/PBCH block |

Table 6.3A.4.1.1.4.3-1: UplinkPowerControlCommon: Test point 1

| Derivation Path: TS 38.508-1 [5] subclause 4.6.3 Table 4.6.3-119 PUSCH-ConfigCommon |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-ConfigCommon::= SEQUENCE { |  |  |  |
| p0-NominalWithGrant | -114 | Test point 1 to verify a UE relative low initial power transmission |  |
| } |  |  |  |

Table 6.3A.4.1.1.4.3-2: UplinkPowerControlCommon: Test point 2

| Derivation Path: TS 38.508-1 [5] subclause 4.6.3 Table 4.6.3-119 PUSCH-ConfigCommon |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-ConfigCommon::= SEQUENCE { |  |  |  |
| p0-NominalWithGrant | -108 | Test point 2 to verify a UE relative high initial power transmission |  |
| } |  |  |  |

6.3A.4.1.1.5	Test requirement

For intra-band contiguous CA, the absolute power control tolerance per component carrier measured in step (5) of the test procedure is not to exceed the values specified in Table 6.3A.4.1.1.5-1 and 6.3A.4.1.1.5-2.

Table 6.3A.4.1.1.5-1: Absolute power tolerance: test point 1

|  |  | Channel bandwidth / expected output power (dBm) |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| Expected Measured power | SCS15 | -17.6 | -14.4 | -12.6 | -11.3 | -10.4 | -9.6 | -8.3 | -7.3 | N/A | N/A | N/A | N/A | N/A |
|  | SCS30 | -18.2 | -14.8 | -12.8 | -11.5 | -10.5 | -9.7 | -8.3 | -7.4 | -6.5 | -5.8 | -5.2 | -4.7 | -4.2 |
|  | SCS60 |  | -15.2 | -13.0 | -11.8 | -10.7 | -9.8 | -8.5 | -7.5 | -6.6 | -5.9 | -5.3 | -4.8 | -4.3 |
| Power tolerance |  | ± (9+TT)dB |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: TT for each duplex, Sub-Carrier Spacing, frequency and channel bandwidth is specified in Table 6.3A.4.1.1.5-3. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.3A.4.1.1.5-2: Absolute power tolerance: test point 2

|  |  | Channel bandwidth / expected output power (dBm) |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| Expected Measured power | SCS15 | -11.6 | -7.6 | -6.6 | -5.3 | -4.4 | -3.6 | -2.3 | -1.3 | N/A | N/A | N/A | N/A | N/A |
|  | SCS30 | -12.2 | -8.8 | -6.8 | -5.5 | -4.5 | -3.7 | -2.3 | -1.4 | -0.5 | 0.2 | 0.8 | 1.3 | 1.8 |
|  | SCS60 | N/A | -9.2 | -7 | -5.8 | -4.7 | -3.8 | -2.5 | -1.5 | -0.6 | 0.1 | 0.7 | 1.2 | 1.7 |
| Power tolerance |  | ± (9+TT)dB |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: TT for each duplex, Sub-Carrier Spacing, frequency and channel bandwidth is specified in Table 6.3A.4.1.1.5-3. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.3A.4.1.1.5-3: Test Tolerance

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| --- | --- | --- | --- |
| BW ≤ 40MHz | 1.0 dB | 1.4 dB | 1.4 dB |
| 40MHz < BW ≤ 100MHz | 1.4 dB | 1.4 dB | 1.4 dB |

## <<< END OF CHANGES >>>