3GPP TSG-RAN5 Meeting #95-e	R5-222362

# Electronic Meeting, 9th May– 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 2906 | rev | - | Current version: | 16.11.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to DRX adaptation test case 7.1.1.12.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_UE_pow_sav-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There are 2 IEs missing in the Table 7.1.1.12.3.3.3-4. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added missing searchSpaceGroupIdList-r16 and freqMonitorLocations-r16 to the Table 7.1.1.12.3.3.3-4. Minor editorial updates. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Message definition will remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.12.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 7.1.1.12.3	DRX adaptation / UE wakeup indication

7.1.1.12.3.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state and long DRX is configured and [(SFN * 10) + subframe number] modulo (drx-LongCycle) = drx-StartOffset and DCP is configured }

ensure that {

when { a DCP indication with the value of wake-up indication 1 associated with the current DRX cycle has been received }

then { UE starts the drx-onDurationTimer after drx-SlotOffset from the beginning of the subframe and monitors the PDCCH }

}

(2)

with { UE in RRC_CONNECTED state and long DRX is configured and [(SFN * 10) + subframe number] modulo (drx-LongCycle) = drx-StartOffset and DCP is configured and ps-wakeup is configured with value true }

ensure that {

when { DCP indication associated with this cycle has not been received }

then { UE starts the drx-onDurationTimer after drx-SlotOffset from the beginning of the subframe and monitors the PDCCH for OnDurationTimer PDCCH-Occasions }

}

(3)

with { UE in RRC_CONNECTED state long DRX is configured and [(SFN * 10) + subframe number] modulo (drx-LongCycle) = drx-StartOffset and DCP is configured }

ensure that {

when { all DCP occasions in time domain occurred in DRX active time }

then { UE starts the drx-onDurationTimer after drx-SlotOffset from the beginning of the subframe and monitors the PDCCH }

}

(4)

with { UE in RRC_CONNECTED state long DRX is configured and DCP is configured }

ensure that {

when { all DCP occasions in time domain occurred during measurement gap }

then { UE starts the drx-onDurationTimer after drx-SlotOffset from the beginning of the subframe and monitors the PDCCH }

}

(5)

with { UE in RRC_CONNECTED state and long DRX is configured and [(SFN * 10) + subframe number] modulo (drx-LongCycle) = drx-StartOffset and DCP is configured }

ensure that {

when { a DCP indication with the value of wake-up indication 0 associated with the current DRX cycle has been received }

then { UE does not start the drx-onDurationTimer after drx-SlotOffset from the beginning of the subframe and skips monitoring the PDCCH }

}

7.1.1.12.3.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.321, clause 5.7, TS 38.213, clause 10.3 and 7.3.1.3.7. Unless otherwise stated these are Rel-16 requirements.

[TS 38.321, clause 5.7]

The MAC entity may be configured by RRC with a DRX functionality that controls the UE's PDCCH monitoring activity for the MAC entity's C-RNTI, CI-RNTI, CS-RNTI, INT-RNTI, SFI-RNTI, SP-CSI-RNTI, TPC-PUCCH-RNTI, TPC-PUSCH-RNTI, TPC-SRS-RNTI, and AI-RNTI. When using DRX operation, the MAC entity shall also monitor PDCCH according to requirements found in other clauses of this specification. When in RRC_CONNECTED, if DRX is configured, for all the activated Serving Cells, the MAC entity may monitor the PDCCH discontinuously using the DRX operation specified in this clause; otherwise the MAC entity shall monitor the PDCCH as specified in TS 38.213 [6].

NOTE 1:	If Sidelink resource allocation mode 1 is configured by RRC, a DRX functionality is not configured.

RRC controls DRX operation by configuring the following parameters:

-	drx-onDurationTimer: the duration at the beginning of a DRX Cycle;

-	drx-SlotOffset: the delay before starting the drx-onDurationTimer;

-	drx-InactivityTimer: the duration after the PDCCH occasion in which a PDCCH indicates a new UL or DL transmission for the MAC entity;

-	drx-RetransmissionTimerDL (per DL HARQ process except for the broadcast process): the maximum duration until a DL retransmission is received;

-	drx-RetransmissionTimerUL (per UL HARQ process): the maximum duration until a grant for UL retransmission is received;

-	drx-LongCycleStartOffset: the Long DRX cycle and drx-StartOffset which defines the subframe where the Long and Short DRX Cycle starts;

-	drx-ShortCycle (optional): the Short DRX cycle;

-	drx-ShortCycleTimer (optional): the duration the UE shall follow the Short DRX cycle;

-	drx-HARQ-RTT-TimerDL (per DL HARQ process except for the broadcast process): the minimum duration before a DL assignment for HARQ retransmission is expected by the MAC entity;

-	drx-HARQ-RTT-TimerUL (per UL HARQ process): the minimum duration before a UL HARQ retransmission grant is expected by the MAC entity;

-	ps-Wakeup (optional): the configuration to start associated drx-onDurationTimer in case DCP is monitored but not detected;

-	ps-TransmitOtherPeriodicCSI (optional): the configuration to report periodic CSI that is not L1-RSRP on PUCCH during the time duration indicated by drx-onDurationTimer in case DCP is configured but associated drx-onDurationTimer is not started;

-	ps-TransmitPeriodicL1-RSRP (optional): the configuration to transmit periodic CSI that is L1-RSRP on PUCCH during the time duration indicated by drx-onDurationTimer in case DCP is configured but associated drx-onDurationTimer is not started.

Serving Cells may be configured by RRC in two groups. When RRC does not configure a secondary DRX group, there is only one DRX group. When two DRX groups are configured each group of Serving Cells, which is called a DRX group, is configured by RRC with its own set of parameters: drx-onDurationTimer, drx-InactivityTimer. When two DRX groups are configured, the two groups share the following parameter values: drx-SlotOffset, drx-RetransmissionTimerDL, drx-RetransmissionTimerUL, drx-LongCycleStartOffset, drx-ShortCycle (optional), drx-ShortCycleTimer (optional), drx-HARQ-RTT-TimerDL, and drx-HARQ-RTT-TimerUL.

When a DRX cycle is configured, the Active Time for Serving Cells in a DRX group includes the time while:

-	drx-onDurationTimer or drx-InactivityTimer configured for the DRX group is running; or

-	drx-RetransmissionTimerDL or drx-RetransmissionTimerUL is running on any Serving Cell in the DRX group; or

-	ra-ContentionResolutionTimer (as described in clause 5.1.5) or msgB-ResponseWindow (as described in clause 5.1.4a) is running; or

-	a Scheduling Request is sent on PUCCH and is pending (as described in clause 5.4.4); or

-	a PDCCH indicating a new transmission addressed to the C-RNTI of the MAC entity has not been received after successful reception of a Random Access Response for the Random Access Preamble not selected by the MAC entity among the contention-based Random Access Preamble (as described in clauses 5.1.4 and 5.1.4a).

When DRX is configured, the MAC entity shall:

1>	if the Long DRX Cycle is used, and [(SFN × 10) + subframe number] modulo (drx-LongCycle) = drx-StartOffset:

2>	if DCP monitoring is configured for the active DL BWP as specified in TS 38.213 [6], clause 10.3:

3>	if DCP indication associated with the current DRX Cycle received from lower layer indicated to start drx-onDurationTimer, as specified in TS 38.213 [6]; or

3>	if all DCP occasion(s) in time domain, as specified in TS 38.213 [6], associated with the current DRX Cycle occurred in Active Time considering grants/assignments/DRX Command MAC CE/Long DRX Command MAC CE received and Scheduling Request sent until 4 ms prior to start of the last DCP occasion, or within BWP switching interruption length, or during a measurement gap; or

3>	if ps-Wakeup is configured with value true and DCP indication associated with the current DRX Cycle has not been received from lower layers:

4>	start drx-onDurationTimer after drx-SlotOffset from the beginning of the subframe.

2>	else:

3>	start drx-onDurationTimer after drx-SlotOffset from the beginning of the subframe.

NOTE 2:	In case of unaligned SFN across carriers in a cell group, the SFN of the SpCell is used to calculate the DRX duration.

1>	if the DRX group is in Active Time:

2>	monitor the PDCCH on the Serving Cells in this DRX group as specified in TS 38.213 [6];

2>	if the PDCCH indicates a DL transmission:

3>	start the drx-HARQ-RTT-TimerDL for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback;

NOTE 3:	When HARQ feedback is postponed by PDSCH-to-HARQ_feedback timing indicating a non-numerical k1 value, as specified in TS 38.213 [6], the corresponding transmission opportunity to send the DL HARQ feedback is indicated in a later PDCCH requesting the HARQ-ACK feedback.

3>	stop the drx-RetransmissionTimerDL for the corresponding HARQ process.

3>	if the PDSCH-to-HARQ_feedback timing indicate a non-numerical k1 value as specified in TS 38.213 [6]:

4>	start the drx-RetransmissionTimerDL in the first symbol after the PDSCH transmission for the corresponding HARQ process.

2>	if the PDCCH indicates a UL transmission:

3>	start the drx-HARQ-RTT-TimerUL for the corresponding HARQ process in the first symbol after the end of the first repetition of the corresponding PUSCH transmission;

3>	stop the drx-RetransmissionTimerUL for the corresponding HARQ process.

2>	if the PDCCH indicates a new transmission (DL or UL) on a Serving Cell in this DRX group:

3>	start or restart drx-InactivityTimer for this DRX group in the first symbol after the end of the PDCCH reception.

1>	if DCP monitoring is configured for the active DL BWP as specified in TS 38.213 [6], clause 10.3; and

1>	if the current symbol n occurs within drx-onDurationTimer duration; and

1>	if drx-onDurationTimer associated with the current DRX cycle is not started as specified in this clause:

2>	if the MAC entity would not be in Active Time considering grants/assignments/DRX Command MAC CE/Long DRX Command MAC CE received and Scheduling Request sent until 4 ms prior to symbol n when evaluating all DRX Active Time conditions as specified in this clause:

3>	not transmit periodic SRS and semi-persistent SRS defined in TS 38.214 [7];

3>	not report semi-persistent CSI configured on PUSCH;

3>	if ps-TransmitPeriodicL1-RSRP is not configured with value true:

4>	not report periodic CSI that is L1-RSRP on PUCCH.

3>	if ps-TransmitOtherPeriodicCSI is not configured with value true:

4>	not report periodic CSI that is not L1-RSRP on PUCCH.

1>	else:

2>	in current symbol n, if the DRX group would not be in Active Time considering grants/assignments scheduled on Serving Cell(s) in this DRX Group and DRX Command MAC CE/Long DRX Command MAC CE received and Scheduling Request sent until 4 ms prior to symbol n when evaluating all DRX Active Time conditions as specified in this clause:

3>	not transmit periodic SRS and semi-persistent SRS defined in TS 38.214 [7] in this DRX group;

3>	not report CSI on PUCCH and semi-persistent CSI configured on PUSCH in this DRX group.

…

Regardless of whether the MAC entity is monitoring PDCCH or not on the Serving Cells in this DRX group, the MAC entity transmits HARQ feedback, aperiodic CSI on PUSCH, and aperiodic SRS defined in TS 38.214 [7] on the Serving Cells in this DRX group when such is expected.

The MAC entity needs not to monitor the PDCCH if it is not a complete PDCCH occasion (e.g. the Active Time starts or ends in the middle of a PDCCH occasion).

[TS 38.213, clause 10.3]

A UE configured with DRX mode operation [11, TS 38.321] can be provided the following for detection of a DCI format 2_6 in a PDCCH reception on the PCell or on the SpCell [12, TS 38.331]

-	a PS-RNTI for DCI format 2_6 by ps-RNTI

-	a number of search space sets, by dci-Format2-6, to monitor PDCCH for detection of DCI format 2_6 on the active DL BWP of the PCell or of the SpCell according to a common search space as described in clause 10.1

-	a payload size for DCI format 2_6 by sizeDCI-2-6

-	a location in DCI format 2_6 of a Wake-up indication bit by ps-PositionDCI-2-6

-	a '0' value for the Wake-up indication bit, when reported to higher layers, indicates to not start the drx-onDurationTimer for the next long DRX cycle [11, TS 38.321]

-	a '1' value for the Wake-up indication bit, when reported to higher layers, indicates to start the drx-onDurationTimer for the next long DRX cycle [11, TS 38.321]

-	a bitmap, when the UE is provided a number of groups of configured SCells by dormancyGroupOutsideActiveTime, where

-	the bitmap location is immediately after the Wake-up indication bit location

-	the bitmap size is equal to the number of groups of configured SCells where each bit of the bitmap corresponds to a group of configured SCells from the number of groups of configured SCells

-	a '0' value for a bit of the bitmap indicates an active DL BWP, provided by dormantBWP-Id, for the UE [11, TS38.321] for each activated SCell in the corresponding group of configured SCells

-	a '1' value for a bit of the bitmap indicates

-	an active DL BWP, provided by firstOutsideActiveTimeBWP-Id, for the UE for each activated SCell in the corresponding group of configured SCells, if a current active DL BWP is the dormant DL BWP

-	a current active DL BWP, for the UE for each activated SCell in the corresponding group of configured SCells, if the current active DL BWP is not the dormant DL BWP

-	the UE sets the active DL BWP to the indicated active DL BWP

-	an offset by ps-Offset indicating a time, where the UE starts monitoring PDCCH for detection of DCI format 2_6 according to the number of search space sets, prior to a slot where the drx-onDurationTimer would start on the PCell or on the SpCell [11, TS 38.321]

-	for each search space set, the PDCCH monitoring occasions are the ones in the first Ts slots indicated by duration, or Ts=1 slot if duration is not provided, starting from the first slot of the first Ts slots and ending prior to the start of drx-onDurationTimer.

On PDCCH monitoring occasions associated with a same long DRX Cycle, a UE does not expect to detect more than one DCI format 2_6 with different values of the Wake-up indication bit for the UE or with different values of the bitmap for the UE.

The UE does not monitor PDCCH for detecting DCI format 2_6 during Active Time [11, TS 38.321].

If a UE reports for an active DL BWP a MinTimeGap value that is X slots prior to the beginning of a slot where the UE would start the drx-onDurationTimer, the UE is not required to monitor PDCCH for detection of DCI format 2_6 during the X slots, where X corresponds to the MinTimeGap value of the SCS of the active DL BWP in Table 10.3-1.

Table 10.3-1: Minimum time gap value X

| SCS (kHz) | Minimum Time Gap X (slots) |  |
| --- | --- | --- |
|  | Value 1 | Value 2 |
| 15 | 1 | 3 |
| 30 | 1 | 6 |
| 60 | 1 | 12 |
| 120 | 2 | 24 |
| 480 | 8 | 96 |
| 960 | 16 | 192 |

If a UE is provided search space sets to monitor PDCCH for detection of DCI format 2_6 in the active DL BWP of the PCell or of the SpCell and the UE detects DCI format 2_6, the physical layer of a UE reports the value of the Wake-up indication bit for the UE to higher layers [11, TS 38.321] for the next long DRX cycle.

If a UE is provided search space sets to monitor PDCCH for detection of DCI format 2_6 in the active DL BWP of the PCell or of the SpCell and the UE does not detect DCI format 2_6, the physical layer of the UE does not report a value of the Wake-up indication bit to higher layers for the next long DRX cycle.

If a UE is provided search space sets to monitor PDCCH for detection of DCI format 2_6 in the active DL BWP of the PCell or of the SpCell and the UE

-	is not required to monitor PDCCH for detection of DCI format 2_6, as described in clauses 10, 11.1, 12, and in clause 5.7 of [11, TS 38.321] for all corresponding PDCCH monitoring occasions outside Active Time prior to a next long DRX cycle, or

-	does not have any PDCCH monitoring occasions for detection of DCI format 2_6 outside Active Time of a next long DRX cycle

the physical layer of the UE reports a value of 1 for the Wake-up indication bit to higher layers for the next long DRX cycle.

[TS 38.212, clause 7.3.1.3.7]

DCI format 2_6 is used for notifying the power saving information outside DRX Active Time for one or more UEs.

The following information is transmitted by means of the DCI format 2_6 with CRC scrambled by PS-RNTI:

-	block number 1, block number 2,…, block number N

where the starting position of a block is determined by the parameter PSPositionDCI2-6 provided by higher layers for the UE configured with the block.

If the UE is configured with higher layer parameter PS-RNTI and dci-Format2-6, one block is configured for the UE by higher layers, with the following fields defined for the block:

-	Wake-up indication - 1 bit

-	SCell dormancy indication – 0 bit if higher layer parameter Scell-groups-for-dormancy-outside-active-time is not configured; otherwise 1, 2, 3, 4 or 5 bits bitmap determined according to higher layer parameter Scell-groups-for-dormancy-outside-active-time, where each bit corresponds to one of the SCell group(s) configured by higher layers parameter Scell-groups-for-dormancy-outside-active-time, with MSB to LSB of the bitmap corresponding to the first to last configured SCell group.

The size of DCI format 2_6 is indicated by the higher layer parameter SizeDCI_2-6, according to Clause 10.3 of [5, TS 38.213].

7.1.1.12.3.3	Test description

7.1.1.12.3.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.1.0 except that set to return no data in uplink.

7.1.1.12.3.3.2	Test procedure sequence

Table 7.1.1.12.3.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits RRCReconfiguration to configure specific DCP parameters. (Note 1) | <-- | RRCReconfiguration | - | - |
| 2 | The UE transmits RRCReconfigurationComplete. (Note 2) | --> | RRCReconfigurationComplete | - | - |
| 3 | Wait 1280ms to ensure UE is out DRX active time. | - | - | - | - |
| 3A | The SS transmits DCI 2-6 on the PDCCH within the PS-offset time before the start of next long DRX drx-onDurationTimer and the DCI 2-6 indicates not to start the next Drx-onDurationTimer. | <-- | (PDCCH (DCI 2-6)) | - | - |
| 3B | In a PDCCH occasion the SS indicates the transmission of a DL MAC PDU on the PDCCH. | <-- | MAC PDU | - | - |
| 3C | Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 28? | --> | HARQ ACK | 5 | F |
| 4 | The SS transmits DCI 2-6 on the PDCCH within the PS-offset time before the start of next long DRX drx-onDurationTimer and the DCI 2-6 indicates to start the next Drx-onDurationTimer. | <-- | (PDCCH (DCI 2-6)) | - | - |
| 5 | In the first PDCCH occasion when the Drx-onDurationTimer is running, the SS indicates the transmission of a DL MAC PDU on the PDCCH. | <-- | MAC PDU | - | - |
| 6 | Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 5? | --> | HARQ ACK | 1 | P |
| 7 | SS transmits RRCReconfiguration to configure ps-wakeup with value true. (Note 1) | <-- | RRCReconfiguration | - | - |
| 8 | The UE transmits RRCReconfigurationComplete. (Note 2) | --> | RRCReconfigurationComplete | - | - |
| 9 | Wait 20ms to ensure UE is out DRX active time. | - | - | - | - |
| 10 | In the first PDCCH occasion when the Drx-onDurationTimer is running, the SS indicates the transmission of a DL MAC PDU on the PDCCH. | <-- | MAC PDU | - | - |
| 11 | Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 10? | --> | HARQ ACK | 2 | P |
| 12 | SS transmits RRCReconfiguration to configure specific DCP parameters. (Note 1) | <-- | RRCReconfiguration | - | - |
| 13 | The UE transmits RRCReconfigurationComplete. (Note 2) | --> | RRCReconfigurationComplete | - | - |
| 14 | Wait 1280ms to ensure UE is out DRX active time. | - | - | - | - |
| 15 | The SS transmits DCI 2-6 on the PDCCH within the PS-offset time before the start of next long DRX drx-onDurationTimer and the DCI 2-6 indicates to start the next Drx-onDurationTimer. | <-- | (PDCCH (DCI 2-6)) | - | - |
| 16 | In the last PDCCH occasion when the Drx-onDurationTimer is running, the SS indicates the transmission of an invalid DL MAC PDU on the PDCCH. | <-- | Invalid MAC PDU | - | - |
| 17 | The UE transmit a HARQ NACK for the DL MAC PDU in Step 16. | --> | HARQ NACK | - | - |
| 17A | The SS transmits DCI 2-6 on the PDCCH within the PS-offset time before the start of next long DRX drx-onDurationTimer and the DCI 2-6 indicates not to start the next Drx-onDurationTimer. | <-- | (PDCCH (DCI 2-6)) | - | - |
| 18 | In the PDCCH occasion when the next Drx-onDurationTimer is running, the SS indicates the transmission of a DL MAC PDU on the PDCCH. | <-- | MAC PDU | - | - |
| 19 | Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 18? | --> | HARQ ACK | 3 | P |
| 20 | SS transmits RRCReconfiguration to configure specific measonfig parameters. (Note 1) | <-- | RRCReconfiguration | - | - |
| 21 | The UE transmits RRCReconfigurationComplete. (Note 2) | --> | RRCReconfigurationComplete | - | - |
| 22 | Wait 10ms to ensure UE is out DRX active time. | - | - | - | - |
| 22A | The SS transmits DCI 2-6 on the PDCCH within the PS-offset time before the start of next long DRX drx-onDurationTimer and the DCI 2-6 indicates not to start the next Drx-onDurationTimer. | <-- | (PDCCH (DCI 2-6)) | - | - |
| 23 | In the first PDCCH occasion when the Drx-onDurationTimer is running, the SS indicates the transmission of a DL MAC PDU on the PDCCH. | <-- | MAC PDU | - | - |
| 24 | Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 23? | --> | HARQ ACK | 4 | P |
| Note 1: For EN-DC the NR RRCReconfiguration message is contained in RRCConnectionReconfiguration 36.508 [7], Table 4.6.1-8 using condition EN-DC_EmbedNR_RRCRecon. Note 2: For EN-DC the NR RRCReconfigurationComplete message is contained in RRCConnectionReconfigurationComplete. |  |  |  |  |  |

7.1.1.12.3.3.3	Specific message contents

Table 7.1.1.12.3.3.3-1: RRCReconfiguration (step 1, 7 and 12)

| Derivation Path: TS 38.508-1 [6] |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |  |
| secondaryCellGroup |  | CellGroupConfig |  | EN-DC |
| nonCriticalExtension := SEQUENCE {} |  | Not present |  | EN-DC |
| nonCriticalExtension SEQUENCE { |  |  |  | NR |
| masterCellGroup |  | CellGroupConfig |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 7.1.1.12.3.3.3-2: CellGroupConfig (Table 7.1.1.12.3.3.3-1: RRCReconfiguration)

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| cellGroupId | CellGroupId | TS 38.508-1 default value |  |
| mac-CellGroupConfig ::= SEQUENCE { |  |  |  |
| drx-Config CHOICE { |  |  |  |
| setup | DRX-Config | TS 38.508-1 default value |  |
| } |  |  |  |
| } |  |  |  |
| physicalCellGroupConfig::= SEQUENCE { |  |  |  |
| dcp-Config-r16 CHOICE { |  |  |  |
| setup | DCP-Config-r16 | TS 38.508-1 default value |  |
| } |  |  |  |
| } |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| spCellConfigDedicated | ServingCellConfig |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.12.3.3.3-3: ServingCellConfig (Table 7.1.1.13.3.3.3-2: CellGroupConfig)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-167 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| initialDownlinkBWP ::= SEQUENCE { |  |  |  |
| pdcch-Config CHOICE { |  |  | Step 1, Step 7, Step 12, Step 20 |
| setup | PDCCH-Config |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.12.3.3.3-4: PDCCH-Config (Table 7.1.1.12.3.3.3-3: BWP-DownlinkDedicated)

| Derivation Path: TS 38.508-1 [4],Table 4.6.3-95 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCCH-Config::= SEQUENCE { |  |  |  |
| controlResourceSetToAddModList SEQUENCE(SEQUENCE(SIZE (1..3)) OF ControlResourceSet ::= SEQUENCE { |  |  |  |
| controlResourceSetId | 1 |  |  |
| } |  |  |  |
| searchSpacesToAddModList SEQUENCE(SIZE (1..10)) OF SearchSpace { |  |  |  |
| searchSpace ::= SEQUENCE { |  |  |  |
| searchSpaceExt-r16 ::= SEQUENCE { |  |  |  |
| controlResourceSetId-r16 | 1 |  |  |
| searchSpaceType-r16 SEQUENCE { |  |  |  |
| common SEQUENCE { |  |  |  |
| dci-Format2-6-r16 SEQUENCE { |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| searchSpaceGroupIdList-r16 | Not present |  |  |
| freqMonitorLocations-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.12.3.3.3-5: CellGroupConfig (Table 7.1.1.13.3.3.3-1: RRCReconfiguration step 7)

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| cellGroupId | CellGroupId | TS 38.508-1 default value |  |
| mac-CellGroupConfig ::= SEQUENCE { |  |  |  |
| drx-Config CHOICE { |  |  |  |
| setup | DRX-Config | TS 38.508-1 default value |  |
| } |  |  |  |
| } |  |  |  |
| physicalCellGroupConfig::= SEQUENCE { |  |  |  |
| dcp-Config-r16 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| ps-WakeUp-r16 | true |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.12.3.3.3-6: CellGroupConfig (Table 7.1.1.13.3.3.3-1: RRCReconfiguration step 12)

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| cellGroupId | CellGroupId | TS 38.508-1 default value |  |
| mac-CellGroupConfig ::= SEQUENCE { |  |  |  |
| drx-Config CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| drx-onDurationTimer CHOICE { | ms10 |  |  |
| milliSeconds | ms10 |  |  |
| } |  |  |  |
| drx-InactivityTimer | ms6 |  |  |
| drx-HARQ-RTT-TimerDL | 56 |  |  |
| drx-HARQ-RTT-TimerUL | 56 |  |  |
| drx-RetransmissionTimerDL | sl320 |  |  |
| drx-RetransmissionTimerUL | sl320 |  |  |
| drx-LongCycleStartOffset CHOICE { |  |  |  |
| ms20 | 0 |  |  |
| } |  |  |  |
| shortDRX | Not present |  |  |
| drx-SlotOffset | ms0 |  |  |
| } |  |  |  |
| physicalCellGroupConfig::= SEQUENCE { |  |  |  |
| dcp-Config-r16 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| ps-Offset-r16 | 40 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.12.3.3.3-7: RRCReconfiguration (step 20)

| Derivation Path: TS 38.508-1 [6], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |  |
| secondaryCellGroup |  | CellGroupConfig |  | EN-DC |
| measConfig ::= SEQUENCE { |  |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasObjectToAddMod { |  | 2 entries |  |  |
| MeasObjectToAddMod[1] SEQUENCE { |  |  | entry 1 |  |
| measObjectId |  | 1 |  |  |
| measObject CHOICE { |  |  |  |  |
| measObjectNR SEQUENCE { |  |  |  |  |
| ssbFrequency |  | ARFCN-ValueNR for SSB of NR Cell 1 |  |  |
| absThreshSS-BlocksConsolidation |  | Not present |  |  |
| nrofSS-BlocksToAverage |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| MeasObjectToAddMod[2] SEQUENCE { |  |  |  |  |
| measObjectId |  | 2 |  |  |
| measObject CHOICE { |  |  |  |  |
| measObjectNR SEQUENCE { |  |  |  |  |
| ssbFrequency |  | ARFCN-ValueNR for SSB of NR Cell 3 |  |  |
| absThreshSS-BlocksConsolidation |  | Not present |  |  |
| nrofSS-BlocksToAverage |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| reportConfigToAddModList SEQUENCE(SIZE (1..maxReportConfigId)) OF ReportConfigToAddMod { |  | 1 entry |  |  |
| ReportConfigToAddMod[1] SEQUENCE { |  |  | entry 1 |  |
| reportConfigId |  | 1 |  |  |
| reportConfig CHOICE { |  |  |  |  |
| reportConfigNR SEQUENCE { |  |  |  |  |
| reportType CHOICE { |  |  |  |  |
| eventTriggered SEQUENCE { |  |  |  |  |
| eventId CHOICE { |  |  |  |  |
| eventA3 SEQUENCE { |  |  |  |  |
| a3-Offset CHOICE { |  |  |  |  |
| rsrp |  | 2 | 1 dB (2*0.5 dB) |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| reportAmount |  | infinity |  |  |
| reportQuantityCell SEQUENCE { |  |  |  |  |
| rsrp |  | true |  |  |
| rsrq |  | false |  |  |
| sinr |  | false |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasIdToAddMod { |  | 1 entry |  |  |
| MeasIdToAddMod[1] SEQUENCE { |  |  | entry 1 |  |
| measId |  | 1 |  |  |
| measObjectId |  | 2 |  |  |
| reportConfigId |  | 1 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| measGapConfig ::= SEQUENCE { |  |  |  |  |
| gapUE CHOICE { |  |  |  |  |
| setup SEQUENCE { |  |  |  |  |
| gapOffset |  | 34 |  |  |
| mgl |  | ms6 |  |  |
| mgrp |  | ms40 |  |  |
| mgta |  | ms0 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  | NR |
| masterCellGroup |  | CellGroupConfig |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 7.1.1.12.3.3.3-8: CellGroupConfig (Table 7.1.1.12.3.3.3-7: RRCReconfiguration step 20)

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| cellGroupId | CellGroupId | TS 38.508-1 default value |  |
| mac-CellGroupConfig ::= SEQUENCE { |  |  |  |
| drx-Config CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| drx-onDurationTimer CHOICE { |  |  |  |
| milliSeconds | ms10 |  |  |
| } |  |  |  |
| drx-InactivityTimer | ms5 |  |  |
| drx-LongCycleStartOffset CHOICE { |  |  |  |
| ms40 | 39 |  |  |
| } |  |  |  |
| shortDRX | Not present |  |  |
| drx-SlotOffset | ms0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| physicalCellGroupConfig::= SEQUENCE { |  |  |  |
| dcp-Config-r16 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| ps-Offset-r16 | 40 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |