3GPP TSG-RAN5 Meeting #95-e	R5-223059

# Online, , 9th May 2022 - 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 2971 | rev | - | Current version: | 16.11.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of NR MDT test case 8.1.6.3.4.x |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to TS 38.331, the measResultNeighCells-r16 is not optional in ConnEstFailReport-r16, therefore the specific message content needs to be updated accordingly. ConnEstFailReport-r16 ::= SEQUENCE { measResultFailedCell-r16 MeasResultFailedCell-r16, locationInfo-r16 LocationInfo-r16 OPTIONAL, measResultNeighCells-r16 SEQUENCE { measResultNeighCellListNR MeasResultList2NR-r16 OPTIONAL, measResultNeighCellListEUTRA MeasResultList2EUTRA-r16 OPTIONAL }, numberOfConnFail-r16 INTEGER (1..8), perRAInfoList-r16 PerRAInfoList-r16, timeSinceFailure-r16 TimeSinceFailure-r16, ... } |  |  |  |  |  |  |  |  |
| Summary of change: |  | The specific message contents were updated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail this case unfairly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.6.3.4.1.3.3, 8.1.6.3.4.2.3.3 and 8.1.6.3.4.3.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

[Start of change]

8.1.6.3.4	Inter-System MDT / Connection Establishment Failure

8.1.6.3.4.1	Inter-System MDT / Connection Establishment Failure / Logging and reporting / Bluetooth measurement collection

8.1.6.3.4.1.1	Test Purpose (TP)

(1)

with { UE after being set up with LoggedMeasurementConfiguration detects connection establishment failure }

ensure that {

when { UE has Bluetooth logged measurements available and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport }

then { UE sends the RRCSetupComplete message with logMeasAvailable-r16, logMeasAvailableBT-r16 and connEstFailInfoAvailable-r16 set to true }

}

(2)

with { UE has sent the RRCSetupComplete message with connEstFailInfoAvailable-r16 set to true }

ensure that {

when { UE receives a UEInformationRequest message with connEstFailReportReq-r16 set to true }

then { UE sends the UEInformationResponse message containing only the measurement of the Bluetooth beacon(s) that included in bt-NameList of LoggedMeasurementConfiguration }

}

8.1.6.3.4.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.331 clauses 5.5a.1.3, 5.5a.3.2, 5.3.3.4, 5.3.3.7 and 5.7.10.3. Unless otherwise stated these are Rel-16 requirements.

[TS 38.331, clause 5.5a.1.3]

Upon receiving the LoggedMeasurementConfiguration message the UE shall:

1>	discard the logged measurement configuration as well as the logged measurement information as specified in 5.5a.2;

1>	store the received loggingDuration, reportType and areaConfiguration, if included, in VarLogMeasConfig;

1>	if the LoggedMeasurementConfiguration message includes plmn-IdentityList:

2>	set plmn-IdentityList in VarLogMeasReport to include the RPLMN as well as the PLMNs included in plmn-IdentityList;

1>	else:

2>	set plmn-IdentityList in VarLogMeasReport to include the RPLMN;

1>	store the received absoluteTimeInfo, traceReference, traceRecordingSessionRef, and tce-Id in VarLogMeasReport;

- store the received bt-NameList, if included, in VarLogMeasConfig;

[TS 38.331, clause 5.5a.3.2]

While T330 is running, the UE shall:

1>	perform the logging in accordance with the following:

2>	if the reportType is set to periodical in the VarLogMeasConfig:

3>	if the UE is in any cell selection state (as specified in TS 38.304 [20]):

4>	perform the logging at regular time intervals, as defined by the loggingInterval in the VarLogMeasConfig;

3>	if the UE is in camped normally state on an NR cell and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

4>	if areaConfiguration is not included in VarLogMeasConfig; or

4>	if the serving cell is part of the area indicated by areaConfig in areaConfiguration in VarLogMeasConfig:

5>	perform the logging at regular time intervals, as defined by the loggingInterval in the VarLogMeasConfig;

…

2>	when performing the logging:

3>	set the relativeTimeStamp to indicate the elapsed time since the moment at which the logged measurement configuration was received;

3>	if location information became available during the last logging interval, set the content of the locationInfo as in 5.3.3.7:

[TS 38.331, clause 5.3.3.4]

The UE shall perform the following actions upon reception of the RRCSetup:

…

1>	set the content of RRCSetupComplete message as follows:

…

2>	if the UE has logged measurements available for NR and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

3>	include the logMeasAvailable in the RRCSetupComplete message;

3>	if Bluetooth measurement results are included in the logged measurements the UE has available for NR:

4>	include the logMeasAvailableBT in the RRCSetupComplete message;

...

2>	if the UE has connection establishment failure or connection resume failure information available in VarConnEstFailReport and if the RPLMN is equal to plmn-Identity stored in VarConnEstFailReport:

3>	include connEstFailInfoAvailable in the RRCSetupComplete message;

…

1>	submit the RRCSetupComplete message to lower layers for transmission, upon which the procedure ends.

[TS 38.331, clause 5.3.3.7]

The UE shall:

1>	if timer T300 expires:

…

2>	store the following connection establishment failure information in the VarConnEstFailReport by setting its fields as follows:

…

3>	if available, set the locationInfo as follows:

4>	if available, set the commonLocationInfo to include the detailed location information;

4>	if available, set the bt-LocationInfo to include the Bluetooth measurement results, in order of decreasing RSSI for Bluetooth beacons;

4>	if available, set the wlan-LocationInfo to include the WLAN measurement results, in order of decreasing RSSI for WLAN APs;

4>	if available, set the sensor-LocationInfo to include the sensor measurement results as follows;

5>	if available, include the sensor-MeasurementInformation;

5>	if available, include the sensor-MotionInformation;

[TS 38.331, clause 5.7.10.3]

Upon receiving the UEInformationRequest message, the UE shall, only after successful security activation:

…

1>	if connEstFailReportReq is set to true and the UE has connection establishment failure or connection resume failure information in VarConnEstFailReport and if the RPLMN is equal to plmn-Identity stored in VarConnEstFailReport:

2>	set timeSinceFailure in VarConnEstFailReport to the time that elapsed since the last connection establishment failure or connection resume failure in NR;

2>	set the connEstFailReport in the UEInformationResponse message to the value of connEstFailReport in VarConnEstFailReport;

2>	discard the connEstFailReport from VarConnEstFailReport upon successful delivery of the UEInformationResponse message confirmed by lower layers;

8.1.6.3.4.1.3	Test description

8.1.6.3.4.1.3.1	Pre-test conditions

System Simulator:

- NR Cell 1, Bluetooth beacon 1 (Cell 40), Bluetooth beacon 2 (Cell 41)

-	Cell 40 and Cell 41 are configured as per 38.508-1 [4] cl 4.4.1.3.2

UE:

None

Preamble:

- The UE is in state 3N-A on NR Cell 1 as defined in TS 38.508-1 [4], subclause 4.4A.

8.1.6.3.4.1.3.2	Test procedure sequence

Table 8.1.6.3.4.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits a LoggedMeasurementConfiguration message to configure the UE to perform logging of measurement results while in RRC_IDLE. | <-- | LoggedMeasurementConfiguration | - | - |
| 2 | The SS transmits an RRCRelease message. | <-- | RRCRelease | - | - |
| 3 | Wait 8s to allow UE to activate logging. | - | - | - | - |
| 4-5 | Steps 1 to 2 of the generic radio bearer establishment procedure in TS 38.508-1 [4] table 4.5.4.2-3. | - | - | - | - |
| 6 | The SS waits for 2sec (T300 expiry). | - | - | - | - |
| 7-9 | Steps 1 to 3 of the generic radio bearer establishment procedure in TS 38.508-1 [4] table 4.5.4.2-3. | - | - | - | - |
| 10 | Check: Does the UE transmit an RRCSetupComplete message with logMeasAvailable-r16, logMeasAvailableBT-r16 and connEstFailInfoAvailable-r16 included? | --> | NR RRC: RRCSetupComplete 5GMM: Service Request | 1 | P |
| 11-14 | Steps 5 to 8 of the generic radio bearer establishment procedure in TS 38.508-1 [4] table 4.5.4.2-3. | - | - | - | - |
| 15 | The SS sends a UEInformationRequest message to get connEstFailReportReq-r16. | <-- | UEInformationRequest | - | - |
| 16 | Check: Does the UE transmit a UEInformationResponse message with LogMeasResultListBT including one entry (Bluetooth beacon 1) measurement result? | --> | UEInformationResponse | 2 | P |

8.1.6.3.4.1.3.3	Specific message contents

Table 8.1.6.3.4.1.3.3-1: LoggedMeasurementConfiguration (step 1, Table 8.1.6.3.4.1.3.2-1)

| Derivation path: TS 38.508-1 [4], table 4.6.1-5AA with condition PERIODICAL |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LoggedMeasurementConfiguration-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| loggedMeasurementConfiguration-r16 SEQUENCE { |  |  |  |
| bt-NameList-r16 CHOICE { |  |  |  |
| setup | BT-NameList-r16 |  |  |
| } |  |  |  |
| reportType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  |  |
| loggingInterval-r16 | ms10240 | 10.24 seconds |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.3.4.1.3.3-1A: BT-NameList-r16 (Table 8.1.6.3.4.1.3.3-1)

| Derivation path: TS 38.508-1 [4], table 4.6.5-0C |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BT-NameList-r16 ::= SEQUENCE (SIZE (1..maxBT-Name-r16)) OF BT-Name-r16 { | 1 entry | Bluetooth beacon 1 |  |
| BT-Name-r16[1] | Set as per Table 4.4.9-1 of 3GPP TS 36.508 | OCTET STRING (SIZE (1..248)) Refers to LOCAL NAME defined in Bluetooth specification |  |
| } |  |  |  |

Table 8.1.6.3.4.1.3.3-2: RRCSetupComplete (step 10, Table 8.1.6.3.4.1.3.2-1)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-22 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCSetupComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcSetupComplete SEQUENCE{ |  |  |  |
| nonCriticalExtension SEQUENCE{ |  |  |  |
| iab-NodeIndication-r16 | Not present |  |  |
| idleMeasAvailable-r16 | Not present |  |  |
| ue-MeasurementsAvailable-r16 SEQUENCE { |  |  |  |
| logMeasAvailable-r16 | true |  |  |
| logMeasAvailableBT-r16 | true |  |  |
| logMeasAvailableWLAN-r16 | Not present |  |  |
| connEstFailInfoAvailable-r16 | true |  |  |
| rlf-InfoAvailable-r16 | Not present |  |  |
| } |  |  |  |
| mobilityHistoryAvail-r16 | Not present |  |  |
| mobilityState-r16 | Not present |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.3.4.1.3.3-3: UEInformationRequest (step 15, Table 8.1.6.3.4.1.3.2-1)

Derivation Path: TS 38.508-1 [4], Table 4.6.1-32A with condition FAIL

Table 8.1.6.3.4.1.3.3-4: UEInformationResponse (step 16, Table 8.1.6.3.4.1.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-32B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueInformationResponse-r16 SEQUENCE { |  |  |  |
| connEstFailReport-r16 SEQUENCE { |  |  |  |
| measResultFailedCell-r16 SEQUENCE { |  |  |  |
| cgi-Info SEQUENCE { |  |  |  |
| plmn-Identity-r16 | plmn-Identity within SIB1 broadcasted in NR Cell 1 |  |  |
| cellIdentity-r16 | cellIdentity within SIB1 broadcasted in NR Cell 1 | BIT STRING (SIZE (36)) |  |
| trackingAreaCode-r16 | Not checked | BIT STRING (SIZE (24)) |  |
| } |  |  |  |
| measResult-r16 SEQUENCE { |  |  |  |
| cellResults-r16 | Not checked |  |  |
| rsIndexResults-r16 | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| locationInfo-r16 SEQUENCE { |  |  |  |
| bt-LocationInfo-r16 SEQUENCE (SIZE (1..maxBT-IdReport-r16)) OF LogMeasResultBT-r16 { | 1 entry |  |  |
| LogMeasResultBT-r16[1] SEQUENCE { |  | Report Bluetooth beacon 1 |  |
| bt-Addr-r16 | Bluetooth public address of Bluetooth beacon 1 | BIT STRING (SIZE (48)) |  |
| rssi-BT-r16 | Any allowed value | INTEGER (-128..127) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultNeighCells-r16 SEQUENCE { |  |  |  |
| measResultNeighCellListNR | Not present |  |  |
| measResultNeighCellListEUTRA | Not present |  |  |
| } |  |  |  |
| numberOfConnFail-r16 | Any allowed value | INTEGER (1..8) |  |
| perRAInfoList-r16 | Any allowed value |  |  |
| timeSinceFailure-r16 | Any allowed value | INTEGER (0..172800) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

8.1.6.3.4.2	Inter-System MDT / Connection Establishment Failure / Logging and reporting / WLAN measurement collection

8.1.6.3.4.2.1	Test Purpose (TP)

(1)

with { UE after being set up with LoggedMeasurementConfiguration detects connection establishment failure }

ensure that {

when { UE has WLAN logged measurements available and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport }

then { UE sends the RRCSetupComplete message with logMeasAvailable-r16, logMeasAvailableWLAN-r16 and connEstFailInfoAvailable-r16 set to true }

}

(2)

with { UE has sent the RRCSetupComplete message with connEstFailInfoAvailable-r16 set to true }

ensure that {

when { UE receives a UEInformationRequest message with connEstFailReportReq-r16 set to true }

then { UE sends the UEInformationResponse message containing only the measurement of the WLAN AP(s) that included in wlan-NameList-r16 of LoggedMeasurementConfiguration }

}

8.1.6.3.4.2.2	Conformance requirements

Same as test case 8.1.6.3.4.1.

8.1.6.3.4.2.3	Test description

8.1.6.3.4.2.3.1	Pre-test conditions

Same as test case 8.1.6.3.4.1 with the following differences:

System Simulator:

-	NR Cell 1, WLAN AP 1 (Cell 27) and WLAN AP 2 (Cell 27b)

- Cell 27 and Cell 27b are configured as per TS 38.508-1 [4] cl 4.4.1.3 with the OFFLOAD condition

8.1.6.3.4.2.3.2	Test procedure sequence

Same as test case 8.1.6.3.4.1 with the following differences: logMeasAvailableBT-r16 is replaced by logMeasAvailableWLAN-r16.

Table 8.1.6.3.4.2.3.2-1: Void

8.1.6.3.4.2.3.3	Specific message contents

Table 8.1.6.3.4.2.3.3-1: LoggedMeasurementConfiguration (step 1, Table 8.1.6.3.4.2.3.2-1)

| Derivation path: TS 38.508-1 [4], table 4.6.1-5AA with condition PERIODICAL |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LoggedMeasurementConfiguration-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| loggedMeasurementConfiguration-r16 SEQUENCE { |  |  |  |
| wlan-NameList-r16 CHOICE { |  |  |  |
| setup | WLAN-NameList-r16 |  |  |
| } |  |  |  |
| reportType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  |  |
| loggingInterval-r16 | ms10240 | 10.24 seconds |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.3.4.2.3.3-1A: WLAN-NameList-r16 (Table 8.1.6.3.4.2.3.3-1)

| Derivation path: TS 38.508-1 [4], table 4.6.5-18 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| WLAN-NameList-r16 ::= SEQUENCE (SIZE (1..maxWLAN-Name-r16)) OF WLAN-Name-r16 { | 1 entry | WLAN AP1 |  |
| WLAN-Name-r16[1] | Set as per Table 4.4.8-1 of TS 36.508 | OCTET STRING (SIZE (1..32)) Refers to Service Set Identifier (SSID) defined in IEEE 802.11-2012 |  |
| } |  |  |  |

Table 8.1.6.3.4.2.3.3-2: RRCSetupComplete (step 10, Table 8.1.6.3.4.2.3.2-1)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-22 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCSetupComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcSetupComplete SEQUENCE{ |  |  |  |
| nonCriticalExtension SEQUENCE{ |  |  |  |
| iab-NodeIndication-r16 | Not present |  |  |
| idleMeasAvailable-r16 | Not present |  |  |
| ue-MeasurementsAvailable-r16 SEQUENCE { |  |  |  |
| logMeasAvailable-r16 | true |  |  |
| logMeasAvailableBT-r16 | Not present |  |  |
| logMeasAvailableWLAN-r16 | true |  |  |
| connEstFailInfoAvailable-r16 | true |  |  |
| rlf-InfoAvailable-r16 | Not present |  |  |
| } |  |  |  |
| mobilityHistoryAvail-r16 | Not present |  |  |
| mobilityState-r16 | Not present |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.3.4.2.3.3-3: UEInformationRequest (step 15, Table 8.1.6.3.4.2.3.2-1)

Derivation Path: TS 38.508-1 [4], Table 4.6.1-32A with condition FAIL

Table 8.1.6.3.4.2.3.3-4: UEInformationResponse (step 16, Table 8.1.6.3.4.2.3.2-1)

| Derivation Path: TS 38.508-1 [4] table 4.6.1-32B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueInformationResponse-r16 SEQUENCE { |  |  |  |
| connEstFailReport-r16 SEQUENCE { |  |  |  |
| measResultFailedCell-r16 SEQUENCE { |  |  |  |
| cgi-Info SEQUENCE { |  |  |  |
| plmn-Identity-r16 | plmn-Identity within SIB1 broadcasted in NR Cell 1 |  |  |
| cellIdentity-r16 | cellIdentity within SIB1 broadcasted in NR Cell 1 | BIT STRING (SIZE (36)) |  |
| trackingAreaCode-r16 | Not checked | BIT STRING (SIZE (24)) |  |
| } |  |  |  |
| measResult-r16 SEQUENCE { |  |  |  |
| cellResults-r16 | Not checked |  |  |
| rsIndexResults-r16 | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| locationInfo-r16 SEQUENCE { |  |  |  |
| wlan-LocationInfo-r16 SEQUENCE (SIZE (1..maxWLAN-Id-Report-r16)) OF LogMeasResultWLAN-r16 { | 1 entry |  |  |
| LogMeasResultWLAN-r16[1] SEQUENCE { |  | Report WLAN AP 1 |  |
| wlan-Identifiers-r16 SEQUENCE { |  |  |  |
| ssid-r12 | ssid for WLAN AP 1 |  |  |
| bssid-r12 | Not present |  |  |
| hessid-r12 | Not present |  |  |
| } |  |  |  |
| rssiWLAN-r16 | Any allowed value |  |  |
| rtt-WLAN-r16 | Any allowed value |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultNeighCells-r16 SEQUENCE { |  |  |  |
| measResultNeighCellListNR | Not present |  |  |
| measResultNeighCellListEUTRA | Not present |  |  |
| } |  |  |  |
| numberOfConnFail-r16 | Any allowed value | INTEGER (1..8) |  |
| perRAInfoList-r16 | Any allowed value |  |  |
| timeSinceFailure-r16 | Any allowed value | INTEGER (0..172800) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

8.1.6.3.4.3	Inter-System MDT / Connection Establishment Failure / Logging and reporting / Sensor measurement collection

8.1.6.3.4.3.1	Test Purpose (TP)

(1)

with { UE after being set up with LoggedMeasurementConfiguration detects connection establishment failure }

ensure that {

when { UE has sensor logged measurements available and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport }

then { UE sends the RRCSetupComplete message with logMeasAvailable-r16 and connEstFailInfoAvailable-r16 set to true }

}

(2)

with { UE has sent the RRCSetupComplete message with connEstFailInfoAvailable-r16 set to true }

ensure that {

when { UE receives a UEInformationRequest message with connEstFailReportReq-r16 set to true }

then { UE sends the UEInformationResponse message containing only the measurement of the sensor(s) that included in sensor-NameList-r16 of LoggedMeasurementConfiguration }

}

8.1.6.3.4.3.2	Conformance requirements

Same as test case 8.1.6.3.4.1.

8.1.6.3.4.3.3	Test description

8.1.6.3.4.3.3.1	Pre-test conditions

Same as test case 8.1.6.3.4.1 with the following differences:

System Simulator:

-	NR Cell 1

UE:

-	UE supports at least one of pc_barometer_r16, pc_orientation_r16 and pc_speed_r16.

8.1.6.3.4.3.3.2	Test procedure sequence

Same as test case 8.1.6.3.4.1 with the following differences: logMeasAvailableBT-r16 is not mentioned.

Table 8.1.6.3.4.3.3.2-1: Void

8.1.6.3.4.3.3.3	Specific message contents

Table 8.1.6.3.4.3.3.3-1: LoggedMeasurementConfiguration (step 1, Table 8.1.6.3.4.3.3.2-1)

| Derivation path: TS 38.508-1 [4], table 4.6.1-5AA with condition PERIODICAL |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LoggedMeasurementConfiguration-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| loggedMeasurementConfiguration-r16 SEQUENCE { |  |  |  |
| sensor-NameList-r16 CHOICE { |  |  |  |
| setup | Sensor-NameList-r16 |  |  |
| } |  |  |  |
| reportType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  |  |
| loggingInterval-r16 | ms10240 | 10.24 second |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.3.4.3.3.3-1A: Sensor-NameList-r16 (Table 8.1.6.3.4.3.3.3-1)

| Derivation path: TS 38.508-1 [4], table 4.6.5-13 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |  |
| Sensor-NameList-r16 ::= SEQUENCE { |  |  |  |  |  |  |  |
| measUncomBarPre-r16 |  | true |  |  |  | pc_barometer_r16 |  |
|  |  | Not present |  |  |  |  |  |
| measUeSpeed |  | true |  |  |  | pc_speed_r16 |  |
|  |  | Not present |  |  |  |  |  |
| measUeOrientation |  | true |  |  |  | pc_orientation_r16 |  |
|  |  | Not present |  |  |  |  |  |
| } |  |  |  |  |  |  |  |

Table 8.1.6.3.4.3.3.3-2: RRCSetupComplete (step 10, Table 8.1.6.3.4.3.3.2-1)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-22 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCSetupComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcSetupComplete SEQUENCE{ |  |  |  |
| nonCriticalExtension SEQUENCE{ |  |  |  |
| iab-NodeIndication-r16 | Not present |  |  |
| idleMeasAvailable-r16 | Not present |  |  |
| ue-MeasurementsAvailable-r16 SEQUENCE { |  |  |  |
| logMeasAvailable-r16 | true |  |  |
| logMeasAvailableBT-r16 | Not checked |  |  |
| logMeasAvailableWLAN-r16 | Not checked |  |  |
| connEstFailInfoAvailable-r16 | true |  |  |
| rlf-InfoAvailable-r16 | Not checked |  |  |
| } |  |  |  |
| mobilityHistoryAvail-r16 | Not present |  |  |
| mobilityState-r16 | Not present |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.3.4.3.3.3-3: UEInformationRequest (step 15, Table 8.1.6.3.4.3.3.2-1)

Derivation Path: TS 38.508-1 [4], Table 4.6.1-32A with condition FAIL

Table 8.1.6.3.4.3.3.3-4: UEInformationResponse (step 16, Table 8.1.6.3.4.3.3.2-1)

| Derivation Path: TS 38.508-1 [4], table 4.6.1-32B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueInformationResponse-r16 SEQUENCE { |  |  |  |
| connEstFailReport-r16 SEQUENCE { |  |  |  |
| measResultFailedCell-r16 SEQUENCE { |  |  |  |
| cgi-Info SEQUENCE { |  |  |  |
| plmn-Identity-r16 | plmn-Identity within SIB1 broadcasted in NR Cell 1 |  |  |
| cellIdentity-r16 | cellIdentity within SIB1 broadcasted in NR Cell 1 | BIT STRING (SIZE (36)) |  |
| trackingAreaCode-r16 | Not checked | BIT STRING (SIZE (24)) |  |
| } |  |  |  |
| measResult-r16 SEQUENCE { |  |  |  |
| cellResults-r16 | Not checked |  |  |
| rsIndexResults-r16 | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| locationInfo-r16 SEQUENCE { |  |  |  |
| sensor-LocationInfo-r16 SEQUENCE { |  | Report sensor measurement information |  |
| sensor-MeasurementInformation-r16 | Not present |  |  |
|  | OCTET STRING including Sensor-MeasurementInformation-r13 according to TS 37.355 cl 6.5.5.2. |  | pc_barometer_r16 |
| sensor-MotionInformation-r16 | Not present |  |  |
|  | OCTET STRING including Sensor-MotionInformation-r15 according to TS 37.355 cl 6.5.5.2. |  | pc_orientation_r16 OR pc_speed_r16 |
| } |  |  |  |
| } |  |  |  |
| measResultNeighCells-r16 SEQUENCE { |  |  |  |
| measResultNeighCellListNR | Not present |  |  |
| measResultNeighCellListEUTRA | Not present |  |  |
| } |  |  |  |
| numberOfConnFail-r16 | Any allowed value | INTEGER (1..8) |  |
| perRAInfoList-r16 | Any allowed value |  |  |
| timeSinceFailure-r16 | Any allowed value | INTEGER (0..172800) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

[End of change]