3GPP TSG-RAN5 Meeting #95-e	R5-223751

# Online, , 9th May 2022 - 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-2 | CR | 0742 | rev | 1 | Current version: | 16.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates across Spherical Coverage test cases to incorporate Rel.16 requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple Portugal |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_FR2_req_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Core specification has added rel.16 minimum conformance requirements for EIS Spherical Coverage with Inter-band CA |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce test case structure for EIS Spherical Coverage for Inter-band CA |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing Rel16 Inter-band CA tests in test specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3A.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1 updatest test case structure and content for the 2CC spherical coverage test in line with existing 2CC tests. MU/TT and test config are FFS and indicated in editor’s notes R5-223038r1->Final TDoc R5-223751 |  |  |  |  |  |  |  |  |

<Unchanged Sections Skipped>

Table 7.3A.3.3-1: ΔRIB,S,n EIS spherical coverage requirement relaxation for inter-band CA for power class 3

| NR CA band combination | NR band | ΔRIB,S,n (dB) |
| --- | --- | --- |
| CA_n260-n261 | n260 | 3.5 |
|  | n261 | 3.5 |

#### 7.3A.3.1	EIS Spherical Coverage for Inter-band CA (2DL CA)

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Measurement Uncertainties and Test Tolerances are FFS

- Test Config is FFS.

- In case of frequency separation larger than 800 MHz and in case the device manufacturer does not explicitly declare that the beam peak for a reference (frequency band, CBW) or (frequency band combination, CA BW class) is applicable for a group of other intra-band contiguous combinations and CA BW classes, according to Table A.4.3.9-6 in 38.508-2, following aspect of beam peak search procedures for CA is FFS: RB allocation, power level, channel bandwidth configuration, per CC approach or all CC combined approach, etc

7.3A.3.1	Test purpose

Same test purpose as in 7.3.4.1

7.3A.3.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that supports FR2 2DL inter-band CA.

7.3A.3.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 7.3A.3.0.

The normative reference for this requirement is TS 38.101-2 [3] clause 7.3A.3.3.

7.3A.3.4	Test description

7.3A.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR CA configurations specified in clause 5.5A. All of these configurations shall be tested with applicable test parameters for each combination of channel bandwidth and subcarrier spacing are shown in Table [TBD], Table [TBD] and Table [TBD]. The details of the uplink and downlink reference measurement channels (RMCs) are specified in Annexes A.2 and A.3. The details of the OCNG patterns used are specified in Annex A.5. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 7.3A.2.1.4.1-1: Test Configuration Table

FFS

7.3A.3.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1, C.2 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 38.508-1 [10] clause 5.5.1. Message contents are defined in clause 7.3A.2.1.4.3.

3.	SS activates SCC by sending the activation MAC CE (Refer TS 38.321[28], clauses 5.9, 6.1.3.10). Wait for at least 2 seconds (Refer TS 38.133[25], clause 9.2).

4.	SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Table 7.3.2.4.1-1. The SS sends downlink MAC padding bits on the DL RMC.

5.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Tables 7.3.2.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

3.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200msec for the UE to reach PUMAX.

4.	Set the UE in the Rx beam peak direction found with a 3D EIS scan as performed in Annex K.1.2. Allow at least BEAM_SELECT_WAIT_TIME (NOTE 1) for the UE Rx beam selection to complete.

5.	For each component carrier, measure UE EIS value for each grid point according to EIS spherical coverage procedure defined in Annex K.1.6, and obtain a Complimentary Cumulative Distribution Function (CCDF) of all EIS dBm values.

6.	Identify the EIS dBm value corresponding to %-tile (UE power class dependent) value in the applicable test requirement table in section 7.3.4.5.

7.	Compare the EIS dBm value identified in step 6, to the limit value in the applicable test requirement table in section 7.3.4.5. If the EIS dBm value is lower or equal to the limit value, pass the UE. Otherwise fail the UE.

NOTE 1:	The BEAM_SELECT_WAIT_TIME default value is defined in Annex K.1.2.

7.3A.3.4.3	Message contents

Same as 7.3.4.4.3

7.3A.3.5	Test requirement

The reference measurement channels and throughput criterion shall be as specified in section 7.3.2.5.

Table 7.3A.3.5-1: ΔRIB,S,n EIS spherical coverage requirement relaxation per component carrier for inter-band CA for power class 3

| NR CA band combination | NR band | ΔRIB,S,n (dB) |
| --- | --- | --- |
| CA_n260-n261 | n260 | 3.5 |
|  | n261 | 3.5 |

Table 7.3A.3.5-2: EIS spherical coverage per component carrier for power class 3 for single band UE or multi-band UE declaring MBs = 0 in all FR2 bands

| Operating band |  | EIS at 50th%ile CCDF (dBm) / Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 50 MHz |  | 100 MHz | 200 MHz | 400 MHz |
| n257 | -77.4 +TT+ ΔRIB,S,n |  | -74.4 +TT+ ΔRIB,S,n | -71.4 +TT+ ΔRIB,S,n | -68.4 +TT+ ΔRIB,S,n |
| n259 | -71.9 +TT+ ΔRIB,S,n |  | -68.9 +TT+ ΔRIB,S,n | -65.9 +TT+ ΔRIB,S,n | -62.9 +TT+ ΔRIB,S,n |
| n258 | -77.4 +TT+ ΔRIB,S,n |  | -74.4 +TT+ ΔRIB,S,n | -71.4 +TT+ ΔRIB,S,n | -68.4 +TT+ ΔRIB,S,n |
| n260 | -73.1 +TT+ ΔRIB,S,n |  | -70.1 +TT+ ΔRIB,S,n | -67.1 +TT+ ΔRIB,S,n | -64.1 +TT+ ΔRIB,S,n |
| n261 | -77.4 +TT+ ΔRIB,S,n |  | -74.4 +TT+ ΔRIB,S,n | -71.4 +TT+ ΔRIB,S,n | -68.4 +TT+ ΔRIB,S,n |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.4. NOTE 2: The EIS spherical coverage requirements are verified only under normal thermal conditions as defined in TS 38.508-1 [10] subclause 4.1.1. |  |  |  |  |  |

Table 7.3A.3.5-2a: EIS spherical coverage per component carrier for power class 3 for multi-band UE declaring MBs > 0 in any FR2 band (Rel-15)

| Operating band | EIS at 50th%ile CCDF (dBm) / Channel bandwidth (NOTE 3) |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n257 | -77.4 +TT+MBs+ ΔRIB,S,n | -74.4 +TT+MBs+ ΔRIB,S,n | -71.4 +TT+MBs+ ΔRIB,S,n | -68.4 +TT+MBs+ ΔRIB,S,n |
| n258 | -77.4 +TT+MBs+ ΔRIB,S,n | -74.4 +TT+MBs+ ΔRIB,S,n | -71.4 +TT+MBs+ ΔRIB,S,n | -68.4 +TT+MBs+ ΔRIB,S,n |
| n260 | -73.1 +TT+MBs+ ΔRIB,S,n | -70.1 +TT+MBs+ ΔRIB,S,n | -67.1 +TT+MBs+ ΔRIB,S,n | -64.1 +TT+MBs+ ΔRIB,S,n |
| n261 | -77.4 +TT+MBs+ ΔRIB,S,n | -74.4 +TT+MBs+ ΔRIB,S,n | -71.4 +TT+MB+ ΔRIB,S,n s | -68.4 +TT+MBs+ ΔRIB,S,n |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.4. NOTE 2: The EIS spherical coverage requirements are verified only under normal thermal conditions as defined in TS 38.508-1 [10] subclause 4.1.1. NOTE 3: Refer Table 7.3.4.5-3b for details for MBs allowance corresponding to supported FR2 band set combination NOTE 4: For a Rel-15 UE supporting FR2 bands set not defined in Table 7.3.2.3.3-1a, Table 7.3.4.5-3c applies. |  |  |  |  |  |

Table 7.3A.3.5-2b: EIS spherical coverage multiband relaxation factors per component carrier for power class 3 (Rel-15)

| ID | Supported FR2 bands set | Maximum sum of MBs, ∑MBs (dB) (Note 3) | Comments |
| --- | --- | --- | --- |
| 1 | n257, n258 | 1.25 | Maximum 0.75 dB relaxation allowed for each band |
| 2 | n257, n260 | 0.75 | Maximum 0.4 dB relaxation allowed for n260 and 0.75 dB relaxation allowed for all other bands |
| 3 | n258, n260 | 0.75 | Maximum 0.4 dB relaxation allowed for n260 and 0.75 dB relaxation allowed for all other bands |
| 4 | n258, n261 | 1.25 | Maximum 0.75 dB relaxation allowed for each band |
| 5 | n260, n261 | 0.75 | No relaxation allowed for n260 and 0.75 dB relaxation allowed for all other bands |
| 6 | n257, n258, n260 | 1.75 | Maximum 0.4 dB relaxation allowed for n260 and 0.75 dB relaxation allowed for all other bands |
| 7 | n257, n258, n261 | 1.75 | Maximum 0.75 dB relaxation allowed for each band |
| 8 | n257, n260, n261 | 1.25 | Maximum 0.4 dB relaxation allowed for n260 and 0.75 dB relaxation allowed for all other bands |
| 9 | n258, n260, n261 | 1.25 | Maximum 0.4 dB relaxation allowed for n260 and 0.75 dB relaxation allowed for all other bands |
| 10 | n257, n258, n260, n261 | 1.75 | Maximum 0.4 dB relaxation allowed for n260 and 0.75 dB relaxation allowed for all other bands |
| NOTE 1: MBs is the Multiband Relaxation factor declared by the UE for the tested band in Table A.4.3.9-3 of TS38.508-2 [11]. This declaration shall fulfil the requirements in Table 7.3.2.3.3-1a. NOTE 2: All UE supported bands needs to be tested to ensure the multiband relaxation declaration is compliant NOTE 3: Max allowed sum of MBs over all supported FR2 bands as defined in clause 7.3.2.3.3. |  |  |  |

Table 7.3A.3.5-2c: EIS spherical coverage per component carrier for power class 3 (Rel-16 and forward)

| Operating band | EIS at 50th%ile CCDF (dBm) / Channel bandwidth (NOTE 3) |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n257 | -77.4 +TT+MBs,n+ ΔRIB,S,n | -74.4 +TT+MBs,n+ ΔRIB,S,n | -71.4 +TT+MBs,n+ ΔRIB,S,n | -68.4 +TT+MBs,n+ ΔRIB,S,n |
| n258 | -77.4 +TT+MBs,n+ ΔRIB,S,n | -74.4 +TT+MBs,n+ ΔRIB,S,n | -71.4 +TT+MBs,n+ ΔRIB,S,n | -68.4 +TT+MBs,n+ ΔRIB,S,n |
| n259 | -71.9 +TT+MBs,n+ ΔRIB,S,n | -68.9 +TT+MBs,n+ ΔRIB,S,n | -65.9 +TT+MBs,n+ ΔRIB,S,n | -62.9 +TT+MBs,n+ ΔRIB,S,n |
| n260 | -73.1 +TT+MBs,n+ ΔRIB,S,n | -70.1 +TT+MBs,n+ ΔRIB,S,n | -67.1 +TT+MBs,n+ ΔRIB,S,n | -64.1 +TT+MBs,n+ ΔRIB,S,n |
| n261 | -77.4 +TT+MBs,n+ ΔRIB,S,n | -74.4 +TT+MBs,n+ ΔRIB,S,n | -71.4 +TT+MBs,n+ ΔRIB,S,n | -68.4 +TT+MBs,n+ ΔRIB,S,n |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.4. NOTE 2: The EIS spherical coverage requirements are verified only under normal thermal conditions as defined in TS 38.508-1 [10] subclause 4.1.1. NOTE 3: Refer Table 7.3.4.5-3d for details for MBs allowance corresponding to supported FR2 band set combination |  |  |  |  |

Table 7.3A.3.5-2d: EIS spherical coverage multi-band relaxation factors per component carrier for power class 3 (Rel-16 and forward)

| ID | FR2 bands/set | Comments |
| --- | --- | --- |
| 1 | n257 |  |
| 2 | n258 |  |
| 3 | n259 |  |
| 4 | n260 |  |
| 5 | n261 |  |
| 6 | n257, n261 | MBs,n relaxation is 0 dB |
| 7 | n260, n261 | MBs,n relaxation is 0 dB |
| NOTE 1: MBs,n is the Multiband Relaxation factor for the tested band. This shall fulfil the requirements in Table 7.3.2.3.3-1b. |  |  |

Table 7.3A.3.5-3: Test Tolerance per component carrier (EIS spherical coverage for power class 3)

| Test Metric | f ≤ 40.8 GHz |
| --- | --- |
| IFF (Max device size ≤ 30 cm) | FFS |

<End of Changes>