3GPP TSG-RAN WG5 Meeting #95-e	R5-223396

# Online, , 9th May 2022 - 20th May 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 2993 | rev | 1 | Current version: | 16.11.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR EIEI test case 11.5.10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_EIEI-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | New NR EIEI test case 11.5.10 needs to be added as per workplan. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added test case 11.5.10. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR EIEI WP will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.5.10 (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 38.523-2 CR 0227 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN Impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-223396 is the final version of R5-223267 with below change: 1. Removed Exception after step 24 as there is no need to modify the PDU session in this reject scenario. |  |  |  |  |  |  |  |  |

## <Start of modified section>

### 11.5.10	eCall Only mode / Automatic initiation / Emergency registration / Abnormal case / IMS CN sends 600 (Busy Everywhere) / UE performs eCall in CS domain / UTRAN or GERAN / 5GS

11.5.10.1	Test Purpose (TP)

(1)

with { UE is in state 5GMM-DEREGISTERED.eCALL-INACTIVE and requested to make an automatic eCall over IMS }

ensure that {

when { UE receives 600 Busy Everywhere in response to INVITE }

then { UE establishes the eCall using CS domain (UTRA or GERAN) }

}

11.5.10.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.229, clauses 5.1.6.11.1, 5.1.6.11.2 and TS 23.167, Annex H.6.

[TS 24.229 clause 5.1.6.11.1]

If the upper layers request establishment of an IMS emergency call of the manually initiated eCall type of emergency service, the service URN shall be "urn:service:sos.ecall.manual" as specified in RFC 8147 [244].

If the upper layers request establishment of an IMS emergency call of the automatically initiated eCall type of emergency service, the service URN shall be "urn:service:sos.ecall.automatic" as specified in RFC 8147 [244].

NOTE 1:	The manually initiated eCall type of emergency service is used when the eCall IMS emergency session is invoked with user input. The automatically initiated eCall type of emergency service is used if the eCall IMS emergency session is invoked without user input.

[TS 24.229 clause 5.1.6.11.2]

If the upper layers request establishment of an IMS emergency call of the automatically initiated eCall type of emergency service or of the manually initiated eCall type of emergency service and if allowed by IP-CAN specific annex, the UE shall send an INVITE request as specified in the procedures in subclause 5.1.6.8 with the following additions:

1)	the UE shall set the Request-URI to "urn:service:sos.ecall.automatic" or "urn:service:sos.ecall.manual"; and

2)	if the IP-CAN indicates the eCall support indication, the UE shall:

a)	insert a multipart/mixed body containing an "application/EmergencyCallData.eCall.MSD" MIME body part as defined in RFC 8147 [244], containing the MSD not exceeding 140 bytes and encoded in binary ASN.1 PER as specified in CEN EN 15722:2015 [245] and include a Content-Disposition header field with a "handling" header field parameter with an "optional" value, as described in RFC 3261 [26];

b)	insert an Accept header field indicating the UE is willing to accept an "application/EmergencyCallData.Control+xml" MIME type as defined in RFC 8147 [244]; and

c)	insert a Recv-Info header field set to "EmergencyCallData.eCall.MSD" as defined in RFC 8147 [244].

NOTE:	Further content for the INVITE is as defined in RFC 8147 [244].

Then the UE shall proceed as follows:

…

3)	if the UE receives a 486 (Busy Here), 600 (Busy Everywhere) or 603 (Decline) response to the INVITE request containing:

a)	a multipart/mixed body containing an "application/EmergencyCallData.Control+xml" MIME body part as defined in RFC 8147 [244] with an "ack" element containing:

i)	a "received" attribute set to "true"; and

ii)	a "ref" attribute set to the Content-ID of the MIME body part containing the MSD sent by the UE;

then the UE shall consider the initial MSD transmission as successful and shall perform domain selection to re-attempt the eCall as specified in 3GPP TS 23.167 [4B]; and

4)	in all other cases, the UE shall perform domain selection to re-attempt the eCall as specified in 3GPP TS 23.167 [4B].

[TS 23.167 clause H.6]

Table H.2: Domain Selection Rules for eCall over IMS session attempts for E-UTRAN or NG-RAN radio access networks

|  | PS Available | VoIMS | EMS | ECL | First eCall Attempt | Second eCall Attempt |
| --- | --- | --- | --- | --- | --- | --- |
| A | Y | Y | Y | Y | PS | PS on another PS RAT if available with EMS=Y and ECL=Y or CS if available |
| B | Y | Y | Y | N | CS if available | PS (UE establishes IMS emergency session) |
| C | Y | Y or N | N | N | CS if available | PS on another PS RAT if available with EMS=Y or EMS unknown |
| D | Y | N | Y | Y | PS or CS if available | CS if first attempt in PS PS if first attempt in CS |
| E | Y | N | Y | N | CS if available | PS (UE establishes IMS emergency session) |
| F | N |  | - | - | CS if available |  |
| VoIMS = Voice over IMS over PS sessions support as indicated by IMS Voice over PS session supported indication as defined in TS 23.401 [28] and TS 23.502 [49]. EMS = IMS Emergency Services supported as indicated by Emergency Service Support indicator as defined in TS 23.401 [28] and TS 23.501 [48] and TS 23.502 [49]. ECL = eCall Over IMS support as indicated by the eCall support indicator defined in TS 23.401 [28] and TS 23.501 [48]. NOTE 1: As an implementation option, when the first attempt uses PS and fails for reasons other than related to IMS, the second attempt may use PS with a different 3GPP RAT. In this case the UE, can make a third attempt using CS. |  |  |  |  |  |  |

11.5.10.3	Test description

11.5.10.3.1	Pre-test conditions

System Simulator:

-	2 cells

-	NR Cell 1 as defined in TS 38.508-1 [4] Table 4.4.2-3.

-	if px_NR_RATComb_Tested = NR_UTRA, UTRA Cell 5 as defined in TS 36.508 [7].

-	cell 5 is configured as ''Suitable Neighbour cell''.

-	if px_NR_RATComb_Tested = NR_GERAN, GERAN cell 24 as defined in TS 36.508 [7].

-	cell 24 is configured as ''Suitable Neighbour cell''.

UE:

-	the eCall capable UE is equipped with ‘eCall only’ enabled USIM configured as per TS 38.508-1 [4] Table 6.4.1-24.

Preamble:

-	 The UE is in test state 0-A (Switched Off) as defined in TS 38.508-1 [4], subclause 4.4A.2.

11.5.10.3.2	Test procedure sequence

Table 11.5.10.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE is switched on. | - | - | - | - |
| 2 | Wait 60s for the UE to enter 5GMM-DEREGISTERED.eCALL-INACTIVE state. | - | - | - | - |
| 3 | An automatic eCall is initiated. (Note 1) | - | - | - | - |
| 4-21 | Steps 2 to 19 of generic procedure specified in Table 4.5.2.2-2 of 38.508-1 [4] takes place. | - | - | - | - |
| 22-24 | Steps 8-10 of generic procedure specified in Table 4.9.11.2.2-1 of TS 38.508-1 [4] with condition ‘eCall’ are performed. | - | - | - | - |
| 25 | Step 1 of Annex A.23 of TS 34.229-5 [41] happens | --> | INVITE | - | - |
| 26 | SS sends 600 Busy Everywhere response | <-- | 600 Busy Everywhere | - | - |
| - | EXCEPTION: Steps 27a1 to 27b20 describe behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that takes place if a capability is supported. | - | - | - | - |
| 27a1 | IF (px_NR_RATComb_Tested = NR_UTRA), Check: Does the UE transmit an RRC CONNECTION REQUEST message on Cell 5 with establishment cause set to Emergency Call? | --> | RRC CONNECTION REQUEST | 1 | P |
| 27a2 | The SS transmits an RRC CONNECTION SETUP message. | <-- | RRC CONNECTION SETUP | - | - |
| 27a3 | The UE transmits an RRC CONNECTION SETUP COMPLETE message. | --> | RRC CONNECTION SETUP COMPLETE | - | - |
| 27a4 | Check: Does the UE transmit a CM SERVICE REQUEST with CM service type IE indicating “Emergency call establishment”? | --> | CM SERVICE REQUEST | 1 | P |
| 27a5 | The SS transmits an AUTHENTICATION REQUEST. | <-- | AUTHENTICATION REQUEST | - | - |
| 27a6 | The UE transmits AUTHENTICATION RESPONSE. | --> | AUTHENTICATION RESPONSE | - | - |
| 27a7 | The SS transmits a SECURITY MODE COMMAND message for the CS domain. | <-- | SECURITY MODE COMMAND | - | - |
| 27a8 | The UE transmits a SECURITY MODE COMPLETE message. | --> | SECURITY MODE COMPLETE | - | - |
| 27a9 | Check: Does the UE transmit an EMERGENCY SETUP message with Emergency Service Category IE bit 7 set to 1 and all other bits are set to 0? | --> | EMERGENCY SETUP | 1 | P |
| 27a10-27a15 | Steps 11 to 16 of the generic test procedure in TS 34.108 subclause 7.2.3.2.3 are performed on Cell 5. NOTE: the CS call setup is completed. | - | - | - | - |
| 27a16 | Traffic channel is kept active for at least 5 seconds. | - | - | - | - |
| 27a17 | The SS transmits DISCONNECT. | <-- | DISCONNECT | - | - |
| 27a18 | The UE transmits RELEASE. | --> | RELEASE | - | - |
| 27a19 | The SS transmits RELEASE COMPLETE. | <-- | RELEASE COMPLETE | - | - |
| 27a20 | The SS transmits an RRC CONNECTION RELEASE message. | <-- | RRC CONNECTION RELEASE | - | - |
| 27a21 | The UE transmits RRC CONNECTION RELEASE COMPLETE. | --> | RRC CONNECTION RELEASE COMPLETE | - | - |
| 27b1 | IF (px_NR_RATComb_Tested = NR_GERAN), Check: Does the UE transmit a CHANNEL REQUEST message on Cell 24 with Establishment cause: Emergency call? | --> | CHANNEL REQUEST | 1 | P |
| 27b2 | The SS transmits an IMMEDIATE ASSIGNMENT message. | <-- | IMMEDIATE ASSIGNMENT | - | - |
| 27b3 | Check: Does the UE transmit a CM SERVICE REQUEST with CM service type IE indicating “Emergency call establishment”? | --> | CM SERVICE REQUEST | 1 | P |
| 27b4 | The SS transmits an AUTHENTICATION REQUEST message. | <-- | AUTHENTICATION REQUEST | - | - |
| 27b5 | The UE transmits an AUTHENTICATION RESPONSE message. | --> | AUTHENTICATION RESPONSE | - | - |
| 27b6 | The SS transmits a CIPHERING MODE COMMAND. | <-- | CIPHERING MODE COMMAND | - | - |
| 27b7 | The UE transmits a CIPHERING MODE COMPLETE. | --> | CIPHERING MODE COMPLETE | - | - |
| 27b8 | Check: Does the UE transmit an EMERGENCY SETUP message with Emergency Service Category IE bit 7 set to 1 and all other bits are set to 0? | --> | EMERGENCY SETUP | 1 | P |
| 27b9-27b15 | Steps 11 to 17 of the generic test procedure in TS 51.010-1 subclause 10.2.3 are performed on Cell 24. NOTE: the CS call setup is completed. | - | - | - | - |
| 27b16 | Traffic channel is kept active for at least 5 seconds. | - | - | - | - |
| 27b17 | The SS transmits DISCONNECT. | <-- | DISCONNECT | - | - |
| 27b18 | The UE transmits RELEASE. | --> | RELEASE | - | - |
| 27b19 | The SS transmits RELEASE COMPLETE. | <-- | RELEASE COMPLETE | - | - |
| 26b20 | The SS transmits CHANNEL RELEASE | <-- | CHANNEL RELEASE | - | - |
| Note 1: The request to originate an automatic eCall may be performed by MMI or AT command. |  |  |  |  |  |

11.5.10.3.3	Specific message contents

Table 11.5.10.3.3-1: SIB1 for NR Cell 1 (All steps, Table 11.5.10.3.2-1)

Derivation path: TS 38.508-1 [4] table 4.6.1-28 Condition eCalloverIMSforNR

Table 11.5.10.3.3-2: INVITE (step 25, Table 11.5.10.3.2-1)

Derivation path: TS 34.299-5 [41] Step 1 in Annex A.23 Condition A21

Table 11.5.10.3.3-3: 600 Busy Everywhere (step 26, Table 11.5.10.3.2-1)

Derivation path: TS 34.229-1 [35] Annex A.2.22 Condition A1

Table 11.5.10.3.3-4: RRC CONNECTION REQUEST (Step 27a1, Table 11.5.10.3.2-1)

| Derivation Path: TS 34.108 clause 9.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Establishment cause | Emergency Call |  |  |

Table 11.5.10.3.3-5: CM SERVICE REQUEST (Steps 27a4 and 27b3, Table 11.5.10.3.2-1)

| Derivation Path: TS 24.008[43] Table 9.2.11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CM service type | 0010 | Emergency call establishment |  |

Table 11.5.10.3.3-6: CHANNEL REQUEST (Step 27b1, Table 11.5.10.3.2-1)

| Derivation Path: TS 44.018 Table 9.1.8.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Establishment cause | 101 | Emergency call |  |

## <End of modifed section>