3GPP TSG-RAN5 Meeting #95-e	R5-222460

# Online, , 9th May 2022 - 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2312 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to REGISTRATION ACCEPT message |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNS_Ph2-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To add Rel-17 eNS Ph2 related IE in REGISTRATION ACCEPT message |  |  |  |  |  |  |  |  |
| Summary of change: |  | - The REGISTRATION ACCEPT message has been updated - IE Extended rejected NSSAI has been added - IE NSSRG information has been added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification is still incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section>

–	Registration accept

Table 4.7.1-7: REGISTRATION ACCEPT

| Derivation Path: 24.501 clause 8.2.7 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0111 1110’B | 5GS mobility management messages |  |
| Security header type |  | ’0000’B | Plain 5GS NAS message, not security protected |  |
| Spare half octet |  | '0000'B |  |  |
| Registration accept message identity |  | ‘0100 0010’B |  |  |
| 5GS registration result |  |  |  |  |
| 5GS registration result value |  | ‘001’B | 3GPP access |  |
| SMS allowed |  | ‘0’B | SMS over NAS not allowed |  |
| 5G-GUTI |  | See Table 4.4.2-3 | For 5GC NAS test cases see Table 6.3.2.2-1 |  |
| Equivalent PLMNs |  | Not Present |  |  |
| TAI list |  |  |  |  |
| Length of tracking area identity list contents |  | '0000 0111'B | 7 octets |  |
| Partial tracking area identity list 1 |  |  |  |  |
| Number of elements |  | '0 0000'B | 1 element |  |
| Type of list |  | '00'B | list of TACs belonging to one PLMN, with non-consecutive TAC values |  |
| MCC |  | See Table 4.4.2-3 | For 5GC NAS test cases see Table 6.3.2.2-1 |  |
| MNC |  | See Table 4.4.2-3 | For 5GC NAS test cases see Table 6.3.2.2-1 |  |
| TAC 1 |  | See Table 4.4.2-3 | For 5GC NAS test cases see Table 6.3.2.2-1 |  |
| Allowed NSSAI |  |  |  |  |
| Length of NSSAI contents |  | 4 entries | Equal to the number of S-NSSAI values included |  |
| S-NSSAI |  |  |  |  |
| Length of S-NSSAI contents |  | ‘0000 0001’B | SST |  |
| SST |  | ‘0000 0001’B | SST value 1 (eMBB) |  |
| S-NSSAI |  |  |  |  |
| Length of S-NSSAI contents |  | ‘0000 0001’B | SST |  |
| SST |  | ‘0000 0010’B | SST value 2 (URLLC) |  |
| S-NSSAI |  |  |  |  |
| Length of S-NSSAI contents |  | ‘0000 0001’B | SST |  |
| SST |  | ‘0000 0011’B | SST value 3 (MIoT) |  |
| S-NSSAI |  |  |  |  |
| Length of S-NSSAI contents |  | ‘0000 0001’B | SST |  |
| SST |  | ‘0000 0100’B | SST value 4 (V2X) |  |
| Rejected NSSAI |  | Not Present |  |  |
| Configured NSSAI |  | Not Present |  |  |
| 5GS network feature support |  | ‘0000 1101 0000 0000 0000 0000’B | IMS voice over PS session supported over 3GPP access, Emergency services supported in NR connected to 5GCN and E-UTRA connected to 5GCN. All other features set to "not supported" including the 'Interworking without N26 interface not supported'. |  |
| 5GS network feature support |  | Not Present |  | SST_URLLC OR SST_V2X OR SST_MIOT |
| PDU session status |  | The same value as the PDU session status IEof the most recently received REGISTRATION REQUEST message |  |  |
| PDU session reactivation result |  | Not Present |  |  |
| PDU session reactivation result error cause |  | Not Present |  |  |
| LADN information |  | Not Present |  |  |
| MICO indication |  | Not Present |  |  |
| Network slicing indication |  | Not Present |  |  |
| Service area list |  | Not Present |  |  |
| T3512 value |  |  |  | INITIAL |
| Timer value |  | '0 0000'B |  |  |
| Unit |  | '111'B | value indicates that the timer is deactivated |  |
| T3512 value |  | Not Present |  |  |
| Non-3GPP de-registration timer value |  | Not Present |  |  |
| T3502 value |  | Not Present |  |  |
| Emergency number list |  | Not Present |  |  |
| Extended emergency number list |  | Not Present |  |  |
| SOR Transparent container |  | Not Present |  |  |
| EAP message |  | Not Present |  |  |
| NSSAI inclusion mode |  | Not Present |  |  |
| Operator-defined access category definitions |  | Not Present |  |  |
| Negotiated DRX parameters |  | Not Present |  |  |
| Non-3GPP NW policies |  | Not Present |  |  |
| EPS bearer context status |  | The same value as the EPS bearer context status IE of the most recently received REGISTRATION REQUEST message |  |  |
| Negotiated extended DRX parameters |  | Not Present |  |  |
| T3447 value |  | Not Present |  |  |
| T3448 value |  | Not Present |  |  |
| T3324 value |  | Not Present |  |  |
| UE radio capability ID |  | Not Present |  |  |
| UE radio capability ID |  | The same value as received in UE radio capability ID; if any of the REGISTRATION REQUEST message |  | pc_5GC_RACS |
| UE radio capability ID deletion indication |  | Not Present |  |  |
| Pending NSSAI |  | Not Present |  |  |
| Ciphering key data |  | Not Present |  |  |
| CAG information list |  | Not Present |  |  |
| Truncated 5G-S-TMSI configuration |  | Not Present |  |  |
| Negotiated WUS assistance information |  | Not Present |  |  |
| Negotiated NB-N1 mode DRX parameters |  | Not Present |  |  |
| Extended rejected NSSAI |  | Not Present |  |  |
| NSSRG information |  | Not Present |  |  |

| Condition | Explanation |
| --- | --- |
| INITIAL | Initial registration |

NOTE:	This message is always sent within SECURITY PROTECTED 5GS NAS MESSAGE message.

<End of modified section>