3GPP TSG-RAN5 Meeting #95-e	R5-222434

# Electronic Meeeting, 9th – 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 5404 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of missing RMC R.86 TDD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Rohde & Schwarz |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test |  |  |  |  |  | Date: |  |  | 2022-04-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | RMC R.86 TDD is required for several TCs, however the CR has not been added to the Annex A yet. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add RMC R.86 to table A.3.4.3.2-1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Required RMC remains missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3.4.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

#### A.3.4.3.2	Two antenna ports (Cell Specific)

The reference measurement channels in Table A.3.4.3.2-1 apply for verifying demodulation performance for CDM-multiplexed UE specific reference symbols with two cell-specific antenna ports.

Table A.3.4.3.2-1: Fixed Reference Channel for CDM-multiplexed DM RS

| Parameter | Unit | Value |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.31 TDD |  | R.32 TDD |  | R.32-1 TDD |  | R.33 TDD |  | R.33-1 TDD |  | R.34 TDD |  | R.86 TDD |  |
| Channel bandwidth | MHz | 10 |  | 10 |  | 5 |  | 10 |  | 10 |  | 10 |  | 10 |  |
| Allocated resource blocks |  | 50 4 |  | 50 4 |  | 25 4 |  | 50 4 |  | 18 6 |  | 50 4 |  | 50 4 |  |
| Uplink-Downlink Configuration (Note 3) |  | 1 |  | 1 |  | 1 |  | 1 |  | 1 |  | 1 |  | 1 |  |
| Allocated subframes per Radio Frame (D+S) |  | 3+2 |  | 3+2 |  | 3+2 |  | 3+2 |  | 3+2 |  | 3+2 |  | 3+2 |  |
| Modulation |  | QPSK |  | 16QAM |  | 16QAM |  | 64QAM |  | 64QAM |  | 64QAM |  | QPSK |  |
| Target Coding Rate |  | 1/3 |  | 1/2 |  | 1/2 |  | 3/4 |  | 3/4 |  | 1/2 |  | 1/3 |  |
| Information Bit Payload |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For Sub-Frames 4,9 | Bits | 3624 |  | 11448 |  | 5736 |  | 27376 |  | 9528 |  | 18336 |  | 4392 |  |
| For Sub-Frames 1,6 |  | 2664 |  | 7736 |  | 3112 |  | 16992 |  | 7480 |  | 11832 |  | 2664 |  |
| For Sub-Frame 5 | Bits | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |
| For Sub-Frame 0 | Bits | 2984 |  | 9528 |  | 3496 |  | 22152 |  | 9528 |  | 14688 |  | 3624 |  |
| Number of Code Blocks per Sub-Frame (Note 5) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For Sub-Frames 4,9 |  | 1 |  | 2 |  | 1 |  | 5 |  | 2 |  | 3 |  | 1 |  |
| For Sub-Frames 1,6 |  | 1 |  | 2 |  | 1 |  | 3 |  | 2 |  | 2 |  | 1 |  |
| For Sub-Frame 5 |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |
| For Sub-Frame 0 |  | 1 |  | 2 |  | 1 |  | 4 |  | 2 |  | 3 |  | 1 |  |
| Binary Channel Bits Per Sub-Frame |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For Sub-Frames 4,9 | Bits | 12000 |  | 24000 |  | 10800 |  | 36000 |  | 12960 |  | 36000 |  | 11400 |  |
| For Sub-Frames 1,6 |  | 7872 |  | 15744 |  | 6528 |  | 23616 |  | 10368 |  | 23616 |  | 7872 |  |
| For Sub-Frame 5 | Bits | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |
| For Sub-Frame 0 | Bits | 9840 |  | 19680 |  | 7344 |  | 29520 |  | 12960 |  | 29520 |  | 9840 |  |
| Max. Throughput averaged over 1 frame | Mbps | 1.556 |  | 4.79 |  | 2.119 |  | 11.089 |  | 4.354 |  | 7.502 |  | 1.7736 |  |
| UE Category |  | ≥ 1 |  | ≥ 2 |  | ≥ 1 |  | ≥ 2 |  | ≥ 1 |  | ≥ 2 |  | 1bis |  |
| Note 1: 2 symbols allocated to PDCCH for 20 MHz, 15 MHz and 10 MHz channel BW; 3 symbols allocated to PDCCH for 5 MHz and 3 MHz; 4 symbols allocated to PDCCH for 1.4 MHz. For subframe 1&6, only 2 OFDM symbols are allocated to PDCCH. Note 2: Reference signal, synchronization signals and PBCH allocated as per TS 36.211 [4]. Note 3: as per Table 4.2-2 in TS 36.211 [4]. Note 4: For R.31, R.32, R.33, R.34 and R.86, 50 resource blocks are allocated in sub-frames 4,9 and 41 resource blocks (RB0–RB20 and RB30–RB49) are allocated in sub-frame 0 and the DwPTS portion of sub-frames 1,6. For R.32-1, 25 resouce blocks are allocated in sub-frames 4,9 and 17 resource blocks (RB0–RB7 and RB16–RB24) are allocated in sub-frame 0 and the DwPTS portion of sub-frames 1, 6. Note 5: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). Note 6: Localized allocation started from RB #0 is applied. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

< End of changes >