3GPP TSG-RAN5 Meeting #95-e	R5-222697

# Electronic Meeting, 9th – 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 5407 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Editorial correction to NB-IoT performance test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Bureau Veritas |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI13_Test, NB_IOT-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The NB-IoT test case in clause 8 have some missing information and shall make it clear. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Corrected to the table format; - Corrected ‘TBD’ information; - Added operating mode information to align same strcuture with different test cases. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NB-IoT test cases will be misleading. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.12.1.1.2.4.3; 8.12.1.1.2.5; 8.12.2.1.1.4.1; 8.12.2.1.2.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## {Start of changes}

##### 8.12.1.1.2	Demodulation of NPDSCH (Cell-Specific Reference Symbols) in standalone and Guard-band mode for category NB1 and NB2

8.12.1.1.2.1	Test purpose

Same test purpose as in clause 8.12.1.1.1.1.

8.12.1.1.2.2	Test applicability

This test applies to all types of NB-IoT FDD UE release 13 and forward of category NB1.

This test applies to all types of NB-IoT FDD UE release 14 and forward of category NB2.

8.12.1.1.2.3	Minimum conformance requirements

The requirements are specified in Table 8.12.1.1.2.3-2, with the addition of the parameters in Table 8.12.1.1.2.3-1 and the downlink physical channel setup according to Annex C.3.2. Using this configuration the fraction of maximum throughput percentage shall meet or exceed the minimum requirements specified in Table 8.12.1.1.2.3-2 for the specified SNR.

Table 8.12.1.1.2.3-1: Test Parameters for NPDSCH under standalone/Guard-band

| Parameter | Unit |  | Test 1, 2 |
| --- | --- | --- | --- |
| at antenna port |  | dBm/15kHz | -93 (Note 1) |
|  |  | dBm/15kHz | -99 (Note 2) |
| NPDCCH repetition number |  | subframe | 32 for Test 1; 256 for Test 2. |
| (npdcch-NumRepetitions-r13) |  | subframe | 64 for Test 1; 512 for Test 2. |
| (nPDCCH-startSF-USS-r13) |  |  | 1.5 |
| Note 1: This noise is applied to all subframes from the end of the NPDCCH to the end of the following NPDSCH transmission; Note 2: This noise is applied to all subframes from the end of the NPDSCH to the end of the following NPDCCH transmission. |  |  |  |

Table 8.12.1.1.2.3-2: Minimum performance under standalone/Guard-band with 1 NRS ports

| Test number | Bandwidth | Carrier Type | Reference Channel | Repetition number | Propagation condition | Number of NRS ports | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 200kHz | Anchor | R.NB.6 FDD | 32 | EPA5 | 1 | 70% | -3.4 | NB1, NB2 |
| 2 (Note 1) | 200kHz | Non-anchor | R.NB.6-1 FDD | 256 | ETU1 | 1 | 70% | -10.2 | NB1, NB2 |
| Note 1: Applicable to UE supporting Non-Anchor mode of operation. |  |  |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 36.101 [2] clause 8.12.1.1.2.

8.12.1.1.2.4	Test description

8.12.1.1.2.4.1	Initial conditions

Same initial conditions as in clause 8.12.1.1.1.4.1 with the following exceptions.

-	Operation mode: Standalone.

-	Instead of Figure A.10 use Figure A.9.

8.12.1.1.2.4.2	Test procedure

1.	SS transmits NPDSCH via NPDCCH DCI format N1 for C_RNTI to transmit the DL RMC according to Tables 8.12.1.1.2.3-1 and 8.12.1.1.2.3-2.

2.	Set the parameters of the bandwidth, MCS, reference channel, the propagation condition, the correlation matrix, antenna configuration and the SNR according to Table 8.12.1.1.2.5-1 as appropriate.

3.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex G clause G.3. Count the number of NACKs, ACKs and statDTXs on the UL during the test interval and decide pass or fail according to Tables G.3.5 and G.3.6 in Annex G clause G.3.

4.	Repeat steps from 1 to 3 for each subtest in Table 8.12.1.1.2.5-1 as appropriate.

8.12.1.1.2.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 8.1.6 with the following exceptions:

Table 8.12.1.1.2.4.3-1: Configure Non-anchor carrier in subtest 2

| Derivation Path: 36.508 Table 8.1.8.2.1.6-1 PhysicalConfigDedicated-NB-DEFAULT |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Information Element |  |  | Value/remark |  | Comment | Condition |
| PhysicalConfigDedicated-NB-DEFAULT ::= SEQUENCE { |  |  |  |  |  |  |
| carrierConfigDedicated-NB-r13 ::= SEQUENCE { |  |  |  |  | Non-anchor carrier |  |
| dl-CarrierConfig-r13::= SEQUENCE { |  |  |  |  |  |  |
| dl-CarrierFreq-r13 |  |  | Note 1 |  |  |  |
| downlinkBitmapNonAnchor-r13 |  |  | NULL |  |  |  |
| dl-GapNonAnchor-r13 |  |  | NULL |  |  |  |
| } |  |  |  |  |  |  |
| ul-CarrierConfig-r13 ::= SEQUENCE { |  |  |  |  |  |  |
| ul-CarrierFreq-r13 |  |  | Note 1 |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| npdcch-ConfigDedicated-r13 |  |  | NPDCCH-ConfigDedicated-NB-DEFAULT |  | See subclause 8.1.6.3 in 36.508 |  |
| npusch-ConfigDedicated-r13 |  |  | NPUSCH-ConfigDedicated-NB-DEFAULT |  | See subclause 8.1.6.3 in 36.508 |  |
| uplinkPowerControlDedicated-r13 |  |  | UplinkPowerControlDedicated-NB-DEFAULT |  | See subclause 8.1.6.3 in 36.508 |  |
| } |  |  |  |  |  |  |
| Note 1: The frequency of Non-anchor carrier is located at 200KHz higher from the centre of the anchor carrier. |  |  |  |  |  |

8.12.1.1.2.5	Test requirement

Table 8.12.1.1.2.3-1 defines the primary level settings.

The fraction of maximum throughput percentage for the downlink reference measurement channels specified in Annex A clause A.3.12.2.1	 for each throughput test shall meet or exceed the specified value in Table 8.12.1.1.2.5-1 for the specified SNR including test tolerances for all throughput tests.

Table 8.12.1.1.2.5-1: Test requirements under standalone with 1 NRS ports

| Test number | Bandwidth | Carrier Type | Reference Channel | Repetition number | Propagation condition | Number of NRS ports | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |
| 1 | 200kHz | Anchor | R.NB.6 FDD | 32 | EPA5 | 1 | 70% | -2.6 |
| 2 (Note 1) | 200kHz | Non-anchor | R.NB.6-1 FDD | 256 | ETU1 | 1 | 70% | -9.4 |
| Note 1: Applicable to UE supporting Non-Anchor mode of operation. |  |  |  |  |  |  |  |  |

<Unchanged Sections Skipped>

##### 8.12.2.1.1	Demodulation of NPDCCH single-antenna performance for category NB1 and NB2

8.12.2.1.1.1	Test purpose

This test verifies the demodulation performance of NPDCCH for single-antenna scenario with a given SNR for which the average probability of miss-detection of the Downlink Scheduling Grant, remains below a given reference value.

8.12.2.1.1.2	Test applicability

This test applies to all types of NB-IoT FDD UE release 13 and forward of category NB1.

This test applies to all types of NB-IoT FDD UE release 14 and forward of category NB2.

8.12.2.1.1.3	Minimum conformance requirements

The purpose of these tests is to verify the probability of miss-detection of the Downlink Scheduling Grant (Pm-dsg). The requirements are specified in Table 8.12.2.1.1.3-1, with the parameters in Table 8.12.2.1-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests is to verify the performance.

Table 8.12.2.1.1.3-1: Minimum performance NPDCCH

| Test number | Deployment mode | Repetition number (R) | Carrier Type | Reference Channel | Propagation Condition | Number of NRS ports | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |  |
| 1 | Stand-alone/Guard-band | 128 | Anchor | R.NB.3 FDD | EPA5 | 1 | 1 | -4.9 | NB1, NB2 |
| 2 (Note 1) | Stand-alone/Guard-band | 1024 | Non-anchor | R.NB.3 FDD | ETU1 | 1 | 1 | -11.4 | NB1, NB2 |
| Note 1: Applicable to UE supporting Non-Anchor mode of operation. |  |  |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 36.101 [2] clause 8.12.2.1.1.

8.12.2.1.1.4	Test description

8.12.2.1.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

Configurations of NPDSCH and NPDCCH before measurement are specified in C.2.

Test Environment: Normal, as defined in TS 36.508 [7] clause 4.1.

Operation mode: Standalone.

Frequencies to be tested: K.2.1.

Channel Bandwidths to be tested: As specified per test number in Table 8.12.2.1.1.3-1.

1.	Connect the SS, the faders and AWGN noise sources to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure A.9 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to Tables 8.12.2.1-1 and 8.12.2.1.1.3-1 as appropriate.

3.	Downlink signals are initially set up according to Annex C.0, C.1 and Annex C.3.2 and uplink signals according to Annex H.0.1, H.1.1, H.2.1 and Annex H.4.2.

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in State 2A-NB with CP CIoT Optimisation according to TS 36.508 [7] clause 8.1.5. Message contents are defined in clause 8.12.2.1.1.4.3.

8.12.2.1.1.4.2	Test procedure

1.	SS transmits NPDSCH via NPDCCH DCI format N1 for C_RNTI to transmit the DL RMC according to Tables 8.12.2.1-1, 8.12.2.1-2 and 8.12.2.1.1.3-1. The details of NPDCCH are specified in Table A.3.13.1. The SS sends downlink MAC padding bits on the DL RMC.

2.	Set the parameters of the bandwidth, MCS, reference channel, the propagation condition and the SNR according to Tables 8.12.2.1.1.5-1 as appropriate.

3.	Measure the Pm-dsg for a duration sufficient to achieve statistical significance according to Annex G clause G.4. Count the number of NACKs, ACKs and statDTXs on the UL NPUSCH during each subtest. Pm-dsg is the radio (statDTX)/(NACK +ACK+statDTX). If Pm-dsg is less than the value specified in table 8.12.2.1.1.5-1, UE pass the subtest. Otherwise fail the UE.

4.	SS release the connection through State 3A-NB and finally ensure the UE is in State 2A-NB with CP CIoT Optimisation according to TS 36.508 [7] clause 8.1.5. Configure the Non-anchor carrier according to table 8.12.2.1.1.4.3-1.

5.	Repeat steps from 1 to 3 for subtest 2 in Table 8.12.2.1.1.5-1 as appropriate.

8.12.2.1.1.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 8.1.6 with the following exceptions:

Table 8.12.2.1.1.4.3-1: Configure Non-anchor carrier in subtest 2

| Derivation Path: 36.508 Table 8.1.8.2.1.6-1 PhysicalConfigDedicated-NB-DEFAULT |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Information Element |  |  | Value/remark |  | Comment | Condition |
| PhysicalConfigDedicated-NB-DEFAULT ::= SEQUENCE { |  |  |  |  |  |  |
| carrierConfigDedicated-NB-r13 ::= SEQUENCE { |  |  |  |  | Non-anchor carrier |  |
| dl-CarrierConfig-r13::= SEQUENCE { |  |  |  |  |  |  |
| dl-CarrierFreq-r13 |  |  | Note 1 |  |  |  |
| downlinkBitmapNonAnchor-r13 |  |  | NULL |  |  |  |
| dl-GapNonAnchor-r13 |  |  | NULL |  |  |  |
| } |  |  |  |  |  |  |
| ul-CarrierConfig-r13 ::= SEQUENCE { |  |  |  |  |  |  |
| ul-CarrierFreq-r13 |  |  | Note 1 |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| npdcch-ConfigDedicated-r13 |  |  | NPDCCH-ConfigDedicated-NB-DEFAULT |  | See subclause 8.1.6.3 in 36.508 |  |
| npusch-ConfigDedicated-r13 |  |  | NPUSCH-ConfigDedicated-NB-DEFAULT |  | See subclause 8.1.6.3 in 36.508 |  |
| uplinkPowerControlDedicated-r13 |  |  | UplinkPowerControlDedicated-NB-DEFAULT |  | See subclause 8.1.6.3 in 36.508 |  |
| } |  |  |  |  |  |
| Note 1: The frequency of Non-anchor carrier is located at 200KHz higher from the centre of the anchor carrier. |  |  |  |  |  |

8.12.2.1.1.5	Test requirement

For the parameters specified in Tables 8.12.2.1-1 and 8.12.2.1.1.3-1 the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 8.12.2.1.1.5-1.

Table 8.12.2.1.1.5-1: Test Parameters for NPDCCH performance

| Test number | Deployment mode | Repetition number (Rmax) | Operated carrier | Reference Channel | Propagation Condition | Number of NRS ports | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | Stand-alone | 128 | Anchor | R.NB.3 FDD | EPA5 | 1 | 1 | -4.1 |
| 2 (Note 1) | Stand-alone | 1024 | Non-anchor | R.NB.3 FDD | ETU1 | 1 | 1 | -10.6 |
| Note 1: Applicable to UE supporting Non-Anchor mode of operation. |  |  |  |  |  |  |  |  |

##### 8.12.2.1.2	Demodulation of NPDCCH Transmit diversity performance for category NB1 and NB2

8.12.2.1.2.1	Test purpose

This test verifies the demodulation performance of NPDCCH for the transmit diversity scenario with a given SNR for which the average probability of misdetection of the Downlink Scheduling Grant, remains below a given reference value.

8.12.2.1.2.2	Test applicability

This test applies to all types of NB-IoT FDD UE release 13 and forward of category NB1.

This test applies to all types of NB-IoT FDD UE release 14 and forward of category NB2.

8.12.2.1.2.3	Minimum conformance requirements

The requirements are specified in Table 8.12.2.1.2.3-1, with the addition of the parameters in Tables 8.12.2.1-1, 8.12.2.1-2 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests is to verify the performance.

Table 8.12.2.1.2.3-1: Minimum performance NPDCCH

| Test number | Deployment mode | Repetition number (R) | Carrier Type | Reference Channel | Propagation Condition | Number of NRS ports | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |  |
| 1 | In-band | 64 | Anchor | R.NB.4 FDD | EPA5 | 2 | 1 | -3.9 | NB1, NB2 |
| 2 (Note 1) | In-band | 512 | Non-anchor | R.NB.4 FDD | ETU1 | 2 | 1 | -10.0 | NB1, NB2 |
| Note 1: Applicable to UE supporting Non-Anchor mode of operation. |  |  |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 36.101 [2] clause 8.12.2.1.2.

8.12.2.1.2.4	Test description

8.12.2.1.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

Configurations of NPDSCH and NPDCCH before measurement are specified in Annex C.2.

Test Environment: Normal, as defined in TS 36.508 [7] clause 4.1.

Operation mode: In-band.

Frequencies to be tested: K.2.1.

Channel Bandwidths to be tested: As specified per test number in Table 8.12.2.1.2.3-1.

EUTRA Bandwidth is 10 MHz, as defined in TS 36.508 [7] clause 4.3.1.1 except for band 31 where 5 MHz bandwidth is used.

1.	Connect the SS, the faders and AWGN noise sources to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure A.10 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to Tables 8.12.2.1-1, 8.12.2.1-2 and 8.12.2.1.2.3-1 as appropriate.

3.	Downlink signals are initially set up according to Annex C.0, C.1 and Annex C.3.2 and uplink signals according to Annex H.0.1, H.1.1, H.2.1 and Annex H.4.2.

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in State 2A-NB with CP CIoT Optimisation according to TS 36.508 [7] clause 8.1.5. Message contents are defined in clause 8.12.2.1.2.4.3.

8.12.2.1.2.4.2	Test procedure

1.	SS transmits NPDSCH via NPDCCH DCI format N1 for C_RNTI to transmit the DL RMC according to Tables 8.12.2.1-1, 8.12.2.1-2 and 8.12.2.1.2.3-1. The details of NPDCCH are specified in Table A.3.13.1. The SS sends downlink MAC padding bits on the DL RMC.

2.	Set the parameters of the bandwidth, MCS, reference channel, the propagation condition and the SNR according to Tables 8.12.2.1.2.5-1 as appropriate.

3.	Measure the Pm-dsg for a duration sufficient to achieve statistical significance according to Annex G clause G.4. Count the number of NACKs, ACKs and statDTXs on the UL NPUCCH during each subtest. Pm-dsg is the radio (statDTX)/(NACK +ACK+statDTX). If Pm-dsg is less than the value specified in table 8.12.2.1.2.5-1, UE pass the subtest. Otherwise fail the UE.

4.	SS release the connection through State 3A-NB and finally ensure the UE is in State 2A-NB with CP CIoT Optimisation according to TS 36.508 [7] clause 8.1.5. Configure the Non-anchor carrier according to table 8.12.2.1.2.4.3-2.

5.	Repeat steps from 1 to 3 for subtest 2 in Table 8.12.2.1.2.5-1 as appropriate.

8.12.2.1.2.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 8.1.6 and clause 8.1.4.3 using condition "Inband_Different" with the following exceptions:

Table 8.12.2.1.2.4.3-1: MasterInformationBlock-NB

| Derivation Path: 36.508 Table 8.1.4.3.2-1: MasterInformationBlock-NB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MasterInformationBlock-NB ::= SEQUENCE { |  |  |  |
| systemFrameNumber-MSB-r13 | A valid value as defined in TS 36.331 [17] |  |  |
| hyperSFN-LSB-r13 | A valid value as defined in TS 36.331 [17] |  |  |
| schedulingInfoSIB1-r13 | 2 | TBS = 208 bits, 16 repetitions |  |
| systemInfoValueTag-r13 | 0 |  |  |
| ab-Enabled-r13 | FALSE |  |  |
| operationModeInfo-r13 CHOICE { |  |  |  |
| inband-DifferentPCI-r13 SEQUENCE { |  |  | Inband_Different |
| eutra-NumCRS-Ports-r13 | four |  |  |
| rasterOffset-r13 | khz-2dot5 |  |  |
| spare | ’00’B |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.12.2.1.2.4.3-2: Configure Non-anchor carrier in subtest 2

| Derivation Path: 36.508 Table 8.1.8.2.1.6-1 PhysicalConfigDedicated-NB-DEFAULT |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Information Element |  |  | Value/remark |  | Comment | Condition |
| PhysicalConfigDedicated-NB-DEFAULT ::= SEQUENCE { |  |  |  |  |  |  |
| carrierConfigDedicated-NB-r13 ::= SEQUENCE { |  |  |  |  | Non-anchor carrier |  |
| dl-CarrierConfig-r13::= SEQUENCE { |  |  |  |  |  |  |
| dl-CarrierFreq-r13 |  |  | Note 1 |  |  |  |
| downlinkBitmapNonAnchor-r13 |  |  | NULL |  |  |  |
| dl-GapNonAnchor-r13 |  |  | NULL |  |  |  |
| } |  |  |  |  |  |  |
| ul-CarrierConfig-r13 ::= SEQUENCE { |  |  |  |  |  |  |
| ul-CarrierFreq-r13 |  |  | Note 1 |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| npdcch-ConfigDedicated-r13 |  |  | NPDCCH-ConfigDedicated-NB-DEFAULT |  | See subclause 8.1.6.3 in 36.508 |  |
| npusch-ConfigDedicated-r13 |  |  | NPUSCH-ConfigDedicated-NB-DEFAULT |  | See subclause 8.1.6.3 in 36.508 |  |
| uplinkPowerControlDedicated-r13 |  |  | UplinkPowerControlDedicated-NB-DEFAULT |  | See subclause 8.1.6.3 in 36.508 |  |
| } |  |  |  |  |  |
| Note 1: The frequency of the non-anchor carrier is 180KHz higher than the centre of the anchor carrier. |  |  |  |  |  |

8.12.2.1.2.5	Test requirement

For the parameters specified in Tables 8.12.2.1-1, 8.12.2.1-2 and 8.12.2.1.2.3-1 the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 8.12.2.1.2.5-1.

Table 8.12.2.1.2.5-1: Test Parameters for NPDCCH performance

| Test number | Deployment mode | Repetition number (Rmax) | Operated carrier | Reference Channel | Propagation Condition | Number of NRS ports | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | In-band | 64 | Anchor | R.NB.4 FDD | EPA5 | 2 | 1 | -3.0 |
| 2 (Note 1) | In-band | 512 | Non-anchor | R.NB.4 FDD | ETU1 | 2 | 1 | -9.1 |
| Note 1: Applicable to UE supporting Non-Anchor mode of operation. |  |  |  |  |  |  |  |  |

## {End of changes}