3GPP TSG-RAN5 Meeting #95-e	R5-222698

# Electronic Meeting, 9th – 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 5408 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to MPR test applicability for category 1bis |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Bureau Veritas |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI13_Test, LTE_UE_cat_1RX-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The MPR test requirement is covered in ACLR test cases and test applicability is incorrectly indicated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Updated test applicability to apply the MPR requirement in TC6.6.2.3EB ACLR for UE category 1bis; - 3GPP stype updated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MPR test requirement will duplicate the tests. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3EB, 6.2.3EB.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS36.521-2 CR 0982 (R5-222699) |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## {Start of changes}

<Unchanged Sections Skipped>

### 6.2.3EB	Maximum Power Reduction (MPR) for UE category 1bis

#### 6.2.3EB.1	Test purpose

Same test purpose as in clause 6.2.2.

#### 6.2.3EB.2	Test applicability

The requirements of this test apply in test case 6.6.2.3EB Adjacent Channel Leakage power Ratio for UE category 1bis to all types of E-UTRA Power Class 2 and 3 UE release 13 and forward of UE category 1bis.

NOTE:	As a result TC 6.2.3EB has not been included in the test case applicability table 4.1-1, TS 36.521-2. This does not preclude the test from being used for R&D or other purposes if deemed useful.

#### 6.2.3EB.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.2.3.3.

#### 6.2.3EB.4	Test description

##### 6.2.3EB.4.1	Initial conditions

Same initial conditions as in clause 6.2.3.4.1 with the following exceptions:

-	Connect the SS to the UE antenna connectors as shown in TS 36.508[7] Annex A Figure A.3 using only main UE Tx/Rx antenna.

##### 6.2.3EB.4.2	Test procedure

Same test procedure as in clause 6.2.2.4.2 for Power Class 3 UE.

Same test procedure as in clause 6.2.2_1.4 for Power Class 2 UE.

##### 6.2.3EB.4.3	Message contents

Same message contents as in clause 6.2.2.4.3 for Power Class 3 UE.

Same message contents as in clause 6.2.2_1.4 for Power Class 2 UE.

#### 6.2.3EB.5	Test requirement

The maximum output power, derived in step 2 of clause 6.2.3EB.4.2, shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2.3EB.5-1.

Table 6.2.3EB.5-1: UE Power Class test requirements

| E-UTRA Band | Class 1 (dBm) | Tol. (Db) | Class 2 (dBm) | Tol. (dB) | Class 3 (dBm) | QPSK partial RB allocation Tol. (dB)) | QPSK full RB allocation Tol. (dB)) | 16QAM partial RB allocation Tol. (dB) | 16QAM full RB allocation Tol. (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  | 23 | ±2.7 | +2.7 / -3.7 | +2.7 / -3.7 | +2.7 / -4.7 |
| 2 |  |  |  |  | 23 | ±2.71 | +2.7 / 1,2 -3.7 | +2.7 / 1,2 -3.7 | +2.7 / 1,2 -4.7 |
| 3 |  |  |  |  | 23 | ±2.71 | +2.7 / 1,2 -3.7 | +2.7 / 1,2 -3.7 | +2.7 / 1,2 -4.7 |
| 4 |  |  |  |  | 23 | ±2.7 | +2.7 / -3.7 | +2.7 / -3.7 | +2.7 / -4.7 |
| 5 |  |  |  |  | 23 | ±2.7 | +2.7 / -3.7 | +2.7 / -3.7 | +2.7 / -4.7 |
| 7 |  |  |  |  | 23 | ±2.71 | +2.7 / 1,2 -3.7 | +2.7 / 1,2 -3.7 | +2.7 / 1,2 -4.7 |
| 8 |  |  |  |  | 23 | ±2.71 | +2.7 / 1,2 -3.7 | +2.7 / 1,2 -3.7 | +2.7 / 1,2 -4.7 |
| 12 |  |  |  |  | 23 | ±2.71 | +2.7 / 1,2 -3.7 | +2.7 / 1,2 -3.7 | +2.7 / 1,2 -4.7 |
| 13 |  |  |  |  | 23 | ±2.7 | +2.7 / -3.7 | +2.7 / -3.7 | +2.7 / -4.7 |
| 18 |  |  |  |  | 23 | ±2.73 | +2.7 / 3 -3.7 | +2.7 / 3 -3.7 | +2.7 / 3 -4.7 |
| 20 |  |  |  |  | 23 | ±2.71 | +2.7 / 1,2 -3.7 | +2.7 / 1,2 -3.7 | +2.7 / 1,2 -4.7 |
| 26 |  |  |  |  | 23 | ±2.71 | +2.7 / 1,2 -3.7 | +2.7 / 1,2 -3.7 | +2.7 / 1,2 -4.7 |
| 28 |  |  |  |  | 23 | +2.7/-3.2 | +2.7/-4.2 | +2.7/-4.2 | +2.7/-5.2 |
| 31 |  |  |  |  | 23 | ±2.7 | +2.7 / -3.7 | +2.7 / -3.7 | +2.7 / -4.7 |
| 34 |  |  |  |  | 23 | ±2.7 | +2.7 / -3.7 | +2.7 / -3.7 | +2.7 / -4.7 |
| 39 |  |  |  |  | 23 | ±2.7 | +2.7 / -3.7 | +2.7 / -3.7 | +2.7 / -4.7 |
| 40 |  |  |  |  | 23 | ±2.7 | +2.7 / -3.7 | +2.7 / -3.7 | +2.7 / -4.7 |
| 41 |  |  | 26 | ±2.71 | 23 | ±2.71 | +2.7 / 1,2 -3.7 | +2.7 / 1,2 -3.7 | +2.7 / 1,2 -4.7 |
| 66 |  |  |  |  | 23 | ±2.7 | +2.7 / -3.7 | +2.7 / -3.7 | +2.7 / -4.7 |
| 72 |  |  |  |  | 23 | ±2.7 | +2.7 / -3.7 | +2.7 / -3.7 | +2.7 / -4.7 |
| Note 1: For transmission bandwidths (Figure 5.4.2-1) confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. Note 2: For the UE maximum output power modified by MPR, the power limits specified in Table 6.2.5.3-1 apply Note 3: For a UE that supports both Band 18 and Band 26, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5dB for transmission bandwidths confined within 815 MHz and 818 MHz |  |  |  |  |  |  |  |  |  |

## {End of changes}