3GPP TSG-RAN5 Meeting #95-e	R5-222388

# Online, , 9th May 2022 - 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4673 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Routine maintenance for TS 36.523-3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI13_Test, NB_IOT-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Clause 7A.1 specifies the derivation of the LTE PCI from the NB-IoT PCI in case of ‘in-band different PCI’ operation mode. However this derivation calculation does not comply with TS 36.211 clause 10.2.6.2, which specifies that the LTE PCI shall be modulo 6 from the NB-IoT PCI for the case of ‘in-band different PCI’ operation mode: If the higher layer does not indicate UE may assume that is equal to , UE may assume - the number of antenna port for the cell-specific reference signals as defined in clause 6.10.1 is obtained from the higher layer parameter eutra-NumCRS-Ports, - the cell-specific reference signals are available in all subframes where the narrowband reference signals are available, and the cell-specific frequency shift for cell-specific reference signals as defined in clause 6.10.1.2 is given by . |  |  |  |  |  |  |  |  |
| Summary of change: |  | The derivation calculation in clause 7A.1 has been modified to comply with the TS 36.211 clause 10.2.6.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail NB-IoT test cases when executed in the ‘in-band different PCI’ operation mode. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7A.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | See also RAN5 TTCN Workshop#57 contribution R5w220105. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# 7A	NB-IoT Test Methods and Design Considerations

NOTE:	Unless explicitly stated otherwise, the present clause and its subclauses apply to the NB-IoT anchor carrier.

## 7A.1	Physical signals and channels

For Signalling testing, single Tx antenna shall be used by the SS, unless specified otherwise in a test case.

The SS shall transmit all NB-IoT physical signals and channels according to TS 36.211 [35], with the following additional requirements, unless specified otherwise in a test case:

-	The NRS shall be transmitted on one antenna port.

-	The NPSS and NSSS shall be transmitted on one and the same antenna port.

-	The NWUS shall be transmitted on one and the same antenna port, which is different from the antenna port where NPSS and NSSS shall be transmitted.

No (WB-E-UTRA) LTE signals and channels shall be transmitted by the SS, except for the case of ‘in-band same PCI’ operation mode where the SS shall transmit the LTE CRS. For the in-band operation modes:

-	TTCN will provide the necessary LTE CRS configuration to the SS ; cyclic prefix (CP) is normal.

-	For the case of ‘in-band different PCI’ operation mode:

-	 The SS shall use this information to skip the resource elements, which should be occupied by the LTE CRS, when transmitting NB-IoT downlink physical channels;

-	The LTE physical cell identity  is calculated from the NB-IoT physical cell identity  as follows:

= ( + 6) MOD 504

For NWUS generation as per TS 36.211 [35] clause 10.2.6B, TTCN provides SS with:

-	time offset between the end of NWUS and start of the first PO in frames, used to calculate  needed for generation of ,

-	with the actual duration of the NWUS signal in subframes,

-	and optionally with the power offset of NWUS relative to NRS in dB.

Actual transmission of the NWUS signal is triggered by the system request at the activation time calculated by TTCN.