3GPP TSG-RAN5 Meeting #95-e	R5-222566

# Electronic Meeting, 9th May 2022 – 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2325 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Editorial update IE FreqBandList |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The connectivities are not fully aligned with TS 38.331. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The connectivities has been clarified. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The RAN5 used RRC terminology might be slightly ambiguous. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of first modified section>

#### –	FreqBandList

Table 4.6.4-21: FreqBandList

| Derivation Path: TS 38.331 [6], clause 6.3.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| FreqBandList::= SEQUENCE (SIZE (1..maxBandsMRDC)) OF FreqBandInformation { | Number of entries depends on the conditions |  |  |
| FreqBandInformation[1] CHOICE { |  | entry 1 | EN-DC, NE-DC |
| bandInformationEUTRA SEQUENCE { |  |  |  |
| bandEUTRA | FreqBandIndicatorEUTRA |  |  |
| ca-BandwidthClassDL-EUTRA | Not checked |  |  |
| ca-BandwidthClassUL-EUTRA | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| FreqBandInformation[2] CHOICE { |  | entry 2 |  |
| bandInformationNR SEQUENCE { |  |  |  |
| bandNR | FreqBandIndicatorNR |  |  |
| maxBandwidthRequestedDL | Not checked |  |  |
| maxBandwidthRequestedUL | Not checked |  |  |
| maxCarriersRequestedDL | Not checked |  |  |
| maxCarriersRequestedUL | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| FreqBandInformation[3] CHOICE { |  | entry 3 | EN-DC AND CA-InterBand |
| bandInformationNR SEQUENCE { |  |  |  |
| bandNR | FreqBandIndicatorNR with condition CA-InterBand |  |  |
| maxBandwidthRequestedDL | Not checked |  |  |
| maxBandwidthRequestedUL | Not checked |  |  |
| maxCarriersRequestedDL | Not checked |  |  |
| maxCarriersRequestedUL | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| FreqBandInformation[1] CHOICE { |  | entry 1 | NR |
| bandInformationNR SEQUENCE { |  |  |  |
| bandNR | FreqBandIndicatorNR |  |  |
| maxBandwidthRequestedDL | Not checked |  |  |
| maxBandwidthRequestedUL | Not checked |  |  |
| maxCarriersRequestedDL | Not checked |  |  |
| maxCarriersRequestedUL | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| FreqBandInformation[2] CHOICE { |  | entry 2 | NR AND CA-InterBand |
| bandInformationNR SEQUENCE { |  |  |  |
| bandNR | FreqBandIndicatorNR with condition CA-InterBand |  |  |
| maxBandwidthRequestedDL | Not checked |  |  |
| maxBandwidthRequestedUL | Not checked |  |  |
| maxCarriersRequestedDL | Not checked |  |  |
| maxCarriersRequestedUL | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| FreqBandInformation[2] CHOICE { |  | entry 2 | NR-DC |
| bandInformationNR SEQUENCE { |  |  |  |
| bandNR | FreqBandIndicatorNR with condition NR-DC-SecondaryBand |  |  |
| maxBandwidthRequestedDL | Not checked |  |  |
| maxBandwidthRequestedUL | Not checked |  |  |
| maxCarriersRequestedDL | Not checked |  |  |
| maxCarriersRequestedUL | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| EN-DC | E-UTRA-NR Dual Connectivity with E-UTRA connected to EPC |
| CA-InterBand | Used in NR CA Inter-band test cases |
| NR | NG-RAN NR Radio Access |
| NR-DC | NR-NR Dual Connectivity |
| NE-DC | NR E-UTRA Dual Connectivity |

<End of modified section>