3GPP TSG-RAN5 Meeting #95-e	R5-222836

# Electronic Meeting, May 9 - 20, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2354 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of Annex C for calculation of SSB and CORESET#0 for PCells |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The current description of CORESET#0 signalling parameters does not reflect that CORESET#0 is only applicable for FR1 to SCS=15 kHZ and SCS=30 kHz, and for FR2 to SCS=60 kHz and SCS=120 kHz. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added clarification to clause C.0, C.1, C.2.4.2.1, C.2.4.3.2 and C.3.2 that calculation of CORESET#0 parameters are only applicable to FR1 carriers wih SCS=15 kHz or SCS=30 kHz, and to FR2 carriers with SCS=60 kHz or SCS=120 kHz. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Limitation of chapter C.3.2 not clear that it is limited to FR1 carriers wih SCS=15 kHz or SCS=30 kHz, and to FR2 carriers with SCS=60 kHz or SCS=120 kHz. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | C.0, C.1, C.2.4.2.1, C.2.4.3.2, C.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Annex C (informative):
Calculation of test frequencies

# C.0	General

Test frequencies are defined in clause 4.3.1 with extensions for signalling test cases in clause 6.2.3. This annex gives a guideline to determine these test frequencies and the associated signalling parameters for a given NR band, NR CA or NR DC band combination.

Clause C.1 describes definitions and parameters used by the procedures to determine test frequencies, SS/PBCH Block (SSB) and CORESET#0 configuration parameters.

Clause C.2 describes how to calculate test frequencies for symmetric NR bands, asymmetric NR bands, NR CA and NR DC configurations.

Clause C.3.2 describes how to determine the SSB, CORESET#0 and signalling parameters for FR1 carriers with SCS=15 kHz or SCS=30 kHz, and for FR2 carriers with SCS=60 kHz or SCS=120 kHz. CORESET#0 is required for a carrier to be used as a PCell.

Clause C.3.3 describes how to determine the SSB and signalling parameters for a carriers without CORESET#0.

Clause C.5 describes how to calculate test frequencies for NR V2X bands.

# C.1	Definitions and Parameters

Figure C.1-1 shows SSB and CORESET#0 and related parameters. CORESET#0 is required for a carrier to be used as a PCell.

Figure C.1-1: location of SSB and CORESET#0 within a channel

The parameters referenced in figure C.1-1 are defined in Table C.1-1.

Table C.1-1: Definition of parameters in Figure C.1-1 used in Annex C.

| Parameter | Description |
| --- | --- |
| FPointA | Reference Point A frequency. |
| Fcarrier | Fcarrier is the centre frequency of a carrier corresponding to its NR-ARFCN value. |
| FcarrierLow | FcarrierLow is the centre frequency of lowest subcarrier of the carrier. FcarrierLow = Fcarrier - 12 * SCSCarrier * (NRB / 2) with NRB according to Table 5.3.2-1 of TS 38.101-1 [7] and TS 38.101-2 [8] for the channel bandwidth of the carrier. |
| ΔFcarrierBandwidth | ΔFcarrierBandwidth is the carrier’s channel bandwidth as provided in carrierBandwidth to the UE (SCS-SpecificCarrier). |
| ΔFoffsetToCarrier | ΔFoffsetToCarrier is the frequency offset between Point A and the lower edge of the carrier. FoffsetToCarrier = offsetToCarrier * PRB size, where PRB size according to the subcarrier spacing of the carrier. offsetToCarrier is signalled to the UE (SCS-SpecificCarrier). |
| FSSref | Centre frequency of SSB. For a cell selectable as PCell the FSSref corresponds to a valid GSCN value according to clause 5.4.3.1 of TS 38.101-1 [7] and TS 38.101-2 [8]. |
| ΔFOffset(RBs) | ΔFOffset(RBs) = 12* Offset(RBs) * subCarrierSpacingCommon, where Offset(RBs) is given in tables 13-1 to 13-10 of TS 38.213 [22]. |
| ΔFkSSB | ΔFKSSB = kSSB * subcarrier spacing of SSB (SCSSSB). |
| ΔFOffsetSSB-CORESET0 | Frequency offset between the lowest subcarrier of the SSB and the lowest subcarrier of CORESET#0. ΔFOffsetSSB-CORESET0 = ΔFOffset(RBs) + ΔFkSSB. |
| ΔFOffsetCORESET-0-Carrier | Frequency offset, FOffsetCORESET#0-Carrier, between the lowest subcarrier of CORESET#0 and the lowest subcarrier of the carrier. |
| ΔFOffsetSSB-Carrier | Frequency offset between the lowest subcarrier of the SSB and the lowest subcarrier of the carrier. |
| FCORESET0Low | Centre frequency of subcarrier 0 of CORESET#0. |
| FOffsetToPointA | Frequency of the lowest subcarrier of the lowest resource block, which has the subcarrier spacing provided by the higher-layer parameter subCarrierSpacingCommon and overlaps with the SS/PBCH block used by the UE for initial cell selection, expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2 (TS 38.211 [29] clause 4.4.4.2). |
| ΔFOffsetToPointA | Frequency offset between FOffsetToPointA and point A. ΔFOffsetToPointA = offsetToPointA * {15 kHz for FR1; 60 kHz for FR2} (TS 38.211 [29] clause 4.4.4.2). |

Additional parameters used in this annex are defined in Table C.1-2.

Table C.1-2: Definition of additional parameters used in Annex C.

| kSSB | as defined in TS 38.211 [29] clause 7.4.3.1 |
| --- | --- |
| SCSCarrier | subcarrier spacing for the carrier (SCS-SpecificCarrier): FR1: 15kHz, 30kHz or 60kHz according to TS 38.101-1 [7] Table 5.3.5-1 FR2: 60kHz or 120kHz according to TS 38.101-2 [8] Table 5.3.5-1 |
| SCSSSB | SS/PBCH block subcarrier spacing FR1: 15kHz or 30kHz according to TS 38.101-1 [7] Table 5.4.3.3-1 FR2: 120kHz or 240kHz according to TS 38.101-2 [8] Table 5.4.3.3-1 NOTE: According to the tables in clause 13 of TS 38.213 [22] not all combinations of SCSSSB and SCSCarrier are applicable. |
| SCScommon | Subcarrier spacing for SIB1, Msg.2/4 for initial access, paging and broadcast SI-messages. Provided to the UE in the MIB in IE subCarrierSpacingCommon. |
| PRBsize | Physical Resource Block size of the carrier = 12 * SCSCarrier. |
| CRBsize | Common Resource Block size = 12 * SCScommon. |
| FDL_Low, FUL_Low | Lowest frequency of the downlink and uplink frequency range of the band as defined in clause 5.2 of TS 38.101-1 [7] and TS 38.101-2 [8]. |
| FDL_High, FUL_High | Highest frequency of the downlink and uplink frequency range of the band as defined in clause 5.2 of TS 38.101-1 [7] and TS 38.101-2 [8]. |
| ΔFRaster | Frequency raster of the band as defined in clause 5.4.2.3 of TS 38.101-1 [7] and TS 38.101-2 [8]. |
| BWDL | Bandwidth of downlink frequency range of the band. |
| BWUL | Bandwidth of uplink frequency range of the band. |
| CBWDL | Downlink channel bandwidth (MHz) of the carrier according to Table 5.3.2-1 of TS 38.101-1 [7] and TS 38.101-2 [8]. |
| CBWUL | Uplink channel bandwidth (MHz) of the carrier according to Table 5.3.2-1 of TS 38.101-1 [7] and TS 38.101-2 [8]. |
| FTx-Rx_separation | Default Tx – Rx carrier centre frequency separation of the band as defined in clause 5.4.4 of TS 38.101-1 [7]. For TDD bands FTx-Rx_separation = 0. |
| ΔFTx-Rx_separation | ΔFTx-Rx = ¦ (BWDL – BWUL)/2 ¦ is the deviation to the default Tx-Rx carrier centre frequency separation (FTx-Rx_separation) for FDD FR1 bands supporting asymmetric channel bandwidths as defined in clause 5.3.6 of TS 38.101-1 [7]. |
| BWSSB | Bandwidth of the SSB. BWSSB = 12 * SCSSSB * 20 |
| ΔGSCN, GSCNMIN, GSCNMAX | GSCN step size, GSCN minimum and GSCN maximum values for the NR band according to table 5.4.3.3-1 of TS 38.101-1 [7] and TS 38.101-2 [8] |
| OffsetRBs | Offset (RBs) according to the applicable table 13-1 to 13-10 in TS 38.213 [22] for the given band and {SCSSSB, SCSCarrier} combination of the carrier. |
| OffsetRBs,max | Maximum value for Offset (RBs) according to the applicable table 13-1 to 13-1o in TS 38.213 [22] for the given band and {SCSSSB, SCSCarrier} combination of the carrier limited to the table indexes with number of RBs equal to the minimum value of in the table. |
| OffsetRBs,min | Minimum value for Offset (RBs) according to the applicable table 13-1 to 13-1o in TS 38.213 [22] for the given band and {SCSSSB, SCSCarrier} combination of the carrier limited to the table indexes with number of RBs equal to the minimum value of in the table. |

# C.2	Determination of test frequencies

## C.2.0	General

Test frequencies are determined as:

For symmetric NR bands (supporting same bandwidth in UL and DL):

-	test frequencies for the supported symmetric channel bandwidth combinations are determined as described in clause C.2.1; and

-	the test frequencies for the supported asymmetric channel bandwidth combinations are determined as described in clause C.2.3.

For asymmetric NR bands (supporting different bandwidth in UL and DL):

-	the test frequencies for the supported symmetric channel bandwidth combinations are determined as described in clause C.2.2; and

-	the test frequencies for the supported asymmetric channel bandwidth combinations are determined as described in clause C.2.3.

For NR CA and NR DC:

-	the test frequencies are determined as described in the relevant subclause in C.2.4 depending to the type of configuration.

The carrier test frequencies are determined considering the channel raster according to clause 5.4.2.3 in TS 38.101-1 [7] for FR1 and in TS 38.101-2 [8] for FR2.

## C.2.1	Determination of test frequencies for symmetric NR bands and symmetric uplink and downlink channel bandwidth combinations

### C.2.1.1	Determination of test frequencies for Low-, Mid- and High-Range

Downlink:

| FDL_LowRange = Ceil((FDL_Low + CBWDL/2) / ΔFRaster) * ΔFRaster | C.2.1.1-Eq1 |
| --- | --- |
| FDL_MidRange = Round((FDL_Low + BWDL/2) / ΔFRaster) * ΔFRaster | C.2.1.1-Eq2 |
| FDL_HighRange = Floor((FDL_High - CBWDL/2) / ΔFRaster) * ΔFRaster | C.2.1.1-Eq3 |

FDL_LowRange is rounded up and FDL_HighRange is rounded down to obey to the minimum guard band according to clause 5.3.3 of TS 38.101-1 [7] and TS 38.101-2 [8].

Uplink:

| FUL_LowRange = FDL_LowRange - FTx-Rx_separation | C.2.1.1-Eq4 |
| --- | --- |
| FUL_MidRange = FDL_MidRange - FTx-Rx_separation | C.2.1.1-Eq5 |
| FUL_HighRange = FDL_HighRange - FTx-Rx_separation | C.2.1.1-Eq6 |

### C.2.1.2	Determination test frequencies for of Mid-Low and Mid-High-Range for signalling tests

| FMid-LowRange = Round((FLowRange+ (FHighRange - FLowRange )/3) / ΔFRaster) * ΔFRaster | C.2.1.2-Eq1 |
| --- | --- |
| FMid-HighRange = Round((FLowRange + 2*(FHighRange - FLowRange )/3) / ΔFRaster) * ΔFRaster | C.2.1.2-Eq2 |

## C.2.2	Determination of test frequencies for asymmetric NR bands and symmetric uplink and downlink channel bandwidth combinations

Determination of test frequencies for asymmetric NR bands, and symmetric uplink and downlink channel bandwidth combinations are determined using the procedure in clause C.2.3 with ΔFTx-Rx = 0.

## C.2.3	Determination of test frequencies for bands supporting asymmetric channel bandwidth combinations

### C.2.3.1	General

The following procedure is used to calculate test frequencies for NR bands supporting asymmetric UL and DL channel bandwidths as described below, where CBWUL and CBWDL refer to the carrier’s UL and DL channel bandwidths; and BWUL and BWDL refer to the band’s total UL and DL bandwidths.

The procedure is also used to calculate test frequencies for symmetric UL and DL bandwidth combinations for asymmetric NR bands.

For FDD bands supporting asymmetric uplink and downlink bandwidth combinations a deviation of ΔFTX-RX (C.2.3.1-Eq1) is to be added to the default Tx-Rx carrier centre frequency separation, FTx-Rx_separation (TS 38.101-1 [7] clause 5.3.6).

| ΔFTX-RX = ¦(CBWDL – CBWUL)/2¦ | C.2.3.1-Eq1 |
| --- | --- |

For the case of asymmetric NR bands and symmetric UL and DL bandwidth combinations ΔFTX-RX = 0.To meet the Tx-Rx frequency separation requirement for asymmetric NR bands were the supported overall UL bandwidth is smaller than the supported overall DL bandwidth it may not be possible to cover the full DL frequency range for all UL and DL channel bandwidth combinations. For CA when such band is only used for DL CC the full range can be used for all DL channel bandwidths.

To maximize the tested frequency range for the non-CA case the UL frequency range, as being smaller than the DL frequency range, need to be used as the starting point to calculate the uplink and downlink test frequencies.

### C.2.3.2	Determination of Low-, Mid- and High-Range for bands supporting asymmetric uplink and downlink bandwidth combinations

The following procedure is used to determine the test frequencies for Low-, Mid- and High-Range for bands supporting asymmetric UL and DL bandwidth combinations.

1.	Calculate uplink carrier centre frequencies:

| FUL_LowRange = Ceil((FUL_Low + CBWUL/2) / ΔFRaster) * ΔFRaster | C.2.3.2-Eq1 |
| --- | --- |
| FUL_MidRange = Round((FUL_Low + BWUL_Band/2) / ΔFRaster) * ΔFRaster | C.2.3.2-Eq2 |
| FUL_HighRange = Floor((FUL_Low + BWUL_Band – CBWUL/2) / ΔFRaster) * ΔFRaster | C.2.3.2-Eq3 |

2.	Calculate the downlink frequencies:

Calculate the DL carrier centre frequencies from the UL frequencies in step 1.

| FDL_LowRange = FUL_LowRange + FTx-Rx_separation + ΔFTx-Rx | C.2.3.2-Eq4 |
| --- | --- |
| FDL_MidRange = FUL_MidRange + FTx-Rx_separation + ΔFTx-Rx | C.2.3.2-Eq5 |
| FDL_HighRange = FUL_HighRange + FTx-Rx_separation + ΔFTx-Rx | C.2.3.2-Eq6 |

3.	Check that the calculated centre test frequencies in step 2 for the BWDL fits within the bands DL frequency range:

If FDL_LowRange is smaller than the lowest frequency of the band then recalculate the minimum FDL_LowRange and modify the associated FUL_LowRange, FDL_MidRange and FUL_MidRange as:

| FDL_LowRange = Ceil((FDL_Low + CBWDL/2) / ΔFRaster) * ΔFRaster | C.2.3.2-Eq7 |
| --- | --- |
| FUL_LowRange = FDL_LowRange - FTx-Rx_separation - ΔFTx-Rx | C.2.3.2-Eq8 |
| FDL_MidRange = Round((FDL_LowRange + FDL_HighRange)/2 / ΔFRaster))* ΔFRaster | C.2.3.2-Eq8a |
| FUL_MidRange = FDL_MidRange - FTx-Rx_separation - ΔFTx-Rx | C.2.3.2-Eq8b |

If FDL_HighRange is larger than the higher frequency of the band then recalculate the maximum FDL_HighRange and modify the associated FUL_HighRange, FDL_MidRange and FUL_MidRange as:

| FDL_HighRange = Floor((FDL_Low + BWDL_Band – CBWDL/2) / ΔFRaster) * ΔFRaster | C.2.3.2-Eq9 |
| --- | --- |
| FUL_HighRange = FDL_HighRange - FTx-Rx_separation - ΔFTx-Rx | C.2.3.2-Eq10 |
| FDL_MidRange = Round((FDL_LowRange + FDL_HighRange)/2 / ΔFRaster))* ΔFRaster | C.2.3.2-Eq11 |
| FUL_MidRange = FDL_MidRange - FTx-Rx_separation - ΔFTx-Rx | C.2.3.2-Eq12 |

### C.2.3.3	Determination of test frequencies for a Mid range adjacent inter-frequency cell for FR2 RRM multicell testing

The following procedure is used to determine the test frequencies for Mid-Range adjacent inter-frequency cell used for RRM FR2 NR multi-cell in NR SA and EN-DC test cases. The reason for using an adjacent inter-frequency cell to the Mid-range cell for FR2 instead of using Low- or High- Range test frequencies as used for FR1 is to reduce test system complexity.

In addition to the definition of parameters in clause C.1 the following parameters are used to calculate the test frequencies for the Mid adjacent inter-frequency cell:

| Parameter | Description |
| --- | --- |
| FMid | Carrier centre frequency (MHz) of the Mid-range cell |
| CBWMid | Channel bandwidth (MHz) of the Mid-range cell |
| CBWAdjecent | Channel bandwidth (MHz) of the adjacent cell |

1.	Calculate the Mid-range adjacent cell carrier centre frequencies:

| FMidRangeAdjecentCell = Ceil((FMid + (CBWMid + CBWAdjecent)/2) / ΔFRaster) * ΔFRaster | C.2.3.3-Eq1 |
| --- | --- |

2.	Calculate SSB and CORESET#0 parameters as described in clause C.4.

## C.2.4	Frequency determination for NR CA and NR DC configurations

### C.2.4.1	Determination of test frequencies for NR Inter-band CA and NR DC

Test frequencies for NR Inter-band CA configurations and NR DC use the single carrier test frequencies for each of the included NR bands in the configuration as specified in clause 4.3.1.1.1 for FR1 bands and in clause 4.3.1.2.1 for FR2 bands.

### C.2.4.2	Determination of test frequencies for NR Intra-band Contiguous CA

#### C.2.4.2.1	General

By default, test frequencies for NR Intra-band Contiguous CA in clause 4.3.1 are specified using the nominal channel spacing between the carrier components as specified in TS 38.101-1 [7] clause 5.4A.1 for FR1 and TS 38.101-2 [8] clause 5.4A.1 for FR2. In addition, some NR bands may have test frequencies specified based on an adjusted channel spacing as specified in in TS 38.101-1 [7] clause 5.4A.1 for FR1 and TS 38.101-2 [8] clause 5.4A.1 for FR2.

The test frequencies for NR Intra-band Contiguous CA with SCS=15kHz or SCS=30 kHz for FR1 and with SCS=60 kHZ or SCS=120 kHz for FR2 is calculated for each CC such that the specific test cases can decide which CC is used as PCell. This means that all CC test frequencies is calculated with a CORESET#0 as specified in clause C.3.2.

The test frequencies for CCs with SCS=60 kHz for FR1 and with SCS=240 kHz for FR2 is calculated without CORESET#0 as specified in C.3.3. CCs with SCS=60 kHz for FR1 and with SCS=240 kHz for FR2 can only be used for NR Intra-band Contiguous CA configurations with mixed numerologies.

Note:	For NR Intra-band Contiguous CA configurations for bands where Tx frequency range is lower than Rx frequency range the RAN4 requirements for reference sensitivity testing is specified having the PCC frequency lower than the SCC frequencies such that UL PRB maximise the Tx-Rx separation. This means that CC1 shall be used as PCell in the reference test case for bands where Tx frequency range is lower than Rx frequency range; and highest CC shall be used as PCell in the reference test case for bands where Tx frequency range is higher than Rx frequency range.

In addition to the definition of parameters in clause C.1 the following parameters are used to calculate carrier components (CC) test frequencies for NR Intra-band Contiguous and Non-contiguous CA configurations:

| Parameter | Description |
| --- | --- |
| NCC | Number of CCs in the for NR Intra-band configuration |
| CCBWDL(i) | Channel bandwidth (MHz) of downlink CC(i), where i = 1 to NCC |
| FChannel_Spacing(i) | Channel spacing between CC(i) and CC(i+1), where i = 1 to (NCC-1) |

#### C.2.4.2.2	Determination of test frequencies for Low-, Mid- and High-Range

Downlink CC(1), lowest frequency CC:

FDL_LowRange_CC(1) is rounded up and FDL_HighRange_CC(1) is rounded down to obey to the minimum guard band according to clause 5.3.3 of TS 38.101-1 [7] and TS 38.101-2 [8].

| FDL_LowRange_CC(1) = Ceil((FDL_Low + CCBWDL(1) / 2) / ΔFRaster) * ΔFRaster | C.2.4.2.2-Eq1 |
| --- | --- |
| FDL_MidRange_CC(1) = Round((FDL_Low + BWDL/2 – ∑k=1 to (Ncc) CCBWDL (k)/2 + CCBWDL(1)/2) / ΔFRaster) * ΔFRaster | C.2.4.2.2-Eq2 |
| FDL_HighRange_CC(1) = Floor((FDL_High - CCBWDL(NCC)/2– ∑k=1 to (Ncc-1) FChannel_Spacing(k)) / ΔFRaster) * ΔFRaster | C.2.4.2.2-Eq3 |

Downlink CC(2) to CC(NCC), in increasing frequency order:

| FDL_LowRange_CC(i) = FDL_LowRange_CC(i-1) + FChannel_Spacing(i)), i=2 to NCC | C.2.4.2.2-Eq4 |
| --- | --- |
| FDL_MidRange_CC(i) = FDL_MidRange_CC(i-1) + FChannel_Spacing(i)), i=2 to NCC | C.2.4.2.2-Eq5 |
| FDL_HighRange_CC(i) = FDL_HighRange_CC(i-1) + FChannel_Spacing(i)), i=2 to NCC | C.2.4.2.2-Eq6 |

Uplink CC(i), i=1 to NCC:

| FUL_LowRange_CC(i) = FDL_LowRange_CC(i) - FTx-Rx_separation | C.2.4.2.2-Eq7 |
| --- | --- |
| FUL_MidRange_CC(i) = FDL_MidRange_CC(i) - FTx-Rx_separation | C.2.4.2.2-Eq8 |
| FUL_HighRange_CC(i) = FDL_HighRange_CC(i) - FTx-Rx_separation | C.2.4.2.2-Eq9 |

### C.2.4.2A	Determination of test frequencies for FR1 NR Intra-band Contiguous CA without UL CA for bands with uplink bandwidth less than downlink bandwidth

#### C.2.4.2A.1	General

By default, test frequencies for FR1 NR Intra-band Contiguous CA for bands with uplink bandwidth less than downlink bandwidth in clause 4.3.1 (e.g. n66 and n70) are specified with CC1 used as PCC and all additional CCs are specified as SCCs without UL to enable the SCCs for High range to extend into the upper DL BW beyond the UL BW. The nominal channel spacing between the carrier components is calculated as specified in TS 38.101-1 [7] clause 5.4A.1.

In addition to the definition of parameters in clause C.1 the defintion of parameters NCC, CCBWDL and FChannel_Spacing in clause C.2.4.2.1, and ΔFTX-RX in clause C.2.3.1 are used to calculate the test frequencies.

#### C.2.4.2A.2	Determination of test frequencies for Low-, Mid- and High-Range

1.	Calculate UL carrier centre frequencies for Low and High ranges:

| FUL_LowRange = Ceil((FUL_Low + CBWUL/2) / ΔFRaster) * ΔFRaster | C.2.4.2A.2-Eq1 |
| --- | --- |
| FUL_HighRange = Floor((FUL_Low + BWUL_Band – CBWUL/2) / ΔFRaster) * ΔFRaster | C.2.4.2A.2-Eq2 |

2.	Calculate the DL CC(1) carrier centre frequencies from the UL frequencies in step 1 for Low and High ranges:

| ΔFTX-RX = ¦(CBWDL(1) – CBWUL)/2¦ | C.2.4.2A.2-Eq3 |
| --- | --- |
| s = +1 if FUL_Low <= FDL_Low else -1 | C.2.4.2A.2-Eq4 |
| FDL_LowRange_CC(1) = FUL_LowRange + s*(FTx-Rx_separation + ΔFTx-Rx) | C.2.4.2A.2-Eq5 |
| FDL_HighRange_CC(1) = FUL_HighRange + s*(FTx-Rx_separation + ΔFTx-Rx) | C.2.4.2A.2-Eq6 |

3.	Check that DL aggregated CBW for the High range fits into the DL bandwidth.

| FDL_HighRange_max = Floor((FDL_HighRange_CC(1) + ∑k=1 to Ncc-1 FChannel_Spacing(k)+ CCBWDL(NCC)/2) | C.2.4.2A.2-Eq7 |
| --- | --- |

If FDL_HighRange_max is less or equal to FDL_High then goto step 4 else modify FDL_HighRange_CC1 such that the full aggregated CBW is located at the DL bandwidth high edge and recalculate FUL_HighRange.

| FDL_HighRange_CC(1) = FDL_High - CCBWDL(NCC)/2 - ∑k=1 to (Ncc-2) FChannel_Spacing(k) | C.2.4.2A.2-Eq8 |
| --- | --- |
| FUL_HighRange = FDL_HighRange_CC(1) – s*(FTx-Rx_separation + ΔFTx-Rx) | C.2.4.2A.2-Eq9 |

4.	Calculate the FUL_MidRange and FDL_MidRange_CC(1):

| FDL_MidRange_CC(1) = Round((FDL_LowRange_CC(1) + FDL_HighRange_CC(1))/2) / ΔFRaster) * ΔFRaster | C.2.4.2A.2-Eq10 |
| --- | --- |
| FUL_MidRange = FUDL_MidRange_CC(1) – s*(FTx-Rx_separation + ΔFTx-Rx) | C.2.4.2A.2-Eq11 |

5.	Calulate DL CC(2) to CC(NCC), in increasing frequency order:

| FDL_LowRange_CC(i) = FDL_LowRange_CC(i-1) + FChannel_Spacing(i), i=2 to NCC | C.2.4.2A.2-Eq12 |
| --- | --- |
| FDL_MidRange_CC(i) = FDL_MidRange_CC(i-1) + FChannel_Spacing(i), i=2 to NCC | C.2.4.2A.2-Eq13 |
| FDL_HighRange_CC(i) = FDL_HighRange_CC(i-1) + FChannel_Spacing(i), i=2 to NCC | C.2.4.2A.2-Eq14 |

### C.2.4.3	Determination of test frequencies for NR Intra-band Non-Contiguous CA

#### C.2.4.3.1	General

The default test frequencies in clause 4.3.1 for NR Intra-band Non-Contiguous CA are based on maximum Wgap between the carrier components of the different bands taking the UE capability of maximum supported frequency separation between the lower edge of lowest component carrier and upper edge of highest component carrier for UL for FR1, and UL and DL for FR2.

Test frequencies with Wgap different from maximum Wgap are specified in the specific test cases using them.

In addition to the definition of parameters in clause C.1 the following parameters are used to calculate carrier components (CC) test frequencies for NR Intra-band Contiguous and Non-contiguous CA configurations:

| Parameter | Description |
| --- | --- |
| NSB | Number of sub-block in the for NR Intra-band non-contiguous configuration |
| SBCBWDL(i) | Downlink channel bandwidth (MHz) of sub-block SB(i), where i = 1 to NSB |
| maxFsBW | Maximum frequency separation bandwidth between the lower edge of lowest component carrier and upper edge of highest component carrier. |
| maxWgap | maxWgap is the maximum separation in MHz between each sub-block in the NR Intra-band non-contiguous configuration within maxFsBW for a given sub-block combination. |

#### C.2.4.3.1A	Selection of maximum frequency separation for FR1

Select the maxFsBW dependent on the type of configuration and aggregated CBW for the sub-block combination in Table C.2.4.3.1A-1:

Table C.2.4.3.1A-1: Selecting maxFsBW for FR1

| Type of configuration | Aggregated CBW | maxFsBW |
| --- | --- | --- |
| Without UL CA | All | Full NR bandwidth |
| With UL CA | <100 MHz | 100 MHz |
|  | >=100 MHz and <200 MHz | 200 MHz |
|  | >=100 MHz and <600 MHz | 600 MHz |
|  | >600 MHz | Full NR bandwidth |

#### C.2.4.3.1B	Selection of maximum frequency separation for FR2

Select the maxFsBW dependent on the type of configuration and aggregated CBW for the sub-block combination in Table C.2.4.3.1B-1:

Table C.2.4.3.1B-1: Selecting maxFsBW for FR2

| Type of configuration | Aggregated CBW | maxFsBW (Note 1) |
| --- | --- | --- |
| With non-contiguous UL CA | <400 MHz | 400 MHz |
|  | >=400 MHz and <600 MHz | 600 MHz |
|  | >=600 MHz and <800 MHz | 800 MHz |
|  | >=800 MHz and <1000 MHz | 1000 MHz |
|  | >=1000 MHz and <1200 MHz | 1200 MHz |
|  | >=1200 MHz and <1400 MHz | 1400 MHz |
|  | >=1400 MHz | N/A |
| Without non-contiguous UL CA | <400 MHz | 400 MHz |
|  | >=400 MHz and <600 MHz | 600 MHz |
|  | >=600 MHz and <800 MHz | 800 MHz |
|  | >=800 MHz and <1000 MHz | 1000 MHz |
|  | >=1000 MHz and <1200 MHz | 1200 MHz |
|  | >=1200 MHz and <1400 MHz | 1400 MHz |
|  | >=1400 MHz and <1600 MHz | 1600 MHz |
|  | >=1600 MHz and <1800 MHz | 1800 MHz |
|  | >=1800 MHz and <2000 MHz | 2000 MHz |
|  | >=2000 MHz and <2200 MHz | 2200 MHz |
|  | >=2200 MHz and <2400 MHz | 2400 MHz |
| Note 1: For FR2 intra-band non-contiguous CA configurations with non-contiguous UL CA the selected value of maxFsBW is based on applicable frequency separation classes for UL non-contiguous operation in TS 38.101-2 [8],Table 5.3A.4-2. For FR2 intra-band non-contiguous CA configurations without non-contiguous UL CA the selected value of maxFsBW is based on applicable frequency separation classes for DL non-contiguous operation in TS 38.101-2 [8],Table 5.3A.4-2. |  |  |

#### C.2.4.3.2	Determination of test frequencies for a sub-block combination

Editor's note: The number of test points for intra-band non-contiguous CA configurations is under investigation, e.g. “Low” and “High”, or “Mid”.

1.	Calculate the maxWgap value between sub-blocks for the sub-block combination:

| maxWgap = (maxFsBW – ∑k=1 to NSB SBCBWDL(k)) / (NSB-1) | C.2.4.3.3-Eq1 |
| --- | --- |

2.	Calculate test frequencies for all sub-blocks in the sub-block combination:

If the maxFsBW is smaller than the full bandwidth of the NR band then calculate test frequencies for both Low and High range else only for the Low range.

For Low range the test frequencies are calculated such that the lower edge of the lowest component carrier of the lowest frequency sub-block is located at the lower edge of the NR bandwidth. The sub-blocks are separated by the calculated maxWgap in step 1.

For High range the test frequencies are calculated such that the upper edge of the highest component carrier of the highest frequency sub-block is located at the upper edge frequency of the NR bandwidth. The sub-blocks are separated by the calculated maxWgap in step 1.

Within each sub-block the test frequencies and parameters of the sub-block are calculated based on the location of the sub-block and the relevant principles in clause C.2, C3 and C4 for the type of component carriers in the sub-block.

The test frequencies for CCs with SCS=60 kHz for FR1 and with SCS=240 kHz for FR2 is calculated without CORESET#0 as specified in C.3.3. CCs with SCS=60 kHz for FR1 and with SCS=240 kHz for FR2 can only be used for NR Intra-band Non-Contiguous CA configurations with mixed numerologies.

#### C.2.4.3.3	Void

## C.2.5	Frequency determination for supplemental uplink

### C.2.5.1	General

The following procedure is used to calculate test frequencies for NR supplemental uplink as described below, where CBWUL refers to the carrier’s UL channel bandwidths and BWUL refers to the band’s total UL bandwidths.

### C.2.5.2	Determination of Low-, Mid- and High-Range for supplemental uplink bands

The following procedure is used to determine the uplink carrier centre frequencies for Low-, Mid- and High-Range for supplemental uplink bands.

| FUL_LowRange = Ceil((FUL_Low + CBWUL/2) / ΔFRaster) * ΔFRaster | C.2.5.2-Eq1 |
| --- | --- |
| FUL_MidRange = Round((FUL_Low + BWUL_Band/2) / ΔFRaster) * ΔFRaster | C.2.5.2-Eq2 |
| FUL_HighRange = Floor((FUL_Low + BWUL_Band – CBWUL/2) / ΔFRaster) * ΔFRaster | C.2.5.2-Eq3 |

## C.2.6	Frequency determination for EN-DC configurations

### C.2.6.1	Determination of test frequencies for EN-DC Inter-band

Test frequencies for EN-DC Intra-band non-contiguous configurations use the Low and High ranges test frequencies for each of the included single carrier E-UTRA and NR bands, and E-UTRA and NR CA configurations in the configuration as specified E-UTRA in TS 36.508 [2], clause 4.3.1 and for NR in clause 4.3.1.1.1 for FR1 bands and in clause 4.3.1.2.1 for FR2 bands.

### C.2.6.2	Determination of test frequencies for EN_DC Intra-band Contiguous CA

#### C.2.6.2.1	General

By default, test frequencies for EN-DC Intra-band Contiguous CA in clause 4.3.1 are specified using the nominal channel spacing between the E-UTRA and NR carrier components as specified in TS 38.101-3 [9], clause 5.4B.1.

The test frequencies for EN_DC Intra-band Contiguous CA is calculated for Low and High ranges for the following two cases:

-	with NR CC at the band edge; and

-	 with E-UTRA CC at the band edge.

In addition to the definition of parameters in clause C.1 the following parameters are used to calculate carrier components (CC) test frequencies for EN_DC Intra-band Contiguous:

| Parameter | Description |
| --- | --- |
| NNR_CC | Number of NR CCs in the EN-DC Intra-band configuration |
| NEUTRA_CC | Number of E-UTRA CCs in the EN-DC Intra-band configuration |
| CCBWNR_DL(i) | Channel bandwidth (MHz) of downlink NR CC(i), where i = 1 to NNR_CC |
| CCBWEUTRA_DL(m) | Channel bandwidth (MHz) of downlink E-UTRA CC(m), where m = 1 to NEUTRA_CC |
| FNR_EUTRA_Channel_Spacing | Nominal channel spacing between adjacent NR and E-UTRA CCs as defined in TS 38.101-3 [9], 5.4B.1. |
| FEUTRA_Channel_Spacing(m) | Nominal channel spacing between E-UTRA adjacent contiguous CC(m) and CC(m+1), where m = 1 to (NEUTRA_CC-1) as defined in TS 36.101 [48], 5.7.1A. |
| FNR_Channel_Spacing(i) | Nominal channel spacing between NR adjacent contiguous CC(i) and CC(i+1), where i = 1 to (NNR_CC-1) as defined in TS 38.101-1 [7] clause 5.4A.1 for FR1 and TS 38.101-2 [8] clause 5.4A.1 for FR2 |
| LCMΔFRaster | Least Common Multiple of NR ΔFRaster and E-UTRA ΔFRaster equals to 300 kHz for E-UTRA ΔFRaster = 100 kHz and NR ΔFRaster = 15kHz, 30kHz and 60kHz. |

#### C.2.6.2.2	Determination of test frequencies for Low-, Mid- and High-Range with NR at band edges

Downlink NR CC(1), lowest frequency CC:

FNR_DL_LowRange_CC(1) is rounded up and FNR_DL_HighRange_CC(1) is rounded down to obey to the minimum guard band according to clause 5.3.3 of TS 38.101-1 [7] and TS 38.101-2 [8].

The NR test frequencies are calculated such that both the NR CC and E-UTRA CC adjacent to each other are located at the NR and E-UTRA frequency raster respectively.

| FNR_DL_LowRange_CC(1) = Ceil((FDL_Low + CCBWNR_DL(1) / 2 + FNR_EUTRA_Channel_Spacing) / LCMΔFRaster) * LCMΔFRaster - FNR_EUTRA_Channel_Spacing, where FNR_EUTRA_Channel_Spacing = Round((CCBWEUTRA_DL(1) + CCBWNR_DL(NNR_CC))/(2*ΔFRaster))* ΔFRaster | C.2.6.2.2-Eq1 |
| --- | --- |
| FNR_DL_MidRange_CC(1) = Round((FDL_Low + BWDL/2 – (∑i=1 to NNR_CC CCBWNR_DL (i) + ∑m=1 to NEUTRA_CC CCBWEUTRA_DL(m))/2 + CCBWNR_DL(1))/2) + FNR_EUTRA_Channel_Spacing / LCMΔFRaster) * LCMΔFRaster - FNR_EUTRA_Channel_Spacing, where FNR_EUTRA_Channel_Spacing = Round((CCBWEUTRA_DL(1) + CCBWNR_DL(NNR_CC))/(2*ΔFRaster))* ΔFRaster | C.2.6.2.2-Eq2 |
| FNR_DL_HighRange_CC(1) = Floor((FDL_High - CCBWNR_DL(NNR_CC)/2– ∑i=1 to (NNR_CC-1) FNR_Channel_Spacing(i) - FNR_EUTRA_Channel_Spacing) / LCMΔFRaster) * LCMΔFRaster + FNR_EUTRA_Channel_Spacing, where FNR_EUTRA_Channel_Spacing = Round((CCBWEUTRA_DL(NEUTRA_CC) + CCBWNR_DL(1))/(2*ΔFRaster))* ΔFRaster | C.2.6.2.2-Eq3 |

Downlink NR CC(2) to CC(NNR_CC), in increasing frequency order:

| FNR_DL_LowRange_CC(k) = FDL_LowRange_CC(k-1) + ∑FNR_Channel_Spacing(k), k=2 to NDL_CC | C.2.6.2.2-Eq4 |
| --- | --- |
| FNR_DL_MidRange_CC(k) = FDL_MidRange_CC(k-1) + ∑FNR_Channel_Spacing(k), k=2 to NDL_CC | C.2.6.2.2-Eq5 |
| FNR_DL_HighRange_CC(k) = FDL_HighRange_CC(k-1) + ∑FNR_Channel_Spacing(k), k=2 to NDL_CC | C.2.6.2.2-Eq6 |

Uplink NR CC(k), k=1 to NCC:

| FNR_UL_LowRange_CC(k) = FNR_DL_LowRange_CC(k) - FTx-Rx_separation | C.2.6.2.2-Eq7 |
| --- | --- |
| FNR_UL_MidRange_CC(k) = FNR_DL_MidRange_CC(k) - FTx-Rx_separation | C.2.6.2.2-Eq8 |
| FNR_UL_HighRange_CC(k) = FNR_DL_HighRange_CC(k) - FTx-Rx_separation | C.2.6.2.2-Eq9 |

Downlink E-UTRA CC(1), lowest frequency CC:

ΔFNR_EUTRA_Channel_Spacing is selected in each formula C.2.6.2.2-Eq10, C.2.6.2.2-Eq11 and C.2.6.2.2-Eq12 selected such that FEUTRA_DL_LowRange_CC(1), FEUTRA_DL_MidRange_CC(1) and FEUTRA_DL_HighRange_CC(1) are located on the E-UTRA band frequency raster.

| FEUTRA_DL_LowRange_CC(1) = FNR_DL_LowRange_CC(NDL_CC) + FNR_EUTRA_Channel_Spacing | C.2.6.2.2-Eq10 |
| --- | --- |
| FEUTRA_DL_MidRange_CC(1) = FNR_DL_MidRange_CC(NDL_CC) + FNR_EUTRA_Channel_Spacing | C.2.6.2.2-Eq11 |
| FEUTRA_DL_HighRange_CC(1) = FNR_DL_HighRange_CC(NDL_CC) - ∑i=1 to (NNR_CC-1) FNR_Channel_Spacing(i) - FNR_EUTRA_Channel_Spacing - ∑i=1 to (NEUTRA_CC-1) FEUTRA_Channel_Spacing(i) | C.2.6.2.2-Eq12 |

Downlink E-UTRA CC(2) to CC(NEUTRA_CC), in increasing frequency order:

| FEUTRA_DL_LowRange_CC(k) = FEUTRA_DL_LowRange_CC(k-1) + ∑FEUTRA_Channel_Spacing(k), k=2 to NEUTRA_CC | C.2.6.2.2-Eq13 |
| --- | --- |
| FEUTRA_DL_MidRange_CC(k) = FEUTRA_DL_MidRange_CC(k-1) + ∑FEUTRA_Channel_Spacing(k), k=2 to NEUTRA_CC | C.2.6.2.2-Eq14 |
| FEUTRA_DL_HighRange_CC(k) = FEUTRA_DL_HighRange_CC(k-1) + ∑FEUTRA_Channel_Spacing(k), k=2 to NEUTRA_CC | C.2.6.2.2-Eq15 |

Uplink E-UTRA CC(k), k=1 to NEUTRA_CC:

| FEUTRA_UL_LowRange_CC(k) = FEUTRA_DL_LowRange_CC(k) - FTx-Rx_separation | C.2.6.2.2-Eq16 |
| --- | --- |
| FEUTRA_UL_MidRange_CC(k) = FEUTRA_DL_MidRange_CC(k) - FTx-Rx_separation | C.2.6.2.2-Eq17 |
| FEUTRA_UL_HighRange_CC(k) = FEUTRA_DL_HighRange_CC(k) - FTx-Rx_separation | C.2.6.2.2-Eq18 |

#### C.2.6.2.3	Determination of test frequencies for Low-, Mid- and High-Range with E-UTRA at band edges

To get the NR carrier on the synchcronisation raster the calculations of the E-UTRA carrier components needs to be based on the location of the NR carrier.

Downlink NR CC(1), lowest frequency CC:

FNR_DL_LowRange_CC(1) is rounded up and FNR_DL_HighRange_CC(1) is rounded down to obey to the minimum guard band according to clause 5.3.3 of TS 38.101-1 [7] and TS 38.101-2 [8].

| FNR_DL_LowRange_CC(1) = Ceil((FDL_Low + ∑m=1 to NEUTRA_CC CCBWEUTRA_DL(m)+CCBWNR_DL(1) / 2 - FNR_EUTRA_Channel_Spacing) / LCMΔFRaster) * LCMΔFRaster + FNR_EUTRA_Channel_Spacing, where FNR_EUTRA_Channel_Spacing = Round((CCBWEUTRA_DL(NEUTRA_CC) + CCBWNR_DL(1))/(2*ΔFRaster))* ΔFRaster | C.2.6.2.3-Eq1 |
| --- | --- |
| FNR_DL_MidRange_CC(1) = same formula as C.2.6.2.2-Eq2 | C.2.6.2.3-Eq2 |
| FNR_DL_HighRange_CC(1) = Floor((FDL_High – (∑m=1 to NEUTRA_CC CCBWEUTRA_DL(m) + ∑i=1 to NNR_CC CCBWNR_DL(i)) +CCBWNR_DL(1) / 2 + FNR_EUTRA_Channel_Spacing) / LCMΔFRaster) * LCMΔFRaster - FNR_EUTRA_Channel_Spacing, where FNR_EUTRA_Channel_Spacing = Round((CCBWEUTRA_DL(1) + CCBWNR_DL(NNR_CC))/(2*ΔFRaster))* ΔFRaster | C.2.6.2.3-Eq3 |

Downlink NR CC(2) to CC(NNR_CC), in increasing frequency order:

| FNR_DL_LowRange_CC(k) = same formula as C.2.6.2.2-Eq4 | C.2.6.2.3-Eq4 |
| --- | --- |
| FNR_DL_MidRange_CC(k) = same formula as C.2.6.2.2-Eq5 | C.2.6.2.3-Eq5 |
| FNR_DL_HighRange_CC(k) = same formula as C.2.6.2.2-Eq6 | C.2.6.2.3-Eq6 |

Uplink NR CC(k), k=1 to NCC:

| FNR_UL_LowRange_CC(k) = same formula as C.2.6.2.2-Eq7 | C.2.6.2.3-Eq7 |
| --- | --- |
| FNR_UL_MidRange_CC(k) = same formula as C.2.6.2.2-Eq8 | C.2.6.2.3-Eq8 |
| FNR_UL_HighRange_CC(k) = same formula as C.2.6.2.2-Eq9 | C.2.6.2.3-Eq9 |

Downlink E-UTRA CC(1), lowest frequency CC:

| FEUTRA_DL_LowRange_CC(1) = FNR_DL_LowRange_CC(1) - FNR_EUTRA_Channel_Spacing - ∑m=1 to (NEUTRA_CC - 1) FEUTRA_Channel_Spacing(m) | C.2.6.2.3-Eq10 |
| --- | --- |
| FEUTRA_DL_MidRange_CC(1) = same formula as C.2.6.2.2-Eq11 | C.2.6.2.3-Eq11 |
| FEUTRA_DL_HighRange_CC(1) = FNR_DL_HighRange_CC(NDL_CC) + ∑i=1 to (NNR_CC-1) FNR_Channel_Spacing(i) + FNR_EUTRA_Channel_Spacing + ∑i=1 to (NEUTRA_CC-1) FEUTRA_Channel_Spacing(i) | C.2.6.2.3-Eq12 |

Downlink E-UTRA CC(2) to CC(NEUTRA_CC), in increasing frequency order:

| FEUTRA_DL_LowRange_CC(k) = same formula as C.2.6.2.2-Eq13 | C.2.6.2.3-Eq13 |
| --- | --- |
| FEUTRA_DL_MidRange_CC(k) = same formula as C.2.6.2.2-Eq14 | C.2.6.2.3-Eq14 |
| FEUTRA_DL_HighRange_CC(k) = same formula as C.2.6.2.2-Eq15 | C.2.6.2.3-Eq15 |

Uplink E-UTRA CC(k), k=1 to NEUTRA_CC:

| FEUTRA_UL_LowRange_CC(k) = same formula as C.2.6.2.2-Eq16 | C.2.6.2.3-Eq16 |
| --- | --- |
| FEUTRA_UL_MidRange_CC(k) = same formula as C.2.6.2.2-Eq17 | C.2.6.2.3-Eq17 |
| FEUTRA_UL_HighRange_CC(k) = same formula as C.2.6.2.2-Eq18 | C.2.6.2.3-Eq18 |

### C.2.6.3	Determination of test frequencies for EN-DC Intra-band non-contiguous

Test frequencies for EN-DC Intra-band non-contiguous configurations use the Low and High ranges test frequencies for each of the included single carrier E-UTRA and NR bands, and E-UTRA and NR CA configurations in the configuration as specified E-UTRA in TS 36.508 [2], clause 4.3.1 and for NR in clause 4.3.1.1.1 for FR1 bands and in clause 4.3.1.2.1 for FR2 bands.

The following cases of test frequencies are specified for relevant E-UTRA and NR CBW combinations, and NR SCS:

-	Low with maxWgap (NR - E-UTRA): NR Low range and E-UTRA High range

-	High with maxWgap (E-UTRA - NR): NR High range and E-UTRA Low range

# C.3	Determination of SSB and CORESET#0

## C.3.1	General

The requirements to be met and the principles used for determining the SSB and CORESET#0 for a PCell are:

1.	The complete SSB and CORESET#0 shall be within the carrier’s channel bandwidth.

2.	The SSB centre frequency (SSref) shall be on the synchronisation raster.

3.	The SSB shall be kept as close as possible to the carrier’s lower edge centre frequency.

4.	CORESET#0 configuration is selected using lowest number of RBs and symbols in applicable table in TS 38.213 [22], clause 13.

5.	The first SSB subcarrier shall be aligned with the defined resource grid given by SCS indicated by subCarrierSpacingCommon in the MIB.

## C.3.2	Determination of SSB, CORESET#0 and signalling parameters for a PCell

Calculation of SSB and CORESET#0 parameters is limited to FR1 carriers with SCS=15 kHz or SCS=30 kHz, and to FR2 carriers with SCS=60 kHz or SCS=120 kHz. CORESET#0 is required for a carrier to be used as a PCell.

The following procedure is used to determine an SSB on the synchronisation raster (GSCN) and a CORESET#0 configuration (kSSB, OffsetRBs and OffsetToPointA) as close as possible to the carrier’s lower edge. See figure C1-1 and clause C.1 for definition of parameters referenced in the procedure.

1.	Determine SSB and CORESET#0:

1a.	Calculate the lower of FSSref, FSSref_Min, correspondent to SSB lowest subcarrier being at the same frequency as the carrier’s lowest subcarrier; and the higher limit of FSSref, FSSref_Max, correspondent to SSB highest subcarrier being at the same frequency as the carrier’s highest subcarrier FSSref_Min

FcarrierLow = see formula for FcarrierLow in Table C.1-1

FSSref_Min = FcarrierLow + CRBsize * OffsetRBs,min + BWSSB / 2

FSSref_Max = FcarrierLow + ΔFcarrierBandwidth - BWSSB / 2

1b.	Calculate GSCNMIN correspondent to FSSref_Min in accordance to TS 38.101-1 [7], clause 5.4.3.1 for FR1 and TS 38.101-2 [7], clause 5.4.3.1 for FR2 and select the closest valid GSCN value with GSCN >= GSCNMIN for the carrier in according to the carrier’s synchronisation raster as specified in clause 5.4.3.3 in TS 38.101-1 [7] and TS 38.101-2 [8].

1c.	Calculate the FSSref for the selected GSCN value in step 1b in accordance to TS 38.101-1 [7], clause 5.4.3.1 for FR1 and TS 38.101-2 [7], clause 5.4.3.1 for FR2.

1d.	Calculate the frequency FoffsetToPointA, which is the lowest subcarrier of the lowest resource block with the subcarrier spacing being a multiple of resource blocks expressed in terms of common PRB size and overlaps with the SS/PBCH block subcarrier 0 of the first resource block of the SS/PBCH block, FSSBlow (TS 38.211 [3], clause 7.4.3.1):

FSSBlow = FSSref - BWSSB / 2

FoffsetToPointA = CRBsize * Floor((FSSBlow - FcarrierLow) / CRBsize) + FcarrierLow

1e.	Calculate the maximum OffsetRBs value with FCORESET0Low >= FcarrierLow:

Max_OffsetRBs = (FoffsetToPointA - FcarrierLow) / CRBsize

1f.	Select the largest valid OffsetRBs value equal or smaller than the calculated max value, Max_OffsetRBs in step 1e within the applicable values for the carrier in TS 38.213 [4], table 13-1 to 13-10 limited to the table indexes with number of RBs and number of symbols  equal to the minimum value of in the table and minimum value of  for the selected .If a valid OffsetRBs value is found, then continue from step 1g.

If no valid OffsetRBs value is found, then select the next valid GSCN with FSSref <= FSSref_Max within the valid GSCN range for the carrier and repeat steps 1b to 1f.

If no valid OffsetRBs value found within the valid GSCN range then will the carrier not be possible to use as PCell and FSSref, kSSB, FPointA, OffsetToCarrier and OffsetToPointA are calculated as described in clause C.3.3 and the procedure is completed.

1g.	Calculate kSSB

kSSB = (FSSBlow - FoffsetToPointA) / {15 kHz for FR1, subCarrierSpacingCommon (MIB) for FR2} (TS 38.211 [3], clause 7.4.3.1).

N = SCSSSB / {15 kHz for FR1; subCarrierSpacingCommon (MIB) for FR2}. kSSB MOD N <> 0 indicates that the SSB subcarriers are not aligned with the resource grid given by the SCS indicated by subCarrierSpacingCommon in the MIB.

If kSSB is an integer and kSSB MOD N = 0, then continue from step 2.

If kSSB is not an integer value or kSSB MOD N <> 0, then select the next valid GSCN with FSSref < FSSref_Max within the valid GSCN range for the carrier and repeat steps 1b to 1g. 

If N > 1 and no valid kSSB value found within the valid GSCN range for the currently selected carrier frequency Fcarrier then shift Fcarrier up by ΔFRaster for Low range; or down by ΔFRaster for Mid, Mid-Low, Mid-High and High ranges and repeat steps 1a to 1g for a maximum shift of 3 * ΔFRaster (see clause C.3.1, Note 1).

If no valid kSSB value found within the valid GSCN range then will the carrier not be possible to use as PCell and FSSref, kSSB, FPointA, OffsetToCarrier and OffsetToPointA are calculated as described in clause C.3.2 and the procedure is completed.

2.	Determine OffsetToCarrier

Select offsetToCarrier value for the carrier in accordance to Table C.3.2-1.

Table C.3.2-1: Downlink and uplink offsetToCarrier default values for different frequency ranges

| Frequency range | Downlink offsetToCarrier | Uplink offsetToCarrier |
| --- | --- | --- |
| Low range | 0 | 0 |
| Mid range | 102 | 504 |
| High range | 504 | 6 |
| Mid-Low range | 12 | 36 |
| Mid-High range | 24 | 114 |
| Note: Different values of offsetToCarrier have been selected for Low, Mid-Low, Mid, Mid-High and High ranges to achieve enhanced test coverage of the offsetToCarrier range of values. |  |  |

In case low, mid and high range are exactly the same frequency, use offsetToCarrier associated to low range.

2b.	Determine FpointA:

FPointA = FcarrierLow - offsetToCarrier * PRBsize

3.	Calculate ΔFOffsetCORESET-0-Carrier:

The ΔFOffsetCORESET-0-Carrier value is used to calculate the Offset Carrier CORESET#0 parameter included in the test frequency tables in sub-clauses 4.3.1 and 6.2.3.

ΔFOffsetCORESET-0-Carrier = FoffsetToPointA - OffsetRB * CRBsize - FcarrierLow

4.	Calculate signalling parameters:

| IE field | Value |
| --- | --- |
| ssb-SubcarrierOffset | Set to the 4 least significant bits of kSSB. For the case kSSB > 15 the extended by an additional most significant bit encoded within PBCH as specified in TS 38.213 [22]. The IE field ssb-SubcarrierOffset is signalled in the MIB. |
| controlResourceSetZero | Set to the index associated with the selected OffsetRBs value in the applicable table, 13-1 to 13-10, in TS 38.213 [22]. The IE field controlResourceSetZero is signalled in the IE pdcch-ConfigSIB1 in the MIB. |
| absoluteFrequencySSB | Set to FSSref expressed in ARFCN as defined in TS 38.101-1 [15] and TS 38.101-2 [39], clause 5.4.2. |
| absoluteFrequencyPointA | Set to FPointA expressed in ARFCN as defined in TS 38.101-1 [15] and TS 38.101-2 [39], clause 5.4.2. |
| offsetToPointA | (FOffsetToPointA - FPointA) / (12*{15 kHz for FR1; 60 kHz for FR2}). The IE field offsetToPointA is signalled in IE FrequencyInfoDL-SIB. |
| offsetToCarrier | Set to value calculated in step 2a. The IE field offsetToCarrier is signalled in IE SCS-SpecificCarrier. |

## C.3.3	Determination of SSB and signalling parameters for a carrier without CORESET#0

The following procedure is used for calculation of SSB and signalling parameters for a carrier without a CORESET#0.

1. 	Calculate FSSref, kSSB and FPointA with the SSB lowest subcarrier at the carrier’s lowest subcarrier (ΔFOffsetSSB-Carrier in Figure C.1-1 = 0):

FSSref = FcarrierLow + BWSSB / 2

kSSB = {31 for FR1; 15 for FR2} indicating that no CORESET#0 is present for the carrier (TS 38.213 [4], clause 13).

offsetToCarrier = target value for offsetToCarrier dependent on frequency range as specified in Table C.3.1-1.

FPointA = FcarrierLow - offsetToCarrier * PRBsize

2.	Calculate signalling parameters:

| IE field | Value |
| --- | --- |
| ssb-SubcarrierOffset | Set to the 4 least significant bits of kSSB. For the case kSSB > 15 the extended by an additional most significant bit encoded within PBCH as specified in TS 38.213 [22]. The IE field ssb-SubcarrierOffset is signalled in the MIB. |
| controlResourceSetZero | Set to 0 indicating that no CORESET#0 exist (TS 38.213 [22], clause 13). The IE field controlResourceSetZero is signalled in the IE pdcch-ConfigSIB1 in the MIB. |
| searchSpaceZero | Set to 0 indicating that no CORESET#0 exist (TS 38.213 [22], clause 13). The IE field searchSpaceZero is signalled in the IE pdcch-ConfigSIB1 in the MIB. |
| absoluteFrequencySSB | Set to FSSref expressed in ARFCN as defined in TS 38.101-1 [15] and TS 38.101-2 [39], clause 5.4.2. |
| absoluteFrequencyPointA | Set to FPointA expressed in ARFCN as defined in TS 38.101-1 [15] and TS 38.101-2 [39], clause 5.4.2. |
| offsetToCarrier | Set to offsetToCarrier target value selected in step 1. |