3GPP TSG-RAN5 Meeting #95-e	R5-223797

# Electronic Meeting, 9th May– 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-2 | CR | 0319 | rev | 1 | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Alignment of of EN-DC Physical Layer Baseline Implementation Capabilities with 38.521-3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CAICT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - EN-DC configurations in A.4.3.2B.2 are not aligned with 38.521-3. - EN-DC NR part power class UE RF Baseline Implementation Capabilities defined in 6.1 of 38.521-3 for Rel-15 are missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - EN-DC configurations in A.4.3.2B.2 were revised to align with 38.521-3. - EN-DC NR part power class UE RF Baseline Implementation Capabilities defined in 6.1 of 38.521-3 for Rel-15 were added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - EN-DC configurations in A.4.3.2B.2 will be incomplete and incorrect. - Test lab can not get the NR part power class information for a Rel-15 EN-DC within FR1 UE that indicates IE maxNumberSRS-Ports-PerResource = n2 in NR standalone operation mode and indicates IE maxNumberSRS-Ports-PerResource = n1 for EN-DC on this NR band. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4.3.1, A.4.3.2B.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Other WIs included in this CR: NR_CADC_NR_LTE_DC_R16-UEConTest, NR_CADC_NR_LTE_DC_R17-UEConTest. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: - The overlapping revisions with R5-223157 in Table A.4.3.2B.2.3.1-3 and Table A.4.3.2B.2.3.1-3a were removed. - The addition of DC_5A_78A in Table A.4.3.2B.2.3.1-3a were cancelled. - DC_5A_78A were removed from Table A.4.3.2B.2.3.1-3 to align with Table A.4.3.2B.2.3.1-3a and 38.521-3. - Table A.4.3.1-4f, Table A.4.3.2B.2.1-4a, Table A.4.3.2B.2.1-4b,Table A.4.3.2B.2.2-4a,Table A.4.3.2B.2.2-4b,able A.4.3.2B.2.3.1-3b, Table A.4.3.2B.2.3.1-3c were added or updated. |  |  |  |  |  |  |  |  |

<< Unchanged sections omitted >>

A.4.3.1	RF Baseline Implementation Capabilities

NOTE:	The values indicated in column "Release" in tables A.4.3.1-1 and A.4.3.1-2 below are to be understood as the specifications release version in which a band was introduced and not as a mandate that a UE conforming to particular release shall support a particular band. For further guidance to release independent bands see TS 38.307 [19].

Table A.4.3.1-1: NR FDD FR1 RF Baseline Implementation Capabilities

| Item |  | NR FDD FR1 RF Baseline Implementation Capabilities |  | Ref. |  | Release |  | Mnemonic |  | Comments |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  | NR Frequency band: 1920-1980 MHz (UL), 2110-2170 MHz (DL) |  | 38.101-1, 5.2 |  | Rel-15 |  | pc_nrBand1_Supp |  | NR FDD FR1 Band 1 |  |
| 2 |  | NR Frequency band: 1850-1910 MHz (UL), 1930-1990 MHz (DL) |  | 38.101-1, 5.2 |  | Rel-15 |  | pc_nrBand2_Supp |  | NR FDD FR1 Band n2 |  |
| 3 |  | NR Frequency band: 1710-1785 MHz (UL), 1805-1880 MHz (DL) |  | 38.101-1, 5.2 |  | Rel-15 |  | pc_nrBand3_Supp |  | NR FDD FR1 Band n3 |  |
| 4 |  | NR Frequency band: 824-849 MHz (UL), 869-894 MHz (DL) |  | 38.101-1, 5.2 |  | Rel-15 |  | pc_nrBand5_Supp |  | NR FDD FR1 Band n5 |  |
| 5 |  | NR Frequency band: 2500-2570 MHz (UL), 2620-2690 MHz (DL) |  | 38.101-1, 5.2 |  | Rel-15 |  | pc_nrBand7_Supp |  | NR FDD FR1 Band n7 |  |
| 6 |  | NR Frequency band: 880-915 MHz (UL), 925-960 MHz (DL) |  | 38.101-1, 5.2 |  | Rel-15 |  | pc_nrBand8_Supp |  | NR FDD FR1 Band n8 |  |
| 6a to 6c |  | Reserved |  |  |  |  |  |  |  |  |  |
| 6d |  | NR Frequency band: 699-716 MHz (UL), 729-746 MHz (DL) |  | 38.101-1, 5.2 |  | Rel-15 |  | pc_nrBand12_Supp |  | NR FDD FR1 Band n12 |  |
| 6e |  | Reserved |  |  |  |  |  |  |  |  |  |
| 6f |  | NR Frequency band: 788-798 MHz (UL), 758-768 MHz (DL) |  | 38.101-1, 5.2 |  | Rel-16 |  | pc_nrBand14_Supp |  | NR FDD FR1 Band n14 |  |
| 6g to 6i |  | Reserved |  |  |  |  |  |  |  |  |  |
| 6j |  | NR Frequency band: 815-830 MHz (UL), 860-875 MHz (DL) |  | 38.101-1, 5.2 |  | Rel-16 |  | pc_nrBand18_Supp |  | NR FDD FR1 Band n18 |  |
| 6k |  | Reserved |  |  |  |  |  |  |  |  |  |
| 7 |  | NR Frequency band: 832-862 MHz (UL), 791-821 MHz (DL) |  | 38.101-1, 5.2 |  | Rel-15 |  | pc_nrBand20_Supp |  | NR FDD FR1 Band n20 |  |
| 7a to 7c |  | Reserved |  |  |  |  |  |  |  |  |  |
| 7d |  | NR Frequency band: 1626.5-1660.5 MHz (UL), 1525-1559 MHz (DL) |  | 38.101-1, 5.2 |  | Rel-17 |  | pc_nrBand24_Supp |  | NR FDD FR1 Band n24 |  |
| 7e |  | NR Frequency band: 1850-1915 MHz (UL), 1930- 1995 MHz (DL) |  | 38.101-1, 5.2 |  | Rel-15 |  | pc_nrBand25_Supp |  | NR FDD FR1 Band n25 |  |
| 7f |  | NR Frequency band: 814-849 MHz (UL), 859-894 MHz (DL) |  | 38.101-1, 5.2 |  | Rel-16 |  | pc_nrBand26_Supp |  | NR FDD FR1 Band n26 |  |
| 7g |  | Reserved |  |  |  |  |  |  |  |  |  |
| 8 |  | NR Frequency band: 703-748 MHz (UL), 758-803 MHz (DL) |  | 38.101-1, 5.2 |  | Rel-15 |  | pc_nrBand28_Supp |  | NR FDD FR1 Band n28 |  |
| 8a |  | Reserved |  |  |  |  |  |  |  |  |  |
| 8b |  | NR Frequency band: 2305-2315 MHz (UL), 2350-2360 MHz (DL) |  | 38.101-1, 5.2 |  | Rel-16 |  | pc_nrBand30_Supp |  | NR FDD FR1 Band n30 |  |
| 8c to 8d |  | Reserved |  |  |  |  |  |  |  |  |  |
| 8e |  | NR Frequency band: 1920-2010 MHz (UL),2110-2200 MHz (DL) |  | 38.101-1, 5.2 |  | Rel-16 |  | pc_nrBand65_Supp |  | NR FDD FR1 Band n65 |  |
| 9 |  | NR Frequency band: 1710-1780 MHz (UL), 2110-2200 MHz (DL) |  | 38.101-1, 5.2 |  | Rel-15 |  | pc_nrBand66_Supp |  | NR FDD FR1 Band n66 |  |
| 9a to 9c |  | Reserved |  |  |  |  |  |  |  |  |  |
| 10 |  | NR Frequency band: 1695-1710 MHz (UL), 1995-2020 MHz (DL) |  | 38.101-1, 5.2 |  | Rel-15 |  | pc_nrBand70_Supp |  | NR FDD FR1 Band n70 |  |
| 11 |  | NR Frequency band: 663-698 MHz (UL), 617-652 MHz (DL) |  | 38.101-1, 5.2 |  | Rel-15 |  | pc_nrBand71_Supp |  | NR FDD FR1 Band 71 |  |
| 12 to 13 |  | Reserved |  |  |  |  |  |  |  |  |  |
| 14 |  | NR Frequency band: 1427-1470 MHz (UL), 1475-1518 MHz (DL) |  | 38.101-1, 5.2 |  | Rel-15 |  | pc_nrBand74_Supp |  | NR FDD FR1 Band n74 |  |
| 15 |  | NR Frequency band: 832-862 MHz (UL), 1427-1432 MHz (DL) |  | 38.101-1, 5.2 |  | Rel-16 |  | pc_nrBand91_Supp |  | NR FDD FR1 Band n91 |  |
| 16 |  | NR Frequency band: 832-862 MHz (UL), 1432-1517 MHz (DL) |  | 38.101-1, 5.2 |  | Rel-16 |  | pc_nrBand92_Supp |  | NR FDD FR1 Band n92 |  |
| 17 |  | NR Frequency band: 880-915 MHz (UL), 1427-1432 MHz (DL) |  | 38.101-1, 5.2 |  | Rel-16 |  | pc_nrBand93_Supp |  | NR FDD FR1 Band n93 |  |
| 18 |  | NR Frequency band: 880-915 MHz (UL), 1432-1517 MHz (DL) |  | 38.101-1, 5.2 |  | Rel-16 |  | pc_nrBand94_Supp |  | NR FDD FR1 Band n94 |  |

Table A.4.3.1-2: NR TDD FR1 RF Baseline Implementation Capabilities

| Item | NR TDD FR1 RF Baseline Implementation Capabilities | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- |
| 0 | NR Frequency band: 2010-2025 MHz (UL / DL) | 38.101-1, 5.2 | Rel-15 | pc_nrBand34_Supp | NR TDD FR1 Band 34 |
| 0a to 0c | Reserved |  |  |  |  |
| 1 | NR Frequency band: 2570-2620 MHz (UL / DL) | 38.101-1, 5.2 | Rel-15 | pc_nrBand38_Supp | NR TDD FR1 Band n38 |
| 1a | NR Frequency band: 1880-1920 MHz (UL / DL) | 38.101-1, 5.2 | Rel-15 | pc_nrBand39_Supp | NR TDD FR1 Band n39 |
| 1b | NR Frequency band: 2300-2400 MHz (UL / DL) | 38.101-1, 5.2 | Rel-15 | pc_nrBand40_Supp | NR TDD FR1 Band n40 |
| 2 | NR Frequency band: 2496-2690 MHz (UL / DL) | 38.101-1, 5.2 | Rel-15 | pc_nrBand41_Supp | NR TDD FR1 Band n41 |
| 2a to 2d | Reserved |  |  |  |  |
| 2e | NR Frequency band: 5150-5925 MHz (UL / DL) | 38.101-1, 5.2 | Rel-16 | pc_nrBand46_Supp | NR TDD FR1 Band n46 |
| 2f | Reserved |  |  |  |  |
| 2g | NR Frequency band: 3550-3700 MHz (UL / DL) | 38.101-1, 5.2 | Rel-16 | pc_nrBand48_Supp | NR TDD FR1 Band n48 |
| 2h | Reserved |  |  |  |  |
| 2i | NR Frequency band: 1432-1517 MHz (UL / DL) | 38.101-1, 5.2 | Rel-15 | pc_nrBand50_Supp | NR TDD FR1 Band n50 |
| 2j | NR Frequency band: 1427-1432 MHz (UL / DL) | 38.101-1, 5.2 | Rel-15 | pc_nrBand51_Supp | NR TDD FR1 Band n51 |
| 2k | Reserved |  |  |  |  |
| 2l | NR Frequency band: 2483.5-2495 MHz (UL / DL) | 38.101-1, 5.2 | Rel-16 | pc_nrBand53_Supp | NR TDD FR1 Band n53 |
| 3 | NR Frequency band: 3300–4200 MHz (UL / DL) | 38.101-1, 5.2 | Rel-15 | pc_nrBand77_Supp | NR TDD FR1 Band n77 |
| 4 | NR Frequency band: 3300–3800 MHz (UL / DL) | 38.101-1, 5.2 | Rel-15 | pc_nrBand78_Supp | NR TDD FR1 Band n78 |
| 5 | NR Frequency band: 4400–5000 MHz (UL / DL) | 38.101-1, 5.2 | Rel-15 | pc_nrBand79_Supp | NR TDD FR1 Band n79 |
| 6 | NR Frequency band: 2496–2690 MHz (UL / DL) | 38.101-1, 5.2 | Rel-16 | pc_nrBand90_Supp | NR TDD FR1 Band n90 |
| 7-11 | Reserved |  |  |  |  |
| 12 | NR Frequency band: 5925–7125 MHz (UL / DL) | 38.101-1, 5.2 | Rel-16 | pc_nrBand96_Supp | NR TDD FR1 Band n96 |

Table A.4.3.1-3: NR TDD FR2 RF Baseline Implementation Capabilities

| Item | NR TDD FR2 RF Baseline Implementation Capabilities | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | NR Frequency band: 26500-29500 MHz (UL / DL) | 38.101-2, 5.2 | Rel-15 | pc_nrBand257_Supp | NR TDD FR2 Band n257 |
| 2 | NR Frequency band: 24250-27500 MHz (UL / DL) | 38.101-2, 5.2 | Rel-15 | pc_nrBand258_Supp | NR TDD FR2 Band n258 |
| 2a | NR Frequency band: 39500-43500 MHz (UL / DL) | 38.101-2, 5.2 | Rel-16 | pc_nrBand259_Supp | NR TDD FR2 Band n259 |
| 3 | NR Frequency band: 37000–40000 MHz (UL / DL) | 38.101-2, 5.2 | Rel-15 | pc_nrBand260_Supp | NR TDD FR2 Band n260 |
| 4 | NR Frequency band: 27500–28350 MHz (UL / DL) | 38.101-2, 5.2 | Rel-15 | pc_nrBand261_Supp | NR TDD FR2 Band n261 |

Table A.4.3.1-4: NR FR1 PC2 RF Baseline Implementation Capabilities

| Item | NR FR1 PC2 RF Baseline Implementation Capabilities | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- |
| 0 | NR Frequency band: 2300-2400 MHz (UL / DL) | 38.101-1, 6.2.1 | Rel-16 | pc_nrBand40_PC2_Supp | NR FR1 PC2 Band n40 |
| 1 | NR Frequency band: 2496-2690 MHz (UL / DL) | 38.101-1, 6.2.1 | Rel-15 | pc_nrBand41_PC2_Supp | NR FR1 PC2 Band n41 |
| 2 | NR Frequency band: 3300-4200 MHz (UL / DL) | 38.101-1, 6.2.1 | Rel-15 | pc_nrBand77_PC2_Supp | NR FR1 PC2 Band n77 |
| 3 | NR Frequency band: 3300–3800 MHz (UL / DL) | 38.101-1, 6.2.1 | Rel-15 | pc_nrBand78_PC2_Supp | NR FR1 PC2 Band n78 |
| 4 | NR Frequency band: 4400–5000 MHz (UL / DL) | 38.101-1, 6.2.1 | Rel-15 | pc_nrBand79_PC2_Supp | NR FR1 PC2 Band n79 |
| 5 | NR Frequency band: 2010–2025 MHz | 38.101-1, 6.2.1 | Rel-16 | pc_nrBand34_PC2_Supp | NR FR1 PC2 Band 34 |
| 6 | NR Frequency band: 1880–1920 MHz | 38.101-1, 6.2.1 | Rel-16 | pc_nrBand39_PC2_Supp | NR FR1 PC2 Band 39 |

Table A.4.3.1-4a: NR FR2 PC2 RF Baseline Implementation Capabilities

| Item | NR FR2 PC2 RF Baseline Implementation Capabilities | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | NR Frequency band: 26500-29500 MHz (UL / DL) | 38.101-2, 6.2.1 | Rel-15 | pc_nrBand257_PC2_Supp | NR FR2 PC2 Band n257 |
| 2 | NR Frequency band: 24250-27500 MHz (UL / DL) | 38.101-2, 6.2.1 | Rel-15 | pc_nrBand258_PC2_Supp | NR FR2 PC2 Band n258 |
| 3 | NR Frequency band: 27500–28350 MHz (UL / DL) | 38.101-2, 6.2.1 | Rel-15 | pc_nrBand261_PC2_Supp | NR FR2 PC2 Band n261 |

Table A.4.3.1-4b: NR FR1 PC1 RF Baseline Implementation Capabilities

| Item | NR FR1 PC1 RF Baseline Implementation Capabilities | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | NR Frequency band: 788-798 MHz (UL), 758-768 MHz (DL) | 38.101-1, 6.2.1 | Rel-16 | pc_nrBand14_PC1_Supp | NR FR1 PC1 Band n14 |

Table A.4.3.1-4c: NR FR2 PC1 RF Baseline Implementation Capabilities

| Item | NR FR2 PC1 RF Baseline Implementation Capabilities | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | NR Frequency band: 26500-29500 MHz (UL / DL) | 38.101-2, 6.2.1 | Rel-15 | pc_nrBand257_PC1_Supp | NR FR2 PC1 Band n257 |
| 2 | NR Frequency band: 24250-27500 MHz (UL / DL) | 38.101-2, 6.2.1 | Rel-15 | pc_nrBand258_PC1_Supp | NR FR2 PC1 Band n258 |
| 3 | NR Frequency band: 37000-40000 MHz (UL / DL) | 38.101-2, 6.2.1 | Rel-15 | pc_nrBand260_PC1_Supp | NR FR2 PC1 Band n260 |
| 4 | NR Frequency band: 27500–28350 MHz (UL / DL) | 38.101-2, 6.2.1 | Rel-15 | pc_nrBand261_PC1_Supp | NR FR2 PC1 Band n261 |

Table A.4.3.1-4d: NR FR2 PC4 RF Baseline Implementation Capabilities

| Item | NR FR2 PC4 RF Baseline Implementation Capabilities | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | NR Frequency band: 26500-29500 MHz (UL / DL) | 38.101-2, 6.2.1 | Rel-15 | pc_nrBand257_PC4_Supp | NR FR2 PC4 Band n257 |
| 2 | NR Frequency band: 24250-27500 MHz (UL / DL) | 38.101-2, 6.2.1 | Rel-15 | pc_nrBand258_PC4_Supp | NR FR2 PC4 Band n258 |
| 3 | NR Frequency band: 37000-40000 MHz (UL / DL) | 38.101-2, 6.2.1 | Rel-15 | pc_nrBand260_PC4_Supp | NR FR2 PC4 Band n260 |
| 4 | NR Frequency band: 27500–28350 MHz (UL / DL) | 38.101-2, 6.2.1 | Rel-15 | pc_nrBand261_PC4_Supp | NR FR2 PC4 Band n261 |

Table A.4.3.1-4e: NR FR1 PC1.5 RF Baseline Implementation Capabilities

| Item | NR FR1 PC1.5 RF Baseline Implementation Capabilities | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | NR Frequency band: 3300-4200 MHz | 38.101-1, 6.2.1 | Rel-15 | pc_nrBand77_PC1.5_Supp | NR FR1 PC1.5 Band 77 |
| 2 | NR Frequency band: 3300–3800 MHz | 38.101-1, 6.2.1 | Rel-15 | pc_nrBand78_PC1.5_Supp | NR FR1 PC1.5 Band 78 |
| 3 | NR Frequency band: 4400-5000 MHz | 38.101-1, 6.2.1 | Rel-15 | pc_nrBand79_PC1.5_Supp | NR FR1 PC1.5 Band 79 |
| 4 | NR Frequency band: 2496-2690 MHz | 38.101-1, 6.2.1 | Rel-15 | pc_nrBand41_PC1.5_Supp | NR FR1 PC1.5 Band 41 |

Table A.4.3.1-4f: NR FR1 maxNumberSRS-Ports-PerResource RF Baseline Implementation Capabilities (Rel-15)

| Item | NR Band | UE Physical Layer Baseline Implementation Capabilities | Ref. | Release | Mnemonic | Supported maxNumberSRS-Ports-PerResource |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | n41 | NR_n41 maxNumberSRS-Ports-PerResource | 38.306, 4.2.7.7 | Rel-15 | pc_nrBand41_maxNumberSRS-Ports-PerResource_r15 |  |

Table A.4.3.1-5: NR SUL FR1 RF Baseline Implementation Capabilities

| Item |  | NR SUL FR1 RF Baseline Implementation Capabilities |  | Ref. |  | Release |  | Mnemonic |  | Comments |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  | NR Frequency band: 1710-1785 MHz (UL) |  | 38.101-1, 5.2 |  | Rel-15 |  | pc_nrBand80_Supp |  | NR SUL FR1 Band n80 |  |
| 2 |  | NR Frequency band: 880-915 MHz (UL) |  | 38.101-1, 5.2 |  | Rel-15 |  | pc_nrBand81_Supp |  | NR SUL FR1 Band n81 |  |
| 3 |  | NR Frequency band: 832-862 MHz (UL) |  | 38.101-1, 5.2 |  | Rel-15 |  | pc_nrBand82_Supp |  | NR SUL FR1 Band n82 |  |
| 4 |  | NR Frequency band: 703-748 MHz (UL) |  | 38.101-1, 5.2 |  | Rel-15 |  | pc_nrBand83_Supp |  | NR SUL FR1 Band n83 |  |
| 5 |  | NR Frequency band: 1920-1980 MHz (UL) |  | 38.101-1, 5.2 |  | Rel-15 |  | pc_nrBand84_Supp |  | NR SUL FR1 Band n84 |  |
| 6 |  | NR Frequency band: 1710-1780 MHz (UL) |  | 38.101-1, 5.2 |  | Rel-15 |  | pc_nrBand86_Supp |  | NR SUL FR1 Band n86 |  |
| 6a to 6b |  | Reserved |  |  |  |  |  |  |  |  |  |
| 6c |  | NR Frequency band: 824-849 MHz (UL) |  | 38.101-1, 5.2 |  | Rel-16 |  | pc_nrBand89_Supp |  | NR SUL FR1 Band n89 |  |
| 7 |  | NR Frequency band: 2010-2025 MHz (UL) |  | 38.101-1, 5.2 |  | Rel-16 |  | pc_nrBand95_Supp |  | NR SUL FR1 Band n95 |  |
| 8 |  | NR Frequency band: 2300 MHz – 2400 MHz (UL) |  | 38.101-1, 5.2 |  | Rel-17 |  | pc_nrBand97_Supp |  | NR SUL FR1 Band n97 |  |
| 9 |  | NR Frequency band: 1626.5-1660.5 MHz (UL) |  | 38.101-1, 5.2 |  | Rel-17 |  | pc_nrBand99_Supp |  | NR SUL FR1 Band n99 |  |

<< Unchanged sections omitted >>

A.4.3.2B.2.1	Intra-band contiguous EN-DC

Table A.4.3.2B.2.1-1: Downlink Bandwidth Class Combination capabilities for Intra-band contiguous EN-DC configurations (for one or more of the supported configurations in Table A.4.3.2B.2.1-2)

| Item | DL Intra-band contiguous EN-DC Bandwidth Class | Ref. | Mnemonic | Comments |
| --- | --- | --- | --- | --- |
| 1 | DL Intra-band contiguous EN-DC BW Class Combination AA | 36.101, 5.6A.1 38.101-3, 5.3B.1.2 | pc_DL_intra_contiguous_EN_DC_Class_AA |  |
| 2 | DL Intra-band contiguous EN-DC BW Class Combination CA | 36.101, 5.6A.1 38.101-3, 5.3B.1.2 | pc_DL_intra_contiguous_EN_DC_Class_CA |  |
| 3 | DL Intra-band contiguous EN-DC BW Class Combination DA | 36.101, 5.6A.1 38.101-3, 5.3B.1.2 | pc_DL_intra_contiguous_EN_DC_Class_DA |  |

Table A.4.3.2B.2.1-1a: Uplink Bandwidth Class Combination capabilities for Intra-band contiguous EN-DC configurations (for one or more of the supported configurations in Table A.4.3.2B.2.1-2)

| Item | UL Intra-band contiguous EN-DC Bandwidth Class | Ref. | Mnemonic | Comments |
| --- | --- | --- | --- | --- |
| 1 | UL Intra-band contiguous EN-DC BW Class Combination AA | 36.101, 5.6A.1 38.101-3, 5.3B.1.2 | pc_UL_intra_contiguous_EN_DC_Class_AA |  |
| 2 | UL Intra-band contiguous EN-DC BW Class Combination A_A | 36.101, 5.6A.1 38.101-3, 5.3B.1.2 | pc_UL_intra_contiguous_EN_DC_Class_A_A |  |

Table A.4.3.2B.2.1-2: Supported Intra-band contiguous EN-DC configurations

| EN-DC configuration / Item (Note 1, 3) | Release | Supported | Supported EN-DC Bandwidth Class(es) in UL (Note 2) | Supported Bandwidth Combination Set(s) |
| --- | --- | --- | --- | --- |
| DC_(n)41AA | Rel-15 |  |  |  |
| DC_(n)71AA | Rel-15 |  |  |  |
| Note 1: Notation used for intra-band contiguous EN-DC Bands is according to TS 38.101-3 [25] Table 5.3B.1.2-1, e.g. ‘DC_(n)41AA’ indicates contiguous EN-DC operation on E-UTRA band 41 with DL Bandwidth Class A and NR band n41 with DL CA Bandwidth Class A. Note 2: See UL_nCC(table_index) in Note 2 of Table 4.0-3 in TS 38.522 [9]. Note 3: See DL_nCC(table_index) in Note 4 of Table 4.0-3 in TS 38.522 [9]. |  |  |  |  |

Table A.4.3.2B.2.1-3: Intra-band contiguous EN-DC PC2 UE RF Baseline Implementation Capabilities

| Item | Intra-band contiguous EN-DC PC2 UE RF Baseline Implementation Capabilities | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | LTE Frequency band: 2496-2690 MHz NR Frequency band: 2496-2690 MHz | 38.101-3, 6.2B.1.1 | Rel-15 | pc_Band41_nrBand41_C_PC2_Supp | DC_(n)41AA |

Table A.4.3.2B.2.1-4: Intra-band contiguous EN-DC NR part power class UE RF Baseline Implementation Capabilities (Rel-16 and forward)

| Item | EN-DC configuration | UE Physical Layer Baseline Implementation Capabilities | Ref. | Release | Mnemonic | Supported NR part power class |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | DC_(n)41AA | DC_(n)41AA NR part power class | 38.306, 4.2.7.1 | Rel-16 | pc_Band41_nrBand41_C_powerClassNRPart_r16 |  |

Table A.4.3.2B.2.1-4a: Intra-band contiguous EN-DC maxNumberSRS-Ports-PerResource UE RF Baseline Implementation Capabilities (Rel-15)

| Item | EN-DC configuration | UE Physical Layer Baseline Implementation Capabilities | Ref. | Release | Mnemonic | Supported maxNumberSRS-Ports-PerResource |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | DC_(n)41AA | DC_(n)41AA maxNumberSRS-Ports-PerResource on NR band | 38.306, 4.2.7.7 | Rel-15 | pc_Band41_nrBand41_C_maxNumberSRS-Ports-PerResource_NR_r15 |  |

Table A.4.3.2B.2.1-4b: Intra-band contiguous EN-DC NR part power class UE RF Baseline Implementation Capabilities (Rel-15) (maxNumberSRS-Ports-PerResource=n2 in NR standalone operation mode, maxNumberSRS-Ports-PerResource=n1 for EN-DC on NR band)

| Item | EN-DC configuration | UE Physical Layer Baseline Implementation Capabilities | Ref. | Release | Mnemonic | Supported NR part power class |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | DC_(n)41AA | DC_(n)41AA NR part power class | 38.101-3, 6.1 | Rel-15 | pc_Band41_nrBand41_C_powerClassNRPart_r15 |  |

A.4.3.2B.2.2	Intra-band non-contiguous EN-DC

Table A.4.3.2B.2.2-1: Downlink Bandwidth Class Combination capabilities for Intra-band non-contiguous EN-DC configurations (for one or more of the supported configurations in Table A.4.3.2B.2.2-2)

| Item | DL Intra-band non-contiguous EN-DC Bandwidth Class | Ref. | Mnemonic | Comments |
| --- | --- | --- | --- | --- |
| 1 | DL Intra-band non-contiguous EN-DC BW Class Combination A_A | 36.101, 5.6A.1 38.101-3, 5.3B.1.3 | pc_DL_intra_non_contiguous_EN_DC_Class_A_A |  |
| 2 | DL Intra-band non-contiguous EN-DC BW Class Combination A_AA | 36.101, 5.6A.1 38.101-3, 5.3B.1.3 | pc_DL_intra_non_contiguous_EN_DC_Class_A_AA |  |
| 3 | DL Intra-band non-contiguous EN-DC BW Class Combination A-A_A | 36.101, 5.6A.1 38.101-3, 5.3B.1.3 | pc_DL_intra_non_contiguous_EN_DC_Class_A-A_A |  |
| 4 | DL Intra-band non-contiguous EN-DC BW Class Combination C_A | 36.101, 5.6A.1 38.101-3, 5.3B.1.3 | pc_DL_intra_non_contiguous_EN_DC_Class_C_A |  |
| 5 | DL Intra-band non-contiguous EN-DC BW Class Combination D_A | 36.101, 5.6A.1 38.101-3, 5.3B.1.3 | pc_DL_intra_non_contiguous_EN_DC_Class_D_A |  |

Table A.4.3.2B.2.2-1a: Uplink Bandwidth Class Combination capabilities for Intra-band non-contiguous EN-DC configurations (for one or more of the supported configurations in Table A.4.3.2B.2.2-2)

| Item | UL Intra-band non-contiguous EN-DC Bandwidth Class | Ref. | Mnemonic | Comments |
| --- | --- | --- | --- | --- |
| 1 | UL Intra-band non-contiguous EN-DC BW Class Combination A_A | 36.101, 5.6A.1 38.101-3, 5.3B.1.3 | pc_UL_intra_non_contiguous_EN_DC_Class_A_A |  |
| 2 | UL Intra-band non-contiguous EN-DC BW Class Combination AA | 36.101, 5.6A.1 38.101-3, 5.3B.1.3 | pc_UL_intra_non_contiguous_EN_DC_Class_AA |  |

Table A.4.3.2B.2.2-2: Supported Intra-band non-contiguous EN-DC configurations

| EN-DC configuration / Item (Note 1, 3, 4) |  | Release |  | Supported |  | Supported EN-DC Bandwidth Class(es) in UL (Note 2) |  | Supported Bandwidth Combination Set(s) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_2A_n2A6 |  | Rel-16 |  |  |  |  |  |  |  |
| DC_3A_n3A5 |  | Rel-15 |  |  |  |  |  |  |  |
| DC_41A_n41A |  | Rel-15 |  |  |  |  |  |  |  |
| DC_41C_n41A | Rel-15 |  |  |  |
| DC_41D_n41A |  | Rel-15 |  |  |  |  |  |  |  |
| DC_66A_n66A6 | Rel-16 |  |  |  |
| Note 1: Notation used for intra-band non-contiguous EN-DC Bands is according to TS 38.101-3 [25] Table 5.3B.1.3-1, e.g. ‘DC_41A_n41A’ indicates non-contiguous EN-DC operation on E-UTRA band 41 with DL Bandwidth Class A and NR band n41 with DL CA Bandwidth Class A. Note 2: See UL_nCC(table_index) in Note 2 of Table 4.0-3 in TS 38.522 [9]. Note 3: See DL_nCC(table_index) in Note 4 of Table 4.0-3 in TS 38.522 [9]. Note 4: See DL_NR_nCC(table_index) in Note 5 of Table 4.0-3 in TS 38.522 [9]. Note 5: Only single switched UL is supported in Rel-15. Note 6: Only single switched UL is supported. |  |  |  |  |

Table A.4.3.2B.2.2-3: Intra-band non-contiguous EN-DC PC2 UE RF Baseline Implementation Capabilities

| Item | Intra-band non-contiguous EN-DC PC2 UE RF Baseline Implementation Capabilities | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | LTE Frequency band: 2496-2690 MHz NR Frequency band: 2496-2690 MHz | 38.101-3, 6.2B.1.2 | Rel-15 | pc_Band41_nrBand41_NC_PC2_Supp | DC_41A_n41A |

Table A.4.3.2B.2.2-4: Intra-band non-contiguous EN-DC NR part power class UE RF Baseline Implementation Capabilities (Rel-16 and forward)

| Item | EN-DC configuration | UE Physical Layer Baseline Implementation Capabilities | Ref. | Release | Mnemonic | Supported NR part power class |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | DC_41A_n41A | DC_41A_n41A NR part power class | 38.306, 4.2.7.1 | Rel-16 | pc_Band41_nrBand41_NC_powerClassNRPart_r16 |  |

Table A.4.3.2B.2.2-4a: Intra-band non-contiguous EN-DC maxNumberSRS-Ports-PerResource UE RF Baseline Implementation Capabilities (Rel-15)

| Item | EN-DC configuration | UE Physical Layer Baseline Implementation Capabilities | Ref. | Release | Mnemonic | Supported maxNumberSRS-Ports-PerResource |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | DC_41A_n41A | DC_41A_n41A maxNumberSRS-Ports-PerResource on NR band | 38.306, 4.2.7.7 | Rel-15 | pc_Band41_nrBand41_NC_maxNumberSRS-Ports-PerResource_NR_r15 |  |

Table A.4.3.2B.2.2-4b: Intra-band non-contiguous EN-DC NR part power class UE RF Baseline Implementation Capabilities (Rel-15) (maxNumberSRS-Ports-PerResource=n2 in NR standalone operation mode, maxNumberSRS-Ports-PerResource=n1 for EN-DC on NR band)

| Item | EN-DC configuration | UE Physical Layer Baseline Implementation Capabilities | Ref. | Release | Mnemonic | Supported NR part power class |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | DC_41A_n41A | DC_41A_n41A NR part power class | 38.101-3, 6.1 | Rel-15 | pc_Band41_nrBand41_NC_powerClassNRPart_r15 |  |

A.4.3.2B.2.3	Inter-band EN-DC

A.4.3.2B.2.3.1	Inter-band EN-DC within FR1 (two bands)

Table A.4.3.2B.2.3.1-1: Downlink Bandwidth Class Combination capabilities for Inter-band EN-DC within FR1 and two bands (for one or more of the supported DC configurations in Table A.4.3.2B.2.3.1-2)

| Item | DL inter-band EN-DC within FR1 Bandwidth Class | Ref. | Mnemonic | Comments |
| --- | --- | --- | --- | --- |
| 1 | Inter-band EN-DC within FR1 BW Class Combination A_A (two bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4.1 | pc_DL_inter_band_EN_DC_FR1_2B_Class_A_A |  |
| 2 | Inter-band EN-DC within FR1 BW Class Combination A_(2A) (two bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4.1 | pc_DL_inter_band_EN_DC_FR1_2B_Class_A_(2A) |  |
| 3 | Inter-band EN-DC within FR1 BW Class Combination A_B (two bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4.1 | pc_DL_inter_band_EN_DC_FR1_2B_Class_A_B |  |
| 4 | Inter-band EN-DC within FR1 BW Class Combination A_C (two bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4.1 | pc_DL_inter_band_EN_DC_FR1_2B_Class_A_C |  |
| 5 | Inter-band EN-DC within FR1 BW Class Combination (2A)_A (two bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4.1 | pc_DL_inter_band_EN_DC_FR1_2B_Class_(2A)_A |  |
| 6 | Inter-band EN-DC within FR1 BW Class Combination (2A)_(2A) (two bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4.1 | pc_DL_inter_band_EN_DC_FR1_2B_Class_(2A)_(2A) |  |
| 7 | Inter-band EN-DC within FR1 BW Class Combination (2A)_B (two bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4.1 | pc_DL_inter_band_EN_DC_FR1_2B_Class_(2A)_B |  |
| 8 | Inter-band EN-DC within FR1 BW Class Combination (3A)_A (two bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4.1 | pc_DL_inter_band_EN_DC_FR1_2B_Class_(3A)_A |  |
| 9 | Inter-band EN-DC within FR1 BW Class Combination B_A (two bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4.1 | pc_DL_inter_band_EN_DC_FR1_2B_Class_B_A |  |
| 10 | Inter-band EN-DC within FR1 BW Class Combination C_A (two bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4.1 | pc_DL_inter_band_EN_DC_FR1_2B_Class_C_A |  |
| 11 | Inter-band EN-DC within FR1 BW Class Combination C_(2A) (two bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4.1 | pc_DL_inter_band_EN_DC_FR1_2B_Class_C_(2A) |  |
| 12 | Inter-band EN-DC within FR1 BW Class Combination C_B (two bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4.1 | pc_DL_inter_band_EN_DC_FR1_2B_Class_C_B |  |
| 13 | Inter-band EN-DC within FR1 BW Class Combination C_C (two bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4.1 | pc_DL_inter_band_EN_DC_FR1_2B_Class_C_C |  |
| 14 | Inter-band EN-DC within FR1 BW Class Combination D_A (two bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4.1 | pc_DL_inter_band_EN_DC_FR1_2B_Class_D_A |  |
| 15 | Inter-band EN-DC within FR1 BW Class Combination D_C (two bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4.1 | pc_DL_inter_band_EN_DC_FR1_2B_Class_D_C |  |
| 16 | Inter-band EN-DC within FR1 BW Class Combination E_A (two bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4.1 | pc_DL_inter_band_EN_DC_FR1_2B_Class_E_A |  |
| 17 | Inter-band EN-DC within FR1 BW Class Combination E_C (two bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4.1 | pc_DL_inter_band_EN_DC_FR1_2B_Class_E_C |  |

Table A.4.3.2B.2.3.1-1a: Uplink Bandwidth Class Combination capabilities for Inter-band EN-DC within FR1 and two bands (for one or more of the supported configurations in Table A.4.3.2B.2.3.1-2)

| Item | UL inter-band EN-DC within FR1 Bandwidth Class | Ref. | Mnemonic | Comments |
| --- | --- | --- | --- | --- |
| 1 | UL Inter-band EN-DC within FR1 BW Class Combination A_A (two bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4.1 | pc_UL_inter_band_EN_DC_FR1_2B_Class_A_A |  |
| 2 | UL Inter-band EN-DC within FR1 BW Class Combination A_B (two bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4.1 | pc_UL_inter_band_EN_DC_FR1_2B_Class_A_B |  |
| 3 | UL Inter-band EN-DC within FR1 BW Class Combination A_C (two bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4.1 | pc_UL_inter_band_EN_DC_FR1_2B_Class_A_C |  |
| 4 | UL Inter-band EN-DC within FR1 BW Class Combination (2A)_A (two bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4.1 | pc_UL_inter_band_EN_DC_FR1_2B_Class_(2A)_A |  |
| 5 | UL Inter-band EN-DC within FR1 BW Class Combination C_A (two bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4.1 | pc_UL_inter_band_EN_DC_FR1_2B_Class_C_A |  |

Table A.4.3.2B.2.3.1-2: Supported Inter-band EN-DC configurations within FR1 (two bands)

| EN-DC configuration / Item | Release | Supported | Supported EN-DC Bandwidth Class(es) in UL | Supported ULTxSwitching Band Pair |
| --- | --- | --- | --- | --- |
| DC_1A_n3A | Rel-16 |  |  |  |
| DC_1A_n5A | Rel-16 |  |  |  |
| DC_1A_n7A | Rel-16 |  |  |  |
| DC_1A_n28A | Rel-15 |  |  |  |
| DC_1A_n40A | Rel-15 |  |  |  |
| DC_1A_n51A | Rel-15 |  |  |  |
| DC_1A_n77A | Rel-15 |  |  |  |
| DC_1A_n78A | Rel-15 |  |  |  |
| DC_1A_n78C | Rel-15 |  |  |  |
| DC_1A-1A_n78A | Rel-17 |  |  |  |
| DC_1A_n79A | Rel-15 |  |  |  |
| DC_2A_n5A | Rel-15 |  |  |  |
| DC_2A_n41A | Rel-16 |  |  |  |
| DC_2C_n41A | Rel-16 |  |  |  |
| DC_2A_n66A | Rel-15 |  |  |  |
| DC_2A_n71A | Rel-15 |  |  |  |
| DC_2A_n77A | Rel-17 |  |  |  |
| DC_2A_n78A | Rel-15 |  |  |  |
| DC_3A_n1A | Rel-16 |  |  |  |
| DC_3A_n5A | Rel-16 |  |  |  |
| DC_3A_n7A | Rel-15 |  |  |  |
| DC_3A_n28A | Rel-15 |  |  |  |
| DC_3A_n41A | Rel-16 |  |  |  |
| DC_3A_n51A | Rel-15 |  |  |  |
| DC_3A_n77A | Rel-15 |  |  |  |
| DC_3A_n78A | Rel-15 |  |  |  |
| DC_3A_n78C | Rel-15 |  |  |  |
| DC_3A_n79A | Rel-15 |  |  |  |
| DC_3C_n78A | Rel-15 |  |  |  |
| DC_3A_n82A | Rel-15 |  |  |  |
| DC_5A_n2A | Rel-16 |  |  |  |
| DC_5A_n40A | Rel-15 |  |  |  |
| DC_5A_n66A | Rel-15 |  |  |  |
| DC_5A_n77A | Rel-17 |  |  |  |
| DC_5A_n78A | Rel-15 |  |  |  |
| DC_5A_n78C | Rel-17 |  |  |  |
| DC_7A_n1A | Rel-16 |  |  |  |
| DC_7A_n3A | Rel-16 |  |  |  |
| DC_7A_n5A | Rel-16 |  |  |  |
| DC_7A_n28A | Rel-15 |  |  |  |
| DC_7A_n51A | Rel-15 |  |  |  |
| DC_7A_n78A | Rel-15 |  |  |  |
| DC_7A_n66A | Rel-15 |  |  |  |
| DC_7C_n66A | Rel-15 |  |  |  |
| DC_7C_n78A | Rel-15 |  |  |  |
| DC_8A_n1A | Rel-16 |  |  |  |
| DC_8A_n3A | Rel-16 |  |  |  |
| DC_8A_n20A | Rel-16 |  |  |  |
| DC_8A_n40A | Rel-15 |  |  |  |
| DC_8A_n41A | Rel-16 |  |  |  |
| DC_8A_n77A | Rel-15 |  |  |  |
| DC_8A_n78A | Rel-15 |  |  |  |
| DC_8A_n79A | Rel-15 |  |  |  |
| DC_11A_n77A | Rel-15 |  |  |  |
| DC_11A_n78A | Rel-15 |  |  |  |
| DC_11A_n79A | Rel-15 |  |  |  |
| DC_12A_n5A | Rel-15 |  |  |  |
| DC_12A_n66A | Rel-15 |  |  |  |
| DC_12A_n78A | Rel-16 |  |  |  |
| DC_13A_n2A | Rel-16 |  |  |  |
| DC_13A_n66A | Rel-15 |  |  |  |
| DC_13A_n77A | Rel-17 |  |  |  |
| DC_14A_n2A | Rel-16 |  |  |  |
| DC_14A_n66A | Rel-16 |  |  |  |
| DC_18A_n77A |  | Rel-15 |  |  |  |  |  |  |  |
| DC_18A_n78A |  | Rel-15 |  |  |  |  |  |  |  |
| DC_18A_n79A |  | Rel-15 |  |  |  |  |  |  |  |
| DC_19A_n1A | Rel-17 |  |  |  |
| DC_19A_n77A | Rel-15 |  |  |  |
| DC_19A_n77(2A) | Rel-17 |  |  |  |
| DC_19A_n78A | Rel-15 |  |  |  |
| DC_19A_n78(2A) | Rel-17 |  |  |  |
| DC_19A_n79A | Rel-15 |  |  |  |
| DC_20A_n1A |  | Rel-16 |  |  |  |  |  |  |  |
| DC_20A_n3A |  | Rel-16 |  |  |  |  |  |  |  |
| DC_20A_n7A | Rel-16 |  |  |  |
| DC_20A_n8A | Rel-15 |  |  |  |
| DC_20A_n28A | Rel-15 |  |  |  |
| DC_20A_n51A |  | Rel-15 |  |  |  |  |  |  |  |
| DC_20A_n77A |  | Rel-15 |  |  |  |  |  |  |  |
| DC_20A_n78A | Rel-15 |  |  |  |
| DC_20A_n83A | Rel-16 |  |  |  |
| DC_21A_n1A | Rel-17 |  |  |  |
| DC_21A_n28A | Rel-17 |  |  |  |
| DC_21A_n77A | Rel-15 |  |  |  |
| DC_21A_n77(2A) | Rel-17 |  |  |  |
| DC_21A_n78A | Rel-15 |  |  |  |
| DC_21A_n78(2A) | Rel-17 |  |  |  |
| DC_21A_n79A | Rel-15 |  |  |  |
| DC_25A_n41A | Rel-15 |  |  |  |
| DC_26A_n41A | Rel-16 |  |  |  |
| DC_26A_n77A | Rel-16 |  |  |  |
| DC_26A_n78A | Rel-16 |  |  |  |
| DC_26A_n79A | Rel-16 |  |  |  |
| DC_28A_n3A | Rel-16 |  |  |  |
| DC_28A_n5A | Rel-16 |  |  |  |
| DC_28A_n7A | Rel-16 |  |  |  |
| DC_28A n51A | Rel-15 |  |  |  |
| DC_28A_n77A | Rel-15 |  |  |  |
| DC_28A_n78A | Rel-15 |  |  |  |
| DC_28A_n79A | Rel-15 |  |  |  |
| DC_30A_n5A | Rel-15 |  |  |  |
| DC_30A_n66A | Rel-15 |  |  |  |
| DC_38A_n78A | Rel-15 |  |  |  |
| DC_39A_n41A | Rel-16 |  |  |  |
| DC_39A_n78A | Rel-15 |  |  |  |
| DC_39A_n79A | Rel-15 |  |  |  |
| DC_40A_n1A | Rel-16 |  |  |  |
| DC_40A_n41A | Rel-16 |  |  |  |
| DC_40A_n77A | Rel-15 |  |  |  |
| DC_40A_n78A | Rel-16 |  |  |  |
| DC_40A_n79A | Rel-16 |  |  |  |
| DC_40C_n78A | Rel-16 |  |  |  |
| DC_40C_n79A | Rel-16 |  |  |  |
| DC_41A_n77A | Rel-16 |  |  |  |
| DC_41A_n78A | Rel-16 |  |  |  |
| DC_41A_n79A | Rel-15 |  |  |  |
| DC_42A_n1A | Rel-17 |  |  |  |
| DC_42C_n1A | Rel-17 |  |  |  |
| DC_42A_n51A |  | Rel-15 |  |  |  |  |  |  |  |
| DC_42A_n77A |  | Rel-15 |  |  |  |  |  |  |  |
| DC_42A_n78A |  | Rel-15 |  |  |  |  |  |  |  |
| DC_42A_n79A |  | Rel-15 |  |  |  |  |  |  |  |
| DC_48A_n5A | Rel-16 |  |  |  |
| DC_48A_n66A | Rel-16 |  |  |  |
| DC_66A_n2A | Rel-16 |  |  |  |
| DC_66A_n5A | Rel-15 |  |  |  |
| DC_66A_n41A | Rel-16 |  |  |  |
| DC_66A_n71A | Rel-15 |  |  |  |
| DC_66A_n77A | Rel-17 |  |  |  |
| DC_66A_n78A | Rel-15 |  |  |  |

Table A.4.3.2B.2.3.1-3: Inter-band EN-DC within FR1 (two bands) PC2 UE RF Baseline Implementation Capabilities

| Item | EN-DC configuration | Inter-band EN-DC within FR1 (two bands) PC2 UE RF Baseline Implementation Capabilities | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | DC_39A_n41A | LTE Frequency band: 1880-1920 MHz NR Frequency band: 2496-2690 MHz | 38.101-3, 6.2B.1.3 | Rel-16 | pc_Band39_nrBand41_ PC2_Supp |  |
| 2 | DC_39A_n79A | LTE Frequency band: 1880-1920 MHz NR Frequency band: 4400-5000 MHz | 38.101-3, 6.2B.1.3 | Rel-16 | pc_Band39_nrBand79_ PC2_Supp |  |
| 3 | DC_41A_n79A | LTE Frequency band: 2496-2690 MHz NR Frequency band: 4400-5000 MHz | 38.101-3, 6.2B.1.3 | Rel-16 | pc_Band41_nrBand79_ PC2_Supp |  |
| 4 | DC_3A_n78A | LTE Frequency band: 1710-1785 MHz (UL), 1805-1880 MHz (DL) NR Frequency band: 3300-3800 MHz | 38.101-3, 6.2B.1.3 | Rel-16 | pc_Band3_nrBand78_ PC2_Supp |  |
| 5 | DC_3A_n41A | LTE Frequency band: 1710-1785 MHz (UL), 1805-1880 MHz (DL) NR Frequency band: 2496-2690 MHz | 38.101-3, 6.2B.1.3 | Rel-16 | pc_Band3_nrBand41_ PC2_Supp |  |
| 6 | DC_1A_n78A | LTE Frequency band: 1920-1980 MHz (UL),2110- 2170 MHz (DL) NR Frequency band: 3300-3800 MHz | 38.101-3, 6.2B.1.3 | Rel-17 | pc_Band1_nrBand78_ PC2_Supp |  |
| 7 | Void |  |  |  |  |  |
| 8 | DC_8A_n78A | LTE Frequency band: 703-748 MHz (UL), 758-803 MHz (DL) NR Frequency band: 3300-3800 MHz | 38.101-3, 6.2B.1.3 | Rel-17 | pc_Band8_nrBand78_ PC2_Supp |  |

Table A.4.3.2B.2.3.1-3a: Inter-band EN-DC within FR1 (two bands) NR part power class UE RF Baseline Implementation Capabilities (Rel-16 and forward)

| Item |  | EN-DC configuration |  | UE Physical Layer Baseline Implementation Capabilities |  | Ref. |  | Release |  | Mnemonic |  | Supported NR part power class |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  | DC_39A_n41A |  | DC_39A_n41A NR part power class |  | 38.306, 4.2.7.1 |  | Rel-16 |  | pc_Band39_nrBand41_powerClassNRPart_r16 |  |  |  |
| 2 |  | DC_39A_n79A |  | DC_39A_n79A NR part power class |  | 38.306, 4.2.7.1 |  | Rel-16 |  | pc_Band39_nrBand79_powerClassNRPart_r16 |  |  |  |
| 3 |  | DC_41A_n79A |  | DC_41A_n79A NR part power class |  | 38.306, 4.2.7.1 |  | Rel-16 |  | pc_Band41_nrBand79_powerClassNRPart_r16 |  |  |  |
| 4 |  | DC_3A_n78A |  | DC_3A_n78A NR part power class |  | 38.306, 4.2.7.1 |  | Rel-16 |  | pc_Band3_nrBand78_powerClassNRPart_r16 |  |  |  |
| 5 |  | DC_3A_n41A |  | DC_3A_n41A NR part power class |  | 38.306, 4.2.7.1 |  | Rel-16 |  | pc_Band3_nrBand41_powerClassNRPart_r16 |  |  |  |
| 6 |  | DC_1A_n78A |  | DC_1A_n78A NR part power class |  | 38.306, 4.2.7.1 |  | Rel-17 |  | pc_Band1_nrBand78_powerClassNRPart_r17 |  |  |  |
| 7 |  | DC_8A_n78A |  | DC_8A_n78A NR part power class |  | 38.306, 4.2.7.1 |  | Rel-17 |  | pc_Band8_nrBand78_powerClassNRPart_r17 |  |  |  |

Table A.4.3.2B.2.3.1-3b: Inter-band EN-DC within FR1 (two bands) maxNumberSRS-Ports-PerResource UE RF Baseline Implementation Capabilities (Rel-15)

| Item | EN-DC configuration | UE Physical Layer Baseline Implementation Capabilities | Ref. | Release | Mnemonic | Supported maxNumberSRS-Ports-PerResource |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | DC_39A_n41A | DC_39A_n41A maxNumberSRS-Ports-PerResource on NR band | 38.306, 4.2.7.7 | Rel-15 | pc_Band39_nrBand41_maxNumberSRS-Ports-PerResource_NR_r15 |  |
| 2 | DC_39A_n79A | DC_39A_n79A maxNumberSRS-Ports-PerResource on NR band | 38.306, 4.2.7.7 | Rel-15 | pc_Band39_nrBand79_maxNumberSRS-Ports-PerResource_NR_r15 |  |
| 3 | DC_41A_n79A | DC_41A_n79A maxNumberSRS-Ports-PerResource on NR band | 38.306, 4.2.7.7 | Rel-15 | pc_Band41_nrBand79_maxNumberSRS-Ports-PerResource_NR_r15 |  |
| 4 | DC_3A_n78A | DC_3A_n78A maxNumberSRS-Ports-PerResource on NR band | 38.306, 4.2.7.7 | Rel-15 | pc_Band3_nrBand78_maxNumberSRS-Ports-PerResource_NR_r15 |  |
| 5 | DC_3A_n41A | DC_3A_n41A maxNumberSRS-Ports-PerResource on NR band | 38.306, 4.2.7.7 | Rel-15 | pc_Band3_nrBand41_maxNumberSRS-Ports-PerResource_NR_r15 |  |
| 6 | DC_1A_n78A | DC_1A_n78A maxNumberSRS-Ports-PerResource on NR band | 38.306, 4.2.7.7 | Rel-15 | pc_Band1_nrBand78_maxNumberSRS-Ports-PerResource_NR_r15 |  |
| 7 | DC_8A_n78A | DC_8A_n78A maxNumberSRS-Ports-PerResource on NR band | 38.306, 4.2.7.7 | Rel-15 | pc_Band8_nrBand78_maxNumberSRS-Ports-PerResource_NR_r15 |  |
| 8 | DC_2A_n77A | DC_2A_n77A maxNumberSRS-Ports-PerResource on NR band | 38.306, 4.2.7.7 | Rel-15 | pc_Band2_nrBand77_maxNumberSRS-Ports-PerResource_NR_r15 |  |
| 9 | DC_5A_n77A | DC_5A_n77A maxNumberSRS-Ports-PerResource on NR band | 38.306, 4.2.7.7 | Rel-15 | pc_Band5_nrBand77_maxNumberSRS-Ports-PerResource_NR_r15 |  |
| 10 | DC_13A_n77A | DC_13A_n77 maxNumberSRS-Ports-PerResource on NR band | 38.306, 4.2.7.7 | Rel-15 | pc_Band13_nrBand77_maxNumberSRS-Ports-PerResource_NR_r15 |  |
| 11 | DC_66A_n77A | DC_66A_n77A maxNumberSRS-Ports-PerResource on NR band | 38.306, 4.2.7.7 | Rel-15 | pc_Band66_nrBand77_maxNumberSRS-Ports-PerResource_NR_r15 |  |

Table A.4.3.2B.2.3.1-3c: Inter-band EN-DC within FR1 (two bands) NR part power class UE RF Baseline Implementation Capabilities (Rel-15) (maxNumberSRS-Ports-PerResource=n2 in NR standalone operation mode, maxNumberSRS-Ports-PerResource=n1 for EN-DC on NR band)

| Item | EN-DC configuration | UE Physical Layer Baseline Implementation Capabilities | Ref. | Release | Mnemonic | Supported NR part power class |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | DC_39A_n41A | DC_39A_n41A NR part power class | 38.101-3, 6.1 | Rel-15 | pc_Band39_nrBand41_powerClassNRPart_r15 |  |
| 2 | DC_39A_n79A | DC_39A_n79A NR part power class | 38.101-3, 6.1 | Rel-15 | pc_Band39_nrBand79_powerClassNRPart_r15 |  |
| 3 | DC_41A_n79A | DC_41A_n79A NR part power class | 38.101-3, 6.1 | Rel-15 | pc_Band41_nrBand79_powerClassNRPart_r15 |  |
| 4 | DC_3A_n78A | DC_3A_n78A NR part power class | 38.101-3, 6.1 | Rel-15 | pc_Band3_nrBand78_powerClassNRPart_r15 |  |
| 5 | DC_3A_n41A | DC_3A_n41A NR part power class | 38.101-3, 6.1 | Rel-15 | pc_Band3_nrBand41_powerClassNRPart_r15 |  |
| 6 | DC_1A_n78A | DC_1A_n78A NR part power class | 38.101-3, 6.1 | Rel-15 | pc_Band1_nrBand78_powerClassNRPart_r15 |  |
| 7 | DC_8A_n78A | DC_8A_n78A NR part power class | 38.101-3, 6.1 | Rel-15 | pc_Band8_nrBand78_powerClassNRPart_r15 |  |
| 8 | DC_2A_n77A | DC_2A_n77A NR part power class | 38.101-3, 6.1 | Rel-15 | pc_Band2_nrBand77_powerClassNRPart_r15 |  |
| 9 | DC_5A_n77A | DC_5A_n77A NR part power class | 38.101-3, 6.1 | Rel-15 | pc_Band5_nrBand77_powerClassNRPart_r15 |  |
| 10 | DC_13A_n77A | DC_13A_n77A NR part power class | 38.101-3, 6.1 | Rel-15 | pc_Band13_nrBand77_powerClassNRPart_r15 |  |
| 11 | DC_66A_n77A | DC_66A_n77A NR part power class | 38.101-3, 6.1 | Rel-15 | pc_Band66_nrBand77_powerClassNRPart_r15 |  |

Table A.4.3.2B.2.3.1-4: UE Power Class implementation Capabilities for inter-band EN-DC within FR1 (two bands)

| Item | UE Power Class implementation Capabilities | Ref. | Comments |
| --- | --- | --- | --- |
| 1 | UE Power Class 2 for Inter-band EN-DC within FR1 (two bands) | 38.101-3, 6.2B.1.3 | Applicable to the bands in Table A.4.3.2B.2.3.1-3 |
| 2 | UE Power Class 3 for Inter-band EN-DC within FR1 (two bands) | 38.101-3, 6.2B.1.3 | Applicable to the bands in Table A.4.3.2B.2.3.1-2 |

<< End of changes >>