3GPP TSG-RAN5 Meeting #95-e	R5-222339
Electronic Meeting, 9th May – 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 1657 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Editorial corrections for FR1 in annex F.1.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | -Referred clauses 6.4A.2.1, 6.4A.2.2 and 6.4A.2.3 are not test cases but sections containing test cases. -Red font color used unnecessarily. -Position of row for 6.3A.3.1_1 is misaligned with the rest of the table. |  |  |  |  |  |  |  |  |
| Summary of change: |  | -Replaced references to 6.4A.2.1, 6.4A.2.2 and 6.4A.2.3 by 6.4A.2.1.1, 6.4A.2.2.1 and 6.4A.2.3.1 respectively. -Replaced red font color by black font color. -Row for test case 6.3A.3.1_1 realigned with the rest of the table. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test specification will remain inconsistent. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | F.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES >>>

## F.1.2	Measurement of transmitter

Table F.1.2-1: Maximum Test System Uncertainty for transmitter tests

| Subclause |  | Maximum Test System Uncertainty |  | Derivation of Test System Uncertainty |  |
| --- | --- | --- | --- | --- | --- |
| 6.2.1 UE maximum output power |  | f ≤ 3.0GHz ±0.7 dB, BW ≤ 40MHz ±1.4 dB, 40MHz < BW ≤ 100MHz 3.0GHz < f ≤ 4.2GHz ±1.0 dB, BW ≤ 40MHz ±1.6 dB, 40MHz < BW ≤ 100MHz 4.2GHz < f ≤ 6.0GHz ±1.3 dB, BW ≤ 20MHz ±1.5 dB, 20MHz < BW ≤ 40MHz ±1.6 dB, 40MHz < BW ≤ 100MHz |  |  |  |
| 6.2.2 Maximum Power Reduction (MPR) |  | f ≤ 3.0GHz ±0.7 dB, BW ≤ 40MHz ±1.4 dB, 40MHz < BW ≤ 100MHz 3.0GHz < f ≤ 4.2GHz ±1.0 dB, BW ≤ 40MHz ±1.6 dB, 40MHz < BW ≤ 100MHz 4.2GHz < f ≤ 6.0GHz ±1.3 dB, BW ≤ 20MHz ±1.5 dB, 20MHz < BW ≤ 40MHz ±1.6 dB, 40MHz < BW ≤ 100MHz |  |  |  |
| 6.2.3 UE additional maximum output power reduction |  | f ≤ 3.0GHz ±0.7 dB, BW ≤ 40MHz ±1.4 dB, 40MHz < BW ≤ 100MHz 3.0GHz < f ≤ 4.2GHz ±1.0 dB, BW ≤ 40MHz ±1.6 dB, 40MHz < BW ≤ 100MHz 4.2GHz < f ≤ 6.0GHz ±1.3 dB, BW ≤ 20MHz ±1.5 dB, 20MHz < BW ≤ 40MHz ±1.6 dB, 40MHz < BW ≤ 100MHz |  |  |  |
| 6.2.4 Configured transmitted power |  | f ≤ 3.0GHz ±0.7 dB, BW ≤ 40MHz ±1.4 dB, 40MHz < BW ≤ 100MHz 3.0GHz < f ≤ 4.2GHz ±1.0 dB, BW ≤ 40MHz ±1.6 dB, 40MHz < BW ≤ 100MHz 4.2GHz < f ≤ 6.0GHz ±1.3 dB, BW ≤ 20MHz ±1.5 dB, 20MHz < BW ≤ 40MHz ±1.6 dB, 40MHz < BW ≤ 100MHz |  |  |  |
| 6.2A.1.1 UE maximum output power for CA (2UL CA) |  | For Inter-band CA MAX (MUCC1, MUCC2) |  | MUCCX is MU of each UL CC specified in single UL case 6.2.1. |  |
| 6.2A.2.1 UE maximum output power reduction for CA (2UL CA) |  | For Inter-band CA MAX (MUCC1, MUCC2) For intra-band contiguous CA Aggregated BW ≤ 100M: same as 6.2.2 for sum of powers of all CCs Aggregated BW > 100M: TBD |  | MUCCX is MU of each UL CC specified in single UL case 6.2.2. |  |
| 6.2A.3.1 UE additional maximum output power reduction CA (2UL CA) |  | For Inter-band CA MAX (MUCC1, MUCC2) |  | MUCCX is MU of each UL CC specified in single UL case 6.2.3. |  |
| 6.2A.4.1 Configured transmitted power for CA (2UL CA) |  | For Inter-band CA MAX (MUCC1, MUCC2) For intra-band contiguous CA Aggregated BW ≤ 100M: same as 6.2.2 for sum of powers of all CCs Aggregated BW > 100M: TBD |  | MUCCX is MU of each UL CC specified in single UL case 6.2.4. |  |
| 6.2C.1 Configured transmitted power for SUL |  | Same as 6.2.4 |  |  |  |
| 6.2C.3 UE maximum output power for SUL |  | Same as 6.2.1 |  |  |  |
| 6.2C.4 UE maximum output power reduction for SUL |  | Same as 6.2.2 |  |  |  |
| 6.2C.5 UE additional maximum output power reduction for SUL |  | Same as 6.2.3 |  |  |  |
| 6.2D.1 UE maximum output power for UL MIMO |  | Same as 6.2.1 for the sum of power at each of UE antenna connector |  | MU is for the sum of power at each of UE antenna connector, and is the same as the MU of single antenna port in 6.2.1 with SNR assumption reduced by 3dB compared to the single antenna case. |  |
| 6.2D.2 UE maximum output power reduction for UL MIMO |  | Same as 6.2.2 for the sum of power at each of UE antenna connector |  | MU is for the sum of power at each of UE antenna connector, and is the same as the MU of single antenna port in 6.2.2 with SNR assumption reduced by 3dB compared to the single antenna case. |  |
| 6.2D.3 UE additional maximum output power reduction for UL MIMO |  | Same as 6.2.3 for the sum of power at each of UE antenna connector |  | MU is for the sum of power at each of UE antenna connector, and is the same as the MU of single antenna port in 6.2.3 with SNR assumption reduced by 3dB compared to the single antenna case. |  |
| 6.2D.4 Configured transmitted power for UL MIMO |  | Same as 6.2.4 for the sum of power at each of UE antenna connector |  | MU is for the sum of power at each of UE antenna connector, and is the same as the MU of single antenna port in 6.2.4 with SNR assumption reduced by 3dB compared to the single antenna case. |  |
| 6.2F.1 UE maximum output power for shared spectrum channel access |  | 4.2GHz < f ≤ 5.925GHz ±1.3 dB, BW ≤ 20MHz ±1.5 dB, 20MHz < BW ≤ 40MHz ±1.6 dB, 40MHz < BW ≤ 100MHz 5.925GHz < f ≤ 7.125GHz TBD |  |  |  |
| 6.2F.3 UE additional maximum output power reduction for shared spectrum access |  | Same as 6.2F.1 |  |  |  |
| 6.3.1 Minimum output power |  | f ≤ 3.0GHz ±1.0 dB, BW ≤ 40MHz ±1.4 dB, 40MHz < BW ≤ 100MHz 3.0GHz < f ≤ 4.2GHz ±1.3 dB, BW ≤ 40MHz ±1.6 dB, 40MHz < BW ≤ 100MHz 4.2GHz < f ≤ 6.0GHz ±1.5 dB, BW ≤ 40MHz ±1.8 dB, 40MHz < BW ≤ 100MHz |  |  |  |
| 6.3.2 Transmit OFF power |  | f ≤ 3.0GHz ±1.5 dB, BW ≤ 40MHz ±1.7 dB, 40MHz < BW ≤ 100MHz 3.0GHz < f ≤ 4.2GHz ±1.8 dB, BW ≤ 40MHz ±1.9 dB, 40MHz < BW ≤ 80MHz ±2.2 dB, 80MHz < BW ≤ 100MHz 4.2GHz < f ≤ 6.0GHz ±2.0 dB, BW ≤ 20MHz ±2.1 dB, 20MHz < BW ≤ 80MHz ±2.2 dB, 80MHz < BW ≤ 100MHz |  |  |  |
| 6.3.3.2 General ON/OFF time mask |  | f ≤ 3.0GHz ±1.5 dB, BW ≤ 40MHz ±1.7 dB, 40MHz < BW ≤ 100MHz 3.0GHz < f ≤ 4.2GHz ±1.8 dB, BW ≤ 40MHz ±1.9 dB, 40MHz < BW ≤ 80MHz ±2.2 dB, 80MHz < BW ≤ 100MHz 4.2GHz < f ≤ 6.0GHz ±2.0 dB, BW ≤ 20MHz ±2.1 dB, 20MHz < BW ≤ 80MHz ±2.2 dB, 80MHz < BW ≤ 100MHz |  |  |  |
| 6.3.3.4 PRACH time mask |  | f ≤ 3.0GHz ±1.5 dB, BW ≤ 40MHz ±1.7 dB, 40MHz < BW ≤ 100MHz 3.0GHz < f ≤ 4.2GHz ±1.8 dB, BW ≤ 40MHz ±1.9 dB, 40MHz < BW ≤ 80MHz ±2.2 dB, 80MHz < BW ≤ 100MHz 4.2GHz < f ≤ 6.0GHz ±2.0 dB, BW ≤ 20MHz ±2.1 dB, 20MHz < BW ≤ 80MHz ±2.2 dB, 80MHz < BW ≤ 100MHz |  |  |  |
| 6.3.3.6 SRS time mask |  | f ≤ 3.0GHz ±1.5 dB, BW ≤ 40MHz ±1.7 dB, 40MHz < BW ≤ 100MHz 3.0GHz < f ≤ 4.2GHz ±1.8 dB, BW ≤ 40MHz ±1.9 dB, 40MHz < BW ≤ 80MHz ±2.2 dB, 80MHz < BW ≤ 100MHz 4.2GHz < f ≤ 6.0GHz ±2.0 dB, BW ≤ 20MHz ±2.1 dB, 20MHz < BW ≤ 80MHz ±2.2 dB, 80MHz < BW ≤ 100MHz |  |  |  |
| 6.3.4.2 Absolute power tolerance |  | f ≤ 3.0GHz ±1.0 dB, BW ≤ 40MHz ±1.6 dB, 40MHz < BW ≤ 100MHz 3.0GHz < f ≤ 4.2GHz ±1.4 dB, BW ≤ 40MHz ±1.9 dB, 40MHz < BW ≤ 100MHz 4.2GHz < f ≤ 6.0GHz ±2.0 dB, BW ≤ 20MHz ±2.1 dB, 20MHz < BW ≤ 40MHz ±2.2 dB, 80MHz < BW ≤ 100MHz |  | Test System uncertainty = SQRT (UL Meas Uncer2 + DL Meas Uncer2) |  |
| 6.3.4.3 Power Control Relative power tolerance |  | ±0.7 dB, BW ≤ 40MHz ±1.0 dB, 40MHz < BW ≤ 100MHz Absolute Uplink power measurement for step 2.1 same as 6.2.1. Absolute Uplink power measurement for step 1.1 same as 6.3.1. |  |  |  |
| 6.3.4.4 Aggregate power tolerance |  | ±0.7 dB, BW ≤ 40MHz ±1.0 dB, 40MHz < f ≤ 100MHz |  |  |  |
| 6.3A.1.1 Minimum output power for CA (2UL CA) |  | Same as 6.3.1 for each CC |  |  |  |
| 6.3A.3.1 Transmit ON/OFF time mask for CA (2UL CA) |  | Same as 6.3.3.2 for each CC |  |  |  |
| 6.3A.3.1_1 Time mask for switching between two uplink carriers |  | FFS |  |  |  |
| 6.3A.4.1.1 Absolute power tolerance for CA (2UL CA) |  | Same as 6.3.4.2 for each CC |  |  |  |
| 6.3A.4.2.1 Power Control Relative power tolerance for CA (2UL CA) |  | Same as 6.3.4.3 for each CC |  |  |  |
| 6.3A.4.3.1 Aggregate power tolerance for CA (2UL CA) |  | Same as 6.3.4.4 for each CC |  |  |  |
| 6.3D.1 Minimum output power for UL MIMO |  | Same as 6.3.1 for the sum of power at each of UE antenna connector |  | MU is for the sum of power at each of UE antenna connector, and is the same as the MU of single antenna port in 6.3.1 with SNR assumption reduced by 3dB compared to the single antenna case. |  |
| 6.3D.2 Transmit OFF power for UL MIMO |  | Same as 6.3.2 for each antenna |  |  |  |
| 6.3D.3 Transmit ON/OFF time mask for UL MIMO |  | ON power: Same as 6.2D.1 OFF power: Same as 6.3D.2 |  |  |  |
| 6.3D.4.1 Absolute Power tolerance |  | Same as 6.3.4.2 for the sum of power at each of UE antenna connector |  | MU is for the sum of power at each of UE antenna connector, and is the same as the MU of single antenna port in 6.3.4.2 with SNR assumption reduced by 3dB compared to the single antenna case. |  |
| 6.3D.4.2 Relative Power tolerance |  | ±0.9 dB, BW ≤ 40MHz ±1.4 dB, 40MHz < f ≤ 100MHz |  | MU is for the sum of power at each of UE antenna connector |  |
| 6.3D.4.3 Aggregate Power tolerance |  | Same as 6.3.4.4 for the sum of power at each of UE antenna connector |  | MU is for the sum of power at each of UE antenna connector, and is the same as the MU of single antenna port in 6.3.4.4 with SNR assumption reduced by 3dB compared to the single antenna case. |  |
| 6.3F.2 Transmit OFF power for shared spectrum channel access |  | 4.2GHz < f ≤ 5.925GHz ±2.0 dB, BW ≤ 20MHz ±2.1 dB, 20MHz < BW ≤ 80MHz ±2.2 dB, 80MHz < BW ≤ 100MHz 5.925GHz < f ≤ 7.125GHz TBD |  |  |  |
| 6.3F.3.2 General ON/OFF time mask for shared spectrum channel access |  | 4.2GHz < f ≤ 5.925GHz ±2.0 dB, BW ≤ 20MHz ±2.1 dB, 20MHz < BW ≤ 80MHz ±2.2 dB, 80MHz < BW ≤ 100MHz 5.925GHz < f ≤ 7.125GHz TBD |  |  |  |
| 6.4.1 Frequency Error |  | ±15 Hz, f ≤ 3.0GHz ±36 Hz, f > 3.0GHz DL Signal level: ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz ±1.5 dB, 4.2GHz < f ≤ 6.0GHz |  |  |  |
| 6.4.2.1 Error Vector Magnitude |  | For up to 256QAM: f ≤ 6.0GHz, BW ≤ 100MHz 15 dBm < PUL PUSCH, PUCCH, PRACH: ±1.5 % -25 dBm < PUL ≤ 15 dBm PUSCH, PUCCH, PRACH: ±2.5 % -40dBm ≤ PUL ≤ -25dBm PUSCH, PUCCH, PRACH: ±3.0 % Absolute Uplink power measurement same as 6.3.1. Relative Uplink power measurement same as 6.3.4.3. |  |  |  |
| 6.4.2.1a Error Vector Magnitude including symbols with transient period |  | Same as 6.4.2.1 |  |  |  |
| 6.4.2.2 Carrier Leakage |  | f ≤ 3.0GHz ±0.8 dB, BW ≤ 40MHz ±1.5 dB, 40MHz < BW ≤ 100MHz 3.0GHz < f ≤ 4.2GHz ±0.8 dB, BW ≤ 40MHz ±1.6 dB, 40MHz < BW ≤ 100MHz 4.2GHz < f ≤ 6.0GHz ±1.0 dB, BW ≤ 40MHz ±1.6 dB, 40MHz < BW ≤ 100MHz Absolute Uplink power measurement for step 2 and step 4 same as 6.2.1. Absolute Uplink power measurement for step 6 and step 8 same as 6.3.1. Relative Uplink power measurement same as 6.3.4.3. |  |  |  |
| 6.4.2.3 In-band emissions |  | f ≤ 3.0GHz ±0.8 dB, BW ≤ 40MHz ±1.5 dB, 40MHz < BW ≤ 100MHz 3.0GHz < f ≤ 4.2GHz ±0.8 dB, BW ≤ 40MHz ±1.6 dB, 40MHz < BW ≤ 100MHz 4.2GHz < f ≤ 6.0GHz ±1.0 dB, BW ≤ 40MHz ±1.6 dB, 40MHz < BW ≤ 100MHz Absolute Uplink power measurement for steps 1.2, 1.4, 2.2, and 2.4 same as 6.2.1. Absolute Uplink power measurement for steps 1.6, 1.8, 2.6, and 2.8 same as 6.3.1. Relative Uplink power measurement same as 6.3.4.3. |  |  |  |
| 6.4.2.4 EVM equalizer spectrum flatness |  | ±1.4 dB, BW ≤ 40MHz ±1.6 dB, 40MHz < BW ≤ 100MHz |  |  |  |
| 6.4.2.5 EVM equalizer spectrum flatness for Pi/2 BPSK |  | Same as 6.4.2.4 |  |  |  |
| 6.4A.1 Frequency error for CA |  | For inter-band CA: same as 6.4.1 for each CC |  |  |  |
| 6.4A.2.1.1 Error Vector Magnitude for CA |  | For inter-band CA: same as 6.4.2.1 for each CC Absolute Uplink power measurement same as 6.3A.1.1. Relative Uplink power measurement same as 6.3.4.3. |  |  |  |
| 6.4A.2.2.1 Carrier leakage for CA |  | For inter-band CA: same as 6.4.2.2 for each CC Uplink power measurement for step 5 and step 7 same as 6.2A.1.1. Absolute Uplink power measurement for step 9 and step 11 same as 6.3A.1.1] Relative Uplink power measurement same as 6.3.4.3. |  |  |  |
| 6.4A.2.3.1 In-band emission for CA |  | For inter-band CA: same as 6.4.2.3 for each CC Absolute Uplink power measurement for step 5 and step 7 same as 6.2A.1.1. Absolute Uplink power measurement for step 9 and step 11 same as 6.3A.1.1. Relative Uplink power measurement same as 6.3.4.3. |  |  |  |
| 6.4C.1 Frequency error for SUL |  | Same as 6.4.1 |  |  |  |
| 6.4C.2.1 Error Vector Magnitude for SUL |  | Same as 6.4.2.1 |  |  |  |
| 6.4C.2.2 Carrier leakage for SUL |  | Same as 6.4.2.2 |  |  |  |
| 6.4C.2.3 In-band emissions for SUL |  | Same as 6.4.2.3 |  |  |  |
| 6.4C.2.4 EVM equalizer spectrum flatness for SUL |  | Same as 6.4.2.4 |  |  |  |
| 6.4D.1 Frequency error for UL MIMO |  | Same as 6.4.1 for each antenna |  |  |  |
| 6.4D.2.1 Error Vector Magnitude for UL MIMO |  | Same as 6.4.2.1 for each antenna Absolute Uplink power measurement same as 6.3D.1. Relative Uplink power measurement same as 6.3.4.3. |  |  |  |
| 6.4D.2.2 Carrier leakage for UL MIMO |  | Same as 6.4.2.2 for each antenna Absolute Uplink power measurement for step 2 and step 4 same as 6.2D.1. Absolute Uplink power measurement for step 6 and step 8 same as 6.3D.1. Relative Uplink power measurement same as 6.3.4.3. |  |  |  |
| 6.4D.2.3 In-band emissions for UL MIMO |  | Same as 6.4.2.3 for each antenna Absolute Uplink power measurement for steps 1.2 and 1.4 same as 6.2D.1. Absolute Uplink power measurement for steps 1.6 and 1.8 same as 6.3D.1. Relative Uplink power measurement same as 6.3.4.3. |  |  |  |
| 6.4D.2.4 EVM equalizer spectrum flatness for UL MIMO |  | Same as 6.4.2.4 for each antenna |  |  |  |
| 6.4D.3 Time alignment error for UL MIMO |  | ±25ns |  |  |  |
| 6.4D.4 Requirements for Coherent UL MIMO |  | FFS |  |  |  |
| 6.5.1 Occupied bandwidth |  | 1.5% of channel bandwidth |  |  |  |
| 6.5.2.2 Spectrum Emission Mask |  | ±1.5 dB, f ≤ 3.0GHz ±1.8 dB, 3.0GHz < f ≤ 4.2GHz ±2.0 dB, 4.2GHz < f ≤ 6.0GHz |  |  |  |
| 6.5.2.3 Additional spectrum emission mask |  | ±1.5 dB, f ≤ 3.0GHz ±1.8 dB, 3.0GHz < f ≤ 4.2GHz ±2.0 dB, 4.2GHz < f ≤ 6.0GHz |  |  |  |
| 6.5.2.4.1 NR ACLR |  | ±0.8 dB, f ≤ 4.0GHz ±1.0 dB, 4.0GHz < f ≤ 6.0GHz |  |  |  |
| 6.5.2.4.2 UTRA ACLR |  | ±0.8 dB, f ≤ 4.0GHz ±1.0 dB, 4.0GHz < f ≤ 6.0GHz |  |  |  |
| 6.5.3.1 General spurious emissions |  | for results > -60 dBm: ±2.0 dB, 9kHz < f ≤ 3GHz ±2.5 dB, 3GHz < f ≤ 4GHz ±4.0 dB, 4GHz < f ≤ 19GHz ±6.0 dB, 19GHz < f ≤ 26GHz |  |  |  |
| 6.5.3.2 Spurious emission for UE co-existence |  | for results > -60 dBm: ±2.0 dB, 9kHz < f ≤ 3GHz ±2.5 dB, 3GHz < f ≤ 4GHz ±4.0 dB, 4GHz < f ≤ 19GHz ±6.0 dB, 19GHz < f ≤ 26GHz |  |  |  |
| 6.5.3.3 Additional spurious emissions |  | for results > -60 dBm: ±2.0 dB, 9kHz < f ≤ 3GHz ±2.5 dB, 3GHz < f ≤ 4GHz ±4.0 dB, 4GHz < f ≤ 19GHz ±6.0 dB, 19GHz < f ≤ 26GHz |  |  |  |
| 6.5.4 Transmit intermodulation |  | f ≤ 3.0GHz ±2.7 dB, BW ≤ 40MHz ±3.1 dB, 40MHz < BW ≤ 100MHz 3.0GHz < f ≤ 4.2GHz ±3.7 dB, BW ≤ 40MHz ±4.0 dB, 40MHz < BW ≤ 100MHz 4.2GHz < f ≤ 6.0GHz ±5.1 dB, BW ≤ 40MHz ±5.3 dB, 40MHz < BW ≤ 100MHz |  | Overall system uncertainty comprises four quantities: 1. Wanted signal setting error 2. CW Interferer level error 3. Wanted signal meas. error 4. Intermodulation product measurement error The relative level of the wanted signal and the CW interferer has 2 x effect on the intermodulation product. Items 1, 2, 3 and 4 are assumed to be uncorrelated so can be root sum squared to provide the combined effect. Test System uncertainty = SQRT [(2 x SQRT (Wanted setting_error2 + CW_level_error2)) 2 + Wanted_level_meas error2 + Intermodulation product measurement error2] |  |
| 6.5A.1 Occupied bandwidth for CA |  | For inter-band CA: same as 6.5.1 for each CC |  |  |  |
| 6.5A.2.2 Spectrum emission mask |  | For inter-band CA: same as 6.5.2.2 for each CC |  |  |  |
| 6.5A.2.4.1 NR ACLR |  | For inter-band CA: same as 6.5.2.4.1 for each CC |  |  |  |
| 6.5A.2.4.2 URTA ACLR |  | For inter-band CA: same as 6.5.2.4.2 for each CC |  |  |  |
| 6.5A.3.1 General spurious emissions for CA |  | For inter-band CA: same as 6.5.3.1 for each CC |  |  |  |
| 6.5A.3.2 Spurious emission for UE co-existence |  | For inter-band CA: same as 6.5.3.2 for each CC |  |  |  |
| 6.5A.4 Transmit intermodulation for CA |  | For inter-band CA: same as 6.5.2.4.2, at each antenna used for transmission |  |  |  |
| 6.5C.1 Occupied bandwidth for SUL |  | Same as 6.5.1 |  |  |  |
| 6.5C.2.2 Spectrum Emission Mask for SUL |  | Same as 6.5.2.2 |  |  |  |
| 6.5C.2.3 Additional spectrum emission mask for SUL |  | Same as 6.5.2.3 |  |  |  |
| 6.5C.2.4.1 NR ACLR for SUL |  | Same 6.5.2.4.1 |  |  |  |
| 6.5C.2.4.2 UTRA ACLR for SUL |  | Same as 6.5.2.4.2 |  |  |  |
| 6.5C.3.1 General spurious emissions for SUL |  | Same as 6.5.3.1 |  |  |  |
| 6.5C.3.2 Spurious emission for UE co-existence for SUL |  | Same as 6.5.3.2 |  |  |  |
| 6.5C.3.3 Additional spurious emissions for SUL |  | Same as 6.5.3.3 |  |  |  |
| 6.5C.4 Transmit intermodulation for SUL |  | Same as 6.5.4 |  |  |  |
| 6.5D.1 Occupied bandwidth for UL MIMO |  | Same as 6.5.1 for each antenna |  |  |  |
| 6.5D.2.2 Spectrum emission mask for UL MIMO |  | Same as 6.5.2.2 for each antenna |  |  |  |
| 6.5D.2.3 Additional spectrum emission mask for UL MIMO |  | Same as 6.5.2.3 for each antenna |  |  |  |
| 6.5D.2.4.1 NR ACLR for UL MIMO |  | Same as 6.5.2.4.1 for each antenna |  |  |  |
| 6.5D.2.4.2 UTRA ACLR for UL MIMO |  | Same as 6.5.2.4.2 for each antenna |  |  |  |
| 6.5D.3.1 General spurious emissions for UL MIMO |  | Same as 6.5.3.1 for each antenna |  |  |  |
| 6.5D.3.2 Spurious emissions for UE co-existence for UL MIMO |  | Same as 6.5.3.2 for each antenna |  |  |  |
| 6.5D.3.3 Additional spurious emissions for UL MIMO |  | Same as 6.5.3.3 for each antenna |  |  |  |
| 6.5D.4 Transmit intermodulation for UL MIMO |  | Same as 6.5.4 for each antenna |  |  |  |

## <<< END OF CHANGES >>>