3GPP TSG-RAN5 Meeting #95-e	R5-223085

# Online, , 9th May 2022 - 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 2981 | rev | - | Current version: | 16.11.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR RLC test case 7.1.2.3.7 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to 38.523-1 Table 7.1.2.3.7.3.2-1: Step 12: After 200 ms the SS transmits 2 AMD PDUs with SN=3, SN=7. The SS sets the P field of all the AMD PDUs to 0 except for that of the AMD PDU with SN=7. A time spacing of 20 ms is applied. Considering the test purpose of this case, the 200ms here should be after step 10 instead of the current system time, as otherwise there is risk that the t-StatusProhibit = 300ms gets expired before receiving the missing PDUs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modified step 12 to clarify that SS transmits AMD PDUs 200 ms after step 10 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant device may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.2.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 7.1.2.3.7	AM RLC / Receiver status triggers

7.1.2.3.7.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state and using AM RLC }

ensure that {

when { Reception failure of an RLC data PDU is detected and t-Reassembly expires }

then { UE initiates Status Reporting }

}

(2)

with { UE in RRC_CONNECTED state and using AM RLC }

ensure that {

when { Status Reporting is triggered and t-StatusProhibit is running }

then { UE wait until t-StatusProhibit has expired to send Status Report}

}

(3)

with { UE in RRC_CONNECTED state and using AM RLC }

ensure that {

when { Polling from peer AM RLC entity is detected and the sequence number ‘x’ of the PDU that carries the Poll satisfies x < RX_Highest_Status or x >= RX_Next + AM_Window_Size }

then { UE initiates Status Reporting }

}

(4)

with { UE in RRC_CONNECTED state and using AM RLC }

ensure that {

when { Polling from peer AM RLC entity is detected and the sequence number ‘x’ of the PDU that carries the Poll does not satisfies x < RX_Highest_Status or x >= RX_Next + AM_Window_Size }

then { UE waits until ‘x < RX_Highest_Status or x >= RX_Next + AM_Window_Size’ before initiating Status Reporting}

}

(5)

with { UE in RRC_CONNECTED state and using AM RLC }

ensure that {

when { the UE needs to send a Status Report and the UL grant is not large enough to accommodate the whole report }

then { UE includes as many NACK_SNs in the Status Report as allowed by the UL grant }

}

(6)

with { UE in RRC_CONNECTED state and using AM RLC }

ensure that {

when { the UE needs to send a Status Report and continuous sequence of RLC SDUs that have not been received yet }

then { UE includes NACK_SN with NACK range }

}

7.1.2.3.7.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.322, clause 5.3.4. Unless otherwise stated these are Rel-15 requirements.

[TS 38.322, clause 5.3.4]

An AM RLC entity sends STATUS PDUs to its peer AM RLC entity in order to provide positive and/or negative acknowledgements of RLC SDUs (or portions of them).

Triggers to initiate STATUS reporting include:

-	Polling from its peer AM RLC entity:

-	When an AMD PDU with SN = x and the P field set to "1" is received from lower layer, the receiving side of an AM RLC entity shall:

-	if the AMD PDU is to be discarded as specified in subclause 5.2.3.2.2; or

-	if x < RX_Highest_Status or x >= RX_Next + AM_Window_Size:

-	trigger a STATUS report.

-	else:

-	delay triggering the STATUS report until x < RX_Highest_Status or x >= RX_Next + AM_Window_Size.

NOTE 1:	This ensures that the RLC Status report is transmitted after HARQ reordering.

-	Detection of reception failure of an AMD PDU

-	The receiving side of an AM RLC entity shall trigger a STATUS report when t-Reassembly expires.

NOTE 2:	The expiry of t-Reassembly triggers both RX_Highest_Status to be updated and a STATUS report to be triggered, but the STATUS report shall be triggered after RX_Highest_Status is updated.

When STATUS reporting has been triggered, the receiving side of an AM RLC entity shall:

-	if t-StatusProhibit is not running:

-	at the first transmission opportunity indicated by lower layer, construct a STATUS PDU and submit it to lower layer.

-	else:

-	at the first transmission opportunity indicated by lower layer after t-StatusProhibit expires, construct a single STATUS PDU even if status reporting was triggered several times while t-StatusProhibit was running and submit it to lower layer.

When a STATUS PDU has been submitted to lower layer, the receiving side of an AM RLC entity shall:

-	start t-StatusProhibit.

When constructing a STATUS PDU, the AM RLC entity shall:

-	for the RLC SDUs with SN such that RX_Next <= SN < RX_Highest_Status that has not been completely received yet, in increasing SN order of RLC SDUs and increasing byte segment order within RLC SDUs, starting with SN = RX_Next up to the point where the resulting STATUS PDU still fits to the total size of RLC PDU(s) indicated by lower layer:

-	for an RLC SDU for which no byte segments have been received yet:

-	include in the STATUS PDU a NACK_SN which is set to the SN of the RLC SDU.

-	for a continuous sequence of byte segments of a partly received RLC SDU that have not been received yet:

-	include in the STATUS PDU a set of NACK_SN, SOstart and SOend.

-	for a continuous sequence of RLC SDUs that have not been received yet:

-	include in the STATUS PDU a set of NACK_SN and NACK range;

-	include in the STATUS PDU, if required, a pair of SOstart and SOend.

-	set the ACK_SN to the SN of the next not received RLC SDU which is not indicated as missing in the resulting STATUS PDU.

7.1.2.3.7.3	Test description

7.1.2.3.7.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.2.1.1 with the exception that the AM DRB is configured according to Table 7.1.2.3.7.3.1-1.

Table 7.1.2.3.7.3.1-1: RLC parameters

| t-Reassembly | ms150 |
| --- | --- |
| t-StatusProhibit | ms300 |
| t-PollRetransmit | ms500 |

7.1.2.3.7.3.2	Test procedure sequence

Table 7.1.2.3.7.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | The SS ignores scheduling requests and does not allocate any uplink grant. | - | - | - | - |
| 1 | The SS transmits 4 AMD PDUs with SN=0, 1, 2, and 4. The SS sets the P field of all the AMD PDUs to 0. A time spacing of 20 ms is applied. Record time TA when the AMD PDU with SN=4 is sent. | <-- | AMD PDU (SN=0, P=0) AMD PDU (SN=1, P=0) AMD PDU (SN=2, P=0) AMD PDU (SN=4, P=0) | - | - |
| 2 | The SS waits for 70 ms after the transmission of the first AMD PDU to ensure UE RLC has all the required SDUs available and then assigns 3 UL grants (UL grant allocation type 2) with a time spacing of 20 ms of size 848 bits (UL Grant Allocation type 2). (Note 1) | <-- | (UL grants, 848 bits) | - | - |
| 3 | The UE transmits RLC SDU#1. | --> | (RLC SDU#1) | - | - |
| 4 | The UE transmits RLC SDU#2. | --> | (RLC SDU#2) | - | - |
| 5 | The UE transmits RLC SDU#3. | --> | (RLC SDU#3) | - | - |
| 6 | 60 ms after step 5, the SS transmits a STATUS PDU | <-- | STATUS PDU | - | - |
| 7 | 80 ms after step 5, the SS starts the UL default grant transmission. | - | - | - | - |
| 8 | Check 1: Does the UE transmit a Status Report with NACK_SN=3 and ACK_SN=5? Record time TB (Note 5) Check 2: (TB – TA) = t-Reassembly? | --> | STATUS PDU | 1 | P |
| 9 | 100 ms after the Status Report is received at Step 8, the SS transmits 4 AMD PDUs with SN=5, 6, 8 and 9. The SS sets the P field of all the AMD PDUs to 0. A time spacing of 20 ms is applied. | <-- | AMD PDU (SN=5, P=0) AMD PDU (SN=6, P=0) AMD PDU (SN=8, P=0) AMD PDU (SN=9, P=0) | - | - |
| 10 | Check 1: Does the UE transmit a Status Report with NACK_SN=3 and ACK_SN=7? Record time TC Check 2: (TC – TB) = t-StatusProhibit? | --> | STATUS PDU | 2 | P |
| 11 | The SS ignores scheduling requests unless otherwise specified and does not allocate any uplink grant. | - | - | - | - |
| 12 | 200 ms after step 10, the SS transmits 2 AMD PDUs with SN=3, SN=7. The SS sets the P field of all the AMD PDUs to 0 except for that of the AMD PDU with SN=7. A time spacing of 20 ms is applied. | <-- | AMD PDU (SN=3, P=0) AMD PDU (SN=7, P=1) | - | - |
| 13 | The SS waits for 100 ms after the transmission of the last AMD PDU to ensure UE RLC has all the required SDUs available and then assigns 1 UL grant (UL grant allocation type 3) of size 88 bits. (Note 2) | <-- | (UL grant, 88 bits) | - | - |
| 14 | Check: Does the UE transmit a Status Report with no NACK_SN and ACK_SN = 10? | --> | STATUS PDU | 3 | P |
| 15 | In the second frame following the one scheduled in step 13 the SS assigns 7 UL grants (UL grant allocation type 2) with a time spacing of 20 ms of size 848 bits. (Note 1) | <-- | (UL grant, 848 bits) | - | - |
| 16 | The UE transmits RLC SDU#4. | --> | (RLC SDU#4) | - | - |
| 17 | The UE transmits RLC SDU#5. | --> | (RLC SDU#5) | - | - |
| 18 | The UE transmits RLC SDU#6. | --> | (RLC SDU#6) | - | - |
| 19 | The UE transmits RLC SDU#7. | --> | (RLC SDU#7) | - | - |
| 20 | The UE transmits RLC SDU#8. | --> | (RLC SDU#8) | - | - |
| 21 | The UE transmits RLC SDU#9. | --> | (RLC SDU#9) | - | - |
| 22 | The UE transmits RLC SDU#10. | --> | (RLC SDU#10) | - | - |
| 23 | The SS transmits a STATUS PDU | <-- | STATUS PDU | - | - |
| 24 | After 300 ms the SS transmits an AMD PDU with SN=11 and P=0, and an AMD PDU with SN=12 and P=1.A time spacing of 20 ms is applied. | <-- | AMD PDU (SN=11, P=0) AMD PDU (SN=12, P=1) | - | - |
| 25 | Check: Does the UE transmit a scheduling request within t-Reassembly/ 2 ms after the transmission of the first AMD PDU of Step 24? | --> | (SR) | 4 | F |
| 26 | At t-Reassembly/ 2 ms after the transmission of the second AMD PDU of Step 24, the SS transmits an AMD PDU with SN=10 and P=0. | <-- | AMD PDU (SN=10, P=0) | - | - |
| 27 | The SS waits for 60 ms to ensure UE RLC has all the required SDUs available and then assigns 1 UL grant (UL grant allocation type 3) of size 88 bits. (Note 2) | <-- | (UL grant, 88 bits) | - | - |
| 28 | Check: Does the UE transmit a Status Report with no NACK_SN and ACK_SN=13? | --> | STATUS PDU | 4 | P |
| 29 | The SS assigns 3 UL grants (UL grant allocation type 2) with a time spacing of 20 ms of size 848 bits. (Note 1) | <-- | (UL grant, 848 bits) | - | - |
| 30 | The UE transmits RLC SDU#11. | --> | (RLC SDU#11) | - | - |
| 31 | The UE transmits RLC SDU#12. | --> | (RLC SDU#12) | - | - |
| 32 | The UE transmits RLC SDU#13. | --> | (RLC SDU#13) | - | - |
| 33 | The SS transmits a STATUS PDU. | <-- | STATUS PDU | - | - |
| 34 | After 300 ms the SS transmits an AMD PDU with SN=17 and P=0, and an AMD PDU with SN=19 and P=1. A time spacing of 20 ms is applied. | <-- | AMD PDU (SN=17, P=0) AMD PDU (SN=19, P=1) | - | - |
| 35 | The SS waits for t-Reassembly ms to ensure expiry. | - | - | - | - |
| 36 | 60 ms after step 35 the SS assigns an UL grant (UL grant allocation type 3) of size 88 bits. (Note 3) | <-- | (UL Grant, 88 bits) | - | - |
| 37 | Check: Does the UE transmit a Status Report with ACK_SN=18 and NACK_SN: 13 including NACK Range 4 (SN 13, 14, 15, 16)? | --> | STATUS PDU | 5,6 | P |
| 38 | After 300 ms the SS transmits an AMD PDU with SN=16 and P=1. | <-- | AMD PDU (SN=16, P=1) | - | - |
| 39 | 30 ms after step 38 the SS assigns an UL grant (UL grant allocation type 3) of size 112 bits. (Note 4) | <-- | (UL Grant, size 112) | - | - |
| 40 | Check: Does the UE transmit a Status Report with ACK_SN=20 and NACK_SN: 13 including NACK Range 3 (SN 13, 14, 15) and NACK_SN=18 without NACK Range? | --> | STATUS PDU | 5,6 | P |
| 41 | 60 ms after step 38 the SS transmits 4 AMD PDUs with SN=13, 14, 15 and 18. A time spacing of 20 ms is applied. | <-- | AMD PDU (SN=13, P=0) AMD PDU (SN=14, P=0) AMD PDU (SN=15, P=0) AMD PDU (SN=18, P=0) | - | - |
| 42 | 130 ms after the transmission of the first AMD PDU the SS assigns 7 UL grant (UL grant allocation type 3 with a time spacing of 20 ms of size 848 bits. (Note 1) | <-- | (UL grant, 848 bits) | - | - |
| 43 | The UE loopbacks the complete RLC SDU. | --> | (RLC SDU#14) | - | - |
| 44 | The UE loopbacks the complete RLC SDU. | --> | (RLC SDU#15) | - | - |
| 45 | The UE loopbacks the complete RLC SDU. | --> | (RLC SDU#16) | - | - |
| 46 | The UE loopbacks the complete RLC SDU. | --> | (RLC SDU#17) | - | - |
| 47 | The UE loopbacks the complete RLC SDU. | --> | (RLC SDU#18) | - | - |
| 48 | The UE loopbacks the complete RLC SDU. | --> | (RLC SDU#19) | - | - |
| 49 | The UE loopbacks the complete RLC SDU. | --> | (RLC SDU#20) | - | - |
| 50 | The SS transmits a STATUS PDU. | <-- | STATUS PDU | - | - |
| Note 1: UL grant of 848 bits (LRBs & IMCS as per 38.523-3[3] annex B) is chosen to allow the UE to transmit one PDU at a time ( 99 bytes RLC SDU + 3 bytes RLC Header + 2 bytes MAC Sub PDU header + 2 bytes for short BSR or padding). Note 2: UL grant of 88 bits (LRBs & IMCS as per 38.523-3[3] annex B) is chosen to allow the UE to transmit a Status Report with ACK_SN(3 byte) + 2 byte MAC PDU subheader and (2 byte short BSR). 4 Bytes additional space provided to confirm UE does not include NACK_SN and conformant UE instead will include MAC Padding. Note 3: UL grant of 88 bits (LRBs & IMCS as per 38.523-3[3] annex B) is chosen to allow the UE to transmit (a Status Report with ACK_SN (3 Bytes)and 1 NACK_SNs with NACK Range(4 Bytes) + MAC PDU subheader (2 Bytes) + Short BSR (2 Byte). Note 4: UL grant of 112 bits (LRBs & IMCS as per 38.523-3[3] annex B) is chosen to allow the UE to transmit (a Status Report with ACK_SN (3 Bytes)and 1 NACK_SNs with NACK Range(4 Bytes) +NACK SN (3 Bytes) + MAC PDU subheader (2 Bytes) + Short BSR (2 Byte). Note 5: Time TB should be recorded by factoring in the SR-Periodicity as configured in the SchedulingRequestResourceConfig->periodicityAndOffset |  |  |  |  |  |

7.1.2.3.7.3.3	Specific message contents

None