3GPP TSG RAN meeting #96e									        RP-221205

Electronic Meeting, Jun 6-9, 2022

## Status Report to TSG

Agenda item:			9.4.4.7

Source:				Nokia

For:					Approval

| WI / SI Name | Further enhancements of NR RF requirements for frequency range 2 (FR2) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_RF_FR2_req_enh2 |  |  |  |  |
| Unique ID | 890059 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-220968 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: n/a | Core part: 06/2022 | Performance part: 09/2022 | Testing part: n/a |  |
| Overall Completion level | Study Item: n/a | Core part: 100% | Performance Part: 80% | Testing part: n/a |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN4 |
| --- | --- | --- |
| Rapporteur | Name | Vasenkari, Petri |
|  | Company | Nokia |
|  | Email | Petri.j.vasenkari@nokia.com |

# 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | Yes |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

# 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.4	RAN4

#### 2.4.1	Agreements

RAN4 #103-e Agreements

DL and UL CA:

| Tdoc number | Revised to | Title | Source | Status | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-2210573 |  | WF on FR2 CA relaxations | Nokia | Approved |  |
| R4-2207638 | R4-2210777 | CR to 38.101-2: FR2+FR2 IBM DLCA for PC1/2/5 | Qualcomm, Nokia, Verizon, LGE | Agreed | Capture agreed values in WF |
| R4-2209106 | R4-2210779 | Addition of downlink CA_n258-n261 configuration | Nokia, Qualcomm Inc | Agreed |  |
| R4-2207636 | R4-2210780 | CR to 38.101-2 FR2+FR2 ULCA Feature | Qualcomm, Nokia, Verizon, LGE | Agreed | Capture agreed values in WF |

UL GAPS:

| Tdoc number | Revised to | Title | Source | Status | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-2210574 | R4-2211221 | WF on UL gap for FR2 | Apple | Approved | Remove Section 3 UL coherent MIMO |
| R4-2210575 | R4-2211222 | LS on UL gap for FR2 | Apple | Approved |  |
| R4-2210576 |  | Draft CR on RF related UL gap for FR2 (38.101-2) | Apple | Endrosed |  |
| R4-2210781 |  | Draft CR on UL gaps for BPS | Apple, Ericsson, Nokia | Endorsed | Capture RRM related agreements on top of the endorsed CR R4-2206511 |
| R4-2211140 R4-2209155 (Cat-A) R4-2209156 (Cat-A) |  | Draft CR to add ‘Annex G Difference of relative phase and power errors’ for FR2 UL coherent MIMO | Anritsu Limited | Endorsed |  |

DC-Location and new CA BW Classes

| Tdoc number | Revised to | Title | Source | Status | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-2210782 |  | LS on DC location for intra-band CA | vivo | Approval |  |
| R4-2210783 |  | FR2 CA BW classes up to 2400 MHz aggregated BW with mixed channel bandwidths | Ericsson, Verizon | Endorsed |  |
| R4-2210784 |  | LS to introduce new FR2 CA BW Classes | Xiaomi | Noted |  |

RRM:

| Tdoc number | Revised to | Title | Source | Status | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-2210606 |  | WF on RRM requirements for FR2 Inter-band DL CA and UL CA | Nokia | Approved | Capture agreed values in WF |
| R4-2209788 | R4-2211079 | Correction for Big CR on RRM requirements for FR2 Inter-band CA | Nokia, Nokia Shanghai Bell | Endorsed |  |
| R4-2208499 |  | CR on RRM requirements for IBM inter-band FR2 UL CA | Nokia, Nokia Shanghai Bell | Agreed |  |
| R4-2210125 | R4-2211080 | Draft CR on RRM requirements for FR2 inter-band UL CA for IBM UE | Ericsson | Endorsed |  |
| R4-2209790 | R4-2211081 | dratCR on UE UL carrier RRC reconfiguration delay for FR2 | Nokia, Nokia Shanghai Bell | Endorsed |  |

GTW Agreements

DL and UL CA:

Agreement:

- Alt 1: For PC1/2, CA relaxations add MBR aspects into ΔRIB and ΔTIB. Capture the components of ΔRIB and ΔTIB either in the CR cover page or in CR as informative note.

Agreement:

- Relaxation values for n258-n261 PC3

| ΔRIB,P,n (dB) |  | ΔRIB,S,n (dB) |  |
| --- | --- | --- | --- |
| n258 | n261 | n258 | n261 |
| 3.5 | 3.5 | 3.5 | 3.5 |

Note: there is no simultaneous Rx/Tx operation.

UL GAPS:

Agreement: Step 3 discussion is decoupled with UL gap.

Agreement: UE will meet OFF power requirement defined in this clause for the band with activated UL Gap.

Agreement: Add the following clarification to TS38.101-2 6.2: The measured peak EIRP PUMAX,f,c_GAP_ON shall be measured outside of the UL gap symbol.

DC-Location

Agreement:

- Keep R~U and inform RAN2 that R~U is under consideration

  - Further discuss whether to remove R~U and if not how to specify the fallback rule

- FFS to introduce the new IE to limit the maximum aggregated bandwidth

- Put the 1600MHz limitation in the corresponding baseket WID

- Agreement: agree on Option 3.

  - Option 3: Applicable for both cases:

    - non-CA case in UL (i.e. UL SCCs not exist or removed by network, only DL CA is configured)

    - UL CA are configured, but only one CC is activated.

Agreement:

- Only Rel-17 reporting scheme can be used throughout different UL CC groups on the same band

Agreement: TX LO cannot locate in the DL only spectrum

- The DL only spectrum here means the spectrum described in TS 38.101-2 clause 5.3A.4 and defined by Fsd according to Table 5.3A.4-3.

Agreement:

- Clarify CC groups do not always consist of multiple CCs, but it may consist of single CC

- To help RAN2 design in Rel-17, RAN4 provides the following information

- Considering current spec restriction (1LO for FR2, maximum 2CCs UL for FR1), limit the CC groups so that each group contains only one UL CC

new CA BW Classes

Agreement:

- Take option 2c as baseline

  - For CA BW class R9, R10, R11, R12 of FBG5, the maximum aggregated BW is limited to 1600MHz in Rel-17.

    - FFS on whether and how to capture it in Rel-17 RAN4 specification, and whether to capture it in Rel-17 RAN2 specification

  - Option 2c:

  - Further refinement is needed:

    - Further discussion on proposal 4 in Issue 2-1-2.

    - Considering other issues

Agreement:

- Keep R~U and inform RAN2 that R~U is under consideration

  - Further discuss whether to remove R~U and if not how to specify the fallback rule

- FFS to introduce the new IE to limit the maximum aggregated bandwidth

- Put the 1600MHz limitation in the corresponding baseket WID

Email discussion summaries (For information)

RAN4 #103-e

| R4-2210263 | Email discussion summary for [103-e][128] NR_RF_FR2_enh2_Part_1 |
| --- | --- |
| R4-2210460 | Email discussion summary for [103-e][128] NR_RF_FR2_enh2_Part_1 |
| R4-2210264 | Email discussion summary for [103-e][129] NR_RF_FR2_enh2_Part_2 |
| R4-2210461 | Email discussion summary for [103-e][129] NR_RF_FR2_enh2_Part_2 |
| R4-2210265 | Email discussion summary for [103-e][130] NR_RF_FR2_enh2_Part_3 |
| R4-2210462 | Email discussion summary for [103-e][130] NR_RF_FR2_enh2_Part_3 |
| R4-2210291 | Email discussion summary for [103-e][219] NR_RF_FR2_enh2_RRM |
| R4-2210488 | Email discussion summary for [103-e][219] NR_RF_FR2_enh2_RRM |

#### 2.4.2	Remaining Open issues

Core part none.

# 4.	References

RAN4 #103-e contributions 108 submitted

| R4-2210263 | Email discussion summary for [103-e][128] NR_RF_FR2_enh2_Part_1 | Moderator (Nokia) | Carolyn Taylor |
| --- | --- | --- | --- |
| R4-2210460 | Email discussion summary for [103-e][128] NR_RF_FR2_enh2_Part_1 | Moderator (Nokia) | Carolyn Taylor |
| R4-2210573 | WF on FR2 CA relaxations | Nokia | Carolyn Taylor |
| R4-2208545 | PC2 RF requirements for inter-band CA_n257A_n259A based on IBM | LG Electronics, KT, LG Uplus | YOONOH YANG |
| R4-2208546 | CR on FR2-1 PC2 in n259 for supporting PC2 CA_n257-n259 | LG Electronics, KT, LG Uplus | YOONOH YANG |
| R4-2208644 | CR on FR2-1 PC2 in n259 for supporting PC2 CA_n257-n259 | LG Electronics, KT, LG Uplus | YOONOH YANG |
| R4-2209107 | 38.101-1: Introduction of 900 MHz to 5G NR for RMR | Nokia, UIC | Petri Vasenkari |
| R4-2207637 | On delta(RIB) for n258+n261 DL inter-CA | Qualcomm Incorporated | Sumant Iyer |
| R4-2207638 | CR to 38.101-2: FR2+FR2 IBM DLCA for PC1/2/5 | Qualcomm, Nokia, Verizon, LGE | Sumant Iyer |
| R4-2207710 | UE DL requirements for IBM | Sony, Ericsson | Olof Zander |
| R4-2208309 | Discussion on UE relaxation values for inter-band DL CA for CA_n258-n261 | MediaTek Beijing Inc. | Ting-Wei Kang |
| R4-2208484 | EIS requirements for IBM based DL CA_n258-n261 | Samsung | Bozhi Li |
| R4-2208607 | Discussion on requirements of n258-n261 | vivo | Ruixin Wang |
| R4-2208682 | Discussion on relaxation value FR2 inter-band UL CA | ZTE Corporation | Wubin Zhou |
| R4-2208683 | Discussion on IBM inter-band DL CA_n258-n261 | ZTE Corporation | Wubin Zhou |
| R4-2208750 | Introduction of requirements for DL inter-band CA for CA_n258-n261 | Ericsson, Sony | Christian Bergljung |
| R4-2208862 | Discussion on delta RIB for CA_n258-n261 based on IBM | Xiaomi | juan zhang |
| R4-2209106 | Addition of downlink CA_n258-n261 configuration | Nokia, Qualcomm Inc | Petri Vasenkari |
| R4-2209321 | UE relaxation for CA_n258-n261 with IBM | Apple | Anatoliy Ioffe |
| R4-2209427 | R17 FR2 Inter-band DL CA requirements for n258+n261 | OPPO | Jinqiang Xing |
| R4-2210777 | CR to 38.101-2: FR2+FR2 IBM DLCA for PC1/2/5 | Qualcomm, Nokia, Verizon, LGE | Carolyn Taylor |
| R4-2210778 | Discussion on delta RIB for CA_n258-n261 based on IBM | Xiaomi | Carolyn Taylor |
| R4-2210779 | Addition of downlink CA_n258-n261 configuration | Nokia, Qualcomm Inc | Carolyn Taylor |
| R4-2207635 | On MPR and delta(TIB) for FR2+FR2 ULCA | Qualcomm Incorporated | Sumant Iyer |
| R4-2207636 | CR to 38.101-2 FR2+FR2 ULCA Feature | Qualcomm, Nokia, Verizon, LGE | Sumant Iyer |
| R4-2207711 | UE UL CA requirements based on IBM | Sony, Ericsson | Olof Zander |
| R4-2208311 | Discussion on inter-band UL CA for PC1 CA_n260-n261 and PC5 CA_n257A-n259A | MediaTek Beijing Inc. | Ting-Wei Kang |
| R4-2208485 | EIRP requirements for IBM based UL CA for FWA | Samsung | Bozhi Li |
| R4-2208606 | Discussion on FR2 inter-band UL CA | vivo | Ruixin Wang |
| R4-2208863 | RF requirements for inter-band UL CA_n260-n261 and CA_n257-n259 based on IBM | Xiaomi | juan zhang |
| R4-2209429 | R17 FR2 Inter-band UL CA requirements | OPPO | Jinqiang Xing |
| R4-2210192 | TIB for inter-band UL CA with IBM | NTT DOCOMO, INC. | Ryu Kitagawa |
| R4-2210193 | CR to TS38.101-2 TIB for inter-band UL CA | NTT DOCOMO, INC. | Ryu Kitagawa |
| R4-2210780 | CR to 38.101-2 FR2+FR2 ULCA Feature | Qualcomm, Nokia, Verizon, LGE | Carolyn Taylor |
| R4-2210264 | Email discussion summary for [103-e][129] NR_RF_FR2_enh2_Part_2 | Moderator (Apple) | Carolyn Taylor |
| R4-2210461 | Email discussion summary for [103-e][129] NR_RF_FR2_enh2_Part_2 | Moderator (Apple) | Carolyn Taylor |
| R4-2210574 | WF on UL gap for FR2 | Apple | Carolyn Taylor |
| R4-2210575 | LS on UL gap for FR2 | Apple | Carolyn Taylor |
| R4-2210576 | Draft CR on RF related UL gap for FR2 (38.101-2) | Apple | Carolyn Taylor |
| R4-2211221 | WF on UL gap for FR2 | Apple | Carolyn Taylor |
| R4-2211222 | LS on UL gap for FR2 | Apple | Carolyn Taylor |
| R4-2207661 | On RF requirements for UL Gap | Qualcomm Incorporated | Ville Vintola |
| R4-2207706 | draft CR on UL Gap RF requirements | Qualcomm Incorporated | Ville Vintola |
| R4-2207814 | UL gaps for Tx power management RF aspect | Apple | Tang Yang |
| R4-2208608 | Discussion on UL gap for Tx power management | vivo | Ruixin Wang |
| R4-2208751 | UE Tx power management test case for MPE compliance | Ericsson, Sony | Christian Bergljung |
| R4-2208875 | Draft CR for UL gap for Tx power management RF aspect | Nokia Denmark | Samantha Caporal |
| R4-2208876 | Requirements and test cases of UE FR2 UL Gap for UE Tx power enhancement | Nokia Denmark | Samantha Caporal |
| R4-2209425 | R17 FR2 UL gap for power management | OPPO | Jinqiang Xing |
| R4-2207815 | UL gaps for coherent UL MIMO | Apple | Tang Yang |
| R4-2209145 | Draft CR to 38.101-2 on requirements for coherent UL MIMO | Huawei,HiSilicon | Lingyu Kong |
| R4-2209146 | Discussion on UL coherent MIMO | Huawei,HiSilicon | Lingyu Kong |
| R4-2209153 | FR2 UL coherent MIMO | Anritsu Limited | Hassen Chouli |
| R4-2209154 | Draft CR to add ‘Annex G Difference of relative phase and power errors’ for FR2 UL coherent MIMO | Anritsu Limited | Hassen Chouli |
| R4-2209155 | Draft CR to add ‘Annex G Difference of relative phase and power errors’ for FR2 UL coherent MIMO | Anritsu Limited | Hassen Chouli |
| R4-2209156 | Draft CR to add ‘Annex G Difference of relative phase and power errors’ for FR2 UL coherent MIMO | Anritsu Limited | Hassen Chouli |
| R4-2211140 | Draft CR to add ‘Annex G Difference of relative phase and power errors’ for FR2 UL coherent MIMO | Anritsu Limited | Carolyn Taylor |
| R4-2207660 | Offset and DL CA handling for DC location | Qualcomm Incorporated | Ville Vintola |
| R4-2207679 | Further views on DC location for intra-band UL CA | Apple | Anatoliy Ioffe |
| R4-2208385 | Handling of multiple DC locations for intra-band CA | Nokia, Nokia Shanghai Bell | Hiromasa Umeda |
| R4-2208387 | [Draft] Reply on LS Reply LS on DC location for >2CC | Nokia, Nokia Shanghai Bell | Hiromasa Umeda |
| R4-2208609 | Discussion on remaining issue on DC location | vivo | Ruixin Wang |
| R4-2208610 | LS on DC location for intra-band CA | vivo | Ruixin Wang |
| R4-2209166 | Further study on DC location reporting | Huawei, HiSilicon | Ye Liu |
| R4-2209424 | R17 FR2 DC reporting | OPPO | Jinqiang Xing |
| R4-2210265 | Email discussion summary for [103-e][130] NR_RF_FR2_enh2_Part_3 | Moderator (vivo) | Carolyn Taylor |
| R4-2210462 | Email discussion summary for [103-e][130] NR_RF_FR2_enh2_Part_3 | Moderator (vivo) | Carolyn Taylor |
| R4-2210782 | LS on DC location for intra-band CA | vivo | Carolyn Taylor |
| R4-2207680 | Further views on new FR2 CA BW classes | Apple | Anatoliy Ioffe |
| R4-2208314 | Discussion on FR2 mixed-CC CA BW class | MediaTek Beijing Inc. | Ting-Wei Kang |
| R4-2208486 | Discussion on CA BW class of mixed 100MHz and 200MHz CCs | Samsung | Bozhi Li |
| R4-2208752 | Draft LS to RAN2 on FR2 bandwidth classes covering up to 2400 MHz aggregated bandwidth with mixed carrier bandwidths | Ericsson, Verizon | Christian Bergljung |
| R4-2208753 | FR2 CA BW classes up to 2400 MHz aggregated BW with mixed channel bandwidths | Ericsson, Verizon | Christian Bergljung |
| R4-2208864 | Discussion on FR2 new CA BW classes | Xiaomi | juan zhang |
| R4-2208865 | LS to introduce new FR2 CA BW Classes | Xiaomi | juan zhang |
| R4-2208866 | Draft CR for TS 38.101-2 to introduction of FR2 new CA BW classes | Xiaomi | juan zhang |
| R4-2209617 | Discussion on FR2 CA BW class with aggregated BW up to 1600MHz | ZTE Corporation | Zhifeng Ma |
| R4-2210783 | FR2 CA BW classes up to 2400 MHz aggregated BW with mixed channel bandwidths | Ericsson, Verizon | Carolyn Taylor |
| R4-2210784 | LS to introduce new FR2 CA BW Classes | Xiaomi | Carolyn Taylor |
| R4-2208498 | Correction for Big CR on RRM requirements for FR2 Inter-band CA | Nokia, Nokia Shanghai Bell | Yueji Chen |
| R4-2208992 | DraftCR on correction to interruption requirements for IBM R17 | Huawei, Hisilicon | Jing Han |
| R4-2209788 | Correction for Big CR on RRM requirements for FR2 Inter-band CA | Nokia, Nokia Shanghai Bell | Yueji Chen |
| R4-2210124 | Update of Big CR for RRM requirements of FR2 Inter-band CA | Ericsson | VENKATARAO GONUGUNTLA |
| R4-2210291 | Email discussion summary for [103-e][219] NR_RF_FR2_enh2_RRM | Moderator (Nokia, Nokia Shanghai Bell) | Carolyn Taylor |
| R4-2210488 | Email discussion summary for [103-e][219] NR_RF_FR2_enh2_RRM | Moderator (Nokia, Nokia Shanghai Bell) | Carolyn Taylor |
| R4-2210606 | WF on RRM requirements for FR2 Inter-band DL CA and UL CA | Nokia | Carolyn Taylor |
| R4-2211078 | DraftCR on correction to interruption requirements for IBM R17 | Huawei, Hisilicon | Carolyn Taylor |
| R4-2211079 | Correction for Big CR on RRM requirements for FR2 Inter-band CA | Nokia, Nokia Shanghai Bell | Carolyn Taylor |
| R4-2208499 | CR on RRM requirements for IBM inter-band FR2 UL CA | Nokia, Nokia Shanghai Bell | Yueji Chen |
| R4-2208993 | Discussion on carrier numbers for FR2 inter-band UL CA | Huawei, Hisilicon | Jing Han |
| R4-2208994 | DraftCR on number of serving carriers for FR2 inter-band CA R17 | Huawei, Hisilicon | Jing Han |
| R4-2210125 | Draft CR on RRM requirements for FR2 inter-band UL CA for IBM UE | Ericsson | VENKATARAO GONUGUNTLA |
| R4-2211080 | Draft CR on RRM requirements for FR2 inter-band UL CA for IBM UE | Ericsson | Carolyn Taylor |
| R4-2207816 | UL gaps for Tx power management RRM aspect | Apple | Tang Yang |
| R4-2207817 | Draft CR for UL gap for Tx power management RRM aspect | Apple | Tang Yang |
| R4-2208590 | Discussion on remaining aspects for UL Gaps (RRM) | Nokia, Nokia Shanghai Bell | Lars Dalsgaard |
| R4-2208591 | CR for UL gaps for Tx power management | Nokia, Nokia Shanghai Bell | Lars Dalsgaard |
| R4-2208787 | LS on priority for PHR including MPE indication | Nokia Corporation | Lars Dalsgaard |
| R4-2210126 | Discussion on UL gaps prioritization over critical UL signals | Ericsson | VENKATARAO GONUGUNTLA |
| R4-2210127 | Draft CR on UL gaps for BPS | Ericsson | VENKATARAO GONUGUNTLA |
| R4-2210781 | Draft CR on UL gaps for BPS | Apple, Ericsson, Nokia | Carolyn Taylor |
| R4-2209789 | Discussion on RRM performance requirements for IBM inter-band FR2 UL CA | Nokia, Nokia Shanghai Bell | Yueji Chen |
| R4-2209790 | dratCR on UE UL carrier RRC reconfiguration delay for FR2 | Nokia, Nokia Shanghai Bell | Yueji Chen |
| R4-2210128 | Scope of RRM tests for UL inter-band CA for IBM | Ericsson | VENKATARAO GONUGUNTLA |
| R4-2211081 | dratCR on UE UL carrier RRC reconfiguration delay for FR2 | Nokia, Nokia Shanghai Bell | Carolyn Taylor |
| R4-2208595 | Discussion on test for UL gaps for Tx power management | Nokia, Nokia Shanghai Bell | Lars Dalsgaard |
| R4-2208778 | Discussion on performance requirements of UL gap for Tx power management | ZTE Corporation | Chenchen Zhang |
| R4-2210129 | Scope of the RRM test cases for UL gaps for TX power management | Ericsson | VENKATARAO GONUGUNTLA |