3GPP TSG RAN meeting #96										RP-221226

Budapest, Hungary, June 6-9, 2022

## Status Report to TSG

Agenda item:			9.4.4.3

| WI / SI Name |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: No | Performance part: Yes |  | Testing part: No |
| Acronym | NR_DL1024QAM_FR1 |  |  |  |  |
| Unique ID | 890156 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-213654 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: mm/yyyy | Core part: mm/yyyy | Performance part: 12/2022 | Testing part: mm/yyyy |  |
| Overall Completion level | Study Item: xx % | Core part: xx% | Performance Part: 100% | Testing part: xx% |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN4 |
| --- | --- | --- |
| Rapporteur | Name | Wanuga, Kevin (RAN1); Uesaka, Kazuyoshi (RAN4) |
|  | Company | Nokia, Ericsson |
|  | Email | kevin (dot) wanuga (at) nokia (dot) com; kazuyoshi (dot) uesaka (at) ericsson (dot) com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

## 2.1	RAN1

#### 2.1.1	Agreements

#### 2.1.2	Remaining Open issues

## 2.2	RAN2

#### 2.2.1	Agreements

#### 2.2.2	Remaining Open issues

## 2.3	RAN3

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

## 2.4	RAN4

#### 2.4.1	Agreements

The following agreements are made on RF performance part in RAN4#103-e (R4-2210504):

- Agreed with CR of the introduction of RMC for UE Rx 1024QAM maximum input level test

- Agreed with CRs of BS RF conformance requirements.

The following agreements are made on UE demodulation performance part in RAN4#103-e (R4-2210520).

- SDR tests

  - For Rank 2 with 2RX, reuse the existing SDR requirements for 2 MIMO layers with 256QAM for 1024QAM with 2RX.

    - Add a note for 1024QAM MCS table for 2RX UE as: For the band(s) on which UE supporting “Maximum modulation format” of 10, with 2 RX and 2 MIMO layers, the MCS index is derived from the rows with “Maximum modulation format” of 8 in Table 5.5A-5

- CQI reporting tests

  - Set the SNR test points as follows

    - 2Rx UE: SNR = [28/29] dB

    - 4Rx UE: SNR = [25/26] dB

- Agree with the draft CRs of UE demodulation and CSI reporting requirements.

#### 2.4.2	Remaining Open issues

None

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

## 4.	References

RAN4#103-e

- R4-2209063	"CR: Introduction of RMC for 1024QAM maximum input level", Ericsson.

- R4-2208790	"Discussion on BS RF conformance requirements for 1024QAM in FR1", NEC.

- R4-2208791	"CR to 38.141-1: BS RF conformance requirements for 1024QAM in FR1", NEC.

- R4-2208792	"CR to 38.141-2: BS RF conformance requirements for 1024QAM in FR1", NEC.

- R4-2208793	"CR to 37.141: BS RF conformance requirements for 1024QAM in FR1", NEC.

- R4-2208794	"CR to 37.145-1: BS RF conformance requirements for 1024QAM in FR1", NEC.

- R4-2208795	"CR to 37.145-2: BS RF conformance requirements for 1024QAM in FR1", NEC.

- R4-2209138	"CR to TS 38.141-2: Introduction of 1024 QAM in FR1", Ericsson.

- R4-2210700	"CR to TS 38.141-2: Introduction of 1024 QAM in FR1", Ericsson.

- R4-2207794	"Simulation results for PDSCH demod requirements with 1KQAM", Apple.

- R4-2207795	"Draft CR for PDSCH demodulation requirements for 1KQAM in TDD", Apple.

- R4-2209064	"Summary of PDSCH simulation results for DL 1024QAM in FR1", Ericsson.

- R4-2209065	"UE demodulation requirements for DL 1024QAM", Ericsson.

- R4-2209175	"PDSCH Simulation results for 1024QAM FR1 UE Demod Tests", Qualcomm Incorporated.

- R4-2209376	"Draft CR on Applicability Rules and TDLD30-5 delay profile for FR1 DL 1024QAM PDSCH Requirements", Qualcomm Incorporated.

- R4-2209801	"Simulation results and discussion on the PDSCH requirements for 1024QAM", MediaTek inc.

- R4-2209803	"Draft CR to TS38.101-4, PDSCH requirements for 1024QAM in FR1 FDD", MediaTek inc.

- R4-2209871	"Simulation results on 1024QAM PDSCH", Huawei, HiSilicon.

- R4-2210323	"Email discussion summary for [103-e][317] NR_DL1024QAM_Demod", Moderator (Ericsson).

- R4-2210520	"Email discussion summary for [103-e][317] NR_DL1024QAM_Demod", Moderator (Ericsson).

- R4-2210651	"WF on DL1024QAM UE demodulation requirement", Ericsson.

- R4-2210652	"Big CR to 38.101-4: Introduction of FR1 1024QAM UE demodulation and CQI reporting requirements", Ericsson.

- R4-2210897	"Draft CR for PDSCH demodulation requirements for 1KQAM in TDD", Apple.

- R4-2210898	"Draft CR on Applicability Rules and TDLD30-5 delay profile for FR1 DL 1024QAM PDSCH Requirements", Qualcomm Incorporated.

- R4-2210899	"Draft CR to TS38.101-4, PDSCH requirements for 1024QAM in FR1 FDD", MediaTek inc.

- R4-2207796	"Discussion on SDR requirements with 1KQAM", Apple.

- R4-2209067	"draft CR: Introduction of SDR requirements for DL 1024QAM", Ericsson.

- R4-2210900	"draft CR: Introduction of SDR requirements for DL 1024QAM", Ericsson.

- R4-2209066	"CQI reporting requirements for DL 1024QAM", Ericsson.

- R4-2209068	"draft CR: Introduction of TDD CQI reporting requirements for DL 1024QAM", Ericsson.

- R4-2209176	"CQI requirements for 1024QAM FR1 UE Demod Tests", Qualcomm Incorporated.

- R4-2209430	"Draft CR on Applicability Rules for FR1 DL 1024QAM CQI Requirements", Qualcomm Incorporated.

- R4-2209802	"Simulation results and discussion on the CQI requirements for 1024QAM", MediaTek inc.

- R4-2209872	"Discussion and simulation results on 1024QAM CQI", Huawei, HiSilicon.

- R4-2209873	"Draft CR on FDD CQI reporting cases for 1024QAM and CSI RMC (TS38.101-4, Rel-17)", Huawei, HiSilicon.

- R4-2210901	"draft CR: Introduction of TDD CQI reporting requirements for DL 1024QAM", Ericsson.

- R4-2210902	"Draft CR on Applicability Rules for FR1 DL 1024QAM CQI Requirements", Qualcomm Incorporated.

- R4-2210903	"Draft CR on FDD CQI reporting cases for 1024QAM and CSI RMC (TS38.101-4, Rel-17)", Huawei, HiSilicon.

21.05.2022		minor adaptations for RAN #96

10.01.2022		minor adaptations for RAN #95e

04.10.2021		minor adaptations for RAN #94e

08.08.2021		minor adaptations for RAN #93e

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template