3GPP TSG RAN meeting #96									        	       RP-221231

Budapest, Hungary, June 6-9, 2022

Status Report to TSG

Agenda item:			9.2.7

| WI / SI Name | Study on NR network-controlled repeaters |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: Yes | Core part: No | Performance part: No |  | Testing part: No |
| Acronym | FS_NR_netcon_repeater |  |  |  |  |
| Unique ID | 940083 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-213700 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: 09/2022 | Core part: | Performance part: | Testing part: |  |
| Overall Completion level | Study Item: 40% for Overall 60% for RAN1 0% for RAN2 0% for RAN3 | Core part: | Performance Part: | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN1 |
| --- | --- | --- |
| Rapporteur | Name | Nan Zhang |
|  | Company | ZTE |
|  | Email | Zhang.nan152@zte.com.cn |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

2.1	RAN1

2.1.1	Agreements

RAN1#109e

Agreement

The TR skeleton for TR 38.867 Study on NR network-controlled repeaters is endorsed in R1-2205231.

Note: Cost analysis on network controlled repeaters can be captured in the annex pending RAN1 consensus

Agreement

The TR is updated to R1-2205496.

Agreement

Capture the following model of network-controlled repeater in TR 38.867.

- The NCR-MT is defined as a function entity to communicate with a gNB via Control link (C-link) to enable the information exchanges (e.g. side control information). The C-link is based on NR Uu interface.

  - Note: Side control information is at least for the control of NCR-Fwd

- The NCR-Fwd is defined as a function entity to perform the amplify-and-forwarding of UL/DL RF signal between gNB and UE via backhaul link and access link. The behavior of the NCR-Fwd will be controlled according to the received side control information from gNB.

Agreement

At least for FR2, beam information is beneficial and recommended as the side control information for network-controlled repeater to control the behaviour of NCR at least for access link

- FFS: Detailed mechanism of indication.

- Note: There are no supporting evaluation results on FR1 at this point to reach similar conclusion

Agreement

Both fixed beam and adaptive beam can be considered at NCR for both C-link and backhaul-link.

- FFS: the mechanism for indication and determination of beam.

- Note: Fixed beam refers to the case that beam at NCR for both C-link and backhaul-link cannot be changed.

Agreement

For the TDD UL/DL configuration of network controller repeater:

- At least semi-static TDD UL/DL configuration is needed for network-controlled repeater for links including C-link, backhaul link and access link.

  - FFS: handling of flexible symbols

- Note1: The same TDD UL/DL configuration is always assumed for backhaul link and access link

- Note2: The same TDD UL/DL configuration is assumed for C-link and backhaul link and access link if NCR-MT and NCR-Fwd are in the same frequency band.

Agreement

For the timing of NCR, the following assumption is considered as baseline:

- The DL receiving timing of the NCR-Fwd is aligned with the DL receiving timing of the NCR-MT.

- The UL transmitting timing of the NCR-Fwd is aligned with the UL transmitting timing of the NCR-MT.

- FFS: the impact of internal delay on the following timing relationships:

  - The DL receiving timing and DL transmitting timing of the NCR-Fwd

  - The UL transmitting timing and UL receiving timing of the NCR-Fwd

Agreement

ON-OFF information is beneficial and recommended for network-controlled repeater to control the behaviour of NCR-Fwd.

- FFS: Detailed mechanism of ON-OFF indication and determination

- FFS: explicit indication or implicit indication of ON-OFF information

Agreement

Both the dynamic indication and semi-static indication can be considered for the beam of access link for NCR-Fwd.

- FFS: the details of each indication

- FFS: the maximum number of beams configured for NCR-Fwd access link

Agreement

The following options can be considered to indicate the ON-OFF information from gNB to NCR for controlling the behaviour of NCR-Fwd:

- Option 1: Explicit indication with on-off state (e.g., via dynamic or semi-static signalling) or on-off pattern (e.g., periodic/semi-static ON-OFF pattern or new DRX-like pattern for ON-OFF)

- Option 2: Implicit indication via the signalling for other information (e.g., beam, DL/UL configuration, or PC information)

  - Note: This example does not imply that PC information is necessary or not.

- Other solutions (e.g., potential combination of explicit and implication solution) can be further discussed.

Agreement

Capture the following assumption of network-controlled repeater in TR 38.867.

- At least one of the NCR-MT’s carrier(s) should be within the set of carriers forwarded by the NCR-Fwd in same frequency range.

  - The NCR-MT and NCR-Fwd operating in the same carrier is prioritized for the study.

Agreement

Capture the following assumption of network-controlled repeater in TR 38.867.

- As baseline, same large-scale properties of the channel, i.e., channel properties in Type-A and Type-D (if applicable), are expected to be experienced by C-link and backhaul link (at least when the NCR-MT and NCR-Fwd operating in same carrier).

Agreement

As baseline, the same TCI states as C-link are assumed for beam at NCR-Fwd for backhaul link if the NCR-MT’s carrier(s) is within the set of carriers forwarded by the NCR-Fwd.

- FFS: additional indication from gNB to determine the beam at NCR-Fwd for backhaul link or implicit determination of the beam at NCR-Fwd for backhaul link

Note: the same assumption of the beam correspondence is applied for DL/UL of the backhaul link at NCR-Fwd as the DL/UL of the C-link at NCR-MT.

Agreement

The beam correspondence is assumed for:

- the DL/UL of the access link at NCR-Fwd

Agreement

Recommend to capture the following observation in TR 38.867:

- The benefits of power information used to control the behavior of NCR-Fwd for the DL of access link and/or UL of backhaul link are observed by the following inputs:

  - [Source-1, Huawei] shows that for the uplink transmission via NCR, a fixed NCR amplifying gain may lead to interference to the gNB or NCR UL coverage loss. For the downlink transmission via NCR, a fixed NCR amplifying gain may lead to NCR RU saturation or NCR DL coverage loss.

  - [Source-2, vivo] shows that the optimal system performance can be achieved when repeater’s gain is set to a proper value.

  - [Source-3, ETRI] shows that dynamic repeater gain/power control can provide additional SINR gain over semi-static repeater gain/power configuration.

  - [Source-4, Ericsson] mentions that the gain control is needed for self-interference management due to repeater oscillation.

- This agreement does not change the prioritization of PC

Agreement

The controlling of the amplifying gain of NCR-Fwd is considered to enable the power control of NCR-Fwd if PC is recommended as side control information for NCR in Rel-18

- FFS: Controlling of the transmission power of NCR-Fwd

Agreement

Recommend to capture the following examples of the transmission/reception of C-link and backhaul link by NCR in TR 38.867.

- The DL of C-link and DL of backhaul link can be performed simultaneously or in TDM way.

- The UL of C-link and UL of backhaul link can be performed in TDM way

- Note-1: Multiplexing is under the control of gNB with consideration for NCR capability

- Note-2: Simultaneous transmission of the UL of C-link and UL of backhaul link is subject to NCR’s capability

Agreement

The NCR-MT can obtain the necessary configuration for receiving the L1/L2 signaling of the side control information.

- Option 1: The necessary configuration is from RRC.

- Option 2: The necessary configuration is from OAM or hard-coded.

- Option 3: The necessary configuration is partially configured by RRC and partially configured by OAM or hard-coded.

Agreement

For indication of NCR-Fwd ON-OFF for efficient interference management and improved energy efficiency, both dynamic and semi-static indication can be considered

- FFS: RAN1 to consider whether/how to handle the forwarding of broadcast and cell-specific signals/channels.

Agreement

From the perspective of signaling design, following mechanisms can be considered for the access link beamforming of the NCR-Fwd.

- Option #2-1: Dynamic beam indication only

- Option #2-2: Semi-static beam indication only

- Option #2-3: Dynamic beam indication and semi-static beam indication

Agreement

For an NCR-MT, the necessary configurations from RRC and/or OAM(or hard-coded) contain:

- The configurations of PHY channels to carry the L1/L2 signaling:

  - The configurations for receiving PDCCH and PDSCH.

  - The configurations for transmitting PUCCH, if needed.

  - The configurations for transmitting PUSCH, if needed.

- The configurations of L1/L2 signaling:

  - The configurations for DCI.

  - The configurations for UCI, if needed.

  - The configurations for MAC CE, if needed.

Agreement

In the access link beam indication, an access link beam can be indicated by:

- Option 1: A beam index

  - FFS: How to indicate the corresponding time domain resource of the beam.

- Option 2: An index of a source RS (e.g. a TCI-like indicator)

  - FFS: The definition of the source RS.

  - FFS: How to indicate the corresponding time domain resource of the beam.

  - FFS: The definition of the association between the source RS and the beam.

- Note: The above does not imply that the NCR can or cannot generate and transmit reference signals to a UE or receive and process reference signals from a UE.

RAN1 to select one of the two options, combine the two options, or select both options in RAN1#110

Conclusion

Legacy UE mechanism is sufficient to achieve DL/UL timing for NCR-MT

Agreement

For the parameters in the necessary configurations for L1/L2 signaling, the existing parameters for PDCCH, PDSCH, PUCCH, PUSCH, DCI, UCI and MAC CE in Rel-17 are the baseline for further discussion.

- Note 1: This does not imply that all Rel-17 parameters will be supported for the NCR-MT.

- Note 2: This does not imply that PUCCH, PUSCH, UCI and MAC CE are currently agreed to be supported. Further consideration is needed.

Agreement

For the signaling of information on UL-DL TDD configuration, if the NCR-MT can acquire the TDD configuration as legacy UEs or from the OAM, new signaling may not be necessary.

- Note 1: The same TDD UL/DL configuration is assumed for C-link and backhaul link and access link if the NCR-MT and the NCR-Fwd are in the same frequency band.

- FFS: Other cases where new signaling may be necessary.

Agreement

For the signaling of the side control information of timing to align transmission / reception boundaries, new signaling may be unnecessary.

- FFS: the impact of internal delay

Agreement

The time at which the NCR applies an access link beam indication should be considered.

Agreement

As for the time-domain granularity of the access link beam indication, one or both of the following options can be considered:

- Option 1: slot-level

- Option 2: symbol-level

- FFS: The details of indication signaling

2.1.2	Remaining Open issues

Remaining FFS on the details of candidate approaches for the indication of beam information and ON-OFF information.

Remaining FFS on the necessity of new signalling for TDD configuration and Timing.

Further study on the PC information for NCR, e.g., details of signalling for indication, and conclude whether it’s recommend to NCR in Rel-18.

2.2	RAN2

2.2.1	Agreements

Void

2.2.2	Remaining Open issues

Study the following aspects of network-controlled repeater management

- Identification and authorization of network-controlled repeaters [RAN2, RAN3]

NOTE2: Coordination with SA3 may be needed.

2.3	RAN3

2.3.1	Agreements

Void

2.3.2	Remaining Open issues

Study the following aspects of network-controlled repeater management

- Identification and authorization of network-controlled repeaters [RAN2, RAN3]

NOTE2: Coordination with SA3 may be needed.

4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

R1-2203236	Work plan for NR network-controlled repeaters

R1-2205231	TR 38.867 skeleton for Study on NR network-controlled repeaters

R1-2205496	TR 38.867 skeleton for Study on NR network-controlled repeaters

R1-2203133	Discussion on side control information to enable NR network-controlled repeaters

R1-2203237	Discussion on side control information to enable NR network-controlled repeaters

R1-2203343	Discussion on side control information to enable NR network-controlled repeaters

R1-2203354	Discussion on side control information to enable NR network-controlled repeaters

R1-2203376	Discussions on side control information to enable NR network-controlled repeaters

R1-2203476	Discussion on side control information for NR network-controlled repeaters

R1-2203578	Discussion on side control information to enable NR network-controlled repeaters

R1-2203697	Discussion on side control information for network-controlled repeaters

R1-2203741	Considerations on side control information to enable NR network-controlled repeaters

R1-2203832	Discussion on side control information to enable NR network-controlled repeaters

R1-2203921	Side control information to enable NR network-controlled repeaters

R1-2204064	Discussion on side control information for network controllable repeater

R1-2204066	Side Control Information for Network-Controlled Repeaters

R1-2204086	Side Control Information for supporting initial access

R1-2204119	Side control information for network-controlled repeaters

R1-2204258	Potential side control information for NW-controlled repeater

R1-2204321	Discussion on Side control information to enable NR network-controlled repeaters

R1-2204393	Discussion on side control information to enable NR network-controlled repeaters

R1-2204532	Discussion on side control information for NCR

R1-2204565	Discussion on side control signals for network-controlled repeaters

R1-2204610	Discussion on side control information to enable NR network-controlled repeaters

R1-2204642	Control information for enabling NW-controlled repeaters

R1-2204653	Discussions on side control information for network-controlled repeater

R1-2204688	Side control information for network-controlled repeaters

R1-2204757	Discussion on Side control information to enable NR network-controlled repeaters

R1-2204813	Discussion on Side control information to enable NR network-controlled repeater

R1-2204847	Discussion on side control information to enable NR network-controlled repeaters

R1-2204864	Side control information to enable NR network-controlled repeaters

R1-2205047	On side control information for network controlled repeaters (NCR)

R1-2205068	Initial view on Network-controlled repeater

R1-2205085	Side control information for NR network-controlled repeaters

R1-2205233	Summary#1 of Email discussion on side control information

R1-2205340	Summary#2 of Email discussion on side control information

R1-2205495	Summary#3 of Email discussion on side control information

R1-2203134	Discussion on L1/L2 signaling for side control information

R1-2203238	Discussion on L1/L2 signaling for side control information

R1-2203355	Discussion on L1/L2 signaling for side control information of network-controlled repeaters

R1-2203377	Discussion on L1/L2 signaling for side control information

R1-2203477	Discussion on signalling of  side control informaion  for NR network-controlled repeaters

R1-2203579	Discussion on signaling for side control information

R1-2203698	Discussion on signaling for side control information

R1-2203742	Considerations on L1/L2 signaling for side control information

R1-2203833	Discussion on L1/L2 signalling for side control information

R1-2203922	L1/L2 signaling for side control information

R1-2204065	Discussion on L1/L2 signaling for side control information of network-controlled repeaters

R1-2204259	Discussion on L1/L2 signaling for side control information

R1-2204322	Discussion on L1/L2 signaling for side control information

R1-2204394	Discussion on L1/L2 signaling for side control information

R1-2204533	Discussion on L1/L2 signaling for side control information

R1-2204643	Signaling of control information for NW-controlled repeaters

R1-2204654	Discussions on L1/L2 signaling for side control information

R1-2204689	L1/L2 control signaling for enabling network controlled repeaters

R1-2204758	Discussion on L1/L2 signaling for side control information for NCR

R1-2204814	Disucssion on L1/L2 signaling for side control information

R1-2205048	On L1-L2 signaling for side control information for NCR

R1-2205069	Initial view on signaling for Network-controlled repeater

R1-2205086	NR network-controlled repeaters receive signaling of side control information

R1-2205237	Summary#1 on L1/L2 signaling for side control information

R1-2205456	Summary#2 on L1/L2 signaling for side control information

R1-2205519	Summary#3 on L1/L2 signaling for side control information

R1-2203239	Discussion on other issues for NR network-controlled repeater

R1-2203580	Discussion on in-band repeater

R1-2204067	Deployment Scenarios for Network-Controlled Repeaters

R1-2204534	Discussion on operation scenarios for NCR

R1-2204644	Terminology for NW-controlled repeaters

R1-2204930	Other considerations for NR network-controlled repeaters

R1-2205087	Other issues on network-controlled repeaters

10.01.2022		minor adaptations for RAN #95e

04.10.2021		minor adaptations for RAN #94e

08.08.2021		minor adaptations for RAN #93e

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template