3GPP TSG-RAN5 Meeting #95-e	R5-223105

# Electronic Meeting, 09th – 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-2 | CR | 0336 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add PICS for MBS test |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS_5MBS_5MBUSA-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1, Accroding to 23.247, UE may join in multicast session by sending a PDU Session Modification Request or PDU Session Establishment Request. [TS 23.247 7.2.1.3] 1. To join the multicast group, the UE sends a PDU Session Modification Request for the associated PDU session which additionally contains one or several MBS Session ID(s) and join request. The MBS Session ID(s) indicate the multicast MBS session(s) that UE wants to join. The UE may alternatively join the multicast MBS session by sending PDU Session Establishment Request for associated PDU session together with one or several MBS Session ID(s) and join request. In that case, before step 2, the network proceeds with establishment of the associated PDU session executing steps 4 to 10 of PDU Session Establishment procedure as specified in TS 23.502 [6] clause 4.3.2.2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1, Add PICS for different UE implement. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not easy to distinguish the different UE join in multicast session procedure without PICS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4.4-2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

Table A.4.4-2: Definition of UE implementation capabilities

| Item | Definition of UE implementation capabilities | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | Void |  |  |  |  |
| 2 | Void |  |  |  |  |
| 3 | Number of UE-requested PDU session establishments after REGISTRATION during the same signalling connection | 24.501 | Rel-15 | pc_noOf_PDUsSameConnection |  |
| 4 | Number of UE-requested PDU session establishments after REGISTRATION in a new signalling connection | 24.501 | Rel-15 | pc_noOf_PDUsNewConnection |  |
| 5 | Number of UE-requested PDN connection establishments after ATTACH during the same signalling connection | 24.301 | Rel-15 | pc_noOf_PDNsSameConnection |  |
| 6 | Number of UE-requested PDN connection establishments after ATTACH in a new signalling connection | 24.301 | Rel-15 | pc_noOf_PDNsNewConnection |  |
| 7 | Void | 24.501 | Rel-15 | pc_IMS_DNN_default |  |
| 8 |  | Support of Emergency PDU session transfer from N1 mode to S1 mode when network does not support N26 interface | TS 24.501, 6.1.4.2 | Rel-15 | pc_TransferEmergencyPDUN1toS1noN26 | Will the UE attempt to transfer an existing Emergency PDU session upon inter-system change from N1 mode to S1 mode in EMM-IDLE mode if the network does not support N26 interface |
| 9 |  | Support of Emergency PDN connection transfer from S1 mode to N1 mode when network does not support N26 interface | TS 24.501, 6.1.4.2 | Rel-15 | pc_TransferEmergencyPDUS1toN1noN26 | Will the UE attempt to transfer an existing Emergency PDN connection upon inter-system change from S1 mode to N1 mode in EMM-IDLE mode if the network does not support N26 interface |
| 10 |  | Support of UE's usage setting as data centric | TS 24.501, 4.3.1 | Rel-15 | pc_data_centric | UE supports to be configured to consistently behave as a Data centric UE. |
| X |  | Support of join in MBS multicast session by sending a PDU Session Modification Request | TS 23.247 7.2.1 | Rel-17 | pc_Join_MBS_by_PDU_Modification | If pc_Join_MBS_by_PDU_Modification, UE join in MBS multicast session by sending a PDU Session Modification Request, else UE join in MBS multicast session by sending a PDU Session Establishment Request |

<End of modified section 1>