3GPP TSG RAN Meeting #96	RP-221347

Budapest, Hungary, June 6-9, 2022

## Status Report to TSG

Agenda item:			9.2.8

| WI / SI Name | Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: Yes | Core part: No | Performance part: No |  | Testing part: No |
| Acronym | FS_NR_AIML_Air |  |  |  |  |
| Unique ID | 940084 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-213599 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: 12/2023 | Core part: N/A | Performance part: N/A | Testing part: N/A |  |
| Overall Completion level | Study Item: 5% | Core part: N/A | Performance Part: N/A | Testing part: N/A |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | TSG RAN WG1 |
| --- | --- | --- |
| Rapporteur | Name | Juan Montojo |
|  | Company | Qualcomm |
|  | Email | juanm@qti.qualcomm.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

General aspects of AI/ML framework

Agreement

Use 3gpp channel models (TR 38.901) as the baseline for evaluations.

Note: Companies may submit additional results based on other dataset than generated by 3GPP channel models

Working Assumption

Include the following into a working list of terminologies to be used for RAN1 AI/ML air interface SI discussion.

The description of the terminologies may be further refined as the study progresses.

New terminologies may be added as the study progresses.

It is FFS which subset of terminologies to capture into the TR.

Table: Working list of terminologies

| Terminology | Description |
| --- | --- |
| Data collection | A process of collecting data by the network nodes, management entity, or UE for the purpose of AI/ML model training, data analytics and inference |
| AI/ML Model | A data driven algorithm that applies AI/ML techniques to generate a set of outputs based on a set of inputs. |
| AI/ML model training | A process to train an AI/ML Model [by learning the input/output relationship] in a data driven manner and obtain the trained AI/ML Model for inference |
| AI/ML model Inference | A process of using a trained AI/ML model to produce a set of outputs based on a set of inputs |
| AI/ML model validation | A subprocess of training, to evaluate the quality of an AI/ML model using a dataset different from one used for model training, that helps selecting model parameters that generalize beyond the dataset used for model training. |
| AI/ML model testing | A subprocess of training, to evaluate the performance of a final AI/ML model using a dataset different from one used for model training and validation. Differently from AI/ML model validation, testing does not assume subsequent tuning of the model. |
| UE-side (AI/ML) model | An AI/ML Model whose inference is performed entirely at the UE |
| Network-side (AI/ML) model | An AI/ML Model whose inference is performed entirely at the network |
| One-sided (AI/ML) model | A UE-side (AI/ML) model or a Network-side (AI/ML) model |
| Two-sided (AI/ML) model | A paired AI/ML Model(s) over which joint inference is performed, where joint inference comprises AI/ML Inference whose inference is performed jointly across the UE and the network, i.e, the first part of inference is firstly performed by UE and then the remaining part is performed by gNB, or vice versa. |
| AI/ML model transfer | Delivery of an AI/ML model over the air interface, either parameters of a model structure known at the receiving end or a new model with parameters. Delivery may contain a full model or a partial model. |
| Model download | Model transfer from the network to UE |
| Model upload | Model transfer from UE to the network |
| Federated learning / federated training | A machine learning technique that trains an AI/ML model across multiple decentralized edge nodes (e.g., UEs, gNBs) each performing local model training using local data samples. The technique requires multiple interactions of the model, but no exchange of local data samples. |
| Offline field data | The data collected from field and used for offline training of the AI/ML model |
| Online field data | The data collected from field and used for online training of the AI/ML model |
| Model monitoring | A procedure that monitors the inference performance of the AI/ML model |
| Supervised learning | A process of training a model from input and its corresponding labels. |
| Unsupervised learning | A process of training a model without labelled data. |
| Semi-supervised learning | A process of training a model with a mix of labelled data and unlabelled data |
| Reinforcement Learning (RL) | A process of training an AI/ML model from input (a.k.a. state) and a feedback signal (a.k.a. reward) resulting from the model’s output (a.k.a. action) in an environment the model is interacting with. |
| Model activation | enable an AI/ML model for a specific function |
| Model deactivation | disable an AI/ML model for a specific function |
| Model switching | Deactivating a currently active AI/ML model and activating a different AI/ML model for a specific function |

Conclusion

As indicated in SID, although specific AI/ML algorithms and models may be studied for evaluation purposes, AI/ML algorithms and models are implementation specific and are not expected to be specified.

Observation

Where AI/ML functionality resides depends on specific use cases and sub-use cases.

Conclusion

- RAN1 discussion should focus on network-UE interaction.

  - AI/ML functionality mapping within the network (such as gNB, LMF, or OAM) is up to RAN2/3 discussion.

Agreement

Take the following network-UE collaboration levels as one aspect for defining collaboration levels

- Level x: No collaboration

- Level y: Signaling-based collaboration without model transfer

- Level z: Signaling-based collaboration with model transfer

Note: Other aspect(s), for defining collaboration levels is not precluded and will be discussed in later meetings, e.g., with/without model updating, to support training/inference, for defining collaboration levels will be discussed in later meetings

FFS: Clarification is needed for Level x-y boundary

Evaluation on AI/ML for CSI feedback enhancement

Agreement

For the performance evaluation of the AI/ML based CSI feedback enhancement, system level simulation approach is adopted as baseline

- Link level simulation is optionally adopted

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, for the calibration purpose on the dataset and/or AI/ML model over companies, consider to align the parameters (e.g., for scenarios/channels) for generating the dataset in the simulation as a starting point.

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, for ‘Channel estimation’, ideal DL channel estimation is optionally taken into the baseline of EVM for the purpose of calibration and/or comparing intermediate results (e.g., accuracy of AI/ML output CSI, etc.)

- Note: Eventual performance comparison with the benchmark release and drawing SI conclusions should be based on realistic DL channel estimation.

- FFS: the ideal channel estimation is applied for dataset construction, or performance evaluation/inference.

- FFS: How to model the realistic channel estimation

- FFS: Whether ideal channel is used as target CSI for intermediate results calculation with AI/ML output CSI from realistic channel estimation

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, companies can consider performing intermediate evaluation on AI/ML model performance to derive the intermediate KPI(s) (e.g., accuracy of AI/ML output CSI) for the purpose of AI/ML solution comparison.

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, Floating point operations (FLOPs) is adopted as part of the ‘Evaluation Metric’, and reported by companies.

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, AI/ML memory storage in terms of AI/ML model size and number of AI/ML parameters is adopted as part of the ‘Evaluation Metric’, and reported by companies who may select either or both.

- FFS: the format of the AI/ML parameters

Agreement

For the evaluation of the AI/ML based CSI compression sub use cases, a two-sided model is considered as a starting point, including an AI/ML-based CSI generation part to generate the CSI feedback information and an AI/ML-based CSI reconstruction part which is used to reconstruct the CSI from the received CSI feedback information.

- At least for inference, the CSI generation part is located at the UE side, and the CSI reconstruction part is located at the gNB side.

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, if SLS is adopted, the following table is taken as a baseline of EVM

- Note: the following table captures the common parts of the R16 CSI enhancement EVM table and the R17 CSI enhancement EVM table, while the different parts are FFS.

- Note: the baseline EVM is used to compare the performance with the benchmark release, while the AI/ML related parameters (e.g., dataset construction, generalization verification, and AI/ML related metrics) can be of additional/different assumptions.

  - The conclusions for the use cases in the SI should be drawn based on generalization verification over potentially multiple scenarios/configurations.

- FFS: modifications on top of the following table for the purpose of AI/ML related evaluations.

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD (TDD is not precluded), OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Dense Urban (Macro only) is a baseline. Other scenarios (e.g. UMi@4GHz 2GHz, Urban Macro) are not precluded. |
| Frequency Range |  | FR1 only, FFS 2GHz or 4GHz as a baseline |
| Inter-BS distance |  | 200m |
| Channel model |  | According to TR 38.901 |
| Antenna setup and port layouts at gNB |  | Companies need to report which option(s) are used between - 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ - 16 ports: (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ Other configurations are not precluded. |
| Antenna setup and port layouts at UE |  | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for (rank 1-4) 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) Other configuration is not precluded. |
| BS Tx power |  | 41 dBm for 10MHz, 44dBm for 20MHz, 47dBm for 40MHz |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Modulation |  | Up to 256QAM |
| Coding on PDSCH |  | LDPC Max code-block size=8448bit |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 15kHz for 2GHz, 30kHz for 4GHz |
| Simulation bandwidth |  | FFS |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | FFS |
| MIMO layers |  | For all evaluation, companies to provide the assumption on the maximum MU layers (e.g. 8 or 12) |
| CSI feedback |  | Feedback assumption at least for baseline scheme - CSI feedback periodicity (full CSI feedback) : 5 ms, - Scheduling delay (from CSI feedback to time to apply in scheduling) : 4 ms |
| Overhead |  | Companies shall provide the downlink overhead assumption (i.e., whether the CSI-RS transmission is UE-specific or not and take that into account for overhead computation) |
| Traffic model |  | FFS |
| Traffic load (Resource utilization) |  | FFS |
| UE distribution |  | - 80% indoor (3km/h), 20% outdoor (30km/h) FFS whether/what other indoor/outdoor distribution and/or UE speeds for outdoor UEs needed |
| UE receiver |  | MMSE-IRC as the baseline receiver |
| Feedback assumption |  | Realistic |
| Channel estimation |  | Realistic as a baseline FFS ideal channel estimation |
| Evaluation Metric |  | Throughput and CSI feedback overhead as baseline metrics. Additional metrics, e.g., ratio between throughput and CSI feedback overhead, can be used. Maximum overhead (payload size for CSI feedback)for each rank at one feedback instance is the baseline metric for CSI feedback overhead, and companies can provide other metrics. |
| Baseline for performance evaluation |  | FFS |

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, as a starting point, take the intermediate KPIs of GCS/SGCS and/or NMSE as part of the ‘Evaluation Metric’ to evaluate the accuracy of the AI/ML output CSI

- For GCS/SGCS,

  - FFS: how to calculate GCS/SGCS for rank>1

  - FFS: whether GCS or SGCS is adopted

- FFS other metrics, e.g., equivalent MSE, received SNR, or numerical spectral efficiency gap.

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, if LLS is preferred, the following table is taken as a baseline of EVM

- Note: the baseline EVM is used to compare the performance with the benchmark release, while the AI/ML related parameters (e.g., dataset construction, generalization verification, and AI/ML related metrics) can be of additional/different assumptions.

  - The conclusions for the use cases in the SI should be drawn based on generalization verification over potentially multiple scenarios/configurations.

- FFS: modifications on top of the following table for the purpose of AI/ML related evaluations.

- FFS: other parameters and values if needed

| Parameter | Value |
| --- | --- |
| Duplex, Waveform | FDD (TDD is not precluded), OFDM |
| Carrier frequency | 2GHz as baseline, optional for 4GHz |
| Bandwidth | 10MHz or 20MHz |
| Subcarrier spacing | 15kHz for 2GHz, 30kHz for 4GHz |
| Nt | 32: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ |
| Nr | 4: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ |
| Channel model | CDL-C as baseline, CDL-A as optional |
| UE speed | 3kmhr, 10km/h, 20km/h or 30km/h to be reported by companies |
| Delay spread | 30ns or 300ns |
| Channel estimation | Realistic channel estimation algorithms (e.g. LS or MMSE) as a baseline, FFS ideal channel estimation |
| Rank per UE | Rank 1-4. Companies are encouraged to report the Rank number, and whether/how rank adaptation is applied |

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, study the verification of generalization. Companies are encouraged to report how they verify the generalization of the AI/ML model, including:

- The configuration(s)/ scenario(s) for training dataset, including potentially the mixed training dataset from multiple configurations/scenarios

- The configuration(s)/ scenario(s) for testing/inference

- Other details are not precluded

Agreement

For the evaluation of the AI/ML based CSI compression sub use cases, companies are encouraged to report the details of their models, including:

- The structure of the AI/ML model, e.g., type (CNN, RNN, Transformer, Inception, …), the number of layers, branches, real valued or complex valued parameters, etc.

- The input CSI type, e.g., raw channel matrix estimated by UE, eigenvector(s) of the raw channel matrix estimated by UE, etc.

  - FFS: the input CSI is obtained from the channel with or without analog BF

- The output CSI type, e.g., channel matrix, eigenvector(s), etc.

- Data pre-processing/post-processing

- Loss function

- Others are not precluded

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, if SLS is adopted, the following parameters are taken into the baseline of EVM

- Note: The 2nd column applies if R16 TypeII codebook is selected as baseline, and the 3rd column applies if R17 TypeII codebook is selected as baseline.

  - Additional assumptions from R17 TypeII EVM Same consideration with respect to utilizing angle-delay reciprocity should be considered taken for the AI/ML based CSI feedback and the baseline scheme if R17 TypeII codebook is selected as baseline

  - FFS baseline for potential sub use cases involving CSI enhancement on time domain

- Note: the baseline EVM is used to compare the performance with the benchmark release, while the AI/ML related parameters (e.g., dataset construction, generalization verification, and AI/ML related metrics) can be of additional/different assumptions.

  - The conclusions for the use cases in the SI should be drawn based on generalization verification over potentially multiple scenarios/configurations.

- FFS: modifications on top of the following table for the purpose of AI/ML related evaluations.

| Parameter | Value (if R16 as baseline) | Value (if R17 as baseline) |
| --- | --- | --- |
| Frequency Range | FR1 only, 2GHz as baseline, optional for 4GHz. | FR1 only, 2GHz with duplexing gap of 200MHz between DL and UL, optional for 4GHz |
| Simulation bandwidth | 10 MHz for 15kHz as a baseline, and configurations which emulate larger BW, e.g., same sub-band size as 40/100 MHz with 30kHz, may be optionally considered. Above 15kHz is replaced with 30kHz SCS for 4GHz. | 20 MHz for 15kHz as a baseline (optional for 10 MHz with 15KHz), and configurations which emulate larger BW, e.g., same sub-band size as 40/100 MHz with 30kHz, may be optionally considered. Above 15kHz is replaced with 30kHz SCS for 4GHz |
| MIMO scheme | SU/MU-MIMO with rank adaptation. Companies are encouraged to report the SU/MU-MIMO with RU | SU/MU-MIMO with rank adaptation. Companies are encouraged to report the SU/MU-MIMO with RU |
| Traffic load (Resource utilization) | 20/50/70% Companies are encouraged to report the MU-MIMO utilization. | 20/50/70% Companies are encouraged to report the MU-MIMO utilization. |

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, if SLS is adopted, the ‘Baseline for performance evaluation’ in the baseline of EVM is captured as follows

| Baseline for performance evaluation | Companies need to report which option is used between - Rel-16 TypeII Codebook as the baseline for performance and overhead evaluation. - Rel-17 TypeII Codebook as the baseline for performance and overhead evaluation. - FFS: Whether Type I Codebook can be optionally considered at least for performance evaluation |
| --- | --- |

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, if the GCS/SGCS is adopted as the intermediate KPI as part of the ‘Evaluation Metric’ for rank>1 cases, companies to report the GCS/SGCS calculation/extension methods, including:

     Method 1: Average over all layers

o    Note:  is the eigenvector of the target CSI at resource unit i and K is the rank. is the  output vector of the output CSI of resource unit i.  is the total number of resource units.  denotes the average operation over multiple samples.

     Method 2: Weighted average over all layers

o    Note: Companies to report the formula (e.g., whether normalization is applied for eigenvalues)

     Method 3: GCS/SGCS is separately calculated for each layer (e.g., for K layers, K GCS/SGCS values are derived respectively, and comparison is performed per layer)

       Other methods are not precluded

       FFS: Further down-selection among the above options or take one/a subset of the above methods as baseline(s).

Other aspects on AI/ML for CSI feedback enhancement

Agreement

Spatial-frequency domain CSI compression using two-sided AI model is selected as one representative sub use case.

  - Note: Study of other sub use cases is not precluded.

  - Note: All pre-processing/post-processing, quantization/de-quantization are within the scope of the sub use case.

Conclusion

- Further discuss temporal-spatial-frequency domain CSI compression using two-sided model as a possible sub-use case for CSI feedback enhancement after evaluation methodology discussion.

- Further discuss improving the CSI accuracy based on traditional codebook design using one-sided model as a possible sub-use case for CSI feedback enhancement after evaluation methodology discussion.

- Further discuss CSI prediction using one-sided model as a possible sub-use case for CSI feedback enhancement after evaluation methodology discussion

- Further discuss CSI-RS configuration and overhead reduction as a possible sub-use case for CSI feedback enhancement after evaluation methodology discussion

- Further discuss resource allocation and scheduling as a possible sub-use case for CSI feedback enhancement after evaluation methodology discussion

- Further discuss joint CSI prediction and compression as a possible sub-use case for CSI feedback enhancement after evaluation methodology discussion.

Evaluation on AI/ML for beam management

Agreement

- For dataset construction and performance evaluation (if applicable) for the AI/ML in beam management, system level simulation approach is adopted as baseline

  - Link level simulation is optionally adopted

Agreement

- At least for temporal beam prediction, companies report the one of spatial consistency procedures:

  - Procedure A in TR38.901

  - Procedure B in TR38.901

Agreement

- At least for temporal beam prediction, Dense Urban (macro-layer only, TR 38.913) is the basic scenario for dataset generation and performance evaluation.

  - Other scenarios are not precluded.

- For spatial-domain beam prediction, Dense Urban (macro-layer only, TR 38.913) is the basic scenario for dataset generation and performance evaluation.

  - Other scenarios are not precluded.

Agreement

- At least for spatial-domain beam prediction in initial phase of the evaluation, UE trajectory model is not necessarily to be defined.

Agreement

- At least for temporal beam prediction in initial phase of the evaluation, UE trajectory model is defined. FFS on the details.

Agreement

- UE rotation speed is reported by companies.

  - Note: UE rotation speed = 0, i.e., no UE rotation, is not precluded.

Agreement

- For AI/ML in beam management evaluation, RAN1 does not attempt to define any common AI/ML model as a baseline.

Conclusion

- Further study AI/ML model generalization in beam management evaluating the inference performance of beam prediction under multiple different scenarios/configurations.

  - FFS on different scenarios/configurations

  - Companies report the training approach, at least including the dataset assumption for training

Agreement

- For evaluation of AI/ML in BM, the KPI may include the model complexity and computational complexity.

  - FFS: the details of model complexity and computational complexity

Agreement

- For spatial-domain beam prediction, further study the following options as baseline performance

  - Option 1: Select the best beam within Set A of beams based on the measurement of all RS resources or all possible beams of beam Set A (exhaustive beam sweeping)

    - FFS CSI-RS/SSB as the RS resources

  - Option 2: Select the best beam within Set A of beams based on the measurement of RS resources from Set B of beams

    - FFS: Set B is a subset of Set A and/or Set A consists of narrow beams and Set B consists of wide beams

    - FFS: how conventional scheme to obtain performance KPIs

    - FFS: how to determine the subset of RS resources is reported by companies

  - Other options are not precluded.

Agreement

- For dataset generation and performance evaluation for AI/ML in beam management, take the parameters (if applicable) in Table 1.2-1b for Dense Urban scenario for SLS

Table 1.2-1b Assumptions for Dense Urban scenario for AI/ML in beam management

| Parameters | Values |
| --- | --- |
| Frequency Range | FR2 @ 30 GHz - SCS: 120 kHz |
| Deployment | 200m ISD, - 2-tier model with wrap-around (7 sites, 3 sectors/cells per site) Other deployment assumption is not precluded |
| Channel mode | UMa with distance-dependent LoS probability function defined in Table 7.4.2-1 in TR 38.901. |
| System BW | 80MHz |
| UE Speed | - For spatial domain beam prediction, 3km/h - For time domain beam prediction: 30km/h (baseline), 60km/h (optional) - Other values are not precluded |
| UE distribution | - FFS UEs per sector/cell for evaluation. More UEs per sector/cell for data generation is not precluded. - For spatial domain beam prediction: FFS: - Option 1: 80% indoor ,20% outdoor as in TR 38.901 - Option 2: 100% outdoor - For time domain prediction: 100% outdoor |
| Transmission Power | Maximum Power and Maximum EIRP for base station and UE as given by corresponding scenario in 38.802 (Table A.2.1-1 and Table A.2.1-2) |
| BS Antenna Configuration |  [One panel: (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1), (dV, dH) = (0.5, 0.5) λ as baseline]  [Four panels: (M, N, P, Mg, Ng) = (4, 8, 2, 2, 2), (dV, dH) = (0.5, 0.5) λ. (dg,V, dg,H) = (2.0, 4.0) λ as optional]  Other assumptions are not precluded. Companies to explain TXRU weights mapping. Companies to explain beam selection. Companies to explain number of BS beams |
| BS Antenna radiation pattern | TR 38.802 Table A.2.1-6, Table A.2.1-7 |
| UE Antenna Configuration | [Panel structure: (M,N,P) = (1,4,2)]  2 panels (left, right) with (Mg, Ng) = (1, 2) as baseline  Other assumptions are not precluded Companies to explain TXRU weights mapping. Companies to explain beam and panel selection. Companies to explain number of UE beams |
| UE Antenna radiation pattern | TR 38.802 Table A.2.1-8, Table A.2.1-10 |
| Beam correspondence | Companies to explain beam correspondence assumptions (in accordance to the two types agreed in RAN4) |
| Link adaptation | Based on CSI-RS |
| Traffic Model | FFS: - Option 1: Full buffer - Option 2: FTP model Other options are not precluded |
| Inter-panel calibration for UE | Ideal, non-ideal following 38.802 (optional) – Explain any errors |
| Control and RS overhead | Companies report details of the assumptions |
| Control channel decoding | Ideal or Non-ideal (Companies explain how it is modelled) |
| UE receiver type | MMSE-IRC as the baseline, other advanced receiver is not precluded |
| BF scheme | Companies explain what scheme is used |
| Transmission scheme | Multi-antenna port transmission schemes Note: Companies explain details of the using transmission scheme. |
| Other simulation assumptions | Companies to explain serving TRP selection Companies to explain scheduling algorithm |
| Other potential impairments | Not modelled (assumed ideal). If impairments are included, companies will report the details of the assumed impairments |
| BS Tx Power | [40 dBm] |
| Maximum UE Tx Power | 23 dBm |
| BS receiver Noise Figure | 7 dB |
| UE receiver Noise Figure | 10 dB |
| Inter site distance | 200m |
| BS Antenna height | 25m |
| UE Antenna height | 1.5 m |
| Car penetration Loss | 38.901, sec 7.4.3.2: μ = 9 dB, σp = 5 dB |

Agreement

- For temporal beam prediction, the following options can be considered as a starting point for UE trajectory model for further study. Companies report further changes or modifications based on the following options for UE trajectory model. Other options are not precluded.

  - Option #2: Linear trajectory model with random direction change.

    - UE moving trajectory: UE will move straightly along the selected direction to the end of an time interval, where the length of the time interval is provided by using an exponential distribution with average interval length, e.g., 5s, with granularity of 100 ms.

      - UE moving direction change: At the end of the time interval, UE will change the moving direction with the angle difference A_diff from the beginning of the time interval, provided by using a uniform distribution within [-45°, 45°].

      - UE move straightly within the time interval with the fixed speed.

    - FFS on UE orientation

  - Option #3: Linear trajectory model with random and smooth direction change.

    - UE moving trajectory: UE will change the moving direction by multiple steps within an time internal, where the length of the time interval is provided by using an exponential distribution with average interval length, e.g., 5s, with granularity of 100 ms.

      - UE moving direction change: At the end of the time interval, UE will change the moving direction with the angle difference A_diff from the beginning of the time interval, provided by using a uniform distribution within [-45°, 45°].

      - The time interval is further broken into N sub-intervals, e.g. 100ms per sub-interval, and at the end of each sub-interval, UE change the direction by the angle of A_diff/N.

      - UE move straightly within the time sub-interval with the fixed speed.

    - FFS on UE orientation

  - Option #4: Random direction straight-line trajectories.

- Initial UE location, moving direction and speed: UE is randomly dropped in a cell, and an initial moving direction is randomly selected, with a fixed speed.

  - The initial UE location should be randomly drop within the following blue area

where d1 is the minimum distance that UE should be away from the BS.

    - Each sector is a cell and that the cell association is geometry based.

    - During the simulation, inter-cell handover or switching should be disabled.

For training data generation

- For each UE moving trajectory: the total length of the UE trajectory can be set as T second if it is in time, of set as D meter if it is in distance.

  - The value of T (or D) can be further discussed

  - The trajectory sampling interval granularity depends on UE speed and it can be further discussed.

- UE can move straightly along the entire trajectory, or

- UE can move straightly during the time interval, where the time interval is provided by using an exponential distribution with average interval length ΔT

  - UE may change the moving direction at the end of the time interval. UE will change the moving direction with the angle difference A_diff from the beginning of the time interval, provided by using a uniform distribution within [-45°, 45°]

- If the UE trajectory hit the cell boundary (the red line), the trajectory should be terminated.

  - If the trajectory length (in time) is less than the length of observation window + prediction window, the trajectory should be discarded.

  - At the current stage, the length of observation window + prediction window is not fixed and the companies can report their values.

    - FFS on UE orientation

- Generalization issue is FFS

Agreement

- For temporal beam prediction, further study the following options as baseline performance

  - Option 1a: Select the best beam for T2 within Set A of beams based on the measurements of all the RS resources or all possible beams from Set A of beams at the time instants within T2

  - Option 2: Select the best beam for T2 within Set A of beams based on the measurements of all the RS resources from Set B of beams at the time instants within T1

    - Companies explain the detail on how to select the best beam for T2 from Set A based on the measurements in T1

  - Where T2 is the time duration for the best beam selection, and T1 is a time duration to obtain the measurements of all the RS resource from Set B of beams.

    - T1 and T2 are aligned with those for AI/ML based methods

  - Whether Set A and Set B are the same or different depend on the sub-use case

  - Other options are not precluded.

Agreement

- For dataset generation and performance evaluation for AI/ML in beam management, take the following assumption for LLS as optional methodology

| Parameter | Value |
| --- | --- |
| Frequency | 30GHz. |
| Subcarrier spacing | 120kHz |
| Data allocation | [8 RBs] as baseline, companies can report larger number of RBs First 2 OFDM symbols for PDCCH, and following 12 OFDM symbols for data channel |
| PDCCH decoding | Ideal or Non-ideal (Companies explain how is oppler) |
| Channel model | FFS: LOS channel: CDL-D extension, DS = 100ns NLOS channel: CDL-A/B/C extension, DS = 100ns Companies explains details of extension methodology considering spatial consistency Other channel models are not precluded. |
| BS antenna configurations | - One panel: (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1), (dV, dH) = (0.5, 0.5) λ as baseline - Other assumptions are not precluded. Companies to explain TXRU weights mapping. Companies to explain beam selection. Companies to explain number of BS beams |
| BS antenna element radiation pattern | Same as SLS |
| BS antenna height and antenna array downtile angle | 25m, 110° |
| UE antenna configurations | Panel structure: (M, N, P) = (1, 4, 2), - 2 panels (left, right) with (Mg, Ng) = (1, 2) as baseline - 1 panel as optional - Other assumptions are not precluded Companies to explain TXRU weights mapping. Companies to explain beam and panel selection. Companies to explain number of UE beams |
| UE antenna element radiation pattern | Same as SLS |
| UE moving speed | Same as SLS |
| Raw data collection format | Depends on sub-use case and companies’ choice. |

Agreement

- For UE trajectory model, UE orientation can be independent from UE moving trajectory model. FFS on the details.

  - Other UE orientation model is not precluded.

Agreement

- Companies are encouraged to report the following aspects of AI/ML model in RAN 1 #110. FFS on whether some of aspects need be defined or reported.

  - Description of AI/ML model, e.g, NN architecture type

  - Model inputs/outputs (per sub-use case)

  - Training methodology, e.g.

    - Loss function/optimization function

    - Training/ validity /testing dataset:

      - Dataset size, number of training/ validity /test samples

      - Model validity area: e.g., whether model is trained for single sector or multiple sectors

      - Details on Model monitoring and model update, if applicable

  - Others related aspects are not precluded

Agreement

- To evaluate the performance of AI/ML in beam management, further study the following KPI options:

  - Beam prediction accuracy related KPIs, may include the following options:

    - Average L1-RSRP difference of Top-1 predicted beam

    - Beam prediction accuracy (%) for Top-1 and/or Top-K beams, FFS the definition:

      - Option 1: The beam prediction accuracy (%) is the percentage of “the Top-1 predicted beam is one of the Top-K genie-aided beams”

      - Option 2: The beam prediction accuracy (%) is the percentage of “the Top-1 genie-aided beam is one of the Top-K predicted beams”

    - CDF of L1-RSRP difference for Top-1 predicted beam

    - Beam prediction accuracy (%) with 1dB margin for Top-1 beam

      - The beam prediction accuracy (%) with 1dB margin is the percentage of the Top-1 predicted beam “whose ideal L1-RSRP is within 1dB of the ideal L1-RSRP of the Top-1 genie-aided beam”

    - the definition of L1-RSRP difference of Top-1 predicted beam:

      - the difference between the ideal L1-RSRP of Top-1 predicted beam and the ideal L1-RSRP of the Top-1 genie-aided beam

    - Other beam prediction accuracy related KPIs are not precluded and can be reported by companies.

  - System performance related KPIs, may include the following options:

    - UE throughput: CDF of UE throughput, avg. and 5%ile UE throughput

    - RS overhead reduction at least for spatial-domain beam prediction at least for top-1 beam:

      - 1-N/M,

      - where N is the number of beams (with reference signal (SSB and/or CSI-RS)) required for measurement

      - where (FFS) M is the total number of beams

      - Note: Non-AI/ML approach based on the measurement of these M beams may be used as a baseline

      - FFS on whether to define a proper value for M for evaluation.

    - Other System performance related KPIs are not precluded and can be reported by companies.

o   Other KPIs are not precluded and can be reported by companies, for example:

  Reporting overhead reduction: (FFS) The number of UCI report and UCI payload size, for temporal /spatial prediction

  Latency reduction:

  (FFS) (1 – [Total transmission time of N beams] / [Total transmission time of M beams])

       where N is the number of beams (with reference signal (SSB and/or CSI-RS)) in the input beam set required for measurement

       where M is the total number of beams

  Power consumption reduction: FFS on details

Other aspects on AI/ML for beam management

Agreement

For AI/ML-based beam management, support BM-Case1 and BM-Case2 for characterization and baseline performance evaluations

- BM-Case1: Spatial-domain DL beam prediction for Set A of beams based on measurement results of Set B of beams

- BM-Case2: Temporal DL beam prediction for Set A of beams based on the historic measurement results of Set B of beams

- FFS: details of BM-Case1 and BM-Case2

- FFS: other sub use cases

Note: For BM-Case1 and BM-Case2, Beams in Set A and Set B can be in the same Frequency Range

Agreement

Regarding the sub use case BM-Case2, the measurement results of K (K>=1) latest measurement instances are used for AI/ML model input:

- The value of K is up to companies

Agreement

Regarding the sub use case BM-Case2, AI/ML model output should be F predictions for F future time instances, where each prediction is for each time instance.

- At least F = 1

- The other value(s) of F is up to companies

Agreement

For the sub use case BM-Case1, consider both Alt.1 and Alt.2 for further study:

- Alt.1: AI/ML inference at NW side

- Alt.2: AI/ML inference at UE side

Agreement

For the sub use case BM-Case2, consider both Alt.1 and Alt.2 for further study:

- Alt.1: AI/ML inference at NW side

- Alt.2: AI/ML inference at UE side

Conclusion

For the sub use case BM-Case1, consider the following alternatives for further study:

- Alt.1: Set B is a subset of Set A

o   FFS: the number of beams in Set A and B

o   FFS: how to determine Set B out of the beams in Set A (e.g., fixed pattern, random pattern, …)

- Alt.2: Set A and Set B are different (e.g. Set A consists of narrow beams and Set B consists of wide beams)

o   FFS: the number of beams in Set A and B

o   FFS: QCL relation between beams in Set A and beams in Set B

o   FFS: construction of Set B (e.g., regular pre-defined codebook, codebook other than regular pre-defined one)

- Note1: Set A is for DL beam prediction and Set B is for DL beam measurement.

- Note2: The narrow and wide beam terminology is for SI discussion only and have no specification impact

- Note3: The codebook constructions of Set A and Set B can be clarified by the companies.

Conclusion

Regarding the sub use case BM-Case1, further study the following alternatives for AI/ML input:

- Alt.1: Only L1-RSRP measurement based on Set B

- Alt.2: L1-RSRP measurement based on Set B and assistance information

  - FFS: Assistance information. The following were mentioned by companions in the discussion:  Tx and/or Rx beam shape information (e.g., Tx and/or Rx beam pattern, Tx and/or Rx beam boresight direction (azimuth and elevation), 3dB beamwidth, etc.), expected Tx and/or Rx beam for the prediction (e.g., expected Tx and/or Rx angle, Tx and/or Rx beam ID for the prediction), UE position information, UE direction information, Tx beam usage information, UE orientation information, etc.

    - Note: The provision of assistance information may be infeasible due to the concern of disclosing proprietary information to the other side.

- Alt.3: CIR based on Set B

- Alt.4: L1-RSRP measurement based on Set B and the corresponding DL Tx and/or Rx beam ID

- Note1: It is up to companies to provide other alternative(s) including the combination of some alternatives

- Note2: All the inputs are “nominal” and only for discussion purpose.

Conclusion

For the sub use case BM-Case2, further study the following alternatives with potential down-selection:

- Alt.1: Set A and Set B are different (e.g. Set A consists of narrow beams and Set B consists of wide beams)

  - FFS: QCL relation between beams in Set A and beams in Set B

- Alt.2: Set B is a subset of Set A (Set A and Set B are not the same)

  - FFS: how to determine Set B out of the beams in Set A (e.g., fixed pattern, random pattern, …)

- Alt.3: Set A and Set B are the same

- Note1: Predicted beam(s) are selected from Set A and measured beams used as input are selected from Set B.

- Note2: It is up to companies to provide other alternative(s)

- Note3: The narrow and wide beam terminology is for SI discussion only and have no specification impact

Conclusion

Regarding the sub use case BM-Case2, further study the following alternatives of measurement results for AI/ML input (for each past measurement instance):

- Alt.1: Only L1-RSRP measurement based on Set B

- Alt 2: L1-RSRP measurement based on Set B and assistance information

  - FFS: Assistance information. The following were mentioned by companies in the discussion:, Tx and/or Rx beam angle, position information, UE direction information, positioning-related measurement (such as Multi-RTT), expected Tx and/or Rx beam/occasion for the prediction (e.g., expected Tx and/or Rx beam angle for the prediction, expected occasions of the prediction), Tx and/or Rx  beam shape information (e.g., Tx and/or Rx beam pattern, Tx and/or Rx beam pointing angles beam boresight directions (azimuth and elevation), 3dB beamwidth, etc.) , increase ratio of L1-RSRP for best N beams, UE orientation information

    - Note: The provision of assistance information may be infeasible due to the concern of disclosing proprietary information to the other side.

- Alt.3: L1-RSRP measurement based on Set B and the corresponding DL Tx and/or Rx beam ID

- Note1: It is up to companies to provide other alternative(s) including the combination of some alternatives

Note2: All the inputs are “nominal” and only for discussion purpose.

AI/ML for positioning accuracy enhancement

Agreement

The IIoT indoor factory (InF) scenario is a prioritized scenario for evaluation of AI/ML based positioning.

Agreement

For evaluation of AI/ML based positioning, at least the InF-DH sub-scenario is prioritized in the InF deployment scenario for FR1 and FR2.

Agreement

For InF-DH channel, the prioritized clutter parameters {density, height, size} are:

- {60%, 6m, 2m};

- {40%, 2m, 2m}.

  - Note: an individual company may treat {40%, 2m, 2m} as optional in their evaluation considering their specific AI/ML design.

Agreement

For evaluation of AI/ML based positioning, reuse the common scenario parameters defined in Table 6-1 of TR 38.857.

Agreement

For evaluation of InF-DH scenario, the parameters are modified from TR 38.857 Table 6.1-1 as shown in the table below.

  - The parameters in the table are applicable to InF-DH at least. If another InF sub-scenario is prioritized in addition to InF-DH, some parameters in the table below may be updated.

Parameters common to InF scenario (Modified from TR 38.857 Table 6.1-1)

|  |  | FR1 Specific Values |  | FR2 Specific Values |
| --- | --- | --- | --- | --- |
| Channel model |  | InF-DH |  | InF-DH |
| Layout | Hall size | InF-DH: (baseline) 120x60 m (optional) 300x150 m |  |  |
|  | BS locations | 18 BSs on a square lattice with spacing D, located D/2 from the walls. - for the small hall (L=120m x W=60m): D=20m - for the big hall (L=300m x W=150m): D=50m |  |  |
|  | Room height | 10m |  |  |
| Total gNB TX power, dBm |  | 24dBm | 24dBm EIRP should not exceed 58 dBm |  |
| gNB antenna configuration |  | (M, N, P, Mg, Ng) = (4, 4, 2, 1, 1), dH=dV=0.5λ – Note 1 Note: Other gNB antenna configurations are not precluded for evaluation | (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1), dH=dV=0.5λ – Note 1 One TXRU per polarization per panel is assumed |  |
| gNB antenna radiation pattern |  | Single sector – Note 1 | 3-sector antenna configuration – Note 1 |  |
| Penetration loss |  | 0dB |  |  |
| Number of floors |  | 1 |  |  |
| UE horizontal drop procedure |  | Uniformly distributed over the horizontal evaluation area for obtaining the CDF values for positioning accuracy, The evaluation area should be selected from - the convex hull of the horizontal BS deployment. - the whole hall area if the CDF values for positioning accuracy is obtained from whole hall area. FFS: which of the above should be baseline. FFS: if an optional evaluation area is needed |  |  |
| UE antenna height |  | Baseline: 1.5m (Optional): uniformly distributed within [0.5, X2]m, where X2 = 2m for scenario 1(InF-SH) and X2= for scenario 2 (InF-DH) FFS: if the optional UE antenna height is needed |  |  |
| UE mobility |  | 3km/h |  |  |
| Min gNB-UE distance (2D), m |  | 0m |  |  |
| gNB antenna height |  | Baseline: 8m (Optional): two fixed heights, either {4, 8} m, or {max(4,), 8}. FFS: if the optional gNB antenna height is needed |  |  |
| Clutter parameters: {density , height ,size } |  | High clutter density: - {40%, 2m, 2m} - {60%, 6m, 2m} - Note: an individual company may treat {40%, 2m, 2m} as optional in their evaluation considering their specific AI/ML design. |  |  |
| Note 1: According to Table A.2.1-7 in TR 38.802 |  |  |  |  |

Agreement

For AI/ML-based positioning evaluation, the baseline performance to compare against is that of existing Rel-16/Rel-17 positioning methods.

  - As a starting point, each participating company report the specific existing positioning method (e.g., DL-TDOA, Multi-RTT) used as comparison.

Agreement

For all scenarios and use cases, the main KPI is the CDF percentiles of horizonal accuracy.

- Companies can optionally report vertical accuracy.

Agreement

The CDF percentiles to analyse are: {50%, 67%, 80%, 90%}.

  - 90% is the baseline. {50%, 67% 80%} are optional.

Agreement

Target positioning requirements for horizonal accuracy and vertical accuracy are not defined for AI/ML-based positioning evaluation.

Agreement

For evaluation of AI/ML based positioning, the KPI include the model complexity and computational complexity.

- FFS: the details of model complexity and computational complexity

Agreement

Synthetic dataset generated according to the statistical channel models in TR38.901 is used for model training, validation, and testing.

Agreement

The dataset is generated by a system level simulator based on 3GPP simulation methodology.

Agreement

As a starting point, the training, validation and testing dataset are from the same large-scale and small-scale propagation parameters setting. Subsequent evaluation can study the performance when the training dataset and testing dataset are from different settings.

Agreement

For AI/ML-based positioning evaluation, RAN1 does not attempt to define any common AI/ML model as a baseline.

Agreement

The entry “UE horizontal drop procedure” in the simulation parameter table for InF is updated to the following.

| UE horizontal drop procedure | Uniformly distributed over the horizontal evaluation area for obtaining the CDF values for positioning accuracy, The evaluation area should be selected from - (baseline) the whole hall area, and the CDF values for positioning accuracy is obtained from whole hall area. - (optional) the convex hull of the horizontal BS deployment, and the CDF values for positioning accuracy is obtained from the convex hull. |
| --- | --- |

Agreement

The entries “UE antenna height” and “gNB antenna height” in the simulation parameter table for InF is updated to the following.

| UE antenna height | Baseline: 1.5m (Optional): uniformly distributed within [0.5, X2]m, where X2 = 2m for scenario 1(InF-SH) and X2= for scenario 2 (InF-DH) |
| --- | --- |
| … | … |
| gNB antenna height | Baseline: 8m (Optional): two fixed heights, either {4, 8} m, or {max(4,), 8}. |

Agreement

If spatial consistency is enabled for the evaluation, companies model at least one of: large scale parameters, small scale parameters and absolute time of arrival, where

- the large scale parameters are according to Section 7.5 of TR 38.901 and correlation distance =  for InF (Section 7.6.3.1 of TR 38.901)

- the small scale parameters are according to Section 7.6.3.1 of TR 38.901

- the absolute time of arrival is according to Section 7.6.9 of TR 38.901

Agreement

If spatial consistency is enabled for the evaluation of AI/ML based positioning, the baseline evaluation does not incorporate spatially consistent UT/BS mobility modelling (Section 7.6.3.2 of TR 38.901).

-         It is optional to implement spatially consistent UT/BS mobility modelling (Section 7.6.3.2 of TR 38.901).

Agreement

For evaluation of AI/ML based positioning, companies are encouraged to evaluate the model generalization.

- FFS: the metrics for evaluating the model generalization (e.g., model performance based on agreed KPIs under different settings)

Agreement

Companies are encouraged to provide evaluation results for:

- Direct AI/ML positioning

  - Companies are encouraged to describe at least the following implementation details for the evaluation

    - details of the channel observation used as the input of the AI/ML model inference (e.g., type and size of model input), model input acquisition and pre-processing

- AI/ML assisted positioning

  - Companies are encouraged to describe at least the following implementation details for the evaluation

    - details of the channel observation used as the input of the AI/ML model inference (e.g., type and size of model input), model input acquisition and pre-processing

    - details of the output of the AI/ML model inference, how the AI/ML model output is used to obtain the UE’s location

Agreement

When reporting evaluation results with direct AI/ML positioning and/or AI/ML assisted positioning, proponent company is expected to describe if a one-sided model or a two-sided model is used.

- If one-sided model (i.e., UE-side model or network-side model), the proponent company report which side the model inference is performed (e.g. UE, network), and any details specific to the side that performs the AI/ML model inference.

- If two-sided model, the proponent company report which side (e.g., UE, network) performs the first part of interference, and which side (e.g., network, UE) performs the remaining part of the inference.

Agreement

For evaluation of AI/ML based positioning, the computational complexity can be reported via the metric of floating point operations (FLOPs).

- Note: For AI/ML assisted methods, computational complexity for the AI/ML model is only one component of the overall complexity for estimating the UE’s location.

- Note: Other metrics to measure the computational complexity are not precluded.

Agreement

For evaluation of AI/ML based positioning, details of the training dataset generation are to be reported by proponent company. The report may include (in addition to other selected settings, if applicable):

- The size of training dataset, for example, the total number of UEs in the evaluation area for generating training dataset;

- The distribution of UE location for generating the training dataset may be one of the following:

  - Option 1: grid distribution, i.e., one training data is collected at the center of one small square grid, where, for example, the width of the square grid can be 0.25/0.5/1.0 m.

  - Option 2: uniform distribution, i.e., the UE location is randomly and uniformly distributed in the evaluation area.

Other aspects on AI/ML for positioning accuracy enhancement

Agreement

Study further on sub use cases and potential specification impact of AI/ML for positioning accuracy enhancement considering various identified collaboration levels.

- Companies are encouraged to identify positioning specific aspects on collaboration levels if any in agenda 9.2.4.2.

- Note1: terminology, notation and common framework of Network-UE collaboration levels are to be discussed in agenda 9.2.1 and expected to be applicable to AI/ML for positioning accuracy enhancement.

- Note2: not every collaboration level may be applicable to an AI/ML approach for a sub use case

Agreement

For further study, at least the following aspects of AI/ML for positioning accuracy enhancement are considered.

- Direct AI/ML positioning: the output of AI/ML model inference is UE location

  - E.g., fingerprinting based on channel observation as the input of AI/ML model

  - FFS the details of channel observation as the input of AI/ML model, e.g. CIR, RSRP and/or other types of channel observation

  - FFS: applicable scenario(s) and AI/ML model generalization aspect(s)

- AI/ML assisted positioning: the output of AI/ML model inference is new measurement and/or enhancement of existing measurement

  - E.g., LOS/NLOS identification, timing and/or angle of measurement, likelihood of measurement

  - FFS the details of input and output for corresponding AI/ML model(s)

  - FFS: applicable scenario(s) and AI/ML model generalization aspect(s)

- Companies are encouraged to clarify all details/aspects of their proposed AI/ML approaches/sub use case(s) of AI/ML for positioning accuracy enhancement

Agreement

Companies are encouraged to study and provide inputs on potential specification impact at least for the following aspects of AI/ML approaches for sub use cases of AI/ML for positioning accuracy enhancement.

- AI/ML model training

  - training data type/size

  - training data source determination (e.g., UE/PRU/TRP)

  - assistance signalling and procedure for training data collection

- AI/ML model indication/configuration

  - assistance signalling and procedure (e.g., for model configuration, model activation/deactivation, model recovery/termination, model selection)

- AI/ML model monitoring and update

  - assistance signalling and procedure (e.g., for model performance monitoring, model update/tuning)

- AI/ML model inference input

  - report/feedback of model input for inference (e.g., UE feedback as input for network side model inference)

  - model input acquisition and pre-processing

  - type/definition of model input

- AI/ML model inference output

  - report/feedback of model inference output

  - post-processing of model inference output

- UE capability for AI/ML model(s) (e.g., for model training, model inference and model monitoring)

- Other aspects are not precluded

- Note: not all aspects may apply to an AI/ML approach in a sub use case

- Note2: the definitions of common AI/ML model terminologies are to be discussed in agenda 9.2.1

#### 2.1.2	Remaining Open issues

- Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98

  - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels

- AI/ML model, terminology and description to identify common and specific characteristics for framework investigations:

  - Characterize the defining stages of AI/ML related algorithms and associated complexity:

  - Model generation, e.g., model training (including input/output, pre-/post-process, online/offline as applicable), model validation, model testing, as applicable

  - Inference operation, e.g., input/output, pre-/post-process, as applicable

  - Identify various levels of collaboration between UE and gNB pertinent to the selected use cases, e.g.,

    - No collaboration: implementation-based only AI/ML algorithms without information exchange [for comparison purposes]

    - Various levels of UE/gNB collaboration targeting at separate or joint ML operation.

  - Characterize lifecycle management of AI/ML model: e.g., model training, model deployment, model inference, model monitoring, model updating

  - Dataset(s) for training, validation, testing, and inference

  - Identify common notation and terminology for AI/ML related functions, procedures and interfaces

- Evaluate performance benefits of AI/ML based algorithms for the agreed use cases in the final representative set

- Assess potential specification impact, specifically for the agreed use cases in the final representative set and for a common framework:

  - PHY layer aspects, e.g., (RAN1)

    - Consider aspects related to, e.g., the potential specification of the AI Model lifecycle management, and dataset construction for training, validation and test for the selected use cases

    - Use case and collaboration level specific specification impact, such as new signalling, means for training and validation data assistance, assistance information, measurement, and feedback

## 2.3	RAN2

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

Work has not started for any of the objectives.

- Assess potential specification impact, specifically for the agreed use cases in the final representative set and for a common framework:

  - Protocol aspects, e.g., (RAN2) - RAN2 only starts the work after there is sufficient progress on the use case study in RAN1

    - Consider aspects related to, e.g., capability indication, configuration and control procedures (training/inference),  and management of data and AI/ML model, per RAN1 input

    - Collaboration level specific specification impact per use case

## 2.3	RAN3

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

## 2.4	RAN4

#### 2.4.1	Agreements

#### 2.4.2	Remaining Open issues

Work has not started for any of the objectives.

- Assess potential specification impact, specifically for the agreed use cases in the final representative set and for a common framework:

  - Interoperability and testability aspects, e.g., (RAN4)

    - Requirements and testing frameworks to validate AI/ML based performance enhancements and ensuring that UE and gNB with AI/ML meet or exceed the existing minimum requirements if applicable

    - Consider the need and implications for AI/ML processing capabilities definition

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN1#109-e contributions:

- R1-2205021	Work plan for Rel-18 SI on AI and ML for NR air interface	Qualcomm Incorporated

- R1-2205022	TR skeleton for Rel-18 SI on AI and ML for NR air interface	Qualcomm Incorporated

- R1-2205476	TR skeleton for Rel-18 SI on AI and ML for NR air interface	Qualcomm Incorporated

- R1-2205522	Summary#4 of General Aspects of AI/ML Framework	Moderator (Qualcomm)

- R1-2205474	Summary#3 of General Aspects of AI/ML Framework	Moderator (Qualcomm)

- R1-2205401	Summary#2 of General Aspects of AI/ML Framework	Moderator (Qualcomm)

- R1-2205285	Summary#1 of General Aspects of AI/ML Framework	Moderator (Qualcomm)

- R1-2203067	Discussion on common AI/ML characteristics and operations	FUTUREWEI

- R1-2203139	Discussion on general aspects of AI/ML framework	Huawei, HiSilicon

- R1-2203247	Discussion on common AI/ML framework	ZTE

- R1-2203280	General aspects of AI PHY framework	Ericsson

- R1-2203404	Discussions on AI-ML framework	New H3C Technologies Co., Ltd.

- R1-2203450	Discussion on AI/ML framework for air interface	CATT

- R1-2203549	General discussions on AI/ML framework	vivo

- R1-2203656	Discussion on general aspects of AI/ML for NR air interface	China Telecom

- R1-2203690	Discussion on general aspects of AI ML framework	NEC

- R1-2203728	Consideration on common AI/ML framework	Sony

- R1-2203807	Initial views on the general aspects of AI/ML framework	xiaomi

- R1-2203896	General aspects of AI ML framework and evaluation methodogy	Samsung

- R1-2204014	On general aspects of AI/ML framework	OPPO

- R1-2204062	Evaluating general aspects of AI-ML framework	Charter Communications, Inc

- R1-2204077	General aspects of AI/ML framework	Panasonic

- R1-2204120	Considerations on AI/ML framework	SHARP Corporation

- R1-2204148	General aspects on AI/ML framework	LG Electronics

- R1-2204179	Views on general aspects on AI-ML framework	CAICT

- R1-2204237	Discussion on general aspect of AI/ML framework	Apple

- R1-2204294	Discussion on general aspects of AI/ML framework	CMCC

- R1-2204374	Discussion on general aspects of AI/ML framework	NTT DOCOMO, INC.

- R1-2204416	General aspects of AI/ML framework	Lenovo

- R1-2204498

- Discussion on general aspects of AIML framework	Spreadtrum Communications

- R1-2204570	ML terminology, descriptions, and collaboration framework	Nokia, Nokia Shanghai Bell

- R1-2204650	Discussion on AI/ML framework for NR air interface	ETRI

- R1-2204792	Discussion of AI/ML framework	Intel Corporation

- R1-2204839	On general aspects of AI and ML framework for NR air interface	NVIDIA

- R1-2204859	General aspects of AI/ML framework for NR air interface	AT&T

- R1-2204936	General aspects of AI/ML framework	Mavenir

- R1-2205023	General aspects of AIML framework	Qualcomm Incorporated

- R1-2205065	AI/ML Model Life cycle management	Rakuten Moible

- R1-2205075	Discussions on general aspects of AI/ML framework	Fujitsu Limited

- R1-2205099	Overview to support artificial intelligence over air interface	MediaTek Inc.

- R1-2203650	Evaluation on AI-based CSI feedback	SEU

- R1-2204041	Considerations on AI-enabled CSI overhead reduction	CENC

- R1-2204606	Discussion on the AI/ML methods for CSI feedback enhancements      	Fraunhofer IIS, Fraunhofer HHI

- R1-2205492	Summary#5 of [109-e-R18-AI/ML-03]	Moderator (Huawei)

- R1-2205491	Summary#4 of [109-e-R18-AI/ML-03]	Moderator (Huawei)

- R1-2205224	Summary#3 of [109-e-R18-AI/ML-03]	Moderator (Huawei)

- R1-2205223	Summary#2 of [109-e-R18-AI/ML-03]	Moderator (Huawei)

- R1-2203068	Discussion on evaluation of AI/ML for CSI feedback enhancement use case	FUTUREWEI

- R1-2203140	Evaluation on AI/ML for CSI feedback enhancement	Huawei, HiSilicon

- R1-2203248	Evaluation assumptions on AI/ML for CSI feedback	ZTE

- R1-2203281	Evaluations on AI-CSI	Ericsson

- R1-2203451	Discussion on evaluation on AI/ML for CSI feedback	CATT

- R1-2203550	Evaluation on AI/ML for CSI feedback enhancement	vivo

- R1-2203808	Discussion on evaluation on AI/ML for CSI feedback enhancement	xiaomi

- R1-2203897	Evaluation on AI ML for CSI feedback enhancement	Samsung

- R1-2204015	Evaluation methodology and preliminary results on AI/ML for CSI feedback enhancement			OPPO

- R1-2204050	Evaluation on AI/ML for CSI feedback enhancement	InterDigital, Inc.

- R1-2204055	Evaluation of CSI compression with AI/ML	Beijing Jiaotong University

- R1-2204063	Performance evaluation of ML techniques for CSI feedback enhancement	Charter Communications, Inc

- R1-2204149	Evaluation on AI/ML for CSI feedback enhancement	LG Electronics

- R1-2204180	Some discussions on evaluation on AI-ML for CSI feedback	CAICT

- R1-2204238	Initial evaluation on AI/ML for CSI feedback	Apple

- R1-2204295	Discussion on evaluation on AI/ML for CSI feedback enhancement	CMCC

- R1-2204375	Discussion on evaluation on AI/ML for CSI feedback enhancement	NTT DOCOMO, INC.

- R1-2204417	Evaluation on AI/ML for CSI feedback	Lenovo

- R1-2204499	Discussion on evaluation on AI/ML for CSI feedback enhancement	Spreadtrum Communications, BUPT

- R1-2204571	Evaluation on ML for CSI feedback enhancement	Nokia, Nokia Shanghai Bell

- R1-2204793	Evaluation for CSI feedback enhancements	Intel Corporation

- R1-2204840	On evaluation assumptions of AI and ML for CSI feedback enhancement	NVIDIA

- R1-2204860	Evaluation of AI/ML for CSI feedback enhancements		AT&T

- R1-2205024	Evaluation on AIML for CSI feedback enhancement		Qualcomm Incorporated

- R1-2205076	Evaluation on AI/ML for CSI feedback enhancement		Fujitsu Limited

- R1-2205100	Evaluation on AI/ML for CSI feedback enhancement		MediaTek Inc.

- R1-2205556	Summary #2 on other aspects of AI/ML for CSI enhancement		Moderator (Apple)

- R1-2203069	Discussion on sub use cases of AI/ML for CSI feedback enhancement use case	FUTUREWEI

- R1-2203141	Discussion on AI/ML for CSI feedback enhancement	Huawei, HiSilicon

- R1-2203249	Discussion on potential enhancements for AI/ML based CSI feedback	ZTE

- R1-2203282	Discussions on AI-CSI	Ericsson

- R1-2203452	Discussion on other aspects on AI/ML for CSI feedback	CATT

- R1-2203551	Other aspects on AI/ML for CSI feedback enhancement	vivo

- R1-2203614	Discussion on AI/ML for CSI feedback enhancement	GDCNI

- Late submission

- R1-2203729	Considerations on CSI measurement enhancements via AI/ML	Sony

- R1-2203809	Discussion on AI for CSI feedback enhancement	xiaomi

- R1-2203898	Representative sub use cases for CSI feedback enhancement	Samsung

- R1-2203939	Discussion on AI/ML for CSI feedback enhancement	NEC

- R1-2204016	On sub use cases and other aspects of AI/ML for CSI feedback enhancement	OPPO

- R1-2204051	Discussion on AI/ML for CSI feedback enhancement	InterDigital, Inc.

- R1-2204057	CSI compression with AI/ML	Beijing Jiaotong University

- R1-2204150	Other aspects on AI/ML for CSI feedback enhancement	LG Electronics

- R1-2204181	Discussions on AI-ML for CSI feedback	CAICT

- R1-2204239	Discussion on other aspects on AI/ML for CSI feedback	Apple

- R1-2204296	Discussion on other aspects on AI/ML for CSI feedback enhancement	CMCC

- R1-2204376	Discussion on other aspects on AI/ML for CSI feedback enhancement	NTT DOCOMO, INC.

- R1-2204418	Further aspects of AI/ML for CSI feedback	Lenovo

- R1-2204500	Discussion on other aspects on AI/ML for CSI feedback	Spreadtrum Communications

- R1-2204568	Discussions on Sub-Use Cases in AI/ML for CSI Feedback Enhancement	TCL Communication

- R1-2204572	Other aspects on ML for CSI feedback enhancement	Nokia, Nokia Shanghai Bell

- R1-2204659	Discussion on AI/ML for CSI feedback enhancement	Panasonic

- R1-2204794	Use-cases and specification for CSI feedback	Intel Corporation

- R1-2204841	On other aspects of AI and ML for CSI feedback enhancement	NVIDIA

- R1-2204861	CSI feedback enhancements for AI/ML based MU-MIMO scheduling and parameter configuration			AT&T

- R1-2204937	AI/ML for CSI feedback enhancement	Mavenir

- R1-2205025	Other aspects on AIML for CSI feedback enhancement	Qualcomm Incorporated

- R1-2205077 	Views on sub-use case selection and STD impacts on AI/ML for CSI feedback enhancement     Fujitsu Limited

- R1-2205101 	On the challenges of collecting field data for training and testing of AI/ML for CSI feedback enhancement	MediaTek Inc.

- R1-2205641	Feature lead summary #4 evaluation of AI/ML for beam management 	Moderator (Samsung)

- R1-2205271	Feature lead summary #3 evaluation of AI/ML for beam management 	Moderator (Samsung)

- R1-2205269	Feature lead summary #1 evaluation of AI/ML for beam management	Moderator (Samsung)

- R1-2203142	Evaluation on AI/ML for beam management	Huawei, HiSilicon

- R1-2203250	Evaluation assumptions on AI/ML for beam management	ZTE

- R1-2203255	Model and data-driven beam predictions in high-speed railway scenarios	PML

- R1-2203283	Evaluations on AI-BM	Ericsson

- R1-2203374	Discussion for evaluation on AI/ML for beam management	InterDigital, Inc.

- R1-2203453	Discussion on evaluation on AI/ML for beam management	CATT

- R1-2203552	Evaluation on AI/ML for beam management	vivo

- R1-2203810	Evaluation on AI/ML for beam management	xiaomi

- R1-2203899	Evaluation on AI ML for Beam management	Samsung

- R1-2204017	Evaluation methodology and preliminary results on AI/ML for beam management	OPPO

- R1-2204059	Evaluation methodology of beam management with AI/ML	Beijing Jiaotong University

- R1-2204102	Discussion on evaluation of AI/ML for beam management use case	FUTUREWEI

- R1-2204151	Evaluation on AI/ML for beam management	LG Electronics

- R1-2204182	Some discussions on evaluation on AI-ML for Beam management	CAICT

- R1-2204240	Evalulation on AI based Beam Management	Apple

- R1-2204297	Discussion on evaluation on AI/ML for beam management	CMCC

- R1-2204377	Discussion on evaluation on AI/ML for beam management	NTT DOCOMO, INC.

- R1-2204419	Evaluation on AI/ML for beam management	Lenovo

- R1-2204573	Evaluation on ML for beam management	Nokia, Nokia Shanghai Bell

- R1-2204795	Evaluation for beam management	Intel Corporation

- R1-2204842	On evaluation assumptions of AI and ML for beam management	NVIDIA

- R1-2204862	Evaluation methodology aspects on AI/ML for beam management	AT&T

- R1-2205026	Evaluation on AIML for beam management	Qualcomm Incorporated

- R1-2205078 	Evaluation on AI/ML for beam management	Fujitsu Limited

- R1-2205102 	AI-assisted Target Cell Prediction for Inter-cell Beam Management	MediaTek Inc.

- R1-2205253	Discussion summary#2 for other aspects on AI/ML for beam management	Moderator (OPPO)

- R1-2203143	Discussion on AI/ML for beam management	Huawei, HiSilicon

- R1-2203251	Discussion on potential enhancements for AI/ML based beam management	ZTE

- R1-2203284	Discussions on AI-BM	Ericsson

- R1-2203375	Discussion for other aspects on AI/ML for beam management	InterDigital, Inc.

- R1-2203454	Discussion on other aspects on AI/ML for beam management	CATT

- R1-2203553	Other aspects on AI/ML for beam management	vivo

- R1-2203691	Discussion on other aspects on AI/ML for beam management	NEC

- R1-2203730	Consideration on AI/ML for beam management	Sony

- R1-2203811	Other aspects on AI/ML for beam management	xiaomi

- R1-2203900	Representative sub use cases for beam management	Samsung

- R1-2204018	Other aspects of AI/ML for beam management	OPPO

- R1-2204060	Beam management with AI/ML	Beijing Jiaotong University

- R1-2204078	Discussion on sub use cases of beam management	Panasonic

- R1-2204103	Discussion on sub use cases of AI/ML for beam management use case	FUTUREWEI

- R1-2204152	Other aspects on AI/ML for beam management	LG Electronics

- R1-2204183	Discussions on AI-ML for Beam management	CAICT

- R1-2204241	Enhancement on AI based Beam Management	Apple

- R1-2204298	Discussion on other aspects on AI/ML for beam management	CMCC

- R1-2204378	Discussion on other aspects on AI/ML for beam management	NTT DOCOMO, INC.

- R1-2204420	Further aspects of AI/ML for beam management	Lenovo

- R1-2204501	Discussion on other aspects on AI/ML for beam management	Spreadtrum Communications

- R1-2204569	Discussions on Sub-Use Cases in AI/ML for Beam Management	TCL Communication

- R1-2204574	Other aspects on ML for beam management	Nokia, Nokia Shanghai Bell

- R1-2204796	Use-cases and specification for beam management	Intel Corporation

- R1-2204843	On other aspects of AI and ML for beam management	NVIDIA

- R1-2204863	System performance aspects on AI/ML for beam management	AT&T

- R1-2204938	AI/ML for beam management	Mavenir

- R1-2205027	Other aspects on AIML for beam management	Qualcomm Incorporated

- R1-2205079	Sub-use cases and spec impact on AI/ML for beam management	Fujitsu Limited

- R1-2205094	Discussion on Codebook Enhancement with AI/ML	Charter Communications, Inc

- R1-2205633	Summary #4 of [109-e-R18-AI/ML-07] Email discussion on evaluation of AI/ML for positioning accuracy enhancement	Moderator (Ericsson)

- R1-2205219	Summary #3 of [109-e-R18-AI/ML-07] Email discussion on evaluation of AI/ML for positioning accuracy enhancement	Moderator (Ericsson)

- R1-2203144	Evaluation on AI/ML for positioning accuracy enhancement	Huawei, HiSilicon

- R1-2203252	Evaluation assumptions on AI/ML for positioning	ZTE

- R1-2203285	Evaluations on AI-Pos	Ericsson

- R1-2203455	Discussion on evaluation on AI/ML for positioning	CATT

- R1-2203554	Evaluation on AI/ML for positioning accuracy enhancement	vivo

- R1-2203812	Initial views on the evaluation on AI/ML for positioning accuracy enhancement	xiaomi

- R1-2203901	Evaluation on AI ML for Positioning	Samsung

- R1-2204019	Evaluation methodology and preliminary results on AI/ML for positioning accuracy enhancement			OPPO

- R1-2204104	Discussion on evaluation of AI/ML for positioning accuracy enhancements use case				FUTUREWEI

- R1-2204153	Evaluation on AI/ML for positioning accuracy enhancement	LG Electronics

- R1-2204159	Evaluation assumptions and results for AI/ML based positioning	InterDigital, Inc.

- R1-2204184	Some discussions on evaluation on AI-ML for positioning accuracy enhancement	CAICT

- R1-2204242	Evaluation on AI/ML for positioning accuracy enhancement	Apple

- R1-2204299	Discussion on evaluation on AI/ML for positioning accuracy enhancement	CMCC

- R1-2204421	Discussion on AI/ML Positioning Evaluations	Lenovo

- R1-2204575	Evaluation on ML for positioning accuracy enhancement	Nokia, Nokia Shanghai Bell

- R1-2204797	Evaluation for positioning accuracy enhancements	Intel Corporation

- R1-2204837	Evaluation on AI/ML for positioning accuracy enhancement	Fraunhofer IIS, Fraunhofer HHI

- R1-2204844	On evaluation assumptions of AI and ML for positioning enhancement	NVIDIA

- R1-2205028	Evaluation on AIML for positioning accuracy enhancement	Qualcomm Incorporated

- R1-2205066	Initial view on AI/ML application to positioning use cases	Rakuten Moible

- R1-2205080 	Discussion on Evaluation related issues for AI/ML for positioning accuracy enhancement	Fujitsu Limited

- R1-2205229	Discussion summary #1 of [109-e-R18-AI/ML-08]	Moderator (vivo)

- R1-2203145	Discussion on AI/ML for positioning accuracy enhancement	Huawei, HiSilicon

- R1-2203253	Discussion on potential enhancements for AI/ML based positioning	ZTE

- R1-2203286	Discussions on AI-Pos	Ericsson

- R1-2203456	Discussion on other aspects on AI/ML for positioning	CATT

- R1-2203555	Other aspects on AI/ML for positioning accuracy enhancement	vivo

- R1-2203692	Discussion on other aspects on AI/ML for positioning accuracy enhancement	NEC

- R1-2203731	Considerations on AI/ML for positioning accuracy enhancement	Sony

- R1-2203813	Initial views on the other aspects of AI/ML-based positioning accuracy enhancement	xiaomi

- R1-2203902	Representative sub use cases for Positioning	Samsung

- R1-2204020	On sub use cases and other aspects of AI/ML for positioning accuracy enhancement	OPPO

- R1-2204105	Discussion on sub use cases of AI/ML for positioning accuracy enhancements use case				FUTUREWEI

- R1-2204154	Other aspects on AI/ML for positioning accuracy enhancement	LG Electronics

- R1-2204160	Potential specification impacts for AI/ML based positioning	InterDigital, Inc.

- R1-2204185	Discussions on AI-ML for positioning accuracy enhancement	CAICT

- R1-2204243	Discussion on other aspects on AI/ML for positioning accuracy enhancement	Apple

- R1-2204300	Discussion on other aspects on AI/ML for positioning accuracy enhancement	CMCC

- R1-2204422	AI/ML Positioning use cases and Associated Impacts	Lenovo

- R1-2204576	Other aspects on ML for positioning accuracy enhancement	Nokia, Nokia Shanghai Bell

- R1-2204798	Use-cases and specification for positioning	Intel Corporation

- R1-2204838	On potential specification impact of AI/ML for positioning	Fraunhofer IIS, Fraunhofer HHI

- R1-2204845	On other aspects of AI and ML for positioning enhancement	NVIDIA

- R1-2205029	Other aspects on AIML for positioning accuracy enhancement	Qualcomm Incorporated

- R1-2205081 	Sub-use cases and spec impacts for AI/ML for positioning accuracy enhancement	Fujitsu Limited

- R1-2203254	Discussion on other use cases for AI/ML	ZTE

- R1-2203405	Discussions on AI-ML chanllenges and limiations	New H3C Technologies Co., Ltd.

- R1-2203457	Views on UE capability of AI/ML for air interface	CATT

- R1-2203556	Discussions on AI/ML for DMRS	vivo

- R1-2203670	Draft skeleton of TR 38.843	Ericsson

- R1-2204577	On ML capability exchange, interoperability, and testability aspects	Nokia, Nokia Shanghai Bell

- R1-2204846	GPU hosted 5G virtual RAN baseband processing and AI applications	NVIDIA

- R1-2204911	Discussion on other potential use cases of AI/ML for NR air interface	Huawei, HiSilicon

- R1-2205067	Consideration on UE processing capability for AI/ML utilization	Rakuten Moible

10.01.2022		minor adaptations for RAN #95e

04.10.2021		minor adaptations for RAN #94e

08.08.2021		minor adaptations for RAN #93e

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66