3GPP TSG RAN Meeting #96	RP-221351

Budapest, Hungary, June 6-9, 2022

Status Report to TSG

Agenda item:			9.2.6

| WI / SI Name | Study on evolution of NR duplex operation |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: Yes | Core part: No | Performance part: No |  | Testing part: No |
| Acronym | FS_NR_duplex_evo |  |  |  |  |
| Unique ID | 940082 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-220633 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: 12/2023 | Core part: N/A | Performance part: N/A | Testing part: N/A |  |
| Overall Completion level | Study Item: 10% | Core part: N/A | Performance Part: N/A | Testing part: N/A |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | TSG RAN WG1 |
| --- | --- | --- |
| Rapporteur | Name | Fei WANG |
|  | Company | CMCC |
|  | Email | wangfei@chinamobile.com |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

2.1	RAN1

2.1.1	Agreements

TR

Agreement

TR skeleton for TR 38.858 skeleton for study on evolution of NR duplex operation is endorsed in R1-2205399.

Evaluation on NR duplex evolution

Agreement

For discussion purpose for evaluation, define the following deployment cases for SBFD:

- Deployment Case 1 (Non-coexistence case with single SBFD subband configuration): One single operator using one single carrier is considered. All the cells belonging to the operator use SBFD operation with the same SBFD subband configuration.

- Deployment Case 2 (Non-coexistence case with multiple SBFD subband configurations): One single operator using one single carrier is considered. All the cells belonging to the operator use SBFD operation, but different cells may use different SBFD subband configurations.

- Deployment Case 3 (Co-channel co-existence case): One single operator using one single carrier is considered. Among the cells belonging to the operator, some of them use legacy TDD operation (static TDD operation) while the others use SBFD operation with the same SBFD subband configuration.

  - Deployment Case 3-1: Only 1-layer is considered

  - Deployment Case 3-2: 2-layer is considered

- Deployment Case 4 (Adjacent-channel co-existence case): Two operators each using one carrier are considered and the two carriers are adjacent carriers. One operator uses legacy TDD operation (static TDD operation) while the other operator uses SBFD operation with the same SBFD subband configuration.

Note: This definition has no intention to preclude any potential solutions for SBFD in AI9.3.2

Note: SBFD subband configuration is from gNB perspective.

Agreement

For SBFD Deployment Case 1, at least consider the following scenarios for evaluation:

- For FR1,

  - Indoor office (use Indoor office defined in TR38.802/TR38.901 as starting point)

  - Urban macro (use Urban macro defined in TR38.802/TR38.901 as starting point)

    - FFS: UE outdoor/indoor proportion, clustering, etc

  - Optional: Dense Urban with 1-layer or 2-layer (use Dense Urban defined in TR38.802/TR38.901 as starting point)

  - FFS: Rural

- For FR2-1,

  - Indoor office (use Indoor office defined in TR38.802/TR38.901 as starting point)

  - Dense Urban Macro layer (use Dense Urban defined in TR38.802 as starting point)

    - FFS: UE outdoor/indoor proportion, clustering, etc

  - Optional: Dense Urban micro (use Dense Urban micro defined in TR38.802/TR38.901 as starting point)

  - FFS: Whether FR2-2 is considered or not in Rel-18.

Note: For optional scenarios, they can be captured in TR and it is up to each company to provide the results. The results can be used to draw conclusion/recommendation depending on the number of companies providing the results.

Agreement

Regarding gNB self-interference modelling for system level simulation purpose, consider introducing ratio of self-interference (RSI) to represent the overall self-interference suppression capability of gNB by means of spatial isolation, subband frequency isolation, digital interference cancellation and beamform nulling/isolation, etc. RSI also takes into account the impact of Tx/Rx antenna element gain on self-interference. The RSI, denoted as αSIm,n,  can be defined as the ratio of the total power transmitted by gNB across all transmit chains on a frequency unit m (e.g., subband/RB/subcarrier m) in a SBFD carrier to the residual self-interference received by the same gNB on a single receiver chain on a different frequency unit n (e.g., another subband/RB/subcarrier n) in the same SBFD carrier.

- FFS: Model for link level simulations and relevant questions to ask RAN4

- FFS: details of gNB self-interference modelling using RSI in SLS. As one example based on per-RB-RSI, the gNB self-interference on a single receiver chain at UL RB n can be modelled as

  - ISIn=m≠nm∈DL subbandISIm,n, wherein,

  - dBmISIm,n=Ptxm-dBαSIm,n

  - ISIm,nis the gNB self-interference on a single receiver chain at UL RB n caused by DL transmission on DL RB m.

  - m is the DL RB index in DL subbands.

  - Ptxm is gNB’s DL transmission power across all transmit chains at RB m (in dBm).

  - αSIm,n is the per-RB-RSI.

  - FFS: consider a statistical clutter model based on statistics of clutter strength and AoA.

- The following should be asked to RAN4:

  - What is the value range of RSI αSI(n,m) for each frequency range, and under what assumptions on the self-interference suppression means the value range of RSI is provided?

    - RAN1 understands the RSI can be described per subband, per RB, or per subcarrier depending on the granularity of the frequency unit, and it is up to RAN4 to provide the RSI in which granularity.

  - Whether it is possible for RAN4 to provide RAN1 the respective capabilities of different self-interference suppression means? e.g., is it possible to provide the separate estimates for spatial isolation, subband frequency isolation, beamform nulling/isolation, and digital cancellation, etc., as below?

    - dB(αSI(m,n))= dB(αSI-spatial)+dB(αSI-frequency (m,n))+dB(αSI-beam)+dB(αSI-digtial) +…

      - αSI-spatial denotes the spatial isolation.

      - αSI-frequency (m,n) denotes the suband frequency isolation between the Tx frequency unit m and the Rx frequency unit n.

      - αSI-beam denotes the beamform nulling or beam isolation.

      - αSI-digtial denotes the digital cancellation capability.

  - Whether it is possible to simplify the RSI as frequency flat model, and under which condition(s) the dependency of the RSI on frequency can be ignored?

  - The feasibility of provided value range of RSI regarding factors such as blocking, AGC, etc.

  - Does RSI have any dependency with the following factors or any other factors? What are the dependencies?

    - gNB’s antenna aspects, e.g., the assumed antenna architecture, the number of transmit chains and receive chains, etc.

    - Frequency aspects, e.g., the frequency distance between the Tx frequency unit m and the Rx frequency unit n, the number of RBs allocated for DL transmission, etc.

    - Beam aspects, e.g., Tx/Rx beam-pair for FR1/FR2 especially for clutter echo, etc.

- Note: RAN1’s consideration on the frequency locations and sizes of SBFD DL subband and SBFD UL subband assumed in SBFD operation can be provided to RAN4.

Agreement

For discussion of gNB-gNB and UE-UE co-channel inter-subband CLI modelling in system level simulation, RAN1 understands at least the following two aspects need to be considered:

- Aspect 1: The unwanted emissions due to Tx non-linearity at the transmitter of the aggressor from the allocated RBs to the non-allocated RBs in the same carrier.

- Aspect 2: The receiver selectivity at the victim to receive the desired signal in the allocated RBs in the presence of the unwanted signals at the non-allocated RBs. (e.g. receiver blocking at the victim, overload of the receiver dynamic range, etc)

The following questions should be asked to RAN4:

- Whether it is feasible to consider the above two aspects for gNB-gNB and UE-UE co-channel inter-subband CLI modelling in system level simulation? Are there any other aspects should also be taken into account?

- For a specific pair of DL frequency unit m (e.g., subband/RB m) and UL frequency unit n (e.g., subband/RB n) of gNB-gNB link, where the DL frequency unit m and UL frequency unit n are in the same carrier and non-overlapping in frequency, and assuming the aggressor gNB transmits on the DL frequency unit m and the victim gNB receives on the UL frequency unit n,

  - How to model the interference from DL frequency unit m to UL frequency unit n due to Aspect 1 (defined above) at the gNB transmitter?

  - How to model the interference from DL frequency unit m to UL frequency unit n due to Aspect 2 (defined above) at the gNB receiver?

  - How to model the above interferences for the following two cases:

    - inter-site gNB-gNB co-channel inter-subband CLI

    - co-site inter-sector co-channel inter-subband CLI

- For a specific pair of DL frequency unit m (e.g., subband/RB m) and UL frequency unit n (e.g., subband/RB n) of UE-UE link, where the DL frequency unit m and UL frequency unit n are in the same carrier and non-overlapping in frequency, and assuming the aggressor UE transmits on the UL frequency unit n and the victim UE receives on the DL frequency unit m,

  - How to model the interference from UL frequency unit n to DL frequency unit m due to Aspect 1 (defined above) at the UE transmitter?

  - How to model the interference from UL frequency unit n to DL frequency unit m due to Aspect 2 at the UE receiver?

FFS: Usage of the above model provided by RAN4 in the evaluation

Agreement

At least the following metrics are considered for SBFD and dynamic/flexible TDD evaluation.

- DL/UL UPT or user throughput (CDF or {mean, 5%, 50%, 95%}) using SLS

- Latency (CDF or {mean, 5%, 50%, 95%}) using SLS

- Resource utilization using SLS

- DL/UL received SINR using SLS

- Coverage metric

  - FFS: MPL to achieve a certain bit rate in UL and DL

- FFS: definitions of the above metrics

- FFS: other metrics

Agreement

Regarding traffic model for SBFD and dynamic/flexible TDD evaluation, at least FTP3 is considered. Performance evaluation comparison between different duplex modes (e.g., legacy static TDD vs. SBFD) should be performed based on the same amount of input traffic.

- FFS: other traffic models, e.g., XR, VoIP

- FFS: Packet size, traffic load, ratio of DL/UL traffic

- FFS: additionally consider different amount of input traffic at least for adjacent-channel coexistence studies

Agreement

For discussion for duplex evolution study (all agenda items), consider the following as RAN1’s common understanding:

- Co-channel interference: The interference is from the aggressor to the victim in the same carrier.

  - Co-channel intra-subband interference: The interference is caused by transmission of the aggressor on a set of contiguous RBs in a carrier to reception of the victim on the same set of contiguous RBs in the same carrier.

  - Co-channel inter-subband interference: The interference is caused by transmission of the aggressor in a first set of contiguous RBs in a carrier to reception of the victim in a second set of contiguous RBs in the same carrier, where the two contiguous RB sets are non-overlapping in frequency.

- Adjacent channel interference: The interference is from the aggressor in carrier#1 to the victim in carrier#2, where the carrier#1 and carrier#2 are adjacent carriers.

Note 1: ‘Co-channel’ here means ‘co-carrier’. ‘Adjacent-channel’ here means ‘adjacent-carrier’.

Agreement

For discussion for duplex evolution study (all agenda items), consider the following as the common understanding in RAN1 on the definition of interference types for SBFD operation:

- gNB self-interference (SI): Interference caused by DL transmission on a set of DL RBs in a carrier to UL reception on a set of UL RBs in the same carrier at the gNB side, where the two RB sets are non-overlapping in frequency.

- gNB-UE co-channel intra-subband interference: This is the same as the legacy DL interference type in legacy TDD network with static TDD UL/DL configuration.

- UE-gNB co-channel intra-subband interference: This is the same as the legacy UL interference type in legacy TDD network with static TDD UL/DL configuration.

- (inter-cell) inter-site gNB-gNB co-channel intra-subband CLI: CLI caused by DL transmission of the aggressor gNB on a set of RBs in one carrier to UL reception of the victim gNB in a different site on the same set of RBs in the same carrier.

- (inter-cell) co-site inter-sector co-channel intra-subband CLI: CLI caused by DL transmission of the aggressor gNB on a set of RBs in one carrier to UL reception of the victim gNB in another sector of the same site on the same set of RBs in the same carrier.

- (inter-cell) UE-UE co-channel intra-subband CLI: CLI caused by UL transmission of the aggressor UE on a set of RBs in one carrier to DL reception of the victim UE on the same set of RBs in the same carrier.

- (inter-cell) inter-site gNB-gNB co-channel inter-subband CLI: CLI caused by DL transmission of the aggressor gNB on a first set of RBs in a carrier to UL reception of the victim gNB in a different site on a second set of RBs in the same carrier, where the two RB sets are non-overlapping in frequency.

- (inter-cell) co-site inter-sector co-channel inter-subband CLI: CLI caused by DL transmission of the aggressor gNB on a first set of RBs in a carrier to UL reception of the victim gNB in another sector of the same site on a second set of RBs in the same carrier, where the two RB sets are non-overlapping in frequency.

- (intra-cell/inter-cell) UE-UE co-channel inter-subband CLI: CLI caused by UL transmission of the aggressor UE on a first set of RBs in a carrier to DL reception of the victim UE on a second set of RBs in the same cell or neighboring cell in the same carrier, where the two RB sets are non-overlapping in frequency.

- gNB-gNB adjacent-channel CLI: CLI caused by DL transmission of the aggressor gNB in a carrier to UL reception of the victim gNB in another adjacent carrier.

  - This includes adjacent-channel CLI between gNBs in the same and different sectors of the same site, i.e., co-site intra and inter-sector adjacent-channel CLI.

- UE-UE adjacent-channel CLI: CLI caused by UL transmission of the aggressor UE in a carrier to DL reception of the victim UE in another adjacent carrier.

Note: Some of the interferences may not be used according to the deployment scenarios, e.g, whether the SBFD subband configurations are the same or different across gNBs.

Note: This does not imply we need to consider all the above interference types in evaluation for SBFD.

Agreement

Regarding gNB-gNB and UE-UE adjacent-channel CLI modelling for system level simulation, RAN1 understands at least the following aspects need to be considered:

- Aspect 1: The unwanted emissions due to Tx non-linearity at the transmitter of the aggressor from the allocated RBs in one carrier to the non-allocated RBs in the adjacent carrier.

- Aspect 2: The receiver selectivity at the victim to receive the desired signal in the allocated RBs in one carrier in the presence of the unwanted signals at the non-allocated RBs in the adjacent carrier. (e.g. receiver blocking at the victim, overload of the receiver dynamic range, etc)

The following questions should be asked to RAN4:

- Whether it is feasible to consider the above two aspects for gNB-gNB and UE-UE adjacent-channel CLI modelling in system level simulation? Are there any other aspects should also be taken into account?

- For a specific pair of DL frequency unit m (e.g., subband/RB m) and UL frequency unit n (e.g., subband/RB n) of gNB-gNB link, where the DL frequency unit m and UL frequency unit n are in adjacent carriers and non-overlapping in frequency, and assuming the aggressor gNB transmits on the DL frequency unit m and the victim gNB receives on the UL frequency unit n,

  - How to model the interference from DL frequency unit m to UL frequency unit n due to Aspect 1 (defined above) at the gNB transmitter?

  - How to model the interference from DL frequency unit m to UL frequency unit n due to Aspect 2 (defined above) at the gNB receiver?

  - How to model the above interferences for the following cases:

    - the two gNBs are from the same sector of the same site in adjacent carriers, i.e., co-site co-sector gNB-gNB adjacent-channel CLI

    - the two gNBs are from different sectors of the same site in adjacent carriers, i.e., co-site inter-sector gNB-gNB adjacent-channel CLI

    - the two gNBs are from different sites in adjacent carriers, i.e., inter-site gNB-gNB adjacent-channel CLI

  - Whether it is feasible to define a similar interference ratio as BS-BS ACIR in TR38.828 but in the subband of the adjacent carrier, with finer granularity (e.g., per subband or per RB), to represent the overall effect of the Aspect 1 and Aspect 2 described above?

    - For example, whether it is feasible to define gNB-gNB-adjacent-channel-per-RB/subband interference ratio as the ratio of the power transmitted by the aggressor gNB on DL frequency unit m to the interference received by the victim gNB on UL frequency unit n? If it is feasible, then what is the value range of the gNB-gNB-adjacent-channel-per-RB/subband interference ratio for each frequency range?

- For a specific pair of DL frequency unit m (e.g., subband/RB m) and UL frequency unit n (e.g., subband/RB n) of UE-UE link, where the DL frequency unit m and UL frequency unit n are in adjacent carriers and non-overlapping in frequency, and assuming the aggressor UE transmits on the UL frequency unit n and the victim UE receives on the DL frequency unit m,

  - How to model the interference from UL frequency unit n to DL frequency unit m due to Aspect 1 (defined above) at the UE transmitter?

  - How to model the interference from UL frequency unit n to DL frequency unit m due to Aspect 2 at the UE receiver?

  - Whether it is feasible to define a similar interference ratio as UE-UE ACIR in TR38.828 but in the subband of the adjacent carrier, with finer granularity (e.g., per subband or per RB), to represent the overall effect of the Aspect 1 and Aspect 2 described above?

    - For example, whether it is feasible to define UE-UE-adjacent-channel-per-RB/subband interference ratio as the ratio of the power transmitted by the aggressor UE on UL frequency unit n to the interference received by the victim UE on DL frequency unit m? If it is feasible, then what is the value range of the UE-UE-adjacent-channel-per-RB/subband interference ratio for each frequency range?

FFS: How to make use of the interference model in RAN1

Agreement

For SBFD evaluation, consider the following for SBFD subband configurations:

- SBFD Subband configuration#1 with {DUD} pattern, which means one SBFD slot consists of one UL subband at the center of the channel bandwidth and two DL subbands at two sides of the channel bandwidth.

- SBFD Subband configuration#2 with {DU} pattern, which means one SBFD slot consists of one UL subband at one side of the channel bandwidth and one DL subband at the other side of the channel bandwidth.

- Use the following parameters for description of SBFD subband configuration in evaluation assumptions:

  - ND: the number of RBs in one DL subband

  - NU: the number of RBs in one UL subband

  - NG: the number of RBs in one guard band between one UL subband and one DL subband

Agreement

For performance evaluation and comparison between baseline legacy TDD operation and SBFD operation under SBFD Deployment Case 1 (Non-coexistence case with single SBFD subband configuration), consider the following alternatives:

- Alt 2 (No SBFD DL subband in the slots/symbols that correspond to UL slots/symbols in legacy TDD):

  - Legacy TDD: Static TDD UL/DL configuration with {DDDSU}, where S=[12D:2G:0U]

  - SBFD: Frame structure#2 (XXXXU), where X denotes a SBFD slot. In time domain, SBFD UL subband spans all the symbols in a SBFD slot. In frequency domain, SBFD UL subband is about [20%] of the channel bandwidth.

- Alt 4 (strive for the same UL/DL resource ratio between Legacy TDD and SBFD):

  - Legacy TDD: Static TDD UL/DL configuration with {DDDSU}, where S=[12D:2G:0U]

  - SBFD: Frame structure#3 (XXXXX), where X denotes a SBFD slot. In time domain, SBFD UL subband spans all the symbols in a SBFD slot. In frequency domain, SBFD UL subband is about [20%] of the channel bandwidth.

- Alt 1 (No SBFD DL subband in the slots/symbols that correspond to UL slots/symbols in legacy TDD):

  - Legacy TDD: Static TDD UL/DL configuration with {DDDSU}, where S=[12D:2G:0U]

  - SBFD: Frame structure#1 (DXXXU), where X denotes a SBFD slot. In time domain, SBFD UL subband spans all the symbols in a SBFD slot. In frequency domain, SBFD UL subband is about [20%] of the channel bandwidth.

- Alt 3 (strive for the same UL/DL resource ratio between Legacy TDD and SBFD):

  - Legacy TDD: Static TDD UL/DL configuration with {DDSUU}, where S=[12D:2G:0U]

  - SBFD: Frame structure#2 (XXXXU), where X denotes a SBFD slot. In time domain, SBFD UL subband spans all the symbols in a SBFD slot. In frequency domain, SBFD UL subband is about [20%] of the channel bandwidth.

FFS: whether dynamic TDD can optionally be used for legacy TDD for comparison.

Agreement

For gNB-gNB co-channel/adjacent-channel channel model and UE-UE co-channel/adjacent-channel channel model in RAN1 SLS,

- Large scale fading (e.g., path loss, penetration loss, shadowing) should be modelled, and companies report whether small scale fading (e.g., fast fading including antenna gain) is also modelled in their simulation.

- Note: Antenna gain is calculated based on the gNB-gNB or UE-UE LOS direction instead on the multi-path directions if fast fading is not modeled.

- FFS: how to model realistic LOS probability for gNB-gNB and UE-UE channel model.

- FFS: How to set aligned channel model amongst companies for SLS calibration (if needed).

Agreement

For gNB-gNB channel model, reuse gNB-to-UE channel model in TR 38.901 with necessary modification

- Replacing the UE’s antenna height with gNB’s antenna height, updating the angular spread

- FFS: whether/how to update LOS probability.

- FFS: Other details and necessary modifications

Agreement

LS to RAN4 on interference modelling for duplex evolution is endorsed in R1-2205543.

Agreement

For SBFD simulation, consider 4GHz for FR1 and 30GHz for FR2-1.

Agreement

For evaluation of SBFD operation, BS uses separate panels for simultaneous downlink transmission and uplink reception, we can call it separate-Tx/Rx antenna array for description of evaluation assumption.

- Companies can report the separation of the Tx panel and Rx panel assumed in their simulation.

- Companies can report how the antenna elements are used for transmission or reception in a slot if BS does not perform simultaneous downlink transmission and uplink reception.

Agreement

For evaluation of legacy TDD operation, BS uses the same antenna array for downlink transmission and uplink reception, we can call it shared-Tx/Rx antenna array for description of evaluation assumption.

Agreement

For evaluation and comparison between SBFD and legacy TDD, assume the total number of TxRUs of the antenna array for SBFD is the same as the total number of TxRUs of the antenna array for legacy TDD. Regarding antenna elements, both of the two options can be used.

- Opt 1: The total number of antenna elements of the antenna array for SBFD is the same as the total number of antenna elements of the antenna array for legacy TDD.

- Opt 2: The total number of antenna elements of the antenna array for SBFD is two times of the total number of antenna elements of the antenna array for legacy TDD.

- Companies report which option is assumed in their simulation.

Agreement

For SBFD Deployment Case 4, at least consider the following scenarios for evaluation from RAN1 perspective:

- FR1: Urban Macro

- FR2-1: Dense Urban Macro layer

- FFS: UE outdoor/indoor proportion, clustering, etc

- FFS: the grid shift between two networks, e.g., 0%, 100%

- FFS: Indoor hotspot, Dense Urban Micro layer

Subband non-overlapping full duplex

Agreement

Study whether/how to inform the UE of the time and/or frequency location of subbands that gNB would use for SBFD operation.

Agreement

Study the impact/potential enhancements of resource allocation in symbols with subbands that gNB would use for SBFD operation.

Agreement

At least study SBFD operation within a TDD carrier

Conclusion

For discussion purpose only, SBFD symbols is defined as symbols with subbands that gNB would use for SBFD operation.

Conclusion

For discussion purpose, for SBFD operation within a TDD carrier, a SBFD subband consists of 1 RB or a set of consecutive RBs for the same transmission direction.

Agreement

The time and frequency location of subbands within a TDD carrier are not fixed in the specification.

- Subject to any RAN4 guidance on minimum or maximum subband and guardband size and subband location within TDD carrier.

- Note that whether the time and/or frequency location of subbands are informed to UE is separately discussed.

Potential enhancements on dynamic/flexible TDD

Agreement

- For discussion in AI 9.3.3, consider the deployment scenarios for dynamic/flexible TDD which are agreed for evaluation purpose under AI 9.3.1 in RAN1#109-e.

- Under AI 9.3.3., no more discussion about the deployment scenario for potential enhancement on dynamic/flexible TDD

Agreement

At least, following interference scenarios can be considered for study of dynamic/flexible TDD:

- gNB-to-gNB inter-cell co-channel interference

- UE-to-UE inter-cell co-channel interference

Guideline for future meetings

- Note: AI 9.3.3 handles the potential inter-gNB and inter-UE CLI handling schemes that are specific for dynamic TDD and schemes that are common for both SBFD and dynamic/flexible TDD.

- Note: AI 9.3.2 handles the potential inter-gNB and inter-UE CLI handling schemes that are specific for SBFD.

Agreement

For study of potential enhancement to dynamic/flexible TDD and/or SBFD, followings are considered as candidates of potential enhancement method of gNB-to-gNB CLI handling, where further prioritization/down-scoping of candidate schemes for study can be done in the future meetings:

- gNB-to-gNB CLI measurement and reporting

- Coordinated scheduling

- Spatial domain enhancements

- Advanced receiver

- UE and gNB transmission and reception timing

- Power control based solution

- Potential enhancements to Rel-16 RIM

- Sensing based mechanism

- Note: Whether or not a particular scheme requires OTA or backhaul information exchange should be identified

- Note: Any other scheme(s) for inter-gNB CLI handling is/are not precluded.

- Note: For potential enhancements to dynamic/flexible TDD and/or SBFD, utilize the outcome of discussion in Rel-15 and Rel-16 while avoiding the repetition of the same discussion.

- Note: Potential enhancements specific for SBFD will be discussed in 9.3.2

Agreement

For study of potential enhancement to dynamic/flexible TDD and/or SBFD, followings are considered as candidates of potential enhancement method of UE-to-UE CLI handling, where further prioritization/down-scoping of candidate schemes for study can be done in the future meetings:

- Potential enhancements to UE-to-UE CLI measurement/reporting

- Coordinated scheduling

- Spatial domain enhancements,

- Advanced Receiver

- UE and gNB transmission and reception timing

- Power control based solution

- Sensing based mechanism

- Note: Whether or not a particular scheme requires OTA or backhaul information exchange should be identified

- Note: Any other scheme(s) for UE-to-UE CLI handling is/are not precluded.

- Note: For potential enhancements to dynamic/flexible TDD and/or SBFD, utilize the outcome of discussion in Rel-15 and Rel-16 while avoiding the repetition of the same discussion.

- Note: Potential enhancement specific for SBFD will be discussed in 9.3.2

Conclusion

The following self-interference scenario and inter-subband CLI scenarios are not considered under AI 9.3.3 (Potential enhancements on dynamic/flexible TDD).

- gNB self-interference

- UE-to-UE intra-cell co-channel inter-subband CLI

- UE-to-UE inter-cell co-channel inter-subband CLI

- gNB-to-gNB inter-cell co-channel inter-subband CLI

2.1.2	Remaining Open issues

- Identify applicable and relevant deployment scenarios (RAN1).

- Develop evaluation methodology for duplex enhancement (RAN1).

- Study the subband non-overlapping full duplex and potential enhancements on dynamic/flexible TDD (RAN1, RAN4).

- Identify possible schemes and evaluate their feasibility and performances (RAN1).

- Study inter-gNB and inter-UE CLI handling and identify solutions to manage them (RAN1).

  - Consider intra-subband CLI and inter-subband CLI in case of the subband non-overlapping full duplex.

- Study the performance of the identified schemes as well as the impact on legacy operation assuming their co-existence in co-channel and adjacent channels (RAN1).

2.3	RAN2

2.3.1	Agreements

2.3.2	Remaining Open issues

2.3	RAN3

2.3.1	Agreements

2.3.2	Remaining Open issues

2.4	RAN4

2.4.1	Agreements

2.4.2	Remaining Open issues

Work has not started for any of the objectives.

- Study the subband non-overlapping full duplex and potential enhancements on dynamic/flexible TDD (RAN1, RAN4).

- Study the feasibility of and impact on RF requirements considering adjacent-channel co-existence with the legacy operation (RAN4).

- Study the feasibility of and impact on RF requirements considering the self-interference, the inter-subband CLI, and the inter-operator CLI at gNB and the inter-subband CLI and inter-operator CLI at UE (RAN4).

- Note: RAN4 should be involved early to provide necessary information to RAN1 as needed and to study the feasibility aspects due to high impact in antenna/RF and algorithm design, which include antenna isolation, TX IM suppression in the RX part, filtering and digital interference suppression.

- Summarize the regulatory aspects that have to be considered for deploying the identified duplex enhancements in TDD unpaired spectrum (RAN4).

2.5	RAN5

2.5.1	Agreements

2.5.2	Remaining Open issues

2.5.3	Remaining Open issues with cross-WG dependencies

2.6	RAN6

2.6.1	Agreements

2.6.2	Remaining Open issues

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

3.1	SAx/CTs

3.1.1	Agreements with cross-TSG impacts

3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN1#109-e contributions:

- R1-2204301	TR 38.858 skeleton for study on evolution of NR duplex operation	CMCC

- R1-2204302	Work plan on Rel-18 evolution of NR duplex operation SI	CMCC, Samsung

- R1-2205187	TR 38.858 skeleton for study on evolution of NR duplex operation	Rapporteur (CMCC)

- R1-2205399	TR 38.858 skeleton for study on evolution of NR duplex operation	Rapporteur (CMCC)

- R1-2205188	Summary on email discussion of TR skeleton for Rel-18 SI on evolution of NR duplex operation	Moderator (CMCC)

- R1-2205310	Summary#2 on email discussion of TR skeleton for Rel-18 SI on evolution of NR duplex operation	Moderator(CMCC)

- R1-2204379	Discussion on evaluation on NR duplex evolution	NTT DOCOMO, INC.

- R1-2203156	Overview of evaluation on NR duplex evolution	Huawei, HiSilicon

- R1-2203203	Discussion of evaluation on NR duplex evolution	ZTE

- R1-2203214	Discussion for Evaluation on NR duplex evolution	New H3C Technologies Co., Ltd.

- R1-2203327	Discussion on evaluation on NR duplex evolution	Spreadtrum Communications,BUPT

- R1-2203458	Discussion on deployment scenario and evaluation methodology for duplex operation	CATT

- R1-2203557	Evaluation on NR duplex evolution	vivo

- R1-2203814	Discussion on evaluation on NR duplex evolution	xiaomi

- R1-2203903	Deployment scenario and evaluation methodology for duplex evolution	Samsung

- R1-2204021	Discussion on evaluation on NR duplex evolution	OPPO

- R1-2204053	Evaluation on NR duplex evolution	SHARP Corporation

- R1-2204068	Evaluation assumption and methodology for study on NR-duplex	InterDigital, Inc.

- R1-2204106	Evaluation of NR duplex evolution	Ericsson

- R1-2204122	Discussion on deployment scenario of NR duplex evolution	KT Corp.

- R1-2204135	Discussion on evaluation on NR duplex evolution	Panasonic

- R1-2204244	Initial evaluation on NR duplex evolution	Apple

- R1-2204303	Discussion on evaluation on NR duplex evolution	CMCC

- R1-2204430	On the evaluation methodology for NR duplexing enhancements  Nokia, Nokia Shanghai Bell

- R1-2204529	Study on Evaluation for NR duplex evolution	LG Electronics

- R1-2204721	Deployment scenarios and evaluation methodology for NR duplex evolution	MediaTek Inc.

- R1-2204750	Discussion on evaluation on NR duplex evolution	CEWiT

- R1-2204799	On evaluations for NR duplex evolution	Intel Corporation

- R1-2205030	On Deployment scenarios and evaluation Methodology for NR duplex evolution	Qualcomm Incorporated

- R1-2205311	Summary#1 on email discussion on evaluation of NR duplex evolution Moderator(CMCC)

- R1-2205312	Summary#2 on email discussion on evaluation of NR duplex evolution Moderator(CMCC)

- R1-2205313	Summary#3 on email discussion on evaluation of NR duplex evolution Moderator(CMCC)

- R1-2205540	Summary#4 on email discussion on evaluation of NR duplex evolution Moderator(CMCC)

- R1-2205541	Summary#5 on email discussion on evaluation of NR duplex evolution Moderator(CMCC)

- R1-2205542	Draft LS on interference modelling for duplex evolution	Moderator(CMCC)

- R1-2205543	LS on interference modelling for duplex evolution	RAN1, CMCC

- R1-2205031	Feasibility and techniques for Subband non-overlapping full duplex	Qualcomm Incorporated

- R1-2203459	Discussion on subband non-overlapping full duplex	CATT

- R1-2203157	Discussion on subband non-overlapping full duplex	Huawei, HiSilicon

- R1-2203204	Discussion of subband non-overlapping full duplex	ZTE

- R1-2203215	Discussion for subband non-overlapping full duplex	New H3C Technologies Co., Ltd.

- R1-2203328	Discussion on subband non-overlapping full duplex	Spreadtrum Communications

- R1-2203558	Discussion on subband non-overlapping full duplex	vivo

- R1-2203732	Adjacent Channel Interference in non-overlapping subband Full Duplex TDD operations	Sony

- R1-2203815	Discussion on subband non-overlapping full duplex	xiaomi

- R1-2203904	Subband non-overlapping full duplex for duplex evalution	Samsung

- R1-2203945	Discussion on subband non-overlapping full duplex	NEC

- R1-2204022	Discussion on subband non-overlapping full duplex	OPPO

- R1-2204054	Subband non-overlapping full duplex	SHARP Corporation

- R1-2204069	Discussion on subband non-overlapping full duplex	InterDigital, Inc.

- R1-2204107	Subband non-overlapping full duplex	Ericsson

- R1-2204156	Discussion on subband non-overlapping full duplex	Panasonic

- R1-2204245	Views on subband non-overlapping full duplex	Apple

- R1-2204304	Discussion on subband non-overlapping full duplex	CMCC

- R1-2204380	Discussion on subband non-overlapping full duplex	NTT DOCOMO, INC.

- R1-2204412	Discussion on sub band non-overlapping full duplex	CENC

- R1-2204423	Subband non-overlapping full duplex	Lenovo

- R1-2204431	On subband non-overlapping full duplex for NR	Nokia, Nokia Shanghai Bell

- R1-2204441	Discussion on sub-band non-overlapping full duplex	ITRI

- R1-2204530	Study on Subband non-overlapping full duplex	LG Electronics

- R1-2204550	Discussion on sub-band non overlapping full duplex	WILUS Inc.

- R1-2204637	Introduction of subband non-overlapping full duplex	ASUSTeK

- R1-2204651	Discussions on subband non-overlapping full duplex enhancements	ETRI

- R1-2204722	Discussion on subband non-overlapping full duplex for NR	MediaTek Inc.

- R1-2204751	Discussion on subband non-overlapping full duplex	CEWiT

- R1-2204800	Discussions on subband non-overlapping full duplex	Intel Corporation

- R1-2204866	Considerations for subband non-overlapping full duplex	Charter Communications

- R1-2205361	Summary #1 of [109-e-R18-Duplex-03] Email discussion on subband non-overlapping full duplex	Moderator (CATT)

- R1-2205520	Summary #2 of [109-e-R18-Duplex-03] Email discussion on subband non-overlapping full duplex	Moderator (CATT)

- R1-2203944	Views on enhancements of dynamic/flexible TDD	NEC

- R1-2204246	Views on potential enhancements on dynamic/flexible TDD	Apple

- R1-2203158	Potential enhancements on dynamic/flexible TDD	Huawei, HiSilicon

- R1-2203205	Discussion of enhancements on dynamic/flexible TDD	ZTE

- R1-2203216	Discussion for potential enhancements on dynamic/flexible TDD 	New H3C Technologies Co., Ltd.

- R1-2203221	Potential enhancement on dynamic/flexible TDD	TCL Communication Ltd.

- R1-2203329	Discussion on potential enhancements on dynamic/flexible TDD	Spreadtrum Communications

- R1-2203460	Discussion on potential enhancements on dynamic/flexible TDD	CATT

- R1-2203559	Potential enhancements on dynamic/flexible TDD	vivo

- R1-2203733	Enhancements to dynamic/flexible TDD for Full Duplex operation	Sony

- R1-2203816	Discussion on potential enhancements on dynamic TDD	xiaomi

- R1-2203905	Dynamic and flexible TDD for duplex evalution	Samsung

- R1-2204023	Discussion on potential enhancements on dynamic/flexible TDD	OPPO

- R1-2204056	Potential enhancements on dynamic/flexible TDD	SHARP Corporation

- R1-2204070	Discussion on enhancements of dynamic TDD operations	InterDigital, Inc.

- R1-2204076	Potential enhancements on dynamic/flexible TDD for subband full duplex	Panasonic

- R1-2204108	Flexible/dynamic TDD	Ericsson

- R1-2204305	Discussion on potential enhancements on flexible/dynamic TDD	CMCC

- R1-2204381	Discussion on potential enhancements on dynamic/flexible TDD	NTT DOCOMO, INC.

- R1-2204432	Dynamic TDD enhancements	Nokia, Nokia Shanghai Bell

- R1-2204442	Discussion on potential enhancements on dynamic/flexible TDD	ITRI

- R1-2204503	Potential enhancements on dynamic/flexible TDD	Lenovo

- R1-2204531	Study on Potential enhancements on dynamic/flexible TDD	LG Electronics

- R1-2204551	Discussion on potential enhancements on dynamic/flexible TDD	WILUS Inc.

- R1-2204638	Enhancement on dynamic TDD	ASUSTeK

- R1-2204723	Discussion on potential enhancements on dynamic/flexible TDD	MediaTek Inc.

- R1-2204752	Discussion on enhancements on dynamic/flexible TDD	CEWiT

- R1-2204801	On potential enhancements to dynamic/flexible TDD in NR systems	Intel Corporation

- R1-2205032	On potential enhancements on dynamic-flexible TDD	Qualcomm Incorporated

- R1-2205371	Summary #1 of [109-e-R18-Duplex-04] Email discussion on dynamic/flexible TDD Moderator (LG Electronics)

- R1-2205372 Summary #2 of [109-e-R18-Duplex-04] Email discussion on dynamic/flexible TDD	Moderator (LG Electronics)

- R1-2205373 Summary #3 of [109-e-R18-Duplex-04] Email discussion on dynamic/flexible TDD	Moderator (LG Electronics)

- R1-2205374 Summary #4 of [109-e-R18-Duplex-04] Email discussion on dynamic/flexible TDD	Moderator (LG Electronics)

- R1-2203206 Discussion of preliminary simulation results for NR duplex evolution	ZTE

- R1-2203222 Backhaul Signaling Reduction for Inter gNB Information exchange	TCL Communication Ltd.

- R1-2203330 Further considerations on duplex operation	Spreadtrum Communications

- R1-2203560 Other issues on NR duplex evolution	vivo

- R1-2203633 Discussion on Rel-18 duplex evolution	CATT

- R1-2203817 Other issues on NR duplex evolution	xiaomi

- R1-2204109 Inputs needed from RAN4 to facilitate RAN1 study	Ericsson

- R1-2204433 RF considerations of dynamic TDD and SBFD	Nokia, Nokia Shanghai Bell

- R1-2204914 Discussion on the work plan for the evolution of duplex operation	Huawei, HiSilicon

10.01.2022		minor adaptations for RAN #95e

04.10.2021		minor adaptations for RAN #94e

08.08.2021		minor adaptations for RAN #93e

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66