3GPP TSG RAN Meeting #96										     RP-221404

Budapest, Hungary, June 6-9, 2022

## Status Report to TSG

Agenda item:			9.4.4.5

| WI / SI Name | NR support for high speed train scenario in frequency range 2 (FR2) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_HST_FR2 |  |  |  |  |
| Unique ID | 890060 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-210800 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: n/a | Core part: 03/2022 | Performance part: 09/2022 | Testing part: n/a |  |
| Overall Completion level | Study Item: n/a | Core part: 100% | Performance Part: 80% | Testing part: n/a |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN4 |
| --- | --- | --- |
| Rapporteur | Name | WANG, He (Jackson) |
|  | Company | Samsung |
|  | Email | h0809.wang@samsung.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.4	RAN4

#### 2.4.1	Agreements

RAN4 #103-e (May. 2022, Electronic)

- General:

- A summary of the electronic meeting email discussion on FR2 HST RF maintenance was provided in [1, R4-2210244].

- A summary of the electronic meeting email discussion on FR2 HST RRM part 1 was provided in [2, R4-2210490].

- A summary of the electronic meeting email discussion on FR2 HST RRM part 2 was provided in [3, R4-2210491].

- A summary of the electronic meeting email discussion on FR2 HST Demod requirement (part 1) was provided in [4, R4-2210522].

- A summary of the electronic meeting email discussion on FR2 HST Demod requirement (part 2) was provided in [5, R4-2210523].

- TR 38.854 related maintenance:

- CR to TR 38.854 to remove the square brackets for identified requirements was endorsed in [38, R4-2209332].

- CR to TR 38.854 on Bi-directional Scenario-A Mobility Performance was agreed in [43, R4-2207880].

- CR to TR 38.854 on Throughput Performance in HST FR2 Scenarios was agreed in [44, R4-2207881].

- CR to TR 38.854 on HST FR2 RA-Based Timing Adjustment was agreed in [51, R4-2211087]

- RRM core requirement maintenance:

- The following correction CRs were agreed:

  - CR to TS 38.133 on FR2 HST neighboring cell measurement requirement was agreed in [32, R4-2211218].

  - CR to TS 38.133 on FR2 HST core requirements for removing brackets for IE was agreed in [30, R4-2208156].

  - CR to TS 38.133 for the applicability of requirement for FR2 HST was agreed in [41, R4-2211088]

  - CR to TS 38.133 for intra-freq. measurements with gaps for FR2 NR HST was agreed in [45, R4-2209521].

  - CR to TS 38.133 on UL timing adjustment was agreed in [52, R4-2211217].

- The following agreements were made on RRM core requirement maintenance, captured in approved WF [31, R4-2210608]:

  - WF on conditions when one-shot large UL timing adjustment apply

    - A need for DL timing difference threshold

GtW Agreement:

        - UE shall apply one shot large timing adjustment on TCI switching occasion if UE measurement on DL timing difference is larger than a timing difference threshold

    - The value of DL timing difference threshold

Agreement:

        - The value of DL timing difference threshold equals 4.5*64*Tc = Tq= CP/4

        - NOTE: the value of the threshold in bi-directional deployment shall be evaluated further in Rel-18.

  - WF on UL transmit timing accuracy

GtW Agreement:

        - If new TCI state within active TCI state list: Adopt ±Te immediately after TCI state switch as the accuracy otherwise ±[7Ts] adopted.

  - WF on highSpeedLargeOneStepUL-TimingFR2-r17 is not enabled

    - Requirements for the case when highSpeedLargeOneStepUL-TimingFR2-r17 is disabled

Agreement:

        - No need to introduce new UL timing requirements for the case when highSpeedLargeOneStepUL-TimingFR2-r17 is disabled.

    - Scheduling/Transmit restriction after TCI state switch

Way Forward:

        - Further discussion is needed whether and how to define transmit or scheduling restriction for UL after the TCI state switch when highSpeedLargeOneStepUL-TimingFR2-r17 is disabled.

          - Option 2: No impact on UE behavior

          - Option 3: Define scheduling restriction on DL and UL after inter-RRH TCI state switch and before PRACH transmission when highSpeedLargeOneStepUL-TimingFR2-r17 is disabled

          - Option 4: after the TCI state switch, the UE shall not transmit except for RACH preamble in the new target TCI before one of the following conditions is fulfilled:

- the new timing advance is acquired and applied in the target TCI state according to the requirements in clause 7.3;

- the UL transmission is scheduled by the gNB.

In this case, the requirements in clause 7.1.2.1 apply.

          - Other options are not precluded

  - WF on UL capabilities

    - Support of one shot large UL timing adjustment feature group

GtW Agreement:

        - Define feature as optional with capability signaling

  - WF on remaining issues in RRM requirements

    - Scheduling restriction on SSB

Agreement:

        - FR2 HST requirement is applicable only when the two SSBs from adjacent RRHs are not on the adjacent symbols.

    - Time period for PSS/SSS detection and Measurement period for intra-frequency measurements

Agreement:

        - Define Mpss/sss_sync_w/o_gaps = 18, and Mmeas_preriod_w/o_gaps = 18 for set 2.

    - SMTC limit in HST FR2 enhanced requirements

GtW Agreement:

        - Option 1 agreed as starting point and further work on the drafting CR revision including table heading and note 3.

Way forward:

        - Further discussion is needed on how to define HST FR2 requirements if SMTC periodicity is > 40 ms.

          - Option 1: Apply the FR2 HST enhanced requirement only when SMTC <=40ms cases. When SMTC period > 40ms, requirements in Table 9.2.5.2-2 apply.

          - Option 2: Delete NOTE 3, keep table titles without changes and set M2 = 1.5

          - Other options are not precluded

    - Intra-frequency measurements with gaps

Agreement:

        - For the enhancement of SA intra-frequency measurements with gaps in connected mode for FR2 HST, the same enhancement as for intra-frequency measurements without gaps should be applied.

    - L1-SINR measurements with SSB-based CMR and dedicated IMR

Way forward:

        - Open issue needs further discussion

          - Option 1: For L1-SINR measurements with SSB-based CMR and dedicated IMR configured for FR2 HST, the same enhancements as SSB-based L1-RSRP measurements should be applied.

          - Option 2: Do not define enhancement for L1-SINR

          - Other options are not precluded

- RRM performance requirement:

- The following agreements were made on RRM performance requirement, captured in approved WF [72, R4-2210609]:

WF on the scope of RRM Performance Requirement:

  - Necessity of Test Cases for Cell Re-selection Requirement

    - TC for Cell Re-selection Requirement:

      - New test case for A.7.1.1.X, Cell reselection to FR2 intra-frequency NR case for PC6 UE configured with highSpeedMeasFlagFR2-r17, DRX cycle=0.32s:

        - Test case definition and applicability:

          - Option-1: 1 test case defined from either unidirectional or bi-directional:

            - FFS unidirectional or bi-directional:

              - Consider bi-directional as starting point

          - Option-2: 2 test cases defined for unidirectional and bi-directional respectively:

            - Applicability rule is introduced: UE needs to be tested with one of the tests in either unidirectional or bidirectional.

        - SMTC period = 20 ms,

        - Propagation condition: AWGN (serving cell) and AWGN with [9722] Hz frequency offset (neighbour cell) for unidirectional or [19444] Hz frequency offset (neighbour cell) for bi-directional case.

  - Necessity of Test Cases for Re-establishment Delay Requirement

    - TC for Re-establishment Delay Requirement:

      - FFS new test case for A.7.3.2.1.X, Intra-frequency RRC Re-establishment in FR2 for PC6 UE configured with highSpeedMeasFlagFR2-r17.

        - Test case definition and applicability:

          - Option-1: 1 test case defined from either unidirectional or bi-directional:

            - FFS unidirectional or bi-directional:

              - Consider bi-directional as starting point

          - Option-2: 2 test cases defined for unidirectional and bi-directional respectively:

            - Applicability rule is introduced: UE needs to be tested with one of the tests in either unidirectional or bidirectional.

        - SMTC period = 20 ms,

        - Propagation condition: AWGN (serving cell) and AWGN with [9722] Hz frequency offset (neighbour cell) for unidirectional or [19444] Hz frequency offset (neighbour cell) for bi-directional case.

      - FFS the necessity of introducing TC for re-establishment delay requirement:

        - To be decided in next RAN4 meeting.

  - Necessity of Test Cases for Timing Related Requirement

    - TC for Timing Related Requirement:

      - TC for gradual timing adjustment requirement for FR2 PC6 UE:

        - Option 1: Need new test case (similar as A.7.4.1.1)

        - Option 2: Applying PC6 UE new requirement into A.7.4.1.1, and no need to define new test case.

        - Option 3: No new test case needed

      - TC for one shot large UL timing adjustment for FR2 PC6 UE

        - Define test case to verify one shot large UL timing adjustment requirement.

          - Option 1: New test defined in clause A.7.4.1

          - Option-2: one shot large UL timing adjustment test is combined into MAC-CE based TCI switch delay test. (The test shall verify the one shot large UL timing adjustment requirement and MAC-CE based TCI switch delay requirement.)

  - Necessity of Test Cases for SSB-based RLM and BFD

    - TC for SSB-based RLM and BFD:

      - No new SSB-based RLM/BFD test cases dedicated for FR2 PC6.

  - Necessity of Test Cases for MAC-CE based TCI State Switch Delay Requirement

    - TC for MAC-CE based TCI State Switch Delay Requirement:

      - New test case is needed for MAC CE based TCI state switching delay for FR2 PC6: A.7.5.8.X, MAC-CE based active TCI state switch for FR2 PC6 UE

        - Test case definition and applicability:

          - Option-1: 1 test case defined from either unidirectional or bi-directional:

            - FFS unidirectional or bi-directional:

              - Consider bi-directional as starting point

          - Option-2: 2 test cases defined for unidirectional and bi-directional respectively:

            - Applicability rule is introduced: UE needs to be tested with one of the tests in either unidirectional or bidirectional.

        - FFS DRX cycle = 40 ms or Non-DRX configured,

        - SMTC period = 20 ms,

        - Propagation condition: AWGN (serving cell) and AWGN with [9722] Hz frequency offset (neighbour cell) for unidirectional or [19444] Hz frequency offset (neighbour cell) for bi-directional case.

  - Necessity of Test Cases for L3 Measurement Requirement

    - TC for L3 Measurement Requirement:

      - New test case is needed for A.7.6.1.X, SA event triggered reporting test without gap for PC6 UE

        - SMTC period = 20 ms,

        - FFS DRX cycle = 40 ms or Non-DRX configured,

        - Propagation condition: AWGN (serving cell) and AWGN with 19444 Hz frequency offset (neighbour cell)

  - Necessity of Test Cases for L1-RSRP/SINR Measurement Requirement

    - TC for L1 Measurement Requirement:

      - New test case for A.7.6.3.2, SSB based L1-RSRP measurement for PC6 UE.

        - FFS DRX cycle = 40 ms or Non-DRX configured,

        - SMTC period = 20 ms,

        - Propagation condition: AWGN with 19444 Hz frequency offset

      - FFS new test case for A.7.6.6.2, SSB based L1-SINR measurement for PC6 UE.

        - FFS DRX cycle = 40 ms or Non-DRX configured,

        - SMTC period = 20 ms,

        - Propagation condition: AWGN with 19444 Hz frequency offset

      - FFS the necessity of introducing TC for SSB based L1-SINR measurement:

        - To be decided in next RAN4 meeting.

WF on scope of RRM Test Configuration and Applicability:

  - Applicability of Set-1 and Set-2 of enhanced RRM requirements

| TC category (if confirmed in Sub-Topic 1) | Applicability of set1 and set2 (either set1 with 2RX beams or set2 with 6RX beams, or both) |
| --- | --- |
| TC for Cell Re-selection Requirement | TBD |
| TC for Re-establishment Delay Requirement | TBD |
| TC for Timing Related Requirement | N/A (not differentiated by set1 and set2) |
| TC for MAC-CE based TCI State Switch Delay Requirement | TBD (for target TCI is unknown, SSB-based TCI switching to a RS with new QCL-TypeD reference) |
| TC for L3 measurement | TBD |
| TC for L1-RSRP/SINR Measurement Requirement | TBD |

  - Test configuration

    - Channel model for RRM test cases:

      - if AWGN is assumed, uni-directional should result in [9722] Hz frequency offset while bi-directional means [19444] Hz.

        - If Test setup is 1AoA based, the only difference between uni- and bi-directional should be the value of Doppler frequency offset.

WF on Measurement Accuracy Requirement for FR2 HST UE

  - Measurement accuracy for FR2 HST UE

    - Measurement accuracy for L1 and L3 measurement:

      - FFS

      - Companies are encouraged to provide simulation results in next meeting.

  - Simulation assumption further alignment for L3 measurement

    - Confirm the simulation assumptions provided in R4-2209525, except:

      - AWGN with 19444 Hz offset between desired and interfering cells, for bi-directional case

      - use SINR = -6dB only, and no need to consider “FFS: -8dB, -7dB”.

  - Simulation assumption further alignment for L1 measurement

    - Confirm the simulation assumptions provided in R4-2209525, except:

      - AWGN with 19444 Hz offset for bi-directional case

- Link simulation assumptions for L1/L3 measurement accuracy for FR2 HST scenarios in approved [69, R4-2209525].

- Demodulation performance requirement:

- The following draft CRs were endorsed, and the big-CR for 38.101-4 FR2 HST UE demod was email approved in [6, R4-2208079]

  - Draft CR on minimum requirements for PDSCH HST-DPS 38.101-4 was endorsed in [7, R4-2210910]

  - Draft CR to TS 38.101-4: Applicability rules for HST FR2 PDSCH requirements was endorsed in [8, R4-2210654]

  - Draft CR to TS 38.101-4: draft CR: FRC for PDSCH demodulation requirement for FR2 HST was endorsed in [9, R4-2209072]

  - Draft CR to TS 38.101-4: CR: FR2 HST channel model was endorsed in [10, R4-2210911]

- The following draft CRs were endorsed, and the big-CR for 38.104/141-2 FR2 HST BS demod was email approved in [11,12, R4-2208078/2210148]

  - Draft CR on HST FR2 BS applicability rule (38.141-2, Rel-17) was endorsed in [13, R4-2210912]

  - draftCR to TS 38.104 on HST FR2 Channel Conditions was endorsed in [14, R4-2210913]

  - draftCR to TS 38.141-2 on HST FR2 Channel Conditions was endorsed in [15, R4-2210914]

  - Draft CR to 38.141-2: Introduction of HST PUSCH requirements was endorsed in [16, R4-2210915]

  - Draft CR on BS Manufacturer declaration for FR2 HST for TS 38.141-2 was endorsed in [17, R4-2210916]

  - DraftCR to TS 38.104: HST FR2 PUSCH performance requirements was endorsed in [18, R4-2210656]

  - DraftCR to TS 38.104: FRC for HST FR2 PUSCH performance requirements was endorsed in [19, R4-2210657]

  - DraftCR to TS 38.141-2: FRC for HST FR2 PUSCH performance requirements was endorsed in [20, R4-2211127]

  - Draft CR for TS 38.104, Introduce performance requirements for UL TA was endorsed in [21, R4-2210917]

  - Draft CR for TS 38.141-2, Introduce performance requirements for UL TA was endorsed in [22, R4-2210918]

  - Draft CR on PRACH minimum requirements for high speed train (38.104, Rel-17) was endorsed in [23, R4-2209868]

  - Draft CR on PRACH minimum requirements for high speed train (38.104, Rel-17) was endorsed in [24, R4-2209869]

- The following agreement were made on UE demodulation requirement, captured in approved WF [25, R4-2210653], and the simulation results was noted in [26, R4-2208074]

  - WF1: Common setup

    - Noc power level

| Operating bands | n257 | n258 | n261 |
| --- | --- | --- | --- |
| Noc level (dBm/Hz) | -162.4 | -162.6 | -162.4 |

  - WF2: PDSCH requirement for Uni-directional scenario

    - Introduce PDSCH requirement with PDSCH allocation timeline for Uni-directional scenario

      - Step 1: Two RRHs of RRH#(2k), RRH#(2k+1) are assumed, and SSB#(2k mod 4) and SSB#((2k+1 )mod 4) are transmitted for each TRPs, separately, where k is the RRH number with k =0, 1, 2, ….

        - UE is configured with TCI#(2k mod 4) and TCI #((2k+1)mod 4) that are associated with TRS #(2k mod 4) and TRS#((2k+1)mod 4) transmitted from RRH#(2k) and RRH#(2k+1) respectively by RRC signalling tci-StatesToAddModList in the PDSCH-Config and tci-PresentInDCI is not configured;

        - All the configured TCI states are known to UE. UE is configured with NZP-CSI-RS resource for L1-RSRP measurements by RRC signalling nzp-CSI-RS-ResourceSet within the CSI-ResourceConfig and periodic CSI reporting by setting reportConfigType to periodic and reportQuantity to cri-RSRP (Note: reported L1-RSRP mesurements are not tested)

      - Step 2: TE actives TCI #0 for PDCCH by “TCI State Indication for UE-specific PDCCH MAC CE”;

      - Step 3: PDSCH associated with TCI #0 is transmitted during the slots from 0 to n+ THARQ + TMAC

      - Step4: In slot n TE start triggering TCI state switching command to TCI #1 by “TCI State Indication for UE-specific PDCCH MAC CE with MCS 4”;

      - Step 5: PDSCH associated with TCI #1 is transmitted in slots from n+2 + THARQ + TMAC to N.

      - PDSCH associated with TCI#(k mod 4) (k=1) is transmitted in slot from 0 to n + THARQ + TMAC

      - PDSCH associated with TCI #(k mod 4) (k=2, 3,…) is transmitted in slot from (k-1)*n+2 + THARQ + TMAC  to (k*n)+ THARQ + TMAC, where n =57600 is the number of slots between the location of (k-1)Ds- DS_offset and the location of (k)⋅DS-DS_offset. And k is the RRH number in the channel model.

      - PDCCH and PDSCH are DTXed in other slots in which throughput statistics are not considered

    - Requirement for Uni-directional scenario

      - Capture SNR = [13.7] dB into draft CR based on simulation summary R4-2208074 in RAN4#103-e meeting, interest companies are encouraged to provide updated simulation results in RAN4#104-e meeting

  - WF3: PDSCH requirement for Bi-directional scenario

    - Introduce PDSCH requirement with PDSCH allocation timeline for Bi-directional scenario

      - Step 1: Two RRHs of RRH#(2k), RRH#(2k+1) are assumed, and SSB#(2k mod 4) and SSB#((2k+1 )mod 4) are transmitted for each TRPs, separately, where k is the RRH number with k =0, 1, 2, ….

        - UE is configured with TCI#(2k mod 4) and TCI #((2k+1)mod 4) that are associated with TRS #(2k mod 4) and TRS#((2k+1)mod 4) transmitted from RRH#(2k) and RRH#(2k+1) respectively by RRC signalling tci-StatesToAddModList in the PDSCH-Config and tci-PresentInDCI is not configured;

        - All the configured TCI states are known to UE. UE is configured with NZP-CSI-RS resource for L1-RSRP measurements by RRC signalling nzp-CSI-RS-ResourceSet within the CSI-ResourceConfig and periodic CSI reporting by setting reportConfigType to periodic and reportQuantity to cri-RSRP (Note: reported L1-RSRP measurements are not tested)

      - Step 2: TE actives TCI #2 for PDCCH by “TCI State Indication for UE-specific PDCCH MAC CE”;

      - Step 3: PDSCH associated with TCI #2 is transmitted during the slots from 0 to n + THARQ + TMAC proc;

      - Step 4: In slot n TE start triggering TCI state switching command to TCI #1 by “TCI State Indication for UE-specific PDCCH MAC CE with MCS 4”;

      - Step 5: PDSCH associated with TCI #1 is transmitted in slots from n+1 + THARQ + TMAC proc + TfirstSSB + TSSB proc + TfirstTRSafterSSB + TTRS proc to 2n+ THARQ + TMAC proc

      - Step 6: In slot 2n TE start triggering TCI state switching command to TCI# 4 by “TCI State Indication for UE-specific PDCCH MAC CE with MCS 4”

      - Step 7: PDSCH associated with TCI #4 is transmitted in slots from 2n+1 + THARQ + TMAC proc + TfirstSSB + TSSB proc + TfirstTRSafterSSB + TSSB proc to 3n+THARQ + TMAC proc;

      - PDSCH associated with TCI#(2k mod 8) (k=1) is transmitted in slot from 0 to n+ THARQ + TMAC proc

      - PDSCH associated with TCI #(2k mod 8) (k=2,3, …) is transmitted in slot from (2k-2)n +1 + THARQ + TMAC proc + TfirstSSB + TSSB proc + TfirstTRSafterSSB + TTRS proc to (2k-1)n + THARQ + TMAC proc

      - PDSCH associated with TCI #((2k+1) mod 8) (k=0,1,2,…) is transmitted in slot from (2k+1)n +1+ THARQ + TMAC proc + TfirstSSB + TSSB proc + TfirstTRSafterSSB + TTRS proc to (2(k+1)n+ THARQ + TMAC proc) where n =28800 slots is the half of the number of slots between two RRHs.

      - PDCCH and PDSCH are DTXed in other slots in which throughput statistic are not considered

    - Requirement for Uni-directional scenario

      - Capture SNR = [13.9] dB into draft CR based on simulation summary R4-2208074 in RAN4#103-e meeting, interest companies are encouraged to provide updated simulation results in RAN4#104-e meeting

- The following agreement were made on BS demodulation requirement, captured in approved WF [27, R4-2210655], and the simulation results summary was noted in [28, R4-2208075]

  - WF1: PUSCH requirements

    - Requirement selection procedure

      - Ideal threshold =2.5dB, Impaired Threshold =4dB

    - Clarify Rel-16 manufacturer’s declarations and applicability rules to explicitly apply to FR1

      - Existing manufacture declaration D.109 can be reused for FR2 HST.

      - Modify this declaration to also be applicable to BS type 2-O, i.e., n/a is changed to x for BS type 2-O for D.109.

    - Implement Issue 2-2-1 in NR_HST or NR_HST_FR2

      - Use NR_HST_FR2 to implement any required FR1/FR2 distinctions starting from Rel-17.

    - Terminology to use for FR1 and FR2 distinction

      - Use either BS type 1-O or type 2-O to distinguish FR1 and FR2 in applicability rules and requirements.

  - WF on UL timing adjustment

    - SRS configuration

      - SRS Transmission periodicity

        - The intention of the prior agreements was to have SRS sent once per radio frame, in the last symbol of the first special slot of the radio frame.

        - The slot-based value of TSRS in NOTE1 of the “Test parameters for testing UL timing adjustment” table shall be scaled accordingly with the SCS, i.e., 15 kHz: TSRS =10, 30 kHz: TSRS =20, 60 kHz: TSRS =40, 120 kHz: TSRS =80.

        - It is recommended to also add this correction in Rel-16 maintenance.

#### 2.4.2	Remaining Open issues

- FR2 HST deployment scenario related:

- No outstanding issues.

- UE RF core requirement:

- No outstanding issues.

- RRM core requirement:

- No outstanding issues.

- Some remaining details will be completed during maintenance phase.

- RRM test requirement (performance part)

- Specify the RRM performance requirements of measurement accuracy.

- Specify the RRM test cases related to new core requirements:

  - TC for cell re-selection requirement

  - TC for re-establishment delay requirement (if introduced)

  - TC for gradual timing adjustment requirement for FR2 PC6 UE (if introduced)

  - TC for one shot large UL timing adjustment for FR2 PC6 UE (if introduced)

  - TC for MAC-CE based TCI state switch delay requirement

  - TC for L3 measurement requirement

  - TC for L1-RSRP measurement requirement

  - TC for L1-SINR measurement requirement (if introduced)

- Demodulation and CSI performance requirement (performance part):

- UE demodulation

  - No outstanding issues.

- BS demodulation

  - No outstanding issues.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN4 #103-e (May. 2022, Electronic)

- R4-2210244	Email discussion summary for [103-e][108] NR_HST_FR2_maintenance	         	Moderator (Samsung)

- R4-2210490	Email discussion summary for [103-e][221] NR_HST_FR2_RRM_1		          	Moderator (Nokia)

- R4-2210491	Email discussion summary for [103-e][222] NR_HST_FR2_RRM_2		        	Moderator (Samsung)

- R4-2210522	Email discussion summary for [103-e][319] NR_HST_FR2_Demod_Part1		Moderator (Samsung)

- R4-2210533	Email discussion summary for [103-e][320] NR_HST_FR2_Demod_Part2		Moderator (Nokia)

- R4-2208079     Big CR on FR2 HST UE demodulation requirement for TS 38.101-4                             Samsung

- R4-2210910     Draft CR on minimum requirements for FR2 PDSCH HST-DPS requirements       	Huawei

- R4-2210654     Draft CR to TS 38.101-4: Applicability rules for HST FR2 PDSCH requirements	Intel

- R4-2209072     Draft CR to TS 38.101-4: draft CR: FRC for PDSCH demod. req. for FR2 HST           	Ericsson

- R4-2210911     Draft CR to TS 38.101-4: CR: FR2 HST channel model				Qualcomm

- R4-2208078     Big CR on FR2 HST BS demodulation requirement for TS 38.104			Samsung

- R4-2210148     Big CR on FR2 HST BS demodulation requirement for TS 38.141-2			Nokia

- R4-2210912     Draft CR on HST FR2 BS applicability rule (38.141-2, Rel-17)			Huawei

- R4-2210913     draftCR to TS 38.104 on HST FR2 Channel Conditions				Nokia

- R4-2210914     draftCR to TS 38.141-2 on HST FR2 Channel Conditions				Nokia

- R4-2210915     Draft CR to 38.141-2: Introduction of HST PUSCH requirements			Ericsson, Samsung

- R4-2210916     Draft CR on BS Manufacturer declaration for FR2 HST for TS 38.141-2		Samsung, Nokia

- R4-2210656     DraftCR to TS 38.104: HST FR2 PUSCH performance requirements			Intel

- R4-2210657    DraftCR to TS 38.104: FRC for HST FR2 PUSCH performance requirements		Intel

- R4-2211127    DraftCR to TS 38.141-2: FRC for HST FR2 PUSCH performance requirements	Intel

- R4-2210917    Draft CR for TS 38.104, Introduce performance requirements for UL TA		CATT

- R4-2210918    Draft CR for TS 38.141-2, Introduce performance requirements for UL TA		CATT

- R4-2209868    Draft CR on PRACH minimum requirements for high speed train (38.104, Rel-17)	Huawei

- R4-2209869    Draft CR on PRACH test requirement for high speed train (38.141-2, Rel-17)		Huawei

- R4-2210653    WF on UE demodulation requirement for FR2 HST					Samsung

- R4-2208074    Simulation results summary for Rel-17 FR2 HST UE demod				Samsung

- R4-2210655    WF on BS demodulation requirement for FR2 HST					Nokia

- R4-2208075    Simulation results summary for Rel-17 FR2 HST BS demod				Samsung

- R4-2208504	Discussion on Spherical Coverage requirements for FR2 HST				ZTE

- R4-2208156	CR on FR2 HST core requirements							CATT

- R4-2210608	WF on HST FR2 RRM Core Requirement Maintenance				Nokia, Nokia Shanghai Bell

- R4-2211218	FR2 HST neighboring cell measurement requirement correction			Qualcomm, Inc.

- R4-2207818	Discussion on general aspect of FR2 HST						Apple

- R4-2208153	Discussion on remaining issues for FR2 HST						CATT

- R4-2208843	Further Discussion on capability and feature list for FR2 HST UE			Samsung

- R4-2208963	Correction on singaling name for FR2 HST						Huawei, Hisilicon

- R4-2208964	Discussion on capability for HST in FR2						Huawei, Hisilicon

- R4-2209332	CR for TR 38.854 to remove the squar brackets for identified requirements		ZTE Corporation

- R4-2209520	Discussions on remaining issues in RRM enhancements for FR2 HST 			Nokia, Nokia Shanghai Bell

- R4-2210180	Introduction of FR2 HST bands for power class 6 in TS 38.133			Ericsson

- R4-2211088	CR to TS38.133 for the applicability of requirement for FR2 HST			Samsung

- R4-2207879	On Throughput and Bi-directional Scenario-A Mobility Performance			Nokia, Nokia Shanghai Bell

- R4-2207880	CR to TR 38.854 on Bi-directional Scenario-A Mobility Performance			Nokia, Nokia Shanghai Bell

- R4-2207881	CR to TR 38.854 on Throughput Performance in HST FR2 Scenarios			Nokia, Nokia Shanghai Bell

- R4-2209521	CR to TS 38.133: intra-frequency measurements with gaps for for FR2 NR HST	Nokia, Nokia Shanghai Bell

- R4-2207890	CR to TS 38.133 on UL Timing Adjustment						Nokia, Nokia Shanghai Bell

- R4-2208346	Discussion on timing requirements for HST FR2					OPPO

- R4-2208770	Discussion on remaining issues of Timing for HST FR2				ZTE Corporation

- R4-2208845	Discussion on Remaining Issues on Timing Requirements for FR2 HST		Samsung

- R4-2208846	CR to TS38.133 for the corrections on one shot UL timing adj. for FR2 PC 6 UE	Samsung

- R4-2211087	CR to TR 38.854 on HST FR2 RA-Based Timing Adjustment				Nokia, Nokia Shanghai Bell

- R4-2211217	CR to TS 38.133 on UL Timing Adjustment						Nokia, Nokia Shanghai Bell

- R4-2207819	Discussion on signalling characteristics requirements					Apple

- R4-2207821	Draft CR for SSB scheduling restriction						Apple

- R4-2208154	Discussion on scheduling restriction on SSB for FR2 HST				CATT

- R4-2208347	Scheduling restriction on SSB								OPPO

- R4-2208769	Discussion on remaining issue of Signaling characteristics for HST FR2		ZTE Corporation

- R4-2211086	Draft CR for SSB scheduling restriction						Apple

- R4-2207820	Discussion on measurement procedure requirement for FR2 HST			Apple

- R4-2208771	Discussion on Measurement Procedure Requirements for HST FR2			ZTE Corporation

- R4-2211089	CR to TS 38.133: SSB-based L1-SINR measurements for FR2 NR HST		Nokia, Nokia Shanghai Bell

- R4-2207733	FR2 HST RRM									Qualcomm, Inc.

- R4-2208155	Discussion on RRM test cases for FR2 HST						CATT

- R4-2208348	Discussion on performance requirements for FR2 HST					OPPO

- R4-2208847	Discussion on RRM Performance Requirements for FR2 HST				Samsung

- R4-2208965	Discussion on measurement accuracy and test case for FR2 HST			Huawei, Hisilicon

- R4-2208966	Simulation results of measurement accuracy for FR2 HST				Huawei, Hisilicon

- R4-2209098	NR FR2 HST RRM performance requirements						Ericsson

- R4-2209525	Link simulation assumptions for L1 and L3 meas. accuracy for FR2 HST scenarios	Nokia, Nokia Shanghai Bell

- R4-2209526	On RRM Tests for Rel-17 FR2 HST scenarios						Nokia, Nokia Shanghai Bell

- R4-2210214	Simulation results for measurement accuracy for FR2 HST				Nokia, Nokia Shanghai Bell

- R4-2210609	WF on RRM performance requirement for FR2 HST					Samsung

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template